3GPP TSG RAN WG1 #121	R1-2503683

St Julian’s, Malta, May 19th – 23rd, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | Draft | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on SRS frequency hopping with repetition in TS 38.214 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_evo_DL_UL-Core |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding SRS frequency hopping procedure as described in section 6.2.1.1 in TS 38.214, it was specified that each of the antenna ports of the SRS resource in each slot is mapped to the same set of subcarriers within each set of R adjacent OFDM symbols. However, when comb offset hopping is enabled to the SRS resource (i.e., the higher layer parameter combOffsetHopping is configured) and the corresponding comb offset hopping pattern is symbol-level (i.e., the higher layer parameter hoppingWithRepetition is set to 'symbol'), it is intuitive that each of the antenna ports of the SRS resource in each slot is mapped to different sets of subcarriers in different OFDM symbols within each set of R adjacent OFDM symbols. More precisely, the set of subcarriers for each of the antenna ports of the SRS resource in each slot is changed symbol by symbol within each set of R adjacent OFDM symbols. Besides, for the case of frequency hopping within an SRS resource in each slot is configured without repetition (R=1), if the higher layer parameter combOffsetHopping is configured to the SRS resource, it should be clarified that different transmission comb values are assumed for different OFDM symbols. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If symbol-level comb offset hopping is configured for an SRS resource, each of the antenna ports of the SRS resource in one slot should be mapped to different sets of subcarriers in different OFDM symbols within each set of R adjacent OFDM symbols, and different transmission comb values should be assumed for different OFDM symbols. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If symbol-level comb offset hopping is configured for an SRS resource, each of the antenna ports of the SRS resource in one slot is wrongly mapped to the same set of subcarriers in different OFDM symbols within each set of R adjacent OFDM symbols, and the same transmission comb value is wrongly assumed for different OFDM symbols. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: This CR has no isolated impact on network and UE behavior. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version of this CR. |  |  |  |  |  |  |  |  |

#### 6.2.1.1	UE SRS frequency hopping procedure

For a given SRS resource, the UE is configured with repetition factor R∈{1,2,4} or R∈{1,2,3,4,5,6,7,8,10,12,14} by higher layer parameter resourceMapping in SRS-Resource where R≤Ns. When frequency hopping within an SRS resource in each slot is not configured (R=Ns), except for the case when the higher layer parameter combOffsetHopping is configured for the SRS resource and the higher layer parameter hoppingWithRepetition is set to 'symbol', each of the antenna ports of the SRS resource in each slot is mapped in all the  symbols to the same set of subcarriers in the same set of PRBs. When frequency hopping within an SRS resource in each slot is configured without repetition (R=1), according to the SRS hopping parameters , and defined in clause 6.4.1.4 of [4, TS 38.211], each of the antenna ports of the SRS resource in each slot is mapped to different sets of subcarriers in each OFDM symbol, where the same transmission comb value is assumed for different sets of subcarriers if the higher layer parameter combOffsetHopping is not configured for the SRS resource, otherwise the transmission comb value is defined in clause 6.4.1.4.3 of [4, TS 38.211]. When both frequency hopping and repetition within an SRS resource in each slot are configured (Ns ≥ 4, R ≥ 2), except the higher layer parameter combOffsetHopping is configured for the SRS resource and the higher layer parameter hoppingWithRepetition is set to 'symbol', each of the antenna ports of the SRS resource in each slot is mapped to the same set of subcarriers within each set of R adjacent OFDM symbols, and frequency hopping across the NsR sets is according to the SRS hopping parameters , and , where Ns should be divisible by R.

For operation with shared spectrum channel access in FR1, the UE does not expect that multiple hops of an SRS resource transmission are in different RB sets.

A UE may be configured Ns=2,4,8,10,12 or 14  adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across  symbols when frequency hopping is configured with R=1. A UE may be configured Ns ≥ 4 adjacent symbols aperiodic SRS resource with intra-slot frequency hopping within a bandwidth part, where the full hopping bandwidth is sounded with an equal-size subband across NsR sets of R adjacent OFDM symbols, when frequency hopping is configured with R ≥ 2, Ns ≥ R and Ns  should be divisible by R. Except the higher layer parameter combOffsetHopping is configured for the SRS resource and the higher layer parameter hoppingWithRepetition is set to 'symbol', each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each set of R adjacent OFDM symbols of the resource.

A UE may be configured symbol periodic or semi-persistent SRS resource with inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location in each slot. A UE may be configured Ns=2,4,8,10,12 or 14 symbol periodic or semi-persistent SRS resource with intra-slot and inter-slot hopping within a bandwidth part, where the SRS resource occupies the same symbol location(s) in each slot. For Ns ≥ 4, when frequency hopping is configured with R ≥ 2, intra-slot and inter-slot hopping is supported with each of the antenna ports of the SRS resource mapped to different sets of subcarriers across NsR sets of R adjacent OFDM symbol(s) of the resource in each slot, where Ns should be divisible by R. Except the higher layer parameter combOffsetHopping is configured for the SRS resource and the higher layer parameter hoppingWithRepetition is set to 'symbol', each of the antenna ports of the SRS resource is mapped to the same set of subcarriers within each set of R adjacent OFDM symbols of the resource in each slot. For Ns= R, when frequency hopping is configured, except the higher layer parameter combOffsetHopping is configured for the SRS resource and the higher layer parameter hoppingWithRepetition is set to 'symbol', inter-slot frequency hopping is supported with each of the antenna ports of the SRS resource mapped to the same set of subcarriers in R adjacent OFDM symbol(s) of the resource in each slot.

When the higher layer parameter combOffsetHopping is configured for an SRS resource and the higher layer parameter hoppingWithRepetition is set to 'symbol', each of the antenna ports of the SRS resource is mapped to the same set of PRBs within each set of R adjacent OFDM symbols of the resource in each slot and is mapped to different sets of subcarriers in different OFDM symbols within each set of R adjacent OFDM symbols of the resource in each slot as defined in clause 6.4.1.4.3 of [4, TS 38.211].