3GPP TSG RAN WG1 #121		R1-2503756

Malta, May 19th – May 23rd, 2024

Agenda Item:	9.1.4.1

Source:	Indian Institute of Technology Madras (IITM), IIT Kanpur

Title:	Discussion on Additional Study for AI/ML CSI Compression

Document for:	Discussion

# Introduction

This document includes our discussions, simulation results and observations on the Release 19 additional study on AI/ML for CSI Compression. Specifically, we provide observations and proposals related to inter-vendor training collaboration options and directions. In this section, we reproduce the relevant agreements from the prior meeting.

RAN 1 #120b [1] Agreement For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, confirm SGCS and NMSE as the type of performance metric that may be used for the performance target shared as additional information along with the exchanged dataset or the model parameters. · FFS: when to use SGCS, NMSE, and which one to use or both, and relationship with the inter-vender collaboration sub-options. · FFS: details of the format of the performance target o Option 1: Average performance target, e.g. average SGCS and/or average NMSE o Option 2: distribution of the performance target, e.g., SGCS / NMSE for 5, 10, 20, 30 percentiles, etc. · FFS: whether multiple performance targets should be exchanged for different configurations, such as antenna ports configuration, subband configuration and payload configuration, etc., along with each exchanged dataset or model parameters Agreement In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then · The same ID can be used for UE to collect UE-side target CSI for UE-side training · The same ID can be used for applicability inquiry and reporting · The same ID can be used for inference configuration · FFS: whether ID/even same ID is needed for monitoring configuration · FFS: where the ID is assigned · Note: whether the purpose for pair will be specified will be discussed separately. Agreement In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then · The same ID can be used for UE to collect UE-side target CSI for UE-side training · The same ID can be used for applicability inquiry and reporting · The same ID can be used for inference configuration · The same ID can be used for NW-side data collection · FFS: whether ID/even same ID is needed for monitoring configuration · FFS: where the ID is assigned or how the ID is specified Agreement For inter-vendor-collaboration Options 3a-1 and 4-1 in Direction A, performance target is confirmed as additional information along with the exchanged dataset or the model parameters. · FFS: type of performance metric · FFS: input data for evaluating the performance Agreement Study performance-complexity trade-off by comparing different AI/ML models, e.g. by optimizing existing designs, and/or by comparing different precoder representation in (spatial-frequency and angle-delay) or (spatial-frequency-time and angle-delay-doppler), by considering the following aspects - Performance comparison between different AI/ML designs and benchmark schemes - Complexity numbers (FLOP, calculated/projected latency or power consumption if available, measured latency or power consumption if available) of different AI/ML designs and benchmark schemes

# Inter-vendor compatibility: Proposals

Proposal 1: Consider the following performance metric choices for each of the inter-vendor collaboration options.

| Inter-vendor Option | Choice of Performance Metric |
| --- | --- |
| Option 1 (Direction C) (Fully standardize reference model) | NMSE (order of micro units) and SGCS (> 0.9) can be used For performance target, consider the same metric used during training |
| Option 2 (Standardize dataset) | NMSE (order of nano units) is preferred over SGCS (> 0.9) |
| Option 3a (Direction A - all sub-options) (Standardized reference model structure + Parameter exchange between NW-side and UE-side; parameters received at the UE or UE-side goes through offline engineering at the UE-side) | NMSE is preferred over SGCS |
| Option 3b (Direction B) (Standardized reference model structure + Parameter exchange between NW-side and UE-side; parameters received at the UE are directly used for inference at the UE without offline engineering) | NMSE is preferred over SGCS |
| Option 4 (Direction A - all sub-options) (Standardized data / dataset format + Dataset exchange between NW-side and UE-side) | NMSE is preferred over SGCS |
| Option 5a (Direction A - all sub-options) (Standardized model format + Reference model exchange between NW-side and UE-side; model received at the UE or UE-side goes through offline engineering at the UE-side) | NMSE is preferred over SGCS |
| Option 5b (Direction B) (Standardized model format + Reference model exchange between NW-side and UE-side; model received at the UE are directly used for inference at the UE without offline engineering) | NMSE is preferred over SGCS |

Proposal 2: For Direction A of inter-vendor compatibility, an ID is assigned at the NW during dataset/model parameters exchange from the NW to the UE. The same ID is shared with the UE for further communications. Note that additionally a performance target is shared by the NW to the UE.

Proposal 3: For Direction A of inter-vendor compatibility, the ID assigned during exchange of model parameters/dataset can be used for monitoring configuration by the NW.

Proposal 4: In Direction A, post offline engineering, an UE would report the status of the performance target using the same ID to the NW. The UE’s report of performance target can be made conditional. For example, UE reports a) if and only if the performance target is not met or b) whenever NW requests (periodic or aperiodic).

Proposal 5 : For Direction C of inter-vendor compatibility, a model ID is assigned for a standardised reference model. Multiple standardised reference models will have multiple IDs. In the cases of model transfer/exchange,

- Offline collaboration

- Over the Air (Case z5)

the same model ID is exchanged between the NW and the UE(s).

Proposal 6 :  For Direction C of inter-vendor compatibility, the UE(s) can use the model ID of the standardised reference model for applicability inquiry, reporting, inference configuration and monitoring purposes.

Proposal 7: For Direction A, performance target is confirmed as additional information along with exchanged dataset or model parameters. For the choice of performance metric, average NMSE or 90th percentile of NMSE, can be used.

Proposal 8: For Direction A, for performance evaluation, the UE can use a sub-set of Target CSI data which is not used during training at the UE.

# Inter-vendor compatibility: Simulation Results

This section provides various simulation results to study the performance-complexity analysis of different AI/ML models and benchmark schemes.

Observation 1 : The following table captures the performance-complexity analysis of different AI/ML models for spatial-frequency representation and loss function being NMSE.

| Dataset-Architectures | Loss -Quantization (scalar) | Total Model parameters (Enc+Dec) x105 | Model Complexity (Enc+Dec) (# flops) | Performance (SGCS) | Performance (NMSE) |
| --- | --- | --- | --- | --- | --- |
| SF1-EVCSINet | SGCS-No | 9.6+21.1 | 1.92M+1.1B | 0.9172 | 190 micro units |
| SF1-EVCSINet | NMSE-Yes | 9.6+21.1 | 1.92M+1.1B | 0.793 | 326 micro units |
| SF1-Transformer | NMSE-No | 24.8+31.8 | 59.9M+61.1M | 0.841 | 208 micro units |
| SF1-Transformer | NMSE-Yes | 24.8+31.8 | 59.9M+61.1M | 0.603 | 624 micro units |
| SF2-EVCSINet | NMSE-No | 9.6+21.1 | 1.92M+1.1B | 0.812 | 350 micro units |
| SF2-EVCSINet | NMSE-Yes | 9.6+21.1 | 1.92M+1.1B | 0.711 | 402 micro units |
| SF2-Transformer | NMSE-No | 24.8+31.8 | 59.9M+61.1M | 0.894 | 130 micro units |
| SF2-Transformer | NMSE-Yes | 24.8+31.8 | 59.9M+61.1M | 0.765 | 300 micro units |
| SF1-Benchmark: Rel 16 eType-II | No quantisation | - | - | 0.881 | - |

Note : For NMSE computation, every eigen vector is normalised with the norm of the vector and then mean squared error is computed. The average across such eigen vectors is reported as NMSE metric. For model complexity, we used TensorFlow’s legacy v1 profiler to analyse the graph and count the number of floating point operations.

Datasets used for observation-1 simulations :

SF1:  This is our own eigen-vector dataset generated using Quadriga [3]

- The number of sub-bands considered for this dataset are 13. Each sub-band is of size 4 PRBs.

SF2: This is one of the RAN4 datasets [4]

Observation 2: The following table studies the performance comparison between different AI/ML designs (CSINet [5] and EVCSINet [6]) with no quantisation.

| Model/Dataset |  | Train : CDLA-30 (AD1 for CSINet & EV1 for EVCsiNet) | Train : CDLA-300 (AD2 for CSINet & EV2 for EVCsiNet) | Train : Own Data (AD3 for CSINet & EV3 for EVCsiNet) |
| --- | --- | --- | --- | --- |
| Model | Parameter | Test : CDLA-30 | Test : CDLA-300 | Test : Own Data |
| CSINet | SGCS | 0.9973 | 0.9707 | 0.9322 |
| EVCSINet | SGCS | 0.9865 | 0.9859 | 0.9172 |

Datasets used for observation-2 simulations :

AD1:  This is a CDLA30 channel scenario angular-delay domain dataset which is publicly available at [2]

- The number of multi-paths/taps for this dataset are 5.

EV1: This is a CDLA30 channel scenario eigen-vector dataset which is publicly available at [2]

- The number of sub-bands for this dataset are 12.

AD2: This is a CDLA300 channel scenario angular-delay domain dataset which is publicly available at [2]

- The number of multi-paths/taps for this dataset are 25.

EV2: This is a CDLA300 channel scenario eigen-vector domain dataset which is publicly available at [2]

- The number of sub-bands for this dataset are 12.

EV3: This eigen-vector dataset generated for urban-macro (Uma) scenario using Quadriga [3].

- The number of sub-bands considered for this dataset are 13. Each sub-band is of size 4 PRBs.

AD3: This angular-delay domain dataset generated for urban-macro (Uma) scenario using Quadriga [3].

- The number of multi-paths/taps for this dataset are 25.

The following table lists the parameters considered to generate the AD3 & EV3 (our own) datasets

Observation 3 : The following table captures the performance comparison with different loss functions namely NMSE and SGCS using the standard architecture provided in previous RAN1 meeting.

| Dataset-Architectures | # tokens | Loss | Quantization (scalar) | Performance (SGCS) | Performance (NMSE) | Codeword |
| --- | --- | --- | --- | --- | --- | --- |
| SF1-Transformer | 64 | NMSE | No | 0.841 | 208 micro units | 64 |
| SF1-Transformer | 64 | SGCS | No | 0.65 | 24 micro units | 64 |
| SF2-Transformer | 64 | NMSE | No | 0.891 | 61 micro units | 64 |
| SF2-Transformer | 64 | SGCS | No | 0.894 | 130 micro units | 64 |
| SF2-Transformer | 64 | NMSE | Yes | 0.765 | 300 micro units | 64 |
| SF2-Transformer | 64 | SGCS | Yes | 0.699 | 700 micro units | 64 |

Datasets used for observation-3 simulations :

SF1:  This is our own eigen-vector dataset generated using Quadriga [3]

- The number of sub-bands considered for this dataset are 13. Each sub-band is of size 4 PRBs.

SF2: This is one of the RAN4 datasets [4]

Proposal 9 : It can be observed from above results, that satisfying NMSE as a performance target will satisfy SGCS also. Hence we recommend NMSE as primary performance metric for target performance for inter-vendor collaboration.

# 3 Conclusion

In this contribution, we have provided our comments and discussion details on various aspects of inter-vendor collaboration along with simulations for comparing performance-complexity trade-off between different AI/ML models. Further discussion should consider the proposals and observations that we have provided above.

# 4 References

[1] Chair Notes RAN1#120b Wuhan, China, April 2025

[2] Wireless Intelligence,” https://wireless-intelligence.com.

[3] QuaDRiGa, “QuaDRiGa : The Next Generation Radio Channel Model,” https://quadriga-channel-model.de, accessed: 2025-05-03.

[4] https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/Data_sharing/NR_AIML_air/CSI_compression

[5] C. -K. Wen, W. -T. Shih and S. Jin, "Deep Learning for Massive MIMO CSI Feedback," in IEEE Wireless Communications Letters, vol. 7, no. 5, pp. 748-751, Oct. 2018, doi: 10.1109/LWC.2018.2818160.

[6] W. Liu, W. Tian, H. Xiao, S. Jin, X. Liu, and J. Shen, “EVCsiNet: Eigenvector-Based CSI Feedback Under 3GPP Link-Level Channels,” IEEE Wireless Communications Letters, vol. 10, no. 12, pp. 2688–2692, 2021.