3GPP TSG RAN WG1 #121			R1-2503966

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.6.3

Source:	InterDigital, Inc.

Title:	Discussion on RRC CONNECTED Mode LP-WUS Monitoring

Document for:	Discussion

# Introduction

In RAN#102, a new work item [1] for low-power wake-up signal and receiver for NR has been approved. Based on the agreed work item, in RAN#103, detailed work scope for RAN4 [2] has been updated. As identified by the WID, both RRC IDLE/INACTIVE and RRC CONNECTED modes of LP-WUS are expected to be supported for NR. To this end, several agreements were made in [3-10]. In this contribution, we discuss potential enhancements for the remaining aspects of RRC CONNECTED mode LP-WUS monitoring.

# Discussions

## Enabling/Disabling and Activating/Deactivating LP-WUS Monitoring

In this section, we present our views on enabling/disabling and activating/deactivating LP-WUS monitoring.

In RAN1#116 [3], the following agreement was made on enabling/disabling and activating/deactivating LP-WUS monitoring.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

Enabling/Disabling and Activating/Deactivating LP-WUS Monitoring

To achieve desired results (e.g., saving UE power, avoiding frequent activation and deactivation of LP-WUS monitoring, and managing signaling overhead at a reasonable level), LP-WUS monitoring must be activated under appropriate conditions and should operate with proper LP-WUS monitoring configurations. The appropriate conditions can include UE experiencing good signal quality (to avoid MR activation due to low LP-WUS coverage), UE is less active, not about to handover (to avoid activating MR for cell-reselection), not moving fast, not about to experience beam failure (to avoid activating MR for BFR), etc. The configuration of LP-WUS monitoring can include LP-WUS periodicity, DRX configuration, etc. It is difficult for gNB to have up-to-date information on all these conditions. This makes gNB unable to select appropriate LP-WUS monitoring configuration and activate/deactivate LP-WUS monitoring without UE assistance. Thus, both enabling/disabling and activating/deactivating LP-WUS monitoring for UE should be performed with UE assistance.

Observation 1: LP-WUS monitoring configuration should be selected to match the conditions UE is experiencing.

Observation 2: Higher power saving can be achieved by activating LP-WUS monitoring only when UE is in appropriate conditions.

Observation 3: gNB is unable to select all the appropriate LP-WUS monitoring configuration and activate/deactivate LP-WUS monitoring without UE assistance.

Proposal 1: PDCCH monitoring triggered by LP-WUS is enabled/disabled or activated/deactivated with UE assistance.

To achieve UE power saving, LP-WUS monitoring has to be performed at least when both MR and LP-WUR experience good signal quality. To this end, activating/deactivating LP-WUS monitoring based on preconfigured thresholds on MR and LP-WUR signal quality can be used. For example, preconfigured thresholds on MR signal quality (e.g., RRM measurements and BFR measurements via MR) and LP-WUR signal quality (e.g., signal quality of LP-SS and NR-SS measured via LP-WUR) can be used for activating LP-WUS monitoring. Further, preconfigured thresholds on LP-WUR signal quality (e.g., signal quality of LP-SS and NR-SS measured via LP-WUR), can be used for deactivating LP-WUS monitoring.

Proposal 2: For LP-WUS monitoring enabled UE, support activating/deactivating LP-WUS monitoring based on at least following preconfigured conditions.

- For activating LP-WUS monitoring: Signal quality based on MR and LR measurements.

- For deactivating LP-WUS monitoring: Signal quality based on LP-WUR measurements.

It has been agreed to support both OFDM based and OOK based receivers for LP-WUS monitoring. OFDM based LP-WUR can receive both NR-SS (e.g., existing SSB or part of SSB) and LP-SS (if supported). Therefore, a UE equipped with an OFDM based receiver can use both NR-SS and LP-SS to measure signal/channel quality for activating/deactivating LP-WUS monitoring simultaneously. Given the situation, how to handle both NR-SS and LP-SS should be further discussed if OFDM based receiver is capable of monitoring both synchronization signals.

Proposal 3:  For OFDM based LP-WUR, support activating/deactivating LP-WUS monitoring based on both NR-SS (existing SSB) and LP-SS based signal/channel quality measurements.

## Fallback Operation

In RAN1#120bis [10] whether and how to support a fallback behavior for RRC CONNECTED model LP-WUS monitoring was discussed and the following two proposals were made [11].

#### Proposal 4-1a: - When LP-WUS monitoring is enabled for RRC CONNECTED mode, support UE fallback to PDCCH monitoring when UE monitors LP-WUS - FFS whether/how the condition is considered for the fallback - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior - FFS For LP-WUS operation option 1-2 Proposal 4-1b - When LP-WUS monitoring Option 1-1 is enabled for RRC CONNECTED mode, UE can fallback to PDCCH monitoring when UE monitors LP-WUS - FFS whether/how the condition is considered for the fallback - Alt1: When UE does not detect LP-WUS targeting the UE for a certain time configured by gNB (i.e., timer1): - FFS: UE needs to send feedback - FFS: For the next timer, UE monitors LP-WUS - UE reset timer1 when UE receives LP-WUS targeting the UE - Alt2: Without specified condition (by UE implementation): - FFS UE needs to send feedback - For LP-WUS operation option 1-1, UE fallbacks to legacy C-DRX behavior

The coverage of LP-WUS is limited. Consequently, LP-WUS can be susceptible to variations in the radio channel. This is applicable to both LP-WUS operation option 1-1 and option 1-2. Therefore, supporting a fallback behavior for LP-WUS monitoring, regardless of LP-WUS operation option, is desired.

In RAN1#116 [3] (agreement captured in Section 2.1), it was agreed that LP-WUS monitoring by UE should be known to gNB. Further, a UE in RRC CONNECTED state should be in full control of gNB. Therefore, fallback from LP-WUS monitoring, a gNB activated operation, should not be performed based on UE implementation. To this end, fallback from LP-WUS monitoring can be performed based on a preconfigured condition by the gNB.

As limited coverage is one of the key reasons to support fallback from LP-WUS monitoring, a LP-WUS detection based fallback condition is preferred. To this end, UE can track the duration it does not detect any LP-WUS targeting the UE (i.e., UE does not receive LP-WUS on configured MOs) by using a preconfigured timer by gNB. For example, UE can start the timer upon not detecting an LP-WUS targeting the UE in a configured MO. Each time UE detects an LP-WUS targeting the UE, UE can re-start the timer. In the case timer expires, UE can initiate fallback behavior. To make UE comply with the agreement that UE monitoring LP-WUS is aware by gNB, upon fallback from LP-WUS monitoring, UE can indicate its decision to gNB.

UE behavior upon fallback from LP-WUS monitoring

For LP-WUS operation Option 1-1, two alternative fallback behaviors are possible.

- Alt1: UE following C-DRX behavior.

- Alt 2: Continuous PDCCH monitoring.

Although both Alt 1 and Alt 2 enable UE to receive PDCCHs transmitted by gNB targeting the UE, Alt 2 is not a power efficient solution. Therefore, Alt 1 is preferred for LP-WUS Operation Option 1-1.

For LP-WUS operation Option 1-2, three alternatives are possible.

- Alt1: UE following C-DRX behavior.

- Alt 2: Continuous PDCCH monitoring.

- Alt 3: Follow LP-WUS cycle for PDCCH monitoring.

Since Alt 2 has higher power consumption compared to Option 1 and 3, it is not desired. Among Alt 1 and 3, Alt 3 will have equal or higher power consumption compared to Alt 1. This is because the period of LP-WUS monitoring with LP-WUS Operation Option 1-2 is expected to be lower compared to DRX cycle duration. Further, selecting Alt 1 for both LP-WUS operation Option 1-1 and option 1-2 results in a unified solution. Therefore, Alt 1 is preferred for LP-WUS operation Option 1-2.

Based on the discussion, we make the following observations and proposals.

Observation 4: Due to limited coverage, LP-WUS is susceptible to variations in the radio channel.

Observation 5: Based on prior agreements, LP-WUS monitoring by UE should be known to gNB.

Observation 6: Supporting fallback from LP-WUS monitoring based on UE implementation can result in undesired UE behavior in RRC CONNECTED state.

Observation 7: Limited coverage is one of the key reasons to support a fallback behavior for LP-WUS monitoring.

Proposal 4: For RRC CONNECTED mode LP-WUS monitoring, support fallback from LP-WUS monitoring based on a gNB configured timer that tracks time duration that UE does not detect LP-WUS targeting the UE.

Proposal 5: For both LP-WUS monitoring Option 1-1 and Option 1-2 in RRC CONNECTED mode, upon fallback from LP-WUS monitoring, UE follow legacy DRX behavior and indicates its decision to gNB.

## Minimum Time Gap Between LP-WUS Reception and MR to start PDCCH Monitoring

In RAN1#120 [9] and 120bis[10] following agreement was made on supporting UE capability reporting for minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

Agreement - For the X = [2, 3] candidate values for the UE capability report on the minimum time gap for each SCS, consider {0.25, 0.5, 1, 2, 3, 6, 10, 20, 30, 40}ms - Note: Other values are not precluded - Only for reported value of [20ms,] 30ms and 40ms (if supported), the reported values assume SSB periodicity of 20ms and ensures SSB(s) before PDCCH reception and during time gap - FFS: translation of minimum time gap for different SSB periodicities including different candidates for different SSB periodicities - FFS whether the same set of candidates are used for different SCSs - FFS whether different values are selected for OOK-based WUR and OFDM-based WUR - FFS whether/how to consider the case of CA - FFS: candidate values that can be indicated by UAI Agreement For the UE capability report on the minimum time gap is between the end of the last symbol of LP-WUS and the time where MR starts PDCCH monitoring regardless of SCS, support X=3 candidate values corresponding to {V1=[3,5,6], V2=[10,13,20], V3=[33,42]}ms where V1<V2<V3. - FFS: Whether to support different set of values for different receiver types

In the following, we present our views on the remaining aspects identified based on the agreements.

Whether to define minimum time gap based on periodicity of SSBs

One of the remaining aspects is whether to support different minimum time gaps for different SSB periodicities and if supported how to support different minimum time gaps. It is worth noting that in RRC CONNECTED mode LP-WUS monitoring, UEs perform all measurements, i.e., RRM, RLM, BFD, etc., by using MR. Therefore, time/frequency synchronization of MR is not impacted by RRC CONNTECTED mode LP-WUS monitoring. Consequently, time/frequency synchronization using SSBs/TRSs is not necessary for MR upon waking-up. Based on the discussion, we make the following observation and the proposal.

Observation 8: Time/frequency synchronization of MR is not impacted by RRC CONNTECTED mode LP-WUS monitoring.

Proposal 6: For RRC CONNECTED mode LP-WUS monitoring, translation of reported minimum time gap for different SSB periodicities is not supported.

Whether/how to select different minimum time gaps based on WUR type

Unlike DCP which is received via MR, LP-WUS can be received by different types of LP-WURs, i.e., OOK-based LP-WUR or OFDM-based LP-WUR. The wake-up time for different types of WURs can be different due to different shared components with MR and receiver capabilities. Based on the discussion, we make the following proposal.

Proposal 7: In RRC CONNECTED mode LP-WUS monitoring, for minimum time gap between LP-WUS reception to starting PDCCH monitoring, different sets of candidates are used for OOK-based WUR and OFDM-based WUR.

## CSI and L1-RSRP reporting

In RAN1#118-bis [7] and RAN1#119 [8], the following agreements were made on periodic CSI and L1-RSRP reporting.

Agreement For Option 1-1 of LP-WUS CONNECTED mode operation, the followings are assumed from RAN1 perspective. - … - For periodic CSI/L1-RSRP reporting, UE can be configured with one of the following (same as Rel-16 DCP) - Periodic CSI/L1-RSRP is not reported during the time given by the configured drx-onDurationTimer if UE is not indicated to wake-up - Periodic CSI/L1-RSRP is periodically reported during the time given by the configured drx-onDurationTimer regardless if UE is indicated to wake-up or not - Note: Periodic CSI/L1-RSRP is reported during CDRX active time as in legacy specification Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.

Based on the agreement, configuration of whether to wake up for reporting periodic CSI/L1-RSRP can be configured in RRC CONNECTED mode LP-WUS monitoring, as supported in Rel-16 DCP. If not configured, UE does not report periodic CSI/L1-RSRP if UE is not indicated to wake-up. Otherwise, if UE is configured by gNB to report periodic CSI/L1-RSRP regardless of waking up, UE needs to wake up to report CSI/L1-RSRP regardless of receiving LP-WUS. Although, the semi-static configuration works in case of short inactive durations, the semi-static configuration for CSI/L1-RSRP reporting may not be suitable for long inactive durations. For example, when there is a sudden change of UE (e.g., channel quality due to UE movement and, etc.), there can be a need to change the configuration, but the only method for changing the configuration is RRC reconfiguration, which requires huge overhead and latency. As supporting longer inactive duration is important to achieve higher power saving gains from LP-WUS, a mechanism which can trigger CSI/L1-RSRP reporting when inactivity periods exceed a preconfigured threshold is beneficial.

Observation 9: Configuration of periodic CSI/L1-RSRP reporting mechanism can be inefficient in case of long inactive durations.

Proposal 8: For RRC CONNECTED mode LP-WUS monitoring, introduce a mechanism to support CSI/L1-RSRP reporting triggered when inactivity periods exceed a preconfigured threshold.

## Supporting LP-SS for RRC CONNECTED mode

In RAN1#120 [9], a question was raised on whether to support LP-SS for RRC CONNECTED mode LP-WUS monitoring. It is important to note that, as LP-SS is a common signal in a cell, all UEs in the cell can receive LP-SSs. For example, among UEs in a cell performing LP-WUS monitoring, some UEs can be in RRC CONNECTED mode while some other UEs in RRC IDLE/INACTIVE mode. All these UEs can receive LP-SSs regardless of their mode of operation. Given the situation, there is no need to restrict the use of LP-SS for RRC CONNECTED model. Based on the discussion, we make the following proposal.

Proposal 9: LP-SS is supported for RRC CONNECTED mode LP-WUS monitoring.

## TCI State of LP-WUS

In RAN1#120bis [10], the following agreement was made on TCI state of LP-WUS.

Agreement The case where a UE does NOT supports Rel-17 unified TCI framework but supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, select one of the following in RAN1#121 for this case: - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them The case where a UE supports Rel-17 unified TCI framework and supports LP-WUS is supported in Rel-19. For the TCI state of LP-WUS in RRC CONNECTED mode, - When Rel-17 unified TCI framework is configured, LP-WUS follows the activated/indicated TCI-state by MAC-CE/DCI, same as other DL channels/signals (no change to DCI format) - When Rel-17 unified TCI framework is NOT configured, select one of the following in RAN1#121 for this case - Alt1: RRC provides the CORESET ID that UE shall derive the active TCI state for LP-WUS - Alt2: RRC configures K TCI states for LP-WUS and new MAC-CE for TCI activation activates one of them Same alternative to be chosen for both cases.

Based on the agreement, in the case a UE supports and configured with Rel-17 unified TCI framework, LP-WUS follows the activated/indicated TCI state by MAC-CE/DCI, same as other DL channels/signals. TCI state for LP-WUS in two cases, i.e., the case of UE supports Rel-17 unified TCI framework, but not configured, and the case of UE does not support Rel-17 unified TCI framework, are FFS. Further, for both cases, a unified solution is desired. To limit specification impact, we propose using Alt 2 for both these cases.

Proposal 10: When a UE does NOT support Rel-17 unified TCI framework, or UE supports Rel-17 unified TCI framework but NOT configured, for TCI state of LP-WUS, support Alt 2.

- Alt2: RRC configures K TCI states for LP-WUS and a new MAC-CE for TCI activation activates one of them.

# Summary

In this contribution, we discussed LP-WUS operation in RRC CONNECTED mode. Based on the discussion, we made the following observations and the proposals.

Observation 1: LP-WUS monitoring configuration should be selected to match the conditions UE is experiencing.

Observation 2: Higher power saving can be achieved by activating LP-WUS monitoring only when UE is in appropriate conditions.

Observation 3: gNB is unable to select all the appropriate LP-WUS monitoring configuration and activate/deactivate LP-WUS monitoring without UE assistance.

Observation 4: Due to limited coverage, LP-WUS is susceptible to variations in the radio channel.

Observation 5: Based on prior agreements, LP-WUS monitoring by UE should be known to gNB.

Observation 6: Supporting fallback from LP-WUS monitoring based on UE implementation can result in undesired UE behavior in RRC CONNECTED state.

Observation 7: Limited coverage is one of the key reasons to support a fallback behavior for LP-WUS monitoring.

Observation 8: Time/frequency synchronization of MR is not impacted by RRC CONNTECTED mode LP-WUS monitoring.

Observation 9: Configuration of periodic CSI/L1-RSRP reporting mechanism can be inefficient in case of long inactive durations.

Proposal 1: PDCCH monitoring triggered by LP-WUS is enabled/disabled or activated/deactivated with UE assistance.

Proposal 2: For LP-WUS monitoring enabled UE, support activating/deactivating LP-WUS monitoring based on at least following preconfigured conditions.

- For activating LP-WUS monitoring: Signal quality based on MR and LR measurements.

- For deactivating LP-WUS monitoring: Signal quality based on LP-WUR measurements.

Proposal 3:  For OFDM based LP-WUR, support activating/deactivating LP-WUS monitoring based on both NR-SS (existing SSB) and LP-SS based signal/channel quality measurements.

Proposal 4: For RRC CONNECTED mode LP-WUS monitoring, support fallback from LP-WUS monitoring based on a gNB configured timer that tracks time duration that UE does not detect LP-WUS targeting the UE.

Proposal 5: For both LP-WUS monitoring Option 1-1 and Option 1-2 in RRC CONNECTED mode, upon fallback from LP-WUS monitoring, UE follow legacy DRX behavior and indicates its decision to gNB.

Proposal 6: For RRC CONNECTED mode LP-WUS monitoring, translation of reported minimum time gap for different SSB periodicities is not supported.

Proposal 7: In RRC CONNECTED mode LP-WUS monitoring, for minimum time gap between LP-WUS reception to starting PDCCH monitoring, different sets of candidates are used for OOK-based WUR and OFDM-based WUR.

Proposal 8: For RRC CONNECTED mode LP-WUS monitoring, introduce a mechanism to support CSI/L1-RSRP reporting triggered when inactivity periods exceed a preconfigured threshold.

Proposal 9: LP-SS is supported for RRC CONNECTED mode LP-WUS monitoring.

Proposal 10: When a UE does NOT support Rel-17 unified TCI framework, or UE supports Rel-17 unified TCI framework but NOT configured, for TCI state of LP-WUS, support Alt 2.

- Alt2: RRC configures K TCI states for LP-WUS and a new MAC-CE for TCI activation activates one of them.

# References

- RP-234056, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, 3GPP RAN meeting#102, December 11th – 15th, 2023,

- RP-240801, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, 3GPP RAN meeting #103, March 18th – 21st, 2024,

- “Final report of 3GPP TSG RAN WG1 #116”, 3GPP RAN1 Meeting #116, Athens, Greece, February 26th – March 1st, 2024,

- “Final report of 3GPP TSG RAN WG1 #116bis”, 3GPP RAN1 Meeting #116bis, Changsha, Hunna Province, China, April 15th – 19th, 2024,

- “Final report of 3GPP TSG RAN WG1 #117”, 3GPP RAN1 Meeting #117, Fukuoka City, Fukuoka, Japan, May 20h – May 24th, 2024,

- “Final report of 3GPP TSG RAN WG1 #118”, 3GPP RAN1 Meeting #118, Maastricht, NL, August 19 – 23rd, 2024,

- “Final report of 3GPP TSG RAN WG1 #118bis”, 3GPP RAN1 Meeting #118bis, Hefei, China, October 14th – 18th, 2024,

- “Final report of 3GPP TSG RAN WG1 #119”, 3GPP RAN1 Meeting #119, Orlando, USA, November 18th – 22nd, 2024,

- “Final report of 3GPP TSG RAN WG1 #120”, 3GPP RAN1 Meeting #120, Athens, GR, February 17th – 21st, 2025,

- “Final report of 3GPP TSG RAN WG1 #120bis”, 3GPP RAN1 Meeting #120bis, Wuhan, China, April 07th – 11th, 2025,

- R1-2503121, “Final summary on LP-WUS operation in CONNECTED mode”, 3GPP RAN1 Meeting #120bis, Wuhan, China, April 07th – 11th, 2025.