3GPP TSG RAN WG1 Meeting #121	R1-2504002

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	Fujitsu

Title:	Discussion on uplink capacity/cell throughput enhancement for FR1-NTN

Agenda Item:	9.11.3

Document for:	Discussion

Introduction

In the WID for Rel-19 NR NTN [1], the detailed objectives for uplink capacity/cell throughput enhancements for FR1-NTN are described as follows:

- Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design

In this contribution, we provide our views on the details of UL capacity/cell throughput enhancement by using OCC.

OCC indication mechanism in Rel-19 NR NTN

In the RAN1#120-bis [2], following agreement was made for the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2.

Agreement in RAN1#120-bis For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2.

For the clarification, we summarized the characteristic of each option in Table 1. In our opinion, Option 2 is the most suitable option considering the amount of new signalling field, specification impact and OCC controllability. At first, for OCC controllability, OCC length should be controlled based on DCI similar to scheduling for DG and CG Type 2 because UL capacity is proportional to the number of UEs performing OCC. Hence, Option 1 should be excluded. Secondly, for the amount of new signalling field, we prefer to configure OCC length in RRC only because adding DCI field constantly increases the amount of processing. From this perspective, Option 3, 4, and 5 can be excluded. Moreover, in our understanding, the relationship between OCC length and actual repetition number in the Option 2 can be specified by the formula: [applied OCC length value] = min {[actual repetition number], [UE capability of OCCC length], [configured maximum value in RRC]}.
As a precaution, if repetition number is larger than applied OCC length, OCC can be applied N times within PUSCH repetition (N = floor {[repetition number]/[applied OCC length]}). As example, when OCC length 4 is applied in the cell, there is a possibility that some UEs have a performance degradation due to UE grouping. In such case, the performance degradation can be avoided by multiplexing only 2 UEs (e.g., applying [+1, +1, +1, +1], [+1, -1, +1, -1] respectively) and using de-spreading function of OCC length 2 at gNB side. We believe the specification impact of Option 2 is small because these changes are only associated with the actual repetition number.

Proposal 1: For OCC length indication for DG/CG type 2 PUSCH, support Option 2 with small specification impact, DCI-based OCC control and only adding RRC parameter.

Table 1: The characteristic of each option in agreement for OCC length

|  | New signalling field | Specification impact | OCC controllability |
| --- | --- | --- | --- |
| Option 1 | - applied value of OCC length in RRC | - definition of new signalling field | RRC-based |
| Option 2 | - max value of OCC length in RRC | - definition of new signalling field - the relationship between OCC length and repetition number | DCI-based |
| Option 3 | - candidate values of OCC length in RRC - applied value in activation/scheduling DCI | - definition of new signalling field | DCI-based |
| Option 4 | - single value in activation/scheduling DCI | - definition of new signalling field | DCI-based |
| Option 5 | - max value of OCC-length in RRC - applied value in activation/scheduling DCI | - definition of new signalling field - the association between OCC length and applied value in DCI | DCI-based |

Furthermore, in the RAN1#120-bis [2], the following agreement was made for the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2.

Agreement in RAN1#120-bis For the OCC sequence applied for OCC DG PUSCH and CG PUSCH Type 2, the sequence is indicated dynamically in DCI - FFS: with a new field or reusing an existing field

For the indication of OCC sequence, we prefer reusing an existing field to a new field in the FFS point. In our opinion, OCC sequence can be associated with DM-RS antenna port field in DCI because DM-RS antenna port should be designed to be orthogonal among OCC UEs and antenna port may be the same among OCC UEs. For example, when OCC length 2 is applied, the following sentence could be inserted in the description of the antenna port field in TS 38.212 [3], "The UE using the DM-RS port 0 applies the OCC sequence index 1, and the UE using the DM-RS port 1 applies the OCC sequence index 2". As a result, this indication can reduce signalling overhead compared with the case that a new field is added in DCI.

Proposal 2: For OCC sequence indication for DG/CG type 2 PUSCH, reusing an existing field should be supported to keep DCI payload size.

Conclusion

In this contribution, we provide our views as below.

Proposal 1: For OCC length indication for DG/CG type 2 PUSCH, support Option 2 with small specification impact, DCI-based OCC control and only adding RRC parameter.

Proposal 2: For OCC sequence indication for DG/CG type 2 PUSCH, reusing an existing field should be supported to keep DCI payload size.

References

- RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

- Chair’s notes RAN1# 120-bis.

- TS 38.212, Multiplexing and channel coding, V18.5.0, Jan, 2025.