3GPP TSG-RAN WG1 #121			R1-2504012

Malta, Malta, May 19th – 23rd, 2025

Agenda item:	9.7.1

Title:	Discussion on ISAC Deployment Scenarios

Source:	          National Institute of Standards and Technology (NIST)

Document for:	Discussion and Decision

# Introduction

A Study Item (SI) on channel modelling for integrated sensing and communication (ISAC) for NR for release 19 has been approved in RAN meeting #112 [1].

The focus and scope of this SI have been defined as follows:

At RAN1#120b, progress was made with the agreements endorsed in the ISAC session report regarding revised evaluation parameters for UAV sensing scenarios [2]. In summary, the agreements reached at RAN1#120b, as documented in the draft chair notes and the calibration summary table, are as follows:

Agreement For the purposes of large scale calibration for UAV sensing targets, the following revised calibration parameters are proposed below in Table x. Note that the change bars are against the agreements from RAN1#120. Table x. Simulation assumptions for large scale calibration for UAV sensing targets Parameters; Values. Scenario; UMa-AV. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic. Target type; UAV of small size (0.3m x 0.4m x 0.2m). Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for terrestrial UTs,. UT Tx power; 23dBm. UT Distribution; • The overall number of UTs is 30 uniformly distributed in the center cell. • All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. • Vertical distribution of aerial UE: Fixed height value of 200 m. • FR1 is assumed for aerial UE.. Sensing target distribution; 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m.. Component A of the RCS for each scattering point; -12.81 dBsm. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Coupling loss for target channel; power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2. Sensing Tx/Rx selection; Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel. Agreement For the purposes of full calibration for UAV sensing targets, the following calibration parameters are proposed below in Table x. Table x. Simulation assumptions for full calibration for UAV sensing targets Parameters; Values. Scenario; UMa-AV. Sensing mode; TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic. Target type; UAV of small size (0.3m x 0.4m x 0.2m). Sectorization; Single 360-degree sector can be assumed. Carrier Frequency; FR1: 6 GHz FR2: 30 GHz. BS antenna configurations; Single dual-pol isotropic antenna. BS Tx power; FR1: 56dBm FR2: 41dBm. Bandwidth; FR1: 100MHz FR2: 400MHz. BS noise figure; FR1: 5dB FR2: 7dB. UT antenna configurations; Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1). UT noise figure; FR1: 9dB FR2: 10dB. UT height; 1.5m for terrestrial UTs. UT Tx power; 23dBm. UT Distribution; - The overall number of UTs is 30 uniformly distributed in the center cell. - All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. - Vertical distribution of aerial UE: Fixed height value of 200 m. - FR1 is assumed for aerial UE.. Sensing target distribution; 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m.. RCS for each scattering point; Component A: -12.81 dBsm Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS. Minimum 3D distances between pairs of Tx/Rx and sensing target; 10 m. Wrapping Method; No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping. Fast fading model; TR 36.777 Annex B.1.3. (u, std) for XPR of target; Mean 13.75 dB, deviation 7.07 dB. The power threshold for path dropping after concatenation for target channel; FFS. The power threshold for removing clusters in step 6 in section 7.5, TR 38.901 for background channel; FFS. Coupling loss for target channel; By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,p ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p. Sensing Tx/Rx selection; Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel.. Absolute delay; The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref] is reused for UMa-AV for all sensing modes.. Metrics; Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996..

In this contribution, we present the calibration results for the ISAC-UAV sensing scenario within the Urban Macrocell with Aerial Vehicle (UMa-AV) scenario. These results include both Large Scale Calibration and Full-Scale Calibration for the following configurations:

- TRP monostatic 6 GHz target channel

- TRP-TRP 6 GHz channel

- TRP monostatic 6 GHz background channel

- TRP monostatic 30 GHz target channel

- TRP-TRP 30 GHz channel

- TRP monostatic 30 GHz background channel

# UAV Sensing Scenario

In the RAN1 #120bis meeting, deployment scenarios and simulation parameters for the UAV sensing target were agreed upon. The specific parameter used for calibration in this contribution for large-scale calibration and full-scale calibration are listed in Tables 1 and 2 respectively.

The ISAC channel is composed of two distinct components: the target channel, representing scattering from the sensing target, and the background channel, capturing multipath components originating from the surrounding environment. In accordance with the previous agreements, calibration is conducted separately for the target and background channels. In Sections 2.1 and 2.2, we highlight the main calibration assumptions used to obtain the calibration results in Sections 3 and 4.

Table 1: Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for terrestrial UTs, |
| UT Tx power | 23dBm |
| UT Distribution | • The overall number of UTs is 30 uniformly distributed in the center cell. • All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. • Vertical distribution of aerial UE: Fixed height value of 200 m. • FR1 is assumed for aerial UE. |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel |

Table 2: Simulation assumptions for full calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| RCS for each scattering point | Component A: -12.81 dBsm Component B1: 0 dB Component B2: 3.74 dB for standard deviation The same values are used for monostatic RCS and bistatic RCS |
| Fast fading model | TR 36.777 Annex B.1.3 |
| (u, std) for XPR of target | Mean 13.75 dB, deviation 7.07 dB |
| The power threshold for path dropping after concatenation for target channel | FFS |
| Coupling loss for target channel | By definition, need to consider all direct and indirect paths. The following parameters are included in the calculation: - power scaling factor (pathloss, shadow fading, and RCS component A included) - for small scale RCS B1/B2 and power of rays in Tx-target/target-Rx links (Pn',m',n,mk,p), Tx/Rx antenna pattern, 3 polarization matrixes, i.e., Pn',m',n,mk,pFrx,u,θθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pFrx,u,ϕθrx,n',m',ZOAk,p,ϕrx,n',m',AOAk,pTCPMrx,n',m'k,pCPMn',m',n,mk,pCPMtx,n, mk,pd112+d2222 ∙Ftx,s,θθtx,n,m,ZODk,p,ϕtx,n,m,AODk,pFtx,s,ϕθtx,n,m,ZODk,p,ϕtx,n,m,AODk,p |
| Absolute delay | The model of UMa scenario defined in TR 38.901 7-24GHz channel modeling [ref] is reused for UMa-AV for all sensing modes. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the linear sum of coupling losses between Tx/Rx and all reference points) Note: CDFs can be separately generated for target channel, background channel CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA). Definition of Delay Spread is similar to the definition of angle spread in Annex A of TR 25.996, Definition of Angle Spread can ref to Annex A of TR 25.996. |

# Target Channel

# Link selection

For both large-scale and full-scale calibration procedures, the four strongest links are selected as serving links per target realization. These selections are used for statistical evaluation and represent the links with the most significant contribution to sensing performance.  In the monostatic sensing scenario, we consider 19 TRPs, each with a monostatic link, resulting in 19 total links. In the bi-static sensing scenario, each of the 19 TRPs can form a link with the remaining 18 TRPs, yielding 19 × 18/2 = 171 links. For each simulation instance, the top 4 strongest links are retained and treated as one grouped simulation result. To compile robust statistical results, we simulate 2,000 independent target realizations, resulting in a total of 2,000 × 4 = 8,000 data points used for calibration statistics.

# Link concatenation

To simulate the ISAC target channel using existing TRs, we construct two single-segment links: one from the transmitter to the target, and one from the target to the receiver. For the target-to-receiver link, we reuse the channel parameters from the existing TRs, treating the sensing target as the transmitter and the receiver as the receiver in the reference TR configuration. For the concatenation of the two channel segments, we assume a structured one-to-one coupling between multipath components. Specifically, the line-of-sight (LOS) component of the first segment is coupled with the LOS component of the second segment. In addition, the LOS of the first segment is associated with all non-line-of-sight (NLOS) components of the second segment, and conversely, the LOS of the second segment is associated with all NLOS components of the first segment. The remaining NLOS components from both segments are randomly paired in a one-to-one fashion.

# Background Channel

Regarding the background channel calibration, we assume that arrival angles are equal to departure angles, and rays with zenith angles greater than 80 degrees are dropped. For large-scale calibration, the number of reference points, Nrp is set to Nrp=1 so that one reference point is randomly dropped following the agreed distribution. For full-scale calibration, the number of reference points is set to Nrp=3 and the channel amplitudes are scaled by 3​.

# Calibration Results Target Channel for UMa-AV

# Target Channel: TRP monostatic

# Large Scale

| TRP monostatic - 6GHz | TRP monostatic - 30GHz |
| --- | --- |

Proposal 1: Include our TRP monostatic large-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# Full Scale

| TRP monostatic - 6GHz | TRP monostatic - 30GHz |
| --- | --- |

Proposal 2: Include our TRP monostatic full-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# Target Channel: TRP-TRP

# Large Scale

| TRP-TRP - 6GHz | TRP-TRP - 30GHz |
| --- | --- |

Proposal 3: Include our TRP-TRP bi-static large-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# Full Scale

| TRP-TRP - 6GHz | TRP-TRP - 30GHz |
| --- | --- |

Proposal 4: Include our TRP-TRP bi-static full-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# Calibration Results Background Channel for UMa-AV

# Background Channel: TRP monostatic

# Large Scale

| TRP monostatic background - 6GHz | TRP monostatic background - 30GHz |
| --- | --- |

Proposal 5: Include our large-scale background channel calibration results for the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# Full Scale

| TRP monostatic background - 6GHz | TRP monostatic background - 30GHz |
| --- | --- |

Proposal 6: Include our full-scale background channel calibration results for the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# Conclusion

In summary, our proposals are listed as follows:

Proposal 1: Include our TRP monostatic large-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

Proposal 2: Include our TRP monostatic full-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

Proposal 3: Include our TRP-TRP bi-static large-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

Proposal 4: Include our TRP-TRP bi-static full-scale calibration results for UAV sensing targets in the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

Proposal 5: Include our large-scale background channel calibration results for the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

Proposal 6: Include our full-scale background channel calibration results for the UMA-AV scenario to support ISAC channel model calibration efforts and incorporate them in the TR.

# References

| [1] | RP-234069, “New SID: Study on channel modelling for Integrated Sensing And Communication (ISAC) for NR,” Edinburgh, UK, December, 2023 (RAN #112). |
| --- | --- |
| [2] | R1-2502734, FL Summary #4 on ISAC Scenarios and Calibrations, Wuhan, China, April, 2025 (RAN1#120b). |