3GPP TSG-RAN WG1 #121			R1-2504013

Malta, Malta, May 19th – 23rd, 2025

Agenda item:	9.7.2

Title:	Discussion on ISAC Channel Modeling

Source:	          National Institute of Standards and Technology (NIST)

Document for:	Discussion and Decision

# Introduction

In 3GPP TSG RAN#120bis, the feature lead outlined the following items in their priority orders, which could be considered in the next meeting [1].

# Physical Object Model

In the RAN1 #120bis meeting, the following modeling principles for bistatic RCS were proposed as a working assumption.

In this section, we validate the working assumption against our measurement data and investigate extending the working assumption for a large-size UAV.

## 2.1 Large-size UAV

Recall that in our measurement campaign [2], we characterized a UAV with dimensions of 1.1 m in length, 1.2 m in width, and 0.5 m in height, as shown in Figure 2-1. To simulate operational conditions, we illuminated the underside of the UAV. The UAV was kept stationary and powered off during the measurements. This inverted setup was mounted on a support structure connected to a rotator, as illustrated in Figure 2-1 (c), which allowed for controlled rotation during the measurements. To simulate its typical operational state, we placed the battery on top of the UAV and folded the legs to reflect the compact form used during transportation.

In this setup, the TX and RX heights were fixed at 1.3 m. A total of nine configurations—one pseudo-monostatic and eight bistatic—were arranged by moving the TX while maintaining the RX fixed, as shown in Figure 2-2. Per configuration, the UAV rotated around its z-axis, forming an aspect angle β that varied from 0∘ to 180∘ in 5∘ increments. Per UAV rotation, we captured a total of 37 channel acquisitions with a sampling rate of 78 milliseconds (ms), over a total measurement time of 2.88 seconds. Note that acquisitions were made only when the target was static.

Figure 2-1: UAV under measurement

Figure 2-2:  Floor plan and bistatic configuration

### 2.2.1 Bi-static RCS

To extend the monostatic RCS model for a bistatic configuration, we start with the NIST UAV monostatic RCS model shown in Figure 2-3(a), which is a 2D RCS pattern with the zenith angle fixed at 95 degrees.

The following equation is adopted to extend the monostatic RCS pattern to obtain the bistatic RCS value for a given incident angle (θi,ϕi) and scattering angle (θs,ϕs) pair.

The monostatic model was agreed upon in RAN#120 as

rcsmonoθi,s,  φi,s= Gmax-min-σdBVθi,s+σdBHφi,s , σmax,

where θi,s and φi,s are the incident and scattering azimuth and elevation angles, respectively.

The working assumption for the vehicle bistatic RCS is modelled as

rcsdBθi,ϕi,θs,ϕs=max⁡(rcsmono(θ,  φ)-K1*sin⁡(K2*β/2), -Inf) 	(Equation 2.1)

In this equation, the angle (θ,ϕ) is defined as the projection of the bisector angle on the vertical plane and the horizontal plane, respectively, and β is the bistatic angle. We searched the parameters K1 and K2 by minimizing the overall fitting MSE, and obtained the optimal values as K1=4.09 and K2=0.8. The fitted results are shown in Figure 2-3 (b)-(h).

Proposal 1: When extending the monostatic UAV RCS model to develop its bistatic RCS, based on the work assumptions proposed in RAN1#120b, we recommend modifying the K1 and K2 parameters. Specifically, we suggest setting K1 to 4.09 and K2 to 0.8 to minimize fitting errors based on our measurements.

In Figure 2-3(b)-(h), we overlay the measurement results with the computed RCS model. Each RCS polar plot corresponds to a specific bistatic angle. The polar plot illustrates the RCS values for aspect angles ranging from 0 to 180 degrees.

From these figures, we observe that when the bistatic angle is relatively small (no greater than 90 degrees), the two patterns match well. However, with the increase of the bistatic angle, the fitting of the bistatic model to the measurements degrades significantly. The MSE  between the measurements and the bistatic model is shown in Fig. 2-4.

(a) Monostatic RCS pattern (A*B1)           (b) 9-degree bistatic angle (TX1)             (c) 22.5-degree bistatic angle  (TX2)

(d) 45-degree bistatic angle (TX3)                   (e) 67.5-degree bistatic angle  (TX4)             (f) 90-degree bistatic angle (TX5)

(g)112.5-degree bistatic angle (TX6)                   (h) 135-degree bistatic angle (TX7)

Figure 2-3: Comparison of the measurements (blue) with the bistatic RCS model (red). Each sub-figure corresponds to a specific bistatic angle, and the polar plot illustrates the RCS for aspect angles ranging from 0 to 180 degrees.

Figure 2-4: MSE between the bistatic model and measurements for UAV at various bistatic angles.

### 2.1.2 Angular Correlation of RCS Component B2

We extract the small-scale RCS component, B2, by eliminating the contribution of the bistatic RCS model (Equ. 2.1), i.e., A*B1, from the measured RCS σtgt. This process can be expressed mathematically in the linear scale as:

|  | B2t=σtgtA×B1(θit, ϕi(t),θst, ϕs(t)), | (Equation 2.2) |
| --- | --- | --- |

In Figure 2-3, B2 is essentially the difference between the blue curve (the RCS measurement) and the red curve (the computed A*B1)

Figure 2-5: Angular correlation of the RCS B2 component for a UAV. The angle refers to the aspect angle of the UAV. The red curve represents the exponential fitting of the averaged correlation (black curve).

Figure 2-5 presents the RCS correlation between the aspect angles of a UAV, across seven fixed bistatic angles, ranging from 9 degrees to 135 degrees.

## 2.2 Vehicle

Recall that in our vehicle measurement campaign [2], the modelled vehicle is a Ram ProMaster 2500 Van, with dimensions of 5.8m (length) x 2m (width) x 2.5m (height), as shown in Figure 2-6(a). The measurement is taken at 28.5 GHz.

Figure 2-6: Vehicle and its measurement floor plan

The floor plan in Figure 2-6(b) illustrates the experimental setup. This setup involves a stationary vehicle and a mobile robotic platform carrying the TX. The TX was mounted on the top of the robot, and its antenna board was deployed in the side direction of the robot to face the vehicle under test.  The RX is positioned at multiple fixed locations (RX1 to RX8). For each RX location, the TX follows a predefined trajectory (shown in red). The test subject, a vehicle, remains stationary (trunk is towards the TX start point) throughout all configurations to ensure consistency in the data collection. The RX positions are carefully distributed around the vehicle to capture different bistatic angles and reflection characteristics of all the sides (front, back, right, and left sides).

### 2.2.1 Bi-static RCS

Figure  2-7 compares the measured RCS with the simulated value using the bistatic RCS mode (Equ 2.1) for RX1 to RX8. The purple dots represent the RX locations, and the polar plot presents the RCS values corresponding to various TX incident angles. The head of the vehicle is zero degrees, and the back is 180 degrees.

Figure 2-7:  Measurements (blue) vs. bistatic RCS Models (K1 =6, K2=1.62) for RX1 to RX8. The purple dots represent the RX locations, and the polar plot presents the RCS values corresponding to various TX incident angles.

Figure 2-7 demonstrates that the bistatic RCS model accurately captures the fundamental RCS pattern when the bistatic angle is relatively small. However, the accuracy of this fitting significantly decreases as the bistatic angle increases.

### 2.2.2 Angular Correlation of RCS Component B2

Using Equation 2-2, we obtain B2, the difference between the measurement and the A*B1 value. Figure 2-8 displays the B2 correlation across all eight RX locations. The ACF is computed regarding TX movement along the trajectory in Figure 2-6(b).

Figure 2-8: Distance correlation of the RCS B2 component for a Van. The red curve represents the exponential fitting of the averaged correlation (black curve).

Proposal 2: The fitting performance of the bistatic model degrades when the bistatic angle exceeds 90 degrees. This indicates that when the bistatic angle becomes large, particularly when it approaches 180 degrees, the bistatic RCS needs to be modeled differently.

Proposal 3: RCS B2 angle correlation and distance correlation have been observed using measurement data, which suggests that B2 correlation should be considered when taget is moving relative to the TX or RX.

# Blockage Model

In RAN1 #120bis meeting, a working assumption was made to not treat RCS values in forward scattering direction in Rel-19 by having RCSfs=-Inf. In this contribution, we explore the reuse of blockage model B from TR38.901 to model the shadow in the forward scattering region. The study is based on our vehicle measurements. Additionally, we propose an enhancement to blockage model B by adapting the originally fixed-size 2D screen to represent a 3D physical object, taking into account its size and orientation.

To modify the screen size in blockage model B, we project the 3D bounding box of the object onto the plane orthogonal to the blocked ray, and the screen size is approximated by the maximum blocked extent in a way consistent to the original model. The proposed modification is compared with the original and the measured shadow gain in the 28.5 GHz vehicle measurement campaign previously mentioned in section 2.2. The RX position 7 which is opposite to the TX trajectory with clear blockage is selected to study the blockage model, shown in Figure 3-1. Three different types of vehicle-under-tests (VUT) were studied: small, medium and large shown in Figure 3-2.

Figure 3-1: Vehicle measurement campaign setup for RX location 7

Figure 3-2: Three types of vehicle-under-tests with their respective size

## 3.1 Modified blockage model B

### 3.1.1 Variable screen size

Step a: Define the path parameters and blocker parameters

- RX position is defined as xr,yr,zrT

- TX position is defined as xt,yt,ztT if LOS ray, or the centroid of ST if NLOS sensing ray

- Blocker position xk,yk,zkT is the centroid of the blocker with size defined as (dx,k,dy,k,dz,k) and rotation (αk,βk,γk). k is the blocker index for K blockers kept consistent with the original blockage model B

Step b: Calculate adjusted screen size

- Calculate rotation matrix (R) for the blocker from the blocker rotation (αk,βk,γk) using 7.1.3 in TR38.901

- Calculate the three basis column vectors of the blocker B=bx,by,bz=R×diag(dx,k,dy,k,dz,k)

- Calculate unit vector of the ray from TX and RX position,  ρray=xr-xt,yr-yt,zr-ztT, or using covert the path azimuth and zenith to cartesian coordinate with eq. 7.1-6 in TR38.901.

- Calculate unit vector first dimension (width) of the blocking screen ρw=ρray×0,0,1T‖ρray×0,0,1T‖

- Calculate unit vector second dimension (height) of the blocking screen ρh=ρw×ρray‖ρw×ρray‖

- Calculate maximum extent in first and second dimension of the blocker dw,dh=(maxBTρw,maxBTρh)

Step c: Determine the blockage attenuation to the path

- Calculate blockage model B attenuation (LdB) using step b in 7.6.4.2 in TR38.901 with screen size (dw,dh).

The shadow gains simulated using both the original blockage model B with screen size equal to blocker width and height, and the proposed variable screen blockage model B are compared in Figure 3-3. The path gains with free-space power removed from the vehicle measurement campaign is plotted (black) to validate the models. The channel is sampled 20 times per each TX location, and the minimum-maximum range across 20 samples in dB is added to the plot as confidence interval. The x axis is the incident angle relative to the heading of the car. As TX is located behind the car back at the start and travel counterclockwise, the incident angle varies from 180° to 360°.

Figure 3-3: Measured path gain (black) with min-max interval, original blockage model B (blue), and variable screen blockage model B (orange)

### 3.1.2 Diffraction pattern of blockage

Attenuation calculated in TR38.901 blockage model B has the origin from the rectangular aperture model and Babinet’s principle also documented in ITU-R P.526-15 [3] under section 5.2.1. With the complex Fresnel cosine and sine integrals being computationally complex with fluctuation unnecessary for the communication channel model, the integrals are approximated using arctan function. Although complex, Boersma had published an approximation formula for Fresnel integrals with polynomial, also documented in the same ITU-R P.526-15 section 2.7. This subsection explores the possibility of using Fresnel integral recommended in ITU-R in the blockage model for ISAC.

Using the variable screen blockage model B geometry previously proposed, the shadow gain calculated from Fresnel integrals is compared to the arctan approximation and presented in Figure 3-4. Fresnel integrals evaluated here uses MATLAB built-in fresnels and fresnelc functions for ease of use.

Figure 3-4: Measured path gain (black) with min-max interval, variable screen blockage model B with arctan approximation (orange), and variable screen blockage model B with Fresnel integral (yellow).

## 3.2 Discussion

The estimated shadow gain using blockage model B with the original fixed screen and the proposed variable screen presented in Figure 3-3(a) appear to be more severe than the measured shadow gain in small car which can be explained by the sedan shape which is not well-represented by its bounding box. The effect of Sedan shape is also confirmed from the shadow gain biasing toward 180° while the vehicle center is at 270°. The mean strongest attenuation of the small car is around 18 dB at 265°, possibly caused by the constructive interference of diffractions antiphase to the free space. Despite not fitting perfectly, the proposed variable screen blockage B resembles the measured data more than the original version. The effect of using Fresnel integral instead of arctan approximation does not improve the estimation for sedan, shown in Figure 3-4(a).

The SUV on the other hands, with the width and depth similar to a sedan but taller, shows better agreement at the shadow onset at 220° in Figure 3-3(b). The agreement can be explained by the more boxlike shape of the SUV back side compared to the sedan. However, both versions of blockage model B do not fit well in the car front side similarly to sedan. Deep shadow is observed at 235° and 290°. The deep shadows can also be explained by the constructive interference of antiphase diffraction rays, in which the estimation using Fresnel integral in Figure 3-4(b) also predicts similar dips. The peak at 275° is likely from the propagation through window which the TX/RX at 1.3 m are located at around the same height as the window center shown in Figure 3-5. The decay rate and depth in Figure 3-4(b) at the onset also shows better prediction.

Figure 3-5: RX height coincides with the SUV window center

With the van being the largest and boxlike than sedan and SUV, the attenuation is much stronger at around 30 dB for this configuration shown in Figure 3-3(c). With the van back side being boxlike similar to SUV, we may expect the adjustable screen model to estimate the shadow onset well. However, the estimated onset is off by 10° and the original model shows closer prediction. A possible explanation owing to the estimated screen size smaller than the measured data is the object thickness (size along the propagation direction). While the effect of thickness can be small for longer propagation distance, the distance in the conducted measurement is only a few multiples of the van thickness. One way to model the effect of blocker thickness is to consider a multiple knife-edge model such as Deygout model or Bullington model. The diffraction pattern estimated using Fresnel integrals shown in Figure 3-4(c) show some correlation with the measured data, showing stronger attenuation in the back side (220°~260°) and weaker attenuation in the front side (270°~320°).

Observation 1: The original blockage model B does not account for the 3D shape of the blocker, which can reduce modeling accuracy for vehicles, particularly when the propagation occurs in the front-back direction.

Proposal 4: To model a 3D physical object using the Blockage B model, RAN1 could consider adjusting the screen size based on the physical object's size and orientation, following the procedures described in Section 3.1.1.

Observation 2: The blockage model may overestimate the shadowing effect caused by a vehicle; for example, signal penetration through vehicle windows can reduce the attenuation from blockage.

Observation 3: The diffraction pattern can be modeled using the Fresnel integral rather than relying on the approximation provided by the arctan function. This approach allows us to better infer the shape of the blocker but need more accurate screen model.  The rectangular screen model used in blockages A and B does not accurately represent most objects, as these objects often exhibit dielectric properties or have non-rectangular silhouettes.

# Micro Doppler

Micro-Doppler models the internal motion of a target, which is due to non-rigid body motion or vibration of a target, such as rotating blades of a drone, breathing, or heartbeat. Micro-Doppler is important for drone detection and classification, heart rate/breathing monitoring. Existing discussions on micro-Doppler characterization, such as those described in [4], primarily rely on simplified sinusoidal models for respiration and heartbeat.

This contribution proposes the physiologically driven displacement model for chest micro-motion due to respiration and cardiac activity proposed in [5] [6]. The proposed model provides greater fidelity to human motion by capturing the asymmetry and non-sinusoidal nature of respiratory cycles and the periodic dynamics of heartbeat. These motion profiles are expected to improve the accuracy of modelling human vital signs. In addition, the model is fully parametric, enabling simulation of a range of cardiopulmonary behaviours typically observed in adult humans under both normal and pathological conditions. This flexibility makes it particularly suitable for evaluating ISAC systems across diverse populations and use cases involving health monitoring or human target recognition.

In the remainder of this contribution, we support the proposed model with monostatic radar measurements conducted using a 28.5 GHz channel sounder in a semi-anechoic chamber. The measurements capture the RF response of a human subject positioned 2 meters from the radar. Ground-truth vital sensor data is used to parameterize and validate the model. The resulting simulation closely replicates the subject’s breathing pattern and heart rate and demonstrates excellent agreement with the measured phase evolution over ten breathing cycles.

## 4.1 Measurement setup

| Figure 4-1: Measurements geometry | Figure 4-2: Photograph during measurement campaign |
| --- | --- |

The measurements utilized a single dual-polarized phased-array board of 64 microstrip antennas at the transmitter (Tx), and four of the same boards stacked side-by-side (256 microstrip antennas) at the receiver (Rx), a 360° azimuth camera, lidar and vital sensors as described in [6] [7]. For the measurements described in this contribution, a human sat 2 m from the Rx. The geometry and a photograph of the measurement campaign are shown in Figure 4-1 and Figure 4-2, respectively. Samples were collected every 26ms with the subject breathing at a normal rate for 25s while vital signs were monitored. The measurement data comprised time of arrival, angle-of-arrival in the elevation and azimuth planes, path loss, and phase for each multipath component (MPC). After gating the MPC around the chest area, the MPCs are complex summed.

## 4.2 Model

The proposed physiologically based chest displacement model [6] is composed of two components: one capturing respiration and the other capturing heartbeat-induced micro-motion. The model is defined as follows.

### Respiration Motion

The breathing displacement function is modelled as a composite of a parabolic inhalation and an exponential exhalation profile:

rbreatht=-rmaxTiTet2+rmaxTTiTet,  &t≤Tirmax1-e-Teτ e-t-Teτ-e-Teτ,  &Ti<t≤T                         Equation 4-1

Where rmax is the maximum chest displacement due to breathing, Ti is the inspiratory time Te is the expiratory time, T= Ti+ Te is the full respiration cycle period and τis the exhalation decay constant.

### Heartbeat Motion

The heartbeat-induced displacement is modelled using a sinusoidal function based on the subject’s heart rate:

rheartt=rh,maxsin2πtHR60      		Equation 4-2

Where rh,max is the maximum displacement due to cardiac motion and HR is the heart rate in beats per minute. The total displacement of the chest is thus:

rtotalt=rbreatht+rheart(t).			Equation 4-3

### Parameter Extraction

| Figure 4-3: Power spectral density of measured phase [6]. | Figure 4-4: Mathematical model realization [6]. |
| --- | --- |

The RF measurements were used to extract parameters. Fig. 4-3 (red solid line) plots the power spectral density (PSD) of the total measured phase Φtotal. Normal human breathing rates at rest fall between 12 breaths/min and 30 breaths/min and are indicated by the blue box in Fig. 4-3. Similarly, the yellow box in Fig. 4-3 denotes the frequency range of interest due to the heart beating at a resting normal rate (i.e., 60-120 bpm). The peak in the breathing region corresponds to a breathing rate of ~25 breaths/min (0.42 Hz). The heartbeat region shows the dominant heartbeat frequency as 75 bpm (1.25 Hz), which matched the ECG data collected simultaneously.

rmax, rh,max , and breath rate were estimated from the fluctuations in the total received phase. The maximum chest displacement due to breathing, rmax, was estimated from the peak power in the breathing region as 3.5 mm. Since the dominant heartbeat frequency coincided with the third harmonic due to breathing, it is not possible to directly predict the rh,max from the PSD of the measured phase alone. Hence, the phase shift due to rtotal, Φtotal=2πrtotal/λ was defined, where λ is the signal wavelength. Φtotal PSD was plotted against the measurement as shown by the solid green line in Fig. 5.3. By comparing the two PSDs, it was found that rh,max=0.88 mm provided the best match of the power level at 1.25 Hz.

The other parameters estimated to fit the model to measurements: Te=3.125s, Ti=1.875s, τ=Te5.

With these parameters the model in Figure 4-4 is obtained.

## 4.3 Validation of the model

| Figure 4-5: Phase comparison [6] | Figure 4-6: PSD comparison [6] |
| --- | --- |

Excellent agreement between the phase (Figure 4-5) and the PSD (Figure 4-6) show that we have accurately modelled the breathing and heartbeat displacement.

Proposal 5: Equation 4-1 to 4-3 provide a high-fidelity model for body movement regarding human respiration and heartbeat. The corresponding micro-Doppler shift can be computed as follows:

fmicro=1 λ [-drtotaltdt r1(t)-drtotaltdt r2(t) ],

where λ is the wavelength, rtotalt is the total displacement of the chest, and  r1(t) and r2(t) are the unit radial vector from TX to the human subject and from RX to the human subject, respectively.

# Conclusion

In this contribution, we discussed physical object modelling regarding bi-static RCS and angular and distance correlations of RCS component B2. We studied blockage models by considering target as a blocker, and validated the models against our measurements. Finally, we proposed a high-fidelity micro-Doppler regarding human reparation and heartbeat. Our observations and proposals are summarized as follows:

Proposal 1: When extending the monostatic UAV RCS model to develop its bistatic RCS, based on the work assumptions proposed in RAN1#120b, we recommend modifying the K1 and K2 parameters. Specifically, we suggest setting K1 to 4.09 and K2 to 0.8 to minimize fitting errors based on our measurements.

Proposal 2: The fitting performance of the bi-static RCS model degrades when the bistatic angle exceeds 90 degrees. This indicates that when the bistatic angle becomes large, particularly when it approaches 180 degrees, the bistatic RCS needs to be modeled differently.

Proposal 3: RCS B2 angle correlation and distance correlation have been observed using measurement data, which suggests that B2 correlation should be considered when taget is moving relative to the TX or RX.

Observation 1: The original blockage model B does not account for the 3D shape of the blocker, which can reduce modeling accuracy for vehicles, particularly when the propagation occurs in the front-back direction.

Proposal 4: To model a 3D physical object using the Blockage B model, RAN1 could consider adjusting the screen size based on the physical object's size and orientation, following the procedures described in Section 3.1.1.

Observation 2: The Blockage model may overestimate the shadowing effect caused by a vehicle; for example, signal penetration through vehicle windows can reduce the attenuation from blockage.

Observation 3: The diffraction pattern can be modeled using the Fresnel integral rather than relying on the approximation provided by the arctan function. This approach allows us to better infer the shape of the blocker but may need more accurate screen model.  The rectangular screen model used in blockages A and B does not accurately represent most objects, as these objects often exhibit dielectric properties or have non-rectangular silhouettes.

Proposal 5: Equation 4-1 to 4-3 provide a high-fidelity model for body movement regarding human respiration and heartbeat. The corresponding micro-Doppler shift can be computed as follows:

fmicro=1 λ [-drtotaltdt r1(t)-drtotaltdt r2(t) ],

where λ is the wavelength, rtotalt is the total displacement of the chest, and  r1(t) and r2(t) are the unit radial vector from TX to the human subject and from RX to the human subject, respectively.

# References

| [1] | (Xiaomi) Moderator, R1-2502556, "Summary #4 on ISAC channel modelling," Wuhan, China, April, 2025 (RAN1 #120bis). |
| --- | --- |
| [2] | R1-2502572, "Discussion on ISAC Channel Modeling," NIST, RAN#120-bis, April 2025. |
| [3] | ITU-R P.526-15, "Propagation by diffraction," 10/2019. |
| [4] | V. C. Chen, The micro-Doppler effect in radar, Artech house, 2019. |
| [5] | A. Albanese, L. Cheng, M. Ursino and N. Chbat, "An integrated mathematical model of the human cardiopulmonary system: model development.," American Journal of Physiology-Heart and Circulatory Physiology, vol. 310, no. 7, pp. H899-H921, 2016. |
| [6] | S. Mukherjee, B. M. Hardy, G. J. Skidmore, T. Chawla, J. Bang, J. Chuang, J. Senic, S. Berweger, S. Blandino and C. Gentile, "Animating Vital Signs in Radar Simulations: Comparing Physical Optics Against 28.5 GHz Channel Measurements," in 2024 IEEE Radar Conference (RadarConf24), 2024. |
| [7] | 3GPP TSG-RAN WG1 #116 R1-2400175 National Institute of Standards and Technology, "Power Calibration for Radar Cross Section (RCS) Measurement," February 26th – March 1st, 2024. |
| [8] | 3GPP TR 38.901 , "Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)," 2023-12. |