3GPP TSG RAN WG1 Meeting #121		R1- 2504077

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:       9.2.2

Source: 	Nokia

Title: 	CSI enhancement for NR MIMO Phase 5

Document for:     Discussion and Decision

# 1	Introduction

Based on the latest revisions of Rel-19 MIMO draft CRs [3]-[7], the agreements reached in RAN1#120bis [2] and the revised WID [1], in this contribution we address some remaining issues regarding Type-I extension for up to 128 ports.

# 2	Type-I/Type-II refinements for 48, 64 and 128 ports

## 2.1	On CPU occupation for refined eType-II-Doppler

In RAN1#118, the following solution was agreed for the CPU occupancy and active resource count of Rel-19 Type-II-Doppler CSI calculation

Agreement(RAN1#118)

For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports,

- For Capability 1 timeline:

  - OCPU is the legacy Rel-18 Type-II Doppler OCPU multiplied by ceil(P/32) and replace K with KDOPP for AP-CSI-RS, while still not exceeding 8

  - Active resource counting follows the legacy Rel-18 Type-II Doppler

- For Capability 2 timeline:

  - OCPU is the legacy Rel-18 Type-II Doppler OCPU and replace K with KDOPP for AP-CSI-RS

Active resource counting follows the legacy Rel-18 Type-II Doppler

Therefore, if the corresponding CSI-RS Resource Set for channel measurement is aperiodic and configured with KDOPP CSI-RS resource groups of K CSI-RS resources, with total number of CSI-RS ports P∈{48,64,128},

- for capability 1, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP⋅P/32 for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication,

- for capability 2, OCPU=8 for KDOPP=12 and OCPU=Y1⋅KDOPP for KDOPP<12, where Y1∈{1, 2, 3} is reported by UE capability indication

However, for capability 1, there are no values of Y1 that a UE can indicate such that OCPU<8, for KDOPP=4,8. In fact, P/32=2 for 48 and 64 ports, hence for KDOPP=4,8, OCPU is always larger or equal to 8, and for P=128 ports and KDOPP=4,8, OCPU is always larger than 8.

- For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for capability 1, there are no values of Y1∈{1,2,3} that a UE can indicate such that OCPU<8, for KDOPP=4,8.

- For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for capability 1, either conclude, based on the agreed formula for OCPU, that Y1 is not needed because OCPU=8 for all values of KDOPP∈{4,8,12}, or adopt the same OCPU formula as in capability 2 (i.e., CPU occupation is the same for capability 1 and 2).

## 2.2	On the triggering offset of the associated CSI-IM for refined eType-II-Doppler

In legacy specifications, for aperiodic reporting on aperiodic CSI-RS, the triggering offset for the CSI-IM follows that configured for the associated NZP-CSI-RS for channel measurement.

For Rel-19 Type-I/II codebook refinement for 48, 64 and 128 ports, the CMR resources for resource aggregation can be in one or two consecutive slots, and this additional slot offset can be configured per resource also for an aperiodic CSI-RS resource set. Hence, in RAN1#120bis, it was discussed whether the triggering offset for the single CSI-IM resource should follow that of the CMR(s) in the first or second slot. Note that this issue is not present for NCJT/CJT reporting in Rel-17/18, because the CMRs cannot be configured with additional slot offset and only the resource-set level triggering offset can be configured. Hence for an NCJT or CJT report on aperiodic CSI-RS resources, the CMRs are always triggered in the same slot.

In RAN1#120bis, it was agreed, for Rel-19 Type-I and Type-II codebooks, except for Type-II-Doppler, that the triggering offset for the CSI-IM follows that of the associated CMR(s) in the first of the two slot.

Agreement(RAN1#120bis)

For the Rel-19 Type-I and Type-II (except for Type-II Doppler) codebook refinement for 48, 64, and 128 CSI-RS ports, when the K>1 aggregated NZP aperiodic CSI-RS resources for CMR are within two consecutive slots, the triggering offset of the associated aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) for CMR in the first slot.

- FFS: How to resolve this for Rel-19 Type-II Doppler

For Rel-19 Type-II-Doppler the associated aperiodic resource set for channel measurement is configured with KDOPP∈{4,8,12} CSI-RS resource groups of K aperiodic CSI-RS resources each. The KDOPP ⋅K CSI-RS resources are triggered by the same triggering instance and the separation between the first CSI-RS resources of two consecutive CSI-RS resource groups is m∈{1,2} slots, which is configured by higher layer parameter aperiodicResourceOffset. The KDOPP aperiodic CSI-RS resource groups are transmitted following the order of the CSI-RS resource groups configured in the CSI-RS resource set. The KDOPP ⋅K CSI-RS resources are ordered ascendingly by the CSI-RS resource ID and group index kDOPP=0,1,…,KDOPP-1, such that CSI-RS resources {kDOPP⋅K,kDOPP⋅K+1,…,kDOPP+1K-1} are associated with the kDOPP-th CSI-RS resource group. Besides, the k-th resource in all the KDOPP resource groups, with k=0,…,K-1, can be configured by higher layer parameter additionalOneSlotOffsetDopp with one additional slot offset relative to their respective CSI-RS resource group slot offset configured by the higher layer parameters aperiodicTriggeringOffset in the resource set and aperiodicResourceOffset in the codebook configuration.

If interference measurement is performed on CSI-IM, only one resource is configured in the corresponding csi-IM-ResourceSet, where the one CSI-IM resource is associated with all K aperiodic CSI-RS resources in each of the KDOPP groups, in the CSI-RS resource set for channel measurement.

Therefore, a simple extension to Rel-19 Type-II-Doppler of the rule agreed for the other Rel-19 Type-I and Type-II codebooks is for the aperiodic triggering offset of the CSI-IM to follow the triggering offset of the associated CMR(s) configured without additionalOneSlotOffsetDopp in the first CSI-RS resource group.

- For Rel-19 Type-II-Doppler configured with KDOPP aperiodic CSI-RS resource groups, the KDOPP CSI-RS resource groups are transmitted following the order of the CSI-RS resource groups configured in the CSI-RS resource set. Hence, the CMRs in the first resource group configured without additionalOneSlotOffsetDopp can be used as reference for the aperiodic triggering offset of the associated CSI-IM resource.

- For Rel-19 Type-II-Doppler codebook refinement for 48, 64 and 128 ports configured with KDOPP aperiodic NZP CSI-RS resource groups for channel measurements, the triggering offset of the aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) in the first resource group configured without additionalOneSlotOffsetDopp.

# 3	Conclusion

Hereafter is a summary of observations and proposals for Type-I extension for up to 128 ports

Observation 1	For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for capability 1, there are no values of Y1∈{1,2,3} that a UE can indicate such that OCPU<8, for KDOPP=4,8.

Observation 2	For Rel-19 Type-II-Doppler configured with KDOPP aperiodic CSI-RS resource groups, the KDOPP CSI-RS resource groups are transmitted following the order of the CSI-RS resource groups configured in the CSI-RS resource set. Hence, the CMRs in the first resource group configured without additionalOneSlotOffsetDopp can be used as reference for the aperiodic triggering offset of the associated CSI-IM resource.

Proposal 1	For the Rel-19 Type-II codebook refinement based on Rel-18 Type-II Doppler for 48, 64, and 128 CSI-RS ports, for capability 1, either conclude, based on the agreed formula for OCPU, that Y1 is not needed because OCPU=8 for all values of KDOPP∈{4,8,12}, or adopt the same OCPU formula as in capability 2 (i.e., CPU occupation is the same for capability 1 and 2).

Proposal 2	For Rel-19 Type-II-Doppler codebook refinement for 48, 64 and 128 ports configured with KDOPP aperiodic NZP CSI-RS resource groups for channel measurements, the triggering offset of the aperiodic CSI-IM follows the associated NZP CSI-RS resource(s) in the first resource group configured without additionalOneSlotOffsetDopp.

# References

- RP-242394, Samsung, “WID revision: NR MIMO Phase 5”, RAN#105, Melbourne, Australia, 9-12 Sep 2024

- RAN1 Chair’s Notes, RAN1#120bis

- R1-2503459, Ericsson, “Rel-19 CR Draft to TS38.211. Introduction of NR MIMO Phase5”, RAN1#120bis

- R1-2503473, Huawei, “Rel-19 CR Draft to TS38.212. Introduction of Rel-19 NR MIMO Phase 5”, RAN1#120bis

- R1-2503175, Samsung, “Rel-19 CR Draft to TS38.213. Introduction of NR MIMO Phase 5”, RAN1#120bis

- R1-2503488, Nokia, “Rel-19 CR Draft to TS38.214. Introduction of CSI enhancements for NR MIMO Phase 5”, RAN1#120bis

- R1-2503450, Intel, “Rel-19 CR Draft to TS38.215. Introduction of Rel-19 MIMO evolution for TS38.215”, RAN1#120bis