3GPP TSG RAN WG1 #121	R1- 2504091

St Julian's, Malta, May 19th – 23rd, 2025

Agenda Item:	9.4.4

Source:	Fujitsu

Title:	Discussion on other aspects of Ambient IoT

Document for:	Discussion

Introduction

In Rel-19, as a promising new IoT type device/service, Ambient IoT was studied as a RAN SI. The study conclusions and observations on devices architectures, potential solutions for ambient IoT, etc., were captured in TR38.769 [1]. In RAN#106 meeting, the Ambient IoT was successfully approved as Rel-19 WI [2].

The scope for this sub-agenda is about multiplexing/multiple access, scheduling information and timing relationships. In this contribution, we provide our views about these aspects.

Discussion

Multiplexing/multiple access

Msg.1 related aspects

In RAN1#120bis meeting, the agreement on TDMA based Msg.1 transmission with X= 1 and X=2 was reached as follows:

Agreement (RAN1#120bis) For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Agreement (RAN1#120bis) For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption(RAN1#120bis) For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access.

According to the discussion in RAN1#120bis meeting, the Toffset1 for X=1 or X=2 is used to define the starting time for the first Msg.1 time resource and Toffset2 for X=2 is used to define the starting time for the second Msg.1 time resource. whether the Toffset1 and Toffset2 are predefined or indicated by control information is still FFS. In our understanding, the starting time of Msg.1 time domain resource i.e., Toffset1/ Toffset2 should be explicitly indicated as the control information carried by PRDCH. Compared with the predefined starting time in specification, the indication by control information can provide more flexibility. The reader can schedule the Msg.1 transmission by its implementation with consideration of interference coordination, SFO handling. Moreover, explicit indication of the starting time of Msg.1 time domain resource has better forward compatibility to other services/traffics to be supported in the future releases.

Proposal 1:

Regarding the starting time of Msg.1 time domain resource, i.e., Toffset1 / Toffset2, an explicit indication as control information carried by PRDCH should be supported.

Figure 1 Examples of reference time for Toffset2.

A working assumption was reached in RNA1#120bis meeting on how to determine the starting time of the second Msg.1 time domain resource. The determination of the starting time of the second Msg.1 time domain resource can rely on either the start or the end of the first Msg.1 time domain resource, i.e., the possible definitions of Toffset2 as shown in the Figure 1 given in the FL summary [3]. The two options of Toffset2 are as follows,

- Option 1: Toffset2 is from the start of the first Msg.1 to the start of the second Msg.1.

- Option 2: Toffset2 is from the end of the first Msg.1 to the start of the second Msg.1.

We prefer Option 2. The key point to the selection between Option 1 and Option 2 is that the reader does not need to separately indicate the duration of the first Msg.1 which the device can derive by itself according to other information rather than Toffset2. Option 1 includes redundancy. Option 2 can save the signaling overhead for the Toffset2 indication compared with Option 1.

In contention-based random access, the reader sends a paging message to all the devices which may utilize the Msg.1 resources. In the paging message, the parameters relevant to the Msg.1s would be included. The device can derive the duration of the first Msg.1 resource based on the parameters.

Observation 1:

The key point to the selection between Option 1 and Option 2 is that the reader does not need to separately indicate the duration of the first Msg.1 which the device can derive by itself according to other information rather than Toffset2.

- Option 1 includes redundancy.

- Option 2 can save the signaling overhead for the Toffset2 indication compared with Option 1.

Proposal 2: Toffset2 should be from the end of the first Msg.1 to the start of the second Msg.1.

Msg.3 related aspects

The agreements on FDMA and TDMA Msg.3 transmission from multiple devices were reached in previous RAN1 meetings as follows.

Agreement (RAN1#120) Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement (RAN1#120b) Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission.

The issue on the data rate for FDMA based Msg.3 transmissions is still open. The different D2R data rates for FDMA based Msg.3 transmission are allowed. From our understanding, the size of Msg.3 is variable. In the case FDMA based Msg.3s are transmitted, the reader transmits the subsequent R2D until it completes receiving the Msg.3 with the longest duration in time domain. If only the same D2R data rate is supported as shown in Figure 2(a), the larger TBS for Msg.3 results in the longer duration in time domain. The spectrum efficiency is low if significant differences occur on time domain durations among the FDMA based Msg.3s. If the different data rates for FDMA based Msg.3 transmissions are allowed as shown in Figure 2(b), the reader can allocate data rate for each Msg3 transmission. For the large TBS, the reader can allocate a high data rate which can shorten the receiving window for Msg.3s on the reader. In that case, the spectrum efficiency can be improved as well.

Figure 2 Examples of the same and different data rate for Msg.3 transmissions

Observation 2:

In the case FDMA based Msg.3s are transmitted with variable TBS, the reader transmits the subsequent R2D until it completes receiving the Msg.3 with the longest duration in time domain.

- If only the same data rate is supported, the larger TBS for Msg.3 results in the longer duration in time domain. The spectrum efficiency is low if significant differences occur on time domain durations among the FDMA based Msg.3s.

- If different data rates are allowed, the reader can allocate a high data rate for large TBS of Msg.3, which can shorten the receiving window for Msg.3s on the reader. In that case, the spectrum efficiency can be improved as well.

Proposal 3:

The different data rates for FDMA based multiple Msg.3 transmissions are supported if the TBS of Msg.3s is variable.

To support the different data rates for FDMA based multiple Msg.3 transmissions, the data rate for each Msg.3 transmission can be implicitly or explicitly indicated in Msg.2. Different from the Msg.1 transmission, the Msg.3 transmission can be scheduled by reader in device-specific manner due to the contention resolution is successfully completed once a device receives a Msg.2 responding to its Msg.1. The reader can implicitly or explicitly schedule the data rate for each Msg.3 transmission in Msg.2. And how data rate is implicitly or explicitly indicated in Msg.2 is up to the discussion in 9.4.2.

Proposal 4:

To support the different data rates for FDMA based multiple Msg.3 transmissions, the data rate for each Msg.3 transmission can be implicitly or explicitly indicated in Msg.2.

Scheduling information

Scheduling information for D2R transmission

The control information for D2R scheduling has been discussed in previous meetings. And the following agreements have been reached.

Agreement (RAN1#120) The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement (RAN1#120) The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information. Agreement (RAN1#120b) For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH.

One issue on scheduling information for D2R transmission is still open. In RAN-120b meeting, a proposal from FL [3] is discussed as follows.

FL3/FL1 Proposal 5.2-3 Down-select from the following options for indicating the FEC and the number of repetition(s) for PDRCH transmission in the corresponding PRDCH: - Option 1: Separate indication of FEC and number of repetitions - Option 2: joint indication of FEC and number of repetitions

For MCS-like information, the joint indication (option 2) of the FEC and the number of repetitions is supported. The joint indication of the FEC including coding rate and the number of repetitions provides the better tradeoff between signaling overhead and scheduling flexibility. If a few combinations of FEC coding rate and repetition number are supported, the joint indication results in low signaling overhead.

Proposal 5:

Regarding MCS-like scheduling information for D2R transmission, the joint indication (Option 2) of FEC and repetition number is supported.

- Conclusion

In this contribution, we have the following observations and proposals considering the other aspects of AIoT:

Proposal 1:

Regarding the starting time of Msg.1 time domain resource, i.e., Toffset1 / Toffset2, an explicit indication as control information carried by PRDCH should be supported.

Observation 1:

The key point to the selection between Option 1 and Option 2 is that the reader does not need to separately indicate the duration of the first Msg.1 which the device can derive by itself according to other information rather than Toffset2.

- Option 1 includes redundancy.

- Option 2 can save the signaling overhead for the Toffset2 indication compared with Option 1.

Proposal 2: Toffset2 should be from the end of the first Msg.1 to the start of the second Msg.1.

Observation 2:

In the case FDMA based Msg.3s are transmitted with variable TBS, the reader transmits the subsequent R2D until it completes receiving the Msg.3 with the longest duration in time domain.

- If only the same data rate is supported, the larger TBS for Msg.3 results in the longer duration in time domain. The spectrum efficiency is low if significant differences occur on time domain durations among the FDMA based Msg.3s.

- If different data rates are allowed, the reader can allocate a high data rate for large TBS of Msg.3, which can shorten the receiving window for Msg.3s on the reader. In that case, the spectrum efficiency can be improved as well.

Proposal 3:

The different data rates for FDMA based multiple Msg.3 transmissions are supported if the TBS of Msg.3s is variable.

Proposal 4:

To support the different data rates for FDMA based multiple Msg.3 transmissions, the data rate for each Msg.3 transmission can be implicitly or explicitly indicated in Msg.2.

Proposal 5:

Regarding MCS-like scheduling information for D2R transmission, the joint indication (Option 2) of FEC and repetition number is supported.

- Reference

[1] TR 38.769 V2.0.0, Study on solutions for ambient IoT (Internet of Things), 2024-12.

[2] RP-243326, New WID: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.

[3] R1-2503104, FL summary#4 on other aspects for Rel-19 Ambient IoT, Wuhan, China, April 7th – 11th, 2025.