3GPP TSG RAN WG1 #121	R1-2504136

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.3.1

Source: 	ETRI

Title:	Discussion on SBFD TX/RX/measurement procedures

Document for:	Discussion/Decision

Introduction

This contribution presents ETRI’s views on remaining details for SBFD TX/RX/measurement procedures.

Discussion

Table 1. Possible resource setting configurations captured by TS38.214 [1].

##### 5.2.1.4.1 Resource Setting configuration For aperiodic CSI, each trigger state configured using the higher layer parameter CSI-AperiodicTriggerState is associated with one or multiple CSI-ReportConfig where the CSI-ReportConfig not configured with groupBasedBeamReporting-r17 or groupBasedBeamReporting-r18 is linked to periodic, or semi-persistent, or aperiodic resource setting(s): - When one Resource Setting is configured, the Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation. - When two Resource Settings are configured, the first one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second one (given by either higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for interference measurement performed on CSI-IM or on NZP CSI-RS. - When three Resource Settings are configured, the first Resource Setting (higher layer parameter resourcesForChannelMeasurement) is for channel measurement, the second one (given by higher layer parameter csi-IM-ResourcesForInterference) is for CSI-IM based interference measurement and the third one (given by higher layer parameter nzp-CSI-RS-ResourcesForInterference) is for NZP CSI-RS based interference measurement. … For semi-persistent or periodic CSI, each CSI-ReportConfig is linked to periodic or semi-persistent Resource Setting(s): - When one Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is configured, the Resource Setting is for channel measurement for L1-RSRP or for channel and interference measurement for L1-SINR computation. - When two Resource Settings are configured, the first Resource Setting (given by higher layer parameter resourcesForChannelMeasurement) is for channel measurement and the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference) is used for interference measurement performed on CSI-IM. For L1-SINR computation, the second Resource Setting (given by higher layer parameter csi-IM-ResourcesForInterference or higher layer parameter nzp-CSI-RS-ResourceForInterference) is used for interference measurement performed on CSI-IM or on NZP CSI-RS.

Table 1 shows the possible resource setting configurations in TS38.214 [1]. As clarified by the green highlighted parts in Table 1, when one Resource Setting is linked to a CSI-ReportConfig, CQI cannot be reported but only L1-RSRP can be measured and reported (and CLI measurement & report in case of Rel-19). It means that a CSI-ReportConfig for CQI report should be linked to two or three Resource Settings, when the first Resource Setting is for channel measurement and the second or third Resource Setting is for interference measurement. The channel measurement resource (CMR) can be configured by using one or more NZP CSI-RS resources. The interference measurement resource (IMR) can be configured by utilizing one or more CSI-IM resources, which may be overlapped with ZP CSI-RS, or one or more NZP CSI-RS resources

Observation 1. A CSI-ReportConfig for CQI report should be linked to two or three Resource Settings:

- The first Resource Setting is for channel measurement from NZP CSI-RS (CMR)

- The second or third Resource Setting is for interference measurement from CSI-IM/ZP CSI-RS or NZP CSI-RS (IMR)

We believe that CQI reporting for SBFD DL subbands is an essential feature to enable SBFD. Therefore, UE behaviours for both CMR and IMR on SBFD and non-SBFD symbols need to be clear, and hopefully be aligned to each other.

Observation 2. CQI reporting for SBFD DL subbands is an essential feature to enable SBFD

- Hence, UE behaviours for both CMR and IMR on SBFD and non-SBFD symbols should be clear.

However, since the previous agreements and the corresponding TP on CSI-RS in Table 2 are mentioning NZP CSI-RS only (i.e., no clarification on CSI-IM), UE behaviours for CQI reporting for SBFD DL subbands are ambiguous.

Table 2. Agreements and the corresponding TP on valid symbol type for CSI derivation.

| Agreements | Endorsed TP in R1-2503480 |
| --- | --- |
| Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. | For a CSI report associated with semi-persistent or periodic CSI-RS, if a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for the CSI derivation for the CSI report. |

Observation 3. Agreements and TPs on valid symbol type for CSI derivation so far are limited to the followings, only:

- (NZP) CSI-RS, i.e., not explicitly covering CSI-IM

- Semi-persistent or periodic CSI-RS, i.e., not covering aperiodic CSI-RS

According to the observations so far, we can conclude that at least the following UE behaviours are missing for SBFD DL subband CQI report:

- Valid symbol type of IMR for CSI derivation

- Valid symbol type of aperiodic CSI-RS for CSI derivation

Observation 4. The following UE behaviours are missing for SBFD DL subband CQI report:

- Valid symbol type of IMR for CSI derivation

- Valid symbol type of aperiodic CSI-RS for CSI derivation

Per the discussions above, we suggest RAN1 to apply the previous agreements on valid symbol type of CSI-RS for CSI derivation to the missing parts above.

Proposal 1. RAN1 to apply/extend the previous agreements on valid symbol type of CSI-RS for CSI derivation to the following cases as well:

- Valid symbol type of IMR for CSI derivation

- Valid symbol type of aperiodic CSI-RS for CSI derivation

More specifically, one possible way to define the valid symbol type of IMR for CSI derivation is given by proposal 2.

Proposal 2. For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies for all CMRs and IMRs associated with the CSI report

If we consider that “CSI reference resource” is an essential element for CSI derivation, in addition to the aforementioned CMR/IMR, it would be better to clarify the similar aspect for CSI reference resource (i.e., clarification on valid symbol type for CSI reference resource), like proposal 3.

Proposal 3. For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies for CSI reference resource associated with the CSI report

- If the valid symbol type configured by the CSI report is SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

There had been an argument that valid symbol type of aperiodic CSI-RS can be handled together with the other dynamic channels/signals. However, from our understanding on the endorsed CR in R1-2503480, it seems that the valid symbol type of aperiodic CSI-RS has not been clearly captured, yet. In this regard, we believe that the following proposal can be considered.

Proposal 4. For a CSI report associated with aperiodic CSI-RS,

- the valid symbol type for CSI derivation determined by the symbol type of the first measurement occasion among the all CMRs and IMRs associated with the CSI report

- the same valid symbol type applies for all CMRs, IMRs, and CSI reference resource associated with the CSI report

- if the valid symbol type is SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

We think the specification needs to ensure that the UE will report a CSI, only if all of the corresponding CSI reference resource, CMR, and IMR is in the same type of symbols, i.e., either of SBFD symbols or non-SBFD symbols. Otherwise, there will be no need to report such CSI, which should be contaminated by mixed measurements/assumptions across SBFD and non-SBFD symbol types.

Proposal 5. A SBFD-aware UE reports a CSI, only if all of the corresponding CSI reference resource, CMRs, and IMRs is in the valid symbol type of the CSI report.

- Otherwise, the CSI report is dropped.

As shown by one of the agreements in Table 2, CSI computation delay requirements were relaxed for SBFD subbands CSI report. Figure 1 summarizes our analysis on the remaining issues to finalize specification supports on SBFD CSI report. Per the previous agreements, we think that CPU occupation (first bullet in Figure 1), CSI processing timeline (second bullet in Figure 1), and subband size for subband CQIs (forth bullet in Figure 1) had been well clarified already. However, at least two remaining issues, i.e., 1) number of wideband CQI(s) for 2 DL subbands (1 for both DL subbands or 2), and 2) CSI dropping rule (currently, the same value of PriiCSIy,k,c,s can be assumed for 2 DL subbands) should be further discussed.

Figure 1. Remaining issues on SBFD CSI report.

Proposal 6. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the number of wideband CQI is one for a given CSI reference resource

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: the number of wideband CQI can be up to two for a given CSI reference resource. (One wideband CQI per DL subband)

Proposal 7. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the same value of PriiCSIy,k,c,s is applied for all DL subbands.

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: different values of PriiCSIy,k,c,s are applied across different DL subbands.

Conclusion

In this contribution, ETRI’s views on SBFD were shown and the following observation and proposals were made:

Observation 1. A CSI-ReportConfig for CQI report should be linked to two or three Resource Settings:

- The first Resource Setting is for channel measurement from NZP CSI-RS (CMR)

- The second or third Resource Setting is for interference measurement from CSI-IM/ZP CSI-RS or NZP CSI-RS (IMR)

Observation 2. CQI reporting for SBFD DL subbands is an essential feature to enable SBFD

- Hence, UE behaviours for both CMR and IMR on SBFD and non-SBFD symbols should be clear.

Observation 3. Agreements and TPs on valid symbol type for CSI derivation so far are limited to the followings, only:

- (NZP) CSI-RS, i.e., not explicitly covering CSI-IM

- Semi-persistent or periodic CSI-RS, i.e., not covering aperiodic CSI-RS

| Agreements | Endorsed TP in R1-2503480 |
| --- | --- |
| Agreement For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation for periodic/semi-persistent CSI-RS resources for the CSI report is explicitly configured. Only CSI-RS transmission occasions within the valid symbol types are used for CSI derivation. | For a CSI report associated with semi-persistent or periodic CSI-RS, if a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType, the UE only considers the CSI-RS occasions within either SBFD symbol(s) or non-SBFD symbol(s) as indicated by symbolType for the CSI derivation for the CSI report. |

Observation 4. The following UE behaviours are missing for SBFD DL subband CQI report:

- Valid symbol type of IMR for CSI derivation

- Valid symbol type of aperiodic CSI-RS for CSI derivation

Proposal 1. RAN1 to apply/extend the previous agreements on valid symbol type of CSI-RS for CSI derivation to the following cases as well:

- Valid symbol type of IMR for CSI derivation

- Valid symbol type of aperiodic CSI-RS for CSI derivation

Proposal 2. For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies for all CMRs and IMRs associated with the CSI report

Proposal 3. For a CSI report associated with periodic/semi-persistent CSI-RS, the valid symbol type for CSI derivation configured in CSI-ReportConfig applies for CSI reference resource associated with the CSI report

- If the valid symbol type configured by the CSI report is SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

Proposal 4. For a CSI report associated with aperiodic CSI-RS,

- the valid symbol type for CSI derivation determined by the symbol type of the first measurement occasion among the all CMRs and IMRs associated with the CSI report

- the same valid symbol type applies for all CMRs, IMRs, and CSI reference resource associated with the CSI report

- if the valid symbol type is SBFD symbols, only the REs on SBFD symbols and DL usable PRBs in the corresponding CSI reference resource shall be used for TBS determination of the CSI report

Proposal 5. A SBFD-aware UE reports a CSI, only if all of the corresponding CSI reference resource, CMRs, and IMRs is in the valid symbol type of the CSI report.

- Otherwise, the CSI report is dropped.

Proposal 6. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the number of wideband CQI is one for a given CSI reference resource

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: the number of wideband CQI can be up to two for a given CSI reference resource. (One wideband CQI per DL subband)

Proposal 7. For CSI report associated with the CSI-RS across two DL subbands,

- Option 1: the same value of PriiCSIy,k,c,s is applied for all DL subbands.

  - Without further RAN1 agreements, we believe that the option 1 shall be assumed

- Option 2: different values of PriiCSIy,k,c,s are applied across different DL subbands.

References

- 3GPP TS 38.214 V18.6.0