3GPP TSG RAN WG1 #121                                                                                                 R1-2504167

St Julian’s Malta, May 19th – 23th, 2025

Source:               TCL

Title:                    Discussion on HD-FDD Redcap UEs and eRedCap UEs for FR1-NTN

Agenda item:      9.11.2

Document for:    Discussion and Decision

- Introduction

In the RAN #106 meeting, a revised WID was approved for Rel-19 NR NTN [1], one of the objectives of the WID is related to Reduced Capability (RedCap) UEs with NR NTN operating, the details are shown below:

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In the RAN1#120bis meeting [2], the following agreements and conclusion have been achieved.

Agreement

Update the working assumption in RAN1 #120 with the following updates:

For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported:

- Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2.

- FFS: handling of PDCCH ordered PRACH transmission

- For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized.

  - Network is allowed to indicate UL overriding DL for all cases

    - This is signaled by one UE specific RRC parameter

  - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized.

Note2: UE shall comply to the following existing procedure in 38.331:

- UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

In this contribution, further enhancements for HD-FDD RedCap UEs and eRedCap UEs to support operating in FR1-NTN are discussed.

- Discussion

The handling rules for DL and UL overlapping and back-to-back non-overlapping cases were defined in TS 38.213 under the TN scenario, further enhancements of case 3 and case 4 for HD-FDD RedCap UEs and eRedCap UEs are needed.

- Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission

Handle rules: DL reception except UL transmission within processing time.

- Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission

Handle rules: UL transmission.

- Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission

Handle rule: Error case.

- Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission

Handle rule: Error case

- Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission

Handle rule: Prioritize SSB reception.

- Case 6: Dynamic or semi-static DL collides with valid RO

Handle rule: Up to UE implementation.

- Case 7: Collision due to direction switching

Handle rule: Prioritize SSB reception or up to UE implementation based on sub-case

TA misalignment between gNB and UE

For the TN scenario, the distance between gNB and UE is small, accordingly, the RTT between gNB and UE is also small, thus, the weak impact of time advance on TN for data scheduling and potential TA misalignment to cause data collision can be avoided based on gNB implementation. However, for the NTN scenario, due to non-negligible misalignment TA between gNB and UE, based on the scheduling mechanism at the gNB side to avoid collision between DL reception and UL transmission may not be suitable anymore.

In the current specification, the granularity of TA reporting is based on slot with 15Khz SCS, the Timing Advance field indicates the least integer number of slots for NTN or symbols for ATG, using sub-carrier spacing of 15 kHz for NTN and either 15 kHz or 30 kHz for ATG, greater than or equal to the Timing Advance value, which mean a TA reported by a UE need to align with the granularity of 1ms, however, there is an uncertainty of 1 ms about actual TA between gNB and UE. In other words, gNB may not know the actual location of a TA within a slot/ms, thus, based on the gNB scheduling mechanism to avoid collision between DL reception and UL transmission will cause resource waste due to gNB need to adopt a very conservative scheduling way.

Observation1: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, if the collision between DL reception and UL transmission is avoided based on the gNB scheduling mechanism, resources waste will occur due to TA reporting granularity.

In addition, for NTN, a Timing Advance report (TAR) shall be triggered if any of the following events occur:

-	upon indication from upper layers to trigger a Timing Advance report;

-	upon the configuration of offsetThresholdTA by upper layers, if the UE has not previously reported Timing Advance value to the current Serving Cell;

-	if the variation between the current estimate of the Timing Advance value and the last reported Timing Advance value is equal to or larger than offsetThresholdTA, if configured.

However, for timing advance report based on the variation between the current estimate of the timing advance value and the last reported timing advance value is equal to or larger than an offsetThresholdTA, with this way, there is a gap between the last reported timing advance and the current estimate of the timing advance value and the gap value is equal to or large than the configured offsetThresholdTA,  if  DL reception and UL transmission collision need to avoid based on gNB scheduling mechanism, thus, for a data transmission/reception, gNB need to reserved large of resources, for instance, as shown in figure 1, due to TA mismatching between gNB and UE, then gNB cannot judge the actual overlapping area of DL reception and UL transmission. If gNB needs to avoid collision between DL reception and UL transmission, then many resources need to be reserved to guarantee one of data transmission, this will cause large resource wasting.

Figure 1, scheduling limitation due to TA mismatching

Observation2: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, if the collision between DL reception and UL transmission is avoided based on the gNB scheduling mechanism, resources waste will occur due to misalignment of TA between gNB and UE.

Enhancements for a collision of Case 3 and Case 4

Based on the analysis above, a collision between DL reception and UL transmission overlapping and/or back-to-back non-overlapping symbols without sufficient gap caused by TA misalignment between gNB and UE for redcap UE under NTN scenario needs to be studied.

One potential solution is to enhance the TA report, finer granularity for TA reporting by UE can be considered. Reserved bits within a TA-reported MAC-CE in current spec can be used to indicate a re-fined TA granularity, e.g., 2 bits can be used, and the symbols within a slot can be split into 4 groups. Then a symbol group-based granularity of TA can be reported, it’s beneficial for gNB scheduling and potential scheduling collision between DL reception and UL transmission caused by TA granularity misalignment can be relieved. In addition, since the TA value will be changed due to UE and satellite movement, however, due to the configured value of OffsetThresholdTA, UE does not need to report TA until a trigger condition is satisfied, which means, even though TA is changed, if the changed value is not large than configured OffsetThresholdTA value, the actual TA cannot know by gNB timely. To handle this, the most straightforward way is to enable UE to update TA frequently, as a result, a gNB can receive the actual TA timely.

Proposal 1: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, TA reporting with finer granularity can be considered.

Proposal 2: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, it is recommended to consider frequent TA updates to ensure the gNB is informed of the actual TA in a timely manner.

- Conclusion

In this contribution, the following observations and proposals have been made:

Observation1: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, if the collision between DL reception and UL transmission is avoided based on the gNB scheduling mechanism, resources waste will occur due to TA reporting granularity.

Observation2: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, if the collision between DL reception and UL transmission is avoided based on the gNB scheduling mechanism, resources waste will occur due to misalignment of TA between gNB and UE.

Proposal 1: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, TA reporting with finer granularity can be considered.

Proposal 2: For HD-FDD Redcap UEs and eRedCap UEs in FR1-NTN, it is recommended to consider frequent TA updates to ensure the gNB is informed of the actual TA in a timely manner.

4. References

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3.

[2] 3GPP RAN1 #120bis meeting, Chairman notes.