3GPP TSG RAN WG1 #121	R1-2504179

Malta, MT, 19 - 23 May 2025

Agenda item:		9.11.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussions on downlink coverage enhancements

WI code:	NR_NTN_Ph3

Release:	Rel-19

Document for:		Discussion and Decision

# Introduction

In RAN#104, work item description (WID) for NTN is revised [4]. One of the main objectives of the revised WID is related to downlink (DL) coverage enhancement, as outlined here. Revised versions of the WID are highlighted in yellow.

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

As one can see, both potential system level and link level enhancements are within the scope of DL coverage. Furthermore, particular attention is given to the impact (e.g., backward compatibility) that SSB periodicity extension has on both system-level and link level enhancement.

# Discussion on System Level Aspects

## DL beam activity discussions

As outlined in the WID, one approach for the work is to define system parameters and assumptions “… such that satellite beams may not all be simultaneously active… ”. In the session notes for agenda item 9.11 from RAN #120bis [5] it was further noted that (but not agreed at RAN1#120bis):

Proposal 6-2-v1

For NTN with extended SSB periodicity, a subset of RACH occasions is configured to support valid ROs for PRACH transmission.

- The subset of ROs can be configured with bitmap setting.

Proposed conclusion

For NTN with extended SSB periodicity, no further work is needed in RAN1 for Rel-19, except for specification update.

Proposal 6-4-v0

A necessary condition for UE to wake up during On duration of paging DRX cycle starting at time that coincides with an extended SSB transmission to monitor PDCCH Type-0 CSS (SIB1) and PDCCH Type-2 CSS (Paging) is that the PagingCycle is a multiple of extended SSB periodicities

The aspects of Proposal 5-2-v1 and Proposal 6-4-v0 may work in a very regular deployment situation where cells turn on and off but for the case where a cell consists of multiple satellite beams, where the beams are turned on and off depending on the traffic present in those beams, i.e. the pattern is not necessarily regular, some open issues exist:

- The bitmap indicating the subset of RO which are applicable for a UE suffers from the fact that the RO may not fall into the activity period of the beam (unless the RO are set to a very high frequency, leading to a data capacity loss), as it relies on a regular RO pattern. Another issue is that if the RO is at the end of a guaranteed active period, the time window for the random access response window may fall outside the guaranteed active (downlink) period, so this should be avoided, as the UE may not be monitoring for the RA response.

- To maximize the power savings at a UE, it should be aware of the activity of the relevant beam. Alternatively, it will be listening continuously, even if the beam is not active, which from a UE perspective would be highly inefficient.

One solution to provide flexibility and at the same time keep the signaling overhead limited, is to have a fixed guaranteed period per beam besides the cell based SSB. On top of that, a flexible period can be configured which can be used by the beam(s) which is/are having traffic. This is illustrated in Figure 1 for the case of 4 beams in a cell.

Figure 1 Long SSB period with guaranteed transmission per beam and a flexible period which can be used by any of the beams.

Proposal 1: RAN1 should further define the assumptions for the beam activity.

Proposal 2: RAN1 to clarify if there is a coupling between DL and UL beams.

Proposal 3: RAN1 should focus on defining signals and procedures that allows flexible use of the beams in the cell.

Proposal 4: RAN1 should ensure signaling to maximize UE power savings in the case of flexible use of the beams.

## Impact on neighbor measurements and mobility

In case the concerns expressed in the previous section are addressed and resolved and larger SSB periodicities are considered, there are some further considerations related to neighbor measurements which will be further discussed here. The serving cell is providing SMTC windows and measurement gaps (MG) to UEs from the assumption that the neighboring cell is transmitting the SSB with a certain periodicity. If this periodicity is increased, but the value is static the serving cell can adjust the SMTC windows and measurement gaps, which may already be challenging, but if the SSB periodicity is dynamic, i.e. changes occasionally, reconfiguring the SMTC windows and measurement gaps through RRC signalling may on top of that lead to extra signalling overhead.

Observation 1: Adjusting the SMTC window and measurement gaps in case of dynamically changing SSB periodicity will increase the signalling overhead.

One way to avoid this overhead can be to provide the UE with information about the changes in the SSB periodicities of the cells, such that the UE can adjust its behavior. Besides the fact that a changing SSB periodicity impacts the time instants where a UE should monitor for SSBs, one other consequence of a larger SSB periodicity is that there will be less measurement opportunities. This is for instance illustrated in Figure 2, where a UE is served by cell #0. It is assumed that the system is configured in such a way that there is only one SSB defined per SSB burst for each cell (single NR beam per cell), and the SSB of the neighboring cells are spread uniformly over the SSB period, i.e. the intercell SSB interval is constant. Serving cell #0 needs to set the SMTC window for capturing these SSBs of neighboring cells, but this is hard as a SMTC window has a limited length and some UEs cannot have more than 2 SMTC windows (depending on UE capability). The intercell SSB interval is typically not uniform, as the SSB can be distributed in many different ways. At the same time, it can be seen from the Figure that the UE only has 2 close neighboring cells (cell #2 and cell #1) and therefore the system would function fine if the UE just measured those cells.

At this point it is worth noting that the increased SSB periodicity is introduced to allow for a gNB (or satellite) to limit the simultaneous transmission onto “beam footprints” as agreed in the assumptions for FR1 set 1-1, set 1-2 and set 1-3 (with configurations for amount of simultaneous beams from the satellite being active). With only a selected subset of satellite beam footprints being active at the same time, the SSBs of the neighboring cells will not have the possibility to be active at the same time, thereby creating time wise offsets of the transmitted SSBs as illustrated in Figure 2.

Figure 2 Example illustration of a UE in serving cell #0 close to neighboring cells #2 and #1 and example of spread of SSBs across time.

Observation 2: A UE typically needs to measure only a very limited number of cells and not all neighbors of the serving cell.

The network could provide area specific SMTC windows, but this would increase the overhead significantly, as the format in SIB2,3,4 would for instance go from

smtc                                SSB-MTC                                         OPTIONAL,       -- Need S

to

smtc_area1                         SSB-MTC                                         OPTIONAL,       -- Need S

area1

….

smtc_areax                         SSB-MTC                                         OPTIONAL,       -- Need S

areax

where the areax is the area where smtc_areax is to be applied. Depending on the area format the overhead can be considerable. Furthermore, for some measurements like inter satellite measurements in active mode measurement gaps are needed, which would mean they also should be area dependent. A further additional drawback is that idle UEs would need to update their understanding of their own location through GNSS update and SIB reading in order to get the SMTC information belonging to their location.

Observation 3: Providing area specific SMTC windows increases the overhead and UE power consumption due to the extra signaling.

The UE in question would ideally detect which of the neighboring cells are relevant by scanning over one or a few SSB periods. After these scanning periods the UE could limit the measurements and corresponding measurement occasions to the relevant neighboring cells. In this way, the UE would be able to autonomously detect the relevant neighboring cells for monitoring, while at the same time not having to monitor for cells that are not relevant for the UE measurements.

Proposal 5: RAN1 to study how a UE can do a scan for neighboring cells over a period with the purpose of limiting the measurements afterwards to the relevant cells.

Cell specific measurements of the SSBs are for most cases needed at times of initial access and during mobility events. During mobility the measurements are used to find the candidate cell and to synchronize to the new cell. Having the SSBs less frequently available during mobility events may degrade the performance significantly. In case of Earth Fixed Cells (EFC) the mobility events are well defined and deterministic (one set of cells will disappear at the trailing end of the satellite coverage area and a new set of cells will appear at the front end of the satellite coverage area). To optimize the mobility performance, the SSB periodicity of the candidate cell(s) may be set to a lower value in order to maximize the mobility performance during expected mobility events.

Proposal 6: RAN1 to ask RAN2 to study the possibility of setting the SSB periodicity to a lower value during mobility events in case of Earth Fixed Cells.

# Discussion on Link Level Aspects

This section discusses link level coverage enhancement of PDCC, PDSCH for SIB1/SIB19, and PDSCH for Msg4, respectively.

## PDCCH Coverage Enhancement

For PDCCH, in particular, we focus on Type-0 and Type-2A/1/2 common search space (CSS) in the following.

### Considerations on Type-0 CSS PDCCH

In RAN1#120b, the following agreement is achieved regarding coverage improvements of PDCCH.

Agreement For PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within MIB pdcch-ConfigSIB1, support repeated PDCCH candidates in the two consecutive slots n0 and n0+1 associated with the same SSB index ( n0 as defined in section 13 of TS 38.213) at least for M=2. - Repeated PDCCH candidates share the same aggregation level (AL), coded bits and same candidate index - Note: if the network repeats the Type 0 PDCCH across two consecutive slots, a legacy UE might decode the PDCCH and associated PDSCH in one slot and skip PDCCH monitoring in the other slot. Note: further discuss the potential solution for M=1 and M=1/2.

According to the agreement above, Type0-PDCCH is repeated in two consecutive slots associated with the same SSB index. Therefore, in case M=1 or M=1/2, collision occurs across the transmitted PDCCH (original transmission and the repeated transmission) in the slots associated with different SSB indexes. This leads to an increase in interference across different SSB indexes. Overall, the adopted approach in the agreement above is not applicable for cases where M=1 or M=1/2. Additionally, when backward compatibility is considered, it imposes a restriction and limit scheduling flexibility as original transmission must always be transmitted in nominal slot n0.

Observation 4: For Option 1, and when M=1 or M=1/2, the repetitions in slots associated with different SSB indexes lead to an increase in interference and impact NW scheduling flexibility.

Potential solutions targeting resolving cross SSB interference for the case of M=1 or M=1/2, eventually lead to huge specification changes as table 13-11 in 38.213 would need to be updated. Additionally, solutions that are targeting postponing of SIB1 transmission and/or alternating SIB1 transmission across different SSB beams to resolve the interference are not seen as feasible, especially when also considering the aspect of extended SSB periodicities. Further,  it is important to consider a design that takes the backward compatibility into account. Therefore, we propose two solutions that take the above-mentioned issues into account.

As a first solution, we propose that Type0 CSS repetition if enabled, only be configured with M=2. In other words, configuration settings when M=1 or M=1/2 are discarded. Additionally, as a second solution, the gNB may still support the repetition Type0 CSS repetition when M=1 or M=1/2, but would reduce the number of SSB beams to avoid the interference.

Proposal 7: PDCCH repetitions for Type0 CSS is only supported for M=2.

### Considerations on Other CSS Types-0A/1/2

In the previous meeting (RAN1#120bis), the following working assumption is achieved.

Working assumption For PDCCH CSS other than Type-0 CSS and other than Type-3 CSS for common search spaces other than SearchSpaceZero, intra-slot PDCCH repetition is supported. RAN1 to down select between option 1 and option 2: Option 1: Use same CORESET and two different SS (SS Set1 and SS Set2) - Linking two PDCCH candidates (adopt the same mechanism for SS linking specified in Release 17) - FFS: Blind decoding limit Option 2: Use same CORESET associated with one SS which is repeated by introducing symbol domain offset X - FFS: Blind decoding limit - FFS: details configuration and signalling Nokia expressed the concern on the above working assumption that this will take physical resources away from intra-slot scheduling for legacy PDSCH.

Option-1: Proposes to use the framework of “linking two SS sets” developed in Rel-17 for multi-TRP operations, with the assumption that same CORESET is used for linking the two SS sets. From the current specification, TS38.213, clause 10.1, determination of the PDCCH MOs when two SS set are linked is as follows:

Observation 5: In the framework of two linked SS sets, for MOs with the same index in a slot (from each SS set), UE monitors PDCCH candidates ms,nCI(L) with the same value (from each set, i.e., msi,nCI(L)= msj,nCI(L)). Furthermore, UE’s expectation is that both SS sets have the same periodicity, offset, duration, maximum number of PDCCH candidates per aggregation level, and a same number of non-overlapping PDCCH MOs within a slot.

It is important to understand the current implication of linking two search space set mechanism. From the text above, the two SS sets that are linked together must have non-overlapping monitoring occasions within a slot. Therefore, this implies that even if, as in Option 1, the same CORESET is used, due to the constraint that the two non-overlapping monitoring occasions associated to the two search space sets, respectively, the CORESET is repeated two times within a slot.

Observation 6: With Option 1 of the working assumption, the CORESET is repeated two times.

Option-2 On the other hand proposes that UE is configured with one SS set (in contrast to Option-1, wherein UE must be configured with 2 SS sets), and instead the CORESET is repeated in the slot with an offset X (in unit of OFDM symbol) with respect to the first CORESET. Therefore, based on the evaluation above, both Options lead to the same result in terms of the number of resources consumed for transmission of control. Both options lead to two-time transmission of a CORESET, which will take physical resources away for intra-slot scheduling for legacy PDSCH, as Nokia already pointed out at the last meeting.

Observation 7: Option 2 of the working assumption led to two repetitions of the CORESET in a slot.

Observation 8: Both Option 1 and Option 2 of the working assumption will lead to the same control overhead.

Given the analysis above, that both options lead to same control overhead, Option 1 may lead to more specification change. Additionally, Option 2 is preferred as no additional SS set configuration is required for the UE. Furthermore, the offset value X for Option 2 can be determined implicitly without the need for NW signaling and configuration. For instance, offset value X can be determined by the UE accordingly, while taking other monitoring occasions that UE receive and determines as part of all other search space set configuration. In particular, offset value X can be determined by the UE to be e.g., after the last MO in a slot, or immediately after the MO where the first PDCCH is transmitted.

Proposal 8: RAN1 to down-select Option 2, i.e., repetition of the same CORESET that is associated with one SS by introducing symbol domain offset X.

Proposal 9: RAN1 to discuss determination of the symbol domain offset X in an implicit manner and autonomously by the UE.

## SIB1/SIB19 Coverage Enhancement

This section covers aspects related to repetition enabling/disabling of SIB1 repetition, and time domain resource determination for SIB1/19 repetition.

### Signaling of enabling repetitions for PDSCH conveying SIB1

In RAN1#120b, the following agreement is achieved for enabling/disabling of SIB1 repetition.

Agreement For enabling/disabling SIB1 PDSCH repetition, RAN1 to consider the following options: - Option 1: Using reserved bit(s) in PBCH payload. - Option 2: Using scheduling PDCCH. - Option 3: The enabling/disabling of SIB1 PDSCH repetition is implicitly indicating by the enabling/disabling of Type-0 CSS PDCCH repetition.

Proposal 10: RAN1 to down-select Option 2 for enabling/disabling SIB PDSCH repetition, i.e., using scheduling DCI.

According to TS38.214, the modulation order and target code rate is determined as follows:

For the PDSCH scheduled by a PDCCH with DCI format 1_0, format 1_1, format 1_2, format 1_3, format 4_0, format 4_1 or format 4_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, TC-RNTI, CS-RNTI, SI-RNTI, RA-RNTI, MSGB-RNTI, G-RNTI, G-CS-RNTI, multicast-MCCH-RNTI, MCCH-RNTI or P-RNTI, or for the PDSCH scheduled without corresponding PDCCH transmissions using the higher-layer-provided PDSCH configuration SPS-Config, if (…) else - the UE shall use IMCS and Table 5.1.3.1-1 to determine the modulation order (Qm) and Target code rate (R) used in the physical downlink shared channel. end The UE is not expected to decode a PDSCH scheduled with P-RNTI, RA-RNTI, SI-RNTI and Qm > 2

Observation 9: The UE is not intended to decode a PDSCH scheduled with SI-RNTI when modulation order is greater than 2 (QPSK). Therefore, MCS bits can be exploited to carry signalling for enabling SIB PDSCH repetitions. However, backwards compatibility must be considered for such an approach.

According to TS38.212, the DCI for scheduling PDSCH conveying SI is as follows:

The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by SI-RNTI: - Frequency domain resource assignment - bits - is the size of CORESET 0 - Time domain resource assignment - 4 bits as defined in Clause 5.1.2.1 of [6, TS38.214] - VRB-to-PRB mapping - 1 bit according to Table 7.3.1.2.2-5 - Modulation and coding scheme - 5 bits as defined in Clause 5.1.3 of [6, TS38.214], using Table 5.1.3.1-1 - Redundancy version - 2 bits as defined in Table 7.3.1.1.1-2 - System information indicator - 1 bit as defined in Table 7.3.1.2.1-2 - Reserved bits - 17 bits for operation in a cell with shared spectrum channel access in frequency range 1 or for operation in a cell in frequency range 2-2; otherwise 15 bits

Observation 10: There is some room for reserved bits to carry the signaling information for enabling SIB PDSCH repetitions, but the feasibility of such solutions may be challenging, given that UE does not know at this stage if the network supports the feature or not.

To overcome the problem mentioned in the observation above, we provide the following analysis. One of the solutions is to introduce a new DCI format for scheduling enhanced PDSCH carrying SIB1 with repetitions. More specifically, with the new DCI format 1-0 with CRC scrambled by an “enhanced SI-RNTI (ESI-RNTI)” which might use one of the reserved states for RNTI values in Table 7.1-1 in TS 38.321, some fields existing in legacy DCI could be repurposed to indicate SIB PDSCH repetitions, e.g., MCS fields or Reserved bits. Additionally, the new DCI could be more compact, i.e., having reduced and necessary fields only, and therefore the PDCCH coverage be improved. To support Rel-19 and prior Rel-19 NTN UEs, gNB should transmit both types of DCIs mentioned above. This will lead to an increase in total overhead and complexity of scheduling broadcast information.

Proposal 11: RAN1 to consider introducing a new DCI format 1-0 with different RNTI to support SIB PDSCH repetitions and PDCCH coverage enhancement.

### Time Domain Resource Determination

With respect to time domain resource determination of PDSCH carrying SIB1, the following agreement is achieved in RAN1#120b.

Agreement When PDCCH CSS type-0 repetition is performed, for SIB1 link level enhancement, support PDSCH repetition of SIB1 transmitted within the same slot as the type0-CSS PDCCH repetition. - UE supporting SIB1 PDSCH coverage enhancement assumes that the PDCCH and associated PDSCH to be repeated in both slots where the corresponding PDCCHs are transmitted. - Each PDSCH SIB1 repetition is within the same slot of each PDCCH candidate for scheduling DCI - The two associated PDSCHs have the same RV FFS: Whether it is supported that type-0 PDCCH repetition is not performed while the PDSCH-SIB1 repetition is performed, and if so whether/how to handle the slot determination.

According to the agreement in RAN1#120bis, if PDSCH-SIB1 repetition without Type0-PDCCH repetition is supported, there is one Type0-PDCCH to schedule the repetitions of PDSCH carrying SIB1, and UE may skip the monitoring occasion corresponding to the repeated PDSCH rather than the first PDSCH. This scenario is shown in the following figure.

Observation 11: The first PDSCH in slot n can be determined based on legacy specification, and the time/frequency resource for repeated PDSCH is indicated by the Type0-PDCCH in slot n.

However, as described in the agreement above, the problem is how to determine the repeated slot, here denoted by X in the above sketch. In particular, for determination of X which is the offset from slot n, the SSB index and Type0-PDCCH multiplex factor M are the factors that must be taken into account. The objective is that the slot for repeated PDSCH associated with an SSB index does not cause interference to original and/or repeated transmission of SIB1 PDSCH in other SSB beams.

According to TS 38.213, the slot n for transmission of SIB1 is determined as an index of slot n0, where n0=O∙2μ+i∙MmodNslotframe,μ. Therefore one design option would be to have the repeated PDSCH for SIB1, which is associated with SSB beam index i, in the slot n+X, i.e., the slot after all nominal slots n0 associated to the other SSB beam indexes.

Proposal 12: UE determines an index of slot n0+X for repeat PDSCH as following equation:

n0+X=O∙2μ+i+Lmax.MmodNslotframe,μ

Where Lmax is the maximum number of SSB indexes that is specified in clause 4.1 of TS 38.213, i is SSB index and M is Type0-PDCCH multiplex factor.

## PDSCH Carrying Msg4

In the following, we provide a discussion regarding coverage improvement of PDSCH Msg4. In the previous RAN1#120b meeting the following agreement was reached.

Agreement For enabling/disabling Msg4 PDSCH repetition, RAN1 to down-select among the following options: - Option 1: UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0 - FFS: indication details. - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) - Option 2: The enabling/disabling of Msg4 PDSCH repetition is implicitly indicated by the enabling/disabling of SIB1 PDSCH repetition. - Option 3: The enabling/disabling is indicated by SIB1 configuration - FFS: whether/how network is informed by the UE that certain conditions are met to trigger Msg4 PDSCH repetition (e.g. RSRP detected at UE is less than x dB) FFS: Whether UE reports its capability

From the options provided in the agreement above, in Option 2, enabling/disabling is indirectly associated with enabling/disabling of SIB1 PDSCH repetition, which is not a very practical design option as it enforces the NW that if PDSCH for SIB1 is repeated, PDSCH for Msg4 is also transmitted with repetition. Option 3 is also based on configuration/indication provided in SIB1 which is a cell wide signalling ignoring the UE’s capability in even supporting the feature or not and also ignoring individual UE’s channel condition. Therefore, we believe that Option 1 is one good candidate for down-selection as it does not have the shortcomings of Option 2/3, and it is dynamic design in nature.

Proposal 13: RAN1 to down-select Option 1 for enabling/disabling of PDSCH Msg4 repetition, i.e., UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0.

Now, we address assuming that RAN1 agrees to down-select for Option 1 for enabling/disabling of PDSCH Msg4 repetitions, some of the remaining open issues for this topic will be the following:

Assuming Msg4 repetition activation indicated DCI - Aspects related to UE assistance information:

Regarding the assistance information that can potentially be carried by UE to inform the NW/gNB about UE’s need in reception of PDSCH Msg4 with repetition, the information may include measurements reflecting the DL channel quality in very limited measurement samples rather than only RSRP measurements. In general, the DL channel quality measurement at this stage can only be performed on commonly available signals and channels, while UE specific signals and channels are not available for such measurement. The signals and channels that may be considered for the measurements are for instance SSB measurements as one set of input signals, or based on DMRS of PBCH, Type0-PDCCH, Type1-PDCCH or PDSCH of Msg2. Furthermore, the report as part of the assistance information may include several levels of the considered measured quantity in case multiple thresholds are defined by the gNB. In other words, the information received from the UE with respect to PDSCH Msg4 repetitions may be based on channel conditions and the UE receiver capabilities.

Furthermore, with respect to signalling aspects of assistance information, one can consider that information to be transmitted as part of the PUSCH carrying Msg3, which is the only information carrying channel which is available at this point of the initial access procedure. For instance, one could consider using the reserved bits in the MAC-CE sub-header of the Msg3, introducing an extended MAC-CE which may have independent LCID states for indicating which measurement thresholds for number for Msg4 PDSCH transmissions is being met. Furthermore, the network may configure the UE (through SIB1) for threshold(s) under which the UE is allowed to or expected to provide the assistance information, of course under the condition that the UE supports this feature.

Proposal 14: RAN1 to discuss solutions for UE to report scheduling assistant information for assisting the gNB scheduling of Msg4 PDSCH repetition factor.

- FFS: Information that needs to be included in the assistant information

- FFS: Means for signaling of assistant information such as MAC-CE sub-header of Msg3

Assuming Msg4 repetition activation indicated DCI - Indication details [first FFS bullet]:

For signaling aspects of enabling/disabling PDSCH for Msg4 repetition, since there is no spare bit in the fields of the DCI 1_0 with CRC scrambled by TC-RNTI, existing fields must be used to carry enabling/disabling (and/or repetition factor) information. Based on the clause 7.3.1.2.1 of 38.212, the following information is carried with DCI format 1_0 with CRC bits scrambled by TC-RNTI.

Therefore, we propose to exploit “PDSCH-to-HARQ_feedback timing indicator” field for carrying the information. For this to be done, for instance, the range of the values can be reduced to 4 values (e.g., to {1,3,5,7}), instead of 8 values based on the current specification [clause 9.2.3 of 38.213], and the result spare bit can be used for indication.

Alternatively, some (or all) states of the 3 bits in “PDSCH-to-HARQ_feedback timing indicator” can now carry additional information about enabling/disabling of PDSCH Msg4 repetition together with repetition factor. One example is shown below:

| Bitfield/information | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDSCH-to-HARQ_feedback timing | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| Repetition Enabled/disabled | Disabled | Disabled | Disabled | Disabled | Enabled | Enabled | Enabled | Enabled |
| Repetition Factor | X | X | X | X | 2 | 4 | 2 | 4 |

Note: Please note that in the example above, enable/disabling pattern (3rd row), and allocation of the repetition factor (4th row) are up to design.

Proposal 15: RAN1 to decide expanding the functionality of “PDSCH-to-HARQ_feedback timing indicator” field in DCI 1_0 with CRC bits scrambled by TC-RNTI, where additional information to be carried related to enabling/disabling of PDSCH for Msg4 with repetition and/or repetition factor.

# Discussion on higher-layer parameters

As indicated by Chairman guidance for this meeting, companies are encouraged to provide suggestions for higher layer parameters that would be needed for configuring/defining the feature.

Based on earlier agreements and our current proposals for this meeting, we are therefore suggesting the following parameters (with main focus on what is expected to be achieved by having the parameter rather than the actual parameter name). We have listed the parameters in the following table:

| Suggested parameter name | Range | Description |
| --- | --- | --- |
| PDCCH_type0_rep_offset | [L_max, 10] | This parameter introduces an offset to the nominal slot n0 for indication of the slot in which PDCCH repetition is carried. The range of this parameter is between the maximum number of SSBs (L_max) and 10 sub-frames. The parameter is part of broadcast signaling. |
| PDSCH_Msg4_rep | [1,2,4] | This parameter configures a set of repetition factors for PDSCH carrying Msg4. The parameter is part of broadcast signaling. |

When observing the agreements reached over the past meetings, the list above is quite limited, and we find that RAN1 needs to have further progress prior to being able to make a complete list of higher layer parameters.

Proposal 16: RAN1 to further discuss the higher layer parameters to use for DL coverage enhancements for NR over NTN.

# Conclusion

In this contribution we have presented our observations and proposals for the DL coverage enhancements. These are as follows:

Observation 1: Adjusting the SMTC window and measurement gaps in case of dynamically changing SSB periodicity will increase the signalling overhead.

Observation 2: A UE typically needs to measure only a very limited number of cells and not all neighbors of the serving cell.

Observation 3: Providing area specific SMTC windows increases the overhead and UE power consumption due to the extra signaling.

Observation 4: For Option 1, and when M=1 or M=1/2, the repetitions in slots associated with different SSB indexes lead to an increase in interference and impact NW scheduling flexibility.

Observation 5: In the framework of two linked SS sets, for MOs with the same index in a slot (from each SS set), UE monitors PDCCH candidates ms,nCI(L) with the same value (from each set, i.e., msi,nCI(L)= msj,nCI(L)). Furthermore, UE’s expectation is that both SS sets have the same periodicity, offset, duration, maximum number of PDCCH candidates per aggregation level, and a same number of non-overlapping PDCCH MOs within a slot.

Observation 6: With Option 1 of the working assumption, the CORESET is repeated two times.

Observation 7: Option 2 of the working assumption led to two repetitions of the CORESET in a slot.

Observation 8: Both Option 1 and Option 2 of the working assumption will lead to the same control overhead.

Observation 9: The UE is not intended to decode a PDSCH scheduled with SI-RNTI when modulation order is greater than 2 (QPSK). Therefore, MCS bits can be exploited to carry signalling for enabling SIB PDSCH repetitions. However, backwards compatibility must be considered for such an approach.

Observation 10: There is some room for reserved bits to carry the signaling information for enabling SIB PDSCH repetitions, but the feasibility of such solutions may be challenging, given that UE does not know at this stage if the network supports the feature or not.

Observation 11: The first PDSCH in slot n can be determined based on legacy specification, and the time/frequency resource for repeated PDSCH is indicated by the Type0-PDCCH in slot n.

Proposal 1: RAN1 should further define the assumptions for the beam activity.

Proposal 2: RAN1 to clarify if there is a coupling between DL and UL beams.

Proposal 3: RAN1 should focus on defining signals and procedures that allows flexible use of the beams in the cell.

Proposal 4: RAN1 should ensure signaling to maximize UE power savings in the case of flexible use of the beams.

Proposal 5: RAN1 to study how a UE can do a scan for neighboring cells over a period with the purpose of limiting the measurements afterwards to the relevant cells.

Proposal 6: RAN1 to ask RAN2 to study the possibility of setting the SSB periodicity to a lower value during mobility events in case of Earth Fixed Cells.

Proposal 7: PDCCH repetitions for Type0 CSS is only supported for M=2.

Proposal 8: RAN1 to down-select Option 2, i.e., repetition of the same CORESET that is associated with one SS by introducing symbol domain offset X.

Proposal 9: RAN1 to discuss determination of the symbol domain offset X in an implicit manner and autonomously by the UE.

Proposal 10: RAN1 to down-select Option 2 for enabling/disabling SIB PDSCH repetition, i.e., using scheduling DCI.

Proposal 11: RAN1 to consider introducing a new DCI format 1-0 with different RNTI to support SIB PDSCH repetitions and PDCCH coverage enhancement.

Proposal 12: UE determines an index of slot n0+X for repeat PDSCH as following equation:

n0+X=O∙2μ+i+Lmax.MmodNslotframe,μ

Where Lmax is the maximum number of SSB indexes that is specified in clause 4.1 of TS 38.213, i is SSB index and M is Type0-PDCCH multiplex factor.

Proposal 13: RAN1 to down-select Option 1 for enabling/disabling of PDSCH Msg4 repetition, i.e., UE specific PDSCH with Msg4 repetition activation indicated via PDCCH- DCI Format 1_0.

Proposal 14: RAN1 to discuss solutions for UE to report scheduling assistant information for assisting the gNB scheduling of Msg4 PDSCH repetition factor.

- FFS: Information that needs to be included in the assistant information

- FFS: Means for signaling of assistant information such as MAC-CE sub-header of Msg3

Proposal 15: RAN1 to decide expanding the functionality of “PDSCH-to-HARQ_feedback timing indicator” field in DCI 1_0 with CRC bits scrambled by TC-RNTI, where additional information to be carried related to enabling/disabling of PDSCH for Msg4 with repetition and/or repetition factor.

Proposal 16: RAN1 to further discuss the higher layer parameters to use for DL coverage enhancements for NR over NTN.

# References

- RP-241667, ”Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R1-2401844, ”FL Summary #2: NR-NTN downlink coverage enhancements”, Thales, RAN WG1 #116 March 2024.

- R2-24095xx, “Draft report of 3GPP TSG RAN WG2 meeting #127bis”, Hefei China, 14-18 October, 2024.

- RP-243300, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- “Session notes for 9.11 Non-Terrestrial Networks (NTN) for NR Phase 3, Internet of Things (IoT) Phase 3, and IoT-NTN TDD mode (v05)”, Wuhan, China, April 7-11th 2025