3GPP TSG RAN WG1 #121             	                                                 R1-2504182

St Julian's, Malta, 19 - 23 May, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR | --- | rev | --- | Current version: | 16.17.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on the PDSCH mapping type B starting symbol determination in TS 38.214. Source to WG:; NEC. Source to TSG:; ---. Work item code:; NR_L1enh_URLLC; Date:; 2025-05-19. Category:; F; Release:; Rel-16. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20). Reason for change:; In the current specification: 1) S0 is first defined as the reference point for determining the starting symbol S of the scheduled PDSCH in the third bullet. 2) Then S0 is further defined as the starting symbol of the PDCCH occasion where DCI format 1_2 is detected in the first half of the first sub-bullet in the third bullet. 3) S is defined as the starting symbol in the third bullet The definition and physical meaning of S0 are inconsistent in this clause. S and S0 are misdefined with each other.. Summary of change:; Revise the first sub-bullet in the third bullet and clarify the definition of S0 and S, respectively.. Consequences if not approved:; - The content of clause 5.1.2.1 is not consistent with itself on the physical meaning and definition of the S0 and S. - The starting symbol S of the scheduled PDSCH will be misdetermined.. Clauses affected:; 5.1.2.1. Y; N. Other specs; N; Other core specifications; TS/TR ... CR .... affected:; N; Test specifications; TS/TR ... CR .... (show related CRs); N; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:. - 5.1.2.1 Resource allocation in time domain When the UE is scheduled to receive PDSCH by a DCI, the Time domain resource assignment field value m of the DCI provides a row index m + 1 to an allocation table. The determination of the used resource allocation table is defined in Clause 5.1.2.1.1. The indexed row defines the slot offset K0, the start and length indicator SLIV, or directly the start symbol S and the allocation length L, and the PDSCH mapping type to be assumed in the PDSCH reception. Given the parameter values of the indexed row: - The slot allocated for the PDSCH is Ks, where , if UE is configured with ca-SlotOffset for at least one of the scheduled and scheduling cell, and Ks = , otherwise, and where n is the slot with the scheduling DCI, and K0 is based on the numerology of PDSCH, and and are the subcarrier spacing configurations for PDSCH and PDCCH, respectively, and - Nslot, offset, PDCCHCA and μoffset,PDCCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset, for the cell receiving the PDCCH respectively, Nslot, offset, PDSCHCA and μoffset,PDSCH are the Nslot, offsetCA and the, respectively, which are determined by higher-layer configured ca-SlotOffset for the cell receiving the PDSCH, as defined in clause 4.5 of [4, TS 38.211]. - The reference point S0 for starting symbol S is defined as: - if configured with referenceOfSLIVDCI-1-2, and when receiving PDSCH scheduled by DCI format 1_2 with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI with K0=0, and PDSCH mapping Type B, the starting symbol S is relative to the reference point S0 of the starting symbol of PDCCH monitoring occasion where DCI format 1_2 is detected; - otherwise, the starting symbol S is relative to the start of the slot using S0=0. - The number of consecutive symbols L counting from the starting symbol S allocated for the PDSCH are determined from the start and length indicator SLIV: if then else where, and - the PDSCH mapping type is set to Type A or Type B as defined in Clause 7.4.1.1.2 of [4, TS 38.211]. <Unchanged part omitted>. |