3GPP TSG RAN WG1 #121		        						 R1-2504232

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	OPPO

Title:	UE features for NR MIMO Phase 5

Agenda Item:	9.15.2

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the UE feature for Rel-19 MIMO evolution.

UE feature for Rel-19 MIMO

  - UE feature for UE-initiated/event-driven beam management

For supporting UE-initiated/event-driven beam reporting event-2, event-1 or event-7, the UE needs to support unified TCI framework. Thus supporting unified TCI framework shall be the pre-requisite of FG 59-1-1, 59-1-4 and 59-1-5. However, supporting event-2 shall not be the pre-requisite of supporting event 1 or event 7.

Proposal 1: The pre-requisite for FG 59-1-1, 59-1-4 and 59-1-5 is FG of supporting the unified TCI framework.

And for the contents in each FGs

- The FG 59-1-1: the FG name shall include the wording of event-2 and mode-A to clarify the contents of FG 59-1-1.

- For FG 59-1-2: we shall keep the components 3. The pre-requisite of 59-1-2 is 59-1-1 or 59-1-4 or 59-1-5.

- FG 59-1-3: the function of M >1 with a time window shall be applied to event 2 or event 1, but not event 7. Thus, the pre-requisite is 59-1-1 or 59-1-4.

- FG 59-1-4: it should be UE capability for mode-A only. The pre-requisite shall be the supporting of unified TCI framework.

- FG 59-1-5: similar to FG 59-1-4, it should be UE capability for mode A only. The pre-requisite shall be the supporting of unified TCI framework.

Therefore, the following proposal is made for the UE features of UEI beam measurement reporting:

Proposal 2: Updating the UE features of UEI beam reporting as follows:

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [for Event-2 and Mode A] | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 4 candidate values: {1, 2, … , 64}] Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Maximum number of the configured RS(s) for new beam in the RS resource set] [3. Supported values of X] symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH] | 59-1-1 Or 59-1-4 Or 59-1-5 FFS for other potential FG(s) | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-2 | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to have separate FGs for event 2/1/7 for mode B | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. | 1. Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) for at least one new beam within a time window [with M>1 per one new beam], where M > 1 | 59-1-1 or 59-1-4 FFS for other potential FG(s) | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | FFS Per band | FFS n/a | FFS | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] [4. Support of Mode A for Event-1 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported report [for Mode A] | Per Band | n/a | FFS | n/a | [Component 3 candidate values: {FFS}] Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] [2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set] [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] [4. Support of Mode A for Event-7 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS | n/a | Component 1 candidate values for Q: FFS [{1, 2, ..., 8}] [Component 2 candidate values: {1, 2, ..., 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UE feature for CSI enhancements

Agreement For the Rel-19 Type-I SP and Type-II codebook refinements (except based on Rel-18 Type-II Doppler) for 48, 64, and 128 CSI-RS ports, regarding CPU occupation - For Capability 1 timeline: OCPU = ceil(P/32) - For Capability 2 timeline: OCPU = 1

According to the agreements, we have following suggestions for UE feature for up to 128 ports CSI-RS/codebook:

- The prerequisite of group-specific 3-bit scaling factors should be Rel-19 Type-I SP codebook (FG 59-2-1-1).

- The prerequisite of NES SD Type 1 for Rel-19 Type-I single-panel codebook should be Rel-18 NES SD Type 1 (FG 42-1). The note for FG 59-2-1-9 should be deleted since a UE supporting Rel-19 codebook refinement is not mandated to support NES SD Type1 codebook.

- For FG 59-2-1-10, based on the above agreement, the number of CPU is also added to Capability 2 timeline, similar to Capability 2 timeline. The FFS part is deleted.

- Add prerequisite for some other FGs.

Proposal 3: The FGs for 128 ports codebook refinement can be updated as:

| 59. NR_MIMO_Ph5 | 59-2-1-7 | Group-specific 3-bit scaling factors for up to 128 ports | Support of group-specific 3-bit scaling factors | 59-2-1-1 | yes | n/a | Group-specific 3-bit scaling factors is not supported | FFS | FFS | FFS | FFS | Candidate values: {’rank-1’, ‘rank-1 and rank-2’} Note: 3-bit scaling applies only to the Type-I SP codebook | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-9 | NES SD Type1 for Rel-19 Type-I single-panel codebook | 1. Support NES SD Type1 for Rel-19 Type-I single-panel codebook 2. Supported NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook | 42-1 | Yes | n/a | NES SD Type1 for Rel-19 Type-I single-panel codebook is not supported | [Per-band and per-BC] | n/a | n/a | n/a | Component 2 candidate values: - Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) - Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources | Optional with capability signaling |
| 59. NR_MIMO_Ph5 | 59-2-1-10 | CSI processing timeline for Rel-19 Type-I SP and Type-II codebook | Support from two timeline capabilities, for Rel-19 Type-I single-panel codebook and Type-II codebook | [59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a] | Yes | n/a | gNB does not know UE’s CSI processing timeline | [Per-band and per-BC] | n/a | n/a | n/a | Component 1 candidate values: - Capability 1: Reuse legacy Z/Z’ values, OCPU = ceil(P/32) - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources, OCPU = 1 Note: A UE that supports 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a must signal this FG | Optional with capability signaling |
| 59. NR_MIMO_Ph5 | 59-2-2-3a | Configuration of MR always-reported resources with Rel-15 Type-I SP codebook | Support MR={1,2} for hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | 59-2-2-1 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 with Rel-15 Type-I SP codebook | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-3b | Configuration of MR always-reported resources with Rel-16 eType-II codebook with R=1 | Support MR={1} for hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1 | 59-2-2-2 | Yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 with Rel-16 eType-II codebook with R=1 | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4 | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH | 1. Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in a set of aggregated resources, Max # of sets of aggregated resource, and total # of Tx ports} simultaneously | 2-15a | Yes | n/a | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in in a set of aggregated resources is {48, 64, 128} The candidate value set of the max # of sets of aggregated resource is: {2, …, 64} The candidate value set of total # of ports is: {48, …, 256} | Optional with capability signalling |

During the Rel-19 MIMO discussion, the following agreements were made for UE capability on CJT calibration reporting explicitly.

Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, regarding active resource counting and OCPU, when ReportQuantity is ‘cjtc-Dd’ (Doffset+d) or cjtc-F’ (frequency offset), fully reuse those from Rel-18 TDCP reporting - OCPU =X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type Agreement For the Rel-19 aperiodic standalone CJT calibration reporting, when ReportQuantity is ‘cjtc-Dd-F’ (joint Doffset+d and FO) - Fully reuse timeline and active resource counting from Rel-18 TDCP reporting - OCPU = 2X.NTRP where X≥1 is defined based on UE capabilities and determined by the UE for each CJT calibration report type

Proposal 4: The FGs for CJT calibration (CJTC) reporting can be updated as:

| 59. NR_MIMO_Ph5 | 59-2-3-1 | CJTC Dd report | 1. Configured minimum quantization range for CJTC Dd reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC Dd reporting 3. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC Dd report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {half cyclic prefix, full cyclic prefix} Component 2 candidate values: {32, 64, 128, 256} Component 3 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3-2 | CJTC FO report | 1. Configured minimum quantization range for CJTC FO reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC FO reporting 3. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC FO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {0.1ppm, 0.2ppm}] Component 2 candidate values: {16, 32, 256}] Note: parts per million (ppm) of the carrier frequency Component 3 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-3 | CJTC wideband PO report | 1. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC WB PO reporting 2. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC PO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {16, 32} Component 2 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-4 | CJTC subband PO report | 1. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC SB PO reporting 2. Configured minimum subband size in resource blocks for the CJTC subband PO report 3. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC subband PO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {16, 32} Component 2 candidate values: {1, 2, 4, 8, 16} Component 3 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-5 | CJTC Dd+FO report | 1. Configured minimum quantization range for CJTC Dd reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC Dd reporting 3. Configured minimum quantization range for CJTC FO reporting 4. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC FO reporting 5. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC Dd+FO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {half cyclic prefix, full cyclic prefix} Component 2 candidate values: {32, 64, 128, 256} Component 3 candidate values: {0.1ppm, 0.2ppm} Component 4 candidate values: {16, 32, 256} Note: parts per million (ppm) of the carrier frequency Component 5 candidate values: {1, 1.5, 2}. Note：OCPU =2X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-7 | Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering | 1. Support of joint triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI | 59-2-3-1, 40-3-1-1 | yes | n/a | Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-8 | Linkage of CJTC Dd and Rel-18 eType-II CJT with separate triggering | Support separate triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI | 59-2-3-1, 40-3-1-1 | yes | n/a | Separate triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-8a | Separate triggering with configuration of 1-bit indicator per CSI trigger state | Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting | 59-2-3-8 | yes | n/a | Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-7a | Relaxed timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT | Support of relaxed timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT, i.e., Drelax = drelax | 59-2-3-7 | yes | n/a | Relaxed timeline for joint triggering od CJTC Dd and Rel-18 eType-II CJT is not supported, i.e., Drelax = 0 | Per band and per BC | n/a | n/a | n/a | Component candidate values: 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448} | Optional with capability signalling |

During the Rel-19 MIMO discussion, the following agreements were made for UE capability on SRS port grouping explicitly.

Agreement For a UE configured with a total of PSRS=6 or 8 ports across ≥1 SRS resources for antenna switching intended for xT6R or xT8R, respectively, support the following fixed SRS port grouping where (with the PSRS ports indexed in an ascending order according to SRS resource ID and port number within each SRS resource): - SRS port group 0, corresponding to CW0, comprises the even PSRS/2 out of PSRS ports; and - SRS port group 1, corresponding to CW1, comprises the odd PSRS/2 out of PSRS ports The above feature is applicable only for reportQuantity = ‘cri-RI-CQI’ No other spec enhancement is introduced on new CW-to-layer mapping, DL resource allocation, CSI feedback, and DCI format Note: The above grouping assumption is to align NW and UE on the association between SRS ports and reported CQIs for the two CWs when reportQuantity = ‘cri-RI-CQI’. Note: different SRS ports are associated with different UE antenna ports. Note: if one single CW is scheduled, both SRS port groups can correspond to the same CW, i.e. no enhancement is needed for the single-CW case Note: This feature is a separate UE capability and, for UEs supporting this capability, configured via RRC (FFS details on the extend of RRC configuration) Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI FFS (by RAN1#118bis): Whether there is impact on mapping between CWs to CSI-RS ports For SRS antenna switching with multiple aperiodic SRS resource sets, PSRS ports indexed in an ascending order according to SRS resource set ID and SRS resource ID in a set and port number within each SRS resource

For SRS port grouping, separate UE capabilities are needed for 8Rx and 6Rx UEs, and the prerequisite should be FG 2-38 (DL CSI without PMI), since it is only applicable for reportQuantity = ‘cri-RI-CQI’.

Proposal 5: The FGs for SRS port grouping: can be updated as:

| 59. NR_MIMO_Ph5 | 59-2-1-8 | SRS Port Grouping | Support of SRS port grouping | 2-38 | yes | n/a | SRS Port Grouping is not supported | FFS | FFS | FFS | FFS | Candidate values: {xT8R, xT6R} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UE feature for 3-antenna-port codebook-based transmissions

According to the agreements in last meeting, we have following suggestions for UE feature for 3Tx uplink transmission:

- For the agreed UE features in last meeting, we highlight our changes on granularities in revisions mode.

- For codebook-based 3Tx transmission, the following agreement was reached in the RAN1#117 meeting. However, the second bullet has not yet been included in the UE capability list. This is closely related to the description of power scaling in TS 38.214. We support to add a new component 5 in FG 59-3-2 to reflect the second bullet in the agreement.

Agreement For codebook-based UL transmission by a 3TX UE, subject to its capability, - A 3TX UE may report a maximum number of 3 layers - A 3TX UE may report a maximum number of SRS ports of up to 3 Note: SRS resource definition is not changed nor the number of SRS ports in the SRS resource.

Proposal 6: The following UE features can be supported for 3Tx uplink transmission:

| 59. NR_MIMO_Ph5 | 59-3-1a |  | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP | 1. Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} simultaneously |  | 59-3-1 |  | yes | n/a |  | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported |  | FFS |  | FFS |  | FFS |  | FFS |  | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: {2 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 |  | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP |  | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 5. Support 3 SRS ports with port 1003 disabled for codebook-based 3Tx PUSCH |  | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported |  | Per FSPC |  | n/a |  | n/a |  | n/a |  | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} |  | Optional with capability signalling |
| NR MIMO Phase 5 |  | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission |  | Number of supported PTRS ports for PUSCH transmission |  | [59-3-1 or] 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission |  | FSPC |  | n/a |  | n/a |  | n/a |  | [Candidate values: {1,2}] |  | Optional with capability signalling |

Conclusion

In this contribution, we discuss the potential UE features for Rel-19 MIMO with the following proposals:

Proposal 1: The pre-requisite for FG 59-1-1, 59-1-4 and 59-1-5 is FG of supporting the unified TCI framework.

Proposal 2: Updating the UE features of UEI beam reporting as follows:

| 59. NR_MIMO_Ph5 | 59-1-1 | UE-initiated/event-driven beam management [for Event-2 and Mode A] | 1. Support of UE-initiated/event-driven beam report based on one event instance 2. Support of Event-2 based measurement and report 3. Support of Mode A UE-initiated/event-driven beam report 4. Maximum number of the configured RS(s) for new beam in the RS resource set 5. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively 6. Support the first PUCCH and second PUSCH from the same PUCCH group | FFS | yes | n/a | UEI/ED beam report is not supported [for Event-2 and Mode A] | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 4 candidate values: {1, 2, … , 64}] Note For Component 4 and Component 5, an SSB can be associated with the serving cell PCI or a PCI other than the serving cell PCI | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-2 | UE-initiated/event-driven beam management Mode B | 1. Support of Mode B UE-initiated/event-driven beam report [2. Maximum number of the configured RS(s) for new beam in the RS resource set] [3. Supported values of X] symbols between the last symbol of sending first PUCCH and the first available symbol of transmission occasion of second PUSCH] | 59-1-1 Or 59-1-4 Or 59-1-5 FFS for other potential FG(s) | yes | n/a | UEI/ED beam report Mode-B is not supported for Event-2 | FFS Per band | FFS n/a | FFS | FFS n/a | [Component 2 candidate values: {0,1,…,[N]}] FFS: whether to have separate FGs for event 2/1/7 for mode B | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-3 | Triggering event determination via detecting ≥ M event instances for at least one new beam within a time window. | 1. Triggering UEI/ED beam report procedure via detecting ≥ M event instance(s) for at least one new beam within a time window [with M>1 per one new beam], where M > 1 | 59-1-1 or 59-1-4 FFS for other potential FG(s) | yes | n/a | Triggering event determination via detecting ≥ M event instances within a time window is not supported | FFS Per band | FFS n/a | FFS | FFS n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-4 | UE-initiated/event-driven beam management for Event-1 based measurement and report [for Mode A] | 1. Support of Event-1 based measurement and report that L1-RSRP of the current beam becomes worse than a configured threshold based on one event instance [within a time window] [2. Support of current beam measurement by using QCL RS in the indicated TCI state and the corresponding QCL SSB for Scheme-1 and Scheme-2, respectively] [3. Maximum number of the configured RS(s) for new beam measurement in the RS resource set] [FFS: memory] [4. Support of Mode A for Event-1 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-1 based measurement is not supported report [for Mode A] | Per Band | n/a | FFS | n/a | [Component 3 candidate values: {FFS}] Note: the event definition Event 1: Quality of the current beam is worse than a certain threshold | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 59. NR_MIMO_Ph5 | 59-1-5 | UE-initiated/event-driven beam management for Event-7 based measurement and report [for Mode A] | 1. Support of Event-7 that L1-RSRP of at least one new beam becomes a threshold value better than the RS derived from the activated TCI state with the Q-th best quality based on one event instance [within a time window] [2. Maximum number of the configured RS(s) for new beam measurement and event evaluation in the RS resource set] [3. Support of the RS derived from the activated TCI state with the Q-th best quality measurement by using QCL RS in the activated TCI state with the Q-th best quality and the corresponding QCL SSB for the activated TCI state with the Q-th best quality for Scheme-1 and Scheme-2, respectively] [4. Support of Mode A for Event-7 UE-initiated/event-driven beam report] | FFS | yes | n/a | UE-initiated/event-driven beam management for Event-7 based measurement is not supported [for Mode A] | Per Band | n/a | FFS | n/a | Component 1 candidate values for Q: FFS [{1, 2, ..., 8}] [Component 2 candidate values: {1, 2, ..., 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: The FGs for up to 128 ports codebook enhancement can be updated as:

| 59. NR_MIMO_Ph5 | 59-2-1-7 | Group-specific 3-bit scaling factors for up to 128 ports | Support of group-specific 3-bit scaling factors | 59-2-1-1 | yes | n/a | Group-specific 3-bit scaling factors is not supported | FFS | FFS | FFS | FFS | Candidate values: {’rank-1’, ‘rank-1 and rank-2’} Note: 3-bit scaling applies only to the Type-I SP codebook | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-1-9 | NES SD Type1 for Rel-19 Type-I single-panel codebook | 1. Support NES SD Type1 for Rel-19 Type-I single-panel codebook 2. Supported NES SD Type1 timeline from two timeline capabilities, for Rel-19 Type-I single-panel codebook | 42-1 | Yes | n/a | NES SD Type1 for Rel-19 Type-I single-panel codebook is not supported | [Per-band and per-BC] | n/a | n/a | n/a | Component 2 candidate values: - Capability 1: Reuse legacy Z/Z’ values (i.e., Z2 and Z’2) - Capability 2 timeline: Scale the legacy timeline Z/Z’ (i.e., Z2 and Z’2) by where M is the number of sub-configurations that refer to the any of the K aggregated CSI-RS resources | Optional with capability signaling |
| 59. NR_MIMO_Ph5 | 59-2-1-10 | CSI processing timeline for Rel-19 Type-I SP and Type-II codebook | Support from two timeline capabilities, for Rel-19 Type-I single-panel codebook and Type-II codebook | [59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a] | Yes | n/a | gNB does not know UE’s CSI processing timeline | [Per-band and per-BC] | n/a | n/a | n/a | Component 1 candidate values: - Capability 1: Reuse legacy Z/Z’ values, OCPU = ceil(P/32) - Capability 2: Scale the legacy timeline Z/Z’ by ceil(P/32) where P is the total number of ports across all the K aggregated CSI-RS resources, OCPU = 1 Note: A UE that supports 59-2-1-1, 1a, 1b, 1c, 1d, 1e, 3, 3a, 3b, 4, or 4a must signal this FG | Optional with capability signaling |
| 59. NR_MIMO_Ph5 | 59-2-2-3a | Configuration of MR always-reported resources with Rel-15 Type-I SP codebook | Support MR={1,2} for hybrid BF (CRI-based) with Rel-15 Type-I SP codebook | 59-2-2-1 | yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 with Rel-15 Type-I SP codebook | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-2-3b | Configuration of MR always-reported resources with Rel-16 eType-II codebook with R=1 | Support MR={1} for hybrid BF (CRI-based) with Rel-16 eType-II codebook with R=1 | 59-2-2-2 | Yes | n/a | Configuration of MR always-reported resources is not supported, i.e. MR=0 with Rel-16 eType-II codebook with R=1 | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-4 | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH | 1. Support association between {48, 64, 128} CSI-RS ports and SRS resource set for non-codebook-based PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in a set of aggregated resources, Max # of sets of aggregated resource, and total # of Tx ports} simultaneously | 2-15a | Yes | n/a | Association up to 128 CSI-RS ports and SRS for non-codebook-based PUSCH is not supported | Per FS | n/a | n/a | n/a | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in in a set of aggregated resources is {48, 64, 128} The candidate value set of the max # of sets of aggregated resource is: {2, …, 64} The candidate value set of total # of ports is: {48, …, 256} | Optional with capability signalling |

Proposal 4: The FGs for CJT calibration (CJTC) reporting can be updated as:

| 59. NR_MIMO_Ph5 | 59-2-3-1 | CJTC Dd report | 1. Configured minimum quantization range for CJTC Dd reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC Dd reporting 3. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC Dd report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {half cyclic prefix, full cyclic prefix} Component 2 candidate values: {32, 64, 128, 256} Component 3 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-2-3-2 | CJTC FO report | 1. Configured minimum quantization range for CJTC FO reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC FO reporting 3. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC FO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {0.1ppm, 0.2ppm}] Component 2 candidate values: {16, 32, 256}] Note: parts per million (ppm) of the carrier frequency Component 3 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-3 | CJTC wideband PO report | 1. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC WB PO reporting 2. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC PO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {16, 32} Component 2 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-4 | CJTC subband PO report | 1. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC SB PO reporting 2. Configured minimum subband size in resource blocks for the CJTC subband PO report 3. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC subband PO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {16, 32} Component 2 candidate values: {1, 2, 4, 8, 16} Component 3 candidate values: {1, 1.5, 2}. Note：OCPU =X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-5 | CJTC Dd+FO report | 1. Configured minimum quantization range for CJTC Dd reporting 2. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC Dd reporting 3. Configured minimum quantization range for CJTC FO reporting 4. Configured maximum resolution (number of steps) for the quantization alphabet for CJTC FO reporting 5. Supported value of scaling factor X for OCPU calculation | FFS | yes | n/a | CJTC Dd+FO report is not supported | FFS | FFS | FFS | FFS | Component 1 candidate values: {half cyclic prefix, full cyclic prefix} Component 2 candidate values: {32, 64, 128, 256} Component 3 candidate values: {0.1ppm, 0.2ppm} Component 4 candidate values: {16, 32, 256} Note: parts per million (ppm) of the carrier frequency Component 5 candidate values: {1, 1.5, 2}. Note：OCPU =2X.NTRP | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-7 | Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering | 1. Support of joint triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI | 59-2-3-1, 40-3-1-1 | yes | n/a | Linkage of CJTC Dd and Rel-18 eType-II CJT with joint triggering is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-8 | Linkage of CJTC Dd and Rel-18 eType-II CJT with separate triggering | Support separate triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI | 59-2-3-1, 40-3-1-1 | yes | n/a | Separate triggering for linked CJTC Delay offset reporting and Rel-18 eType-II CJT CSI is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-8a | Separate triggering with configuration of 1-bit indicator per CSI trigger state | Support of 1 bit indicate per trigger state for separate triggering of linked DO reporting and Type II CJT reporting | 59-2-3-8 | yes | n/a | Separate triggering with configuration of 1-bit indicator per CSI trigger state is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |
| 59. NR_MIMO_Ph5 | 59-2-3-7a | Relaxed timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT | Support of relaxed timeline for joint triggering of CJTC Dd and Rel-18 eType-II CJT, i.e., Drelax = drelax | 59-2-3-7 | yes | n/a | Relaxed timeline for joint triggering od CJTC Dd and Rel-18 eType-II CJT is not supported, i.e., Drelax = 0 | Per band and per BC | n/a | n/a | n/a | Component candidate values: 15kHz SCS: {2, 4, 8} 30kHz SCS: {4, 8, 14, 28} 60kHz SCS: {8,14, 28} 120kHz SCS: {14,28, 56} 480kHz SCS: {56, 112, 224} 960kHz SCS: {112, 224, 448} | Optional with capability signalling |

Proposal 5: The FGs for SRS port grouping can be updated as:

| 59. NR_MIMO_Ph5 | 59-2-1-8 | SRS Port Grouping | Support of SRS port grouping | 2-38 | yes | n/a | SRS Port Grouping is not supported | FFS | FFS | FFS | FFS | Candidate values: {xT8R, xT6R} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: The following UE features can be supported for 3Tx uplink transmission:

| 59. NR_MIMO_Ph5 | 59-3-1a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH transmission for single TRP | 1. Support the association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH 2. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources, and total # of Tx ports} simultaneously | 59-3-1 | yes | n/a | Association between CSI-RS and SRS for non-codebook-based 3Tx PUSCH is not supported | FFS | FFS | FFS | FFS | Component 2 candidate value: Maximum size of the list is 16. The candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {1 to 64} The candidate value set of total # of ports is: {2 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 59. NR_MIMO_Ph5 | 59-3-2 | Codebook based PUSCH transmission for 3TX for single TRP | 1. Maximal number of PUSCH MIMO layers for codebook-based PUSCH 2. Maximum number of 4-port SRS resources per SRS resource set with usage set to 'codebook’ for codebook-based 3Tx PUSCH [3. Maximum number of simultaneous transmitted SRS resources at one symbol with usage set to 'codebook’ for codebook-based 3Tx PUSCH] 4. Codebook based PUSCH transmission with port 1003 disabled when 4 port SRS resources with port 1003 disabled are configured to the UE 5. Support 3 SRS ports with port 1003 disabled for codebook-based 3Tx PUSCH | FFS | yes | n/a | Codebook based PUSCH transmission for 3TX is not supported | FFS Per FSPC | FFS n/a | FFS n/a | FFS n/a | Component 1 candidate values: {1, 2,3} Component 2 candidate values: {1,2} [Component 3 candidate values: {1,2,3}] | Optional with capability signalling |
| NR MIMO Phase 5 | 59-3-6 | PTRS of 3-antenna-port PUSCH transmission | Number of supported PTRS ports for PUSCH transmission | [59-3-1 or] 59-3-2 | yes | n/a | PTRS is not supported for 3TX PUSCH transmission | FSPC | No | No | No | [Candidate values: {1,2}] | Optional with capability signalling |