3GPP TSG RAN WG1 #121		R1-2504234

St Julian’s, Malta, May 19th – 23th, 2025

Source:	OPPO

Title:	Discussions on UE features for NR mobility enhancements

Agenda Item:	9.15.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we will provide our views on the UE features for rel-19 NR mobility enhancements including L1-RSRP based on CSI-RS, LTM event triggering L1 measurement reporting and CSI acquisition of candidate cell.

- Discussions on UE features

For FG 63-1, the maximum number of CSI-RS resource for L1 measurement of each candidate cell shall be UE capability. And the maximum number of all CSI-RS resources for all candidate cell for L1 measurement shall also be a UE capability. However, the maximum number of CSI report configuration of CSI-RS resource of candidate cell is not a UE capability. The is applicable to FG 63-2 of semi-persistent CSI-RS resource.

Therefore, the FGs 63-1 and 63-2 shall be updated as follows:

Proposal 1: Update the FGs 63-1 and 63-2 as follows:

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of CSI-RS resources per candidate cell configured for intra-frequency L1-RSRP measurement 5. Maximum of the total number of CSI-RS resources of candidate cells configured for intra-frequency L1-RSRP measurement. 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 4. Maximum number of CSI-RS resources per candidate cell configured for intra-frequency L1-RSRP measurement 5. Maximum of the total number of CSI-RS resources of candidate cells configured for intra-frequency L1-RSRP measurement. | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |

UE FGs of supporting CSI-RS in TCI state were agreed. For FG-3a, the maximum number of activated joint LTM TCI states with CSI-RS as QCL source shall be UE capability and the maximum number of all activated LTM TCI states of all candidate cells shall also be a component. The same components shall be included in FG-4a for the case of separate TCI states.

Therefore, the FGs 63-3a and 63-4a shall be updated as follows:

Proposal 2: Update the FGs 63-3a and 63-4a as follows:

| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | 1. Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of activated joint LTM TCI states with CSI-RS for BM as Type-D QCL source per candidate cell. 3. Maximum number of activated joint LTM TCI states with CSI-RS for BM as Type-D QCL source of all candidate cell. | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states 2. Maximum number of activated separate LTM DL/UL TCI states with CSI-RS for BM as Type-D QCL source per candidate cell. 3. Maximum number of activated separated LTM DL/UL TCI states with CSI-RS for BM as Type-D QCL source of all candidate cell. | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states |

Re the FGs 63-6 and 63-7 for CSI acquisition of candidate cell, the maximum number of CSI report configs is not needed because in RAN1, we have agreed one CSI report configuration for one candidate cell. The maximum number of candidate cells for CSI acquisition is sufficient.

Proposal 3: Update the FGs 63-6 and 63-7 as follows:

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of candidate cells configured for CSI measurement and reporting. 3. Maximum number of CSI-RS resource for CSI measurement and reporting per candidate cell 4. Maximum number of CSI-RS resources for CSI measurement and reporting across all candidate cells. 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources for CSI measurement before LTM CSC MAC CE per candidate cell 4. Maximum number of CSI-RS resources across all candidate cells for CSI measurement before LTM CSC MAC CE | 63-6 | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported |

- Conclusion

In this contribution, we discuss the UE features for Rel-19 mobility with the following proposals:

Proposal 1: Update the FGs 63-1 and 63-2 as follows:

| 63. NR_Mob_Ph4 | 63-1 | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on periodic CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resource 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of CSI-RS resources per candidate cell configured for intra-frequency L1-RSRP measurement 5. Maximum of the total number of CSI-RS resources of candidate cells configured for intra-frequency L1-RSRP measurement. 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on periodic CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-2 | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure | 1. Support of intra-frequency L1- RSRP measurement and reporting based on CSI-RS(s) of candidate cell(s) 2. Maximum number of RRC configured candidate cells for intra-frequency L1-RSRP measurement on CSI-RS resouce 3. Support of up to L candidate cells and M beams in one report where a CRI-RSRP pair is used for each beam report for intra-frequency L1-RSRP measurement 4. Maximum number of LTM CSI report configs using CSI-RS as measurement resource 4. Maximum number of CSI-RS resources per candidate cell configured for intra-frequency L1-RSRP measurement 5. Maximum of the total number of CSI-RS resources of candidate cells configured for intra-frequency L1-RSRP measurement. | FFS | Yes |  | NW triggered intra-frequency L1-RSRP measurement based on semi-persistent CSI-RS (s) for L1-L2 Triggered Mobility (LTM) procedure is not supported |

Proposal 2: Update the FGs 63-3a and 63-4a as follows:

| 63. NR_Mob_Ph4 | 63-3a | CSI-RS as Type-D QCL source RS for MAC-CE activated joint LTM TCI states | 1. Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states 2. Maximum number of activated joint LTM TCI states with CSI-RS for BM as Type-D QCL source per candidate cell. 3. Maximum number of activated joint LTM TCI states with CSI-RS for BM as Type-D QCL source of all candidate cell. | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated joint LTM TCI states |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-4a | CSI-RS as Type-D QCL source RS for MAC-CE activated separate DL/UL LTM TCI states | Support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states 2. Maximum number of activated separate LTM DL/UL TCI states with CSI-RS for BM as Type-D QCL source per candidate cell. 3. Maximum number of activated separated LTM DL/UL TCI states with CSI-RS for BM as Type-D QCL source of all candidate cell. | FFS | Yes | No | UE does not support CSI-RS for BM as Type-D QCL source RS and TRS as Type-A QCL source RS for MAC-CE activated separate DL/UL LTM TCI states |

Proposal 3: Update the FGs 63-6 and 63-7 as follows:

| 63. NR_Mob_Ph4 | 63-6 | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement and CSI reporting after reception of LTM CSC MAC CE based on periodic CSI-RS(s) of cell indicated in CSC MAC CE 2. Maximum number of candidate cells configured for CSI measurement and reporting. 3. Maximum number of CSI-RS resource for CSI measurement and reporting per candidate cell 4. Maximum number of CSI-RS resources for CSI measurement and reporting across all candidate cells. 2. Maximum number of the RRC configured candidate cells and CSI-RS resources 3. Maximum number of CSI report configs | FFS | Yes |  | Intra-frequency CSI-RS measurement and CSI reporting for cell indicated in CSC MAC CE after reception of LTM CSC MAC CE is not supported |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 63. NR_Mob_Ph4 | 63-7 | Intra-frequency CSI-RS measurement for candidate cell [after the RRC configuration of configured CSI-RS resource(s) and] before reception of LTM CSC MAC CE | 1. Support of CSI-RS measurement before reception of CSC MAC CE 2. Maximum number of candidate cells for CSI measurement before LTM CSC MAC CE 3. Maximum number of CSI-RS resources for CSI measurement before LTM CSC MAC CE per candidate cell 4. Maximum number of CSI-RS resources across all candidate cells for CSI measurement before LTM CSC MAC CE | 63-6 | Yes |  | Intra-frequency CSI-RS measurement for candidate cell after the RRC configuration of configured CSI-RS resource(s) and before reception of LTM CSC MAC CE is not supported |