3GPP TSG RAN WG1 #121				R1-2504243

Saint Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.4.1

Source: 	LG Electronics

Title: 	A-IoT PHY layer design – waveform and modulation aspects

Document for:	Discussion and decision

- Introduction

The Re1-19 WI on solutions for Ambient IoT was approved in RAN#106 meeting. The WI objective relevant to this agenda item is as follows [1].

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI This WI is to standardize RAN aspects of Ambient IoT, a new 3GPP IoT technology, suitable for deployment in a 3GPP system, which relies on ultra-low complexity devices with ultra-low power consumption for the very-low end IoT applications. The work shall provide clear differentiation, i.e. addressing use cases and scenarios that cannot otherwise be fulfilled based on existing 3GPP LPWA IoT technology e.g. NB-IoT including with reduced peak Tx power. General Scope The definitions provided in TR 38.848, TR 38.769, and decisions, etc. made during the Rel-19 SI in RAN WGs are taken into this WI, and the following is the exclusive general scope: - The overall objective shall be to standardize the following Ambient IoT device: Device 1: ~1 µW peak power consumption, has energy storage, RF envelope detector receiver, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Deployment scenario 1 with Topology 1, according to D1T1-B. - FR1 licensed spectrum in FDD, with R2D in DL spectrum and D2R and CW in UL spectrum. - Spectrum deployment in-band to NR and standalone, with A-IoT BS located indoor. - Traffic types DO-DTT, DT, for rUC1 (indoor inventory) and rUC4 (indoor command). - Carrier wave transmission for waveform 1 only, without hopping, per the following cases in TR 38.769: - Case 1-4 for D1T1-B - Proximity determination via Solution 1 in TR 38.769 only. - Device (un)availability via Direction 1 in TR 38.769 only. WGs begin their discussions from the decisions already made in TR 38.769, with the following refinements for the scope: The following objectives are set, within the General Scope: - RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis). - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 (unless RAN1 decides to use other generator polynomials by RAN1#120bis) for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission. …

- Discussion

In this contribution, we present our views on waveform and modulation aspects of Ambient IoT physical layer design.

  - CP handling for R2D transmission

Following candidate methods were captured in TR 38.769 for CP handling and agreed in RAN1#120bis.

For CP handling, the following candidate methods are studied, on the basis of e.g., CP impact on R2D timing acquisition, and decoding & performance of PRDCH, reader and device implementation complexities, interference between R2D and NR DL/UL if in the same NR band, spectrum efficiency. Method Type 1: Removal of CP at device without specified transmit-side. Method Type 2: Ensure the CP insertion of OFDM-based waveform will not introduce false rising/falling edge between the last OOK chip in OFDM symbol (n-1) and the first OOK chip in OFDM symbol n. For Method Type 1, two ways that CP location/length can be determined are studied: Alt M1-1: Device assumes same CP length for each OFDM symbol, i.e. does not distinguish exact CP length among different OFDM symbols Alt M1-2: Duration between transition edges is utilized by device to determine CP location/length, i.e. if the duration appears to be invalid based on known chip duration For Method Type 1, at least for Alt M1-1, device needs to be aware of or determine the boundary of an OFDM symbol (i.e. the beginning of an OFDM symbol) to determine CP location, including the start and the length of CP. Alt M1-1 and/or Alt M1-2 (if both are supported) can be up to device implementation which may depend on M values. For Method Type 2, two approaches regarding subcarrier orthogonality are studied: Alt M2-1: Method Type 2 retains subcarrier orthogonality, i.e. CP is copied from the end of an OFDM symbol. Alt M2-1-1: The first OOK chip(s) and the last OOK chip(s) in an OFDM symbol are the same. Alt M2-1-2: Ensure a transition edge occurs only at the start or only at the end of the CP, and no transition edge occurs during the CP. Alt M2-2: Method Type 2 does not retain subcarrier orthogonality. For Method Type 2, depending on the design, the chip duration generation of OOK-4 for M-chip per OFDM symbol transmission could possibly be determined by: - M, and the length of OFDM symbol with CP - M, and the length of OFDM symbol without CP - Depending on detailed solutions, chip duration may or may not be constant.

Agreement For the below agreement, further update on the followings Agreement For further down-selection among CP handing which retains subcarrier orthogonality, at least for PRDCH, at least Method Type 1 is supported - For supported M values <= 12 - RAN1 will not further pursue additional CP handling design - For supported M values > 12 - RAN1 will further down-select one from the followings - Option 1: Candidate 3 of M2-1-1 (as per agreements from RAN1#120) - Insert padding chips only at the end OOK chips of OFDM symbol - The last 2 out of M OOK chips at the end of an OFDM symbol are always ‘ON’ - Option 3: RAN1 will not further pursue additional CP handling design

For R2D, CP handling methods were captured as above. As agreed in RAN1#120bis, Method Type 1 is considered as the baseline for CP handling and few companies have concerns about Method Type 1 when large M value (e.g. M=24) is used for R2D transmission. After reviewing a few commercial implementations of Manchester decoding, we think that for larger M value, the Alt M1-2 can also be used with the aid of knowledge of NR OFDM symbol boundaries which can be achieved by e.g., preamble or sync signals for R2D. If Alt M1-1 also assumes awareness of the NR OFDM symbol boundaries, one can also say that hybrid of Alt M1-2 and Alt M1-1 can handle the larger M value. With the understanding that it is feasible for Ambient IoT devices to handle the CP or receive R2D transmissions either without the need for handling the CP or without specification impact at the transmitter side, then we think Method Type 1 should be supported because it would provide better resource utilization with less specification impact regardless of M value and additionally it would be beneficial to have unified CP handling method regardless of M value for implementation issue.

Observation 1: Using Method Type 1 Alt 2 alone, Ambient IoT devices can handle the CP for M values up to 8.

Observation 2: Using Method Type 1 Alt 2 combined with Method Type 1 Alt 1, Ambient IoT devices can also handle the CP with large M values.

Proposal 1: Support Option 3 for supported M values > 12

Additionally, Option 1, which is a candidate of Method Type 2 related to the CP handling approach from the transmitter’s perspective, introduces additional resource overhead (e.g., 2 chips per OFDM symbol) compared to Method Type 1. Furthermore, the chip duration may become inconsistent due to CP insertion in Option 1, which can lead to issues with OOK decoding.

Observation 3: Additional resource overhead is required for Option 1

As agreed in RAN1#120bis, the above CP handling methods are supported at least for PRDCH. In the case of PRDCH, accurate timing of the OFDM symbol boundary is ensured through the support of R-TAS, which is transmitted prior to PRDCH. However, since R-TAS is transmitted first, it does not inherently guarantee accurate timing for the subsequent PRDCH transmission.

Regarding SIP detection, since it primarily involves pattern detection, the impact of CP insertion or CP handling on performance is expected to be minimal. However, CAP, which conveys the M value of PRDCH, differs significantly in this regard. Due to the large SFO, the CP handling method may considerably affect CAP performance, as CAP is not merely a pattern detection but carries critical information that directly impacts the correct decoding of the PRDCH M value.

From the perspective of maintaining orthogonality, it is essential to preserve the CP relationship even in the OFDM symbol used for CAP transmission. To achieve this, one common method is to copy the last chip of the preceding OFDM symbol (which corresponds to a PRDCH transmission chip when M > 2) as the CP. However, this approach can alter the ON chip duration of the first chip in the CAP symbol, leading to performance degradation in CAP detection.

Therefore, when designing CAP, it is crucial to ensure that the CP relationship is maintained within the CAP transmission OFDM symbol, while also minimizing the impact on CAP detection performance by specifying the CP as an OFF state.

Proposal 2: CAP should be designed to ensure reliable operation irrespective of the CP handling method

  - Chip duration for R2D transmission

Following proposal was made by FL in RAN1#120bis.

#### Proposal 4.2.1c-v1: Proposal: For R2D chip duration, the length of R2D chip duration (denoted as ‘C’) in one OFDM symbol is defined as ‘C = (1/SCS)/M’ at least for Method Type 1, - FFS: the length of R2D chip duration for other CP handling design is defined as one of the followings to be down-selected, - Option 1: all chips same as ‘C = (1/SCS)/M’ - Note: It implies up to device implementation to handle with knowing the length of R2D chip duration ‘C’ and the length of CP defined in the spec respectively. - Option 2: one or more chip(s) different to ‘C = (1/SCS)/M’ - FFS: details (e.g. the first chip adjacent to CP is defined as ‘C = (1/SCS)/M+CP’)

As discussed in previous meetings, R2D chip duration can defined by the consequence of CP handling. As aforementioned in section 2.1 (CP handling for R2D transmission), non-uniform chip duration may cause issued about OOK decoding. In this manner, at least when Method Type 1 is supported for CP handling, Option 1 should be supported.

Proposal 3: Support Option 1 for R2D chip duration

For R2D, a chip corresponds to one modulated symbol. The chip duration for R2D can be indicated by CAP in R-TAS. Additionally, M value for the chips for R2D data can also be indicated by a combination of CAP and L1 R2D control if the chips for L1 R2D control have a predefined M value or the same M value as the CAP, in which case the chips for L1 R2D control and those for R2D data may have different M values. In our view, the PRDCH is the only PHY channel for R2D transmission which means the chip duration should be the same throughout the PRDCH transmission.

Proposal 4: R2D chip duration is indicated in CAP of R-TAS.

Proposal 5: The chip durations for L1 R2D control (if supported) and R2D data are the same.

  - R2D numerology

For R2D, according to the WID, R2D transmission is generated by DFT-s-OFDM-based transmitter in TR 38.769 (see Section 2.3). For the case when PRDCH (including R2D postamble, if supported) does not fully occupy the last OFDM symbol, padding may be required to fulfil the OFDM symbol. Following was agreed in RAN1#120bis for the methods of padding.

Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

In RAN1#118bis meeting, it was agreed in AI 9.4.2.2 that for R2D waveform generation using DFT-s-OFDM-based transmitter, from transmitter perspective, when the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, at least padding can be used. From R2D transmitter’s perspective, we think aligning the end of R2D transmission with NR/LTE OFDM symbol boundaries is beneficial because otherwise those resources in the OFDM symbol not occupied by the Ambient IoT symbols would be wasted anyway. In this manner, padding should be supported to align the end of R2D transmission with NR/LTE OFDM symbol boundaries.

Although allowing reader-specific padding content may offer flexibility, specifying the padding content explicitly can bring multiple benefits. It facilitates additional synchronization, ensures accurate timing for the end of PRDCH, and enhance postamble’s (if supported) reliability. Additionally, unspecified padding content might lead to potential synchronization issues or difficulty in identifying the end of PRDCH, particularly when device awareness of padding duration is uncertain.

Proposal 6: Support Alt 2. The content of the padding can be specified to meet at least one of the following objectives

- Facilitating additional synchronization

- Indicating the end timing of PRDCH

- Enhance postamble’s (if supported) reliability

- Conclusion

In this contribution, we shared our views on general aspects of Ambient IoT physical layer design.

Observation 1: Using Method Type 1 Alt 2 alone, Ambient IoT devices can handle the CP for M values up to 8.

Observation 2: Using Method Type 1 Alt 2 combined with Method Type 1 Alt 1, Ambient IoT devices can also handle the CP with large M values.

Proposal 1: Support Option 3 for supported M values > 12

Observation 3: Additional resource overhead is required for Option 1

Proposal 2: CAP should be designed to ensure reliable operation irrespective of the CP handling method

Proposal 3: Support Option 1 for R2D chip duration

Proposal 4: R2D chip duration is indicated in CAP of R-TAS.

Proposal 5: The chip durations for L1 R2D control (if supported) and R2D data are the same.

Proposal 6: Support Alt 2. The content of the padding can be specified to meet at least one of the following objectives

- Facilitating additional synchronization

- Indicating the end timing of PRDCH

- Enhance postamble’s (if supported) reliability

- References

RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR