3GPP TSG RAN WG1 #121					  R1-2504251

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.6.2

Source: 	LG Electronics

Title: 	Discussion on LP-WUS operation in IDLE/INACTIVE modes

Document for:	Discussion and decision

Introduction

The following were agreed in the RAN1#120bis [1].

Agreement Confirm the following working assumption with modifications in red: Working Assumption If LP-WUS design support 32 subgroups within one MO, do not support Option 3 for LO to PO mapping or Option B for MO configuration. Agreement Option A for MO configuration is supported. Agreement A LP-WUS MO can span across multiple slots. - FFS the limitation on the maximum length for an LP-WUS MO Agreement Only R = 1 is supported for Option A. Conclusion For the offset value(s) between an LO and a reference PO/PF, do not support the configuration of 3 offset values. Agreement The maximum number of POs per LO is 4, and the number of POs per LO can be 1, 2 or 4. Proposal 2-3r2 For Option 2, the maximum value of M (number of LP-WUS MOs per beam) in Option A for MO configuration is 4. Proposal 4-2r1 For LP-WUS, the N * M LP-WUS MOs in an LO are indexed sequentially in time, from 1 to N*M, where N is the number of beams corresponding to LP-WUS, and M is the number of LP-WUS MOs for each beam. - The (n*M+m+1)-th LP-WUS MO corresponds to the (n+1)-th beam, where m=0,1,…,M-1, n=0,1,2,…,N-1. (multiple MOs first, beam second) Note: Above does not change the previous agreement on association between LP-WUS and SSB beams. Proposal 7-1 For OFDM-based LP-WUR, reuse the LP-SS based LP-RSRP/LP-RSRQ definition of OOK-based LP-WUR. FFS: Whether OFDM receiver can measure LP-SS if overlaid OFDM sequence is not configured (M=1).

In this contribution, we discuss and provide views on LP-WUS operation in IDLE/INACTIVE modes.

Discussion

  - LO and LMO allocation in time domain

Previous RAN1 agreements established an interconnected framework for LP-WUS, addressing three critical and interrelated aspects:

- First, symbol availability determination, which is fundamental for TDD operation

- Second, the determination of starting positions for LP-WUS MOs and transmission

- Third, the methods for determining symbol duration for each LP-WUS MO

Agreement UE determines whether a symbol is available for LP-WUS based on: - Alt 1: Time-domain pattern configured by the gNB - Alt 1A: Periodic time-domain pattern - E.g. (1) 1-slot periodicity, the pattern indicates the available symbols in each slot; (2) multi-slot or frame-level periodicity with a bitmap indication; (3) Search space set-like pattern; (4) multi-level time-domain patterns; (5) reuse the mechanism of rate matching pattern - Alt 1B: Per-MO pattern, applicable for all Mos - Alt 2: Information from existing configurations available for idle/inactive UEs such as [SSB, CORESET/Type-0 CSS, TDD DL/UL configuration, etc]. - Alt 3: Combination of Alt 1 and Alt 2 - Alt 4: NW ensures LP-WUS configuration without collision with existing signal(s)

For the first part, there had been similar discussions for various aspects. If we adopt the existing framework for Alt. 1 and 2, ratematching pattern or invalidSymbolPattern, (which are having same structure, Alt. 1A) can be considered for Alt.1 and at least one of following configuration can be considered for Alt. 2:

- A symbol that is indicated as uplink by tdd-UL-DL-ConfigurationCommon or tdd-UL-DLConfigurationDedicated is considered as an invalid symbol

- For operation in unpaired spectrum, symbols indicated by ssb-PositionsInBurst in SIB1 or ssb-PositionsInBurst in ServingCellConfigCommon for reception of SS/PBCH blocks are considered as invalid symbols

- For a reduced capability half-duplex UE in paired spectrum, symbols that do not start or end at least 𝑁Rx-Tx ⋅ 𝑇c or 𝑁Tx-Rx ⋅ 𝑇c, respectively, from the last or first symbol of an SS/PBCH block with index indicated by ssb- PositionsInBurst in SIB1 or by ssb-PositionsInBurst in ServingCellConfigCommon or by NonCellDefiningSSB, or by ssb-PositionsInBurst in SSB-MTC-AdditionalPCI associated to physical cell ID with active TCI states for PDCCH or PDSCH, or for a set of symbols of a slot corresponding to SS/PBCH blocks configured for L1 beam measurement/reporting for reception of SS/PBCH blocks are considered as invalid symbols

- For operation in unpaired spectrum, symbol(s) indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set are considered as invalid symbol(s)

For simple implementation, Alt. 2 may be considered. For ensuring future system flexibility, Alt. 1A may also be considered in addition.

Proposal 1: Support Alt. 2 for TDD handling

Proposal 2: For further flexibility, Alt. 1A can be considered in addition to Alt. 2.

- For Alt. 1A, re-use the structure of RateMatchPattern or invalidSymbolPattern

Agreement For the determination of starting time locations of LP-WUS MOs and LP-WUS transmissions in a LO, - A reference point is the start of a reference frame determined by the frame-level offset from the start of the first PF of the PF(s) associated with the LO. - The starting time location of the first LP-WUS MO in a LO is indicated by an offset w.r.t. the reference point. - FFS slot-level or symbol-level offset - The starting time locations of the subsequent LP-WUS MOs in a LO are determined based one of the following alternatives: - Alt 1: An offset is indicated for each of the subsequent LP-WUS MOs. - FFS slot-level or symbol-level offset - Alt 2: The start time location of a subsequent LP-WUS MO is determined implicitly at least based on the previous LP-WUS MO. - FFS additional configuration to control the subsequent MO locations, e.g., - Alt 2A: configuration of a single gap between the end of the previous MO (or a set of previous MOs) and the start of the next MO - Alt 2B: configuration of candidate starting locations for MOs, similar to search space configuration - FFS restriction on MO locations, e.g. only on DL slots - FFS minimum gap is needed between two MOs to ensure LR processing time

For starting position determination, LP-WUS MOs and transmissions within an LO are timed using a reference point and specific offsets for the initial and subsequent MOs, which can be explicit or implicitly derived. In the agreement, Alt. 1 and Alt. 2 have a lot of similarities. We can distinguish the approaches into two categories:

- Support configuring potential starting points: Alt 1 or Alt. 2B

- Support back-to-back allocation for LP-WUS MO within a LO: Alt. 2A.

Considering the complexity of TDD handling, we prefer deterministic starting point. Deterministic points reduce the UE monitoring burden by ensuring LP-WUS monitoring primarily occurs during valid DL periods and simplify gNB scheduling to avoid conflicts with other DL or UL transmissions. In addition, defining potential MO starting locations offers more granular and flexible control than configuring the gap between Mos. This granularity can be beneficial for placing MOs within available DL resources across diverse or dynamic TDD patterns, thereby ensuring efficient and predictable LP-WUS opportunities.

Proposal 3: Support Alt 1 or Alt. 2B for starting position determination of LP-WUS MO

Agreement Terminology definition - Nominal MO duration: this includes both the available and unavailable symbols - Actual LP-WUS duration: the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE Nominal MO duration and actual LP-WUS duration, if defined, are determined using one of the following alternatives: - Alt A: Nominal MO duration is configured. (e.g. in unit of slots) - Actual LP-WUS duration is the number of available symbols within the MO. - Actual LP-WUS duration can vary from one MO to another MO. - Alt B: Actual LP-WUS duration is configured. (e.g. in unit of OFDM symbols, or M and L values) - From the start of a MO, MO extends until the number of available OFDM symbols reaches the configured actual LP-WUS duration. - FFS: Additional termination condition such as time window - Alt C: Both nominal MO duration and actual LP-WUS duration are configured. - If the number of available OFDM symbols within the nominal MO duration is less than the actual LP-WUS duration, the MO is considered as invalid (no LP-WUS monitoring in this MO) and dropped/deferred. - FFS UE behavior if there is no valid MO for the beam(s) that the UE monitors - Note: the number of available OFDM symbols within a nominal MO duration can be different for different MOs. - Alt D: Nominal MO duration is configured. The actual LP-WUS duration is determined based on the same pattern for the available symbols for all the MOs (e.g. by using a per-MO pattern), which is the same for all the MOs.

Regarding the determination of LP-WUS MO duration, this is addressed through the agreed definitions of 'Nominal MO duration' (which includes both available and unavailable symbols within the MO) and 'Actual LP-WUS duration' (the actual number of OFDM symbols used for LP-WUS transmission as assumed by the UE). It is our understanding that the 'Actual LP-WUS duration' effectively means the 'required LP-WUS duration' from the UE's perspective and corresponds to the duration of the LP-WUS transmission that would eventually be transmitted.

From our perspective, several considerations are important:

- Considering beam-specific operations, it is crucial that every LP-WUS MO contains an adequate number of available symbols for reliable reception.

- However, it is problematic if a single LP-WUS MO, or consequently an entire LO, becomes excessively long in an attempt to accumulate these available symbols. This can lead to scheduling complexities, such as potential collisions with other LP-WUS MOs or with the occurrences of subsequent LOs.

- To mitigate this, the duration occupied by a single LP-WUS MO, and potentially the entire duration of the LO, should be maintained below a certain maximum threshold.

Among the discussed alternatives for duration determination, Our views are as follows:

- Alt A is considered a sufficiently supportable option, if there is appropriate gNB configuration of the nominal duration and a well-defined UE behavior for handling MOs that might be dropped or skipped invalid MO.

- Alt B, we think a time window as a termination condition is essential for ensuring that the MO or LO duration is bounded. Furthermore, limiting the overall LO duration could also be a beneficial consideration when Alt B is employed, to prevent excessive extensions.

- Alt C can be viewed as a complementary approach drawing elements from Alt A and Alt B. However, when compared specifically to Alt B (particularly with a time window termination condition), Alt C does not appear to offer significant distinct advantages in terms of managing MO duration while ensuring the required LP-WUS symbols.

- Alt D offers a deterministic actual LP-WUS duration based on a common pattern for available symbols. However, this approach may be less desirable as it could impose specific the TDD handling or the pattern of available symbols, which might be difficult to guarantee universally across diverse TDD configurations and could restrict scheduling flexibility.

Proposal 4: Deprioritize Alt. D for LP-WUS MO duration determination

Proposal 5: Consider following alternatives for LP-WUS MO duration determination:

- Alt. A

- Alt. B with a time window as a termination condition.

  - The time window can be configured per MO or per LO

Effective management of LO is crucial for reliable LP-WUS operation, complementing the determination of individual MO durations (as discussed with Alt. A, B, C, and D) and applying universally across these alternatives. An LO typically groups multiple, often sequentially allocated, LP-WUS MOs. It can be an issue that accommodating these MOs within potentially restrictive TDD UL/DL patterns may collide with an adjacent LO. Such overlaps would force the gNB to discard one of the LOs, thereby reducing LP-WUS availability and increasing the risk of reception errors or false alarms. While a gNB could attempt to adjust each LO's length on a case-by-case basis to avoid these collisions, this approach is impractical; ensuring no overlaps under all TDD conditions would likely require aligning all LOs in a cell to a worst-case TDD pattern, leading to significant inefficiency in resource utilization. Therefore, a more robust and explicit mechanism is required to define clear LO boundaries and mitigate these operational issues.

To address this, the gNB can configure an LO duration where LMO can be accommodated. The UE does not expect or receive any LP-WUS MOs outside of this LO duration.

Proposal 6: To avoid the overlapping between adjacent two LOs, introduce RRC parameter to configure the duration of a LO.

  - Handling for invalid LP-WUS MO

Regarding TDD handling for LP-WUS MOs, existing UE behavior related to PO can be considered. If this approach is adopted: The LP-WUS MOs that do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero, starting from the first LP-WUS MO in the LO.

However, given that the LO duration will be likely longer than the PO period, continuously postponing occasions as it has been done in PO can lead to persistent overlapping issues. Therefore, for LP-WUS, we propose skipping/dropping opportunities instead of postponing, at least for the case of more than one LP-WUS MO per beam.

Proposal 7: For LP-WUS operation in TDD configurations,

- If multiple LMO (for repetitions) are configured per beam, skip/drop the LP-WUS monitoring opportunity when it overlaps with UL symbols.

- Otherwise, postpone the LP-WUS monitoring opportunity for N*K LP-WUS MO when it overlaps with UL symbols.

By TDD handling, An LP-WUS MO may be validate or invalidated (e.g., skipped/dropped due to UL overlap) or its actual monitoring period may be skipped or postponed. The UE needs clear procedures to determine the start of monitoring for LP-WUS MOs in such situations. Since it is highly related to how to allocate MO and how to determine starting positions of MOs, we list two cases, 1) when potential starting points of LP-WUS MO are configured and 2) back-to-back LP-WUS MO operation.

When potential starting points of LP-WUS MO are configured and if a set of potential starting points for LP-WUS MOs is configured for the UE:

- Option 1-1: If a configured potential starting point is associated with an LP-WUS MO that becomes invalid (e.g., it is skipped/dropped due to UL overlap):

  - The UE shall consider this potential starting point as dropped/unavailable for the invalidated MO.

  - The UE shall then evaluate subsequent configured potential starting points to identify the next valid LP-WUS MO.

Fig. 1 An example of Option 1-1

- Option 1-2: If a nominal MO duration is configured:

  - the UE finds for the next configured potential starting point that occurs after the nominal end of this invalidated MO.

Fig. 2 An example of Option 1-2

- Option 1-3: If an actual MO duration is configured along with a termination window:

  - the UE shall identify the next configured potential starting point that occurs only after the end of the termination window associated with the preceding MO.

Fig. 3 An example of Option 1-3

- Some options may require specific method of MO duration determination.

In a back-to-back operation scenario,

- Option 2-1: If a nominal MO duration is configured:

  - Following an LP-WUS MO that is deemed invalid, the subsequent LP-WUS MO is expected to start at the beginning of the next nominal MO period that follows the invalid nominal MO period.

Fig. 4 An example of Option 2-1

- Option 2-2: If an actual MO duration is configured along with a termination window:

  - Following the completion (or invalidation) of an LP-WUS MO, a subsequent LP-WUS MO starts from the first available valid symbol that occurs after the end of the termination window of the preceding MO.

Fig. 5 An example of Option 2-2

Proposal 8: Consider following options for clarification on procedures to determine the start of monitoring for LP-WUS MOs with invalid symbols

- When potential starting points of LP-WUS MO are configured:

  - Option 1-1: If a configured potential starting point is associated with an invalid LP-WUS MO, the UE drops this starting point and evaluates subsequent configured potential starting points to identify the next valid MO.

  - Option 1-2: If nominal MO duration is configured and an LP-WUS MO at a given potential starting point is invalid, the UE finds the next configured potential starting point that occurs after the nominal end of this invalidated MO.

  - Option 1-3: If actual MO duration with a termination window is configured and an LP-WUS MO at a potential starting point is invalid or its duration adjusted, the UE identifies the next configured potential starting point occurring after the termination window of the preceding MO.

- In a back-to-back operation scenario:

  - Option 2-1: If nominal MO duration is configured, an LP-WUS MO following an invalid MO starts at the beginning of the next nominal MO period.

  - Option 2-2: If actual MO duration with a termination window is configured, an LP-WUS MO following a completed or invalid MO starts from the first available valid symbol occurring after the end of the termination window of the preceding MO.

Some options may require a specific method for MO duration determination.

Even if an LP-WUS MO is valid, its actual monitoring duration might be extended. This can occur, for example, the number of symbol of actual LP-WUS MO duration is maintain by extend MO duration due to TDD UL symbol overlap. When potential starting points of MO are configured, such an extension of a current MO can cause it to overlap with one or more subsequent configured potential starting points, potentially making these subsequent starting points unusable for subsequent MO occurrences. To address this, the UE may determine the start of the subsequent LP-WUS MO using one of the following mechanisms:

- Alt. 1: LP-WUS MOs are primarily intended to occur sequentially at their configured potential starting points.

  - If the actual duration of an ongoing LP-WUS MO extends such that it overlaps in time with the next configured potential starting point for a subsequent MO, it is possible that the gNB may select to transmit either one of these

Fig. 6 An example of Alt. 1

- Alt. 2: The subsequent LP-WUS MO starts at the earliest configured potential starting point of an MO that occurs entirely after the termination of the actual duration of the preceding LP-WUS MO.

Fig. 7 An example of Alt. 2

Proposal 9: Consider following alternatives for clarification on procedures to determine the start of monitoring for LP-WUS MOs with invalid symbols

- Alt. 1: LP-WUS MOs are primarily intended to occur sequentially at their configured potential starting points.

- Alt. 2: The subsequent LP-WUS MO starts at the earliest configured potential starting point of an MO that occurs entirely after the termination of the actual duration of the preceding LP-WUS MO.

  - SSB periodicity and wake-up delays

Following three alternatives were agreed in the last meeting for the different SSB periodicities:

Agreement On how to handle the UE capability report on the wake-up delay for SSB periodicities other than 20ms, consider the following alternatives for possible down-selection in RAN1#121. - Alt 1: Do not report for SSB periodicities other than 20ms - Note: LP-WUS is not supported for SSB periodicities larger than 20ms - Alt 2: For UE capability report on the wake-up delay, the UE reports one of the following 3 capabilities (the values in one of the columns): SSB periodicity (ms); Wake-up delay (ms) UE capability 1; Wake-up delay (ms) UE capability 2; Wake-up delay (ms) UE capability 3. [5]; [x]; [x]; [x]. [10]; [x]; [x]; [x]. 20; [70]; [500]; [900]. [40]; [x]; [x]; [x]. [80]; [x]; [x]; [x]. [160]; [x]; [x]; [x]. - Alt 3: It is up to RAN4

We prefer Alt. 2 for UE capability reporting on wake-up delay, as it provides a crucial and structured framework enabling LP-WUS operation across the diverse range of SSB periodicities from 5ms to 160ms. Relying on wake-up delay capabilities defined solely for a 20ms SSB periodicity, as more restrictive approaches, would be insufficient given the wide operational spectrum of SSB configurations and could compromise the ability to guarantee timely UE wake-up for LP-WUS. Alt 2, by allowing UEs to report one of three distinct capability profiles that map wake-up delays to these various SSB periodicities, offers essential deployment flexibility for networks, thereby maximizing the overall applicability and effectiveness of the LP-WUS feature.

To further enhance its practical application and ensure comprehensive coverage, we propose that for an operational SSB periodicity used by the network but not explicitly listed as a row in the UE's reported capability profile, the UE should utilize the wake-up delay value associated with the next larger SSB periodicity that is available in that reported profile. This refinement, complementing the structure of Alt 2, maximizes the overall applicability and effectiveness of the LP-WUS feature.

Proposal 10: Support Alt. 2 for UE capability reporting on wake-up delay for different SSB periodicities

- For current SSB periodicity not in the UE's reported capability, the UE shall use the wake-up delay value corresponding to the next larger SSB periodicity available

  - Handling different wake-up times for UE subgrouping

When multiple POs are associated with a single LO through different offset values, grouping UEs based on their wake-up delay capabilities can provide operational benefits. Particularly, UEs that can utilize the same offset value due to similar wake-up delay capabilities can be efficiently managed through assigning into same group.

For efficient resource utilization and simplified network operation, UEs with similar wake-up delay capabilities can be grouped together to share the same offset value. This approach enables the network to manage wake-up signaling more efficiently by aligning subgroup operations with offset configurations. This approach has the advantage that it can be operated in a manner of Core Network (CN)-based UE grouping, without complicate RRC configuration per-UE basis.

Proposal 11: UEs with different wake-up delay should be assigned to different UE subgroups

The UE subgroups formed based on wake-up delay reporting can operate in a manner similar to CN-based UE subgroups. For example, UE having certain wake-up delay can be assigned to CN-based UE subgroup. These subgroups occupy a portion of the total number of UE subgroups, and the remaining UE subgroups will be used for grouping other UEs based on the number of remaining subgroups.

For example, if the total number of UE subgroups is configured as 16, and the number of subgroups allocated for UEs with shorter wake-up delays is set to 4, the remaining 12 subgroups can be used for grouping UEs based on their UE identification.

Proposal 12: UEs having shorter wake-up delay can be assigned to a specific subgroup(s) separately

- The number of those subgroups can be separately configured

- Wake-up delays below a certain threshold are considered as shorter wake-up delay

  - EPRE Ratio Configuration for LP-WUS and LP-SS

In the last meeting, it was agreed configuring the EPRE ratio between LP-WUS/LP-SS and and SS/PBCH block power, which is the power of SSS REs:

Agreement The EPRE ratio between LP-WUS/LP-SS and SSB can be configured by the gNB. - FFS whether there is a common configuration or separate configuration for LP-WUS and LP-SS - Above is not applicable for the case when all the subcarriers for LP-WUS/LP-SS are transmitted with zero power (from baseband perspective) - Above does not mandate any UE implementation for different receiver types - FFS: whether/how to support for the case when there are different number of OOK ON symbols in different OFDM symbols (if supported)

One thing need to be clarified is whether to introduce common parameter or separated parameter between LP-WUS and LP-SS. We propose to allow the network to independently configure the EPRE ratio between LP-WUS RE and LP-SS. This provides more flexibility in optimizing the power balance between the LP-WUS and LP-SS, potentially improving detection reliability and reducing UE power consumption.

Furthermore, the currently endorsed CR (3GPP TS 38.211, R1-2503457) already specifies separate scaling factors, βWUS​ and βLPSS​, for WUS and LP-SS respectively within the physical layer parameters.

7.4.4.2 Mapping to physical resources The UE shall assume the block of complex-valued symbols rWUS(0),…,rWUS(MbitMZC-1) is scaled by a factor βWUS and mapped to resource elements k,lp,μ used for WUS transmission in increasing order of first k and then l. […] 7.4.5.2 Mapping to physical resources The UE shall assume the block of complex-valued symbols rLPSS(0),…,rLPSS(NOOKMZC-1) is scaled by a factor βLPSS and mapped to resource elements k,lp,μ used for LPSS transmission in increasing order of first k and then , then l.

The fact that separate power-related scaling factors are already defined for WUS and LP-SS supports the argument that configuring separate EPRE ratios for LP-WUS and LP-SS is reasonable and natural extension. This would enable more finer power control to the specific characteristics and requirements of each channel, thereby contributing to optimized system performance.

Proposal 13: Introduce separated parameters of EPRE ratio to SSS between LP-WUS and LP-SS

- For both of parameters, the value range is same as existing parameters: ENUMERATED{db-3, db0, db3, db6}

- For the description of EPRE ratio of LP-WUS RE,

  - Power offset (dB) of LP-WUS RE to SSS RE.

- For the description of EPRE ratio of LP-SS RE,

  - Power offset (dB) of LP-SS RE to SSS RE.

Conclusion

In this contribution, we shared our views on the LP-WUS operation in IDLE/INACTIVE modes. Followings are proposed for LP-WUS operation in IDLE/INACTIVE modes:

Proposal 1: Support Alt. 2 for TDD handling

Proposal 2: For further flexibility, Alt. 1A can be considered in addition to Alt. 2.

- For Alt. 1A, re-use the structure of RateMatchPattern or invalidSymbolPattern

Proposal 3: Support Alt 1 or Alt. 2B for starting position determination of LP-WUS MO

Proposal 4: Deprioritize Alt. D for LP-WUS MO duration determination

Proposal 5: Consider following alternatives for LP-WUS MO duration determination:

- Alt. A

- Alt. B with a time window as a termination condition.

  - The time window can be configured per MO or per LO

Proposal 6: To avoid the overlapping between adjacent two LOs, introduce RRC parameter to configure the duration of a LO.

Proposal 7: For LP-WUS operation in TDD configurations,

- If multiple LMO (for repetitions) are configured per beam, skip/drop the LP-WUS monitoring opportunity when it overlaps with UL symbols.

- Otherwise, postpone the LP-WUS monitoring opportunity for N*K LP-WUS MO when it overlaps with UL symbols.

Proposal 8: Consider following options for clarification on procedures to determine the start of monitoring for LP-WUS MOs with invalid symbols

- When potential starting points of LP-WUS MO are configured:

  - Option 1-1: If a configured potential starting point is associated with an invalid LP-WUS MO, the UE drops this starting point and evaluates subsequent configured potential starting points to identify the next valid MO.

  - Option 1-2: If nominal MO duration is configured and an LP-WUS MO at a given potential starting point is invalid, the UE finds the next configured potential starting point that occurs after the nominal end of this invalidated MO.

  - Option 1-3: If actual MO duration with a termination window is configured and an LP-WUS MO at a potential starting point is invalid or its duration adjusted, the UE identifies the next configured potential starting point occurring after the termination window of the preceding MO.

- In a back-to-back operation scenario:

  - Option 2-1: If nominal MO duration is configured, an LP-WUS MO following an invalid MO starts at the beginning of the next nominal MO period.

  - Option 2-2: If actual MO duration with a termination window is configured, an LP-WUS MO following a completed or invalid MO starts from the first available valid symbol occurring after the end of the termination window of the preceding MO.

Proposal 9: Consider following alternatives for clarification on procedures to determine the start of monitoring for LP-WUS MOs with invalid symbols

- Alt. 1: LP-WUS MOs are primarily intended to occur sequentially at their configured potential starting points.

Proposal 10: Support Alt. 2 for UE capability reporting on wake-up delay for different SSB periodicities

- For current SSB periodicity not in the UE's reported capability, the UE shall use the wake-up delay value corresponding to the next larger SSB periodicity available

Proposal 11: UEs with different wake-up delay should be assigned to different UE subgroups

Proposal 12: UEs having shorter wake-up delay can be assigned to a specific subgroup(s) separately

- The number of those subgroups can be separately configured

- Wake-up delays below a certain threshold are considered as shorter wake-up delay

Proposal 13: Introduce separated parameters of EPRE ratio to SSS between LP-WUS and LP-SS

- For both of parameters, the value range is same as existing parameters: ENUMERATED{db-3, db0, db3, db6}

- For the description of EPRE ratio of LP-WUS RE,

  - Power offset (dB) of LP-WUS RE to SSS RE.

- For the description of EPRE ratio of LP-SS RE,

  - Power offset (dB) of LP-SS RE to SSS RE.

References

[1] Chairman’s notes, RAN1#120

[2] R1-2410486, LP-WUS and LP-SS Design, Qualcomm, RAN1#119

[3] R1-2409689, Discussion on LP-WUS and LP-SS design, vivo, RAN1#119