3GPP TSG RAN WG1 #121		R1-2504258

St Julian, Malta, May 19th – 23rd, 2025

Agenda Item: 9.1.1

Source: MediaTek Inc.

Title:	Discussion on specification support for AIML-based beam management

Document for: Discussion & Decision

# Introduction

During the RAN Plenary RP#94 meeting, Artificial Intelligence (AI)/Machine Learning (ML)-based beam management has been identified as one of the three use cases for investigation and evaluation in a new Study Item (SI) to study on AI/ML. In RAN1 109e the SI on AI/ML for NR air interface has been initiated. The SI includes “Evaluation on AI/ML for beam management” under the agenda 9.3.2.1 and “Other aspects on AI/ML for beam management” under the agenda 9.3.2.2. In the study, two basic use-cases for AI/ML-based beam management has been evaluated, that is “AI/ML for Beam Management in Spatial Domain” and “AI/ML for Beam Management in Temporal Domain”, which are defined as:

•	Spatial-domain beam prediction for Set A of beams based on measurement results of Set B of beams.

•	Temporal DL beam prediction for Set A of beams based on the historic measurement results of Set B of beams.

The study results are later noted in the technical report TR 38.843[1]. Consequently, a Rel-19 work item has been proposed for specification support for AI/ML-based beam management during the RAN Plenary RP#102. The scope of this Rel-19 work item for supporting AI/ML beam management can be identified as follows[2] :

Provide specification support for the following aspects: - … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”) - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”) - Specify necessary signalling/mechanism(s) to facilitate LCM operations specific to the Beam Management use cases, if any - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE NOTE: Strive for common framework design to support both BM-Case1 and BM-Case2 - …

This document will mainly focus on the following topics:

- UE report content

- Report timeline and CSI reference report

- CSI processing units

- Performance monitoring configurations and metrics

- Configuration of SetA/B

- Associated ID discussions

In this contribution we will discuss these potential specification supports.

# Discussion

## Model inference

### UE-side model

#### Reporting content

For UE-sided model, for the content in the report of inference results, currently we support two options: Option1: Beam ID and Option2: Beam ID + predicted RSRP. There is an remaining option (Option3) pending discussion. In Option3, UE will report beam ID on predicted Top K beam(s) and the corresponding probability related information. The probability is the probability of the beam to be the Top 1 beam, which is derived from the model output. The issue with this option is that it might be hard to find a way to test the probability reported by UE. For example, assuming Set A has 3 beams, Model A outputs probability [0.8,0.1,0.1], Model B outputs probability [0.4,0.3,0.3]. It does not necessarily mean Model B has less confidence on its prediction. Its output probability highly depends on the training dataset, and the dataset used by each company can be totally different. Therefore, it is not fair to conclude that Model B fails the test since it still gives the correct beam prediction.

One benefit mentioned so far for this option is that it can be used by NW to monitor the temporal domain beam probability change on the same model. For example, the reported best beam's probability history is: 0.8, 0.8, 0.8, 0.8, 0.4, 0.8, ... . For the report with best probability suddenly dropped to 0.4, NW may choose not to believe that report. If this situation happens frequently (i.e., the best probability drops frequently), we believe it will and should be captured by the performance monitoring process and NW should deactivate this model. If this situation rarely happens, then it should not impact on the overall performance much, also we still have BFR to take care of this. For both cases, we already have methods to solve this issue. Therefore, we prefer not to introduce this new option unless we are clear how to use and test it.

Proposal 1: For UE-sided model, for the content in the report of inference results, current agreed options are sufficient.

## Report timeline and CSI reference resources

### DCI triggered PUSCH report

For the reports that is triggered by DCI (including AP and DCI triggered SP), currently there are two restrictions that defines the report timeline: the Z and Z’ delay requirement. As illustrated in Figure 1, the Z requirement defines the required duration between the last symbol of the triggering DCI and the first symbol of reporting PUSCH. The Z’ requirement defines the duration between the last symbol of the CSI-RS resource configured for channel measurement and the first symbol of reporting PUSCH. The value of Z and Z’ is defined in Table 5.4-2 of TS 38.214 [3], with Z1/Z1’ defined for beam reporting with SINR and Z3/Z3’ defined for beam reporting with L1-RSRP. This timeline restriction applies to each AP report, while for DCI triggered SP, this timeline only applies to the first report of the triggered SP.

However, the current Z/Z' delay values are designed without the consideration of AI/ML model operation. Before receiving the DCI triggering signal, UE has no prior knowledge on when it will receive the activation signal. UE has no way to prepare its model for inference beforehand. Therefore, all the inference preparation such as model loading and variable activation need to happen after UE receives the DCI. Combining with the current Z/Z' delay requirement, it is almost impossible for UE to complete model inference and prepare the report within the timeline. Also, different UEs may have different AI/ML model implementations and model complexities that requires different Z/Z' timelines. Therefore, we propose that for the report timeline of the DCI triggered AI BM reports, the value of Z and Z’ for the Z/Z' delay requirement should be determined by UE capability. One simple way to achieve this is to introduce a new UE capability on model inference timing (e.g., model_inference_timing), which is added on top to the current Z/Z’ timeline.

Figure 1: the Z/Z’ timeline restriction for DCI triggered PUSCH report.

Proposal 2: For BM Case1, when the report is a DCI triggered PUSCH reports, the Z/Z' report timeline requirement should be determined by UE capability.

Proposal 3: For BM Case2, when the report is a DCI triggered PUSCH reports, the Z/Z' report timeline requirement should be determined by UE capability.

Proposal 4: For each BM Cases, UE reports its corresponding capability on model inference timing. The timing capability is added to the current value of Z and Z’ in the report timeline requirement.

### Periodic and Semi-persistent reports on PUCCH

In the current spec, for the periodic reports and semi-persistent reports on PUCCH, the report timeline is defined by the CSI reference resource. That is only the channel measurements on the P/SP CSI-RS/SSB resources that are configured no later than the CSI reference resource will be included in the report. The CSI reference resource is defined mostly based on a variable named: nCSI_ref.  In AI BM cases, since Set B consists of multiple CSI-RS resources, according to Clause 5.2.2.5 of TS 38.214 [3], nCSI_ref is the smallest value greater than or equal to 5*2μDL, such that it corresponds to a valid downlink slot.  However, the current value of nCSI_ref does not consider any AI/ML model operation. Meanwhile, AI/ML inference can only start after all the model input (i.e., Set B measurement) is ready at UE. Therefore, the duration left for UE to perform model inference maybe insufficient. Therefore, we propose that the same UE capability on model inference timing can be introduced to the CSI reference resource timeline.

Proposal 5: the CSI reference resource definition should consider model inference delay, which is expected to be higher than the legacy beam management operations, especially for BM Case2.

Proposal 6: For each BM Cases, UE reports its corresponding capability on model inference timing. The timing capability is added to the n_CSI_ref variable in the definition of CSI reference resource.

## CSI processing units

### CSI processing units for UE-sided model

For CSI processing unit, three related agreements have been made so far:

Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Agreement For UE-side model, existing CPU mechanism is used as a starting point for AI/ML-based CSI processing. - FFS whether the overall CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting, and among AI/ML features/functionalities. - FFS whether it is fully applicable for BM-Case 1 and/or BM-Case 2 Agreement - For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, considering the following options for potential down selection: - Option 1: only dedicated AI/ML PU is occupied, OAPU=N1 is reported by UE. - And OCPU=0 - Option 2: only legacy CPU is occupied, OCPU=M it is reported by UE. - Option 3: both dedicated AI/ML PU and legacy CPU are occupied, OAPU=N2 is reported by UE. - And OCPU=1 Note: The supported option by UE is reported by UE capability, if multiple options are supported. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

For the three options for defining the processing unit of a CSI report, we prefer option1. Note that the current legacy CPU mechanism cannot accurately capture the complexity difference between different CSI reports. For example, BM will report 1 port Wide Band L1-RSRP only, and a CSI-ACQ report may require UE to measure and calculate for up to 32 ports of CRI/RI/PMI/CQI for either Wideband or Sub-Band. It is very hard to accommodate the corresponding complexity difference into the current granularity of O_CPU. Moreover, the UE implementation on the legacy BM/CSI algorithms and AI/ML-based algorithms are totally different. They may even use different hardware resources for computation. Hence, we don’t think it is a good idea to add the AI/ML report complexity on top of the current CPU mechanism. Option1 should be supported so that the AI/ML feature can have its own CPU mechanism for defining the complexity.

Following the same reason above, we believe it is more useful for UE to report dedicated processing capability for each AI/ML use cases. With this approach, we don’t need to standardize on how to capture the complexity difference among different use cases and NW can have a clearer understanding on how many AI/ML BM and AI/ML CSI reports that an UE can support simultaneously. Moreover, it is possible that UE has dedicated AI engines for BM and CSI reports, where the resources are not shared with each other. In this situation, it may confuse NW on exactly how many AI/ML BM and AI/ML CSI reports that an UE can support simultaneously. Eventually, UE can only report the worst-case scenario as the total number of APU, i.e., the smaller number among how many BM reports it can support and how many CSI report it can support. Therefore, we support that for each AI/ML use cases, the AI/ML PU will not be shared with other use cases.

Proposal 7: For the processing unit of a CSI report for AI BM, support Option1: only dedicated AI/ML PU is occupied, O_APU=N1 is reported by UE.

Proposal 8: For the processing unit of a CSI report for AI BM, do not support Option2: only legacy CPU is occupied, O_CPU=M it is reported by UE.

Proposal 9: The total number of dedicated AI/ML PU for AI/ML-based beam management will not be shared with other use cases.

## Performance monitoring (Metrics/events/procedures)

### UE-side model

#### Monitoring with a subset of Set A

We would like to emphasize that for Type1 Option2, designing a mechanism for monitoring with a subset of Set A, instead of full Set A, can significantly reduce the RS and measurement overhead and enhance the robustness of performance monitoring. One of the reasons is that this subset of Set A can be configured with higher frequency than the full set of Set A. Therefore, the performance monitoring can be done more frequently.

Regarding how to do the mapping between Set A and monitoring resources, using the same NZP-CSI-RS-ResourceId/SSB-Index as Set A RS resources in the monitoring resource Set may be the most straightforward method.  However, since they share the same resource id, the time domain behavior must be the same between the monitoring resource and Set A resource. It may be possible that we ignore the time domain behavior of Set A because it is a virtual resource set. But, if there are multiple monitoring report configurations, each with its own resource set, the same Set A beam that is shared between these monitoring resource set must be the same resource id and thus share the same time and frequency domain behavior. Therefore, to maintain the flexibility that for the same Set A beam, different monitoring resource set can configure different time domain behavior, we support NW to additionally configure a bitmap of a set of CRI or SSBRI of a resource in Set A CRI or SSBRI of a resource in Set A in the monitoring report configuration, to indicate the mapping of RS resources in Set A and monitoring set.

Proposal 10:  When the size of a set for monitoring is smaller than the size of Set A. The mapping between the resources for Set A and resources for monitoring can be configured in RRC.

Proposal 11:  For option1 type2, to map RS resources in Set A and monitoring set, NW configures an additional bitmap or a set of ID (CRI or SSBRI of a resource in Set A) in the monitoring report configuration.

Performance metric(s)

In RAN1#120b, it is agreed that the performance metric of Top 1 or Top K beam prediction accuracy is defined as comparing the Top M beam(s) of the resource set(s) for monitoring and the Top-K predicted beam(s) of Set A. This definition may underestimate the model performance and relies heavily on how the NW choose the monitoring set. Assuming model can 100% predict the best beams, if the ground-truth Top-K beams are not even within the monitoring set, then the Top M from the monitoring set will never overlap with Top-K predicted beams. As below Figure 2 (a) shows, the ground-truth best beam in Set A is beam 6 and beam 7, even though the model can predict it correctly, BAI is always 0 since the monitoring set does not cover beam 6 and beam 7. To make a meaningful comparison between the Top-M monitored beams and Top-K predicted beams, the Top-K predicted beams should be selected from the monitoring set, as Figure 2 (b) shows.

Figure 2: (a) the issue for the current definition of  Np , where the model performance may be underestimated. (b) the proposed methods to address the issue, where Top-K predicted beam for monitoring comparison purpose is selected only among the monitoring set

Proposal 12: For option1 type2, the performance metric of Top 1 or Top K beam prediction accuracy, Top-K predicted beam(s) should also be selected from the resource set(s) for monitoring, by filtering out the beams not included in the monitoring resource sets. K is the number of predicted beam(s) in the corresponding inference report.

## Consistency for additional condition for UE-sided model

Currently, we have the following agreement on how to ensure the consistency of NW-side additional condition between the training and inference phases:

Agreement For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list. Agreement For UE-sided model, for configuring the resource for data collection purpose, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - One CSI-ResourceConfigId is configured for Set A. - One CSI-ResourceConfigId is configured for Set B. - Note: UE performs measurement on all resources - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: whether/how to support 'aperiodic' CSI RS Note: This is not related to whether/how to support delivery/transmission of the collected data for training for UE-sided model. Agreement For UE-sided model, in CSI-ReportConfig for inference - One or two associated IDs can be configured in CSI-ReportConfig - When Set B is equal or a subset of set A (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), one associated ID is configured, - Otherwise, one associated ID is configured for Set A and another one associated ID is configured for Set B - FFS: At least BM-Case 1, the applicability for 'aperiodic' CSI RS

### How to use associated ID to ensure consistency?

In our view, the associated ID is used to represent the properties that will impact the performance of AI/ML-based beam management, which are not observable/configurable through the air interface. Therefore, UE should not expect that there is an one-to-one mapping between the associated ID and inference configurations (e.g., Set A and Set B-related information). Note that, the training and inference consistency can be secured even under the condition that the same associated ID is used in different inference configurations.  Figure 3 illustrates an example that during training and data collection phase, when NW keeps the same beam property, NW signals the same associated ID for different data collection configurations, each configuration contains the Set A and Set B-related information. Assuming at UE side, UE trains different models for different combinations of data collection configurations and associated ID. Then, during the inference phase, UE can tell from the configuration of associated ID and inference report configuration, which contains the Set A and Set B-related information, to identify the corresponding model. Note that in this example, only one associated ID is used across multiple Set A and Set B configurations and UE tells which model to use by referring to the inference report configuration.

Figure 3: Examples on how associated ID can be used to ensure training and inference consistency with different report configurations.

Proposal 13: RAN1 does not consider a one-to-one mapping between the associated ID and inference configurations.

### Global associated ID or per cell associated ID

There are multiple issues when considering the applicable level of associated IDs.

- First, if multiple NW venders can use the same set of associated IDs, it may confuse UE when it enters a new cell. For example, if associated ID is cell-specific, UE cannot assume cell 1’s associated ID 1 has the same configuration as cell2’s associated ID 2. Even though UE can distinguish the NW configurations by the combination of cell ID and associated ID, UE cannot guarantee that the cell ID will not change over time.

- Second, even within one cell, there may be multiple sets of NW-side beam related configurations. With the time goes, there might be new NW-side beam related configurations, or a new environment change which makes a trained AI/ML model not feasible to use anymore. Eventually, the number of global associated IDs will keep growing.

Therefore, we think there can be two parts defined for the associated ID. The first part is a global associated ID defined at the PLMN level. Each NW vendor will be assigned an exclusive set of the global associated IDs by the PLMN operator. When UE receives the same global associated IDs from different cells, UE can assume the NW-side beam-related properties is the same since only one NW-vendor can use this associated ID. On the other hand, the second part of the associated ID is cell or gNB-determined. The second part can be used in two different ways, even though the first part of the associated ID is the same, NW can use the second part to indicate the minor difference of NW-side additional condition between cells. For example, if an UE uses one ID to collect a dataset from this cell and trains its model. After a long time, when the UE re-enters this cell, NW may some minor environment changes. Then, the cell/gNB-specifc second part of the associated ID can be used to indicate UE such differences and to trigger potential model retrain/fine-tuning. On the other hand, the second part can also be used by NW to indicate the same NW-side additional condition across PLMNs. In this way, the data collection effort can be reduced since the data collected under the same NW vendor, but at different PLMNs, can be shared together.

Proposal 14: RAN1 makes a working assumption that each NW vendor has its own exclusive set of associated IDs, which can be managed by PLMN.

Proposal 15: For associated ID, considering

- First part is a PLMN-specific ID, different NW vendors may not use the same ID under one PLMN

- Second part is a cell/gNB-determined ID

## Configuration of RS for Set A and Set B

### UE-side model

During, we have the following agreement on how to configure RS for Set A and Set B within the CSI framework for UE-sided model:

Agreement For UE-sided model at least for BM Case-1, CSI-ReportConfig is used for the configuration of inference results reporting - FFS on the details in the CSI-ReportConfig, at least considering: - Alt 1: one CSI-ResourceConfigId is configured for Set B - FFS: how UE can determine the information about set A - Alt 2: one CSI-ResourceConfigId is configured for both Set A and Set B - FFS: How to configure resource set(s) for Set A and Set B in CSI-ResourceConfig - Alt 3: two CSI-ResourceConfigId s are configured for Set A and Set B separately - Alt 4: one CSI-ResourceConfigId is configured for Set B, Set A is configured using separate resource set(s) other than that represented by CSI-ResourceConfigId - FFS: how to configure/indicate separate resource set(s) for Set A - Note: separate CSI-ReportConfig for Set A and Set B are not precluded. - Note: Not perform measurement for Set A and only perform measurement for Set B subject to the CSI-ReportConfig - FFS on the association between Set A and Set B with or without additional IE Other necessary configuration are not precluded. Agreement For UE-sided model at least for BM Case-1, for inference results report - Two resource sets can be configured for Set A and Set B separately in the CSI report configuration for the report - FFS whether support only resource set for Set B is configured - UE performs measurement on the resource set for Set B for inference, and UE is not expected to measure resource set for Set A for inference, - The beam information in the inference report refers to the resource set for Set A Agreement For both BM-Case 1 and BM-Case 2, for UE-sided model for inference, when Set A and Set B are configured within CSI report configuration, Two CSI-ResourceConfigId s are configured for Set A and Set B separately

To our understanding, Set A should be always configured with physical resources, but may be configured with longer periodicity, for data collection and model monitoring purposes. Furthermore, considering that there may have legacy UEs in the NW that require measuring Set A for its beam report. Therefore, before the discussion on which alternatives are better, we suggest RAN1 makes a working assumption that Set A is always configured with physical resources.

Proposal 16: Set A should always be configured with physical resources (i.e., have corresponding resource ID/resource set ID, ResourceConfig ID), for the following reasons:

- Set A is required for monitoring/data collection, and can be configured with longer periodicity

- Set A is required for legacy UE

Consider that Set A must have corresponding csi-ResourceSetId(s), we think Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig and Set B can be configured by a new IE which has csi-ResourceConfigId as its value. The reason is that by following this configuration, the same design can be used for data collection too. For data collection, NW can configure a legacy beam report (i.e., reportQuantity = “cri-RSRP”) to UE, with resourcesForChannelMeasurement linked to Set A, along with a new IE configured for Set B. Once UE is triggered with this report configuration, UE measures Set A and reports the best beams. Then potentially measures Set B (if Set B is not a subset of Set A), together with Set A measurements to formulate a data sample.

Proposal 17: For the design of csi-ReportConfig for UE-sided model, Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig. Set B can be configured by a new IE which has csi-ResourceConfigId as its value.

### Max number of reported beam & Max number of measured RS

In RAN1#120 and [4], a proposal has been brought out to extend the value of maxNrofNZP-CSI-RS-ResourcesPerSet. Currently, the maximum number of NZP-CSI-RS-Resources per resource set is 64. We understand the motivation of extending it to higher value (e.g., 256) to accommodate more finer beams in the future FR2 environment. However, such modification needs to be considered more thoroughly. For example, whether this extension is for applicable only for the AI/ML features? The overhead of measuring 256 beams for a normal FR2 BM report may be prohibitively large. Therefore, the support of 256 beams in one resource set might only be considered under the case when AI/ML feature is enabled, which cannot be implemented simply as changing the value of maxNrofNZP-CSI-RS-ResourcesPerSet. Furthermore, the current spec already support configuring multiple resource sets in one CSI-ResourceConfig. To configure 256 beams for Set A for measurement, NW can configure 4 resource sets, each set contains 64 beams, in the CSI-ResourceConfig for Set A.

Proposal 18: For both NW-side model and UE-side model, for monitoring and data collection (when applicable), to support number of Set A beams > 64, up to 4 resource sets can be configured for Set A in one CSI report configuration.

## Beam indication

For beam indication after model inference, currently there is one agreement made in RAN1#116:

Agreement - For NW-sided model and for UE-sided model, beam indication is based on unified TCI state framework - FFS on whether/how potential enhancement is needed

During the offline discussions of RAN1#116 to RAN1#117, there are some discussions on the possible enhancements on unified TCI state framework for beam indication for AI/ML-based beam management. One of the proposals is the following:

- For BM-Case2 (both UE-sided and NW-sided model), extend the Rel-17 TCI state activation/indication signalling methods to activate/indicate N TCI states which are corresponding to N future time instances

  - FFS: maximum number for N

  - FFS: Time periods that each indicated TCI state is applicable.

In our view, this proposal is not essential and can be deprioritized, for the following reason:

(a) the overhead saving would only occur if model output is Top-1. If the model output is Top-K>1 beams for each future time instance, a P2 beam sweeping would be needed at each future time instance.

(b) In the TCI mechanism, only activated TCI states can be indicated by DCI. But currently only 8 active TCI states are supported. Therefore, the number of the distinct future beams cannot be more than 8. (e.g., NW cannot pre-indicate 10 future time slots, where all the time slots have different best beams, that will require > 8 active TCI states)

(c) RAN1 may need to specify another mechanism to handle the case when NW want to change a pre-indicated beam

(d) it may potentially limit the capability of NW for choosing a different Tx beam based on newly reported beam information from other UEs during the future N time instances. The newly reported beam information by other UEs at the future N time instances may make NW changing its mind and indicates a new beam (TCI state) other than using the pre-indicated predicted beams. Below is a simple example with a hypothesis on NW will tend to pick the beam which can serve more UEs simultaneously. As shown in Figure 4, consider the following case where an AI/ML UE1 is static and operates a BM Case2 model and another UE2 is moving and operates a BM Case1 model. If NW indicates future N TCI states at time 0 to UE1 based on UE1’s prediction, NW will indicate beam 1 for all the future N time instances. However, since UE2 is moving, at time T (one of the future time instances), UE2 will report that beam 1 is no longer within its Top-K beam while beam 2 is. At this time, the beam that can simultaneously serve both UE1 and UE2 becomes beam2. In this case, NW still needs one DCI to activate beam2 at time T, then the additional DCI overhead for activating N future TCI states for UE 1 at time 0 is wasted.

Therefore, the overall benefit needs to be further justified for this enhancement. Furthermore, this may require RAN4 involvement for the new time limitation for known/unknown TCI states. The corresponding standardization effort is huge while the potential benefit so far is not clear.

Proposal 19: For AI/ML-based BM, at this stage, there is no further enhancement needed for TCI and beam indication based on unified TCI state framework.

Figure 4: An example of NW changes the serving beam based on the new reported conditions from the legacy UE.

## Discussion on UE features:

### Beam report timing for UE-sided model

In the current spec, there is a UE capability associated with beam-based operation, that is beamReportTiming, which may be impacted by the new behavior of AI/ML-based beam management. According to TS 38.306 [6], the definition of beamReportTiming is defined as the following,

beamReportTiming, beamReportTiming-v1710 Indicates the number of OFDM symbols between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report. The UE provides the capability for the band number for which the report is provided (where the measurement is performed). The UE includes this field for each supported sub-carrier spacing.

Also, the corresponding available values are defined in TS38.331[5] as follows,

The maximum available values of beamReportTiming in terms of microseconds for each subcarrier spacing are approximately 533.4, 933.2, 466.8, 466.5 us (micro seconds) for SCS-15,30,60,120 kHz, respectively. However, for UE-sided AI/ML-based beam management, the complexity may be different than the legacy beam management. Therefore, we propose to add a new UE capability for beamReportTiming specific for Rel-19 AI BM feature.

Proposal 20:  For Rel-19 AI BM features, introduce a specific beamReportTiming UE capability to distinguish between the legacy beamReportTiming UE capability.

# Conclusion

In summary, based on the above discussion we have the following observations and proposals:

Proposal 1: For UE-sided model, for the content in the report of inference results, current agreed options are sufficient.

Proposal 2: For BM Case1, when the report is a DCI triggered PUSCH reports, the Z/Z' report timeline requirement should be determined by UE capability.

Proposal 3: For BM Case2, when the report is a DCI triggered PUSCH reports, the Z/Z' report timeline requirement should be determined by UE capability.

Proposal 4: For each BM Cases, UE reports its corresponding capability on model inference timing. The timing capability is added to the current value of Z and Z’ in the report timeline requirement.

Proposal 5: the CSI reference resource definition should consider model inference delay, which is expected to be higher than the legacy beam management operations, especially for BM Case2.

Proposal 6: For each BM Cases, UE reports its corresponding capability on model inference timing. The timing capability is added to the n_CSI_ref variable in the definition of CSI reference resource.

Proposal 7: For the processing unit of a CSI report for AI BM, support Option1: only dedicated AI/ML PU is occupied, O_APU=N1 is reported by UE.

Proposal 8: For the processing unit of a CSI report for AI BM, do not support Option2: only legacy CPU is occupied, O_CPU=M it is reported by UE.

Proposal 9: The total number of dedicated AI/ML PU for AI/ML-based beam management will not be shared with other use cases.

Proposal 10:  When the size of a set for monitoring is smaller than the size of Set A. The mapping between the resources for Set A and resources for monitoring can be configured in RRC.

Proposal 11:  For option1 type2, to map RS resources in Set A and monitoring set, NW configures an additional bitmap or a set of ID (CRI or SSBRI of a resource in Set A) in the monitoring report configuration.

Proposal 12: For option1 type2, the performance metric of Top 1 or Top K beam prediction accuracy, Top-K predicted beam(s) should also be selected from the resource set(s) for monitoring, by filtering out the beams not included in the monitoring resource sets. K is the number of predicted beam(s) in the corresponding inference report.

Proposal 13: RAN1 does not consider a one-to-one mapping between the associated ID and inference configurations.

Proposal 14: RAN1 makes a working assumption that each NW vendor has its own exclusive set of associated IDs, which can be managed by PLMN.

Proposal 15: For associated ID, considering

- First part is a PLMN-specific ID, different NW vendors may not use the same ID under one PLMN

- Second part is a cell/gNB-determined ID

Proposal 16: Set A should always be configured with physical resources (i.e., have corresponding resource ID/resource set ID, ResourceConfig ID), for the following reasons:

- Set A is required for monitoring/data collection, and can be configured with longer periodicity

- Set A is required for legacy UE

Proposal 17: For the design of csi-ReportConfig for UE-sided model, Set A can be configured by the existing IE resourcesForChannelMeasurement in csi-ReportConfig. Set B can be configured by a new IE which has csi-ResourceConfigId as its value.

Proposal 18: For both NW-side model and UE-side model, for monitoring and data collection (when applicable), to support number of Set A beams > 64, up to 4 resource sets can be configured for Set A in one CSI report configuration.

Proposal 19: For AI/ML-based BM, at this stage, there is no further enhancement needed for TCI and beam indication based on unified TCI state framework.

Proposal 20:  For Rel-19 AI BM features, introduce a specific beamReportTiming UE capability to distinguish between the legacy beamReportTiming UE capability.

# References

- TR 38.843 V2.0.1, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP Release 18, Dec. 2023

- RP-234039, “New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#102

- TS 38.214, “5G NR Physical layer procedures for data,” 3GPP Release 17, Feb. 2024.

- R1-2501595, “FL summary #6 for AI/ML in beam management”, Samsung, TSG RAN WG1#120

- TS 38.331, “5G NR Radio Resource Control (RRC); Protocol specification; (3GPP TS 38.331 version 18.3.0 Release 18),” 3GPP Release 18, Oct. 2024.

- TS 38.306, “5G NR User Equipment (UE) radio access capabilities,” 3GPP Release 17, Aug. 2022.