3GPP TSG RAN WG1 #121           	                                  R1-2504262

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item: 9.5.1

Source: MediaTek Inc.

Title: On-demand SSB SCell operation

Document for: Discussion and decision

- Introduction

In the WID of Rel-19 network energy savings (NES) [1], the objective of on-demand SSB SCell operation are listed as below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In RAN1 #120-bis [2], the following is agreed:

Agreement

For the case where SCell with on demand SSB transmission and cell with signalling transmission have different numerology, adopt Option 3 to determine SCS for Nslotsubframe,μ for determining T.

- Option 3: The SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication.

Agreement

For the case when the center frequency locations of always-on SSB and on-demand SSB are different, at least the following is supported for QCL between AO-SSB and OD-SSB:

- SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters.

- When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications.

Agreement

For a cell supporting on-demand SSB SCell operation, for configuring the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb-nrofBurst),

- Alt 2: The value range of od-ssb-nrofBurst is {N2 integer values}.

  - N2= [8].

  - If od-ssb-nrofBurst for an on-demand SSB is NOT configured, the on-demand SSB is deactivated via MAC CE.

  - If od-ssb-nrofBurst for an on-demand SSB is configured, the on-demand SSB is deactivated based on the configured value for od-ssb-nrofBurst [or via MAC CE].

Agreement

Upon the reception of MAC CE for deactivating on-demand SSB, UE expects that on-demand SSB is NOT transmitted from time instance B.

- Time instance B’ is T slots after the slot where UE receives a signalling from gNB to deactivate on-demand SSB. If time instance B’ falls within an on-demand SSB burst, time instance B is the ending of the slot containing the last actually transmitted SSB index within the on-demand SSB burst, otherwise, time instance B = time instance B’.

  - As agreed in previous RAN1 meeting, the SSB time domain positions of on-demand SSB burst are configured by gNB.

    - As agreed in previous RAN1 meeting, the location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB

  - (Working assumption) T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to deactivate on-demand SSB ending in slot n, and Nslotsubframe,μ is as defined in current specification.

  - (Working assumption) T=T_min

  - RAN4 to confirm the above two working assumptions

  - Nslotsubframe,μ is based on the SCS of the active UL BWP where the UE transmits ACK corresponding to the MAC-CE for on-demand SSB transmission indication

Agreement

For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell.

- SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases

  - The case where center frequency of AO-SSB and OD-SSB are different

  - Case 1

- Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst)

FFS: Additional restrictions

Agreement

For a cell supporting on-demand SSB SCell operation,

- Frequency of the on-demand SSB (i.e., od-ssb-absoluteFrequency) can be absent for Case #2.

  - If absent, the center frequency of on-demand SSB is the same as that of always-on SSB.

- od-ssb-PositionsInBurst, if provided, has the same bitmap structure as ssb-PositionsInBurst provided in ServingCellConfigCommon, that is od-ssb-PositionsInBurst is choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap.

- od-ssb-PositionsInBurst can be absent for Case #2.

  - If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

Agreement

For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE.

- Scenario #3B and Case #1

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

- Scenario #3B and Case #2

  - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Agreement

For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell)

- UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.

Agreement

For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell),

- A CSI report configuration can be associated with

  - both of AO-SSB and OD-SSB

    - Which SSB to measure is up to UE implementation as long as RAN4 requirements are met.

    - Applies at least for the case where AO-SSB and OD-SSB have the same center frequency and the same positions in the same SSB burst.

    - FFS: It is up to UE to measure both SSBs or one of them if the higher layer timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to “Configured”.

  - OD-SSB only

    - FFS: Applies at least for the case where AO-SSB and OD-SSB have the different center frequencies.

This contribution provides our views on this topic.

- Relations between always-on SSB and on-demand SSB

2.1 Whether/how to indicate time offset between always-on (AO) SSB and on-demand (OD) SSB

It is agreed in RAN1 #120 that "SFN offset" and "half frame index" is provided for the OD SSB with an FFS that:

- FFS: Whether/how to indicate time offset between always-on SSB and on-demand SSB

From our point of view, we prefer a unified solution and it seems not necessary to indicate time offset between always-on SSB and on-demand (OD) SSB.

Proposal 1: When there is always-on SSB transmitted in the cell, use "SFN offset" and "half frame index" to indicate the time domain position of on-demand SSB. No need to indicate time offset between always-on SSB and on-demand SSB.

2.2 Whether PBCH payload for the same SSB index should be the same for AO-SSB and OD-SSB when center frequency locations of always-on SSB and on-demand SSB are different

It was agreed in RAN1 #120 that:

Agreement

PBCH payload for the same SSB index (other than SFN index, half frame index) is the same for AO-SSB and OD-SSB when center frequency locations of always-on SSB and on-demand SSB are the same

- FFS: when center frequency locations of always-on SSB and on-demand SSB are different

After some check on the MIB contents, we think at least ssb-SubcarrierOffset (k_ssb, see Fig. 1) inside MIB should be different for AO-SSB and OD-SSB when center frequency locations of always-on SSB and on-demand SSB are different.

Proposal 2: At least ssb-SubcarrierOffset (k_ssb, see Fig. 1) inside MIB should be different for AO-SSB and OD-SSB when center frequency locations of always-on SSB and on-demand SSB are different.

Figure 1. An example of Kssb usage (Ref)

- Conclusion

In this contribution, we focus on the discussions of on-demand SSB SCell operation and have the following proposals:

Proposal 1: When there is always-on SSB transmitted in the cell, use "SFN offset" and "half frame index" to indicate the time domain position of on-demand SSB. No need to indicate time offset between always-on SSB and on-demand SSB.

Proposal 2: At least ssb-SubcarrierOffset (k_ssb, see Fig. 1) inside MIB would be different for AO-SSB and OD-SSB when center frequency locations of always-on SSB and on-demand SSB are different.

- Reference

RP-242354, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, RAN #105

Chairman’s Notes (Younsun’s Session), RAN1 #120-bis (Rel-19 9.5.1 On-demand SSB SCell operation)