3GPP TSG RAN WG1 #121                                                              R1- 2504311

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.1.4.1

Source:	Apple Inc.

Title:	Discussion on CSI compression and AI processing units

Document for:	Discussion/Decision

# Introduction

CSI compression is identified as a sub-use case for further study in R19 [1].

- CSI feedback enhancement [RAN1]:

  - For CSI compression (two-sided model), further study ways to:

    - Improve trade-off between performance and complexity/overhead

      - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc.

    - Alleviate/resolve issues related to inter-vendor training collaboration.

while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

In addition, for AI/ML PU related aspects that is across use cases, one aspect remains:

- Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates.

In this paper, we discuss the cross-use cases aspects on AI processing unit and remaining open issues for CSI compression is also discussed.

# Discussion on remaining aspects of CSI compression

## Inter-vendor training collaboration

In RAN1 #120bis, it was concluded that

C

Conclusion:

- RAN1 concludes that both Direction A and Direction C are feasible.

- RAN1 concludes that both sub-option 4-1 and sub-option 3a-1 are feasible.

  - This includes both 3a-1 with and without target CSI sharing.

It is also concluded that the n RAN1 #120bis that synthetic data can be used for both direction C and direction A option 3a-1.

In addition, in RAN2 reply LS R2-2503169, RAN2 concluded that:

For non-OTA approaches, i.e., ‘NW dataset/model parameters collection entity -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified the following candidate solutions (see below Table 1).  From RAN2 point of view, it is assumed they can be supported within Rel-19 existing architecture framework.

For OTA approaches, i.e., ‘gNB -> NW dataset/model parameters collection entity (if needed) -> gNB -> UE -> UE training entity (a server inside MNO or an OTT server)’, RAN2 identified following candidate solutions:

- gNB -> UE via CP, where gNB is NW-side dataset/model parameter collection entity

- CN -> UE via gNB, where CN is NW-side dataset/model parameter collection entity

- OAM -> UE via gNB, where OAM is NW-side dataset/model parameter collection entity

RAN2 further have following conclusion on challenges and potential suitable scenarios for above OTA solutions (attached in Table 2), but RAN2 has no consensus on feasibility.

RAN1 concluded that synthetic data can be used to train the reference model, and model performance can be further optimized using field data for both direction C and direction A option 3a-1.

- For Direction C, confirm that the specified model should be trained using synthetic data (answer to issue 8).

- For inter-vendor collaboration option 3a-1 of Direction A, confirm that the specified model structure should be determined using synthetic data.

Given that RAN4 is already defining specified model for testing purpose, and that multiple rounds of calibration are ongoing across datasets and models from different companies, we do not see a need for RAN1 to duplicate efforts by defining additional model structure and model parameters.

With the model structure and parameters specified in RAN4, and additional work in SA on non-OTA approaches for dataset delivery, the remaining specification aspects for inferencing and performance monitoring are largely similar between Direction C and direction A.

Proposal 1: For inter-vendor training collaboration under direction C and direction A, option 3a-1, the reference model structure and parameters reuse RAN4 specified model and parameters.

Proposal 2: Recommend RAN1 to specify 4-1 with non-OTA dataset delivery, with RAN4 specified model structure and parameters for Direction C.

## Discussion on performance monitoring

In RAN1 #116, different cases have been agreed for time-frequency-spatial domain CSI compression. In RAN1 #118, it was agreed that case 2 and case 3 are prioritized for study, as the amount of evaluation work is mainly focused on these two cases. We also observe that case 4 is a combination of case 2 and case 3, which can be enabled naturally once case 2 and case 3 are specified.

Agreement

For the evaluation of temporal domain aspects of AI/ML-based CSI compression using two-sided model in Release 19, adopt the following categorization for study:

| Case | Target CSI slot(s) | Whether the UE uses past CSI information | Whether the network uses past CSI information |
| --- | --- | --- | --- |
| 0 | Present slot | No | No |
| 1 | Present slot | Yes | No |
| 2 | Present slot | Yes | Yes |
| 3 | Future slot(s) | Yes | No |
| 4 | Future slot(s) | Yes | Yes |
| 5 | Present slot | No | Yes |

### Case 3

For case 3, separate CSI prediction and CSI compression can be used as shown in Fig. 1. We draw an AI based CSI prediction in the figure as an example but filter based CSI prediction is also possible.

Fig. 1. Case 3 of time-frequency-spatial domain CSI compression

For performance monitoring of case 3, different options were discussed and captured in RAN1 119.

Agreement

For temporal domain aspects Case 3, study LCM aspects and specification impacts,

consider the following options for training data collection

- Option 1: The target CSI for training is derived based on the predicted CSI of the future slot(s).

- Option 2: The target CSI for training is derived based on the measured CSI of the future slot(s).

- Note: During inference, the input to the CSI generation part is derived based on the predicted CSI.

consider following options for the monitoring labels

- Option 1: The monitoring label is derived based on the predicted CSI of the future slot(s).

  - CSI prediction output is used as input to CSI generation part.

  - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

- Option 2: The monitoring label is derived based on the measured CSI of the future slot(s)

  - Option 2a: CSI prediction output is used as input to CSI generation part.

    - Note: This corresponds to end-to-end monitoring of CSI prediction and compression.

  - Option 2b: Measured CSI of the future slot(s) is used as input to CSI generation part for monitoring purpose.

    - Note: This corresponds to monitoring of CSI compression only. CSI prediction may be monitored separately.

Study how the functionality/model control (activation, deactivation, switching, and fallback) for CSI prediction and CSI compression interacts.

Option 1 separate CSI compression and CSI prediction. When filter-based CSI prediction is used in implementation, and the target CSI is the predicted CSI at the filter output, only AI based CSI compression is monitored. However, if the CSI prediction filter performance is not accurate, the E2E SGCS can be much lower.

Option 2a performs E2E monitoring of CSI prediction and CSI compression. When E2E performance degraded, if AI based CSI prediction is used, separate CSI prediction performance monitoring can be triggered to identify the AI based CSI prediction model using type 1 or type 3 performance monitoring.

Option 2b performs performance monitoring of CSI compression model, similar to option 1. However, the measured CSI is used as input, instead of the predicted CSI. This requires separate compression/de-compression on top of the inference operation.

Observation 1: When traditional filter-based CSI prediction is used, no performance monitoring is performed in MIMO. When AI based CSI prediction is used, separate CSI prediction performance monitoring is specified.

Proposal 3: For case 3 of time-frequency-spatial domain CSI compression, option 2a ensures E2E performance. The intermediate KPI or eventual KPI includes both compression and prediction performance.

## CSI report configuration for case 0, 2, 3

Flexible CSI report configuration and report format should be enabled to support different use cases. One simple method is to use different model ID for different cases and include the model ID in csiReportConfig.

For RI, since case 2 requires layer common/state specific model, or layer specific/state specific model, the number of layers over a longer period seems to be relatively stable. Otherwise, the state information of higher layer can be missing when RI is increased. This can be an extra RI restriction on top of current design that NW can configure to UE.

Proposal 4: For case 2, time-frequency-spatial domain CSI compression, for RI, consider longer term RI update across different CSI reports.

Proposal 5: For time-frequency-spatial domain CSI compression, flexible CSI report configuration to support different cases should be studied.

# Discussion on AI processing unit across use cases

Since the use case specific CPU/APU related discussion is in each use case, in this section, we focus on the issue that is cross different use cases.

## AI processing timeline

The AI model inference timeline depends on factors such as memory structure and size, model parameter size and inference delay. Due to the large storage requirement of different model parameters, the model can be stored in various locations and different times are required to move the model from the server for inference as shown in Fig 2.

- Most trained AI models are stored in the cloud. For each use case, different models can be trained based on factors like association ID, scenarios, configurations, and UE side additional conditions such as UE speed.

- Model(s) are downloaded to UE devices based on different use cases. The models are first stored in the non-volatile memory (NVM). The download process from the cloud to the device may requires up to several hundreds of ms or longer.

- When the model is activated, the UE moves the model parameters from NVM to off-chip memory such as DRAM. The time required for this operation depends on the model size and memory’s read/write speed and may require up to several tens of ms or longer.

- Model inferencing operation is performed with on-chip memory, and it typically much faster to transfer model from off-chip memory to on-chip memory, and this process usually takes on the order of ms.

Fig 2. Different stage of AI model storage and corresponding transfer time requirement

When applicability report procedure, the AI model is ready for inference, which indicates that the AI model is at least in the off-chip memory. Multiple activated AI model can be stored in the off-chip memory, and during active inferencing, the model needs to be moved to on-chip memory.

Due to typical small size of on chip memory, different activated AI models need to be moved to inference state when triggered and move out when another CSI report is requested. The time required to transfer from off chip memory to on chip memory is again tightly coupled with implementation. Therefore, when AI model is in the activated state, whether additional delay is required to switch to inference state can be UE capability.

When no additional delay is required, the processing timeline becomes simpler.  Similar to legacy behaviour, different Z1, Z2 and Z3,  Z1’, Z2’ and Z3’ can be defined for different AI based processing.

When additional delay is required, the processing timeline can depend on the AI function switch between the activated AI functions. As shown in Fig. 3 and Fig. 4 for RS and DCI related processing, when two different AI functions are switching, additional X1 slots can be added to account for the time required to switch from activated state to inference state. Whether the additional processing is required or not, can be up to UE capability reporting.

Fig. 3 Example of processing timeline with activated AI function switch, for RS related processing

Fig. 4 Example of processing timeline with activated AI function switch, for DCI related processing

Proposal 6: Define UE capability for additional processing delays when AI functionalities switch between the activated state and the inference state.

Proposal 7: Additional processing time X1 between transition from “activated state” to “inference state” can be added to Z1/Z2/Z3 and Z1’/Z2’/Z3’ during AI functionalities switch/different CSI reports.

# Complexity and performance tradeoff

One aspect that has not concluded was complexity-performance trade-off. In RAN1 120, it was agreed:

Agreement

Study by comparing different AI/ML models, e.g. by optimizing existing designs, and/or by comparing different precoder representation in (spatial-frequency and angle-delay) or (spatial-frequency-time and angle-delay-doppler), by considering the following aspects

- Performance comparison between different AI/ML designs and benchmark schemes

- Complexity numbers (FLOP, calculated/projected latency or power consumption if available, measured latency or power consumption if available) of different AI/ML designs and benchmark schemes

We focus on spatial-frequency domain input in case 0, and tested different transformer parameters as shown in Table I, comparing to e-type 2 configuration 1. A generalized model is trained over 500 drops, with 21 cells per drop.

| Encoder parameters | Decoder parameters | FLOPs(M) | SGCS | SGCS gain over benchmark |
| --- | --- | --- | --- | --- |
| 636638 | 637440 | 11 | 0.715 | 4% |
| 1034078 | 1034880 | 18 | 0.723 | 5% |
| 2274526 | 2275328 | 39 | 0.732 | 6% |
| 4178334 | 4179136 | 72 | 0.73 | 6% |

Observation 2: For spatial-frequency domain CSI compression (case 0), by adjusting model hyper-parameter, lower FLOPs can be achieved with minor performance degradation.

# Conclusion

In this contribution, we discussed the remaining aspects of CSI compression, and APU processing timeline across different use cases. Based on the discussion, the following proposals have been proposed.

Proposal 1: For inter-vendor training collaboration under direction C and direction A, option 3a-1, the reference model structure and parameters reuse RAN4 specified model and parameters.

Proposal 2: Recommend RAN1 to specify 4-1 with non-OTA dataset delivery, with RAN4 specified model structure and parameters for Direction C.

Observation 1: When traditional filter-based CSI prediction is used, no performance monitoring is performed in MIMO. When AI based CSI prediction is used, separate CSI prediction performance monitoring is specified.

Proposal 3: For case 3 of time-frequency-spatial domain CSI compression, option 2a ensures E2E performance. The intermediate KPI or eventual KPI includes both compression and prediction performance.

Proposal 4: For case 2, time-frequency-spatial domain CSI compression, for RI, consider longer term RI update across different CSI reports.

Proposal 5: For time-frequency-spatial domain CSI compression, flexible CSI report configuration to support different cases should be studied.

Proposal 6: Define UE capability for additional processing delays when AI functionalities switch between the activated state and the inference state.

Proposal 7: Additional processing time X1 between transition from “activated state” to “inference state” can be added to Z1/Z2/Z3 and Z1’/Z2’/Z3’ during AI functionalities switch/different CSI reports.

Observation 2: For spatial-frequency domain CSI compression (case 0), by adjusting model hyper-parameter, lower FLOPs can be achieved with minor performance degradation.

# Reference

[1] RP-242399, Revised WID on artificial intelligence (AI)/Machine learning (ML) for NR air interface, Melbourne, Australia, September 9-12, 2024

[2] TR 38.843, Study on AI/ML for NR air interface, Nov 2023