3GPP TSG RAN WG1 #121			R1-2504348

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.12.2

Source:	Moderator (Apple)

Title:	FL summary #1 of Low band carrier aggregation via switching

Document for:	Discussion/Decision

# Introduction

The following lists the objectives in Rel-19 WI on Low NR band carrier aggregation via switching (RP-250247) with the highlight of RAN1 related part

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

This contribution summarizes the discussions on Low NR band carrier aggregation via switching in RAN1#120bis:

- Section 2 provides contact information for the participating companies

- Section 3 provides the proposals for online sessions.

- Section 4 provides documentation of the open discussions.

- Section 5 provides documentation of the closed discussions.

- Section 6 provides summary of contributions to RAN1#120bis

- Appendix A provides the agreements from the previous and current meetings

- Appendix B provides the LS from other WGs

# Contact Information

| Company | Contact information (Name, Email) |
| --- | --- |
| Moderator (Apple) | Haitong Sun (haitong_sun@apple.com) |
| MediaTek | Chiou-Wei Tsai (cw.tsai@mediatek.com) |
| Apple | Ankit Bhamri (a.bhamri@apple.com) |
| Ofinno | Muhammad Kazmi (mkazmi@ofinno.com) |
| Qualcomm | Fred Takeda (ktakeda@qti.qualcomm.com) |
| Samsung | Sa Zhang (sa.zhang@samsung.com) |
| NTT DOCOMO, INC. | Hiroki Harada (hiroki.harada.sv@nttdocomo.com) |
| Spreadtrum | Huan Zhou (Huan.Zhou@unisoc.com ) |
| Ericsson | Stephen Grant (stephen.grant@ericsson.com ) |
| CATT | Min Zhu (zhumin@catt.cn ) |
| Nokia | Karri Ranta-aho (karri.ranta-aho@nokia.com) |
| LG Electronics | Sukchel Yang (suckchel.yang@lge.com) |
| Lenovo | Haipeng Lei (leihp1@lenovo.com) |
| ZTE | Fei Wang (wang.fei11@zte.com.cn) |
| vivo | Na Li (lina5g@vivo.com) |
| Google | Abdellatif Salah (asalah@google.com) |
| OPPO | Wenfeng Zhang (zhangwenfeng@oppo.com) Jing Xu (xj@oppo.com) |

# Proposals for Online/Offline Sessions

## Proposals for Monday Offline

Proposal 1-1 (offline consensus)

For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration.

Proposal 1-2 (offline consensus)

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support at least the switching gap is located at the “switch from” carrier, i.e., FDD carrier.

- Switching gap ends at the end of the last slot on the “switch from” carrier

Proposed Conclusion 2-1

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, there is no consensus to introduce any SCell dormancy enhancement.

- The same switching pattern should be applied regardless of whether the active DL BWP for the SCell is dormant BWP.

Collision handling discussion 1

Whether collision with switching gap, and FDD/SDL period are designed differently

- Collision with switching gap is avoided by the NW scheduling (switching gap should not take much resource)

- Collision with FDD/SDL period may need UE to perform dropping

Collision handling discussion 2

How to have unified solution. In current TS38.213

Can we consider the following solution

For Rel-19 low NR band carrier aggregation via switching, and for a transmission/reception overlapping with (in) a slot indicated as ‘0’ on SDL carrier or a slot indicated as ‘1’ on FDD carrier by the bitmap or switching gap, reuse the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum

- For a transmission overlapping, even partially, with the set of symbols of the slot indicated as SDL carrier or switching gap, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS

- For a reception on FDD carrier overlapping, even partially, with the set of symbols of the slot indicated as SDL carrier or switching gap, the UE does not receive PDCCH, PDSCH, or CSI-RS

- For a reception on SDL carrier overlapping, even partially, with the set of symbols of the slot indicated as FDD carrier or switching gap, the UE does not receive PDCCH, PDSCH, or CSI-RS

# (Open Discussion) Low band carrier aggregation via switching

## Configuration of semi-static switching pattern

### Number of configuration per cell

In the previous RAN1#120bis meeting, the following agreement was made

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers

During RAN1#120bis discussion, it was also proposed to consider only one semi-static switching pattern configured by the network. However, this was not agreed mainly due to uncertainty about how to handle SCell activation/deactivation.

[Closed][R1] Proposal 1-1

For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration.

| Company | Comments |
| --- | --- |
| Moderator | Based on the latest LS from RAN4 in R1-2503215, moderator thinks it is ready to agree on configuration of only one switching pattern in RAN1. This is proposed by ZTE, Spectrum, Apple |
| Samsung | Support. We think this should be the default, also considering the LS response form RAN4, if no consensus is reached for more than one configuration. |
| MediaTek | Support |
| Ofinno | Support |
| Qualcomm | We are OK with the proposal. |
| Nokia | We are OK with the proposal. |
| vivo | Support |
| Googlw | Ok with the proposal |
| Ericsson | Support Proposal 1-1 |
| Spreadtrum | Support |
| OPPO | Support |
| Apple | Support |
| NTT DOCOMO | Support the proposal 1-1 |
| CATT | Support |
| Lenovo | OK |
| ZTE | Based on the LS from RAN4, we think one TDM pattern should be enough. |
| LGE | Support |
| Moderator | Discussion closed |

### Switching gap location

In the previous RAN1#120bis meeting, the following agreement was made with the highlight part for down-selection

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier

[Closed][R1] Proposal 1-2

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, downselect one of the following solutions

  - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

- When switching gap is located at the “switch from” carrier, switching gap ends at the end of slot on the “switch from” carrier

- When switching gap is located at the “switch to” carrier, switching gap begins at the beginning of slot on the “switch to” carrier

| Company | Comments |
| --- | --- |
| Moderator | We definitely need to complete the down-selection in this meeting. Below is based on companies contributions Alt 1 (9): vivo, Nokia, Spectrum, ETRI, LG, MediaTek, Apple, Ericsson, DOCOMO Alt 2 (8): Huawei, ZTE, Ofinno, CATT, OPPO, Qualcomm, DOCOMO, Google Alt 3 (2): NEC, Samsung The companies’ positions remain similar, and moderator does not see much value to have extended discussion. Based on the number of supporting companies, Alt 3 from the original agreement was removed. Please indicate whether you have strong concern on Alt 1 or Alt 2, and whether you can be flexible |
| Samsung | We still think Alt 3 is the simplest way to go. To support alt 1 or alt 2, we need to clarify UE behaviour regarding UL transmissions for the following cases. - Collision handling for single UL transmission as discussed in section 4.3.1, in addition to the case illustrated in section 4.3.1, we also need to consider the case below. The TA and ending symbol of a UL transmission would impact whether the collision exists or not. - Collision handling for multiple UL transmissions. For example, for the case below, there can be two candidate UE behaviours. It should be clarified in the specs which is the intended UE behaviour. - Option 1) The UE first drops the PUCCH including the UCI due to colliding with the switching gap and only transmits the PUSCH without the UCI. - Option 2) The UE first multiplexes the UCI in the PUSCH and transmits the PUSCH with the UCI. If a clarification for the above is provided, we will be OK with Alt. 1 to avoid having to support the complexities/specifications associated with having both alternatives (that is something that should generally be always avoided). |
| MediaTek | We are supportive for Alt 1 to protect PDCCH. In our view, PDCCH protection should be prioritized over protection of PUSCH/PUCCH/SRS/CSI-RS in the FDD carrier because of the following reasons: - The SDL carrier is aggregated to offload downlink traffic. For uplink traffic heavy UEs, NW don’t need to configure such UEs with SDL and a switching pattern. Scheduling DL on SDL should hence be prioritized over UL on FDD for these UEs. - Without assuming the support for any optional UE features as prerequisite, if we just look at the current NR specifications, we should be able to agree the following observations: - If switching gap (35us or 140us) is located at the “switch-to” carrier (i.e. SDL), PDCCH symbol loss is IRRECOVERABLE regardless of gNB’s implementation and configurable. In addition, PDCCH period is commonly assumed to be EVERY slot (unlike SRS or CSI-RS with longer periods). - On the other hand, if switching gap is located at the “switch-from” carrier (i.e. FDD), the impact on mentioned channels/signals (PUCCH formats 1 & 4, SRS, PUSCH, and CSI-RS) can all be avoided by gNB’s implementation and configurable WITHOUT requiring support for any optional UE features. - PUCCH formats 1 and 4: duration configurable from 4 to 14 symbols and all NR UEs are mandatory to support the configuration. In addition, PUCCH format 0 can be an alternative to PUCCH format 1 to carry 1 or 2 UCI bits. Similarly, PUCCH formats 2 and 3 can be the alternatives to PUCCH format 4 to carry larger UC bits. - Furthermore, the impact on legacy UEs can be avoided by (1) configuring 12 instead of 14 symbols of PUCCH formats 1 and 4. All legacy UEs can support this change by default, or (2) allocating different PUCCH resources for legacy UEs and LBCA UEs, or (3) not scheduling LBCA UEs with PUCCH formats 1 and 4 in the slots when switching from FDD to SDL is configured. - SRS: configurable symbol duration and typical periods observed in field are 40ms, 20ms, and maybe 10ms. All periods are much longer than 1ms which is typically assumed for PDCCH. - 14-symbol PUSCH: Again, PUSCH TDRA is configurable by NW and all NR UEs are mandatory to support PUSCH configuration from 1 to 14 symbols. - CSI-RS in the last two symbols: Again, gNB can choose not to place them in the last two symbols in a slot where UE is configured to perform switching. This implies at least two options for gNB: (1) not configuring CSI-RS in the last two symbols which in fact is not a typical configuration in field base on our observation (2) not to configure CSI-RS in the same slot(s) as FDD-SDL switching. - Regarding the TA issue raised by Samsung in the above, though it may be a valid issue, we don’t think Alt 3 is the only solution. We think such a case should be avoided by gNB’s scheduling in the first place. More specifically, if the switching gap for FDD->SDL switches is configured to 2 symbols (in order to cover the maximum switching period of 140us), then gNB shall not schedule/configure both uplink and downlink in the last two symbols. |
| Ofinno | We support Alt 2 as it gives network sufficient flexibility and is a compromise solution. We are also open for Alt 1 or Alt 3. We don’t think removing Alt 3 is necessary. |
| Qualcomm | We are OK with the FL proposal in general. We think Alt.2 is a simple and compromised solution. Suggest following minor updates. (Suggested updates) - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - When switching gap is located at the “switch from” carrier, switching gap ends at the end of the last slot on the “switch from” carrier - When switching gap is located at the “switch to” carrier, switching gap begins at the beginning of the first slot on the “switch to” carrier We agree with Samsung that the clarification is necessary for the UL transmission around the switch from FDD to SDL carrier. However, we think this is a separate issue. |
| Nokia | We would prefer simplification of the design and stick with Alt.1 Alt.2 could be considered as a compromise, but the argument for network flexibility only holds if the configurability is part of the baseline UE capability. Furthermore for the gap in the beginning of the SDL slot to be useful the UE needs to support delayed PDCCH configuration, i.e. at least one of - Rel-15 FG3-2 PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot Rel-16 FG22-12 PDCCH monitoring with a single span of three contiguous OFDM symbols that is within the first four OFDM symbols in a slot |
| vivo | We support Alt 1 to make the first slot after switching to SDL can be used by gNB for DL transmission considering only self-scheduling is supported and to make the design simple. For the issue commented by Samsung. We have the following vizews. - Collision handling for single UL transmission as discussed in section 4.3.1, in addition to the case illustrated in section 4.3.1, we also need to consider the case below. The TA and ending symbol of a UL transmission would impact whether the collision exists or not. [vivo]: To solve this issue, two options can be considered. - Option 1: it is not expected that gNB schedules/configures a PUXCH transmission overlapping with the switching gap considering TA in the slot with switching gap - Option 2: UE always assumes the last 1/2 symbols (the number of symbols depends on the duration of switching gap) are not available for UL transmission. And same handling as the slot configured on SDL. E.g. drop the UL transmission if it overlaps, even partially with the set of symbols of switching gap. - Collision handling for multiple UL transmissions. For example, for the case below, there can be two candidate UE behaviours. It should be clarified in the specs which is the intended UE behaviour. - Option 1) The UE first drops the PUCCH including the UCI due to colliding with the switching gap and only transmits the PUSCH without the UCI. - Option 2) The UE first multiplexes the UCI in the PUSCH and transmits the PUSCH with the UCI. [vivo]: The issue only happens when above option 2 is adopted. Since the switching gap is semi-statically configured by gNB, for a UL transmission, the symbols in switching gap is similar as symbols configured as semi-static DL symbols. Legacy handling mechanism can be reused. i.e. UE performs UL multiplexing of PUCCH/PUSCH before considering limitations for UE transmission due to switching gap. |
| Google | Alt 2 provides the network with good flexibility. If protecting PDCCH on the SDL carrier is paramount, NW can configure the gap on the FDD carrier. Conversely, if minimizing disruption on the FDD carrier uplink is more critical, NW can choose to place the gap on the SDL carrier. Alt-1 is simpler but it is restrictive. Alt-1 can also be achieved by Alt-2 is the NW finds it necessary to prioritise SDL PDCCH |
| Ericsson | We support Alt-1 with a strong desire to keep the LB CA Switching feature as simple as possible. We are concerned that Alt-2 requires optional UE features to work (cross-carrier scheduling, “Case 2” PDCCH monitoring, etc.). Hence, even if Alt-2 was agreed, the baseline UE capability would need to be defined with the gap on “switch from” carrier since self-scheduling is baseline. Consequently, gap on “switch to” carrier would need to be an optional UE capability (separate, optional FG). The likelihood of this being implemented then becomes very low due to the dependency on other optional UE features. So, in the end, there is no value in Alt-2 even if it appears to result in a more flexibility for the gNB. Regarding the “collision handling for multiple UL transmissions” raised by Samsung, we agree with vivo; UL multiplexing of UCI on PUSCH (due to PUCCH overlapping PUSCH) can be performed before considering collisions of PUSCH/PUCCH with the switching gap on the FDD carrier. Agree with the following update proposed by Qualcomm: When switching gap is located at the “switch from” carrier, switching gap ends at the end of the last slot on the “switch from” carrier |
| Spreadtrum | We support Alt1 to protect PDCCH monitoring on SDL. For the two issues commented by Samsung, the common part is UE behavior when UL colliding with switching gap. For dynamic UL, it can be avoided by gNB. For semi-static, views are diverse, anyway, there will be clear clarifications for them. |
| OPPO | Fine with proposal and alt 2 is a simple and compromised way. |
| Apple | In principle, both alt works, and it is a matter of preference. If we mix the discussion of collisions here, then there could be potential issues with both Alts. However, collision handling is separate discussion, where it might be difficult to converge in terms of UE behavior. Considering these are semi-static configurations, we believe most collisions can be avoided with network scheduling, so we should not use that argument for adding unnecessary flexibility with Alt 2. Therefore, from the perspective of keeping the feature simple from RAN1 perspective, we prefer Alt1 |
| NTT DOCOMO | Support the proposal 1-2 in general and can be flexible between Alt.1 and Alt.2. But even if we go to Alt.2, it should not be subject to UE capability. |
| CATT | We prefer to support Alt 2. It can give network flexibility to configure the location of the switching gap. It can up to gNB configuration to protect the transmission on UL carrier or DL carrier. |
| Lenovo | Our preference is, for switching from FDD carrier to SDL carrier, the switching gap on the FDD carrier or SDL carrier is determined by UE itself, e.g., if the time domain resource allocation of the ongoing DL transmission on the FDD carrier does not include the last symbol, or UE needs to monitor the search space from the first symbol of the SDL carrier, then the switching gap is located on the FDD carrier; otherwise, the switching gap is located on the SDL carrier. But for sake of progress, we can live with Alt 1. |
| ZTE | Alt 2 offers the network great flexibility. The network should have the option to decide which signal/channel to protect based on the operator's requirements, e.g., the PDCCH on the SDL carrier or the PUCCH/SRS on the FDD carrier. And the proposal 1-2 which only rules out Alt 3 is unnecessary. |
| LGE | We are also fine with Proposal 1-2 from FL. And in our view, Alt 1 is preferred for pursuing the unified switching behaviour with consistent reason to protect PDCCH, which is aligned with the motivation of LB-CA via switching. |
| Samsung2 | It seems the majority of companies are fine with Alt 1 or Alt 2. For the sake of progress, we can be constructive for Alt 1 to avoid unnecessary complexity on the condition of no remaining issues for Alt 1 as we mentioned in previous comments. As mentioned by vivo, the collision issue for LB-CA is very similar as for semi-static DL/UL symbols collision in TDD. We can use the same rule to resolve collisions for LB-CA as and propose the following complimentary proposal to support Alt 1 and avoid open issues. Proposal 1-2b For Rel-19 low NR band carrier aggregation via switching, and for a transmission overlapping with a switching gap on FDD carrier, reuse the collision handling rules for a transmission overlapping with semi-static DL symbols for unpaired spectrum, i.e., the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps with a switching gap on FDD carrier. - The UE resolves the overlapping for UL transmissions before considering the limitations for UL transmission due to switching gap. |
| Moderator | Based on the comments so far, the proposed WF is Proposal 1-2 For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, support at least the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier Further check whether to support the following Strongly support: Strong concern: Support switching gap is at the “switch to” carrier, i.e., SDL carrier, via per-UE RRC configuration - Switching gap begins at the beginning of the first slot on the “switch to” carrier Discussion closed |

### Switching pattern restriction

In the previous RAN1#120bis meeting, the following agreement was made with the highlight part for down-selection

Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction

Below is the summary based on companies’ contributions

| Companies | Proposed Restriction X switch(es) within Y slot(s) |
| --- | --- |
| Huawei | (X, Y): (2, 1) |
| ZTE | (X, Y): (>16, 40) |
| vivo | (X, Y): (1, 5) SDL carrier cannot be configured with dormant BWP UE does not stay contiguously on SDL carrier for > 15 slots |
| Nokia | (X, Y): (2, 4) |
| Spectrum | (X, Y): (1, 5) Contiguous slots of SCell cannot be exceed 8 slots At least one SSB is available every 160 ms for both of PCell and SCell if applicable |
| Samsung | (X, Y): (8, 40) The maximum of continuous slots on SCell is 7 |
| Lenovo | (X, Y): (2, 10) |
| Ofinno | (X, Y): (1, 2), (1, 5≥M≥2) maximum consecutive slots assigned to the SDL carrier does not exceed 16 at least one SMTC occasion per 40 msec is allocated to the FDD carrier and SDL carrier respectively. |
| CATT | (X, Y): (2, 10), (3or 4, 20), (6 or 7 or 8, 40) |
| OPPO | (X, Y): (2, 10), (1,1) |
| LG | (X, Y): (1,1) |
| MediaTek | (X, Y): (8, 40), (1,1) at least one SSB every X1 and X2 msec on the FDD carrier and SDL carrier, respectively, with X1 <=min(160, 2*SSB period on the FDD carrier) and X2<=min(160, 2*SSB period on the SDL carrier). |
| Apple | (X, Y): (1,1) |
| Qualcomm | (X, Y): (8, 40), (1,4) |
| Ericsson | No restriction, or (X, Y): (>=10, 40) |
| DOCOMO | (X, Y): Y – X = 3 or 4 maximum 8 consecutive SDL slots |
| Google | (X, Y): X < Y/T |

[Closed][R1] Proposed Conclusion 1-1

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, there is no consensus to introduce additional restrictions in RAN1 specification, including

  - Restriction of maximum X switch(es) within Y slot(s),

  - Other restriction

- Note: restriction can be further discussed in UE feature

| Company | Comments |
| --- | --- |
| Moderator | The table above summarizes diverging views from different companies. Given this is the last RAN1 meeting, and restriction may not be an essential feature, Moderator has concern on the progress, the time that needs to discuss further restriction and high risk of no consensus in the end. This topic can only progress or worthwhile to discuss if the scope can be further reduced. Please indicate only the absolute essential restriction below. |
| Samsung | Although we had a different proposal, we are fine with the proposed conclusion. |
| MediaTek | We don’t agree with this proposal. Without any restriction, how should UE understand what it is expected of? Based on our tdoc review, we think the following two values can serve as starting point: (X, Y) = (1, 1) and (8, 40). |
| Ofinno | Our proposal is though different; but are also fine with moderator’s proposed agreement. However, we still believe it would be good to clarify there is only one switching gap in a slot. |
| Qualcomm | We agree with MediaTek. It is not clear to us what the “no consensus” means here. The proposed conclusion looks to us that any of 240 bitmap patterns shall be supported by a UE. In our understanding, RAN1 already agreed to have restriction with (X, Y), where the values are FFS. We also partly agree with MediaTek that (X, Y) = (8, 40) should at least be starting point. Further, we think (X, Y) = (1, 4) is more reasonable additional constraint. |
| Nokia | In general, agree that the restriction is not necessary to finish the design. At most X switches within Y slots formulation would seem to allow for allocating a single slot for SDL or for FDD (bit-sequence including …101… and …010…), and in that sense the additional restriction would not impact the minimum distance between two switches. However, we don’t think a bit-pattern of …10101010… is meaningful, and if ruling such configurations out is helpful for the UE implementation, we are supportive; no point in requiring the UEs to support configurations that are not meaningful. |
| vivo | We agree with MediaTek, Qualcomm. This issue can even be discussed in the maintenance phase. There is no need to rush a conclusion to say “no consensus”. |
| Google | From UE implementation perspective, a restriction is essential, as no restriction implies potentially very frequent switching. To protect against excessively dense switching patterns and ensure FDD carrier viability, we propose a targeted approach, and instead of (X, Y) based restriction, let’s consider a minimum dwell time on the FDD carrier (e.g., 2 slots). This minimum FDD dwell time offers a baseline protection for UE performance against fragmented carrier usage and could simplify defining acceptable switching behaviour. |
| Ericsson | As discussed in our contribution we don’t think a restriction should be imposed on the gNB given the complexities of scheduling. So in that sense, we agree with the first bullet of Conclusion 1-1. Regarding the 2nd bullet, we don’t think we should punt the discussion to UE capabilities. We would like to hear from other companies what is the complexity concern that motivates specifying X and Y. From our perspective, it would not make sense for the gNB to configure very frequent switching since there is an efficiency loss. However, if it helps progress, we would be fine to agree that the UE expects no more than 1 switch per slot (X,Y) = (1,1) and then not discuss further in UE capabilities. |
| Spreadtrum | We can go with (8, 40), and share a same view that (X,Y) was agreed, only value is FFS. |
| OPPO | From UE perspective, restriction is essential. At least restriction pattern (1,1) should be considered. We do not see that any use case requires more than 2 switch per slot. Note that restriction pattern (1,1) has been specified in Tx switching, we do not see any strong requirement for LBCA to relax this restriction. Other restriction pattern can be discussed further due to it is benefit to reduce UE power consumption and complexity. |
| Apple | As discussed in our contribution, baseline requirement could and should be that no more than 1 switching is allowed per slot. As mentioned earlier, even for a dynamic UL Tx switching, we have a minimum restriction of 1 switching per slot. So as a baseline, this should be agreeable. On any additional restriction, although from UE perspective, we would prefer it, however, we don’t see it really a necessity and to allow flexibility for network pattern, we are okay not to consider any further restrictions |
| NTT DOCOMO | We don’t support the proposal. As several companies pointed, it is not valuable to support full flexibility for the switching pattern that would cause unnecessary testing to report the support of LBCA. We should stick to realistic patterns based on the principle for this WI, and hence we suggest discussing appropriate values for (X, Y) and maximum consecutive SDL slots at least to avoid unnecessary considerations (spec impacts, clarifications, implementation, testing, etc.) for unrealistic cases due to full flexibility on switching patterns. |
| Lenovo | We think this is restriction is necessary for UE. We are OK with this proposal and prefer to discuss this restriction on UE features session. |
| ZTE | We agree with the FL that the restriction on (X, Y) may not be an essential feature. Considering there are so many candidate proposals. if it helps progress, we suggest to reach a consensus on what UE vendors do not expect (we think this issue maybe a absolute essential restriction), e.g., UE is not expected to perform more than one switch within a slot. |
| LGE | We are OK with Proposed Conclusion 1-1 from FL. And as stated in Note, any potential restriction, if necessary, can be further discussed in UE feature session. |
| Moderator | Will provide refined proposal for the next round discussion. # Discussion closed |

## Impact on PHY procedure

### UE processing timeline relaxation

In terms of the UE processing timeline relaxation, it was discussed via email briefly in the last meeting without much progress. In the legacy UL Tx switching, the additional processing time, i.e.,Tswitch, was introduced at least for the following cases, for example

- PUSCH processing time, reference TS38.214 Clause 6.4

- UE CSI computation time, reference TS38.214 Clause 5.4

- DCI triggered SRS transmission, reference TS38.214 Clause 6.2.1

- PRACH transmission, reference TS38.213 Clause 8.1

- Etc.

However, it is worthwhile to note that the UL Tx switching can be triggered by DCI, but Rel-19 low NR band carrier aggregation via switching is configured semi-statically via RRC.

[Closed][R1] Proposal 2-1

For Rel-19 low NR band carrier aggregation via switching, in terms of introducing UE processing timeline relaxation, i.e., TLB, switch, downselect one of the following

- Alt 1: Do not introduce any UE processing timeline relaxation

- Alt 2: Introduce UE processing timeline relaxation

  - Support one or multiple or all of the following cases

    - Case 1: A PUCCH transmission with HARQ-ACK for a dynamically scheduled PDSCH by PDCCH

    - Case 2: A PDCCH reception schedules a dynamic-grant PUSCH transmission

    - Case 3: A PDCCH reception indicates aperiodic SRS transmission

    - Case 4: A RAR schedules a PUSCH transmission

  - The UE processing timeline relaxation, i.e., TLB, switch, is reported by the UE as the switching period defined in RAN4 LS R1-2501702, i.e., among {35us, 140us}

  - UE does not expect to have more than one SDL->FDD switch or more than one FDD->SDL switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission (PUCCH/PUSCH/SRS).

| Company | Comments |
| --- | --- |
| Moderator | Below is the summary based on the companies’ contributions Alt 1 (1): ZTE Alt 2 (5): Samsung, Ofinno, LG, MediaTek, Qualcomm Even though strictly speaking, Alt 1 is the nature outcome if no consensus can be reached for the UE processing timeline relaxation, since this is related to the actual implementation of the feature, some discussion is expected for this proposal. Please be constructive, and indicate (1) whether and which case you can accept for opponents, (2) which cases are important for proponents |
| Samsung | We are fine with the intention of the proposal. We have concerns on the second last sub-sub bullet “The UE processing timeline relaxation, i.e., TLB, switch, is reported by the UE as the switching period defined in RAN4 LS R1-2501702, i.e., among {35us, 140us}”. In RAN4 discussion, they didn’t discuss the timeline relaxation issue for the cases mentioned in the proposal. To simply the discussion, the timeline should be additionally discussed and reported by a UE with a separate UE feature. If UE only report the timeline for one worst case, it would unnecessarily increase the switching gap. We suggest the following update - The UE processing timeline relaxation, i.e., TLB, switch, is reported by the UE with an additional UE capability. as the switching period defined in RAN4 LS R1-2501702, i.e., among {35us, 140us} We suggest to remove the last sub-sub bullet, if there are more than one switching, it is highly likely the timeline should be enough. We can focus on one switching point case for now. For the 4 cases, we think case 1 is of top priority. Case 2 and 3 should also be considered, these two cases are for cross carrier scheduling. For example, PDCCH on SCell schedules PUSCH/SRS on PCell and there is a SDL to FDD switching between PDCCH and PUSCH/SRS on PCell. Case 4 can be deprioritized for now. Instead of listing the two alternatives, we suggest to directly discuss whether Alt 2 is supported to avoid leaving open issues considering this is the last RAN1 meeting. |
| MediaTek | We are in principle supportive for Alt 2 except for Case 4. In our view, once RACH is triggered, UE should stay on the FDD carrier to complete RACH. As for TLB, switch, our understanding is the additional time to be introduced is for UE performing RF retuning and therefore it will be reported by UE based on RAN4’s agreements. We do not see the need to introduce UE capabilities for reporting this additional time. For last bullet, we suggest keeping it. More than SDL->FDD or FDD->SDL switch does not seem necessary for the K1 & K2 scheduling of interest. |
| Ofinno | We support Alt 2. We also agree with MediaTek that when the RACH is triggered then the UE should stay on the FDD carrier and complete the RACH (i.e., do not switch to the SDL carrier during the RACH procedure. Therefore, Case 4 should be excluded from Alt 2. |
| Qualcomm | We think the necessity of timeline relaxation should be discussed after physical layer procedure is well understood. More specifically, we need to first understand the following. - Is there any case where the UE has to ignore the semi-statically configured switching pattern/gap and instead has to follow a DCI-triggered/scheduled UL transmission/DL reception that conflicts with the semi-static pattern/gap? - This requires timeline relaxation, since the UE can tune the RF only after DCI decoding/parsing is completed. - Is there any case where a UE has to drop, defer, postpone, etc, the process for UL transmission/DL reception triggered/scheduled by a DCI, when it conflicts with semi-static pattern/gap? - This would/may require timeline relaxation, depending on what UE shall do to resolve the conflict. - Does UE need timeline relaxation, when a DCI received before a semi-statically configured switch triggers/schedules UL transmission/DL reception after the switch? - RF switching timing is known by the UE preliminarily by RRC, and is irrelevant to the DCI parsing and process of the DL reception/UL transmission. Is relaxation really necessary, and for which case? We are quite unclear about Cases 1-4 and hence would like to get clarifications on them. Is Case 1 “PDSCH is scheduled on the SDL carrier before a switch, and PUCCH for HARQ-ACK transmission for the PDSCH is on FDD carrier after the switch”? Is Case 2 “A PDCCH reception on the SDL carrier before a switch schedules a dynamic-grant PUSCH transmission on the FDD carrier after the switch”? If so, does the case assume Rel-17 DSS cross-CC scheduling from SCell to PCell? Is Case 3 “A PDCCH reception on the SDL carrier before a switch indicates aperiodic SRS transmission on the FDD carrier after the switch”? Or, “A PDCCH reception on the FDD carrier before a switch indicates aperiodic SRS transmission on the SDL carrier using carrier-switching SRS”? Assuming the above Case 1-3 are correct, why “A PDCCH reception on the FDD carrier before a switch triggers aperiodic CSI-RS reception on the SDL carrier after the switch” is not listed? Further, should we also list “A PDCCH reception on the FDD carrier before a first switch (to SDL carrier) triggers aperiodic CSI-RS reception on the SDL carrier after the first switch, and also schedules a PUSCH transmission for the CSI report on the FDD carrier after a second switch (to FDD carrier)”? Regarding Case 4, we are not sure which exact scenario this is about. We think RAR is on the PCell and the corresponding PUSCH is also on the PCell. Or, is this about the case where RAR is received on the FDD carrier before a first switch (from FDD to SDL) and is scheduling the PUSCH on the FDD carrier after a second switch (from SDL to FDD)? |
| Nokia | We have serious concerns in touching the processing timelines. The cross-carrier HARQ-ACK timeline (PDSCH on SCell, HARQ-ACK on PCell) exists since Rel-15, and the Rel-19 design should not force re-wiring those loops in the network implementations. |
| Google | We support Alt- 2 for the UE processing timeline relaxation. This is important for Cases 1 (HARQ-ACK), 2 (DG-PUSCH), and 3 (A-SRS). For Case 4 (RAR PUSCH), we agree with MediaTek, the UE should stay on the FDD cell for the RACH procedure and no switch should occur in this case. |
| Ericsson | We prefer Alt-1 and we agree with the moderator’s statement that processing time relaxation is not relevant for LB CA with switching compared to UL Tx Switching since the switching pattern is semi-statically configured. Moreover, our view is that when the UE is semi-statically configured with a semi-static switching pattern, the UE shall not perform additional switching that violates the configured switching pattern, since that would violate the WID. While we prefer not to introduce processing time relaxation, if the matter must be discussed, we agree with Qualcomm that this discussion should be deferred until the complete switching behavior is understood. We also share the view that Cases 1 – 4 are not clear, and they do not motivate why extra processing time is needed. |
| Spreadtrum | We are open to the timeline relaxation. For the four cases, the focus can only be case 1 - Case 1: A PUCCH transmission with HARQ-ACK for a dynamically scheduled PDSCH on SDL by PDCCH |
| OPPO | Similar view as Spreadtrum, only case 1 can be considered. Case 2-4 are cross-carrier scheduling cases and we do not think that both cross-carrier scheduling and LBCA are supported simultaneously. |
| Apple | From UE perspective, of course, our preference would be to have UE processing timeline relaxation considering switching. However, we also understand that this feature is different compared to previously specific UL Tx switching as it was dynamic in nature, while this featire is semi-static in nature. Therefore, from UE perspective, it shall have reasonable time in advance to prepare for switching and therefore, we would be fine not to consider any processing timeline relaxation. Also, considering this is last meeting, we don’t see we can have reasonable discussion on this considering there are a number of scenarios to consider |
| NTT DOCOMO | We prefer Alt.1 based on similar views with Qualcomm, Nokia and Ericsson. We should discuss and clarify Qualcomm’s three questions first. |
| CATT | We prefer Alt.1 since the switching pattern is configured by RRC. |
| Lenovo | Alt 1 is supported. |
| ZTE | We agree with Qualcomm, if there is no additional switch triggered with ignoring the semi-statically configured switching pattern, then it is not necessary to introduce UE processing timeline. |
| LGE | We are fine with Proposal 2-1 from FL. Specifically, we only support Case 1 of Alt 2 since other Cases 2/3/4 would be true only if cross-CC UL scheduling from FDD Pcell to SDL Scell is considered, however there is no UL in SDL carrier. |
| Moderator | Need to discuss two things - When to discuss this topic, after collision handling is clear? - Which issue is essential among the following 4 cases - PDCCH reception on SDL carrier schedules PDSCH reception on SDL carrier, and corresponding PUCCH for HARQ-ACK transmission on FDD carrier - PDCCH reception on SDL carrier schedules PUSCH transmission on FDD carrier. - PDCCH reception on SDL carrier schedules aperiodic SRS transmission on FDD carrier. - PDCCH reception on SDL/FDD carrier schedules aperiodic CSI-RS reception on FDD/SDL carrier. Discussion closed |

### HARQ-ACK codebook

Similar as the previous RAN1#120bis meeting, HARQ-ACK codebook enhancement was raised based on companies’ contribution this meeting.

[Closed][R1] Proposal 2-2

For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for Type-I HARQ-ACK codebook generation, downselect from one of the followings

- Alt 1: No enhancement

- Alt 2: A row of the TDRA table overlapping with the invalid symbol(s) is excluded from the determination of the set of occasions for candidate PDSCH receptions. Invalid symbol(s) include

  - The symbol(s) overlaps with the configured switching gap

  - The symbol(s) during the semi-static configured PCell (FDD carrier) duration for SCell (SDL carrier)

  - The symbol(s) during the semi-static configured SCell (SDL carrier) duration for PCell (FDD carrier)

| Company | Comments |
| --- | --- |
| Moderator | Below is the summary based on the companies’ contributions Alt 1 (3): Ofinno, Qualcomm, Apple Alt 2 (6): vivo, Spreadtrum, Samsung, Lenovo, CATT, LG, Technically, RAN4 already agrees that SCell deactivation leads to that the switching pattern for SDL SCell is not applied, and MAC-CE is used for SCell activation and deactivation. Unless there is absolute consensus, the default is Alt. 1. Please indicate whether you have strong concern on Alt 2. |
| Samsung | We support Alt 2 which is aligned with the basic design principle for Type-1 HARQ-ACK codebook to minimize the codebook size by removing invalid SLIVs. This is beneficial for resource reduction as well as UE energy savings. The spec impact is minimal, only need to reuse the rules for semi-static UL symbols, and results to consistent design when a PDSCH reception includes invalid symbols. Some editorial suggestion as below for Alt 2 to include the case for PDSCH with/without repetitions. - Alt 2: A row of the TDRA table overlapping with the invalid symbol(s) is excluded from For the determination of the set of occasions for candidate PDSCH receptions, reuse the legacy rules of SLIV overlapping with semi-static UL symbols for SLIV overlapping with invalid symbol(s). Invalid symbol(s) include - The symbol(s) overlaps with the configured switching gap - The symbol(s) during the semi-static configured PCell (FDD carrier) duration for SCell (SDL carrier) - The symbol(s) during the semi-static configured SCell (SDL carrier) duration for PCell (FDD carrier) Instead of listing the two alternatives, we suggest to directly discuss whether Alt 2 is supported to avoid leaving open issues considering this is the last RAN1 meeting. |
| Ofinno | We support Alt 1. We fell Alt 2 seems unnecessary optimization, but open to discuss. |
| Qualcomm | We support Alt.1 and we are not OK with Alt.2. Reason 1: RAN4 agreed that semi-static switching pattern/gap is applicable/not-applicable depending on whether the SCell is activated/deactivated. Legacy Type-1 HARQ-ACK codebook is defined such that PDSCH occasions for an SCell are not dependent on the SCell activation/deactivation. Therefore, Alt.2 will change the basic framework of Type-1 HARQ-ACK codebook. Reason 2: Even without taking into account the RAN4 agreement, Alt.2 is just for an optimization of HARQ-ACK codebook size. Given Type-2 HARQ-ACK codebook is a mandatory feature, there is no point to change the Type-1 HARQ-ACK codebook generation framework. Reason 3: As Ofinno pointed out above, this is unnecessary optimization, and is against the note in the WI scope: “Note 2: Strive to minimize the RAN1 impact” |
| Nokia | Similar view as Qualcomm. |
| vivo | We support alt 2. We think for DL transmission on FDD carrier/SDL, the symbols configured as SDL carrier/FDD carrier can be treated as semi-static UL symbols and can reuse the same rules for HARQ-ACK codebook generation as legacy. The spec change is minor but it is beneficial to reduce the codebook size. In addition, the update from Samsung is fine to us. |
| Google | Support Alt -1. We agree with Qualcomm. This is just an optimization of the HARQ-ACK codebook size and it is not very critical. |
| Ericsson | Similar view as Qualcomm and Nokia. According to Reason 1 stated by Qualcomm, this should immediately rule out making any optimization of the Type-1 HARQ-ACK codebook. Furthermore, minimal spec impact is not a sufficient argument to motivate optimization. |
| Spreadtrum | We can also go with Alt 1 for progress. |
| OPPO | Similar view as Qualcomm, Support Alt 1. |
| Apple | We prefer Alt 1 and don’t see a need for optimization based on Alt 2 |
| NTT DOCOMO | Support Alt.1. |
| Lenovo | We support Alt 2 because, for Type-1 HARQ-ACK codebook, SLIV pruning is essential to keep the HARQ payload concise. |
| ZTE | We support alt 2. We think HARQ-ACK codebook should consider the invalid symbols in the configured switching pattern, and follow the same principle applied to TDD pattern. This can reduce the Type-1 codebook size by excluding some invalid bits (padding bits). And the spec change is minor and the benefit is clear. |
| LGE | Fine with Proposal 2-2 from FL. And Alt 2 is preferred to reduce unnecessary UCI overhead. |
| Samsung2 | As also mentioned by companies, the collision issue for LB-CA switching is practically same as the collision with semi-static DL/UL symbols. There is no reason to use different rules, and have different designs and inconsistent specifications. There is nothing different from the legacy case when a PDSCH overlaps with semi-static UL symbols. The same rule also already applies for the similar case of cell DTX. Therefore, we think that Alt 2 with our update should also apply for LB-CA. |
| Moderator | Based on the comments, towards the end of this meeting, it is likely Proposed conclusion 2-1 For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, there is no consensus for enhancement of Type-I HARQ-ACK codebook generation. Discussion closed |

Similarly, SPS PDSCH handling during HARQ-ACK codebook generation is proposed by [1] vivo and [9] Samsung

[Closed][R1] Proposal 2-3

For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for HARQ-ACK codebook generation, downselect from one of the followings

- Alt 1: No enhancement

- Alt 2: SPS PDSCH overlapping with the invalid symbol(s) is excluded from the generation of HARQ-ACK information. Invalid symbol(s) include

  - The symbol(s) overlaps with the configured switching gap

  - The symbol(s) during the semi-static configured PCell (FDD carrier) duration for SCell (SDL carrier)

  - The symbol(s) during the semi-static configured SCell (SDL carrier) duration for PCell (FDD carrier)

| Company | Comments |
| --- | --- |
| Moderator | Below is the summary based on the companies’ contributions Alt 1 (3): Ofinno, Qualcomm, Apple Alt 2 (3): vivo, Spreadtrum, Samsung Technically, RAN4 already agrees that SCell deactivation leads to that the switching pattern for SDL SCell is not applied, and MAC-CE is used for SCell activation and deactivation. Unless there is absolute consensus, the default is Alt. 1. Please indicate whether you have strong concern on Alt 2. |
| Samsung | Similar comment as for P2-3 and suggest the following update for Alt 2. - Alt 2: For SPS PDSCH overlapping with the invalid symbol(s) is excluded from, reuse the legacy rules for SPS PDSCH overlapping with semi-static UL symbols for the generation of HARQ-ACK information. Invalid symbol(s) include |
| Ofinno | Same view as for Proposal 2-2. We support Alt 1. |
| Qualcomm | Same view as for Proposal 2-2. We support Alt 1. |
| Nokia | Same view as in 2-2. Support Alt.1, could accept Alt.2 as well. |
| vivo | Same view as for Proposal 2-2. The invalid symbols due to switching pattern are treated as semi-static UL symbols and reuse the same rules. Fine with Samsung’s update. Note that when we treat the invalid symbols due to switching pattern as semi-static UL/DL symbols for DL reception/UL transmission. We can reuse the same rules as legacy for the following issues: - Tx/Rx behavior in invalid symbols, e.g. UE does not transmit or receive a UL/DL if the UL/DL overlaps with the invalid symbols. See the discussion in Proposal 3-1 in detail - UCI multiplexing order, e.g. UE performs UL multiplexing of PUCCH/PUSCH before considering limitations for UE transmission due to switching gap. See issue 2 raised by Samsung under discussion of proposal 1-2 - Type 1 HARQ-ACK codebook or SPS PDSCH HARQ-ACK codebook construction, e.g. discussion of Proposal 2-1, Proposal 2-2 - SPS PDSCHs overlapping handling. e.g. UE considers the limitation due to invalid symbols first and then solves the overlapping among multiple SPS PDSCHs. The solution is simple and clear, but meaningful. |
| Google | Same view as for Proposal 2-2. We support Alt 1. |
| Ericsson | Same view as for Proposal 2-2. We support Alt-1. |
| Spreadtrum | We can go with Alt 1 for progress. |
| OPPO | Same view as for Proposal 2-2. We support Alt 1. |
| Apple | Same view as for proposal 2-2 |
| NTT DOCOMO | Support Alt.1 |
| ZTE | Same view as for Proposal 2-2. We support Alt 2. In addition, we think the how to resolve the SPS dropping in the case of LB CA should also be resolved. |
| LGE | Fine with Proposal 2-3 from FL. And Alt 2 is preferred to reduce unnecessary PUCCH transmission. |
| Moderator | Based on the comments, towards the end of this meeting, it is likely Proposed conclusion 2-2 For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, there is no consensus for enhancement of HARQ-ACK codebook generation for SPS PDSCH. Discussion closed |

### SCell dormancy handling

Several companies proposed to SCell dormancy enhancement under Rel-19 low NR band carrier aggregation via switching, however, the justification is not very strong, please see the comment from the Moderator in the Table below

[Closed][R1] Proposed Conclusion 2-1

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, there is no consensus to introduce any SCell dormancy enhancement.

  - The same switching pattern should be applied regardless whether SCell is dormant or not.

| Company | Comments |
| --- | --- |
| Moderator | Below is the contribution summary Introduce SCell dormancy enhancement (2): Ofinno, Spectrum Do not introduce SCell dormancy enhancement (2): ZTE, Docomo However, even in the current specification, UE is expected to operate with limited functionalities on dormant SCell. For example, the following in TS38.321 2> not report CSI on the BWP, report CSI except aperiodic CSI for the BWP; Proposed conclusion 2-1 is a reasonable outcome. Please share your view. |
| Samsung | We should first focus on essential issues should would impact the completion of the WID. We do not need to clarify whether an enhancement is supported for every possible case. We support the proposed conclusion and this should be the default behavior any time an agreement/conclusion cannot be reached. |
| MediaTek | We prefer not to introduce enhancements for SCell dormancy as a conclusion. SCell dormancy to our understanding is not yet deployed. Hence, this topic can be deprioritized considering we only have a meeting remaining in RAN1. |
| Ofinno | In our view switching between FDD and SDL while in SCell dormancy will lead to wastage of resources which can be used for FDD carrier. But we are fine to come back to this issue when other basic issues are resolved. |
| Qualcomm | We do not think an optimization is necessary for this. We suggest the following update: - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern applies regardless of whether the dormant BWP is active on the SCell there is no consensus to introduce any SCell dormancy enhancement. - The same switching pattern should be applied regardless whether SCell is dormant or not. |
| Nokia | This may mean that SCell dormancy is useless for the SDL, but given the tight timeline for the WI, it maybe the only possibility for RAN1 to complete the WI in time. |
| vivo | We think there is no need to adopt Scell dormancy in this scenario. we prefer not to support SCell dormancy when UE is configured with switching pattern. |
| Google | No need for SCell dormancy enhancement for Rel-19 LBCA switching. The configured switching pattern should consistently apply regardless of the dormancy state of the SCell. Focus should be on essential WID objectives and timely completion. |
| Ericsson | We prefer not to introduce enhancements given the limited time. Question to Qualcomm: The wording update you suggest “whether the dormant BWP is active on the SCell,” feels slightly ambiguous. I guess your intention is to convey that for an SCell in activated state, the switching pattern is applicable regardless of whether or not a dormant BWP is active? Question to Nokia: Despite our preference not to introduce enhancements, can you clarify your statement that “SCell dormancy is useless for the SDL?” |
| Spreadtrum | We can go with proposal for progress. |
| OPPO | We prefer not to discuss any enhancement given the limited time. If conclusion is necessary, we are fine with proposal. |
| Apple | No need to consider this |
| NTT DOCOMO | Fine with proposed conclusion with Qualcomm’s update. |
| CATT | OK with the proposal. |
| Lenovo | OK. |
| ZTE | We think conclusion 2-1 is reasonable for simplicity, i.e., the same switching pattern should be applied regardless whether SCell is dormant or not. |
| LGE | OK with the proposal. |
| Moderator | Discussion closed |

## Collision handling

### Collision with switching gap

In the previous RAN1#120bis meeting, the following agreement was made with the highlight part for down-selection

Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

[Closed][R1] Proposal 3-1

For Rel-19 low NR band carrier aggregation via switching,  during a switching gap, a UE is not expected to either transmit or receive on the FDD carrier or receive on the SDL carrier, except the first UL slot corresponding to the first DL slot after SDL switching to FDD.

SDL to FDD switch

FDD to SDL switch

| Company | Comments |
| --- | --- |
| Moderator | The proposal seems to be common understand and nature outcome of the agreement made in the last RAN1 meeting. Please share your view regarding the following - Do you technically agree with the proposal - If you agree, do you think it should be a conclusion (no specification impact), or you prefer agreement (with specification impact) |
| Samsung | We think the issue should be discussed together with overlapping with non-operation period (e.g., bitmap indicating 1 for a slot of PCell or 0 for a slot of SCell) for a serving cell, a unified design is expected. The current wording is too restrictive for network scheduling, we think it should only apply to dynamic scheduled channels/signals. For semi-static channels/signals, if collision happens, UE can simply cancel the transmission/reception. In addition, as clarified for Proposal 1-2, the for Alt1 and Alt2, additional clarifications are needed for UL transmission impact. The following is our proposal Proposal 4: For low band carrier aggregation via switching, - The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell or the switching gap. - The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell or the switching gap. - Note: non-operation period of the serving cell refers to PCell slot indicated as 1 by the bitmap and SCell slot indicated as 0 by the bitmap. The following is agreed in RAN1#120b. - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. |
| Ofinno | We are generally fine with the proposal. The UE behaviour should be defined in the spec. Otherwise, the meaning of the switching gap will leave ambiguity. We prefer clearer term in the proposal. The text, “UE is not expected to receive or transmit…” leaves uncertainty in the UE behavior. We suggest to use “UE is not required to….”, instead of “expected’…as suggested in our proposal. |
| Qualcomm | Our impression of “a UE is not expected to either transmit or receive” is that the UE shall handle the conflict. It maybe good to directly discuss how/whether collision handling is specified as a UE behavior, as Samsung suggested. We tend to agree with Samsung that we can start with their Proposal 4. Although UE based resolution of conflict between semi-static pattern/gap and semi-static DL/UL is not our preference, we are OK with the Proposal if the following changes are made: Proposal 4a: For low band carrier aggregation via switching, except for UL transmission(s) during the configured switching gap for the switch from SDL carrier to FDD carrier, - The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell or the switching gap.tyh4r - For the switch from FDD carrier to SDL carrier, if the switching gap locates at the end of the last slot of the FDD carrier, the overlap between the semi-statically configured UL transmission(s) and the configured switching gap is evaluated with UL TA = 0 and is kept unchanged even when UL TA > 0 - The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell or the switching gap. - Note: non-operation period of the serving cell refers to PCell slot indicated as 1 by the bitmap and SCell slot indicated as 0 by the bitmap. Proposal 4b: For low band carrier aggregation via switching, during the configured switching gap for the switch from SDL carrier to FDD carrier, UE performs UL transmission(s) on FDD carrier - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) One related issue: The two carriers must have a certain DL receive timing difference (the max value should be discussed in RAN4). We think RAN1 should clarify that the configured switching gap is enough also to cover the DL receive timing difference. We suggest to agree following (we believe not controversial). Proposal: For Rel-19 low NR band carrier aggregation via switching, for the duration of the switching gap configured by RRC, NW ensures that the switching gap is enough to also cover the DL receive timing difference between the FDD carrier and SDL carrier - The maximum downlink receive timing difference between the FDD carrier and SDL carrier is up to RAN4 |
| MediaTek | For Proposal 3-1, we have concerns with the wording “UE is not expected to” especially for uplink transmissions. It is not clear which of the following alternatives we are agreeing to: (Let’s take the FDDSDL switch in Figure 2 as an example) - Alt 1: For FDD SDL switches, gNB shall not schedule/configure uplink resources that (partially or fully) overlap with switching gaps. Otherwise, UE is not expected to transmit the entire uplink transmission. - Alt 2: For FDDSDL switches, gNB can schedule/configure uplink resources that overlap with switching gaps. However, UE is only expected to transmit the part that does not overlap with a switching gap. In other words, UE is expected to perform partial cancellation of the scheduled/configured uplink transmission. - Note: partial cancellation is an optional UE feature We suggest segmenting the issue into the following sub-issues as a starting point. If needed, though we prefer not to, further differentiation between “dynamically scheduled” and “higher-layer configured” resources can be discussed and added on top of it. - Case 1: Downlink reception interruption - Case 2: Uplink transmission interruption - Case 2-1: SDL FDL switches - Potential impact is on the beginning symbols of an uplink slot (See slot n-1 and slot n in Figure 1 below) - Case 2-1: FDD SDL switches - Potential impact is on the ending symbols of an uplink slot (See slot n in Figure 2 below) For downlink (Case 1), UE is not expected to receive during switching gaps (i.e. TGAP in Figure 1 and Figure 2 below). Note, in Figure 1 where TGAP for SDL->FDD switches includes at least TSP and TTA, the interruption to downlink reception occurs during the entire TGAP duration (instead of just the duration of TSP). For the beginning symbols in an uplink transmission during switching gaps for SDLFDD switches (i.e. Case 2-1), the common understanding is that NW should configure a switching gap large enough to include at least TSP and TTA. In this way, NW can configure uplink transmissions from the beginning of a slot (i.e. symbol 0) and UE can transmit it without a problem. For the ending symbols in an uplink transmission during switching gaps for FDDSDL switches (i.e. Case 2-2), in our view, gNB shall not schedule/configure uplink resources overlapping with switching gaps. With the above, we propose the following: Modified Proposal 3-1: For Rel-19 low NR band carrier aggregation via switching, - UE is not expected to receive during a switching gap; - For switching from FDD to SDL, UE is not expected to be scheduled or configured with an uplink resource overlapping with a switching gap. Otherwise, UE drops/cancels the entire transmission. - Note: For switching from SDL to FDD, NW shall ensure that the switching gap is enough to cover at least the switching period and UL TA. |
| Nokia | Agree in principle, but also recognize the point MediaTek is raising. This would probably require something similar to what has been specified in 38.211 for TDD: |
| vivo | We think it is a good way to follow the legacy rules of collision handling for transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum. Samsung’s proposal somehow aligns with the current spec. The following is the relates specs. For a set of symbols of a slot that are indicated to a UE as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not receive PDCCH, PDSCH, or CSI-RS when the PDCCH, PDSCH, or CSI-RS overlaps, even partially, with the set of symbols of the slot. For a set of symbols of a slot that are indicated to a UE as downlink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS when the PUSCH, PUCCH, PRACH, or SRS overlaps, even partially, with the set of symbols of the slot. We think the non-operation period of the serving cell refers to PCell slot indicated as 1 by the bitmap and SCell slot indicated as 0 by the bitmap and the switching gap. We have the following suggestion. Proposal X For Rel-19 low NR band carrier aggregation via switching, and for a transmission/reception overlapping with (in) a slot indicated as ‘0’ on SDL carrier or a slot indicated as ‘1’ on FDD carrier by the bitmap or switching gap, reuse the collision handling rules for a transmission/reception overlapping with semi-static DL/UL symbols for unpaired spectrum - For a transmission overlapping, even partially, with the set of symbols of the slot indicated as SDL carrier or switching gap, the UE does not transmit PUSCH, PUCCH, PRACH, or SRS - For a reception on FDD carrier overlapping, even partially, with the set of symbols of the slot indicated as SDL carrier or switching gap, the UE does not receive PDCCH, PDSCH, or CSI-RS - For a reception on SDL carrier overlapping, even partially, with the set of symbols of the slot indicated as FDD carrier or switching gap, the UE does not receive PDCCH, PDSCH, or CSI-RS. |
| Google | We agree with the majority of views that during a switching gap, the UE is not expected to transmit or receive on the carriers involved in the switch. Consequently, the UE should not expect to be scheduled by the gNB with transmissions or receptions that would overlap this switching gap. Clear UE behavior should be defined, such as cancelling the conflicting transmission/reception if an overlap occurs. |
| Ericsson | We agree with MediaTek’s observation that the agreement from last meeting includes the following note which means that the gNB ensures by scheduling/configuration that there is sufficient time to switch before the first UL slot on the FDD carrier after the switch. There is no need to specify a collision rule for the first UL slot on FDD carrier after SDL FDD switch (MediaTek Case 2-1) - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) We think this point should be clarified and emphasized in Proposal 3-1 We also agree with MediaTek that if a particular collision rule is specified, it should be written as “The UE is not expected to be scheduled / configured with a signal channel that overlaps a switching gap.” We understand that this effectively introduces a gNB scheduling restriction. At least for DL, we think this is not too much of a burden. For UL, however, we think there is one exception (other than PRACH which should be discussed separately), and that is for the case of PUCCH and SRS. It is common to configure symbols at the end of the slot which will overlap the switching gap. Configuring additional PUCCH resources and additional SRS resources that would not collide just for the switching slots incurs additional complexity on gNB and UE side. Hence, we think for these signals only, a collision rule can be defined such that the UE omits transmission of symbols of PUCCH and periodic SRS that overlap a switching gap, i.e., partial dropping. In all other cases (except PRACH which should be discussed separately), we think collision can be avoided by gNB implementation. We think MediTek’s proposal is a reasonable starting point and can be updated to take into account our comments above. Regarding Samsung’s view on discussing a signal/channel overlapping with non-operation period, the only relevant case is if a UL signal/channel is configured/scheduled in a slot that overlaps a slot in which the UE is on the SDL carrier. We think this can be avoided by gNB implementation (except for RACH, which should be discussed separately). Hence, we suggest separate proposals, one for overlap with a switching gap (the focus of Proposal 3-1) and another one for the overlap with non-operation period like this: Proposal: Other than PRACH which is discussed separately, a UE is not expected to be scheduled/configured with an UL signal/channel in a slot which is indicated as ‘1’ by the semi-statically configured switching pattern. |
| Spreadtrum | To make UE behavior clearly, we suggest the red words below. For Rel-19 low NR band carrier aggregation via switching, during a switching gap, a UE is not expected to be scheduled/configured to transmit or receive on the FDD carrier or receive on the SDL carrier, except the first UL slot corresponding to the first DL slot after SDL switching to FDD. |
| OPPO | In my understanding, proposal consists of two points to handle switch gap issue: - Scheduling restriction to avoid collision with switching gap. We prefer to MediaTek’s wording “UE is not expected to be scheduled or configured with an uplink resource overlapping with a switching gap” due to it describes restriction for gNB’s scheduling rather than UE behavior, which means it allows UE to transmit in switching gap taking UL TA into account. - Allow UE to transmit in switching gap taking UL TA into account. UE needs to transmit in switching gap due to UL TA but it does not mean UE can transmit in any time during switching gap due to UE has to interrupt transmission during RF switching. So restriction on UE behavior during switching period is necessary. This proposal does not only handle UL TA issue but also DL timing difference issue. So we propose that UE does not expect to transmit and receive in switch-to carrier earlier than switching period after the starting of switching gap. Regarding to whether unified solution for collision with switching gap and collision with switching pattern is applied or not, we are open. Our first preference is to decouple two issues due to collision probability is completely different. Collision with switching gap can be avoided, even for semi-static configuration, due to small duration of switching gap. But collision with switching pattern is not easy to be avoided for semi-static configuration due to long duration of switching pattern. |
| Apple | In principle, key point of discussion here is whether UE behavior needs to be specified or not. In our view, gNB scheduling should avoid such collision and consequently UE is not expect to handle any such issue. Now from spec perspective, we would prefer working that talks specifically in terms of what UE is expected to be scheduled/configured with, rather than UE is not expected to transmit/receive. Basically, the latter wording could mean that gNB could still schedule, but UE then might need some dropping, which we want to avoid. So we prefer to explicitly say that “UE is not expected to be scheduled/configured”. So, updated proposal could be: [Open][R1] Proposal 3-1 For Rel-19 low NR band carrier aggregation via switching, during a switching gap, a UE is not expected to scheduled/configured by network to either transmit or receive on the FDD carrier or receive on the SDL carrier, except the first UL slot corresponding to the first DL slot after SDL switching to FDD. |
| NTT DOCOMO | We think that both switching gaps and invalid periods based on the switching pattern can be handled in the same manner. We are fine with Samsung’s proposal 4. |
| ZTE | First, for the change from ‘a UE is not expected to either transmit or receive’ to ‘a UE is not expected to scheduled/configured’ introduce the gNB scheduling restriction, From our understanding, ‘a UE is not expected to either transmit or receive’ already implies it should be the network to avoid the collision, thus it is not necessary for the latter wording. For Proposal 3-1, we have concerns with the wording ‘except the first UL slot corresponding to the first DL slot after SDL switching to FDD.’ We think it shall be a note based on the agreement of the last meeting: ‘RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. Secondly, we have comments about the a unified design of the switching gap and non-operation period. We think the collision handling of the switching gap and non-operation period may be different. The reason is that the switching gap only occupy 1, 2 symbols which is much more easier for the network to avoid the collision. However, the non-operation period may be multi-slot level, thus it is difficult for the network to ensure there is no collision at all, especially for the semi-statically configured signal/channel. |
| Samsung2 | As also mentioned by companies, the collision issue for LB-CA switching is practically same as the collision with semi-static DL/UL symbols. There is no reason to use different rules, and have different designs and inconsistent specifications. There is nothing different from the legacy case when a PDSCH overlaps with semi-static UL symbols. The same rule also already applies for the similar case of cell DTX. Therefore, we think that Alt 2 with our update should also apply for LB-CA. |
| Moderator | Based on the feedback, there are two questions to discuss - Whether collision with switching gap, and FDD/SDL period are designed differently - How to have unified solution, and which cases are avoided by NW scheduled (UE is not expected), which cases are avoided by UE (UE performs dropping) Discussion closed |

### Collision with RACH operation

In terms of the need for the UE to perform switching in addition to the RRC configured semi-static switching pattern, based on the discussion in the last RAN1 meeting, only possible exception is RACH procedure.

[Closed][R1] Proposal 3-2

For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, the UE is not expected to perform additional switching not following the semi-static switching pattern, except the only candidate exceptional scenario, i.e., PRACH transmission

- Regarding PRACH transmission, downselect one of the following

  - Alt 1: Does not support UE to perform additional switching due to PRACH transmission

    - For a UE in RRC_CONNECTED, if a PRACH occasion occurs in a slot for which the switching pattern indicates the SDL carrier, the UE shall not transmit PRACH.

      - Alt 1.1: The definition of valid PRACH occasions in current specifications is not changed.

      - Alt 1.2: The corresponding PRACH occasion is considered as invalid

  - Alt 2: UE performs additional switching for PRACH transmission

    - UE resumes the semi-statically configured switching pattern after the RACH procedure completes

      - FFS: how to define that the RACH procedure completes

| Company | Comments |
| --- | --- |
| Moderator | Below is the summary of the companies’ proposals Alt 1 (6): ZTE, LG (Alt 1.2), Ericsson (Alt 1.1), Apple (Alt 1.2), Qualcomm, vivo Alt 2 (7): Huawei, Ofinno, OPPO, MediaTek, DOCOMO, Google, CATT However, without consensus, Alt 1.1 is the nature outcome. Please provide your feedback - When you see a strong need for Alt 2. - If Alt 2 is supported, the details, for example in terms of - How to define when RACH procedure completes, or how and when UE resumes the semi-static switching pattern - Which RACH procedure to support, CFRA, CBRA, PDCCH-order PRACH, etc. - Other issues like time line |
| Samsung | We suggest to deprioritize the discussion for this issue for the similar reason as we commented for Proposed Conclusion 2-1. Alt. 1.1 works. The NW can avoid potential problems caused by PRACH transmissions, without material restrictions, because the NW anyway controls the slots with ROs and the RAR window is up to 10 msec. |
| MediaTek | We support the main bullet Alt 2. The RACH procedure can several 10s of msecs long (e.g. the typical RAR window configured by BS is 10msec). For CBRA, gNB cannot know which UE it is and what switching pattern the UE has been configured with. It will be difficult for gNB to do some smart scheduling. For the sub-bullet in Alt 2, when RACH is complete, we would like to hear more views (pros and cons) on whether UE should autonomously resume the switching pattern or it should wait for gNB’s reconfiguration. |
| Ofinno | We support Alt 2. We agree with MediaTek that the RACH procedure can be quite long. It is not always feasible/possible for the network to avoid RACH RO/RARs etc. |
| Qualcomm | We support Alt.1, and do not support Alt.2. We believe at least this is not RAN1’s task as per WI objectives below. If performance/delay of (RACH, HO, BLF, etc) procedure is a concern, the issue should be raised in RAN4. - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4]. |
| Nokia | Agree with MediaTek. For PRACH the UE should suspend the switching operation. It is not possible to configure the FDD cell’s PRACH to match individual UEs switching patterns, and it does not make much sense to restrict the switching patterns to all match to the cell-common PRACH timing. Alt.2 can be understood to facilitate the suspension of switching, but it falls somewhat short on describing the behaviour after the UE has sent the PRACH preamble. |
| vivo | We support alt 1-1. It is the simplest way to finish this WI in short time. |
| Google | We support Alt-2 for the UE to perform additional switching for PRACH transmission. The UE should temporarily switch to FDD for RACH as needed, and then seamlessly resume its RRC-configured switching pattern. However, we want to extend this proposal to other high priority events (e.g., handover, RLF on PCell, LTM, …). |
| Ericsson | We support Alt 1-1 and we do not support Alt 1-2 or Alt-2. As tasked by the WID, RAN1 shall specify a semi-static switching pattern, this means that the UE should not autonomously switch carriers. It is preferrable to have common understanding between network and UE on when switching occurs. At the same time, we understand that for typical cell-common RO configurations, it will be problematic for the network to configure switch patterns to force switch back to PCell for all UEs during all ROs. This will result in inefficient use of the SDL carrier, particularly for short PRACH configuration periods, e.g., 10 ms. Alt 1-1 solves this problem by the UE simply not transmitting PRACH in an RO if that RO is in a slot where the switching pattern indicates a ‘1’ for the slot (i.e., SDL carrier). Of course, the NW should still take care to ensure that a UE configured with a switching pattern will switch back to the PCell during at least 1 or more ROs during the 40 ms period. Regarding MediaTek’s question about CBRA, we would like to hear in what situations this is typically used for UEs in CONNECTED mode compared vs. CFRA. We agree with Qualcomm, that it is not up to RAN1 to specify UE behavior and switching pattern applicability, e.g., during RLF and HO. We disagree with Alt 1-2 since invalidating an RO affects the SSB to RO association, and this may be complicated from a gNB, UE, and spec perspective. |
| Spreadtrum | Alt 1-2 cannot work, RO-SSB mapping is cell specific, not UE specific. |
| OPPO | We support RACH procedure prioritize switching pattern. there are two independent issues to handle: - Whether PRACH transmission triggers switching. This point focuses on the 1st step of RACH procedure. Either Alt1 or Alt 2 is fine for us and slightly prefer to Alt 1. Alt 1 avoids dynamic switching and Alt 2 performs switching as soon as possible. - After PRACH transmission, whether to maintain in FDD carrier until RACH complete. Regardless we go to Alt 1 or Alt 2, we prefer to maintain in FDD carrier until RACH complete. When UE triggers RACH, it usually means transmission hardly can perform normally. So following switching pattern could not provide more efficient transmission but also delay RACH procedure. So, we suggest to formula proposal to decouple two issues as following: For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, the UE is not expected to perform additional switching not following the semi-static switching pattern, except the only candidate exceptional scenario, i.e., PRACH transmission - Regarding PRACH transmission, downselect one of the following - Alt 1: Does not support UE to perform additional switching due to PRACH transmission - For a UE in RRC_CONNECTED, if a PRACH occasion occurs in a slot for which the switching pattern indicates the SDL carrier, the UE shall not transmit PRACH. - Alt 1.1: The definition of valid PRACH occasions in current specifications is not changed. - Alt 1.2: The corresponding PRACH occasion is considered as invalid - Alt 2: UE performs additional switching for PRACH transmission - UE resumes the semi-statically configured switching pattern after the RACH procedure completes - FFS: how to define that the RACH procedure completes Regarding FFS on definition of RACH complete, we suggest that Msg 3 ending as RACH complete for CFRA and PUCCH ending for HARQ_ACK for Msg 4 as RACH complete for CBRA. |
| Apple | We prefer UE not to be triggered to switch as a result of PRACH transmission occasions. So we support Alt 1 in general. Now regarding the two options, our preference would be to adopt Alt 1-2, as for TDD configuration. However, we would be okay also to accept Alt 1-1, for sake of progress |
| NTT DOCOMO | We prefer Alt.1-1 based on the WID principle. |
| CATT | We support Alt 2, since it is possible that the random access procedure falls within the SDL carrier period. To avoid affecting the initial access process, it is better to support Alt2. |
| ZTE | Both Alt 1.1 and Alt 1.2 are fine for us, we do not see the strong motivation for Alt2 (i.e.,UE perform additional switching for RACH. For CBRA). Although gNB cannot know which UE it is and what switching pattern the UE has been configured with, if the PCell duration is flexible enough, e.g., in case the bitmap of switching pattern is ‘0001100011...’, even the legacy UE follow the pattern of the R19 UE for the RACH procedure scheduling, the time delay of msg2, msg 3, msg 4 shall not be influenced. |
| LGE | We support Alt 1, as other companies commented in above. In our view, Alt 1-2 is preferrable since FDD UL carrier with switching can be treated similarly with TDD carrier, but we can also live with Alt 1-1. |
| Moderator | Unfortunately, very strong view on how to specify enhancement on RACH (Samsung, Qualcomm, vivo, Ericsson, Apple, DCM, ZTE LGE) Will revisit this issue after collision handling 4.3.1 and 4.3.3 Discussion closed |

### Other Collisions

Below is the summary of the proposals based on the contributions

| R1-2503331 ZTE | TBoMS |
| --- | --- |
| R1-2503384 vivo | CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI', timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured" Periodic SRS, or semi-persistent SRS |
| R1-2503532 Spreadtrum | semi-static signal/channel |
| R1-2503588 Samsung | semi-static signal/channel |
| R1-2503739 Ofinno | Periodic and semi-persistent CSI-RS SSB Periodic and semi-persistent SRS Semi-static PDCCH SPS-PDSCH CG-PUSCH (Type-1/2) SR on PUCCH |
| R1-2504255 LG | SSB, CSS, CSI-RS for RRM/RLM/BFR |
| R1-2504520 DOCOMO | The collision between part of occasions for configured transmission/reception or scheduled transmission/reception on multiple occasions and unavailable resource according to the semi-static switching pattern should be allowed, and only transmission/reception on the occasion colliding with unavailable re-source should be dropped. |

[Closed][R1] Question 3-1

| Company | Comments |
| --- | --- |
| Moderator | It is still unclear one which cases we can handle in this meeting, and other collisions can be considered as lower priority. For your feedback, please provide - More concrete proposal and the UE behavior, i.e., partial dropping or full dropping |
| Samsung | As we commented for Proposal 3-1, a unified solution is expected for this case and switching gap |
| Ofinno | As a general principle, the UE drops any semi-statically configured signal/channel during slots in a serving colliding with the slots of the other serving cell. One exception is SMTC configuration in the PCell whose collision should be avoided with the slots of the SCelll by the network as described in our paper. |
| Qualcomm | Please see our comment on Proposal 3-1. |
| Ericsson | Please see our comment on Proposal 3-1. |
| Apple | Please see our input to Proposal 3-1 |
| NTT DOCOMO | We share same view with Samsung. |
| ZTE | We think transmission with repetition or TBoMS should be considered as exception. It should be noted the collision between such transmission and the TDD slot format is allowed. In the case of collision, the current UE behavior should be followed, i.e., dropping for repetition type A and postponing for repetition type B. |
| Moderator | Based on the feedback, there are two questions to discuss - Whether collision with switching gap, and FDD/SDL period are designed differently - How to have unified solution, and which cases are avoided by NW scheduled (UE is not expected), which cases are avoided by UE (UE performs dropping) Discussion closed |

## LS to RAN4 and RAN2

[Open][R1] Question 4-1

| Company | Comments |
| --- | --- |
| Moderator | Based on the RAN4 workplan (R4-2500089), RAN1 needs to send LS to RAN4 and RAN2 Please provide your feedback - Are you okay to follow RAN4 workplan to send LS to RAN4 and RAN2 including all RAN1 agreement at the end of this meeting - Your comments about the draft LS in - ( https://www.3gpp.org/ftp/tsg_ran/wg1_rl1/TSGR1_121/Inbox/drafts/9.12.2(NR_LBCA_Sw)/Draft%20LS ) |
| Samsung | Fine |
| Ofinno | Fine with the proposal to send LS to RAN2 and RAN4. |
| Qualcomm | Yes. |
| Ericsson | Yes |
| Apple | Yes |
| NTT DOCOMO | Fine with the proposal |
| ZTE | OK |
| LGE | Fine with the proposal |

## Others

If you have other issues that you believe that need to be discussed, please provide those issues in the following table

[Open][R1] Question 5-1

| Company | Comments |
| --- | --- |
| Moderator | Moderator’s proposals above may not cover all the proposals from contributions, especially the ones that are proposed by one or very few companies, and can be considered as lower priority. Considering this is the last RAN1 meeting before RAN1 Rel-19 functional freeze, please add essential issues that you want to discuss below (due to the time limitation, we still may not be able to handle those, hope you can understand) |
| MediaTek | We think the presence/availability of important RS, at least SSB, should be discussed. After all, once UE is configured with a switching pattern, whether and how often UE can “see” SSBs is different from the case without any switching pattern. We have the following two proposals related to SSB availability: - SSB-based SCell operations is assumed as baseline for this WI. - For an RRC-configured semi-static switching pattern, gNB shall guarantee SSB availability on both carriers at least once every X1 and X2 msec on the FDD carrier and SDL carrier, respectively, with X1 <=min(160, 2*SSB period on the FDD carrier) and X2<=min(160, 2*SSB period on the SDL carrier). |
| Ofinno | We would like to discuss the following two issues: - Clarify the UE behavior on SDL carrier 2 (or FDD carrier 1) during the BWP switching on FDD carrier 1 (or SDL carrier2). While the BWP switching is ongoing on a serving cell, the UE may have to switch to the other carrier (i.e., due to semi-static pattern). It is unclear what will be the UE behaviour i.e., whether the UE completes the BWP switching without switching or abandon the BWP switching. - Discuss CSI feedback, reference resource definition (clause 5.2.2.5, 38.214) in consideration of the semi-static switching pattern. When the semi-static pattern is configured, a CSI reference resource for a serving cell may fall in a slot that is assigned to the other serving cell. The meaning of the valid downlink slot as CSI reference resource needs to be clarified. For example, the valid downlink slot is the one that does not overlap with slots allocated/assigned to the other serving cell. |
| Qualcomm | As we commented on Proposal 3-1, we think it is necessary to conclude following and capture this in the LS to be sent to RAN4. Proposal: - For Rel-19 low NR band carrier aggregation via switching, for the duration of the switching gap configured by RRC, NW ensures that the switching gap is enough to also cover the DL receive timing difference between the FDD carrier and SDL carrier - The maximum downlink receive timing difference between the FDD carrier and SDL carrier is up to RAN4 |
| Google | We would like to discuss the following aspects as well: - The overlapping between the measurement gaps and the switching gaps: whether the UE should prioritize the switching gap or the measurement gap and what about partial overlapping. - Co-existence between DRX and the carriers switching pattern. - For RO validation, whether to determine that an RO in the slot configured for the SDL carrier as invalid RO, where the slot may be the UL slot for the FDD carrier, or consider it as valid RO and perform the RO validation based on the UL/DL configuration for the FDD carrier, then when the UE determines to transmit the PRACH on such RO, the UE drops the PRACH transmission, |
| Ericsson | We agree with the above proposal by Qualcomm to also include the MRTD (Maximum Receive Timing Difference) between FDD and SDL carriers into the switching gap in addition to TA. RAN4. |
| Apple | Fine with Qualcomm’s proposal |
| NTT DOCOMO | We have similar view with MTK that the availability of SSB is an important aspect and should be discussed. |
| LGE | We think the following aspects need to be clarified: - how to handle SR transmission counter/prohibit timer in case where UE is on SDL carrier during the SR PUCCH resource/occasion is on FDD carrier - how to handle available slot counting for PUSCH repetition in case where UE is on SDL carrier during the PUSCH resource/occasion is on FDD carrier |

# (Closed Discussion) Low band carrier aggregation via switching

# Summary of contribution

## R1-2503273 Huawei, HiSilicon

- For switching from FDD carrier to SDL carrier, support Alt 2: NW configures whether the switching gap is at the FDD slot or SDL slot. - For the semi-static switching pattern, maximum 2 switches within 1 slot. - Note: this does not violate the slot-level granularity of the switching pattern. - Maximum time duration on one carrier should be considered in order to minimize its impacts on UE measurement/synchronization, which reply on SSB and CSI-RS receptions, and HARQ-ACK/NACK feedback on PUCCH. - For R19 low band switching, if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully done.

## R1-2503331 ZTE

Proposal 1: The location of switching gap for switching from FDD carrier to SDL carrier should be configurable. Proposal 2: For the restriction of maximum X switch(es) within Y slot(s), X should be greater than 16 for Y=40. Proposal 3: There should be no other restrictions for switching pattern configuration. Proposal 4: There should only one TDM pattern for LB CA in Rel-19. Proposal 5: There should be no specification impact for the switching gap duration. Proposal 6: The additional processing time should not be needed for LB CA. Proposal 7: It should be the NW implementation to avoid the collision, except for repetition or TBoMS, in which UE is expected to drop or postpone the corresponding reception and/or transmission. Proposal 8: The same switching pattern should be applied to the dormant SCell and activated SCell. Proposal 9: Introduce the following RRC parameters for low band carrier aggregation via switching. RAN2 Parent IE; Parameter name in the spec; New or existing?; Default value aspect Description; Value range; Per (UE, cell, TRP, …); UE-specific or Cell-specific. PhysicalCellGroupConfig; LBCA-CarrierSwitchingPattern-r19; New; A bitmap with each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2.; SEQUENCE (SIZE(1..40)) OF INTEGER (0..1); Per CG; UE-specific. PhysicalCellGroupConfig; switchinggapPCelltoSCell_r19; New; Indicates the value of switching period for the switching from PCell to SCell.; FFS; Per CG; UE-specific. PhysicalCellGroupConfig; switchingGapSCelltoPCell_r19; New; Indicates the value of switching period for the from SCell to PCell; FFS; Per CG; UE-specific.

## R1-2503384 vivo

Proposal 1: For a SCell in a SDL carrier in R19 LB CA via switching, the PDCCH monitoring occasions should be configured in SDL carrier to support self-scheduling. Proposal 2: When a switching pattern for LB CA via switching is configured, the SDL carrier cannot be configured with a dormant BWP. Proposal 3: The configured time switching pattern shall guarantee that UE does not stay continuously on SDL carrier for longer than 15 slots. Proposal 4: For the restriction of maximum X switch(es) within Y slot(s), at least support Y=5, X=1. Proposal 5: UE is not expected to be scheduled uplink transmission in the switching gap duration of the last uplink slot before switching. Proposal 6: Support alt1: switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. Proposal 7：For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, RAN1 does not support the UE to perform additional switching not following the semi-static switching pattern Proposal 8: For DL/UL transmission on a serving cell, the symbols in invalid periods can be treated as the symbols semi-statically configured as TDD UL/DL symbols and similar UE behaviors can be defined. Proposal 9: For low band carrier aggregation via switching, UE handles the overlapping among SPS PDSCHs after resolving overlapping with symbols in the slot indicated as invalid periods by semi-statically configured switching pattern including the switching gap. Proposal 10: For periodic CSI-RS and semi-persistent CSI-RS on the serving cell configured in CSI report configuration in CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI', - the UE is not expected to receive the periodic CSI-RS and semi-persistent CSI-RS during the invalid periods of the serving cell based on the configuration of switching pattern. - The most recent CSI measurement occasion of semi-persistent CSI-RS resource or periodic CSI-RS resource on the serving cell occurs in valid time of the serving cell for CSI report configured by CSI-ReportConfig associated with the higher layer parameter reportQuantity comprising at least 'RI'. - the UE reports a CSI report only if receiving at least one CSI-RS transmission occasion of each periodic CSI-RS resource or semi-persistent CSI-RS resource on a serving cell in valid time of the serving cell, no later than CSI reference resource, and the UE drops the CSI report otherwise. Proposal 11: If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", - the UE shall derive the channel measurements for computing CSI reported in uplink slot n based on only the most recent, no later than the CSI reference resource, in cell DTX active time of a serving cell if cell DTX is activated, valid time of the carrier of the serving cell if a switching pattern for the serving cell, occasion of NZP CSI-RS associated with the CSI resource setting on the serving cell. - the UE shall derive the interference measurements for computing the CSI value reported in uplink slot n based on the most recent, no later than the CSI reference resource, in cell DTX active time of a serving cell if cell DTX is activated, valid time of the carrier of the serving cell if a switching pattern for the serving cell, occasion of CSI-IM and/or NZP CSI-RS for interference measurement associated with the CSI resource setting on the serving cell. Proposal 12: For HARQ-ACK codebook generation of SPS PDSCHs, UE will not generate HARQ-ACK information for SPS PDSCH overlapping with symbols indicated as invalid periods of the serving cell based on semi-statically configured switching pattern including the switching gap. Proposal 13: For type 1 HARQ-ACK codebook generation, a row of the TDRA table overlapping with the symbols in invalid periods of the serving cell is excluded when determining the set of occasions of candidate PDSCH receptions. Proposal 14: During invalid periods of the serving cell, the UE configured with LB CA via switching is not expected to transmit the periodic SRS, or semi-persistent SRS for channel acquisition. Proposal 15: UE performs UL multiplexing of PUCCH/PUSCH before considering limitations for UE transmission due to LB CA via switching.

## R1-2503421 NEC

Proposal 1: Within switching gap from SDL to FDD in SDL slot, UE should be able to perform FDD UL transmissions from the first symbol of FDD UL slot after switching from SDL carrier to FDD carrier. Proposal 2: For switching from FDD carrier to SDL carrier, select Alt 3 “Switching gap is always assumed at the SDL carrier”. Proposal 3: Support activation of semi-static FDD/SDL switching pattern upon RRC configuration. Proposal 4: Support dynamic activation of semi-static FDD/SDL switching pattern after RRC configuration. Proposal 5: DCI may include an indicator to update to one of the Special Slot configurations. Proposal 6: DCI can support cross-carrier scheduling with SDL carrier indication. Proposal 7: DCI which schedules SDL carrier from FDD carrier may support multi-slot and multi-carrier scheduling with common scheduling information.

## R1-2503430 Nokia

- The maximum switching frequency is 2 switches (one FDDSDL switch and one SDLFDD switch) over 4 slots, i.e. Restriction of maximum X=2 switch(es) within Y=4 slot(s). - For switching from FDD carrier to SDL carrier, Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - The DL reception after the switch starts at symbol#0 of the slot - The UL transmission on the FDD uplink can start at symbol#0 of the FDD UL slot after the switch - Different switching gap lengths can be configured for the FDD SDL switch and for the SDL FDD switch. - PDSCH scheduling in FDD is managed by the PDCCH in FDD, while PDSCH scheduling in SDL is managed by the PDCCH in SDL. - PUSCH scheduling in FDD is managed by the PDCCH in FDD. - When the SDL carrier is deactivated, the FDD cell should resume ‘normal’ operations, i.e. not apply any switching, and be able to receive and transmit continuously on the FDD carrier. - When Cell DRX operates on the SDL carrier, the SDL carrier reception should be stopped completely and only reactivated based on traffic on the FDD carrier. - When Cell DTX is activated on the SDL carrier, the FDD cell should resume ‘normal’ operations, i.e. not apply any switching, and be able to receive and transmit continuously on the FDD carrier.

## R1-2503532 Spreadtrum

- For Rel-19 low NR band carrier aggregation via switching, support only one semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between PCell and SCell. - At most one switching from PCell to SCell exits in 5ms. - Contiguous slots of SCell cannot be exceed 8 slots in a period. - At least one SSB is available at the UE during every 160 ms for both of PCell and SCell if applicable - For switching from FDD carrier to SDL carrier, switching gap can locate at the “switch from” carrier, i.e., FDD carrier. - For the specification impact of UL TA, UE does not expect the UL dropping due to overlapping with the switching period. - The switching pattern for SDL SCell is not applied if SCell is in dormancy BWP - For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for Type-I HARQ-ACK codebook generation - A row of the TDRA table is excluded from the determination of the set of occasions for candidate PDSCH receptions if the PDSCH associated with the row is not to be received by the UE due to LB-CA switching. - For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for HARQ-ACK codebook generation - HARQ-ACK bit for a SPS PDSCH is not included in the HARQ-ACK codebook if the SPS PDSCH is not to be received by the UE due to LB-CA switching. - For Rel-19 low NR band carrier aggregation via switching, when NW configures a semi-static switching pattern to a UE, RAN1 does not consider the UE perform additional switching not following the semi-static switching pattern - FFS: whether and, if needed, how to support the only candidate exceptional scenario, i.e., CB RACH - For low band carrier aggregation via switching, semi-static pattern indicates UE to operate (receive or transmit) on only one serving cell. - The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell. - The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell.

## R1-2503588 Samsung

Observation 1: For low band carrier aggregation via switching, if the number of continuous slots on SCell is larger than 7, HARQ-ACK of a PDSCH reception in the last slot on PCell cannot be provided due to the K1 value restriction. Observation 2: Alt 1 (Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier) would impact on UL transmissions on PCell as well as UCI multiplexing procedure. Proposal 1: Regarding the restriction of maximum X switch(es) within Y slot(s), support Y = 40 and X = 8. Proposal 2: For low band carrier aggregation via switching, the maximum of continuous slots on SCell is 7. Proposal 3: For switching from FDD carrier to SDL carrier, support Alt 3, i.e., Switching gap is always assumed at the SDL carrier.) Proposal 4: For low band carrier aggregation via switching, - The UE does not receive or transmit a semi-static signal/channel on a serving cell if the semi-static signal/channel overlaps with a non-operation period of the serving cell or the switching gap. - The UE does not expect to be scheduled with a transmission/reception on a serving cell by a DCI format and the transmission/reception overlaps with a non-operation period of the serving cell or the switching gap. Proposal 5: For low band carrier aggregation via switching, additional processing time should be introduced for the following cases if carrier switching happens between a DL reception and a UL transmission, - a PDCCH reception schedules a PUSCH transmission - a PDCCH reception indicates SRS transmission - a RAR schedules a PUSCH transmission - a PUCCH transmission with HARQ-ACK for a PDSCH or PDCCH reception Proposal 6: For low band carrier aggregation via switching, - If a UE is provided with Type-1 HARQ-ACK codebook, and a candidate PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the candidate PDSCH reception is excluded from the set of occasions for candidate PDSCH receptions. - If a SPS PDSCH reception on a serving cell overlaps with a non-operation period of the serving cell, the UE does not receive the SPS PDSCH and does not transmit HARQ-ACK for the SPS PDSCH reception.

## R1-2503657 Lenovo

Observation 1: The carrier switching pattern should be designed to guarantee DL transmission occasions for SSBs and reference signals on one carrier different from those on another carrier to avoid losing the synchronization on two carriers. Observation 2: The carrier switching pattern should be designed to avoid frequent switching as much as possible. Observation 3: The carrier switching pattern should be designed to guarantee the HARQ-ACK feedback corresponding to PDSCHs on DL-only carrier can be transmitted on the paired carrier. Observation 4: Rel-16 HARQ-ACK suspension and one-shot HARQ-ACK request can simplify the carrier switching pattern design to avoid frequent carrier switching for transmitting HARQ-ACK feedback for PDSCHs on Cell 2. Proposal 1: For NR low band CA via switching, the semi-static carrier switching pattern can be designed to have maximum 2 switches within 10 slots (i.e., one radio frame) subject to UE capability. Proposal 2: For NR low band CA via switching, for switching from FDD carrier to SDL carrier, if the time domain resource allocation of the ongoing DL transmission on the FDD carrier does not include the last symbol, or UE needs to monitor the search space from the first symbol of the SDL carrier, then the switching gap is located on the FDD carrier; otherwise, the switching gap is located on the SDL carrier. Proposal 3: UE does not expect the case where the time domain resource allocation of the PDSCH transmission on the FDD carrier includes the last symbol and UE needs to monitor the search space from the first symbol of the SDL carrier. Proposal 4: For Type-1 HARQ-ACK codebook, exclude candidate PDSCH reception occasions from a cell if UE switches to another cell. Proposal 5: For HARQ-ACK feedback timing, support more than 8 values in the K1 set with new values larger than 15.

## R1-2503739 Ofinno

Proposal 1: In terms of restriction, at least X = 1 and Y = 2 is supported. X = 1 and Y = M (5≥M≥2) can be considered. Proposal 2: Network ensures the semi-static switching pattern to satisfy a) maximum consecutive slots assigned to the SDL carrier 2 does not exceed 16 msec; b) at least one SMTC occasion per 40 msec is allocated to the FDD carrier 1 and SDL carrier 2 respectively. Proposal 3: Alt 2 could be more flexible to handle different impact from Alt 1 or Alt 3. If we agree on Alt 2 or Alt 3, network ensures a slot has maximum one switching gap (e.g., by configuring a minimum number of two consecutive slots to SDL carrier). Proposal 4: During a switching gap, a UE is not required to either transmit or receive on the FDD carrier or receive on the SDL carrier. The UE drops a downlink channel/signal or uplink channel/signal if it overlaps with the switching gap. Proposal 5: RAN1 specification should specify the duration of the switching gap switching from SDL carrier to FDD carrier as the sum of Z1 symbols and UL TA, where Z1 is 1 or 2 symbols depending on the UE capability. Proposal 6: Discussion related to the applicability of the semi-static switching pattern during the SCell deactivation is left for RAN4. Proposal 7: The UE starts to apply the semi-static switching pattern at least one slot after transmitting a valid CSI report for the activated SCell (i.e. SDL carrier). Proposal 8: When the SCell (SDL carrier) is put to dormant then the UE operates on PCell (FDD) carrier on both DL and UL without any switching to SCell (SDL carrier). Proposal 9: Do not modify the existing UE procedure on Type-I HARQ-ACK codebook generation when the UE is configured with a semi-static switching pattern. Proposal 10: For Rel-19 low NR band carrier aggregation via switching, the determination of the PDSCH processing time is further based on the switching time from Case 2 (with PDSCH reception) to Case 1 (with HARQ-ACK transmission). Proposal 11: The UE configured with semi-static switching pattern drops the following reception and/or transmission of semi-statically configured reference signals and channels under the collision: - Periodic and semi-persistent CSI-RS - SSB - Periodic and semi-persistent SRS - Semi-static PDCCH - SPS-PDSCH - CG-PUSCH (Type-1/2) - SR on PUCCH Proposal 12: For the UE configured with semi-static switching, the network can ensure that the SMTC duration of the SMTC of a serving cell (e.g., PCell) does not collide/overlap with the time slots of the other serving cell (e.g., SCell). Proposal 13: The UE does not apply the semi-static switching pattern and stays on the PCell (FDD carrier) at least during there is on-going random access procedure on PCell. Proposal 14: Additionally, - For a RACH procedure triggered by a PDCCH order, the UE does not apply the semi-static switching pattern starting from a slot in which the UE has received the PDCCH order for the preamble transmission. - For a contention-based RACH procedure, the UE starts to re-apply the semi-static switching pattern after a slot in which the UE transmits the PUCCH (containing an ACK) for a Msg 4 PDSCH. Proposal 15: Clarify the UE behavior on SDL carrier 2 (or FDD carrier 1) during the BWP switching on FDD carrier 1 (or SDL carrier2). Proposal 16: Discuss CSI feedback, reference resource definition in consideration of the switching pattern.

## R1-2503780 CATT

Proposal 1: For restriction of maximum X switch(es) within Y slot(s), the following combinations can be considered - {X =2, Y=10ms} - {X =3, or 4, Y=20ms} - {X =6, or 7,or 8, Y=40ms} Proposal 2: For switching from FDD carrier to SDL carrier, the Alt 2 is preferred as follows - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier Proposal 3: For switching from SDL carrier to FDD carrier and the switching gap is located on SDL carrier, both SDL and FDD carrier shall omit the reception/transmission duration of the symbols of the switching gap. Proposal 4: For switching from FDD carrier to SDL carrier and the switching gap is located on FDD carrier. - FDD carrier will omit reception duration of the symbols for switching gap. - FDD carrier will omit transmission following symbols at the end of last slot for FDD carrier transmission - Max(0,(GAP_during-TA)/dur_OS). - The GAP during is configured by RRC, The TA value can be configured by gNB to avoid ambiguity。 - dur_OS is duration of one symbol for SCS=15KHz Proposal 5: For switching from FDD carrier to SDL carrier and the switching gap is located on SDL carrier, both SDL and FDD carrier will omitted reception/transmission duration of the symbols for switching gap. Proposal 6: For the UCIs other than the HARQ-ACK feedback, i.e. SR and CSI report, for the resource of PUCCH can be up to gNB configuration, ensuring that it falls within the period of case 1. Proposal 7: When the UE is in case 2, it can suspend the switching pattern configuration and switch to case 1 to start sending preamble, and then resume the configuration until the random access procedure is completed. Proposal 8: For the Type-1 HARQ-ACK codebook, the occasions for candidate PDSCH reception should be pruning based on the LBCA switching pattern for each of carrier.

## R1-2504153 ETRI

Proposal 1: The switching gap for FDD to SDL transition can be configured at the switch-from carrier (i.e., FDD). Proposal 2: Switching between FDD and SDL carriers may be configured to ensure that each carrier duration continues for a reasonable number of consecutive slots. Proposal 3: Collisions between the switching gap and physical signals or channels can be avoided by the network through scheduling. The UE may drop a signal only if it overlaps with the switching gap and belongs to a predefined list of periodic or semi-static resources

## R1-2504217 OPPO

Observation 1: The overlapping between carrier unavailability duration and a channel/signal not involving in RACH procedure can be avoided by proper switching pattern configuration and channel/signal resource allocation. Observation 2: There are two solutions to handle PRACH occasion in FDD-carrier unavailability duration. - Solution 1: PRACH occasion is invalid when FDD-carrier is indicated as unavailable carrier by switching pattern. In other words, UE transmits PRACH only when FDD carrier is indicated as available carrier by switching pattern. - Solution 2: PRACH occasion is still valid even when FDD-carrier is indicated as unavailable carrier by switching pattern. When UE needs to transmit PRACH, UE can autonomously switch to FDD carrier to transmit PRACH on PRACH occasion. Proposal 1: UE does not expect both of the following in switching pattern configuration. - More than two switching within one frame; and - More than one switching within one subframe. Proposal 2: For switching from FDD carrier to SDL carrier, NW configures whether the switching gap is on the FDD carrier or SDL carrier. Proposal 3: UE does not expect to transmit and receive in switch-to carrier earlier than switching period after the starting of switching gap. - No further specification impact is introduced for the relation to TA. Proposal 4: RAN1 chooses from following for handling of collision between switching gap and physical channels/signals. - Option-1: UE does not expect a physical channel/signal occasion to be overlapping with a switching gap. - Option-2: UE cancels the transmission/reception for a physical channel/signal occasion that overlaps with a switching gap, where the physical channel/signal occasion is the outcome of legacy channel multiplexing/prioritization procedure. Proposal 5: For a RACH procedure on FDD carrier, - Once a UE transmits PRACH, the UE stays on FDD carrier before the RACH procedure completes. - After RACH procedure completes, the UE switches to SDL carrier at the next configured FDD-to-SDL switching gap. where the completion of the RACH procedure can refer to the ending of Msg-3 transmission or the ending of PUCCH transmission carrying HARQ-ACK of Msg-4, in both CFRA and CBRA.

## R1-2504255 LG

Proposal #1: Consider X=1 and Y=1 on the restriction of maximum X switch(es) within Y slot(s). Proposal #2: Consider K0 and K1 values on the restriction of configuring consecutive SDL slots. Proposal #3: Support Alt 1 for the switching from FDD carrier to SDL carrier. - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier Proposal #4: Consider how to handle (or what is the UE behaviour in) the situation that the semi-static switching pattern activates Case 2 during essential DL signal/channel transmission (e.g. SSB, CSS, CSI-RS for RRM/RLM/BFR) on FDD carrier 1. Proposal #5: Consider UE behaviour in the situation that the semi-static switching pattern activates Case 2 during periodic UL signal/channel transmission occasion/resource (e.g. RACH occasion, SR PUCCH, CG PUSCH, PUXCH repetition) on FDD carrier 1. - Discuss how to handle SR transmission counter/prohibit timer or available slot counting for PUSCH repetition on FDD carrier 1 during Case 2 - Consider FDD carrier 1 with discontinuous UL slots as TDD carrier, and the UL occasions/ resources on FDD carrier 1 during Case 2 as invalid Proposal #6: Consider to add the switching time from Case 2 (with PDSCH reception) to Case 1 (with HARQ-ACK transmission) in the determination of UE PDSCH processing time. Proposal #7: Consider to exclude the slots invalid for PDSCH scheduling on carrier 1 (or 2) during Case 2 (or 1) in the construction of Type-1 HARQ-ACK codebook. Proposal #8: Consider DAI signalling by DL DCI in case of Type-2 codebook for the CA of carrier 1 and 2 via switching where only one carrier can be scheduled at a time.

## R1-2504275 MediaTek

Observation 1: In NR, CORESET duration can be configured only up to 3 OFDM symbols. And baseline UE capability supports PDCCH monitoring only in the first 3 OFDM symbols. Observation 2: With 140us switching period and 10us transient period defined in RAN4, at least 2 PDCCH symbols will be jeopardized if switching gap is located at the switch-to carrier. Observation 3: If switching gap is located at the “switch-to” carrier, the loss of PDCCH symbols cannot be recovered no matter how hard gNB tries by adjusting its configuration and/or changing its implementation. Observation 4: PUCCH format 0 (short, 1~2 symbols) can serve as alternative to PUCCH format 1 for transmitting 1 or 2 UCI bits for SR and HARQ-ACK. Observation 5: PUCCH formats 2 and 3 can serve as alternative to PUCCH format 4 for moderate UCI payload. Observation 6: Durations of both PUCCH formats 1 and 4 are configurable from 4 to 14 symbols, and all NR UEs are mandatory to support different durations without capability signaling according to FG 4-1 in TR38.822. Observation 7: With gNB’s proper configuration and implementation, legacy UEs on the FDD carrier will not be impacted due to the introduction of R19 LBCA. Observation 8: SRS periods are often much larger than 1msec and NW should be able configure SRS in slots without switching for a bitmap based switching pattern. Observation 9: Configuration of PUSCH TDRA is very flexible and ranges from 1 to 14 symbols. This TDRA configuration flexibility has been mandatorily supported by all NR UEs without capability signaling. Observation 10: Shortening PUSCH to be less than 14 symbols is nothing new. This has been handled when UL Tx switching is supported. Observation 11: It is not a typical configuration to place CSI-RS in the last two symbols in a slot. Observation 12: Even if CSI-RS is placed in the last two symbols in a slot, it will not be impacted if there is no switching in the slot based on the configured switching pattern. Observation 13: If Alt 2 (configurable by NW) is supported, both IODT progress and feature deployment will be delayed, if not infeasible. Observation 14: Per RAN4 LS, SSB should be available at least every 160ms for UE to fulfill UL transmit timing requirements. Observation 15: Some front-end components are shared for uplink transmission path and downlink reception path. Therefore, When SDL->FDD switch is performed, the transmission path for FDD uplink carrier is not ready before the RF retuning is complete. Proposal 1: If an issue can be resolved by gNB’s implementation and/or proper configuration, gNB should just do so. Its impact on RAN1 specifications and UE should be minimized. Proposal 2: Optional UE features shall not be assumed or even mandated to support this feature, including cross-carrier scheduling, SSB-less SCell operation, etc. Proposal 3: For switching from FDD carrier to SDL carrier, switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. Proposal 4: SSB-based SCell operations is assumed as baseline for this WI. Proposal 5: For an RRC-configured semi-static switching pattern, gNB shall guarantee SSB availability on both carriers at least once every X1 and X2 msec on the FDD carrier and SDL carrier, respectively, with X1 <=min(160, 2*SSB period on the FDD carrier) and X2<=min(160, 2*SSB period on the SDL carrier). Proposal 6: In an RRC-configured semi-static switching pattern, the maximum number of switch(es) is X within Y slot(s) with (X, Y) = (8, 40) and (1, 1). Proposal 7: Self-scheduling is assumed as baseline for this WI. Proposal 8: UE is not expected to be scheduled or configured with UE-specific downlink resources overlapping with switching gaps. Proposal 9: UE is not expected to be scheduled or configured with UE-specific uplink resources overlapping with switching periods. Proposal 10: For a received PDSCH at slot n on either FDD or SDL carrier, UE expects to be configured a K1 value so that it is on the FDD carrier at slot n+K1 according to its configured switching pattern where K1 is configured according to existing UE capabilities reported by UE. - No new K1 values and HARQ process numbers are introduced for this feature. Proposal 11: Once configured with a semi-static switching pattern, UE is in principle only expected to switch between the FDD carrier and the SDL carrier, according to the configured switching pattern. No additional switching is expected except for RACH procedure. - The base station shall not schedule or configure the UE in a way conflicting to the configured switching pattern. Proposal 12: When RACH is triggered, UE discards the configured switching pattern and stays on the FDD carrier to complete RACH. - When RACH is completed, select one of the following for UE to continue switching between the two carriers - Alt 1: gNB reconfigures UE a switching pattern - Alt 2: UE autonomously resumes the applicability of its switching pattern. - FFS: detailed timeline to avoid ambiguity between gNB and UE Proposal 13: For low-band carrier aggregation via switching, when there is at least a carrier switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission PUCCH/PUSCH/SRS, - introduce additional processing time for UE in the following cases: - Case 1: A PUCCH transmission with HARQ-ACK for a PDSCH - Case 2: PDCCH reception schedules a PUSCH transmission - Case 3: A PDCCH reception indicates SRS transmission - The additional processing required for a UE is the switching period, 35us or 140us, indicated by the UE. - UE does not expect to have more than one SDL->FDD switch or more than one FDD->SDL switch between the received downlink PDCCH/PDSCH and the corresponding uplink transmission (PUCCH/PUSCH/SRS).

## R1-2504347 Apple

Observation 1: With semi-static switching pattern, further restriction in terms of limiting the number of switching instances across multiple slots could be beneficial as network may not need to switch to different carrier on each slot Observation 2: In Rel-19, since the switching is expected to be based on semi-static configuration of a pattern that triggers UE to switch between case 1 and case 2, therefore, in our view, network should be able to avoid other configurations that overlaps with the switching gap considering relaxed timeline Proposal 1: Only a single switching pattern is configured to UE via UE specific RRC configuration - No further discussion/consideration on additional patterns, as the configuration is only applicable when SCell is activated, as per RAN4 agreement Proposal 2: As a starting point, no more than 1 switching per slot should be considered Proposal 3: Further restriction in terms of number of switching per number of multiple slots can be supported, where candidates for different combinations could be based on UE capability Proposal 4: For switching gap location in case of switching from FDD carrier to SDL carrier, adopt Alt 1, i.e. switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier (same as for the case of switching from SDL carrier to FDD carrier) Proposal 5: Network is not expected to allow misalignment/collision with switching pattern/switching gap, at least for cases other than PRACH occasions - However, if network may not avoid such misalignment/collision, the UE may simply ignore such collision and follow the semi-static switching pattern configuration Proposal 6: For PRACH, adopt one of the three following approaches for special handling in case of misalignment/collision with semi-static switching pattern configuration and switching gap - Approach 1: Network avoids the collision - Approach 2 (if approach 1 is not feasible for network): concept of invalid PRACH occasions is extended to this feature such that whenever in a given slot PRACH transmission occasion(s) are triggered, but according to switching pattern, the slot is associated with SDL carrier, then in that case, PRACH occasion(s) are determined as invalid by the UE - Approach 3 (if approach 1 is not feasible for network and there is strong justification to keep PRACH occasion valid): If the PRACH transmission is triggered for the purpose of uplink timing synchronization, e.g. gNB triggers UE with CFRA, then additional switching is triggered by PRACH transmission where similar parameters/restrictions as configured with semi-static switching pattern are applied including: - Switching gap location - Switching gap duration - Switching instances restrictions - No Rx/Tx during switching gap Proposal 7: For Rel-19 low NR band carrier aggregation via switching, when NW configures the semi-static switching pattern to a UE, for Type-I HARQ-ACK codebook generation, adopt one of the two following approaches: - Approach 1: Network avoids the collision between PDSCH occasions and semi-static switching pattern/switching gap - Approach 2 (if approach 1 is not feasible for network): Row of the TDRA table is excluded from the determination of the set of occasions for candidate PDSCH receptions if the PDSCH associated with the row is not to be received by the UE due to LB-CA switching.

## R1-2504416 Qualcomm

Proposal 1: - For Rel-19 low NR band carrier aggregation via switching, for the duration of the switching gap configured by RRC, NW ensures that the switching gap is enough to also cover the DL receive timing difference between the FDD carrier and SDL carrier - The maximum downlink receive timing difference between the FDD carrier and SDL carrier is up to RAN4 Proposal 2: - For switching from FDD carrier to SDL carrier, adopt Alt.2 - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier Proposal 3: - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, consider following restrictions: - Maximum 8 switches within 40 slots of the P = 40ms periodicity - At most 1 switch per consecutive 4 slots Proposal 4: - UE does not expect a conflict between the semi-statically configured switching pattern (including switching periods) and semi-static DL/UL configured by dedicated RRC or dynamic DL/UL scheduled/triggered by unicast DCI - UE prioritizes the semi-statically configured switching pattern over SSB/PRACH - It is up to RAN4 whether to specify special cases such as RLF or BFD where the UE does not follow the switching pattern to transmit PRACH on the FDD carrier Proposal 5: - For switch from FDD carrier to SDL carrier, availability of UL symbols on the FDD carrier is determined by semi-static switching pattern/gap based on DL timing assuming UL TA = 0 and remains unchanged regardless of the UL TA. Proposal 6: - Type-1 HARQ-ACK codebook is generated as if no semi-static switching pattern/gap is configured - I.e., no RAN1 spec impact on Type-1 HARQ-ACK codebook generation Proposal 7: - RAN1 to discuss potential impact on UE processing timeline after overall UE procedures for low-band CA via switching is clear. - UE processing timeline extension would be necessary at least if a UE is required to switch based on a DCI scheduling DL reception or UL transmission

## R1-2504440 Ericsson

Observation 1 For switching from the SDL to the FDD carrier, there is no need to specify any UL dropping rules to handle overlap of the switching gap with the UL transmission on the FDD carrier since the network ensures that the switching gap is configured large enough to cover at least the switching period and timing advance. Proposal 1 Do not specify the maximum number of switches X within a time period Y Proposal 2 If it is needed to specify the maximum number of switches X within a time period Y, then support Y = 40 (equal to the 40 ms pattern period) and support X to be no less than X = 10. Proposal 3 Do not specify other restrictions on the switching pattern. Proposal 4 For switching from FDD carrier to SDL carrier, the switching gap is always assumed to be on the “switch from” carrier, i.e., FDD carrier (Alt-1 in the RAN1#120bis agreement). Proposal 5 For switching from FDD carrier to SDL carrier, the switching gap ends at the end of the slot on the FDD carrier. Note: as previously agreed, the timing reference for configuration of the switching gap is based on the downlink timing of the PCell. Proposal 6 For switching from the FDD to the SDL carrier, the UE omits UL transmission on the FDD carrier in the symbol(s) that overlap the switching gap. FFS: whether or not a similar omission rule is needed for DL reception on the FDD carrier. Proposal 7 For a UE in CONNECTED mode when the SDL SCell is in activated state, if a PRACH occasion occurs in a slot for which the switching pattern indicates the SDL carrier, the UE shall not transmit PRACH. Note: this does not imply that the definition of valid PRACH occasions in current specifications is changed. Proposal 8 For lowband carrier aggregation via switching, when the UE is configured with a semi-static switching pattern, the UE shall not perform additional switching that violates the configured switching pattern.

## R1-2504520 DOCOMO

Proposal 1: For semi-static switching pattern, the restriction based on Y-X equals to 3 or 4 should be considered. Proposal 2: For the semi-static switching pattern, the restriction of maximum 8 consecutive SDL slots should be considered. Proposal 3: UE does not expect that semi-static switching pattern covers no SSB reception occasion/slot for a carrier. - If SSB reception occasion/slot on FDD carrier and that on SDL carrier are not collided, all SSB reception occasions/slots on each carrier should be ensured. - If SSB reception occasion/slot on FDD carrier and that on SDL carrier are fully collided, SSB reception occasions/slots on two carriers should be alternately ensured. - If subset of SSB reception occasions/slots on one carrier are collided with SSB reception occasions/slots on another carrier, SSB reception occasions/slots on carrier having longer periodicity should be ensured as well as non-collided SSB reception occasions/slots on another carrier. Proposal 4: For switching from FDD carrier to SDL carrier, down-selection should be done from following solutions. - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. Proposal 5: UE does not expect to be scheduled to transmit or receive signal/channel on unavailable resource according to the semi-static switching pattern. Proposal 6: The collision between part of occasions for configured transmission/reception or scheduled transmission/reception on multiple occasions and unavailable resource according to the semi-static switching pattern should be allowed, and only transmission/reception on the occasion colliding with unavailable re-source should be dropped. Proposal 7: When NW configures a semi-static switching pattern to a UE, the only candidate scenario for further discussion that requires UE to perform additional switching not following the semi-static switching pattern is RACH. Proposal 8: RAN1 does not need to discuss SCell activation/deactivation related aspects for LBCA unless triggered by RAN4. Proposal 9: UE is expected to follow the configured switching pattern even when SCell is dormant.

## R1-2504606 Google

- Proposal 1: For the location of the switching gap, support Alt 2 with a network-configurable switching gap: - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - For the selection of the values X and Y, a sliding window within any continuous number Y of slots (which could span across the pattern periodicities) can be specified, the number of carrier switches in the continuous window of Y slots must not exceed X. - A condition X < Y/T can be defined. This means the number of switches X in any window Y must be less than Y divided by the threshold T. T can be viewed as the minimum number of slots between the switches. T is reported as a UE capability - The UE can perform or the network can trigger the UE to perform carrier switching outside the semi-static carrier switching pattern to initiate or respond to specific high-priority procedures. The set of procedures and conditions for such UE deviation needs further study (e.g., RACH initiation on PCell, handover command, RLF, LTM, …). - When a Beam Failure Recovery (BFR) procedure or a handover procedure is initiated while the UE is active on PCell, the UE shall disregard any imminent switch to an SCell as per a configured semi-static carrier switching pattern and shall remain on PCell to complete the ongoing BFR or handover procedure. - When an RRC-configured measurement gap overlaps with a switching gap associated with the semi-static carrier switching pattern, the UE shall prioritize the measurement gap (i.e., performing measurements). Upon completion of the measurement gap, the UE shall resume adherence to the semi-static carrier switching pattern. - When a UE is configured with both DRX and a semi-static carrier switching pattern, the switching pattern shall only be considered during DRX Active Time. - The UE is not expected to do carrier switches during DRX Inactive Time. - If a switching gap, as defined by a semi-static carrier switching pattern, is scheduled to coincide with the expiry of drx-OnDurationTimer or drx-InactivityTimer (thus aligning with the transition to DRX Inactive Time), the UE shall prioritize entering DRX Inactive Time. The scheduled carrier switch shall be deferred, and the UE shall determine the appropriate carrier based on the state of the semi-static switching pattern upon starting the next DRX Active Time. - The UE is not expected to receive or transmit during the switching gap. Any configured/scheduled transmission/reception during switching gap is considered as error case.

## R1-2504304 Apple

Proposal 1: Only a single switching pattern is configured to UE via UE specific RRC configuration - No further discussion/consideration on additional patterns, as the configuration is only applicable when SCell is activated, as per RAN4 agreement

# References

- R1-2503273 Discussion on low band CA via switching	Huawei, HiSilicon

- R1-2503331 Discussion on low band carrier aggregation via switching	ZTE

- R1-2503384 Remaining issues on Low band carrier aggregation via switching	vivo

- R1-2503421 Discussion on low band carrier aggregation via switching	NEC

- R1-2503430 On low-band CA via switching	Nokia

- R1-2503532 Discussion on low band carrier aggregation via switching	Spreadtrum, UNISOC

- R1-2503588 Discussion on low band carrier aggregation via switching	Samsung

- R1-2503657 Discussion on carrier switching pattern for low band carrier aggregation via switching	Lenovo

- R1-2503739 Discussion on a low-band CA with switching	Ofinno

- R1-2503780 Discussion on low band carrier aggregation via switching	CATT

- R1-2504153 Low band carrier aggregation via switching	ETRI

- R1-2504217 Discussion of low-band CA via switching	OPPO

- R1-2504255 Discussion on low band CA operation via switching for Rel-19	LG Electronics

- R1-2504275 Low band carrier aggregation via switching	MediaTek Inc.

- R1-2504347 On low band carrier aggregation via switching	Apple

- R1-2504416 Low-band carrier aggregation via switching	Qualcomm Incorporated

- R1-2504440 Low band carrier aggregation through switching	Ericsson

- R1-2504520 Discussion on Low band carrier aggregation via switching	NTT DOCOMO, INC.

- R1-2504606 On Low band carrier aggregation via switching	Google Ireland Limited

- R1-2504304 Discussion on RAN4 LS on RRM agreements on LB CA via switching	Apple

# Appendix A: Agreements from previous and current meeting

## RAN1#121

## RAN1#120bis

Agreement

- For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases

- Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

- Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

- FDD carrier 1 is PCell and SDL carrier 2 is SCell

- SCS 15KHz on both carriers

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier.

Agreement

For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell.

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS).

Agreement

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design

  - NW configures a periodicity, denoted as  slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2

  - The pattern starts from the beginning of SFN 0 of PCell

- Restriction of maximum X switch(es) within Y slot(s),

  - FFS: detailed value and relation of X and Y.

- FFS: other restriction

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap,

- For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier.

- For switching from FDD carrier to SDL carrier, downselect one of the following solutions

  - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

  - Alt 3: Switching gap is always assumed at the SDL carrier

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW,

- Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch.

- Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed)

  - For the UL TA,

    - Switching gap for SDL to FDD switch ends at the end of slot on SDL

      - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA

    - FFS: whether it has specification impact

# Appendix B: LS from other WGs

## R1-2503215 LS of RAN4 RRM agreements on LB CA via switching

RAN4 discussed the applicability of switching pattern for SDL SCell for RRM requirements and would like to share the following information to RAN1. - For the SCell which has been activated (i.e., after UE reporting valid CQI for SCell activation, e.g., in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - RAN4 will also discuss potential scheduling restriction/interruption on PCell during the SCell activation procedure for receiving the above mentioned RS occasions for the SCell activation - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - RAN4 will discuss and decide whether/how UE to perform the deactivated SDL SCell measurement Note: The switching pattern for SDL SCell referred above is the RAN1 defined semi-static switching pattern configured through RRC for the FDD PCell/SDL SCell switching.

## R1-2501700 LS of RAN4 RRM agreements on LB CA via switching RAN4

RAN4 discussed the impact of LB CA via switching on RAN4 RRM requirements and would like to share the following information to RAN1. - In TS38.133, UL transmit timing requirements are applicable provided that at least one SSB is available at the UE during the last 160 ms. - The required availability of SSB on PCell and/or SCell for RRM requirements is still under discussion in RAN4. RAN1 does not need to wait for RAN4 discussion.

## R1-2501702 LS on switching periods for low-low band switching RAN4

RAN4 would like to update RAN1 on the progress of the discussions related to the switching time for low NR band carrier aggregation via switching. To accommodate different UE implementations RAN4 has reached the following agreement: - Define switching periods between FDD low band and SDL low band with the following values as a UE capability to accommodate different UE implementations - 35us, 140us. Additional value(s) are under discussion. If any additional value(s) are confirmed, RAN4 will provide them in a future LS. RAN4 respectfully requests RAN1 to take this information into account in their work.