3GPP TSG RAN WG1 #121	                                    R1-2504352

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.1

Source: 	Apple Inc.

Title: 	Views on UE features for AI/ML for NR Air interface

Document for:	Discussion/Decision

# AI based beam management

In RAN1 120bis, the following agreements are captured with FFS.

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agree

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management [for NW-sided model inference] | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 FFS: whether/how to have separate FG for some of above components or how to merge components | FFS | yes | n/a | Increased number of reported beams for beam management [for NW-sided model inference] is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {FFS} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case1] [4. Maximum number of inference report(s) activated for BM-Case1] [5. Maximum number of inference report(s) triggered for BM-Case1] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] [12. Supported maximum number of predicted beams in each reporting instance] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for component 12 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | - Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] [2. Supported maximum number of predicted beams in each reporting instance] FFS: other components | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: candidate values for component 2 | Optional with capability signalling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model [2. Supported mapping pattern between set B and set A] [3. Maximum number of inference report(s) configured for BM-Case2] [4. Maximum number of inference report(s) activated for BM-Case2] [5. Maximum number of inference report(s) triggered for BM-Case2] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B FFS: RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] FFS: component 7 or component 7a+7b or 7+7a+7b [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] [11. Supported maximum number of predicted beams in each predicted time instance] [12. Supported maximum number of predicted time instances] [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [14. Supported value(s) of time gap between predicted time instances] [15. Supported value(s) of setB periodicity] FFS: whether/how to report the supported maximum total number of CSI reports across different AI/ML based use-cases FFS: whether some of components will be reported dynamically instead of as capability FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection FFS: whether/how to report each of supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) FFS: whether/how to merge some of above components to report combination(s) of supported values | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] [2. Maximum number of future time instance] FFS: other components | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements | Optional with capability signalling |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and [1-port] CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] FFS: components for maximum number of resources | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS | FFS | FFS | FFS | FFS: separate rows/FGs for BM case 1 and BM case 2 Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The UE feature features need to be clearly described.

Proposal 1-1: Remove the square brackets and keep the enclosed texts for 58-1-1, 58-1-2, 58-1-3, 58-1-4, 58-1-5, 58-1-7.

Proposal 1-2: Split component 1 and component 2 of 58-1-1 into two FGs.

Proposal 1-3: Introduce a UE FG for data collection for UE-side beam prediction for BM Case-2.

# AI based positioning

The AI based positioning UE features was discussed in RAN1 120bis, and the following agreement was captured in [1].

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | [1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | [1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] | [13-1] | N/A | N/A |  | FFS | FFS | FFS | FFS | Need for location server to know if the feature is supported. [the reported value is the total number across all bands in the corresponding BC] Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |

In RAN2 #129, the following agreement was made:

Agreement

Introduce AI/ML positioning Case 1 as a new positioning method.

As such, the AI/ML positioning use case FGs and their components should be at least identical to that of the existing use cases. The FGs have been captured in FG 58-2-3, FG 58-2-3-a and FG 58-2-3-b in RAN1 120-bis. However, the components have not been agreed. Based on this, we make the following proposals.

Proposal 2-1:

FG 58-2-3

- DL PRS Resources for UE-based positioning Case 1

- Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE.

- Values = {1, 2}

- Max number of TRPs across all positioning frequency layers per UE.

- Values = {4, 6, 12, 16, 24, 32, 64, 128, 256}

- Max number of positioning frequency layers UE supports

- Values = {1, 2, 3, 4}

Proposal 2-2:

FG 58-2-3-a

- DL PRS Resources for UE-based positioning Case 1 on a band

- Max number of DL PRS Resources per DL PRS Resource Set

- Values = {1, 2, 4, 8, 16, 32, 64}

- Note: 16, 32, 64 are only applicable to FR2 bands

- Max number of DL PRS Resources per positioning frequency layer.

- Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024}

- Note: 6 is only applicable to FR1 bands

Proposal 2-3:

FG 58-2-3-b

- DL PRS Resources for UE-based positioning Case 1 on a band combination

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only.

- Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for FR1 only BC.

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only.

- Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for FR2 only BC

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation.

- Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for BC containing FR1 and FR2 bands

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation.

- Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for BC containing FR1 and FR2 bands]

Two additional FGs need to be added to ensure support for all the FGs defined for each positioning method. These are on the PRS TEG associated information and for support in the RRC_INACTIVE state. As at now, there is no agreement on whether AI/ML positioning is or is not supported in RRC_INACTIVE. Unless there is an agreement not to support it, it is necessary to have an FG to allow for indication of support like the other positioning methods. Based on these arguments, we make the following proposals:

Proposal 2-4: Add a new FG 58-2-4

- Support of PRS TEG association information for AI/ML based positioning

  - Support of reception of association between PRS and TRP Tx TEG for for AI/ML based positioning Case 1

Proposal 2-5: Add a new FG 58-2-5

- Support of PRS measurement in RRC_INACTIVE state for AI/ML based positioning Case 1

  - Support of PRS measurement in RRC_INACTIVE state for AI/ML based positioning Case 1

Finally, for Case 1, 3a and 3b, the following existing FGs should be applicable to AI/ML positioning

Proposal 2-6: For Case 1,

- From the Rel-16 UE feature list for NR Positioning FG 13-1, 13-1a, 13-7, 13-7a and 13-8 are applicable AI/ML-based positioning

- From the Rel-17 UE feature list for NR Positioning FG 27-3-2, 27-3-3, 27-6, 27-9, 27-10, 27-10a and 27-11 are applicable AI/ML-based positioning

Proposal 2-7: For Cases 3a and 3b

- From the Rel-16 UE feature list for NR Positioning all FGs on SRS transmission are applicable AI/ML-based positioning

- From the Rel-17 UE feature list for NR Positioning all FGs on SRS transmission are applicable AI/ML-based positioning

Proposal 2-8:

- From the Rel-18 UE feature list for NR Positioning, the applicable FGs for AI/ML-based positioning are FFS.

# AI based CSI prediction

The UE features was discussed in RAN1 120bis, and captured in [1].

Agreement: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 [8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) ] [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] [11. Scaling factor for active resource counting Kp] | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR [5. Support for the size of DD-basis, N4>1] [6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously] [7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting] [8. Supported values of the maximum number of observation number Candidate values: {FFS}] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |

On inference related UE capability, it was agreed in the RAN1 120bis that AI/ML PU is introduced. It is under further discussion on whether UE capability is introduced to indicate whether CPU or AI/ML PU is used, and how many AI/ML PU is used for CSI prediction. Whether additional UE capability is introduced can be further discussed in RAN1.

For feature group 58-3-2, component 6 and component 8 are duplicate of component 3, which can be removed.

Proposal 3-1: For feature group 58-3-2, remove component item 6 and 8.

In RAN1 120bis, separate UE capability for performance monitoring was agreed for AI based CSI prediction. On performance metric and reporting procedure itself, additional discussion and agreements are required.

Agreement

For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring.

Proposal 3-2: Add additional feature group 58-3-3, for CSI prediction for UE-sided performance monitoring when N4=1.

Proposal 3-3: Add additional feature group 58-3-4, for CSI prediction for UE-sided performance monitoring when N4>1.

# Conclusion

In this contribution, we discussed UE feature for AI/ML. The proposals are:

For AI based beam management:

Proposal 1-1: Remove the square brackets and keep the enclosed texts for 58-1-1, 58-1-2, 58-1-3, 58-1-4, 58-1-5, 58-1-7.

Proposal 1-2: Split component 1 and component 2 of 58-1-1 into two FGs.

Proposal 1-3: Introduce a UE FG for data collection for UE-side beam prediction for BM Case-2.

For AI based positioning:

Proposal 2-1: For FG 58-2-3

- DL PRS Resources for UE-based positioning Case 1

- Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE.

- Values = {1, 2}

- Max number of TRPs across all positioning frequency layers per UE.

- Values = {4, 6, 12, 16, 24, 32, 64, 128, 256}

- Max number of positioning frequency layers UE supports

- Values = {1, 2, 3, 4}

Proposal 2-2: For FG 58-2-3-a

- DL PRS Resources for UE-based positioning Case 1 on a band

- Max number of DL PRS Resources per DL PRS Resource Set

- Values = {1, 2, 4, 8, 16, 32, 64}

- Note: 16, 32, 64 are only applicable to FR2 bands

- Max number of DL PRS Resources per positioning frequency layer.

- Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024}

- Note: 6 is only applicable to FR1 bands

Proposal 2-3: For FG 58-2-3-b

- DL PRS Resources for UE-based positioning Case 1 on a band combination

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only.

- Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for FR1 only BC.

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only.

- Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for FR2 only BC

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation.

- Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for BC containing FR1 and FR2 bands

- Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation.

- Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048}

- Note this is reported for BC containing FR1 and FR2 bands]

Proposal 2-4: Add a new FG 58-2-4

- Support of PRS TEG association information for AI/ML based positioning

  - Support of reception of association between PRS and TRP Tx TEG for for AI/ML based positioning Case 1

Proposal 2-5: Add a new FG 58-2-5

- Support of PRS measurement in RRC_INACTIVE state for AI/ML based positioning Case 1

  - Support of PRS measurement in RRC_INACTIVE state for AI/ML based positioning Case 1

Proposal 2-6: For Case 1,

- From the Rel-16 UE feature list for NR Positioning FG 13-1, 13-1a, 13-7, 13-7a and 13-8 are applicable AI/ML-based positioning

- From the Rel-17 UE feature list for NR Positioning FG 27-3-2, 27-3-3, 27-6, 27-9, 27-10, 27-10a and 27-11 are applicable AI/ML-based positioning

Proposal 2-7: For Cases 3a and 3b

- From the Rel-16 UE feature list for NR Positioning all FGs on SRS transmission are applicable AI/ML-based positioning

- From the Rel-17 UE feature list for NR Positioning all FGs on SRS transmission are applicable AI/ML-based positioning

Proposal 2-8:

- From the Rel-18 UE feature list for NR Positioning, the applicable FGs for AI/ML-based positioning are FFS.

For AI based CSI prediction:

Proposal 3-1: For feature group 58-3-2, remove component item 6 and 8.

Proposal 3-2: Add additional feature group 58-3-3, for CSI prediction for UE-sided performance monitoring when N4=1.

Proposal 3-3: Add additional feature group 58-3-4, for CSI prediction for UE-sided performance monitoring when N4>1.

Reference

[1] R1-2502959	Session Notes of AI 9.15.1	Ad-Hoc Chair (NTT DOCOMO, INC.)