3GPP TSG RAN WG1 #121		R1-2504355

St Julian’s, Malta, May 19th – 23th, 2025

Agenda item:	9.15.4

Source: 	Apple Inc.

Title: 	Views on UE features for Rel-19 NES

Document for:	Discussion/Decision

- Introduction

In this contribution, we provide our views on UE features for Rel-19 NES based on recent UE feature list [2].

# Views on UE features for Rel-19 NES

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) |  | Yes |  | UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #1 | Per band | n/a | n/a | n/a | FFS: supported deactivation mechanisms Note: RRC based deactivation mechanism is up to RAN2 | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2. Supported time domain relation between on-demand SSB and always-on SSB |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated by RRC based signaling in Case #2 for same center frequency | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Note: RRC based deactivation mechanism is up to RAN2 FFS: supported deactivation mechanisms | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequency | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency between always-on SSB and on-demand SSB | 61-2 | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated by RRC based signaling in Case #2 for different center frequency | Per band | n/a | n/a | n/a | Note: RRC based deactivation mechanism is up to RAN2 FFS: supported deactivation mechanisms | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #1 | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Exp, ExpnImp} Note: - Exp: Explicit deactivation - ExpnImp: Explicit and implicit deactivation | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency 2.Supported time domain relation between on-demand SSB and always-on SSB 3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for same center frequency | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Candidate value of component 3 = {Exp, ExpnImp} Note: - Exp: Explicit deactivation - ExpnImp: Explicit and implicit deactivation | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequency | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequency 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation via Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | 61-4 | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for different center frequency | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Exp, ExpnImp} Note: - Exp: Explicit deactivation - ExpnImp: Explicit and implicit deactivation | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-5 | SIB1 request for idle/inactive UEs | 1. Reception of SIB1 request configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 [in a window] [at least] upon SIB1 request |  | FFS |  |  | FFS Per band | FFS n/a | FFS n/a | FFS n/a | A UE indicates support of this FG if it transmits a SIB1 request | Optional [with/without] capability signaling |
| 61. Netw_Energy_NR_enh | 61-6 | SSB burst periodicity adaptation for SCell operation | Support of adaptation of SSB burst periodicity for SCell by DCI format 2_9 |  | Yes |  | UE does not support adaptation of SSB burst periodicity for SCell | Per band | n/a | n/a | n/a | Note: the SSB for this FG is not cell defining SSB | Optional with capability signaling |
| 61. Netw_Energy_NR_enh | 61-7 | Adaptation of RACH in time domain based on additional RACH resources | 1. Support of adaptation of RACH in time domain based on additional RACH resources in RRC idle/inactive/connected mode 2. Configuration of additional PRACH resources via higher layer signaling 3. DCI-based indication of additional PRACH resources by DCI format 1_0 with P-RNTI 4. DCI-based indication of additional PRACH resources by DCI format 1_0 with C-RNTI for PDCCH-ordered PRACH [5. Support semi-static PRACH mask to identify the subset of additional resources] FFS: component 5 is kept in this FG or is separated to new FG |  | Yes |  | UE does not support adaptation of RACH in time domain based on additional RACH resources | Per band | n/a | n/a | n/a | [A UE that supports transmission of PRACH in additional RO based on configuration of additional PRACH resources via higher layer signaling supports this FG] | Optional with capability signaling |

# Reference

- RP-250343 Revised WID for Enhancements of network energy savings for NR Ericsson

- R1-2502977	Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis		Moderators (AT&T, NTT DOCOMO, INC.)