3GPP TSG RAN WG1 #121	R1-2504397

St Julian’s, Malta, May 19th – 23th, 2025

Source:	Qualcomm Incorporated

Title:	Discussion on other aspects for Rel-19 Ambient IoT

Agenda Item:	9.4.4

Document for: 	Discussion and Decision

# Introduction

In this paper, we focus on the remaining issues for D2R multiplexing/multiple access, scheduling and timing relationships, R2D/D2R control designs.

# Configuration for D2R TDMA/FDMA

Agreement For Msg 1 transmission determined by one R2D transmission triggering random access, support X=1 and X=2 time domain resource(s) for D2R transmission(s) for Msg1 only, where each D2R transmission for Msg1 occurs in one time domain resource of the X time domain resource(s) in Rel-19. - All devices support the above - Note: the impact of specification support (at least including signalling overhead) for X=2 to a reader supporting only X=1 should be minimized Only support X=1 time domain resource for D2R transmission for Msg3 in response to a PRDCH for Msg2 transmission. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Agreement Support FDMA with Y≥1 D2R transmissions for Msg.1 from multiple devices in response to a R2D transmission triggering random access. - The maximum value of Y >1 is determined in AI 9.4.2, based on feasible values of small frequency shift to be decided in AI 9.4.2. - Signalling details should be discussed and determined in AI 9.4.4. Agreement Support FDMA of D2R transmissions for Msg3 from multiple devices in response to a PRDCH for Msg2 transmission corresponding to multiple Msg1 during access procedure. Agreement At least for CBRA, for FDMA of multiple Msg1 transmissions in response to a R2D transmission triggering random access, support only the same data rate for the FDMed Msg1 transmissions.

For CBRA Msg1 with FDMA, RAN1 has agreed to use same data rate for FDMed Msg1 transmission. For CBRA, Msg1 has fixed TBS of 16bits. However, RAN1 needs to clarify the data rate, bit rate, chip rate for other cases of D2R with TDMA and/or FDMA, as listed in Table 1. Note that data rate is different from bit rate and chip rate, as defined as follows.

- Data rate (bps): the number of information bits and CRC (if any) per second before FEC (if any)

- Bit rate (bps): the number of bits per second after FEC (if any)

- Chip rate (bps): the number of chips per second with chip rate=2R×bit rate, where R is the frequency shift factor

Table 1 configuration for D2R with TDMA/FDMA

| Case | D2R with TDMA/FDMA | X | Y | Configurations of D2R transmission for different devices |
| --- | --- | --- | --- | --- |
| 1 | CBRA Msg1 with FDMA | X=1 | Y>=1 | same TBS, same data rate, different chip rate for FS FFS: bit rate |
| 2 | CBRA Msg1 with TDMA | X=2 | Y=1 | same TBS FFS: data rate, bit rate, chip rate |
| 3 | CBRA Msg1 with TDMA + FDMA | X=2 | Y>1 | same TBS FFS: data rate, bit rate, chip rate |
| 4 | CFRA Msg1 with FDMA | X=1 | Y>1 | FFS: TBS, data rate, bit rate, chip rate |
| 5 | CFRA Msg1 with TDMA | X=2 | Y=1 | FFS: TBS, data rate, bit rate, chip rate |
| 6 | CFRA Msg1 with TDMA + FDMA | X=2 | Y>1 | FFS: TBS, data rate, bit rate, chip rate |
| 7 | Msg3 with FDMA | X=1 | Y>=1 | FFS: TBS, data rate, bit rate, chip rate |

For different TBS, it could be flexible for reader to schedule different data rate. It is still unclear whether the CFRA Msg1 and Msg3 may be scheduled with different TBS or not. RAN1 to discuss whether to allow different data rate, which is related with RAN2 decision on the TBS for CFRA Msg1 and Msg3.

Proposal 1: For FDMA of multiple CFRA Msg1 or Msg3 transmissions in response to a R2D transmission, RAN1 to decide

- Opt1: support same data rate for FDMAed D2R transmission with same TBS

- Opt2: support different data rate for FDMAed D2R transmission with different TBS if supported

The Msg1 can support X=1 or X=2. When X=2, the Msg1 transmission time in the 1st time domain resource will impact the starting time of the 2nd time domain resource, as shown in Fig. 1. Fig. 1 (b) is to have a shorter Msg1 transmission time in the 1st time domain resource than that of 2nd one. It could be beneficial for the reader to schedule shorter Msg1 transmission in the 1st time domain resource to reduce the time gap TR→D=Toffset1+Toffset2 for the 2nd time domain resource. The Msg1 transmission time includes, preamble, midamble if added, PDRCH duration based on TBS/CRC and bit rate. For CBRA Msg1 with fixed TBS, shorter Msg1 transmission can use higher data rate in the 1st time domain resource. For CFRA Msg1 with different TBS if supported, the reader can schedule the Msg1 with smaller TBS in the 1st time domain resource.

- same Msg1 transmission time for 1st and 2nd time domain resource

- different Msg1 transmission time in 1st and 2nd time domain resource

Fig. 1 Msg1 transmission time with X=2

Proposal 2: For Msg1 with X=2, RAN1 to down select

- Opt1: support same Msg1 transmission time for 1st and 2nd time domain resource

- Opt2: support shorter Msg1 transmission time in the 1st time domain resource than that of 2nd one

Furthermore, for Msg1 with X=2 and Y>1, RAN1 to decide whether same FDMA configuration (including bit rate, chip rate, R) for the 1st and 2nd time domain resources. For CBRA Msg1, it is simple to have the same TBS and same FDMA configuration. For CFRA Msg1, whether to have same or different FDMA configuration may be dependent on different TBS if supported.

Proposal 3: For CBRA Msg1 or CFRA Msg1 with X=2 and Y>1, down-select

- Opt1: same FDMA configuration is configured for 1st and 2nd time domain resource

- Opt2: different FDMA configuration can be configured for 1st and 2nd time domain resource

# Scheduling and timing relationships

## Discussion on TR→D

Conclusion For any timing requirement that needs to be specified from device perspective by RAN1, the timing(s) do not include the impacts of SFO from the RAN1 perspective. - Whether 3GPP needs to specify some requirement on device’s SFO is up to RAN4 discussion. - Note: RAN1 will not specify TR2D_min or TR2D_max in Rel-19 - Note: SFO may still be considered when discussing time-domain resources for Msg1 when X=2 Agreement For X=1 or X=2, from device perspective, the starting time for the first Msg1 time domain resource is Toffset1 after the end of the corresponding R2D transmission triggering random access. - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access. - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact - FFS: how to define the end of the R2D transmission in 9.4.1 Working assumption For X=2, from device perspective, for determination of the starting time for the second Msg1 time domain resource: - Derived based on the starting time of the first Msg1 time domain resource plus Toffset2 - FFS Toffset2 is predefined in the spec or derived by a rule or indicated implicitly or explicitly by the R2D transmission triggering random access. Agreement For the initial values of the shift register of the CC encoder, the initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits defined in TS 36.212. - Note: K is the constraint length. - For discussion on timing relationships in 9.4.4, the entire processing time for encoding (including finishing CRC encoding) should be taken into account Agreement When the generated number of chips for the R2D transmission does not fully occupy the last OFDM symbol, padding is used. - Padding is to be down-selected among the following alternatives: - Alt 1a: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: it implies the device should be aware of the duration of padding or the last OFDM symbol boundary by implementation. FFS how accurately the device can be aware. - Alt 1b: The content of padding is up to reader implementation and transparent to device. - Note: the timeline determination of any timing relationship refers to the start of the padding. - Alt 2: Specify the detailed content of padding - Note: the timeline determination of any timing relationship refers to the end of padding. - Note: the end chip(s) of the padding content shall follow the CP handling solution determined in RAN1, and may be affected by other agreements.

### Definition of TR→D

For the definition of TR→D, RAN1 needs to define the starting/ending of TR→D.

For the starting point of TR→D,

It depends on whether the padding for R2D transmission counted or not. The padding is required due to OFDM symbol generation at the reader side. Moreover, the padding is not needed if the end of PRDCH transmission for TBS+CRC is aligned with OFDM symbol. When there is no padding, e.g., for M=2 or some TBS, the device cannot use the padding pattern to determine the end of last bit. At the device side, there is no FFT operation and the counting of a valid R2D message does not need to align with a OFDM symbol boundary. As long as the device detects the last bit before the padding, the device can start the processing to prepare D2R transmission. Therefore, the TR→D can be counted from the last bit of PRDCH not including the padding (Alt1b in the RAN1 agreement on padding).

Observation1:

The PRDCH reception can start after receiving the last bit of PRDCH before the padding (Alt1b).

The Manchester coding has midpoint transition per bit. Based on synchronization and rising/falling edge detection, the end of last bit can be defined as 1/2 bit time after the last midpoint transition:

          - If a data 1 (chips{01}) is the last bit, the end of the last bit is 1/2 Tb after the last rising edge;

          - If a data 0 (chips{10}) is the last bit, the end of the last bit is 1/2 Tb after the last falling edge.

Once the receiver detects the midpoint transition, it waits for an additional Tb/2. If there is no additional transition occur during this Tb/2 interval, the bit has ended at Tb. This method avoids false sample(s) from spurious transmissions, as only the midpoint transition is considered valid. The bit length depends on M={2, 6, 12, 24} for PRDCH. For M=2, Tb/2=33.34us; but for M=24, Tb/2=2.78us, which is very short for noise/interference immunity. If the padding is added, it may impact the last bit determination. The uncertainty of midpoint detection at device side may impact the D2R transmission timing to be aligned with TR→D.

If the TBS of PRDCH is indicated before detecting MAC PDU, the device can know the CRC length of 6 or 16bits. The receiver stops PRDCH detection operation after processing the (TBS+CRC)bits. There is no uncertainty to determine the end of last bit.

Observation2: For the starting point of TR→D

          - The midpoint transition detection for the last data bit is dependent on the chip duration for PRDCH. The uncertainty with large M and padding if added may impact alignment of TR→D.

          - With TBS of PRDCH indicated in R2D L1 control, the end of last bit can be derived without uncertainty.

For the ending point of TR→D, the device knows where to start the transmission of D2R.

          - For X=1, TR→D=Toffset1 and the ending point is the start of D2R preamble.

          - For X=2, TR→D=Toffset1 or TR→D=Toffset1+Toffset2 and the ending point is the start of D2R preamble if the Msg1 is transmitted in the 1st or  2nd time domain resources.

Fig. 2 TR→D for X=2 and Y=1

Proposal 4: TR→D is counted from the end of the last bit of PRDCH (before the start of the padding) till the start of D2R preamble of corresponding D2R transmission.

- The end of the last bit for PRDCH can be determined based on the TBS indicated in R2D L1 control.

### Configuration for TR→D=Toffset1

For TR→D=Toffset1, RAN1 to further discuss

  - FFS Toffset1 is predefined in the spec or indicated implicitly or explicitly by the R2D transmission triggering random access.

  - FFS detailed value(s) of Toffset1, considering at least the device processing time including FEC impact

  - FFS: how to define the end of the R2D transmission in 9.4.1

The TR→D needs to be designed to ensure sufficient time for the device reliably detect a valid R2D message and prepare the corresponding D2R response, considering variations in tag capability for power-up timing and processing. There are different alternatives to determine Toffset1:

- Alt1 predefine one absolute value without additional signaling for Toffset1

- Alt2 predefine a list of candidate absolute values and the reader explicitly indicates one of the value for Toffset1

- Alt3: predefine an equation to derive the Toffset1, where the equation is based on the TbitR2D and TchipD2R for the scheduled D2R transmission and no additional explicit signaling

Alt1 is the simplest, but it is not efficient since the value has to be based on the worst case of all possible R2D trigger and D2R responses. Alt2 can provide flexibilities. However, the more flexibility for different R2D/D2R cases the more candidate values. It may not be feasible to let all devices memorize all the candidate values, considering the limited memory and insufficient power. Alt3 is similar as the method to determine RFID T1_norminal. Compared with Alt1 and Alt2, it can derive the appropriate TR→D, with no additional signaling. The device will count TR→D based on the clock frequency of 1/TchipD2R, instead of the absoluted time value.

Take RFID T1_norminal as a reference, it is similar as the case of TR→D=Toffset1 with X=1 and Y=1 and no FEC.

- For RFID with PIE as in Table 6.16 [Gen2 UHF RFID v1.2.0], T1_nominal = MAX(RTcal, 10Tpri), based on the reader-to-tag calibration length (RTcal=data_0+data_1) and the backscatter-link pulse-repetition interval (Tpri). Tpri serves as the fundamental timing reference derived from the tag’s backscatter link frequency (BLF), i.e., Tpri=1/BLF.

- For RFID with Manchester coding as in Table 7.7 [ISO/IEC 18000-63:2021], T1_nominal = MAX(2.5Tbit, 10Tpri), based on the reader-to-tag bit length (Tbit) and the Tpri.

To use a similar way as RFID for Atl3, the R2D chip duration TchipR2D and D2R chip duration TchipD2R can be used to determine TR→D=Toffset1.

For PDRCH without FEC, TR→D=Toffset1=maxαTchipR2D, βTchipD2R, where

- αTchipR2D (e.g., α=4), is to wait for a short time based on multiple times of TchipR2D (chip duration of R2D message) to ensure a valid R2D message considering different device processing time of R2D reception.

- βTchipD2R (e.g., β=10), is to ensure sufficient time for device to prepare its D2R based on the multiple times of TchipD2R (chip duration of scheduled D2R), considering variant device preparation time, including the tx power-up timing.

For PDRCH with FEC, additional time may be needed, TR→D=Toffset1=maxαTchipR2D, βTchipD2R+Tadd_proc, which depends on CRC attachment scheme and whether CRC can be (pre)calculation before the coding,

- According to RAN1 agreement, the “last 6 information bits” are used for FEC register initialization. There can be two ways to interpret last 6 information bits, to be discussed in 9.4.2 [1]:

  - Alt1: Attach CRC not to the end but before the last 6 information bits.

    - Then, no need to consider any additional time for CRC/FEC.

  - Alt2: Attach CRC at the end and the last 6 CRC bits are used for the initialization.

    - If the CRC can be pre-calculated before the R2D trigger, e.g., for Msg1/Msg3 without segmentation, no need to consider additional time for CRC/FEC;

    - Otherwise, CRC calculation time has to be counted. Since D2R preamble is not part of PDRCH and no CRC/FEC is applied to D2R preamble, the additional time can be defined as Tadd_proc=max0,TprocFEC-TpreambleD2R, where

      - TpreambleD2R is D2R preamble time;

      - TprocFEC is CRC calculation time for FEC, defined as TprocFEC=TBS+6/Rsampling.

        - For large TBS, TprocFEC could be very large, e.g., around 0.54ms required for TBS=1024bits with sampling rate of Rsampling=1.92MHz.

Similar as Msg1 with X=1, Msg3 or other D2R with X=1 only should apply TR→D=Toffset1.

Proposal 5: For D2R with X=1, from device perspective, the starting time for the D2R transmission is TR→D=Toffset1 after the end of the corresponding R2D transmission.

If Alt2 is applied, RAN1 should further consider whether to add non-zero additional processing time for different types of messages:

- For CBRA Msg1, the Msg1 is 16bit random ID.

  - The device can pre-calculate CRC for Msg1 RN16, no need to consider TprocFEC.

  - Even if the device cannot pre-calculate CRC for Msg1 RN16, TprocFEC=22/Rsampling=11.4us for TBCC FEC. In this case, TpreambleD2R could be long enough to cover TprocFEC. Therefore, Tadd_proc=0.

    - For example, for the very high data rate of 640kbps=Rsampling/3, the short D2R preamble with 7bits has TpreambleD2R=21/Rsampling, which is similar as TprocFEC.

- For CFRA Msg1, it can have variable TBS but no segmentation is applied.

  - If the device can pre-calculate CRC for CFRA Msg1, no need to consider additional time.

  - If the device cannot pre-calculate CRC for CFRA Msg1, TprocFEC is dependent on CRC attachment scheme and max TBS for Msg1.

    - For example, CFRA Msg1 TBS with 12bytes requires TprocFEC=96/Rsampling. Based on very high bit rate of 640kbps=Rsampling/3, the long preamble for this large TBS has  TpreambleD2R=93/Rsampling, which is similar as TprocFEC Therefore, Tadd_proc=0.

- For Msg3, it can have variable TBS but no segmentation is applied.

  - After Msg1 transmission, the CRC for Msg3 can be precalculated while monitoring Msg2. No need to consider additional time.

  - Even if the device cannot pre-calculate CRC, similar Msg3 TBS restriction can be applied as CFRA Msg1, e.g., TBS up to 12byptes to limit the CRC/FEC preparation time.

- For D2R feedback to R2D data detection, it can have predefined small TBS, pending on RAN2 decision.

  - If using AS ID as ACK feedback, the CRC can be precalculated. No need to consider additional time.

- For D2R data, this is the only case that TBS can be up to 128bytes and segmentation is supported. Tadd_proc=max0,TprocFEC-TpreambleD2R should be counted for Toffset1.

Proposal 6: For TR→D=Toffset1, it can be implicitly derived based on the R2D chip duration TchipR2D, D2R chip duration TchipD2R, and additional processing time if needed, as

- TR→D=Toffset1=max[4]TchipR2D, [10]TchipD2R+Tadd_proc.

- For PDRCH without FEC, Tadd_proc=0

- For PDRCH with FEC,

  - Alt1: attach CRC not at the end, Tadd_proc=0

  - Alt2: attach CRC at the end,

    - For Msg1/Msg3/D2R feedback, apply TBS restriction of [up to 12bytes] or use precalculated CRC, Tadd_proc=0 .

    - For D2R data transmission, Tadd_proc=max{0,TprocFEC-TpreambleD2R} with D2R preamble time TpreambleD2R and TprocFEC processing time for CRC/FEC.

      - Tproc=FEC=TBS+6/Rsampling with sampling rate Rsampling

For Msg1/Msg3 with Y>1 FDMA, the TchipD2R will be different for each frequency shift. A common starting time is useful to align the D2R transmission from different devices, which can significantly reduce the timing hypotheses at the reader side [2]. A reference D2R chip rate is needed to determine TRD. To accommodate the worst case of the FDMA, the TchipD2R of the smallest frequency shift can be used to derive TRD.

Fig. 3 Msg1/Msg3 with Y>1 FDMA

Proposal 7: For Msg1/Msg3 with FDMA, the chip duration TchipD2R of the smallest frequency shift for the FDMA is used to derive Toffset1.

### Configuration for TR→D=Toffset1+Toffset2

For TR→D=Toffset1+Toffset2, we need to consider the SFO impact for Msg1 transmission when X=2. The starting time of the second Msg1 time domain resource Toffset1+Toffset2 may be drifted due to SFO error. Consider the SFO tolerance for 10% SFO, it can be calculated as

- Toffset1+Toffset2=Toffset1+TMsg1_time∙1.2,

where TMsg1_time is the Msg1 transmission time in the 1st time domain resource, e.g., TpreambleD2R+TmidambleD2R+(TBS+CRC)∙TbitD2R. The TBS if not predefined and TbitD2R are the TBS and bit duration to be indicated by reader. The CRC, TpreambleD2R and TmidambleD2R are the CRC length, preamble and midamble time, which can be based on TBS for Msg1.

Proposal 8: For TR→D=Toffset1+Toffset2, it can be implicitly derived based on Toffset1 and Msg1 transmission time TMsg1_time1 in the 1st time domain resource as Toffset1+Toffset2=Toffset1+TMsg1_time1∙1.2.

For Msg1 with X=2 TDMA and Y>1 FDMA, the TchipD2R will be different for each frequency shift. A common starting time for the second time domain resource is also needed to align the D2R transmission from different devices. A reference Msg1 transmission time is needed to determine TR→D=Toffset1+Toffset2.

Fig. 4 Msg1 with X=2 TDMA and Y>1 FDMA

Proposal 9: For Msg1 with X=2 and FDMA, the common Msg1 transmission time in the 1st time domain resource is used to derive TR→D=Toffset1+Toffset2.

For Msg1 with X=2,. TR→D=Toffset1 or TR→D=Toffset1+Toffset2 is randomly selected for CBRA Msg1 or indicated for each CFRA Msg1

Proposal 10: For Msg1 with X=2, TR→D=Toffset1 or TR→D=Toffset1+Toffset2 is randomly selected for CBRA Msg1 or indicated for each CFRA Msg1.

## Discussion on Msg-2 and TD→R

### Definition of TD→R

For the definition of TD→R, RAN1 needs to define the starting/ending of TD→R.

For the starting point of TD→R, the device can count it after the last bit of the D2R transmission, either PDRCH or D2R misamble if added at the end. If there is only one D2R transmission, the reader could transmit the corresponding R2D response immediately after D2R transmission, similar as that of RFID T2 with the minimum time gap defined as 3Tpri.

Fig. 5 TD→R for X=1 and Y=1

However, if there are multiple Msg1s transmitted in FDMA and/or TDMA, the Msg2 would include one or multiple detected Msg1 IDs. If there are more than one Msg1 resources triggered by the paging, the reader will wait until the end of the Msg1 resources to transmit Msg2. Therefore, the starting point of TD→R can be delayed till the end of last Msg1 resources triggered by the same R2D message to align with the reader preparation for Msg2 transmission. The device does not need to count TD→R earlier for Msg2 monitoring.

- For X=1, the end of last Msg1 resource is (Toffset1+TMsg1_time1) as the starting point of TD→R, where TMsg1_time1 is the Msg1 transmission time in the 1st time domain resource;

- For X=2, the end of last Msg1 resource can be derived by using (Toffset1+Toffset2+TMsg1_time2) as the starting point of TD→R, where TMsg1_time2 is the Msg1 transmission time in the 2nd time domain resource

Fig. 6 TD→R for X=2 and Y=1

For the ending point of TD→R, it would be the start of the R2D preamble (R-TAS) transmission. Assuming the SIP is starting from ‘ON’, the start of SIP will be the first rising edge.

Proposal 11: For R2D response associated with a single D2R transmission, TD→R can be counted from the end of the last bit of the D2R transmission till the potential start of the corresponding R2D transmission, where the start of the R2D transmission is the first rising edge of R-TAS.

Proposal 12: For Msg2 corresponding to one or multiple Msg1 transmissions, TD→R can be counted from the end of the last Msg1 transmission resource(s) till the potential start of the corresponding Msg2 transmission.

- For Msg1 with X=1 and FDMA, the TMsg1_time1 in the 1st time domain resource is used to derive the start of TD→Rmin as (Toffset1+TMsg1_time1).

- For Msg1 with X=2 and FDMA, the TMsg1_time2 in the 2nd time domain resource is used to derive the start of TD→Rmin as (Toffset1+Toffset2+TMsg1_time2).

### Configuration for TD→Rmin

The TD→Rmin needs to ensure sufficient device switching time from transmission to monitoring and the reader processing time. For X=1 and Y=1, RFID T2_min = 3Tpri can be used as a reference, where Tpri is the D2R chip duration. TD→Rmin=3TchipD2R.

For Msg1/Msg3 with Y>1 FDMA, the TchipD2R will be different for each frequency shift. A common parameter can be used to align starting/end of TD→R. To accommodate the worst case of the FDMA, the TchipD2R of the smallest frequency shift can be used to derive TD→R_min.

Proposal 13: TD→Rmin=[3]TchipD2R, where TchipD2R is the chip duration.

- For Msg1/Msg3 with FDMA, the chip duration TchipD2R of the smallest frequency shift for the FDMA is used to derive TD→Rmin.

### Configuration for TD→Rmax

In previous meeting, it was discussed whether to define TD→Rmax may be dependent on RAN2 discussion on monitoring window. RAN2 has agreed to use NACK message for Msg3. But this NACK scheme with device ID for Msg3 cannot be applied to Msg1 failure, since the Msg1 RN16 cannot be detected.

For Msg1, the Msg2 monitoring window with [TD→Rmin, TD→Rmax] if defined will be beneficial for device to determine Msg1 failure and help device determine whether to keep monitoring or re-access to a new paging.

- The device is expected to monitor the starting of Msg2 no later than TD→Rmax; otherwise, TD2R expires.

In RFID, T2_max is applied to tags in the reply or acknowledged states. The reply state corresponds to the state when device transmits Msg1 and wait for Msg2. T2_max is used to let tag determine whether the T2 expires (i.e. reaches the T2_max). For TD2R of Msg2 monitoring, TD2R_max can be defined, similar as T2_max=20Tpri, used to determine whether to stop monitoring the Msg2 for the associated Msg1.

Proposal 14: Define TD→Rmax=[20]TchipD2R at least for R2D monitoring corresponding to single D2R transmission.

# R2D/D2R control design

## R2D control information/functionality

Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values Agreement The payload size (i.e. TBS-like) for PDRCH transmission with variable size is explicitly indicated in the corresponding R2D control information. Agreement (in 9.4.2) When CRC is attached to a PRDCH or PDRCH transmission, when the number of information bits is ≤ 24 bits, CRC-6 is used. Otherwise, when the number of information bits is > 24 bits, CRC-16 is used. Agreement For scheduling D2R transmission, any scheduling information related to resource allocation that needs to be signaled is indicated by higher-layer signaling via the corresponding PRDCH. Agreement Use 1 bit to indicate the value of X (X=1 or X=2) time domain resource(s) for Msg1 transmission(s). Working assumption For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size: - 7 bits for byte-level D2R payload size indication Agreement (RAN2 agreement on paging ID) - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer Agreement (RAN2 agreement on AS ID) - For CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. FFS how this is signalled, which message is used and size of AS ID. - From device perspective, it is only required to use one AS ID. - CFRA is not supported for group ID - RAN2 assumes, AS ID is needed for CFRA at least for inventory + command procedure - For CFRA, if a valid AS ID is not already assigned, continue the discussion on AS-ID assignment based on the following options: - Option 2: the device includes a random ID in “Msg 1”. And same as CBRA, it is up to Reader to decide whether to reuse the random ID as the AS ID or to assign a new AS ID. - Option 3: New “Msg 2” for AS ID assignment, complementary option or independent from option 2 - Option 4: “Msg 2” (including the “Command”) for AS ID assignment, complementary option or independent from option 2. Agreement (RAN2 agreement on Transaction ID) - Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. Agreement (RAN2 agreement on MAC PDU format design) - Aim to design simple MAC PDU format design - Support multiplexing of information for multiple devices in R2D message for msg2. FFS others for multicast messages - At least the following field are required for at least for R2D in the MAC header– message type, length for SDU and variable part(s). - FFS whether for D2R we need message type field, any length and need for padding - Specify message types and contents. As starting point consider the following MAC message types. - R2D MAC PDU (Paging/R2D trigger (depending on agreement on WF)) - D2R MAC PDU (MSG1) (FFS if this requires a MAC header or not) - R2D MAC PDU (MSG2) - D2R MAC PDU (MSG3 and data) - R2D MAC PDU (R2D data) - Other message types are FFS. The message types may evolve based on functionality agreements. Agreements (RAN2 agreement on segmentation) - To support segmentation, a 1 bit indication is introduced to indicate whether there is more data or not, if SA2 indicates that CN can provide an estimated expected D2R message size. If not possible, FFS if the 1 bit is sufficient. - Segment retransmission is supported. - For segment retransmission, reader explicitly indicates an offset in the MAC layer– e.g. number of bits successfully received so far (from the start). FFS This implies that unsegmented packet can also be retransmitted. FFS if this applies to msg3 - R2D segmentation is not supported for R19 A-IoT. Agreements (RAN2 agreement on MAC PDU format) - The MAC PDU should be byte-aligned, assuming the allocated TBS value is in the unit of byte. The actual TBS value depends on RAN1. FFS for R2D trigger message - RAN2 assumes that the upper layer data SDU is byte-aligned, and an LS can be sent to CT1. - The D2R MAC PDU size will correspond to the TBS size indicated in the R2D message - The MAC padding is supported at least for D2R from RAN2 perspective. The device includes padding bits if there is no more data and there is still space available in the TBS. - In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required. FFS how this is provided (i.e. SDU length field or padding length field). The size of length field is FFS.

### R2D control to schedule R2D data and D2R feedback

The R2D control to schedule unicast R2D data have multiple components, as summarized in Table 2.

Table 2 R2D control to schedule unicast R2D data

|  | R2D control info | bits | Notes |
| --- | --- | --- | --- |
| R2D control general info | Info type/message type | [3~4] | L1 control |
|  | Transaction ID | [2] | L1 control |
| ID for devices | Msg 1 device ID or AS ID | [16] | L1 control |
| Resource allocation infor (RAI) for R2D | TBS of PRDCH | [7] for TBS up to max 128byptes | L1 control |
|  | Chip duration | 1bit to indicate whether M is different between R2D L1 control and R2D data | L1 control |
| Resource allocation infor (RAI) for D2R feedback | FDRA (TbitR2D, R, TchipD2R) | For unicast D2R feedback: no FDMA | Higher layer control |
|  | TDRA | Starting time for one resource Alt1: explicitly indicated Alt2: implicitly derived |  |
|  | Coding scheme/rate | [2] |  |
|  | Block-level repetitions |  |  |
|  | TBS for PDRCH | For D2R feedback: predefined |  |
|  | D2R preamble/midamble | For D2R feedback: [2] |  |

The R2D/D2R general information can include the message type, which is used to indicate query or command to schedule R2D data or D2R data. The message type is included in the R2D L1 control and also used to indicate the size/length if predefined. The transaction ID can also be included in R2D L1 control. It can be used to differentiate different services. The message type and transaction ID in L1 control can identify the message before MAP PDU processing.

For early indication/termination, the ID associated with devices can also be indicated in R2D control. The short device ID, such as Msg1 ID in or AS ID for R2D data scheduling can be included in R2D L1 control. If the ID is not for the device, the device can terminate the Rx decoder for PRDCH detection (which involves other related clock counting and memory operation). Even if device 1 does not go to sleep, it only needs to keep the SIP detection using ED, comparator and minimum BB detection to monitor upcoming R2D message.

Figure 5.1-1: Architecture of device 1 [TR38.769]

Observation3:

- The Msg type, transaction ID, device ID indicated in R2D L1 control with separate CRC can enable early indication / termination.

  - If the R2D trigger is not for the device, the device can terminate the Rx decoder for PRDCH detection but keep minimum power consumption to monitor upcoming R2D message.

Regarding the TBS of PRDCH, it serves to indicate the end of PRDCH before padding if ayn. RAN2 has agreed to include the "length of MAC SDU" in the MAC header for R2D purposes. However, since the MAC SDU is only one part of the MAC PDU, its length alone is insufficient to ascertain the TBS of PRDCH. For example, for R2D data scheduling, MAC PDU could include at least R2D data payload from higher layer in MAC SDU, and resource allocation information (RAI) for D2R feedback as the response to the R2D data detection. It is essential to know the TBS of the entire MAC PDU to determine whether CRC6 or CRC16 should be applied to PRDCH.

Observation4:

- The MAC PDU may include higher layer data in MAC SDU and other control, such as resource allocation for D2R. The ‘length of MAC SDU’ along is not sufficient to get the TBS of MAC PDU.

- The TBS of PRDCH carrying MAC PDU is indicated in L1 control, based on which the device can determine the CRC length and the end of PRDCH before padding if any.

For PRDCH, RAN1 has agreed to support M={2, 6, 12, 24} as the OOK-4 waveform. The CAP in R-TAS can be used to indicate a subset of M supported for R2D L1 control in PRDCH. For example, M={2, 6, 12} for L1 control. Therefore, 1bit can be used to indicate whether to R2D data uses same M as L1 control.

For unicast R2D data scheduling, the RAI for D2R feedback will be indicated by higher layer, including explicit/implicit configuration of FDRA, TDRA, coding/repetition. A compact TBS can be predefined for D2R feedback. For compact D2R feedback with small TBS, D2R preamble could be sufficient, depending on discussion in 9.4.3. More discussion on the RAI for D2R is discussed in Sect. 4.1.2.

For R2D Msg to schedule R2D data,

  - L1 control is separate from MAC PDU, with separate CRC for early indication

    - Msg type: [3~4]bits

    - Transaction ID: [2]bits

    - AS ID or Msg1 device ID: 16bits

    - TBS of PRDCH: [7]bits

    - R2D chip duration same for L1 control and data: 1bit

    - CRC: 16bits

  - R2D MAC PDU needs to carry

    - R2D data

    - RAI for D2R feedback

Fig. 7 PRDCH to schedule R2D data

### R2D control to schedule Msg1/Msg3/D2R data

The R2D control to schedule Msg1/Msg3/D2R data have multiple components, as summarized in Table 3.

Table 3 R2D control for PDRCH

|  | R2D control info | bits | Notes |
| --- | --- | --- | --- |
| R2D control general info | Info type/message type | [3~4] | L1 control |
|  | Transaction ID | [2] | L1 control |
| ID for devices | Msg1 device ID | [16] Msg1 device ID * number of devices to schedule Msg3; | L1 control |
|  | AS ID | [16] AS ID to schedule D2R unicast data; |  |
| Resource allocation infor (RAI) for R2D | TBS of PRDCH | [4~5] No need for TBS up to max 128byptes | L1 control |
|  | Chip duration | 1bit to indicate whether M is different between R2D L1 control and R2D higher layer data | L1 control |
| Resource allocation infor (RAI) for D2R | FDRA (TbitR2D, R, TchipD2R) | For unicast D2R data: no FDMA For CBRA Msg1: up to 8 FDMA For CFRA Msg1/Msg3: up to 4 FDMA | Higher layer control |
|  | TDRA | For Msg1: 1bit TDMA; Starting time for each resource Alt1: explicitly indicated Alt2: implicitly derived |  |
|  | Coding scheme/rate | [2] |  |
|  | Block-level repetitions |  |  |
|  | TBS for PDRCH | For CBRA Msg1 with predefined length: 0; For Msg1/Msg3 with variable length: [4]; For D2R unicast data with variable length: [7]; |  |
|  | D2R preamble/midamble | [2] |  |

The R2D/D2R general information can include the message type, which is used to indicate query or command to schedule R2D data or D2R data. The message type is included in the R2D L1 control and also used to indicate the size/length if predefined. The transaction ID can also be included in R2D L1 control. It can be used to differentiate different services. The message type and transaction ID in L1 control can identify the message before MAP PDU processing.

For early indication/termination, the ID associated with devices can be indicated in R2D control. The short device ID, such as Msg1 ID for Msg3 or AS ID for D2R data scheduling can be included in R2D L1 control. If the ID is not for the device, the device can terminate the Rx decoder for PRDCH detection (which involves other related clock counting and memory operation). Even if device 1 does not go to sleep, it only needs to keep the SIP detection using ED, comparator and minimum BB detection to monitor upcoming R2D message.

The RAI of Msg1/Msg3/D2R data will be indicated by higher layer, including explicit/implicit configuration of FDRA, TDRA, coding/repetitions, TBS if not predefined, and preamble/midamble indication.

- For FDRA, the (TbitD2R, R, TchipD2R) with or without FDMA should be considered, respectively. The D2R chip duration TchipD2R=TbitD2R2R can be derived by indicting the D2R bit duration TbitD2R and small frequency shift factor R.

  - For unicast D2R: Y=1 no FMDA. FDRA needs to indicate

    - TbitD2R: 3bits

    - R: predefined as R=1 for TbitD2R=720kbps and R=2 otherwise.

  - For CBRA Msg1: TBS=16bits and Y≤8  FDMA can be supported [1]

    - TbitD2R: 3bits

    - R and Y: FFS bitmap, codepoint, Y+Rmin+gap, or Y+Rmax+rule.

  - For Msg3 or CFRA Msg1: TBS can be larger and Y≤4 FDMA can be supported [1]

    - TbitD2R: 3bits

    - R and Y: FFS bitmap, codepoint, Y+Rmin+gap, or Y+Rmax+rule.

Table 4 The bit duration/bit rate, chip duration, frequency shift for D2R [1]

|  |  | Frequency shift (kHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3.75 | 7.5 |  | 15 |  | 30 |  | 45 |  | 60 |  | 90 |  |  | 120 |  | 180 |  | 240 | 360 | 480 |  | 720 | 960 |  |
|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Bit rate (kbps) | Tb (μs) | 133.33 |  | 66.67 |  | 33.33 |  | 16.67 |  | 11.11 |  | 8.33 |  | 5.56 | 4.17 |  | 2.78 |  | 2.08 |  | 1.39 |  | 1.04 | 0.69 |  | 0.52 |
| 3.75 | 266.67 |  |  | R=2 |  | R=4 |  | R=8 |  |  |  | R=16 |  |  | R=32 |  |  |  | R=64 |  |  |  | R=128 |  |  | R=256 |
| 7.5 | 133.33 |  |  |  |  | R=2 |  | R=4 |  |  |  | R=8 |  |  | R=16 |  |  |  | R=32 |  |  |  | R=64 |  |  | R=128 |
| 15 | 66.67 |  |  |  |  |  |  | R=2 |  |  |  | R=4 |  |  | R=8 |  |  |  | R=16 |  |  |  | R=32 |  |  | R=64 |
| 30 | 33.33 |  |  |  |  |  |  |  |  |  |  | R=2 |  |  | R=4 |  |  |  | R=8 |  |  |  | R=16 |  |  | R=32 |
| 45 | 22.22 |  |  |  |  |  |  |  |  |  |  |  |  | R=2 |  |  | R=4 |  |  |  | R=8 |  |  | R=16 |  |  |
| 90 | 11.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R=2 |  |  |  | R=4 |  |  | R=8 |  |  |
| 180 | 5.56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R=2 |  |  | R=4 |  |  |
| 360 | 2.78 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R=2 |  |  |
| 720 | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |  |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- For TDRA,

  - For Msg1: 1bit to indicate X=1 or X=2

  - The Toffset1 and Toffset1+Toffset2 for the 1st and 2nd time domain resource are implicitly derived without additional signaling, discussed in Sect. 3.1

  - Otherwise: X=1 only and Toffset1 is implicitly derived without additional signaling, discussed in Sect. 3.1

- For preamble/midamble, it will be discussed in 9.4.3.

  - In our contribution [2], we discussed the length/location is dependent on the D2R TBS, and illustrate [2]bits in R2D control, where the 1bit MSB can be used to indicate length of preamble/midamble; and 1bit LSB can be used to indicate the Ibit in between preamble and midamble or between midambles.

- Ibit = {33, 66} for 7-bit preamble/midamble​

- Ibit = {66, 132} for 31-bit preamble/midamble

| ​ | MSB = 0​ | MSB = 1​ |
| --- | --- | --- |
| ​Preamble/midamble length | 7-bit sequence​ | 31-bit sequence​ |
| LSB = 0​ | Ibit = 33​ | Ibit = 66​ |
| LSB = 1​ | Ibit = 66​ | Ibit = 132​ |

- For coding and block-level repetition, coding rate and repetition number will be discussed in 9.4.2 but the signaling will be decided in 9.4.4.

  - As coding gain is greater than block repetition, it is generally not recommended to configure repetition greater than 1 when using no FEC or FEC with a higher coding rate. For example, we consider joint indication.

- Alt1: 2bits for {no FEC, FEC1/2, FEC1/3, FEC1/3+rep}

- Alt2: 2bits for {no FEC, FEC 1/3, FEC 1/3+rep2, FEC 1/3+rep4}

- Alt3: 2bits for {no FEC, FEC 1/3, FEC 1/3+rep2, reserved}

- For TBS of PDRCH, RAN1 has agreed 7bits as a working assumption, which is to cover 1~128bytes with granularity of 1byte. However, we think the TBS for different D2R message may have different range.

- For CBRA Msg1, it only includes RN16 in Msg1. The predefined TBS can be derived based on the message type indication for CBRA paging.

- For CFRA Msg1, it can indicate a variable TBS for Msg1 to carry device a part of higher layer device ID. In this case, the TBS will not be as large as 128bytes. Note that RAN2 has agreed no segmentation is applied to Msg1. The TBS size for CFRA Msg1should be limited.

- For Msg3, it can be similar as CFRA Msg1 to carry device a part of higher layer device ID. Note that RAN2 has agreed no segmentation is applied to Msg3. The TBS size for Msg3 should also be limited.

- For D2R data, this is the only use case that we may need to consider max TBS of 128byptes. The 7-bit indication for TBS can be included for the unicast D2R data scheduling.

- For D2R feedback corresponding to R2D data (discussed in 4.1.1), a predefined D2R TBS could be sufficient.

For example:

For R2D paging,

  - L1 control has separate CRC for early indication/termination

    - Msg type: [3~4]bits

    - Transaction ID: [2]bits

    - TBS of PRDCH: [4~5]bits

    - R2D chip duration same for L1 control and data: 1bit

    - CRC: 6bits

  - R2D MAC PDU (paging/R2D trigger) can carry

    - Paging identifier

    - RAI of D2R (for one or multiple Msg1s).

Fig. 8 R2D paging/R2D trigger

For R2D Msg2,

  - L1 control has separate CRC for early indication/termination

    - Msg type: [3~4]bits

    - Transaction ID: [2]bits

    - Number of devices: [2]bits

    - Msg1 device ID(s): 16bits * Number of devices <=64bits

    - TBS of PRDCH: [4~5]bits

    - R2D chip duration same for L1 control and data: 1bit

    - CRC: 16bits

  - R2D MAC PDU for Msg2 can carry

    - RAI for D2R (for one or multiple Msg3s)

    - AS ID if allocated.

Fig. 9 R2D Msg2

### R2D control in L1 and higher layer

Based on the discussion in 4.1.1 and 4.2.1, L1 control with separate CRC should include the scheduling information related to resource allocation for R2D transmission and msg type/transaction/device ID for early indication/termination.

Proposal 15:

- The Msg type, transaction ID, device ID are indicated in R2D L1 control with separate CRC to enable early indication / termination.

Proposal 16:

- The TBS of PRDCH is indicated in L1 control, based on which the device can determine the CRC length and the end of PRDCH before padding if any.

For scheduling information related to resource allocation for D2R transmission, it will be indicated by higher layer. We propose

Proposal 17: For D2R FDRA, the D2R with or without FDMA should be considered, respectively.

  - For unicast D2R, Y=1 no FMDA. FDRA needs to indicate

- TbitD2R: 3bits

- R: predefined as R=1 for TbitD2R=720kbps and R=2 otherwise.

  - For CBRA Msg1: Y≤8  FDMA can be supported

  - TbitD2R: 3bits

  - R and Y: FFS bitmap, codepoint, Y+Rmin+gap, or Y+Rmax+rule.

  - For Msg3 or CFRA Msg1: Y≤4 FDMA can be supported

  - TbitD2R: 3bits

  - R and Y: FFS bitmap, codepoint, Y+Rmin+gap, or Y+Rmax+rule.

Proposal 18: For D2R coding and block-level repetition, joint indication is supported. RAN1 to down select one of alternatives

- Alt1: 2bits for {no FEC, FEC1/2, FEC1/3, FEC1/3+rep}

- Alt2: 2bits for {no FEC, FEC 1/3, FEC 1/3+rep2, FEC 1/3+rep4}

- Alt3: 2bits for {no FEC, FEC 1/3, FEC 1/3+rep2, reserved}

Proposal 19: For TBS of PDRCH

- Confirm the following WA for PDRCH with modification:

  - Working assumption

    - For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size for unicast D2R data scheduling: 7 bits for byte-level D2R payload size indication

- For CFRA Msg1 and Msg3, TBS with variable size: [4]bits for byte-level D2R payload up to [12]bytes.

- Send LS to RAN2 to confirm the TBS of PDRCH.

## D2R control information/functionality

### D2R TBS indication for D2R data transmission

For D2R, RAN2 has agreed to support MAC padding:

- The MAC padding is supported at least for D2R from RAN2 perspective.   The device includes padding bits if there is no more data and there is still space available in the TBS.

- In case where MAC PDU includes both MAC SDU and padding, for D2R a field to indicate how many SDU bits are present is required.  FFS how this is provided (i.e. SDU length field or padding length field).  The size of length field is FFS.

Here, the MAC padding is for the case of less TBS than the indicated D2R TBS. However, there may be the case that the device may not have sufficient energy to transmit D2R indicated TBS. There should be a way to enable the device to adjust the TBS when the reader assigns a TBS with a duration exceeding the device's available time. In this scenario, the device cannot follow the R2D scheduled TBS. The device can transmit the D2R data with a smaller TBS within the allocated PDRCH resources and no MAC padding is added in this case. By using D2R TBS adaptation, the device still responds to the R2D trigger but transmit the actual TBS smaller than the TBS indicated by the reader. The actual D2R TBS can be indicated in MAC control or MAC header.

Proposal 20: For D2R data transmission, the transmitted TBS can be smaller than that of TBS indicated by the reader.

# Conclusion

In summary, we have the following proposals for 9.4.4:

For configuration of TDMA/FDMA:

Proposal 1: For FDMA of multiple CFRA Msg1 or Msg3 transmissions in response to a R2D transmission, RAN1 to decide

- Opt1: support same data rate for FDMed D2R transmission with same TBS

- Opt2: support different data rate for FDMed D2R transmission with different TBS if supported

Proposal 2: For Msg1 with X=2, RAN1 to down select

- Opt1: support same Msg1 transmission time for 1st and 2nd time domain resource

- Opt2: support shorter Msg1 transmission time in the 1st time domain resource than that of 2nd one

Proposal 3: For CBRA Msg1 or CFRA Msg1 with X=2 and Y>1, down-select

- Opt1: same FDMA configuration is configured for 1st and 2nd time domain resource

- Opt2: different FDMA configuration can be configured for 1st and 2nd time domain resource

For TR→D:

Proposal 4: TR→D is counted from the end of the last bit of PRDCH (before the start of the padding) till the start of D2R preamble of corresponding D2R transmission.

- The end of the last bit for PRDCH can be determined based on the TBS indicated in R2D L1 control.

Proposal 5: For D2R with X=1, from device perspective, the starting time for the D2R transmission is TR→D=Toffset1 after the end of the corresponding R2D transmission.

Proposal 6: For TR→D=Toffset1, it can be implicitly derived based on the R2D chip duration TchipR2D, D2R chip duration TchipD2R, and additional processing time if needed, as

- TR→D=Toffset1=max[4]TchipR2D, [10]TchipD2R+Tadd_proc

- For PDRCH without FEC, Tadd_proc=0

- For PDRCH with FEC,

  - Alt1: attach CRC not at the end, Tadd_proc=0

  - Alt2: attach CRC at the end,

    - For Msg1/Msg3/D2R feedback, apply TBS restriction of [up to 12bytes] or use precalculated CRC, Tadd_proc=0 .

    - For D2R data transmission, Tadd_proc=max{0,TprocFEC-TpreambleD2R} with D2R preamble time TpreambleD2R and TprocFEC processing time for CRC/FEC.

      - Tproc=FEC=TBS+6/Rsampling with sampling rate Rsampling

Proposal 7: For Msg1/Msg3 with FDMA, the chip duration TchipD2R of the smallest frequency shift for the FDMA is used to derive Toffset1.

Proposal 8: For TR→D=Toffset1+Toffset2, it can be implicitly derived based on Toffset1 and Msg1 transmission time TMsg1_time1 in the 1st time domain resource as Toffset1+Toffset2=Toffset1+TMsg1_time1∙1.2.

Proposal 9: For Msg1 with X=2 and FDMA, the common Msg1 transmission time in the 1st time domain resource is used to derive TR→D=Toffset1+Toffset2.

Proposal 10: For Msg1 with X=2, TR→D=Toffset1 or TR→D=Toffset1+Toffset2 is randomly selected for CBRA Msg1 or indicated for each CFRA Msg1.

For TD→R:

Proposal 11: For R2D response associated with a single D2R transmission, TD→R can be counted from the end of the last bit of the D2R transmission till the potential start of the corresponding R2D transmission, where the start of the R2D transmission is the first rising edge of R-TAS.

Proposal 12: For Msg2 corresponding to one or multiple Msg1 transmissions, TD→R can be counted from the end of the last Msg1 transmission resource(s) till the potential start of the corresponding Msg2 transmission.

- For Msg1 with X=1 and FDMA, the TMsg1_time1 in the 1st time domain resource is used to derive the start of TD→Rmin as (Toffset1+TMsg1_time1).

- For Msg1 with X=2 and FDMA, the TMsg1_time2 in the 2nd time domain resource is used to derive the start of TD→Rmin as (Toffset1+Toffset2+TMsg1_time2).

Proposal 13: TD→Rmin=[3]TchipD2R, where TchipD2R is the chip duration.

- For Msg1/Msg3 with FDMA, the chip duration TchipD2R of the smallest frequency shift for the FDMA is used to derive TD→Rmin.

Proposal 14: Define TD→Rmax=[20]TchipD2R at least for R2D monitoring corresponding to single D2R transmission.

For R2D L1 control:

Proposal 15:

- The Msg type, transaction ID, device ID are indicated in R2D L1 control with separate CRC to enable early indication / termination.

Proposal 16:

- The TBS of PRDCH is indicated in L1 control, based on which the device can determine the CRC length and the end of PRDCH before padding if any.

For scheduling information related to resource allocation for D2R transmission, it will be indicated by higher layer. We propose

Proposal 17: For D2R FDRA, the D2R with or without FDMA should be considered, respectively.

  - For unicast D2R, Y=1 no FMDA. FDRA needs to indicate

- TbitD2R: 3bits

- R: predefined as R=1 for TbitD2R=720kbps and R=2 otherwise.

  - For CBRA Msg1: Y≤8  FDMA can be supported

  - TbitD2R: 3bits

  - R and Y: FFS bitmap, codepoint, Y+Rmin+gap, or Y+Rmax+rule.

  - For Msg3 or CFRA Msg1: Y≤4 FDMA can be supported

  - TbitD2R: 3bits

  - R and Y: FFS bitmap, codepoint, Y+Rmin+gap, or Y+Rmax+rule.

Proposal 18: For D2R coding and block-level repetition, joint indication is supported. RAN1 to down select one of alternatives

- Alt1: 2bits for {no FEC, FEC1/2, FEC1/3, FEC1/3+rep}

- Alt2: 2bits for {no FEC, FEC 1/3, FEC 1/3+rep2, FEC 1/3+rep4}

- Alt3: 2bits for {no FEC, FEC 1/3, FEC 1/3+rep2, reserved}

Proposal 19: For TBS of PDRCH

- Confirm the following WA for PDRCH with modification:

  - Working assumption

    - For indicating the payload size (i.e. TBS-like) for PDRCH transmission with variable size for unicast D2R data scheduling: 7 bits for byte-level D2R payload size indication

- For CFRA Msg1 and Msg3, TBS with variable size: [4]bits for byte-level D2R payload up to [12]bytes.

- Send LS to RAN2 to confirm the TBS of PDRCH.

For D2R TBS for D2R data:

Proposal 20: For D2R data transmission, the transmitted TBS can be smaller than that of TBS indicated by the reader.

# Reference

[1] R1-2504395, Physical channels design – line coding, FEC, CRC, repetition aspects, Qualcomm, 3GPP TSG RAN1#121, St Julian’s, Malta, May 19th – 23th, 2025.

[2] R1-2504396, Timing acquisition and synchronization, Qualcomm, 3GPP TSG RAN1#121, St Julian’s, Malta, May 19th – 23th, 2025.