3GPP TSG RAN WG1 #121			   									 R1-2504421

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:	9.15.3

Source: 	Qualcomm Incorporated

Title: 	UE features for evolution of NR duplex operation

Document for:	Discussion/Decision

# Introduction

This contribution provides Qualcomm’s views regarding UE features for supporting Rel-19 Duplex evolution (SBFD) and CLI enhancement.

# Discussion

## UE features for supporting gNB SBFD operation

In RAN1 #120bis, the following UE FG was agreed for UE supporting Rel-19 Duplex evolution (SBFD) [2].

Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs 60-2; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Adopt “Per band” type for FG 60-1. Agreement: Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] Agreement: Update FG 60-1 as follows: - Confirm “cell-common” in component 1 - Confirm “UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols” - Confirm Component 3: “3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling” - Add “Support of separate CSI reports for SBFD and non-SBFD symbols 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling.

### Basic SBFD-aware UE FG 60-1

A basic UE feature for supporting gNB SBFD operation should include the basic functionalities and scheduling enhancement to support efficient UL transmission and DL reception in SBFD symbols based on at least the following agreements made in the AI 9.3.1 of Rel-19 duplex enhancement (SBFD). The definition of components 1-6 and component 8 for were agreed. Regarding the number of DL subbands for component 2, it is preferred to have two candidates values {1,2} to indicate the number of supported DL subbands rather than a separate UE FG. In addition, we support to confirm component 7 to support separate power control for UL transmission in SBFD symbols and component 9 to support separate configurations of SRS resource sets for SBFD symbols.

Proposal 1: Support the following for the basic SBFD-aware UE feature (FG 60-1):

- Candidate values of the number of DL subbands for component 2 are {1,2} and remove the square brackets.

- Remove FFS of component 7: Support separate PC parameters for UL transmission in SBFD symbols by additional PC parameters configuration within unified TCI state for SRS/PUCCH/PUSCH in SBFD symbols

- Remove FFS of component 9 and add a note that separate SRS resources set only applicable for SRS usages ‘codebook ,’non codebook’ and ‘beamManagement’.

| 60. NR_duplex_evo | 60-1 | Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures | 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one/two DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols 6. Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols 8. Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols FFS: other component (which may be separate FG) |  | YES | n/a | Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported | Per band | TDD only | n/a | n/a | [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.] Component 2 candidate values for the number of DL subbands {1,2} Component 9 only applicable for SRS sets with usage codebook, non-codebook and beam-management. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### UE FG 60-2 for the support of configuration #2 for UL and DL

For UL transmission (or DL reception) across SBFD and non-SBFD symbols in different slots, the support of configurations #2 should be separate UE FG. UE should report separately the support of configuration #2 for UL transmission and DL reception as they are also separately configured per UL/DL BWP basis.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

Proposal 2: Support a UE feature (FG X-2a) for separately supporting configuration # 2 of UL transmissions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols).

| 60. NR_duplex_evo | 60-2 | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) |  | YES | n/a | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band] | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60. NR_duplex_evo | 60-2a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PUCCH and PUSCH transmission across SBFD symbols and non-SBFD symbols (Configuration 2) |  | YES | n/a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |

### UE FG for non-contiguous CSI-RS across DL subbands

The support of non-contiguous CSI-RS across two downlink subbands and corresponding CSI processing timeline should be a separate UE FG, based on the following RAN1.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy.

Proposal 3: Support a UE feature (FG 60-2b) for supporting non-contiguous CSI-RS resource across two DL subband with one of the following capabilities:

- Cap1: using legacy CSI processing timeline and CPU counting are the same as legacy

- Cap2: using relaxed CSI processing timeline Z/Z’ by factor of 2 and same CPU counting as legacy.

| 60. NR_duplex_evo | 60-2b | Non-contiguous CSI-RS in SBFD symbols | Support of non-contiguous CSI-RS resource across two DL subband with one of the following capabilities: Cap1: using legacy CSI processing timeline and CPU counting are the same as legacy Cap2: using relaxed CSI processing timeline Z/Z’ by factor of 2 and same CPU counting as legacy. |  | YES | n/a | Non-contiguous CSI-RS in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### UE FG for maximum number of partial PRGs in SBFD symbols.

Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols.

Proposal 4: Support a UE feature (FG 60-2C) for supporting maximum number of supported partial PRGs in SBFD symbols with component values {2,4}.

| 60. NR_duplex_evo | 60-2c | Partial PRGs for PDSCH Scheduling in SBFD symbols | Maximum of four partial PRGs for PDSCH in SBFD symbols. |  | YES | n/a | Maximum of four partial PRGs for PDSCH in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### UE FG on Maximum number of DL/UL switching

As the SBFD-aware UE is half duplex, it may frequently switch between DL and UL mode during SBFD slot and/or the SBFD pattern. This will incur some many DL-UL switching as compared to legacy semi-static TDD operation where only UE would switch once for DDDSU TDD pattern. RAN1 should discuss some limitation on the maximum number of DL/UL switching at UE side as frequency switching will affect UE peak throughput.

In current NR specification, component 9 of FG 5-1 puts a limitation of maximum one switching point per flexible slot in TDD deployment as a baseline while FG 5-1b allows for more than one DL/UL switch point in a slot. Then, similarly for an SBFD slot, the maximum number of switching points per SBFD slot should be one. Additionally, there should be a limit on the maximum number of DL/UL switching point per SBFD pattern, otherwise many of the symbols will be wasted for frequency DL-UL switching.  Example in the figure below shows a scenario where UE needs to switch in every SBFD slot from DL mode (PDCCH reception) to UL mode (PUSCH transmission) with 1-2 symbols are not utilized per each SBFD slot to accommodate Tx-Rx switching and UE TA advance.

Figure 2-1: Example of DL-UL switching per each SBFD slot with 1-2 symbols gap for Rx-Tx switching.

Proposal 5: Support a UE feature for reporting the maximum number of DL-UL switching in SBFD slot, and SBFD/TDD period.

| 60. NR_duplex_evo | 60-2e | Maximum number of DL-UL switching in SBFD slots. | 1. The maximum number of DL-UL switching in SBFD slot. 2. The maximum number of DL-UL switching in SBFD/TDD period |  | YES | n/a |  | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### UE FG on supported UL/DL SB configurations

For DUD pattern where the UL subband is in the middle of the carrier, designing a receiver notch filter to suppress the uplink subband is challenging than a filter for a single DL subband. Additionally, some low tier UEs can’t accommodate DL reception across the two DL subbands. This motivates UE reporting of single DL subband scheduling. In addition, SBFD-aware would require a UE-specific guardband to accommodate the UE receiver filters requirement which is different than gNB cell-specific guardband [1]..

Proposal 6: Support a UE feature UE reporting the supported minimum guardband between DL and UL subbands.

| 60. NR_duplex_evo | 60-2d | Guardband between the UL/DL subband | Minimum guardband between UL and DL subband. |  | YES | n/a |  | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## UE features for supporting random-access in SBFD symbols

For random-access in SBFD symbols, CBRA and CFRA using Type-1 random access procedure (4-step RACH) is supported. where two PRACH configuration options were introduced. PRACH configuration option (1) is based on using a single shared legacy RACH configuration where the additional-ROs in SBFD-DL symbols can be valid for SBFD-aware UE. The PRACH configuration options (2) is based on using two separate PRACH configurations, one legacy PRACH configuration indicating legacy-ROs and additional PRACH configuration indicating additional-ROs for random-access in SBFD symbols. For either PRACH configuration option 1 or PRACH configuration option 2, the ROs used for SBFD RA is referred as ‘additional-ROs’.

In RAN1 #120, the following UE FGs was agreed for UE supporting Rel-19 SBFD random access with per-band granularity reporting [2].

Agreement: Adopt “Per band” type for FG 60-3. Agreement: Adopt “Per band” type for FG 60-4 Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) X-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; X-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. X-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [X-1, X-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Per RAN1 agreement, it is up to UE capability reporting to support either or both PRACH configuration. In addition, the support of PRACH transmission with preamble repetitions within additional-ROs of either PRACH configuration should be reported separately for PRACH configuration option 1 and PRACH configuration option 2. Finally, msg3 repetition in SBFD symbols based on configuration #1 should be reported separately.

Proposal 7. Support the following list of UE capabilities for random access in SBFD symbols

| Index | Feature group | Components |  | Consequence if the feature is not supported by the UE | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60-3 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | 1. Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) 2. Support of PRACH transmission without repetition within additional valid-RO in SBFD symbols configured as downlink by tdd-UL-DL-ConfigurationCommon 3. Support of separate PC parameters for PRACH transmission in the additional-ROs. 4. Separate mapping of SSB to the additional ROs and separate determination of the association period and association pattern period 5. Support of CBRA switching procedures between PRACH transmission in additional-ROs and legacy-ROs. 6. Support of explicit indication of RO-type (legacy-RO or additional-RO) for CFRA by PDCCH order. | 60-1 | SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band |  | Optional with capability signalling |

| 60-3a | PRACH repetition in additional-RO of PRACH configuration option 1 | 1. Support of PRACH transmission with preamble repetition in the additional ROs using the same number of repetitions {2,4,8} for PRACH transmission with preamble repetitions within legacy-ROs. 2. Support separate configuration of RSRP thresholds for PRACH transmission with preamble repetitions within additional-ROs | 60-3 | UE doesn’t support PRACH transmission with preamble repetition in additional ROs based on legacy PRACH configuration (PRACH configuration option 1) | Per band |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60-4 | SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode | 1. Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) 2. Support of PRACH transmission without repetition within the valid-RO in SBFD symbols configured by the SBFD-specific PRACH configuration. 3. Support separate mapping of SSB to the additional ROs and separate determination of the association period and association pattern period 4. Support of CBRA switching procedures between PRACH transmission in additional-ROs (SBFD-dedicated PRACH configuration) and legacy-ROs (common PRACH configuration) 5. Support of explicit indication of RO-type for CFRA triggered by PDCCH order. | 60-1 | SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported | Per band |  | Optional with capability signalling |
| 60-4a | PRACH repetition in additional-RO of PRACH configuration option 2 | 1. Support of PRACH transmission with preamble repetition in the additional ROs of SBFD PRACH configuration (PRACH configuration option 2). 2. support separate configuration of RSRP thresholds and repetition numbers {2,4,8} for PRACH transmission with preamble repetitions within additional-ROs | 60-4 | UE doesn’t support PRACH transmission with preamble repetition in additional ROs based on SBFD PRACH configuration (PRACH configuration option 2). | Per band |  | Optional with capability signalling |
| 60-4b | RO across SBFD and non-SBFD symbols | Support of PRACH transmission in RO across SBFD and non-SBFD symbols | 60-4 | UE doesn’t support PRACH transmission in RO across SBFD and non-SBFD symbols | Per band |  | Optional with capability signalling |
| 60-5 | MSG3 transmission with repetition in SBFD symbols | 1. Support of Repetition of msg3 PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with separate power control parameters. | 60-3 or 60-4 | UE doesn’t support the Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols | Per band |  | Optional with capability signalling |

## UE features for supporting UL resource muting on PUSCH

For UL resource muting, RAN1 agreed to support indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain. To determine the time location of UL muting symbols in a slot for a DG PUSCH and type 2 CG PUSCH, RAN1 agreed on that the time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI. And for frequency allocation, if UL resource muting is applied on a symbol for a PUSCH, a comb-2 pattern is applied for all of the allocated PRBs of the PUSCH on that symbol.  And a comb offset {0,1} is configured for CP-OFDM and further discussion for DFT-s-OFDM. And 3dB power boosting is assumed for REs in the PUSCH symbol not containing PT-RS with UL resource muting.

In RAN1 #120bis meeting, RAN1 agreed that:

Agreement Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Proposal 8: Support UE feature for SBFD-aware UE of the following list of UE capabilities for the RE level UL resource muting with comb-2 pattern for PUSCH.

| Index | Feature group | Components | Prerequisite FG | Consequence if the feature is not supported by the UE | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7 | UL resource muting for CP-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH | N/A | YES | n/a | UL resource muting for CP-OFDM waveform is not supported | Per Band |
| 60-7a | UL resource muting for DFTS-OFDM waveform | 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol - Support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH | N/A | YES | n/a | UL resource muting for DFTS-OFDM waveform is not supported | Per Band |

## UE features for L1-CLI measurements and reporting

For L1 CLI measurement and reporting, RAN1 agreed on details of configuration of aperiodic, periodic and semi-persistent SRS-RSRP and CLI-RSSI measurement resources, configuration/determination of QCL typed per aperiodic, periodic and semi-persistent SRS-RSRP and CLI-RSSI measurement resource, and also on details of configuration of at least aperiodic reporting. RAN1 also agreed on Method #1 that UE measures RSSI within DL subband, Method #2 that UE measures RSRP of aggressor UE within UL subband, and Method #3 for CLI measurement that UE measures RSSI within UL subband. We also support have a UE FG for minimum slot offset configuration for aperiodic CLI-RSSI-MeasurementResourceSet and please refer to our contribution for CLI handling for the reasoning.

In RAN1 #120bis meeting, RAN1 agreed that:

Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

Proposal 9: Support UE feature for SBFD-aware UE of the following list of UE capabilities for L1 UE-to-UE CLI measurement and reporting:

| Index | Feature group | Components | Prerequisite FG | Consequence if the feature is not supported by the UE | Type | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| X-7 | L1 based CLI-RSSI aperiodic reporting | The support of aperiodic CLI-RSSI reporting using aperiodic CLI-RSSI resources. | 2-35,2-32 | UE does not support L1 CLI-RSSI measurement and reporting | Per band | Note: CLI-RSSI reports are counted towards the maximum number CSI report (the simultaneousCSI-ReportsPerCC) | Optional with capability signalling |
| X-7a | Slot offset | The support of minimum slot offset configuration for aperiodic CLI-RSSI-MeasurementResourceSet | X-4-1 |  |  | candidate values: [1,32] The candidate value shall be per frequency range based. | Optional with capability signalling |
| X-7b | L1 based CLI -RSSI resources | 1. The maximum number of configured CLI-RSSI measurement resources per CC. 2. The maximum number of configured CLI-RSSI measurement resources across all CCs. 3. The maximum number of simultaneous CLI-RSSI resources per CC 4. The maximum number of simultaneous CLI-RSSI resources in active BWPs across all CCs 5. The maximum number of CLI-RSSI measurements resource per slot per CC 6. The maximum number of CLI-RSSI measurement resource per slot across all CC | X-4-1 | UE does not support L1 CLI-RSSI measurement and reporting | Per band | Components 1-6 candidate values: TBD | Optional with capability signalling |
| X-7c | Periodic CLI-RSSI resource | Support of periodic CLI resources for L1-based aperiodic CLI reporting | X-4-1 | UE does not support periodic CLI-RSSI resources | Per band |  | Optional with capability signalling |
| X-7d | Semi-persistent CLI-RSSI resource | Support of semi-persistent CLI resources for L1-based aperiodic CLI reporting | X-4-1 | UE does not support semi-persistent CLI-RSSI resources | Per band |  | Optional with capability signalling |
| X-7e | Periodic CLI-RSSI resource | Support of periodic CLI resources for L1-based periodic CLI reporting | X-4-1 | UE does not support periodic CLI-RSSI resources for periodic reporting | Per band |  | Optional with capability signalling |
| X-8 | L1 based CLI SRS-RSRP aperiodic reporting | The support of aperiodic SRS-RSRP reporting using aperiodic SRS-RSRP resources. | 2-35,2-32 | UE does not support L1 SRS-RSRP measurement and reporting. |  | Note: CLI SRS-RSPR reports are counted towards the maximum number CSI report (the simultaneousCSI-ReportsPerCC) | Optional with capability signalling |
| X-8a | Slot offset | The support of minimum slot offset configuration for aperiodic SRS-RSRP-MeasurementResourceSet | X-4-2 |  |  | candidate values: [1,32] The candidate value shall be per frequency range based. | Optional with capability signalling |
| X-8b | L1 based CLI-SRS-RSRP resources | 1. The maximum number of configured CLI-SRS-RSRP measurement resources per CC. 2. The maximum number of configured CLI-SRS-RSRP measurement resources across all CCs. 3. The maximum number of simultaneous CLI-SRS-RSRP resources per CC 4. The maximum number of simultaneous CLI-SRS-RSRP resources in active BWPs across all CCs 5. The maximum number of CLI-SRS-RSRP measurements resource per slot per CC 6. The maximum number of CLI-SRS-RSRP measurement resource per slot across all CC | X-4-2 | UE does not support L1 SRS-RSRP measurement and reporting. | Per band | Components 1-6 candidate values: TBD | Optional with capability signalling |
| X-8c | Periodic SRS-RSRP resource | Support of Periodic CLI SRS-RSRP resources for L1-based aperiodic CLI reporting | X-4-2 | UE does not support periodic CLI SRS-RSRP resources | Per band |  | Optional with capability signalling |
| X-8d | Semi-persistent SRS-RSRP resource | Support of semi-persistent CLI SRS-RSRP resources for L1-based aperiodic CLI reporting | X-4-2 | UE does not support semi-persistent CLI SRS-RSRP resources | Per band |  | Optional with capability signalling |
| X-9 | FDM DL reception and L1 CLI-RSSI measurement (Method #3) | Simultaneous reception of DL signals/channels and CLI-RSSI measurement resource | X-4-1 | UE doesn’t support Simultaneous reception of DL signals/channels and CLI RSSI measurement resource | Per band |  | Optional with capability signalling |
| X-10 | FDM DL reception and L1 SRS-RSRP measurement (Method #2) | Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | X-4-2 | UE doesn’t support Simultaneous reception of DL signals/channels and SRS-RSRP measurement resource | Per band |  | Optional with capability signalling |

# Reference

- R1-2501153, “SBFD Transmission, Reception and Measurement Procedures”, Qualcomm Incorporated.

- R1-2503068, Summary#2 of discussion on Rel-19 NR UE features for SBFD, NTT DOCOMO, INC

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, Moderators (AT&T, NTT DOCOMO, INC.)