3GPP TSG RAN WG1 #121			R1-2504487

St Julian’s, Malta, May 19th – 23th, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on reply LS on L1 based UE-to-UE CLI measurement and reporting

Agenda Item:	5

Document for: 	Discussion and Decision

- Introduction

RAN4 sent an reply LS in [1] to RAN1 on L1 CLI measurement and reporting. In this LS, RAN4 provided answers to the questions from RAN1. In this contribution, our views based on the reply LS are provided.

- Discussions

Reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI

Based on the reply LS, RAN4 considers the reporting range and granularity for L1 SRS-RSRP and L1 CLI-RSSI in the RAN1 working assumption as reasonable from RAN4 perspective. And RAN4 considers the range and resolution for differential reporting of L1 SRS-RSRP and L1 CLI-RSSI reporting in the RAN1 working assumption as reasonable from RAN4 perspective. Therefore, the following working assumption should be confirmed.

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

Proposal 1: Confirm the following RAN1 working assumption.

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

Out-of-range reporting

In the reply LS, RAN4 also confirms that the “Infinite” in Table 10.1.22.1.2-1, TS38.133 is also reused for L1 SRS-RSRP.

Table 10.1.22.1.2-1: SRS-RSRP measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| SRS-RSRP_0 | SRS-RSRP<-140 | dBm |
| SRS-RSRP_1 | -140≤ SRS-RSRP<-139 | dBm |
| SRS-RSRP_2 | -139≤ SRS-RSRP<-138 | dBm |
| SRS-RSRP_3 | -138≤ SRS-RSRP<-137 | dBm |
| SRS-RSRP_4 | -137≤ SRS-RSRP<-136 | dBm |
| .. | .. | … |
| SRS-RSRP_95 | -46≤ SRS-RSRP<-45 | dBm |
| SRS-RSRP_96 | -45≤ SRS-RSRP<-44 | dBm |
| SRS-RSRP_97 | -44≤ SRS-RSRP | dBm |
| SRS-RSRP_98 | Infinity |  |
| NOTE: ‘Infinity’ means that UE cannot detect SRS due to too strong signal to measure. |  |  |  |

At the RAN1#120bis meeting, out-of-range reporting for L1 SRS-RSRP was discussed, while no consensus was reached.

Proposal 2-9 If the number of reported L1-SRS-RSRP measurement resources is M, and the number of L1-SRS-RSRP measurements which are either >=-44dBm or infinity (infinity means UE cannot detect SRS due to too strong signal to measure) is X (X<=M), down-select from the following alternatives Alt.1: The following are reported within one report - An indicator corresponding to a codepoint mapping to out-of-range (L1-SRS-RSRP >= -44 dBm) or infinity: 7 bits - A list of “out-of-range” indications based on differential measurement reporting table are reported for each of the X-1 L1-SRS-RSRP measurement: (X-1)*4bits - A codepoint defined as ‘out-of-range’ is introduced in the differential measurement reporting table - For the M-X L1-SRS-RSRP measurements which are within the reporting range (L1-SRS-RSRP <-44Bm), a differential reporting method is adopted using -44dbm as the reference: (M-X)*4bits Alt.2: The following are reported within one report - An indicator corresponding to a codepoint mapping to out-of-range (L1-SRS-RSRP >= -44) or infinity: 7 bits - For the M-1 L1-SRS-RSRP measurement, a differential reporting method is adopted using -44dbm as the reference: (M-1)*4bits

According to the reply from RAN4, out-of-range or infinity reporting should be supported for L1 SRS-RSRP.

Regarding the two alternatives for out-of-range or infinity reporting, Alt 2 allows only one strongest value to indicate out-of-range or infinity, while Alt 1 allows more than 1 value to indicate out-of-range or infinity. Alt 1 seems to be more accurate reporting as it is possible that more than one measurement resource is not measurable due to too strong signal. Therefore, Alt 1 is preferred.

Proposal 2: Support out-of-range or infinity reporting for L1 SRS-RSRP.

- If the number of reported L1-SRS-RSRP measurement resources is M, and the number of L1-SRS-RSRP measurements which are either >=-44dBm or infinity (infinity means UE cannot detect SRS due to too strong signal to measure) is X (X<=M),

  - An indicator corresponding to a codepoint mapping to out-of-range (L1-SRS-RSRP >= -44 dBm) or infinity: 7 bits

  - A list of “out-of-range” indications based on differential measurement reporting table are reported for each of the X-1 L1-SRS-RSRP measurement: (X-1)*4bits

    - A codepoint defined as ‘out-of-range’ is introduced in the differential measurement reporting table

  - For the M-X L1-SRS-RSRP measurements which are within the reporting range (L1-SRS-RSRP <-44Bm), a differential reporting method is adopted using -44dbm as the reference: (M-X)*4bits

- Conclusion

In this contribution, we discussed L1 CLI measurement and reporting based on the reply LS from RAN4. Based on the discussion, we made the following proposals.

Proposal 1: Confirm the following RAN1 working assumption.

Working assumption For L1 based CLI measurement and reporting, the following is adopted - For L1 SRS-RSRP, a 7-bit value in the quantization range [-140, -44] dBm with 1dB step size, and the differential L1 SRS-RSRP is quantized to a 4-bit value. The differential L1 SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1 SRS-RSRP value. - For L1 CLI-RSSI, a 7-bit value in the quantization range [-100, -25] dBm with 1dB step size, and the differential L1 CLI RSSI is quantized to a 4-bit value. The differential L1 CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1 CLI-RSSI value.

Proposal 2: Support out-of-range or infinity reporting for L1 SRS-RSRP.

- If the number of reported L1-SRS-RSRP measurement resources is M, and the number of L1-SRS-RSRP measurements which are either >=-44dBm or infinity (infinity means UE cannot detect SRS due to too strong signal to measure) is X (X<=M),

  - An indicator corresponding to a codepoint mapping to out-of-range (L1-SRS-RSRP >= -44 dBm) or infinity: 7 bits

  - A list of “out-of-range” indications based on differential measurement reporting table are reported for each of the X-1 L1-SRS-RSRP measurement: (X-1)*4bits

    - A codepoint defined as ‘out-of-range’ is introduced in the differential measurement reporting table

  - For the M-X L1-SRS-RSRP measurements which are within the reporting range (L1-SRS-RSRP <-44Bm), a differential reporting method is adopted using -44dbm as the reference: (M-X)*4bits

References

- R1-2503214, Reply LS on L1 based UE-to-UE CLI measurement and reporting

- 3GPP, RAN1#118, RAN1 Chairman’s Notes, August 2024.

- 3GPP, RAN1#118bis, RAN1 Chairman’s Notes, October 2024.

- 3GPP, RAN1#119, RAN1 Chairman’s Notes, November 2024.

- 3GPP, RAN1#120, RAN1 Chairman’s Notes, February 2025.

- 3GPP, RAN1#120bis, RAN1 Chairman’s Notes, February 2025.