3GPP TSG RAN WG1 #121	R1-2504512

St Julian’s, Malta, May 19th – 23th, 2025

Title 	:  Discussion on channel model validation for 7-24 GHz

Source 	:  NTT DOCOMO, INC.

Agenda item	:  9.8.1

Document for:  Discussion

- Introduction

In RAN1#120-bis meeting, regarding to UMa pathloss model in TR38.901, there is conclusion as below [1], [2].

Conclusion - For the following scenarios, there is no consensus to update pathloss models due to lack of consistent and significant observed difference between model and measurements. - UMa LOS/NLOS

In this contribution, we discuss and provide our view on UMa pathloss model in TR38.901 for 7-24GHz.

- Discussion on Pathloss model of UMa

Here, we validate the pathloss model of UMa in TR38.901 by comparing with measurement results. A measurement campaign for pathloss at 2.2, 4.7, 26.4 and 66.5 GHz was conducted [3]. Fig. 1 and Table 1 show the Tx positions, measurement courses and measurement conditions. The Tx was set on the rooftop of buildings at positions Tx1 and Tx2 with heights of 25 m and 21 m from the ground, respectively. The average height of the buildings in measurement area is about 20 m. The Rx was set on the rooftop of a measurement car with 2.5 m height. The Rx was moved on the measurement courses to measure pathlosses at frequencies of 2.2, 4.7, 26.4, 66.5 GHz. After measurement, the median pathloss values at 10 m length were processed. Fig. 2 and 3 and Table 2 show the comparisons between measured pathloss results and calculated results using UMa pathloss model in TR38.901. In cases of frequencies 2.2, 4.7, 26.4 GHz, the comparison results show that the calculated results agree with measured results in both cases of LOS and NLOS. For the case of 66.5 GHz, the different between calculated results and measured results is large due to the small number of measured points. Overall, we can conclude that the UMa pathloss model in TR38.901 is validated and does not need to be updated.

Proposal 1: The pathloss model of UMa in TR38.901 is validated and does not need to be updated.

- Conclusion

In this contribution, we provided our views and discussions on channel model validation of TR38.901 for 7-24GHz related to UMa pathloss model. The following proposal is made:

Proposal 1: The pathloss model of UMa in TR38.901 is validated and does not need to be updated.

References

[1] R1- 2503066, Summary #3 of discussions for Rel-19 7-24 GHz Channel Modeling Validation

[2] TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

[3] M. Sasaki, et. al., “Extension of ITU-R Site-General Path Loss Model in Urban Areas Based on Measurements from 2 to 66 GHz Bands,” IEICE Transactions on Communications, vol. E104.B, no. 7, pp. 849–857, 2021.