3GPP TSG RAN WG1 #121	R1-2504516

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on support of RedCap and eRedCap UEs in FR1-NTN

Agenda Item:	9.11.2

Document for:	Discussion and Decision

- Introduction

At the RAN#104 meeting [1], the R19 NR-NTN WID was updated based on the checkpoint. For (e)RedCap perspective, enhancements will be specified, and the target is cases 3 and 4 only. On the other hand, introduced solution can be applied to other cases, if possible, and it may improve performance in those cases as well as cases 3 and 4. In this contribution, we share our further views for support of RedCap/eRedCap UEs in FR1-NTN.

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

At the RAN1#116 [2] / #116-bis [3] / #117 [4] / #118 [5] / #118bis [6] / #119 [7] / #120 [8] / #120bis [9] meetings, the following agreements/observations/conclusions were reached for this agenda item.

Agreement Study at least the following scenarios for (e)RedCap HD-FDD UEs for NTN: - Whether existing handling rules for the following cases should be reused or updated when taking into account TA mismatch between actual TA used by UE and assumed TA at the gNB based on available TA report: - Case 1: Dynamically scheduled DL reception collides with semi-statically configured UL transmission - Case 2: Semi-statically configured DL reception collides with dynamically scheduled UL transmission - Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Case 5: Configured SSB collides with dynamically scheduled or configured UL transmission - Case 6: Dynamic or semi-static DL collides with valid RO - Case 7: Collision due to direction switching - At least the following potential issues can be further considered for (e)RedCap HD-FDD UEs - Error cases in case 3 and case 4 - SIB19 reception collides with UL transmission - Slot counting for UL repetition transmission colliding with SSB reception - Invalid symbol determination for PUSCH repetition type B - Actual TDW determination due to the collision between DL reception and UL transmission with DMRS bundling - CPU occupation due to omitted DL reception or UL transmission Note: Both GSO and Non-GSO should be considered. Observation To avoid the occurrence of error cases 3 and 4 through network scheduling, there are less resources available for a scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB. Observation For collision cases 1, 2, 5 and 6, when there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there might be less resources available for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB attempts to avoid the collision or there is a loss of DL/UL transmissions due to collision. Observation When there is TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, there may be a BLER performance degradation for the reception of UL transmissions at the gNB for the scheduled HD-FDD RedCap/eRedCap UE in NTN compared to TN if gNB does not attempt to avoid the collision at least in the following cases: - UL transmission with repetitions due to different available slot counting at UE and gNB when colliding with SSB reception - PUSCH repetition type B due to different invalid symbol determination at gNB and UE when colliding with DL transmissions - UL transmission with DMRS bundling due to the different actual TDW determination at gNB and UE when colliding with DL transmissions Note: the above cases happen at least with one of collision cases 1, 2, 5, 6, and 7. Conclusion For Rel-19 HD-FDD RedCap/eRedCap UE in NTN, the issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB should be mitigated for collision cases 3 and 4. - Note: further discussion on other cases is not precluded Conclusion For collision cases 1, 2, 5 and 6, the existing priority rules can be reused for a HD-FDD (e)RedCap UE in NTN. Observation TA reporting is beneficial to mitigate the TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB for HD-FDD RedCap/eRedCap UE in NTN from RAN1 perspective. - Note: complexity, power consumption and signaling overhead impact of TA reporting for (e)redcap UEs was not investigated in this work item Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL Agreement When network indicates UL overriding DL in RRC-Connected mode for collision case 3, one UE-specific RRC parameter is used to indicate overriding for all the applicable other use cases except for collisions with Type-0/0A/1/2-PDCCH CSS. Conclusion For Rel-19 HD-FDD (e)Redcap UE with CG-SDT procedure ongoing in RRC-inactive mode, for collision case 3, - Handling of collision with PDCCH CSS in RRC-inactive mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for SI change indication in its own paging occasion(s) that the UE monitors as specified in TS 38.304 [20]. - UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for SI change indication in any paging occasion at least once per modification period, if the initial downlink BWP on which the SDT procedure is ongoing is associated with a CD-SSB. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is not ongoing shall monitor for indications about PWS notification in its own paging occasion every DRX cycle. - ETWS or CMAS capable UEs in RRC_INACTIVE while SDT procedure is ongoing shall monitor for indication about PWS notification in any paging occasion at least once every defaultPagingCycle. Working assumption For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A[/1][/2]-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that [DL or UL] is prioritized. - Network is allowed to indicate [UL or DL] overriding [DL or UL] for all cases - This is signaled by one UE specific RRC parameter - Note: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement Update the working assumption in RAN1 #120 with the following updates: For Rel-19 NTN HD-FDD (e)Redcap UE in RRC connected mode, the following handling rule for collision case 4 is supported: - Handling of collision with PDSCH (at least for system information) scheduled by Type-0/0A-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - Handling of collision with PDSCH scheduled by Type-1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the note2. - FFS: handling of PDCCH ordered PRACH transmission - For other use cases, default priority rule for collision case 4 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all cases - This is signaled by one UE specific RRC parameter - Note1: if DL is prioritized, the DL prioritization applies only if the UL cancellation timeline can be satisfied, otherwise UL is prioritized. Note2: UE shall comply to the following existing procedure in 38.331: - UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

- Discussion

  - Collision handling – Case 4

At RAN1#120bis meeting, the working assumption reached at RAN1#120 was updated. An FFS for PDCCH ordered PRACH is remaining. For this FFS, necessity of special handling for this rule is unclear. For UE-specific DL, gNB should avoid scheduling DL in the case where PDCCH ordered PRACH is necessary, i.e., when UE sync has trouble. For common DL, the first/second bullets are sufficient, i.e., UE can decide which RX of common DL or TX of PDCCH ordered PRACH should be performed. Which is more important at a given time for a UE is dependent on the UE’s situation. Fixing UE behavior does not bring any benefit even from NW perspective. If some companies prefer to prioritize this UL channel over DL always, the already-agreed NW configuration to prioritize UL can be used for the purpose.

Proposal 1:

- For HD-FDD RedCap/eRedCap UE in Case 4, NOT support special handling for PDCCH ordered PRACH, i.e.,

  - Confirm the working assumption with removing the FFS bullet.

  - UE behavior when UL drop occurs

Even though dropping behavior of either DL or UL has been defined for Case 3 and Case 4, the existing several features to be used for (e)RedCap UEs will not work due to the dropping behavior. One important note is, handling these problems by gNB implementation, e.g., collision avoidance by gNB or blind decoding at gNB, is not reasonable. Collision avoidance by gNB implementation is not aligned with why overlap handling for Case 3 and for Case 4 is to be introduced.

The following are explanations of the problems and solution proposals. With respect to solutions, we submit simpler ones as this meeting is the last of R19 discussion before maintenance phase and thus more complicated mechanism will not be accepted anymore.

- A) Actual TDW determination of PUSCH DMRS bundling

Fig. 1: Why DMRS bundling does not work in Case 4.

If DL reception is prioritized over UL transmission, a possible situation is that some of PUSCH repetitions are dropped. In TN, gNB knows the dropping slots and thus it is expected that gNB and UE have the same understanding of actual TDWs for the PUSCH repetitions. However, this is not the case in NTN as abovementioned. gNB does not know whether or when a nominal TDW is segmented into multiple actual TDWs as above illustrated. It will be impossible for gNB to decode the PUSCH by using the divided actual TDWs.

For the solution, the last window and the next window can also be dropped together with the dropping slots. The wasted UL transmissions should be cancelled as the divided actual TDWs will not be used effectively by gNB.

Proposal 2:

- For HD-FDD RedCap/eRedCap UE in Case 3 and Case 4, for DMRS bundling,

  - Two actual TDWs divided due to HD-FDD are dropped.

- B) UL slot counting for repetitions

In the same situation assumed in A), with TN, gNB knows the dropping slots and thus it is expected that gNB and UE have the same understanding regarding which slots are not used and how many subsequent slots are including repetitions instead of the dropping slots. However, this is not the case in NTN. gNB does not know these aspects. For example, when UE-A is scheduled with 8 PUSCH repetitions from slot n, the transmissions may be completed at slot n+7 or may be completed at slot n+9 due to drop of some repetitions overlapped with DL. gNB does not know which occurs, and thus gNB must skip scheduling any other UE for subsequent slots that are potentially used for the repetitions, such as slots n+8 and n+9. A new mechanism seems to be necessary to enable UL repetitions efficiently. It is noted that this problem becomes larger when DL repetitions are typically applied as discussed in R19 DL coverage enhancement.

Fig. 2: Why PUSCH repetitions does not work efficiently

For the solution, slot counting should be performed regardless of the PUCCH/PUSCH drop due to HD-FDD. Although it may lead to lack of the number of repetitions, at least UL TX at unintended slots is avoidable by this handling.

Proposal 3:

- For HD-FDD RedCap/eRedCap UE in Case 3 and Case 4, for PUCCH/PUSCH slot counting in repetitions,

  - A dropped slot due to HD-FDD is counted as a valid slot.

  - Avoid DL/UL overlap as much as possible – TA report enhancement

    - Report format

At the previous meeting, it was agreed to study TA report enhancement further. The current TA report mechanism is insufficient in NTN, so we are positive to introduce some mechanism. At first, let us have the same understanding of the existing TA report mechanism.

TA report is triggered by configuration of offsetThresholdTA, by comparison between offsetThresholdTA and TA difference from the last reported TA, or by indication from upper layers to MAC layer. When triggered, if there is an available PUSCH for TAR, TAR MAC CE is transmitted via the PUSCH; otherwise, SR for TAR is triggered, if configured. The reported value is the least integer number of slots, using 15 kHz SCS, greater than or equal to the actual TA value. That is, the ceiling function is applied to the actual TA value.

On top of them, our analysis on potential TAR enhancements can be summarized as follows:

- TA report with finer granularity: Unnecessary.

Benefit of TA report with finer granularity will be marginal or even zero since scheduling is slot-level typically. For analysis, one example is illustrated below. As we can see, even when TA report with 1 ms granularity is used, the overlap range with yellow color that is assumed based on the reported TA value is the same as the actual DL/UL-overlapped slots. This clearly means that there is no gain by TA report with finer granularity, in slot-level scheduling.

- The existing spec

- Half-slot-level TA report

Fig. 3: Analysis of TA report with finer granularity.

Observation 1:

- When scheduling margin is not considered, benefit of TA report with finer granularity is not found. In slot-level TA report, gNB can know slots with potential overlap.

One considerable aspect is actual TA variation due to large latency. Actual TA value keeps changing in time mainly due to LEO motion. gNB will need to consider it and have scheduling margin with cyan color. As a result, the small scheduling margin with cyan color makes scheduling efficiency lower.

Then, it may be argued that TA report with finer granularity is beneficial to reduce this scheduling margin, as illustrated below B. When half-slot TA report is applied, one more slot may be available compared to in case of slot-level TA report as A.

Meanwhile, we found that the same is achievable by half-slot shift in 1 ms granularity as C without introducing finer granularity. B needs one or more additional bits or reduction of report range to indicate TA with finer granularity, while C is not involved with such an overhead increase. HD-FDD (e)RedCap UE in NTN just can use this rule always without signaling disadvantage.

- The existing spec  4 slots unavailable

- Half-slot-level TA report  3 slots unavailable

- Slot-level TA report with half-slot shift in 1 ms granularity  3 slots unavailable

Fig. 4: Analysis of TA report with finer granularity, in consideration of scheduling margin.

Observation 2:

- When scheduling margin is considered, the same as what TA report with finer granularity achieves is possible by slot-level TA report with half-slot shift in 1 ms granularity, without additional overhead.

Proposal 4:

- For HD-FDD RedCap/eRedCap UE, TA report with granularity smaller than 1 ms is NOT supported.

Proposal 5:

- For HD-FDD RedCap/eRedCap UE, TA report with half-slot shift in 1 ms granularity is supported.

  - i.e., ‘0’ in TAR MAC CE means 0.5 ms, ‘1’ in TAR MAC CE means 1.5 ms, and so on.

    - New triggering

In addition to TA report format enhancement as abovementioned, new mechanism of TA report trigger is beneficial in our understanding. For the new mechanism, we propose TA report based on collision detection at UE side. When overlap occurs, it may be due to large gap between assumed TA at gNB and actual TA, and thus re-report of TA should be performed immediately.

Proposal 6:

- For HD-FDD RedCap/eRedCap UE, trigger a TA report when the collision is detected at the UE.

  - If PUSCH for the report is available, TA is reported.

  - Otherwise, SR for TA report is transmitted.

  - More than two overlaps

Regarding both collision Case 3 and collision Case 4, more than two channels could overlap. This kind of situation has been discussed during all NR releases. We do not see any valid reason to skip such discussion only for R19 NTN.

Proposal 7:

- For HD-FDD RedCap/eRedCap UE, discuss how to handle more than two overlaps and define handling rule.

- Conclusion

In this contribution, we discussed support of RedCap/eRedCap UEs in FR1-NTN. Observations/Proposals are summarized as following:

Proposal 1:

- For HD-FDD RedCap/eRedCap UE in Case 4, NOT support special handling for PDCCH ordered PRACH, i.e.,

  - Confirm the working assumption with removing the FFS bullet.

Proposal 2:

- For HD-FDD RedCap/eRedCap UE in Case 3 and Case 4, for DMRS bundling,

  - Two actual TDWs divided due to HD-FDD are dropped.

Proposal 3:

- For HD-FDD RedCap/eRedCap UE in Case 3 and Case 4, for PUCCH/PUSCH slot counting in repetitions,

  - A dropped slot due to HD-FDD is counted as a valid slot.

Observation 1:

- When scheduling margin is not considered, benefit of TA report with finer granularity is not found. In slot-level TA report, gNB can know slots with potential overlap.

Observation 2:

- When scheduling margin is considered, the same as what TA report with finer granularity achieves is possible by slot-level TA report with half-slot shift in 1 ms granularity, without additional overhead.

Proposal 4:

- For HD-FDD RedCap/eRedCap UE, TA report with granularity smaller than 1 ms is NOT supported.

Proposal 5:

- For HD-FDD RedCap/eRedCap UE, TA report with half-slot shift in 1 ms granularity is supported.

  - i.e., ‘0’ in TAR MAC CE means 0.5 ms, ‘1’ in TAR MAC CE means 1.5 ms, and so on.

Proposal 6:

- For HD-FDD RedCap/eRedCap UE, trigger a TA report when the collision is detected at the UE.

  - If PUSCH for the report is available, TA is reported.

  - Otherwise, SR for TA report is transmitted.

Proposal 7:

- For HD-FDD RedCap/eRedCap UE, discuss how to handle more than two overlaps and define handling rule.

References

- RP-243300		Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3	Thales, CATT

- 3GPP RAN1#116 meeting,		chair’s notes

- 3GPP RAN1#116bis meeting,	chair’s notes

- 3GPP RAN1#117 meeting,		chair’s notes

- 3GPP RAN1#118 meeting,		chair’s notes

- 3GPP RAN1#118bis meeting,	chair’s notes

- 3GPP RAN1#119 meeting,		chair’s notes

- 3GPP RAN1#120 meeting,		chair’s notes

- 3GPP RAN1#120bis meeting,	chair’s notes