3GPP TSG RAN WG1 #121	R1-2504530

St Julian’s, Malta, May 19th – 23rd, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for R19 NR NTN

Agenda Item:	9.15.8

Document for:	Discussion and Decision

- Introduction

At the last RAN1 meeting, the following FGs were agreed for R19 NR NTN [1]. In this contribution, we will discuss other necessary FGs for this work item and details of the agreed/proposed FGs.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65. NR_NTN_Ph3 | 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial [cell selection or access] [FFS Other components] | FFS | FFS | N/A | FFS: UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial access | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 | Optional [with/without] capability signaling FFS: A UE that supports Rel-19 NR-NTN must support this FG |
| 65. NR_NTN_Ph3 | 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS [of searchSpaceZero configured within MIB pdcch-ConfigSIB1] FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition | FFS | FFS | N/A | PDCCH repetition for Type0 PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN] | Optional [with/without] capability signaling |
| 65. NR_NTN_Ph3 | 65-1-3 | PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | 1. Support reception of PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS | FFS | FFS | N/A | FFS | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable [only] for bands in Tables 5.2.2-1 [and 5.2.3-1] in TS 38.101-5 [FFS: Applicabilitiy of this FG to TN] | Optional [with/without] capability signaling |
| 65. NR_NTN_Ph3 | 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | [1. Support handling collision Case 3] [2. Support handling collision Case 4] FFS: whether to divide this FG into two separate FGs between case 3 and case 4 FFS: whether to divide this FG into two separate FGs between RRC CONNECTED and INACTIVE state | FFS: 26-1, 26-4, 28-1, 28-3, 48-1, 48-2 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | FFS | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FDD only Note: Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only | Optional with capability signaling FFS: A UE that supports FG 28-3 in NTN band must support this FG |

- Discussion

  - DL coverage enhancement

    - FG 65-1-1 (SSB periodicity extension)

- Component 1: In 38.213, “initial cell selection” is the terminology for SSB monitoring/detection. We do not see any reason to adopt different wording here.

- Other components: Component 1 is all of the expected specification impact corresponding to SSB periodicity extension. We suggest removing this FFS.

- Prerequisite: For monitoring SSB with 160 ms periodicity, we do not think that any other FG is essential.

- Report to gNB: UE behavior of this FG is not relevant to any other behaviors after SSB detection, and there is no impact to gNB behavior. No capability signaling is necessary.

- Consequence: Text based on component 1 is reasonable.

- FR2 support: The WID is clearly saying that system level enhancement is applicable to both FR1-NTN and FR2-NTN.

- Mandatory/optional: Based on discussion, R19 assumption is the baseline of practical satellite implementation. From NW deployment perspective, it is helpful for all R19 NTN UEs to support this feature.

Proposal 1: Update FG 65-1-1 as follows.

| 65. NR_NTN_Ph3 | 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection |  | No | N/A | UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial cell selection | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional without capability signaling A UE that supports Rel-19 NR-NTN must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG 65-1-2 (PDCCH repetition for Type0 PDCCH CSS)

- Component 1: 213 editor’s text is “For an NTN cell in FR1, searchSpaceZero in pdcch-ConfigSIB1 indicates …”. We do not see any reason to adopt different wording here.

- 1st FFS of components: The key point may be whether to consider NW applying only either of CSS type 0 PDCCH rep or CSS type 0A/0B/1/1A/2/2A PDCCH rep and/or UE supporting only either of them. Enabling/disabling of the latter rep have not been agreed yet; we suggest waiting for more progress of WI discussion.

- 2nd FFS of components: The same comment as above.

- Prerequisite: We do not think that any other FG is essential.

- Report to gNB: When this FG is dedicated to Type0-PDCCH CSS, UE behavior of this FG is not relevant to any other behaviors before/after detection of this PDCCH, and there is no impact to gNB behavior. No capability signaling is necessary.

- Applicability to TN: This issue was discussed from the first release of NR NTN. As a result, all features introduced in NTN WI have been defined only for NTN. The same principle may be the only possible conclusion.

- Applicability to FR2: The WID is clearly saying that link level enhancement is applicable to FR1-NTN while system level enhancement is intended for both FR1-NTN and FR2-NTN. This may imply that link level enhancement is not applied to FR2.

Proposal 2: Update FG 65-1-2 as follows.

| 65. NR_NTN_Ph3 | 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within pdcch-ConfigSIB1 FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition |  | No | N/A | PDCCH repetition for Type0 PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG 65-1-3 (PDCCH repetition for Type [0A/0B/1/1A/2/2A] PDCCH CSS)

At the last meeting, it was agreed as a working assumption that intra-slot PDCCH repetition is introduced for CSS other than type 0 and type 3 and other than CSS configured by SearchSpaceZero. Regarding the detailed text, it may be better to mention explicitly which CSS type is the target as “other than” may lead to misunderstanding in future.

- FG name: remove the brackets based on the above consideration.

- Component 1: remove the brackets based on the above consideration. In addition, “other than CSS configured by SearchSpaceZero” should be reflected.

- Prerequisite: We do not think that any other FG is essential.

- Report to gNB: NW side will use this capability e.g., for UE-specific PDCCH configurations in consideration of BD capability of each UE.

- Consequence: the text in FG 65-1-2 can be reused for this FG with necessary updates.

- Applicability to TN: same as in FG 65-1-2.

- Applicability to FR2: same as in FG 65-1-2.

Proposal 3: Define FG 65-1-3 as follows.

| 65. NR_NTN_Ph3 | 65-1-3 | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS | 1. Support reception of PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS configured by other than SearchSpaceZero FFS: whether to merge this FG with FG 65-1-2 |  | Yes | N/A | PDCCH repetition for 0A/0B/1/1A/2/2A PDCCH CSS configured by other than SearchSpaceZero is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG for SIB1 PDSCH repetition

SIB1 PDSCH repetition is introduced in R19; a new FG is necessary for this feature.

- Prerequisite: This feature may always be associated with PDCCH repetition for Type0 PDCCH CSS. We suggest waiting for more progress in the WI discussion.

- Report to gNB: We are not sure how NW side can use this capability information for its behavior.

Proposal 4: Define FG 65-1-4 as follows.

| 65. NR_NTN_Ph3 | 65-1-4 | SIB1 PDSCH repetition within 20ms duration | 1. Support reception of SIB1 PDSCH repetition within 20 ms duration FFS: whether to merge this FG with FG 65-1-2 | FFS: 65-1-2 | No | N/A | SIB1 PDSCH repetition within 20ms duration is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG for Msg4 PDSCH repetition

Msg4 PDSCH repetition is introduced in R19; a new FG is necessary for this feature.

- Prerequisite: We do not think that any other FG is essential.

- Report to gNB: Now this is under discussion in the WI. We suggest waiting for more progress in the WI discussion.

Proposal 5: Define FG 65-1-5 as follows.

| 65. NR_NTN_Ph3 | 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition |  | FFS | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

    - FG of PDCCH repetition for CSS type 3 and USS

In addition, although PDCCH repetition for CSS type 3 and USS were introduced in R17 for multi-TRP purpose and the spec can be reused for NTN with single-TRP in general, it seems that R17 FGs are not suitable for R19 NTN in principle. The following is discussed in our contribution [1] and copied here for convenient information.

R17 PDCCH repetition has been specified assuming multi-TRP scenario, thereby these behavior considerations would be reasonable. Either TRP may fall into low SNR, e.g., due to blocking in FR2, while the other may keep higher SNR in LOS. In this case, respective decoding is basically sufficient and ‘A’ above works well. Then especially for ‘B’, for instance, the following BD counting rule is described in 38.213. ## 10.1 UE procedure for determining physical downlink control channel assignment … If a UE - is provided monitoringCapabilityConfig = r16monitoringcapability for a downlink cell, - is provided, by searchSpaceLinkingId a same value for search space sets si and sj on the downlink cell, and - indicates numBD-twoPDCCH-r17 with value of 3 the UE counts each PDCCH candidate for the one of the search space sets si and sj that the UE monitors PDCCH in the later span, as two PDCCH candidates. The UE does not expect a first PDCCH candidate from search space set si or sj and a second PDCCH candidate from a search space set sk that does not include searchSpaceLinkingId to use a same set of CCEs and same scrambling in a same CORESET, and provide respective first and second DCI formats with same size, in any span other than the first span in a slot. …. On the other hand, as discussed for PDCCH repetition for CSS type 0A/0B/1/1A/2/2A in the last section, PDCCH repetition is used to boost SNR by soft-combining in single-TRP scenario in R19 NTN. In this case, a single decoding with soft-combining is enough for the two repeated PDCCHs and thus decode of either of PDCCH repetitions is not necessary in principle. The above two decoding behavior (i.e., A, B) is excessive and undesirable from UE perspective. Therefore, UE should be allowed to do: C: A single blind decoding = Decode of a soft-combined PDCCH and for this purpose, what we need to agree is that description of a corresponding BD counting rule is added in 38.213.

Including the above BD counting perspective, R17 UE capability signaling is not suitable for R19 NTN in principle. Several FGs were defined for R17 PDCCH repetition. In those capabilities, e.g., FG23-2-1, which is the fundamental FG for R17 PDCCH repetition, there is no differentiation between single-TRP scenario and multi-TRP scenario. That is, support indication via R17 UE capability signaling will imply that NTN UE must support both single-TRP scenario and multi-TRP scenario to send this FG as “support”, which is unnecessary requirement for NTN scenario.

To solve this problem, UE capability signaling should be enhanced so that UE can indicate support of PDCCH repetition only in single-TRP scenario. For example, separate capability signaling only for single-TRP scenario is defined and R19 NTN UE can use this new capability signaling instead of R17 capability signaling.

Table 1: R17 FG for PDCCH repetition.

| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs including PDCCH repetition for Type 3 CSS 2. Required number of BDs for the two PDCCH candidates 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier per slot |  | mTRP-PDCCH-Repetition-r17 { numBD-twoPDCCH-r17, maxNumOverlaps-r17, } | FeatureSetDownlink-v1700 | n/a | n/a | n/a | Component 2 candidate values: 2 or 3 Component 3 candidate values: {1,2,3, 5, 10, 20, 40} NOTE 1: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot. NOTE 2: for component 3, each unique pair of overlaps is counted as one. NOTE 3: This FG does not include supporting Two QCL-TypeD in time-domain overlapping CORESETs in FR2. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Define FG 65-1-6 as follows.

| 65. NR_NTN_Ph3 | 65-1-6 | PDCCH repetition for Type3 PDCCH CSS and PDCCH USS in NTN | 1. Support reception of PDCCH repetition for Type3 PDCCH CSS and PDCCH USS in NTN [2. Support 1 for required number of BDs for the two PDCCH candidates] |  | Yes | N/A | PDCCH repetition for CSS type 3 and USS is not supported in NTN | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - RedCap/eRedCap support

    - FG 65-2-1 (Collision Case 3 and collision Case 4)

- Component 1: Remove the bracket and keep the text. We do not believe that it is necessary to capture the detailed handling behavior.

- Component 2: Remove the bracket and keep the text. We do not believe that it is necessary to capture the detailed handling behavior.

- 1st FFS of components: Not support to separate. We should see R17 RedCap FGs to design R19 FGs. In R17, only a single FG (FG 28-3) was defined for HD-FDD UE, which includes overlap handling for multiple collision cases. We should follow this principle, i.e., only a single FG is enough for R19 NTN capability signaling. This is reasonable from technical perspective as well, since both collision Case 3 and collision Case 4 could occur in NTN scenario regardless of whether UE supports collision handling for each case or not.

- 2nd FFS of components: Remove the FFS. RRC_INACTIVE state is relevant to CG-SDT only; another note should be added to clarify this.

- Prerequisite: FG 28-3 (HD-FDD) should be a prerequisite of this FG. We do not think that any other FG is necessary here.

- FR1/FR2 differentiation: Not necessary. This FG is defined only for FR1 and the signaling is per band. FR1/FR2 differentiation is meaningless.

- Mandatory/optional: Remove “FFS” from the FFS part and keep the remaining. This handling is essential in NTN from gNB scheduling perspective, which is why this feature is introduced in R19 NTN.

It is noted that there have NOT been any agreements on support of TA report enhancement so far; thus, no FG should be defined at least in this stage.

Proposal 7: Update FG 65-2-1 as follows.

| 65. NR_NTN_Ph3 | 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | 1. Support handling collision Case 3 2. Support handling collision Case 4 | 28-3 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FDD only Note: Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only Note: Case 3 handling for RRC_INACTIVE state is supported only if the UE supports CG-SDT. | Optional with capability signaling A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - UL capacity/throughput enhancement

Inter-slot OCC for PUSCH is introduced in R19; new FG(s) is necessary for this feature. In the previous RAN1 meeting, it was agreed that separate FGs are defined between OCC length 2 and length 4, and the former is a prerequisite for the latter.

- FG: OCC indication/determination, RV cycling, phase continuity/power consistency and overlap handling (e.g., UCI mux/drop behavior) are fundamental features for PUSCH with OCC and thus these should be covered in a single FG.

- Components: We do not believe that it is necessary to capture detailed behaviors.

- Prerequisite: For inter-slot OCC, at least support of PUSCH repetition type A is essential.

- Applicability to TN: Based on the WID, this feature should cover TN as well as NTN.

Proposal 8: Define FGs 65-3-1 and 65-3-2 as follows.

| 65. NR_NTN_Ph3 | 65-3-1 | PUSCH with inter-slot OCC of length 2 | 1. Support inter-slot OCC with OCC length 2 for DFT-s-OFDM PUSCH transmissions with repetition Type A | One of {5-14, 5-16, 5-17} | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65. NR_NTN_Ph3 | 65-3-2 | PUSCH with inter-slot OCC of length 4 | 1. Support inter-slot OCC with OCC length 4 for DFT-s-OFDM PUSCH transmissions with repetition Type A | 65-3-1 | YES | N/A | UE does not support OCC length 4 for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

- Conclusion

In this contribution, we presented our views regarding UE features for R19 NR NTN. Based on the discussion in this contribution, the following proposals were made.

Proposal 1: Update FG 65-1-1 as follows.

| 65. NR_NTN_Ph3 | 65-1-1 | 160 ms SSB periodicity assumed during initial access | 1. Support additional default value (apart from 20 ms value) of 160ms periodicity of the half frames with SS/PBCH blocks during initial cell selection |  | No | N/A | UE may assume 20 ms periodicity of the half frames with SS/PBCH blocks during initial cell selection | Per band | N/A | N/A | N/A | Note: As described in the WID, this UE feature group is applicable only for bands in Tables 5.2.2-1 and 5.2.3-1 in TS 38.101-5 | Optional without capability signaling A UE that supports Rel-19 NR-NTN must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Update FG 65-1-2 as follows.

| 65. NR_NTN_Ph3 | 65-1-2 | PDCCH repetition for Type0 PDCCH CSS | 1. Support reception of PDCCH repetition for Type0 PDCCH CSS of searchSpaceZero configured within pdcch-ConfigSIB1 FFS: whether to merge this FG with FG(s) for PDCCH repetition for other than Type0 PDCCH CSS FFS: whether to merge this FG with FG(s) for SIB1 PDSCH repetition |  | No | N/A | PDCCH repetition for Type0 PDCCH CSS is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 3: Define FG 65-1-3 as follows.

| 65. NR_NTN_Ph3 | 65-1-3 | PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS | 1. Support reception of PDCCH repetition for Type 0A/0B/1/1A/2/2A PDCCH CSS configured by other than SearchSpaceZero FFS: whether to merge this FG with FG 65-1-2 |  | Yes | N/A | PDCCH repetition for 0A/0B/1/1A/2/2A PDCCH CSS configured by other than SearchSpaceZero is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 4: Define FG 65-1-4 as follows.

| 65. NR_NTN_Ph3 | 65-1-4 | SIB1 PDSCH repetition within 20ms duration | 1. Support reception of SIB1 PDSCH repetition within 20 ms duration FFS: whether to merge this FG with FG 65-1-2 | FFS: 65-1-2 | No | N/A | SIB1 PDSCH repetition within 20ms duration is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5: Define FG 65-1-5 as follows.

| 65. NR_NTN_Ph3 | 65-1-5 | Msg4 PDSCH repetition | 1. Support reception of Msg4 PDSCH repetition |  | FFS | N/A | Msg4 PDSCH repetition is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional [with/without] capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 6: Define FG 65-1-6 as follows.

| 65. NR_NTN_Ph3 | 65-1-6 | PDCCH repetition for Type3 PDCCH CSS and PDCCH USS in NTN | 1. Support reception of PDCCH repetition for Type3 PDCCH CSS and PDCCH USS in NTN [2. Support 1 for required number of BDs for the two PDCCH candidates] |  | Yes | N/A | PDCCH repetition for CSS type 3 and USS is not supported in NTN | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 7: Update FG 65-2-1 as follows.

| 65. NR_NTN_Ph3 | 65-2-1 | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE | 1. Support handling collision Case 3 2. Support handling collision Case 4 | 28-3 | YES | N/A | Handling collision Case 3 and collision Case 4 for half-duplex FDD operation for (e)RedCap UE is not supported | Per band | N/A | N/A | N/A | Note: This UE feature group is applicable only for bands in Tables 5.2.2-1 in TS 38.101-5 Note: collision Case 3: Semi-statically configured DL reception collides with semi-statically configured UL transmission Note: collision Case 4: Dynamically scheduled DL reception collides with dynamic scheduled UL transmission Note: this FG is for FDD only Note: Case 3 handling is for RRC CONNECTED and INACTIVE states, and Case 4 is for RRC CONNECTED state only Note: Case 3 handling for RRC_INACTIVE state is supported only if the UE supports CG-SDT. | Optional with capability signaling A UE that supports FG 28-3 in NTN band must support this FG |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 8: Define FGs 65-3-1 and 65-3-2 as follows.

| 65. NR_NTN_Ph3 | 65-3-1 | PUSCH with inter-slot OCC of length 2 | 1. Support inter-slot OCC with OCC length 2 for DFT-s-OFDM PUSCH transmissions with repetition Type A | One of {5-14, 5-16, 5-17} | YES | N/A | UE does not support inter-slot OCC for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 65. NR_NTN_Ph3 | 65-3-2 | PUSCH with inter-slot OCC of length 4 | 1. Support inter-slot OCC with OCC length 4 for DFT-s-OFDM PUSCH transmissions with repetition Type A | 65-3-1 | YES | N/A | UE does not support OCC length 4 for DFT-s-OFDM PUSCH with repetition Type A | Per band | N/A | N/A | N/A |  | Optional with capability signaling |

Reference

[1]	R1-2502977	Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis	Moderators (AT&T, NTT DOCOMO, INC.)