3GPP TSG RAN WG1 #121	 					        R1- 2504565

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda Item:	9.11.5

Source: 	LG Electronics

Title: 	Discussion on IoT-NTN TDD mode

Document for:	Discussion and decision

- Introduction

In RAN1#120bis meeting [1], the followings are agreed for Rel-19 IoT-NTN TDD mode WI:

Agreement Confirm the following working assumption: - Uplink transmission gaps do not apply in NB-IoT NTN TDD. Agreement NPRACH transmissions are postponed in non-U NB-IoT subframes until the next U NB-IoT subframe(s). The unit of postponement is one PRU. - NOTE: “U NB-IoT subframes” are subframes contained within the set of U=8 usable consecutive uplink subframes in the TDD structure. NPRACH periodicities of 40ms and 80ms are not supported in NB-IoT NTN TDD mode - Instead, periodicities of 90ms and 180ms are supported Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be 50ms at the ULSRP. - NOTE: This roughly corresponds to the largest separation between DL and UL Iridium slot pairs. - Note: Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA. Agreement For the issue of handling DL gaps in NB-IoT NTN TDD mode, down-select among the following options: - Option 1: DL gaps do not apply for NB-IoT NTN TDD mode. - Option 2: DL gaps apply based on current specifications. - Option 3: DL gaps apply, where new periodicity(ies) are introduced (either implicity e.g. the periodicity is scaled by a fixed factor F, or explicitly) Agreement For the issue of handling NPDCCH postponing, down-select among the following options: - Option 1: No new periodicities are introduced for NPDCCH search space monitoring - FFS: Changes in postponing / overlapping rules, if any - FFS: Introduction of constraints on the configuration of search space parameters (e.g. Rmax, G…), if any - Option 2: New periodicities are introduced for NPDCCH search space monitoring - FFS: Details of the new periodicity, e.g. the existing periodicity is scaled by a fixed factor F, explicit signalling of the new periodicity, reusing the NB-IoT TN TDD periodicities, etc. Working assumption DL bitmap (downlinkBitmap / downlinkBitmapNonAnchor) is not present in a NB-IoT NTN TDD cell. - NOTE1: All D NB-IoT subframes (except those carrying NPSS/NSSS/SIB1-NB/NPBCH) are NB-IoT DL subframes. - NOTE2: “D NB-IoT subframes” are subframes contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. Agreement SIB1-NB is dropped in non-D NB-IoT subframes - NOTE 1: “non-D NB-IoT subframes” are subframes not contained within the set of D=8 usable consecutive downlink subframes in the TDD structure. - NOTE 2: RAN1 observes that some combinations of {TBS, number of repetitions, additional SIB1 repetitions}, and depending on aspects such as minimum elevation angle or antenna gain, may not meet the link budget. It is up to the network to properly configure such values to ensure the link budget is met. No specification impact. Agreement For GNSS measurement gap, RAN1 to further discuss at least the following options: - Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap - Option 2: The start of the GNSS measurement gap starts at a non-D/non-U subframe - Option 3: GNSS measurement gap does not apply for NB-IoT NTN TDD Working assumption For precompensation, from RAN1 perspective: - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes. - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed. - FFS: whether spec impact is in RAN1, RAN4 or both Inform RAN4 of the WA above and ask if there are any issues with the above WA.

In this contribution, we discuss mechanism for TDD mode in IoT NTN.

- Discussion

  - NPDCCH design for TDD mode

According to the TS36.213, it seems that NPDCCH USS and Type2-NPDCCH CSS and Type2A-NPDCCH CSS are defined in consecutive NB-IoT DL subframes excluding subframes used for transmission of SI messages. In this case, it would be sufficient to update the definition of NB-IoT DL subframes not to include the non-D NB-IoT subframes. Meanwhile, the first NPDCCH candidate starts in subframe k_0, and the subframe k0 could be non-D NB-IoT subframe. Meanwhile, according to TS36.211, the UE shall not expect NPDCCH in subframe i if it is not a NB-IoT DL subframe. In case of NPDCCH transmission in subframes that are not NB-IoT DL subframes, the NPDCCH transmission is postponed until the next NB-IoT DL subframe. It would result that the NPDCCH candidate defined to be started in subframe k_0 after the postponement will be collided with the next NPDCCH candidate defined to be started in subframe k_R where R is the repetition number of the NPDCCH candidates. They will block each other.

According to TS36.331, for NPDCCH search space, it would be possible that the time difference between the end of the earlier NPDCCH monitoring occasions and the beginning of the later NPDCCH monitoring occasion is less than 4msec. For instance, if the value of G is 1.5 and the value of R_max is 4, then the NPDCCH search space period, denoted by T, is 6 subframes and the time difference is 2msec even though the all the subframes are NB-IoT DL subframes without any gap. In summary, {G=2, R_max=2}, {G=4, R_max=1}, and {G=1.5, R_max=4} will make the time gap between two contiguous NPDCCH occasions less than 4msec. If the following in TS36.213 is applicable even for the same search space (such as USS or CSS), then the above cases will not be allowed even in the current specification. Moreover, in Rel-15, the candidate value for G is extended to be 96 or 128 on top of 1.5, 2, 4, 8, 16, 32, 48, and 64.

TS36.213

the UE is not required to monitor NPDCCH candidates of an NPDCCH search space if an NPDCCH candidate of the NPDCCH search space ends in subframe n, and if the UE is configured to monitor NPDCCH candidates of another NPDCCH search space having starting subframe k0 before subframe n+5.

TS36.331

NPDCCH-ConfigDedicated-NB-v1530 ::= SEQUENCE { npdcch-StartSF-USS-v1530 ENUMERATED {v96, v128} }

If the new periodicity for NPDCCH search space is introduce, since k_0 is defined with respect to the SFN0, the offset would be also modified accordingly. To be specific, the time locations of k_0 will be the same for all the hyper frames while the time locations of the TDD patterns will be different across different hyper frames. For instance, the definition of the k_0 may need to be changed with respect to the beginning of the TDD pattern.

In those points of views, it would not be necessary to change the periodicity of NPDCCH monitoring.

Observation 1: According to the existing specification, for USS and Type 2/2A CSS, the search space including the starting subframe are defined in consecutive NB-IoT DL subframes excluding subframes used for transmission of SI messages.

Observation 2: According to the existing specification, the UE shall not expect NPDCCH in subframe that is not a NB-IoT DL subframe, and the NPDCCH transmission in subframes that are not NB-IoT DL subframes is postponed until the next NB-IoT DL subframe.

Observation 3: According to the existing specification, if the time gap between two contiguous search space is applicable to two contiguous NPDCCH occasions in the same search space, some combinations of G and R_max in the existing specification would not be used even without a consideration of new TDD pattern.

Observation 4: If new NPDCCH search space periodicity to be multiples of 90msec is introduced, the new offset definition would be necessary to allow the time locations of the k0 is different across different hyper frames like the TDD pattern.

Proposal 1: No new periodicities are introduced for NPDCCH search space monitoring.

  - DL transmission gap for TDD mode

Regarding the DL transmission gap, according to TS36.211, since the time location is defined with respect to SFN#0, some DL transmission gap would not be aligned with the DL burst location of the TDD pattern. Therefore, even with the new periodicity which is multiples of 90mesc, depending on the offset between SFN#0 and the beginning of the first TDD pattern, it is not ensured that the DL transmission gap is always inside the DL bursts of the TDD pattern. Meanwhile, even for the DL transmission gap inside the DL bursts, it is not always guaranteed that the eNB schedules and transmits new NPDCCH. In this case, the DL transmission gap will waste the DL resources. Considering that the amount of DL resources within a TDD pattern period is very limited, it would not be desirable. In this case, it can be considered that there is no DL transmission gap in NPDCCH/NPDSCH for the IoT-NTN TDD mode.

Observation 5: If new DL transmission gap periodicity to be multiples of 90msec is introduced, the new offset definition would be necessary to allow the time locations of the DL transmission gap is confined within the DL burst in a TDD pattern period.

Observation 6: During the DL transmission gap within the DL burst of the TDD pattern, it is not guaranteed that eNB always use it for another NPDCCH transmissions, then it will results the DL resource waste.

Proposal 2: DL gaps do not apply for NB-IoT NTN TDD mode.

  - RAR and Msg4 design for TDD mode

Next, due to the reduced chance of UL transmissions and DL receptions, it would be necessary to update all the timelines for the random-access procedure accordingly including NPRACH-to-RAR window timing, RAR-to-Msg3 Tx timing, and Msg3-to-conention resolution window timing.

According to TS36.321, RA Response window starts at the subframe that contains the end of the last preamble repetition plus X (as specified in TS36.321) subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level. Moreover, the maximum RA window size is currently 10.24s for FDD. Meanwhile, depending on the TDD pattern, it would be possible that the determined RAR window does not include any DL subframes or includes insufficient DL subframes. To bypass this problem, it would be necessary to have additional timing offset to derive the location of RAR window. For simplicity, it can be considered that the RAR window starts the earliest DL subframe 3+UE-eNB RTT after the subframe that contains the end of the last preamble repetition. However, in this case, multiple RAR windows associated with different NPRACH repetitions that starts in different timing could be overlapping. In this case, it would be necessary to distinguish different NPRACH transmission by adjusting RAPID or RA-RNTI.

Regarding Msg3-to-Contention resolution window, it can be considered to postpone the starting time of the contention resolution window to be located in the configured DL subframe.

Proposal 3: For IoT-NTN TDD mode, one or more of followings can be considered for initial access procedure:

- RAR window starts in DL subframe provided by the TDD pattern subject to the existing required time.

- UE (re)starts the contention resolution timer at the DL subframe provided by the TDD pattern subject to the existing required time.

  - NPRACH design for TDD mode

In RAN1#120bis meeting, new NPRACH periodicities of 90mesc and 180msec are introduced together with the postponement of NPRACH transmission. The intention of the new periodicity is to allow the UE does not perform the postponement to ensure that the NPRACH transmission is confined within a UL burst of a TDD pattern period. To do this, NPRACH offset would be needed to be updated  to ensure the NPRACH occasions are within the U NB-IoT subframes. Moreover, considering that the time gap between the end of the DL burst and the beginning of the UL burst in a TDD pattern period is 50msec, the UL burst will start at subframe#3.

Proposal 4: For IoT-NTN TDD mode, NPRACH offset is defined with respect to the beginning of the first TDD pattern within a hyper frame.

- Candidate for the new NPRACH offset: 0, 1, 2

  - Pre-compensation mechanism for TDD mode

According to TS36.102, it seems that RAN4 only considers that the UE shall update pre-compensation prior the segment transmission. Meanwhile, there is no restriction on the time locations of the UL segments. In other words, in RAN4 specification point of view, it seems it doesn’t matter whether the position of the UL segment is moved to be aligned with the UL burst in boundary. In this point of view, the working assumption on the pre-compensation for the TDD mode would not be a problem.

TS36.102

When segmentation is applied, then the UE shall update pre-compensation at the beginning of each segment prior to segment transmission.

Depending on the scenario, it would be good for a UE to perform the pre-compensation multiple times within a UL burst in terms of the time/frequency offset accuracy. On the other hand, it will require a number of UL segment gaps which are not used for the actual transmission. If we consider only 8 msec for the segment size without the segment gap, it will maximize the UL resource utilization at the expense of the time/frequency offset error. Considering 0.27ppm/s and 1.6GHz of the carrier frequency, during the time duration of 8msec, the expected frequency error and/or time error would be acceptable.

Meanwhile, the NPUSCH transmission can start in the middle of the UL burst. In this case, the number of transmissions in the first UL burst would be definitely smaller than 8, in this case, the UE does not need to perform the pre-compensation before the starting of the NPUSCH transmission. In our understanding, the WA targets the case when the NPUSCH transmission is postponed to the next UL burst. In this point of view, it would be necessary to clarify when the UE adjust its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes.

Observation 7: Considering the limited number of available UL resources in TDD mode, the segment size of 8msec without any gap can maximize the UL resource utilization at the expense of the small time/frequency offset error.

Observation 8: When the NPUSCH transmission starts in the middle of the UL burst (8 consecutive uplink subframes), the UE does not need to perform the pre-compensation at the beginning of the UL burst. Moreover, if the UE does not have the NPUSCH transmission in the UL burst, the UE is not required to perform the pre-compensation during the UL burst.

Proposal 5: Confirm the following WA with some change:

- Working assumption

  - For precompensation, from RAN1 perspective:

    - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes for the postponed NPUSCH transmission. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

    - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

  - FFS: whether spec impact is in RAN1, RAN4 or both

  - GNSS measurement for TDD mode

Since the DL SCS and the UL SCS can be different, the current specification distinguishes the DL subframe and the UL subframe or slot. In other words, even with applying the TDD pattern, it is not guaranteed that the DL subframes are always inside the DL bursts of the TDD pattern. According to TS36.213, the DL subframe in the specification can be located outside the DL burst. For instance, if the UE shall no provide HARQ-ACK information for the NPDSCH, and if the NPDSCH containing GNSS Measurement Command MAC CE ends in the first DL subframe of the DL burst (subframe#3), then the GNSS measurement gap will start in the guard period as shown in Figure 1-a. If the UE provide HARQ-ACK feedback for the NPDSCH, and if the NPDSCH carrying the HARQ-ACK information ends in the first UL subframe of the UL burst, the GNSS measurement gap will start in the 4th subframe within the UL burst of the TDD pattern as shown in Figure 1-b.

TS36.213

For a NB-IoT UE in a NTN FDD serving cell, when the UE receives a GNSS Measurement Command MAC CE in a NPDSCH ending in DL subframe n, - if the UE shall not provide HARQ-ACK information for the HARQ process associated with the transport block in the NPDSCH carrying GNSS Measurement Command MAC CE, - the UE shall assume the start of the measurement gap in subframe n+13 - otherwise, - the UE shall assume the start of the measurement gap in subframe k+2, where k is the first DL subframe after the end of the transmission of the NPUSCH carrying ACK/NACK response for the HARQ process associated with the transport block in the NPDSCH. For a NB-IoT UE in a NTN FDD serving cell, the UE is not required to monitor NPDCCH within the GNSS measurement gap duration, until it reacquires GNSS position and a contention based Random Access is performed as specified in TS 36.321 [8].

Figure 1: Example of the GNSS measurement gap after applying the TDD pattern.

For both cases, since the GNSS measurement gap starts outside the DL burst, the UE automatically skip the NPDCCH monitoring.

Observation 9: According to the existing specification, even after applying the TDD pattern, the GNSS measurement gap will start outside the DL burst of the TDD pattern regardless of whether the UE provide HARQ-ACK feedback for the NPDSCH containing GNSS measurement command MAC CE.

According to TS36.321, the minimum GNSS measurement gap size is 1 second. During the time duration of 1 second, it will cover multiple TDD pattern periods. According to TS36.213, the UE is not required to monitor NPDCCH within the GNSS measurement gap duration, until it reacquires GNSS position and a contention based Random Access is performed. According to TS36.331, if GNSS position is out-of-date, the UE will leave the RRC_CONNECTED mode. If the UE reacquires GNSS position, the UE will stop the timeAlignmentTimer and initiate the random-access procedure to report the remaining GNSS validity duration. When timeAlignmentTimer expires, all the dedicated resources for SR will be released. In other words, the UE may stop any UL transmissions until timeAlignmentTimer restarts.

TS36.331

### 5.5.9 GNSS measurement triggering and reporting For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods. The UE shall: 1> if an indication to perform GNSS measurement is received from lower layers: 2> perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23]; 2> stop timer T390, if running; 1> if gnss-AutonomousEnabled is configured: 2> if the gap length is indicated by lower layers: 3> set the autonomous gap length to the gap length indicated by lower layers; 2> else: 3> set the autonomous gap length to the latest reported time duration required for the UE to acquire a GNSS position; 2> perform GNSS measurement using the autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry; NOTE: UE can autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid. The exact time of starting GNSS measurements during available idle periods is left to UE implementation. 1> upon starting GNSS measurement: 2> stop timer T318, if running; 1> upon indication that a new GNSS position becomes valid: 2> instruct lower layers to report the remaining GNSS validity duration (see TS 36.321 [6]); 1> upon indication that GNSS measurement has failed: 2> if GNSS position is out-of-date; and 2> if ul-TransmissionExtensionEnabled is not configured or T390 is not running: 3> perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

TS36.321

### 5.4.10 GNSS Validity Duration Reporting For an NB-IoT UE, a BL UE or a UE in enhanced coverage in a non-terrestrial network, an indication may be sent by upper layers to report the remaining GNSS validity duration. If the MAC entity receives an indication from upper layers to report the remaining GNSS validity duration: - stop the timeAlignmentTimer associated with the pTAG, if running. - initiate a Random Access procedure (see clause 5.1). NOTE: When in RRC_CONNECTED, if the UE autonomously starts and completes GNSS acquisition using available idle periods, the UE reports the remaining GNSS validity duration but it is up to UE implementation whether to stop timeAlignmentTimer and initiate a Random Access procedure.

- when a timeAlignmentTimer expires: - if the timeAlignmentTimer is associated with the pTAG: - flush all HARQ buffers for all serving cells; - notify RRC to release PUCCH/SPUCCH for all serving cells; - notify RRC to release SRS for all serving cells; - for NB-IoT, notify RRC to release all dedicated resources for SR; - clear any configured downlink assignments and uplink grants; - consider all running timeAlignmentTimers as expired;

Observation 10: According to the existing specification, even after applying the TDD pattern, UE may stop ongoing DL receptions and UL transmissions until the GNSS measurement and the subsequent procedures including valid GNSS position reacquisition, contention-based random-access procedure, and the remaining GNSS validity duration reporting are completed.

Proposal 6: For GNSS measurement gap, support

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap

- Conclusions

In this contribution, we discussed TDD mode for IoT-NTN. Based on the above discussion, our observations and proposals are given as follows:

Observation 1: According to the existing specification, for USS and Type 2/2A CSS, the search space including the starting subframe are defined in consecutive NB-IoT DL subframes excluding subframes used for transmission of SI messages.

Observation 2: According to the existing specification, the UE shall not expect NPDCCH in subframe that is not a NB-IoT DL subframe, and the NPDCCH transmission in subframes that are not NB-IoT DL subframes is postponed until the next NB-IoT DL subframe.

Observation 3: According to the existing specification, if the time gap between two contiguous search space is applicable to two contiguous NPDCCH occasions in the same search space, some combinations of G and R_max in the existing specification would not be used even without a consideration of new TDD pattern.

Observation 4: If new NPDCCH search space periodicity to be multiples of 90msec is introduced, the new offset definition would be necessary to allow the time locations of the k0 is different across different hyper frames like the TDD pattern.

Observation 5: If new DL transmission gap periodicity to be multiples of 90msec is introduced, the new offset definition would be necessary to allow the time locations of the DL transmission gap is confined within the DL burst in a TDD pattern period.

Observation 6: During the DL transmission gap within the DL burst of the TDD pattern, it is not guaranteed that eNB always use it for another NPDCCH transmissions, then it will results the DL resource waste.

Observation 7: Considering the limited number of available UL resources in TDD mode, the segment size of 8msec without any gap can maximize the UL resource utilization at the expense of the small time/frequency offset error.

Observation 8: When the NPUSCH transmission starts in the middle of the UL burst (8 consecutive uplink subframes), the UE does not need to perform the pre-compensation at the beginning of the UL burst. Moreover, if the UE does not have the NPUSCH transmission in the UL burst, the UE is not required to perform the pre-compensation during the UL burst.

Observation 9: According to the existing specification, even after applying the TDD pattern, the GNSS measurement gap will start outside the DL burst of the TDD pattern regardless of whether the UE provide HARQ-ACK feedback for the NPDSCH containing GNSS measurement command MAC CE.

Observation 10: According to the existing specification, even after applying the TDD pattern, UE may stop ongoing DL receptions and UL transmissions until the GNSS measurement and the subsequent procedures including valid GNSS position reacquisition, contention-based random-access procedure, and the remaining GNSS validity duration reporting are completed.

Proposal 1: No new periodicities are introduced for NPDCCH search space monitoring.

Proposal 2: DL gaps do not apply for NB-IoT NTN TDD mode.

Proposal 3: For IoT-NTN TDD mode, one or more of followings can be considered for initial access procedure:

- RAR window starts in DL subframe provided by the TDD pattern subject to the existing required time.

- UE (re)starts the contention resolution timer at the DL subframe provided by the TDD pattern subject to the existing required time.

Proposal 4: For IoT-NTN TDD mode, NPRACH offset is defined with respect to the beginning of the first TDD pattern within a hyper frame.

- Candidate for the new NPRACH offset: 0, 1, 2

Proposal 5: Confirm the following WA with some change:

- Working assumption

  - For precompensation, from RAN1 perspective:

    - The UE adjusts its time/frequency pre-compensation before the beginning of each set of consecutive 8 uplink subframes for the postponed NPUSCH transmission. No pre-compensation gap is needed before the beginning of each set of consecutive 8 uplink subframes.

    - FFS: Whether it is supported to perform segmented pre-compensation within the set of 8 consecutive uplink subframes, and whether in this case a pre-compensation gap is needed.

  - FFS: whether spec impact is in RAN1, RAN4 or both

Proposal 6: For GNSS measurement gap, support

- Option 1: No enhancement on the Rel-18 timeline for the start of GNSS gap

Reference

[1] RAN1#120bis Chairman’s note, Wuhan, China, April 7 – 11, 2025.