3GPP TSG RAN WG1 #121	R1-2504572

# St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.2.1

Source: 	ASUSTeK

Title:  	Discussion on UE initiated beam report

Document for:	Discussion and Decision

- Introduction

In this contribution, we discuss some aspects for UE initiated beam report.

- Discussion

In last RAN1 meeting, one left open issue is related to overlap between step-1 PUCCH and PUSCH.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121

- Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

  - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report

- Option-4: Reuse the SR dropping rules.

- FFS: whether/how to handle the case of different PHY priorities.

For option-1, as New UCI type has higher priority than SR except LRR and legacy rule from our understanding is dropping SR when overlapping with PUSCH, we think it’s simpler to make new UCI type having higher priority than PUSCH. For option-3, we think UE should always multiplex the 1-bit indication to avoid gNB's blind decoding. And regarding the left FFS for both options, we think the restriction is not needed (i.e. PUSCH should be with UL-SCH), since for Mode-B, step-2 PUSCH is without UL-SCH. For option-4, we don’t think SR dropping rule is needed since new type UCI is different than SR which means reusing legacy SR dropping rule is not technical justified. Thus, we are fine with either Option-1 or Option-3 according to majority support company.

Proposal 1: RAN1 supports Option-1 or Option-3 for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH.

Regarding the last FFS, if PUSCH is to provide critical service, we think it’s more beneficial to prioritize PUSCH than new UCI type. In one example, in Rel-16 URLLC work item, priority index (0, 1) is introduced, where priority index 1 has higher priority than priority index 0. When new UCI type is overlapped with PUSCH with priority index 1, we think UE should drop the new UCI type. Thus, for option-1, if support, we propose that new UCI type has priority than PUSCH except with priority index 1, if configured. On the other hands, for option-3, if support, we think it could follow legacy prioritization rule for different priority index and if priority index is the same, UE should always multiplex the 1-bit indication to avoid gNB's blind decoding.

Proposal 2: For case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, regarding different priority index between PUCCH and PUSCH.

- If option-1 is agreed, RAN1 supports new UCI type has higher priority than PUSCH except PUSCH with priority index 1, if configured.

- If option-3 is agreed, reusing legacy prioritization rule for different priority index

- Conclusion

In this contribution, we have following proposals:

Proposal 1: RAN1 supports Option-1 or Option-3 for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH.

Proposal 2: For case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, regarding different priority index between PUCCH and PUSCH.

- If option-1 is agreed, RAN1 supports new UCI type has higher priority than PUSCH except PUSCH with priority index 1, if configured.

- If option-3 is agreed, reusing legacy prioritization rule for different priority index