3GPP TSG RAN WG1 #121	R1-2504577

St. Julian’s, Malta, May 19th - 23rd, 2025

Agenda item:		9.15.10

Source:	Nordic Semiconductor ASA

Title:					On R19 TDD IoT-NTN features

Document for:		Discussion and Decision

# Introduction

In RAN1#114b in Wuhan, the following agreement was reached on the IoT NTN TDD capability

Agreement:

Introduce the following FG for IoT-NTN TDD mode

| Index | Feature group | Components | Prerequisite feature groups |  | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2-1 | Support of IoT-NTN TDD mode | 1. Support a TDD pattern with periodicity of N=9 radio frames, which consists of D=8 contiguous DL subframes and U=8 contiguous UL subframes. The guard period between the end of the DL subframes and the beginning of the UL subframes is fixed to be 50 ms at the ULSRP 2. The DL subframes of pattern are fixed to subframes [3 4 5 6 7 8 9 0] across two consecutive radio frames. 3. The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” 4. The non-D NB-IoT subframes are not considered by the UE as “NB-IoT DL subframes” 5. Support NPSS/NSSS/NPBCH transmissions dropped in non-D NB-IoT subframes 6. Postponement of NPRACH transmissions in non-U NB-IoT subframes until the next U NB-IoT subframe(s), where the unit of postponement is one PRU. [FFS Whether/how to capture components in Rel-17 2-1b, in case the FG is not defined to be the prerequisite one.] [FFS other component(s)/FG(s)] |  | [Rel-17 2-1b] | Yes | N/A | IoT-NTN TDD mode is not supported | [Per UE or Per band] | [TDD only] | [N/A] | Applicable only for 1616-1626.5 MHz MSS allocated band | FFS |

# Discussion

Regarding FFS points our views are the following:

- [Rel-17 2-1b] A basic NTN feature should be a pre-requisite of NTN IoT TDD. All components from Rel-17 2-1b are applicable also to NTN IoT TDD.

- [Per UE or Per band] For this feature there is no difference, i.e. Per UE = Per band, because there is only single band. On the other hand, if some other MNO would like to use this TDD protocol on a different band, it is better to have this feature “Per band”.

- [Mandatory/Optional] Feature should be mandatory without capability signaling, it is enough that UE indicates support for the b249, then it automatically supports NTN IoT TDD protocol.

- [TDD only] This one is tricky, because protocol follows FDD, even if band is TDD. In addition, this discontinuous TX/RX protocol can equally well run on FDD band. Whether someone would like to do so is of course questionable.