3GPP TSG RAN WG1 #121	R1-2504579

St Julian’s, Malta, May 19th – 23th, 2025

Source:	NICT

Title:	Discussion on Enhancements for UE-initiated/event-driven beam management

Item:	9.2.1

Document for:	Discussion

Introduction

In RAN1#120-bis meeting, the NR MIMO Phase 5 [1] was discussed. The following agreements were made [2].

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7. Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding Event-2, introduce an RRC parameter to indicate ServCellIndex on which the indicated TCI state used to determine the current beam RS is applied - FFS: Indication of the cell that corresponds to the configured first PUCCH resource in CSI report config including whether it is necessary Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, reuse the intra-UE multiplexing/prioritization rules of PUSCH with SP-CSI for Type-1 CG PUSCH with UEI-BR for Mode B Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, down-select one of the following options in RAN1#120bis. - Option-1: The measurement window is from T_PUCCH – T_proc – T_window to T_PUCCH – T_proc, where T_PUCCH is a transmission occasion of a first PUCCH, and T_proc is RRC configured. - Option-3: The length, slot offset and periodicity of a measurement window are configured per CSI report configuration by NW. - Option-4: If an Event-2 instance for a new beam is obtained at the time t [and the time for the new beam is not running], UE (re)starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: T_window is the agreed time window parameter for measurement. Agreement On UE-initiated/event-driven beam reporting, support the following for Event 1 - Regarding report format of L1-RSRP value of current beam, - Option-A2: L1-RSRP of current beam is absolute L1-RSRP (7-bit quantized, rather than 4-bit as in Event-2). Agreement On UE-initiated/event-driven beam reporting, regarding current beam report on Event 1, reuse RRC parameter enabledCurrentBeamReport-r19 to enable/disable the RSRP report of current beam. Conclusion On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the case that one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, there is NO RAN1 consensus on supporting multiple UEI beam reports carrying in a second PUSCH. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support the following rule for the Case-4: the 1-bit first PUCCH is collided/overlapped with a PUCCH format 0/1 carrying HARQ. - Option-1: Follow legacy SR multiplexing rule. - Note: There is no further enhancement on additional phase offset/constellation point(s). Conclusion Regarding resource mapping/configuration between first and second UL channel associated with a same CSI report configuration in Mode-B, there is NO consensus on further supporting the case that first PUCCH resource and pre-configured resource for second UL channel can have different periodicities (in ms). Agreement Regarding triggering event determination for Event 2, on the measurement window for initiating the UE-initiated/event-driven beam reporting procedure, support Option-4. - Option-4: If an Event-2 instance for a new beam is obtained at the time t and the timer for the new beam is not running, UE starts the timer for the new beam, where the expiry time of the timer is equal to the NW-configured length of the time window (T_window) - Note: Timer is new beam specific. - Note: T_window is the agreed time window parameter for measurement. - Introduce separate UE capability to limit the number of timers. There is only 1 timer per new beam. Above agreement is captured in RAN1 specifications. Agreement On UE-initiated/event-driven beam reporting, the procedure of triggering event determination is captured in RAN1 spec. Conclusion There is no RAN1 consensus on the following proposal: On UE-initiated/event-driven beam reporting, support at least Option-1 of following for Event 7 as an extension on report format for Event-2 - Option-1: Additional field to indicate the codepoint of the activated TCI state with Q-th best quality. - FFS: Further report codepoints of other activated TCI state(s) [, e.g., from {Q+1}-th best quality to the worst one]; - FFS: Details of the additional indication. - Option-2: Extending the maximum number of reported RS(s) to 8. - [Note: If Option 2 is agreed, RAN1 can revisit the agreed maximum value of N for event-2.] - Above is applicable only if time window and a configurable value M for event counting on Event-7 are not configured. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting for a CSI report configuration, down-select at least one of the following candidates in RAN1#121: - Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH; - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted. - Candidate#2: To introduce a time interval for mode-A and/or mode-B - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH. - Candidate#3: No further enhancement. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, down-select one of the following rules for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, in RAN1#121 - Option-1: Prioritize first PUCCH over PUSCH, i.e., PUSCH is dropped. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH. - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered. - FFS: If the PUSCH should be with UL-SCH or not for UEI beam report - Option-4: Reuse the SR dropping rules. - FFS: whether/how to handle the case of different PHY priorities. Conclusion There is no RAN1 concensus on the following. No spec change needed. On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, support one of the following rule for the Case-3: the 1-bit first PUCCHs corresponding to different CSI configuration for UE-initiated/event-driven beam reporting are overlapping in the time domain - Option-1: Per CSI report configuration. - Option-2: Up to implementation. - Option-3: UE transmits the first PUCCH with higher priority (e.g., PUCCH resource with lower PUCCH resource ID, or event-type) - Option-4: Multiplexing all 1-bit indications Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, confirming the following working assumption with the following modification. - For Mode-A, the multiple CSI report configurations associated with the same PUCCH resource should be associated with a same CSI-AperiodicTriggerState. - FFS: For Mode-A, the multiple CSI report configurations associated with the same CSI-AperiodicTriggerState should be associated with a same PUCCH resource. Conclusion There is no RAN1 concensus on the following. No spec change needed. For event 2, when one PUCCH resource of first PUCCH can be associated with one or multiple CSI report configurations, and if multiple UE initiated beam report procedures occur, down-select one of the following options: - Option-1: It is up to UE implementation to select one of configuration. - Option-2: The UEI beam report with highest priority is reported - FFS: priority, e.g., lowest CSI-ReportConfigId, event-type or based on legacy CSI report priority rule - Option-3: The report triggered in the latest measurement is reported in PUSCH

In this contribution, we present our views on UE-initiated/event-driven beam management and proposals for moving forward.

Discussion

UL signaling content(s)

Regarding the additional report contents, it has been agreed that L1-RSRP is supported. In our view, L1-SINR should be also supported. Considering that L1-RSRP cannot reflect the level of interference, it is beneficial to also support L1-SINR report.

Proposal 1: As for the contents of the beam report, L1-SINR can be carried in the beam report.

UL signaling medium/container

The first issue is how to support prohibit timer of first PUCCH transmission. In the RAN1#120bis meeting, the following options were given.

- Candidate#1: To introduce a prohibit timer for mode-A and/or mode-B

  - Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

  - Option-1B: In the case that triggering-event associated with the CSI report configuration is determined, if the prohibit timer is NOT running, the UE can transmit first PUCCH;

    - At the first symbol after the end of the first PUCCH transmission, the UE starts the prohibit timer

  - If the prohibit timer is running, the first PUCCH is not allowed to be transmitted.

- Candidate#2: To introduce a time interval for mode-A and/or mode-B

  - Option-2A: For a first PUCCH transmission occasion, if there is a transmission of another second PUSCH [corresponding to the CSI report determined by the same triggering beam(s) as the first PUCCH] within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

  - Option-2B: For a first PUCCH transmission occasion, if there is a transmission of another first PUCCH(s) within a configurable time interval before the first PUCCH transmission occasion, the UE should not transmit the first PUCCH.

- Candidate#3: No further enhancement.

We support Option-1A for Mode A and Mode B. For Mode B, both Option-1A and Option-1B could work. On the other hand, for Mode A, Option-1A should be supported since large latency would happen by waiting for expiry of prohibit timer when first PUCCH is not successfully received by NW.

Proposal 2: Regarding a prohibit timer for Mode A and Mode B, support Option-1A.

- Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

The next issue is multiplexing/dropping rule for first PUCCH of Mode A and B. Option-1 would be simpler solution than Option-3, but performance degradation on latency would happen in case of the collision. On the other hand, that would not happen if Option-3 is supported.

Proposal 3: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, support Option-3,

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - The PUSCH can be with UL-SCH or not for UEI beam report.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

Conclusion

In this contribution, we presented the following proposals:

Proposal 1: As for the contents of the beam report, L1-SINR can be carried in the beam report.

Proposal 2: Regarding a prohibit timer for Mode A and Mode B, support Option-1A.

- Option-1A: In the case that triggering-event associated with the CSI report configuration is determined [by the same triggering beams(s) as the last transmitted PUCCH], if the prohibit timer is NOT running, the UE can transmit first PUCCH;

  - At the first symbol after the end of the PUSCH transmission, the UE starts the prohibit timer

Proposal 3: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing/dropping rule(s) of 1-bit first PUCCH, for the Case-2: the 1-bit first PUCCH is collided/overlapped with a PUSCH, support Option-3,

- Option-3: Piggyback 1-bit indication of first PUCCH into the PUSCH.

  - The PUSCH can be with UL-SCH or not for UEI beam report.

  - FFS: The 1-bit indication is always multiplexed in the PUSCH, regardless that UEI beam report procedure is triggered.

References

- RP-234007, New WID: NR MIMO Phase 5, 3GPP RAN #102, Dec 11-15, 2023.

- Chair’s notes, RAN1#120bis, Wuhan, China, April 7th – 11th, 2025.