3GPP TSG RAN WG1 #121		 R1-2504584

St Julian’s, Malta, May 19th – 23th, 2025

Agenda Item:	9.11.3

Source:	Google

Title:	Discussion on NR-NTN uplink capacity/throughput enhancement

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our view on NR-NTN uplink capacity/throughput enhancement.

# Discussion

## Signaling aspects of PUSCH with OCC

Agreement For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, consider the following options: - Option 1: Configured by RRC higher-layer parameter. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 2: Max value is configured by RRC higher-layer parameter, and the applied value is implicitly determined from the configured repetition number. - Option 3: Candidates values are configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH or by activation DCI for CG PUSCH Type 2. - Note: this does not preclude jointly configuring OCC sequence and OCC length with the same RRC configuration - Option 4: Single value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2 (no RRC configuration of candidate values). - Option 5: Max value is configured by RRC higher-layer parameter, and the applied value is indicated by scheduling DCI for DG PUSCH and by activation DCI for CG PUSCH Type 2. Note: it is not precluded to select a different option for DG PUSCH and CG PUSCH Type 2.

In the last meeting, RAN1 made the agreement above for further discuss how to indicate OCC length for DG PUSCH and CG PUSCH Type 2. In current specifications, the repetition number for DG PUSCH is indicated by TDRA configuration (i.e., PUSCH-Allocation). From our perspective, OCC length can be indicated with repetitions in the TDRA configuration. As for CG PUSCH type 2, the number of repetition is configured by ConfiguredGrantConfig. We didn’t see the necessity of indicate a dynamic OCC length for a configured repetition number. In this regard, we think that RRC configured OCC length is sufficient for DG PUSCH and CG PUSCH Type 2.

Proposal 1: For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, it is configured by RRC higher-layer parameter (e.g., in PUSCH-Allocation and ConfiguredGrantConfig)

## 2.2 OCC group RV cycling

Agreement For OCC with CG-PUSCH, the RV sequence applied across OCC groups is RRC configured among the RV sequences defined for legacy CG PUSCH, i.e., [0,2,3,1], [0,3,0,3] or [0,0,0,0]. - Note: no new RRC parameter is needed for the above. - FFS: if OCC group dropping is later agreed, how to count RVs may need to be discussed for that case.

According to previous agreements, if the length of repetition is larger than the OCC length, RV cycling is applied to each OCC group according to the legacy RV sequences. One remaining issue is that if the UE has to drop an OCC group, how to apply the RV. From our perspective, the applied RV does not affect the orthogonality of the OCC, thus it would be preferred to postpone the RV of a dropped OCC group to the next available OCC group.

Proposal 2: If OCC group dropping is applied, postpone the RV of a dropped OCC group to the next available OCC group.

# Conclusion

Proposal 1: For the OCC length applied for OCC DG PUSCH and CG PUSCH Type 2, it is configured by RRC higher-layer parameter (e.g., in PUSCH-Allocation and ConfiguredGrantConfig)

Proposal 2: If OCC group dropping is applied, postpone the RV of a dropped OCC group to the next available OCC group.

# Reference

- “Draft Report of 3GPP TSG RAN WG1 #120bis v0.1.0, Wuhan, China, April 07th – 11th, 2025

- R1-2503094, “Feature lead summary #4 of AI 9.11.3 on NR-NTN uplink capacity and throughput,” Moderator (MediaTek), Wuhan, China, April 07th – 11th, 2025