3GPP TSG RAN WG1 #120bis		                                       R1-2504597

St Julian’s, Malta, May 19th – 23rd, 2025

Agenda item:        9.1.3

Source        :	CEWiT

Title             :	Discussion on specification support for CSI prediction

Document for:      Discussion

__________________________________________________________________

# Introduction

Rel-19 NR AI/ML for Air Interface WI was revised in [1], we will discuss the following objectives for CSI prediction in this contribution.

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

# Performance Monitoring

In RAN1 #120bis meeting, there were significant discussions with respect to performance monitoring. It was decided that for UE side CSI prediction, support UE assisted performance monitoring subject to an additional UE capability and it was further decided on Option-3 type of monitoring.

Option-3 calculates the performance metric between the ground truth and predicted CSI and reports it to the NW for further decision. The performance metric to be reported was agreed to be SGCS.

In case of reporting the SGCS result to the NW, several options were given and agreed.

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

In Alt 1 based approach, UE calculates SGCS based on predicted CSI and the corresponding ground truth CSI for one inference reporting. This is the simplest option and without any confusion.

Proposal-1: For CSI prediction using UE sided model, for reporting contents of UE assisted performance monitoring, support option Alt-1.

In case of multiple predicted values, study methods to report the SGCS values as per instance or averaged over prediction instances.

With respect to Alt-2, UE calculates two SGCS values, SGCS1 between the predicted CSI and the ground truth CSI and SGCS2between the predicted CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion. In this type, there should be an indication to differentiate between the SGCS values. In our opinion, this requires explicit indication and higher spec impact. It also brings upon additional calculation complexity at UE side.

Proposal-2: For CSI prediction using UE sided model, for reporting contents of UE assisted performance monitoring, deprioritize Alt-2.

# Data Collection

## For training

In RAN1 #120bis meeting, the following agreement was made on data collection for CSI prediction. It was agreed based on the agreement in the BM agenda.

Agreement For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig

In last meeting two options were provided for this case.

- Option 1: reuse existing parameter, i.e., reportQuantity set to ‘none’. - Option 2: introduce new higher layer parameter - Option 2-1: explicit indication of data collection purpose, e.g., resourcesForDataCollection - Option 2-2: reportConfigType-r19 with “none

- In case of Option-1, reportQuantity set to ‘none’; it is simple approach and there is no specification impact.

- In case of Option-2, introduce a new higher layer parameter

  - Option 2-1: explicit indication of data collection purpose, e.g., resourcesForDataCollection; specific to CSI related  AI/ML use-case;

  - Option 2-2 ; reportConfigType-r19 with “none”

Proposal-3: For CSI prediction using UE-side model, for training data collection, support, reportQuantity set to ‘none’.

# Conclusion

In this contribution, the following proposals are made:

Proposal-1: For CSI prediction using UE sided model, for reporting contents of UE assisted performance monitoring, support option Alt-1.

Proposal-2: For CSI prediction using UE sided model, for reporting contents of UE assisted performance monitoring, deprioritize Alt-2.

Proposal-3: For CSI prediction using UE-side model, for training data collection, support, reportQuantity set to ‘none’.

# 5.References

- RP-243244, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm (Moderator), 3GPP TSG RAN #105, September 2024.

- 3GPP RAN1 WG1 #120bis, Chairman Notes

- 3GPP RAN1 WG1 #120, Chairman Notes

- 3GPP RAN1 WG1 #119, Chairman Notes

- 3GPP RAN1 WG1 #118-bis, Chairman Notes