3GPP TSG RAN WG1 #121                                                  R1- 2504606

St Julian’s, Malta, May 19th – 23th, 2025

Source: Google

Title:	On Low band carrier aggregation via switching

Document for: Discussion and Decision

# Introduction

A RAN4-led work item on low-band carrier aggregation via switching was approved  in RAN#106, and it was further revised in RAN#107 with following objectives [1]:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives:

- Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1]

  - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

  - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

  - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary

  - Specify the switching delay and time mask for carrier switching [RAN4]

- Specify necessary RRM requirements [RAN4]

- Define the corresponding UE capabilities [RAN4, RAN2, RAN1]

- Consider the following deployment constraints:

  - The carrier frequency for all cases is <1 GHz

  - Co-located and synchronized network deployment for both carriers

  - Both carriers are in a single TAG

  - SCS 15KHz on both carriers

Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach

Note 2: Strive to minimize the RAN1 impact

In this Tdoc, we share our views and propose techniques to specify the above enhancements.

# Switching Gap Location

In RAN1#120bis, the following agreement has been reached:

Agreement

For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap,

- For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier.

- For switching from FDD carrier to SDL carrier, downselect one of the following solutions

  - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

  - Alt 3: Switching gap is always assumed at the SDL carrier

Alt 2 with a network-configurable switching gap is preferred because of its adaptable nature. This flexibility allows the network to flexibly place the switching gap to effectively match different configurations, different configured signals and to take into account the timing advance.

As a result, the network will be able to make well-informed choice to protect some channels or signals as needed in any particular configuration.

For instance, the effect on PCell transmissions can be minimised by locating the gap on a SDL carrier, whilst placing it at the end of a carrier active time will better protect the reception of PDCCH on the next carrier. But aiming to protect PDCCH on the carrier being switched to, might unintentionally disrupt other important signals on the current active carrier like CSI-RS, SRS or PUCCH often using symbols towards the end of a slot. And this highlights the practical advantage of having a flexible technique where the network can control the location of the switching gap.

- For the location of the switching gap,  support Alt 2 with a network-configurable switching gap:

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

# Switching Pattern

In RAN1#120bis, the following agreement has been reached for the semi-static switching pattern:

Agreement

- For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design

  - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2

  - The pattern starts from the beginning of SFN 0 of PCell

- Restriction of maximum X switch(es) within Y slot(s),

  - FFS: detailed value and relation of X and Y.

- FFS: other restriction

For the selection of the values X and Y, a sliding window within any continuous number Y of slots (which could span across the pattern periodicities) can be specified, the number of carrier switches in the continuous window of Y slots must not exceed X. The sliding window approach is motivated by the need to avoid clustering of switches, which might not be managed by a simple limit within the overall periodicity P. A condition X < Y/T can be defined. This means the number of switches X in any window Y must be less than Y divided by the threshold T. For example:

- P = 10 ms (which is 10 slots at 15kHz SCS, assuming 1ms slots).

- Y = 5 ms (5 slots).

- T = 2 (slots/switch, meaning on average at least 2 slots between switches, or a switch occurs no more frequently than every 2 slots on average within the window).

- So, X < 5 slots / 2 slots/switch => X < 2.5. Since X is an integer, this means X can be 0, 1, or 2.

- For the selection of the values X and Y, a sliding window within any continuous number Y of slots (which could span across the pattern periodicities) can be specified, the number of carrier switches in the continuous window of Y slots must not exceed X.

- A condition X < Y/T can be defined. This means the number of switches X in any window Y must be less than Y divided by the threshold T. T can be viewed as the minimum number of slots between the switches. T is reported as a UE capability

# Potential additional switches

Current understanding of semi-static carrier switching pattern implies a UE follows a semi-static RRC configured schedule. However, certain UE-initiated procedures or network-initiated critical events may necessitate deviation from this pattern. For example, if a UE needs to perform a RACH procedure, it typically does so on the PCell. If the semi-static pattern has scheduled the UE onto an the SDL SCell with no UL transmission for a given duration, the UE might be delayed in initiating RACH if it cannot autonomously switch back to the PCell. More critically, if an event requiring immediate UE attention on the PCell occurs while the UE is operating on an SCell as per its configured pattern, the UE might be unable to switch back to the PCell in a timely manner. Examples of such events include:

  - Imminent transmission of a handover command on PCell.

  - Network initiation of lower-layer triggered mobility procedures requiring PCell interaction.

  - PCell experiencing beam degradation requiring Beam Failure Recovery (BFR) procedures.

  - Pending Radio Link Failure (RLF) on PCell, where the UE might need to perform measurements or receive recovery instructions on PCell.

Strict adherence to the SCell duration in such scenarios could lead to the UE missing critical signaling. This might result in the UE becoming isolated or appearing unresponsive while on the SCell, potentially leading to RLF declaration, call drops, or failure of essential mobility procedures.

To mitigate the risks outlined above, it is crucial to define a mechanism that allows the UE to rapidly switch back to the PCell, even if this means deviating from the ongoing semi-static carrier switching pattern. Such a mechanism would act as a high-priority override to ensure the UE can receive and act upon critical PCell procedures.

The envisioned mechanism would involve a trigger, receivable by the UE on its currently active SCell, instructing it to switch back to the PCell.

- The UE can perform or the network can trigger the UE to perform carrier switching outside the semi-static carrier switching pattern to initiate or respond to specific high-priority procedures. The set of procedures and conditions for such UE deviation needs further study (e.g., RACH initiation on PCell, handover command, RLF, LTM, …).

Also, consideration is required where critical procedures, such as Beam Failure Recovery (BFR) or handover, are triggered while the UE is operating on the PCell, and a switch to an SCell is imminently scheduled according to the RRC-configured semi-static carrier switching pattern. Following the carrier switching pattern in such situations could disrupt the ongoing BFR or handover procedure. Prioritization of these critical procedures on the PCell over a the configured pattern switch is therefore essential for service continuity and robustness.

- When a Beam Failure Recovery (BFR) procedure or a handover procedure is initiated while the UE is active on PCell, the UE shall disregard any imminent switch to an SCell as per a configured semi-static carrier switching pattern and shall remain on PCell to complete the ongoing BFR or handover procedure.

# Co-existence between measurement gaps and switching gaps

The interaction between RRC-configured measurement gaps (MGs) utilized, for example, for inter-frequency or inter-RAT measurements and the configured FDD/SDL semi-static carrier switching pattern needs to be addressed for cases where an MG coincides with a switching gap of the pattern.

It is preferred that in such an overlap, the UE would prioritize the measurement gap. Following the completion of the measurement gap, the UE would then revert to following the configured semi-static switching pattern. The switching gap in that case can be cancelled as the UE would have enough time within the measurement gap to tune back to the carrier of semi-static switching pattern.

- When an RRC-configured measurement gap overlaps with a switching gap associated with the semi-static carrier switching pattern, the UE shall prioritize the measurement gap (i.e., performing measurements). Upon completion of the measurement gap, the UE shall resume adherence to the semi-static carrier switching pattern.

# Co-existence between DRX and the carriers switching pattern

The co-existence of DRX with a configured semi-static carrier switching pattern needs further clarification. As a method to ensure alignment with UE power-saving objectives, it is preferred that the semi-static carrier switching pattern is effective only during periods of DRX Active Time. Consequently, time slots falling within DRX Inactive Time are not counted in the switching pattern. As a result, the UE would not be expected to perform carrier switches, as dictated by such a pattern, while it is in DRX Inactive Time.

- When a UE is configured with both DRX and a semi-static carrier switching pattern, the switching pattern shall only be considered during DRX Active Time.

- The UE is not expected to do carrier switches during DRX Inactive Time.

For a specific scenario, where the switching gap overlaps with the UE transitioning from DRX Active Time to DRX Inactive Time. This can happen upon the expiry of timers such as drx-OnDurationTimer or drx-InactivityTimer. For such scenario, it is proposed that the UE prioritizes the DRX configuration by transitioning to DRX Inactive Time. Consequently, the carrier switch dictated by the pattern would be postponed. Upon the UE re-entry into DRX Active Time, it would then re-evaluate its state within the semi-static switching pattern to determine the appropriate active carrier.

- If a switching gap, as defined by a semi-static carrier switching pattern, is scheduled to coincide with the expiry of drx-OnDurationTimer or drx-InactivityTimer (thus aligning with the transition to DRX Inactive Time), the UE shall prioritize entering DRX Inactive Time. The scheduled carrier switch shall be deferred, and the UE shall determine the appropriate carrier based on the state of the semi-static switching pattern upon starting the next DRX Active Time.

# Overlapping between channels/signals and switching gaps

During periods where a UE is undergoing a switching gap, it is generally understood that the ability of the UE to transmit or receive is temporarily impaired. To avoid complexity and performance degradation, collisions between scheduled or configured channel/signal activity and these switching gaps should be avoided by network scheduling and configuration.

From the UE's perspective, the presence of expected channel/signal transmissions or receptions that overlap with a necessary switching gap creates an ambiguous situation.  Therefore, the UE should not be expected to successfully transmit or receive signals or data that entirely or partially overlap with a configured or indicated switching gap.

If such an overlap were to occur due to network scheduling or configuration, the UE cannot be assumed to be capable of handling the intended transmission or reception. This situation can be considered an exceptional case from the UE operationa perspective.

- The UE is not expected to receive or transmit during the switching gap. Any configured/scheduled transmission/reception during switching gap is considered as error case.

# Conclusion

In this contribution, we have presented several proposals to specify the support of low band carrier aggregation. We made the following proposals:

- For the location of the switching gap,  support Alt 2 with a network-configurable switching gap:

  - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier

- For the selection of the values X and Y, a sliding window within any continuous number Y of slots (which could span across the pattern periodicities) can be specified, the number of carrier switches in the continuous window of Y slots must not exceed X.

- A condition X < Y/T can be defined. This means the number of switches X in any window Y must be less than Y divided by the threshold T. T can be viewed as the minimum number of slots between the switches. T is reported as a UE capability

- The UE can perform or the network can trigger the UE to perform carrier switching outside the semi-static carrier switching pattern to initiate or respond to specific high-priority procedures. The set of procedures and conditions for such UE deviation needs further study (e.g., RACH initiation on PCell, handover command, RLF, LTM, …).

- When a Beam Failure Recovery (BFR) procedure or a handover procedure is initiated while the UE is active on PCell, the UE shall disregard any imminent switch to an SCell as per a configured semi-static carrier switching pattern and shall remain on PCell to complete the ongoing BFR or handover procedure.

- When an RRC-configured measurement gap overlaps with a switching gap associated with the semi-static carrier switching pattern, the UE shall prioritize the measurement gap (i.e., performing measurements). Upon completion of the measurement gap, the UE shall resume adherence to the semi-static carrier switching pattern.

- When a UE is configured with both DRX and a semi-static carrier switching pattern, the switching pattern shall only be considered during DRX Active Time.

- The UE is not expected to do carrier switches during DRX Inactive Time.

- If a switching gap, as defined by a semi-static carrier switching pattern, is scheduled to coincide with the expiry of drx-OnDurationTimer or drx-InactivityTimer (thus aligning with the transition to DRX Inactive Time), the UE shall prioritize entering DRX Inactive Time. The scheduled carrier switch shall be deferred, and the UE shall determine the appropriate carrier based on the state of the semi-static switching pattern upon starting the next DRX Active Time.

- The UE is not expected to receive or transmit during the switching gap. Any configured/scheduled transmission/reception during switching gap is considered as error case.

# References

- RP-250247, “Revised WID on Low NR band carrier aggregation via switching”