3GPP TSG RAN WG1 #121                               		                                       R1-2504656

St Julian’s, Malta, May 19th – 23th, 2025

Source:	Moderator (OPPO)

Title:	Summary #2 on Rel-19 asymmetric DL sTRP/UL mTRP

Agenda Item:	9.2.4

Document for:	Discussion and Decision

# Introduction

This document summarizes remaining issues proposed in company contributions of AI 9.2.4

# Issues for Discussions

## Pathloss Offset

Proposal 1.5 v1:

In asymmetric DL sTRP/UL mTRP deployment scenarios, after the beam failure recovery is completed successfully, support the following for PCell BFR and SCell BFR (not per-TRP BFR)

- the UE shall include the PL offset of the indicated TCI state on PL estimation based on q_new for the transmission of PUSCH/PUCCH/SRS towards the UL TRP;

- The UE does not reset the spatial domain filter for the PUSCH/PUCCH/SRS towards the UL TRP.

Proposal 1.5 v2:

In asymmetric DL sTRP/UL mTRP deployment scenarios, after the beam failure recovery is completed successfully, support the following for PCell BFR and SCell BFR (not per-TRP BFR)

- The UE does not reset the spatial domain filter and power control parameters on the PUSCH/PUCCH/SRS that follows the indicated TCI state if the indicated TCI state is configured with PL offset or associated with an SSB of additionalPCI.

| Company | Comments |
| --- | --- |
| Mod00 | In the offline discussion, it looks like more companies prefer v2. However, as Ericsson commented, this proposal might not be needed and the reason is: when BFR happens, it is ok to temporarily switch those UL transmission to anchor TRP and after than the gNB can switch those UL transmission to UL TRP through TCI state switch, which would be the system behavior per the legacy specification. If you have any further comments, please input here. |
| ZTE | Not support. We share the understanding from Ericsson on the necessity of this discussion, i.e., this enhancement is not needed. |
| QC | Not support. This is specific for beam management enhancement in FR2 which is out of the scope. |
| Panasonic | We do not support. It is not clear whether this suboptimal “enhanced” procedure will perform better than legacy BFR procedure. |
| Ofinno | In our view, legacy BFR procedure in this context needs to be clarified. There is no legacy BFR for this scenario. Following principles of mTRP BFR, we suggest not to reset the spatial domain filter of UL TRP when a BFR occurs on DL TRP. Note that in mTRP with independent BFR procedure, a UE does not reset a TRP that is not under the BFR procedure. We do not see why we need to change a behavior from the legacy. So, we suggest first agree that “The UE does not reset the spatial domain filter for the PUSCH/PUCCH/SRS towards the UL TRP”. For power determination, between v1 and v2, we slightly prefer v1 as it gives at least more reliable pathloss measurement. But we can go with the majority. We are okay with v1 or v2. |
| CATT | Not support. We would like to emphasize that this is out of scope and we should ease the discussion. |

## Two Separate CLPC adjustment states for SRS

N/A

## Two TAs

Proposal 3.2:

For asymmetric DL Strp/UL Mtrp deployment scenario, when the TCI state(s) applied on UL transmission has an expired TA value, the UE shall skip that UL transmission.

In the round 1 discussion, many companies are not ok with the proposal:

- Ok/Support: Xiaomi, google

- Not Ok/Not needed: ZTE, Lenovo, Huawei/HiSilicon, LG, vivo, Nokia (but also open for discussion), ETRI

Many companies (MediaTek/NEC/OPPO/QC/DOCOMO/CATT/Ofinno) suggest that should be an RAN2 issue.

| Company | Comments |
| --- | --- |
| Mod00 | Per the round 1 inputs, majority companies do not support this proposal. The reasons include (1) not needed and the case can be handled by Gnb since Gnb also knows the timer of TA (2) that should be handled by RAN2 if that is an issue. However, if you have further comments, please input here. |
| ZTE | Not support. It is clear to us this is error case from the perspective of NW implementation/scheduling. |
| QC | The UE behavior when TA timer expires is specified in RAN2 spec. We prefer to leave this up to RAN2. |
| Ofinno | We don’t agree with Gnb handling this case, at least not dynamically. So, this is a valid problem. On the other hand, the UE behavior in the TAT expiry has been captured in RAN2. So, in our view, this is RAN2 issue. |
| CATT | Not Support. This is gNB implementation. |

# Proposals for Online Discussion

Proposal 1.5 v2:

In asymmetric DL sTRP/UL mTRP deployment scenarios, after the beam failure recovery is completed successfully, support the following for PCell BFR and SCell BFR (not per-TRP BFR)

- The UE does not reset the spatial domain filter and power control parameters on the PUSCH/PUCCH/SRS that follows the indicated TCI state if the indicated TCI state is configured with PL offset or associated with an SSB of additionalPCI.

# Contributions in 9.2.4

- R1-2503278	Enhancements for asymmetric DL sTRP/UL mTRP scenarios	Huawei, HiSilicon

- R1-2503304	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	MediaTek Inc.

- R1-2503354	Remaining issues on asymmetric DL sTRP/UL mTRP scenarios	vivo

- R1-2503436	Remaining Details of Rel-19 Asymmetric mTRP Operation	InterDigital, Inc.

- R1-2503511	Enhancements for asymmetric DL sTRP/UL mTRP scenarios	Spreadtrum, UNISOC

- R1-2503558	Views on Rel-19 asymmetric DL sTRP/UL mTRP scenarios	Samsung

- R1-2503671	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	ZTE Corporation, Sanechips, China Telecom

- R1-2503712	Enhancement for asymmetric DL sTRP UL mTRP	Tejas Network Limited

- R1-2503730	Enhancements on asymmetric DL STRP/UL MTRP scenarios	Ofinno

- R1-2503789	Remaining issues on asymmetric DL sTRP and UL mTRP scenarios	CATT

- R1-2503817	Enhancement for Asymmetric DL sTRP/UL mTRP Scenarios	Panasonic

- R1-2503826	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	CMCC

- R1-2503878	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	Xiaomi

- R1-2503915	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Lenovo

- R1-2503929	Discussion on enhancements for asymmetric DL sTRP and UL mTRP scenarios	NEC

- R1-2503978	Remaining issues on asymmetric DL sTRP UL mTRP scenarios	Ericsson

- R1-2503986	Discussions on asymmetric DL sTRP/UL mTRP scenarios	LG Electronics

- R1-2504063	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Sony

- R1-2504079	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Nokia

- R1-2504135	Remaining issues in asymmetric UL MIMO operation	ETRI

- R1-2504173	Discussion on enhancements for asymmetric DL sTRP/UL mTRP scenarios	Transsion Holdings

- R1-2504231	Enhancements on asymmetric DL sTRP/UL mTRP scenarios	OPPO

- R1-2504298	Discussion on enhancement for asymmetric DL sTRP and UL mTRP scenarios	Google

- R1-2504314	Enhancements for asymmetric DL sTRP/UL mTRP scenarios	Apple

- R1-2504390	Enhancement for asymmetric DL sTRP and UL mTRP deployment scenarios	Qualcomm Incorporated

- R1-2504458	Enhancement for asymmetric DL sTRP/UL mTRP scenarios	Sharp

- R1-2504463	Discussion on asymmetric DL sTRP/UL mTRP scenarios	TCL

- R1-2504497	Discussion on enhancement for asymmetric DL sTRP/UL mTRP scenarios	NTT DOCOMO, INC.