3GPP TSG RAN WG1 Meeting #121	R1- 2504697

St. Julian, Malta, April 7th – 11st, 2025

Source: 	Moderator (Intel Corporation)

Title:	Summary #1 of discussions for draft CR for 7-24 GHz SI

Agenda item:	9.8

Document for:	Discussion

# Introduction

In this contribution, moderator summarizes discussion on the draft CR for 7-24 GHz SI.

# Discussion on draft CR

#### Discussion

Please provide comments on draft CR v39

https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_121/Inbox/drafts/9.8(FS_NR_7_24GHz_CHmod)/CR/draft%20R1-250xxxx%20Draft%20CR%20for%207-24GHz%20CM_v39.docx

| Company | Comments |
| --- | --- |
| vivo | In 7.3.2, the use of (7.3-3) is not correct. The reason is as follows. In TRP, the orientation of each antenna element is the same, however, the orientation of each antenna element in UE is different. Therefore, two rotations are necessary in each UE. In the first rotation, we need to perform the transformation from Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ to Fu,θ''θ',ϕ' and Fu, ϕ''θ',ϕ', where Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ are the field pattern according to the orientation of the each antenna element in LCS of each antenna element, and Fu,θ''θ',ϕ' and Fu, ϕ''θ',ϕ' are the field pattern according to the orientation of the each antenna UE in LCS of each UE. In such transformation, the following equation is in use Ψ'=argsinγ'cosθ'sinϕ'-α'+cosγ'cosβ'sinθ'-sinβ'cosθ'cosϕ'-α'+jsinγ'cosϕ'-α'+sinβ'cosγ'sinϕ'-α' Then, the transformation relation referred in 7.1.11 in TR38.901 is expressed by Fθ''(θ',ϕ')Fϕ''(θ',ϕ')=cosΨ'-sinΨ'sinΨ'cosΨ'Fθ″″θ″,ϕ″Fϕ″″θ″,ϕ″. In the second rotation, we need to perform the transformation from Fu,θ''θ',ϕ' and Fu, ϕ''θ',ϕ', where Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ to Fθ(θ,ϕ) and Fϕ(θ,ϕ) are the field pattern according to the orientation of the each UE antenna in LCS of each UE antenna, and Fθ(θ,ϕ) and Fϕ(θ,ϕ) are the field pattern according to the orientation of the each antenna UE in GCS. In such transformation, the following equation is in use Ψ=argsinγcosθsinϕ-α+cosγcosβsinθ-sinβcosθcosϕ-α+jsinγcosϕ-α+sinβcosγsinϕ-α Then, the transformation relation referred in 7.1.11 in TR38.901 is expressed by Fθ(θ,ϕ)Fϕ(θ,ϕ)=cosΨ-sinΨsinΨcosΨFθ''(θ',ϕ')Fϕ''(θ',ϕ'). |
| Apple | Section 7.8.3, Table 7.8-7 on “BS antenna downtilt”, we suggest removing “(i.e., array panel downward facing)”, since this is assumed to be electrical downtilt angle, not mechanical downtilt angle. |
| Moderator (ZTE) | The v23 is uploaded to capture the agreements in section 9.8.2 in the Monday online session. Companies are encouraged to check it. |
| Moderator (Intel) | V24 is uploaded to capture agreements from agenda 9.8.1 from Monday online session. @vivo: let’s discuss this as part of the antenna modeling discussion in agenda 9.8.1 @Apple: this may be something that requires some discussion. Moderator understanding is that this should be mechanical downtilt. Moderator will create a proposal for discussion in agenda 9.8.1 |
| Moderator (ZTE) | The v25 is uploaded to capture the agreements in section 9.8.2 in the Wednesday online session. Companies are encouraged to check it. |
| Moderator (Intel) | V26 is uploaded to capture agreements from agenda 9.8.1 from Wednesday online session. |
| Huawei, HiSilicon | Thanks Moderators for the great effort! We have the following suggestion for V26. 7.6.14.1.3 Stochastic based Model < Unchanged parts are omitted > Step II: Generate the visibility region (VR) For the nth cluster which is assumed to be impacted by the invisibility, the visibility region is generated as a rectangle region with dimension a*b by using: a~unif(Vn*W,W), (7.6.14.1.2-3) b=Vn*H*W/a, (7.6.14.1.2-4) where H is the height of TRP antenna array (i.e., in vertical direction) and W is the width of antenna array (i.e. in horizontal direction). The Vn is generated as: Vn=A*e-maxPn-PnR+Β+δ. (7.6.14.1.2-5) where Pn refers to the power of cluster generated by (7.5-6) in dB scale. In the case of LOS condition, the LOS path is considered as an additional cluster, and the power ratio of the LOS path to NLOS clusters follows the Ricean K-factor generated in Section 7.5. The parameters A, B, R and δ are defined in Table 7.6.14.1.2-2. Table 7.6.14.1.2-2: Parameters for A, B, R, and δ RMa; SMa; UMa; UMi; InH; InF. A; 0.16; 0.06; 0.15; 0.12; 0; 0. B; 0.74; 0.56; 0.45; 0,48; 0.6; 0.57. R; 60; 23; 33; 50; NA; NA. δ Nμ, σ2; μ; 0; 0; 0; 0; 0; 0. σ2; 0.0016; 0.0013; 0.0015; 0.001; 0.0011; 0.002. Each visibility region is assumed to be randomly located at a one of the corner of TRP antenna array (i.e., reference corner), which is determined withby the following conditions: - The probability of VR located at either left or right part of antenna array in horizontal domain is 0.5. - The probability of VR located at either upper or lower part of antenna array in vertical domain is 0.5. Step III: Determine the antenna element-wise power attenuation factor For the nth cluster which is assumed not to be impacted by the invisibility, the αu,n=1 or αs,n=1. For the nth cluster which is assumed to be impacted by the invisibility, the αu,n=αi,n or αs,n=αi,n. αi,n=1 ,xi-x0,n≤a,yi-y0,n≤be-xi-xa,nDn∙C ,xi-x0,n>a,yi-y0,n≤b e-yi-yb,nDn∙C ,xi-x0,n≤a,yi-y0,n>b e-xi-xa,n2+yi-yb,n2Dn∙C ,others (7.6.14.1.2-6) where, Dn=xA,n-xa,n2+yB,n-yb,n2 , xA,n, yB,n is the coordinate of the other corner of the antenna array on the diagonal with reference corner x0,n, y0,n, and xa,n, yb,n is the coordinate of the other corner of the visibility region on the diagonal with reference corner x0,n, y0,n. The xi, yi denotes the coordinate of the antenna element i. C is the roll-off factor between the visible and invisible regions and defined in Table 7.6.14.1.2-3. Table 7.6.14.1.2-3: Parameters for C UMa/UMi/RMa/SMa/InF/InH. C; 13. Step IV: Apply the antenna element-wise power attenuation factor in coefficient generation The antenna element-wise power variation is considered in the Step 11 for coefficient generation in Section 7.5 with following: - For the NLOS cluster, the channel coefficients Hu,s,nNLOSt are calculated according to the equation 7.5-28 in Section 7.5 by applying the αu,n or αs,n. - In the LOS case, the LOS channel coefficient Hu,s,1LOSt is calculated according to the equation 7.5-29 in Section 7.5 by applying the αu,1 or αs,1 for LOS path. < Unchanged parts are omitted >. Furthermore, given that DL is assume in 901 (refer to section 7.5), seems gNB-side SNS only needs to consider Tx antenna element s, so we prefer to change the character from i to s (haven’t implemented yet for tidiness). |
| Moderator (ZTE) | @Huawei: The v26-v2 is uploaded to incorporate the editorial updates. For your second comment, since we are now working on the element wise calculation and keep the differentiation between Tx and Rx is helpful to avoid mis-understanding. It’s also aligned with the other section for SNS. @Moderator (Intel): As discussed, due to the WS issue, please incorporate the changes additionally in future version. |
| Ericsson | Clauses 4, 6.4, 7.6, 7.6.13, 7.8.3: The word "near-field" is very unfortunate since what we are really modelling is spherical wavefronts. RAN1 failed to reach a consensus on how to define "near-field" and the use of this terminology will only cause further confusion to readers of the TR. Suggest replacing all occurrences of "near-field" in the draft CR with "spherical wavefront". Clause 7.6.14.1.2 This clause duplicates information provided in existing clause 7.6.4 and 7.6.4.2. It is better to merge the new agreements on antenna element-wise blocking into the existing clause 7.6. Indeed, the agreements relating to blockage modelling specify that Blockage model B (and in some cases A) should be reused or improved. For instance, this agreement from RAN1#118bis specifically targets updates to Table 7.6.4.2-5: RAN1#118bis Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS:The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head Also, clause 7.6.4 already provides guidance on the usage of blockage model B. Any updates to the guidance should go here. Editorial comment “ln”, “arg”, “exp”, “LOS”, “desired”, etc in the equations should not be in italic to align with the style in the rest of the TR. |
| Moderator (ZTE) | @Ericsson: Thanks for your inputs: Regarding your 1st comments: - The terminology of “near-field” is used in the SI, it’s better to keep the consistency. - Regarding the concerns on the whether it will lead to the confusion related to the “reactive” region for each element. Actually, this topic is extensively discussed with couples of agreements e.g., following agreement in RAN1#116-bis as below. The corresponding progress is already captured in the CR. Agreement For the near-field channel model: - The impact of the assumption of wavefront is only considered from the perspective of antenna array. - The near field for each element within the antenna array is not considered in this SI. BTW, based on the offline discussion, companies also prefer to keep it with some arguments including technical views. Regarding your 2nd comment: - Firstly, it’s not duplicated information since the newly added section is capture all progress made for SNS, not only the physical blocker based approach, but also stochastic based approach and UE side SNS. To ensure the readability of the TR, it’s better to keep all SNS in a dedicated section. - Second, for the cited agreement related to the table for blockage. The original agreement is just to say that we will introduce additional blocker for SNS purpose and the “Table 7.6.4.2-5 in TR 38.901” is just used to identify the table without ambiguity. And whether the newly introduced blocker can be used for typical “blockage” simulation, it’s not covered the corresponding agreement. So, with this clarification, hope you can live with the current version. |
| vivo | Regarding the wording change from "near-field" to spherical wavefront, we think it is not necessary. In our understanding, in a near-field, what we can say is that the wavefront is no longer plane wavefront, but could be something else, such as spherical wavefront, cylindrical wavefront. In addition, our agreements are based on the assumptions where we focus on cluster other than ray. In a cluster, there are many rays, and each ray may form a spherical wavefront. However, once multiple rays are summed together, the resultant wavefront is no longer spherical. This is the phenomenon we can confirm either by simulation or by experiment measurement. |
| Ericsson | @Moderator (ZTE): On “near-field”: Can you at least add a note or clarification every time “near field” is mentioned? We remain concerned that readers of the TR will not understand the distinction between what is actually modeled in the TR and the very wide range of possible interpretations of what “near field” could be. In the current draft CR this is not clarified until page 32 in clause 7.6.13. In contrast, in clause 6.1 that references channel modeling work outside 3GPP the term spherical wave modeling appears twice. @vivo: We have concluded that in RAN1#118bis that near field is indeed modeled only with spherical wavefronts: Conclusion RAN1 confirms that the modelling of near-field propagation characteristics (i.e., characteristics of spherical wavefront) is taken as an additional modelling component, and the relevant progress (e.g., the agreement(s) with/without “if necessary”) in AI 9.8.2 will be captured in TR 38.901. @Moderator (ZTE): The agreement from RAN1#118bis clearly states that the new blocker types should be introduced in Table 7.6.4.2-5 in TR 38.901. We expect the draft CR to implement this agreement. We are not opposed to keep both approaches to SNS in the same (new) clause, but the sub-clause on SNS blockage modelling should reference existing procedures, figure, and equations in clause 7.6 to the greatest extent possible. This will respect several other agreements that explicitly mentions adopting Blockage model B with extensions. |
| Moderator (Intel) | Updated v27 that reflects the file error issue in v26. The update also include v26-v2 changes. Comments from vivo and Ericsson above will be addressed by Moderator (ZTE). |
| Moderator (Intel) | Update v28 that moved the UE antenna polarization contents to the polarization section. Updates to minor formatting issues. |
| Moderator (ZTE) | @Ericsson, For the NF part, if my understanding is correct, your preference is to have an earlier “statement/explanation” of meaning of Near field? Otherwise, the current description as the first sentence under the section 7.6.13 for near field should be fine. If so, the following update (in red) can be considered for the sentence under section 6.4 Modelling objectives: - Support near-field channel propagation (i.e., characteristics of spherical wavefront) and spatial non-stationarity For the SNS part, - Firstly, the so-called “should reference existing procedures, figure, and equations in clause 7.6 to the greatest extent possible”, it’s already reflected in the current text, e.g., “The number of blockers (K), their vertical and horizontal extensions (hkand wk), locations (xk,yk,zk) are determined according to ‘Step a’ in the Section 7.6.4.2:” - For the cited agreement, as I said, the context is to allocate the table. From the perspective of applicability, according to the agreement, the “new blocker” is more for SNS instead of full blockage. If we want to add it to the section for blockage, it somehow extends the applicability of the table. |
| Moderator (Intel) | Update v29 incorporates all agreements from RAN1#121 into account. |
| Moderator (Intel) | Update v29 had a file save error and did not include all changes from RAN1 #121. Update v30 now contains all changes. |
| Sharp | - Table 7.4.1-1 Path loss models For Sma path loss model: 0.5 ≤fc≤ 37 GHz -- remove this range from Column3 of LOS/NLOS to be consistent with all other scenarios. No scenario in Table 7.4.1-1 captures the frequency range. The frequency range is captured in Note 2 for all scenarios. And Note 2 in Table 7.4.1-1 already captures the upper limit of Fh for SMa in LOS/NLOS. - Table 7.4.2-1 LOS probability Table Might be good to add the note on what BS height was assumed for SMa LOS probability derviation as well. I think it was 35 m, but E/// can confirm. NOTE: The LOS probability is derived with assuming antenna heights of 3m for indoor, 10m for UMi, 25m for UMa, and [35 m] for SMa. - Low-loss, high-loss, and [DW1] ;ow-loss A -- not sure if there is typo in low-loss A. I can see a ";" only and the text looks like ;ow-loss A. - 7.5-2, 7.5-4: either we should add the values in the Tables for all clusters or remove entries rather than leaving blank. Ex: 13 in 7.5-2 is blank, either remove it or add a value for it. Is there a plan to add values instead of leaving it blank? - Table 7.6-1 Part 1: UMi ASD LOS: -0.05 log10(1+ fc) + 1.21 we should still say “see note 8” like we did for all other parameters. The value for UMi ASD LOS remains unchanged even after combining data from Rel-19 and Rel-14. Without “see note 8” it is not clear why this value wasn’t updated. We need to somehow capture that this value even though it is unchanged used both Rel-19 and Rel-14 data and the easiest way to make sure this is reflected is to just say “see note 8”. - Section 7.6 “Simulation of diverse cluster environments” -- this doesn’t seem to be the appropriate wording for this section. With the changes we have introduced we are trying to give the users the option to break away from 7.5 and have variability in the number of clusters compared to the fixed number in 7.5. Thus, we should choose a wording to reflect the number of clusters is not fixed and variable. Hence, the section should be renamed to something like “Variable Number of Clusters" (preferred) or "Variability in Number of Clusters". Section 7.6.15 heading to be modified to “Variable Number of Clusters" to capture the intention of the optional modelling component. - Section 7.6.15: Suggest using the following description: The number of clusters observed in real-world deployments can vary due to one or more factors, including but not limited to the propagation environment, carrier frequency, bandwidth, and spatial resolution. To capture this variability in simulations, the number of clusters for each link can be chosen between a closed range of [D1, D2]. The values of D1 and D2 are provided in Table 7.6.15-1. The selection of the number of clusters for each link determined by the user. The selected number of clusters for each link replaces the number of clusters N in Table 7.5.6-Part 1 and 7.5.6-Part 2. A similar framework for modeling the variability in number of clusters is also adopted in the METIS channel model [26]." <Table 7.6.15-1 - unchanged text> The above description is a probably a more appropriate description of this optional modeling section and reflects the insights from different measurement data from different sources as well. Measurement data has shown that the variation can be caused by a single factor or multiple factors acting together. Also, the listed factors (environment, frequency, bandwidth, spatial resolution) are examples, but there could be other influencing factors as well. Hence, the above text is maybe more generic in nature and accurately captures the intention of this new optional modeling section. The above description has also been modified for easy readability and clarity. - METIS Reference [6] ICT-317669-METIS/D1.4: "METIS channel model, METIS 2020, Feb, 2015". No need for the new reference [26] as ref [6] in current TR already cites METIS. We can remove either [6] or [26]. Just keep either one of the references. |
| ZTE | Thanks for the update. We have following comments: On calibration assumption Table 7.8-9 for UE side SNS: - According to the agreement in RAN1#120bis in R1-2503114, the UE antenna Config B should be single polarization, so we suggest the following change Config B for 15 GHz: 8 antenna port with dual single polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. - According to the updates on the rotation angles in RAN1#121, for dual hand grip case, the rotation should be updated to: - For dual-hand blockage, ΩUT,α = 0 – 360 degrees, ΩUT,β = 0 degrees, ΩUT,γ = 45 degrees, - In note 3, there is a typo to be fixed from ‘determiend ’ to ‘determined’ On calibration assumption Table 7.8-8 for BS side SNS: According to the agreement in RAN1#120bis in R1-2503114, the BS antenna config2 should be Config 2 for 15 GHz: (M, N, P, Mg, Ng; MP, NP) = (64, 32 16, 2, 1, 1; 8, 16), dH = dV = 0.5λ On SMa: - Delay scaling parameter for NLOS should be 1.5 instead of 2.4 - There are 2 values for the XPR standard deviation of O2I case, among which 5 should be removed - For O2I penetration loss, there is a typo on “Low-loss, high-loss, and ;ow-loss A models are applicable to SMa.” On stochastic based SNS: There is missing parameter R in the following sentence: “The parameters A, B, R and δ are defined in Table 7.6.14.1.2-2.” On near field: In equation (7.6-49), the phase component needs to be changed to align with that in equation (7.6-48), i.e. expj2πd2,n-d2,n∙ rrx,n,m-drx,uλ0expj2πd1,n-d1,n∙ rtx,n,m-dtx,sλ0 needs to be changed to: expj2πd2,n,m-d2,n,m∙ rrx,n,m-drx,uλ0expj2πd1,n,m-d1,n,m∙ rtx,n,m-dtx,sλ0 It may need to be clarified whether to consider ‘The ratio of UT with impact of SNS is 90% and for each UT’, or follows the calibration assumption table to consider only these 3 cases of blockage. From our understanding, both approaches could work for calibration but companies’ assumptions should be aligned. Considering that agreed calibration table does not mention the case without blockage, maybe we can assume the total ratio of 3 cases as 100%, then there is no need to update the calibration table on this part. Furthermore, it appears that companies may have different understanding on the removal of -25dB power offset, since this may have impact on the calibration results, we want to clarify the removal of -25dB power offset is based on equation 7.5-8 instead of equation 7.5-6 for LOS UE. |
| Moderator (Intel) | Updated draft CR based on comments received. @Sharp: all suggested changes were implemented. Removed ref 26 and used 6 instead. @ZTE: all suggested changes were implemented. Added clarification on blockage probability in Table 7.8-9 note 3. Clarified directly in Section 7.5 regarding -25dB cutoff using eq (7.5-6). Moderator also received questions on indoor calibration on whether indoor is sectorized or not. Moderator assumes indoor is NOT sectorized, and this has been clarified in Table 7.8-7. Sectorization; For UMi-street canyon: sectorized as described in Table 7.8-1. For indoor-office: non-sectorized, 1 BS per BS location.. Moderator also received request to correct the coversheet for the data sources. More specifically, the observations that was agreed had redundant and incorrect text. Moderator asks companies to check, changes are simply removing unnecessary text and clarifying the excel sheet reference. https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_121/Inbox/drafts/9.8(FS_NR_7_24GHz_CHmod)/CR/draft%20R1-2504960%207-24GHz%20data%20source-v1.docx |
| Nokia | We have the following comments on the CR version 31: - Table 6.3-1: Channel measurement capabilities - Following the Excel with Data sources, Nokia have provided Material penetration losses and pathloss measurements for the SMa scenario at 28 GHz, line 40. Therefore, we prefer to add Nokia in the following cells of the table: - Suburban macro, 20-30 GHz - O2I, 20-30 GHz - In 7.3.0 Antenna array structure, UE antenna model: - Antennas are assumed to be oriented along the direction determined by the vector connecting the center of the rectangle to the antenna location. - Alternatively, we can say: Each antenna is - The captions of Figures 7.3-7 and 7.3-8 are not updated and copied directly from Figure 7.3-2. The captions should be updated: - Figures 7.3-7: Handheld UT antenna polarization directions for one antenna filed pattern (top down view) - Figures 7.3-8: Handheld UT polarization direction for two antenna filed pattern (side view) - Table 7.4.1-1: Pathloss models - Would it be more correct to add a subscript for the Optional NLOS UMa and UMi? - For example, Optional PLUMa-NLOS=32.4+20log10fc+30log10d3D - Just a question: what are expected to do with Scaling factors in Tables 7.5-2/4? - Are we still waiting for the results or will we need a correction CR to introduce those? - 7.6.15 Number of cluster variability model - The selection of the number of clusters for each link is determined by the user of the model. - 7.7.5.1 CDL extension: Scaling of angles - The first change from Ray angles to Cluster angles is wrong because as it is defined below “ϕn,m,scaled is the resulting scaled ray angle that corresponds to mth subpath angle of nth cluster,” - The following correction are needed: - The translated and scaled ray cluster angles can be obtained according to the following equation: - Alternatively, for CDL-D and CDL-E, if maintaining the LOS angle is desired, the translated and scaled cluster ray angles can be obtained … - Additionally, there is a strange term: cASA, ASD,ZSA,ZSD is the cluster-wise rsm angular spread - What rsm stands for? Should it be RMS instead? - Moreover, shall rms be capitalized, i.e. ther are other place in where rsm is used, e.g., - AS_{model} is the rms RMS cluster angular spread of the tabulated - Table 7.8-2A: Simulation assumptions for full calibration - Carrier frequency 15 GHz is only for the optional scenarios. Hence it should be marked as Optional - Carrier Frequency: 7 GHz, (optional) 15 GHz |
| Moderator (Intel) | @Nokia: (1) Updated as suggested (2) Updated as “each antenna is”. The word “centre” is spelling in UK, which is the official language for 3GPP specification. To be precise the 3GPP specification is based on Oxford English dictionary, which is based on British English. So, we will keep “centre” as is. (3) Updated as suggested. (4) Added subscript for optional PL. (5) The table has been put into two row table instead of single row table to make the total width smaller. Unused entries should be now removed. No need to fill in the missing entries (at least in this release). (6) Updated as suggested (7) Updated as suggestd. ‘rsm’ was a typo for “RMS”. (8) Updated as suggested. Updates are now available as v32. |
| vivo | According to the agreement, d1,n,m and d2,n,m are cluster-based determination. Therefore, they should not capture the index m. We should keep original equation as expj2πd2,n-d2,n∙ rrx,n,m-drx,uλ0expj2πd1,n-d1,n∙ rtx,n,m-dtx,sλ0 |
| ZTE | For the -25dB removal, in our understanding the maximum cluster power for LOS case and NLOS case can be determined based on different formulas, i.e. - For NLOS case, there is no doubt that the maximum cluster power is determined based on equation 7.5-6 - For LOS case, we think the maximum cluster power should be based on equation 7.5-8, while some other companies may assume the maximum cluster power is determined based on equation 7.5-6, then the different assumtion may have impact on the calibration results The following change is only focused on LOS case, I think either we can consider something like "based on equation (7.5-8) in the case of LOS condition, and based on equation (7.5-6) when it's not in the case of LOS condition", or just keep the original wording and clarify this in your summary, it would be OK if companies have the same understanding on this. “Remove clusters with less than -25 dB power compared to the maximum cluster power based on equation (7.5-8).” |
| Moderator (Intel) | @vivo: From the description d1,n,m and d2,n,m are both NOT a function of ‘m’ therefore, subscript ‘m’ may be omitted as suggested by vivo. d1,n,m=sTRP,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sTRP,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N d2,n,m=sUE,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sUE,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N, Rapporteur has updated equation (7.6-47) (7.6-48), and (7.6-49) and removed the subscript ‘m’ from ‘d1,n,m’ and ‘d2,n,m’ @ZTE: Rapporteur thinks it would be good to clarify any ambiguous text in the TR to avoid problems during 6G studies. Clarified the application of -25 dB threshold in Section 7.5 based on ZTE suggestion. |
| Moderator (Intel) | @vivo: it seems Moderator was overlooked the sub-cluster index ‘i’ that needs to be used to determine different value for different rays. d1,n and d2,n has been reverted back to d1,n,m and d2,n,m, It was further clarified that “The ray index, m, should be determined based on the sub-cluster information in Table 7.5-5.” Update in v34. |
| Ericsson | @Moderator (ZTE): Many thanks for introducing the clarification on near field propagation = spherical wave modeling in clause 6.4. Would it also be possible to add the same clarification in clause 7.6.0? @Moderator (ZTE): Likewise, the term “spatial non-stationarity” is by now familiar to all participants in this SI but may not be well-known to everyone. The clarification that this is “antenna element-wise power variation” that is given in clauses 7.6.14.1.1 and 7.6.14.2 could also be mentioned in clauses 6.4 and 7.6.0. @Sharp: The LOS probability for SMa was derived analytically considering any arbitrary antenna height. So, the proposed note does not work for SMa, unless this is captured with the height range 0-infinity. Note that the InF LOS probability is also valid for any height. @Moderator (Intel): The text description of the SMa scenario is added in clause 6.2. However, this clause was first drafted at the beginning of the Rel-14 SI and lists potential scenarios for work. As such it doesn’t reflect actual scenarios defined in the TR, this is instead done in clause 7.2. Therefore, our suggestion would be to move the text description to the appropriate place in clause 7.2 and instead use a much shorter reference to “Suburban macrocell scenarios” in clause 6.2. We support the proposals from Sharp on reformulating clause 7.6.0 and 7.6.15, including changing the clause 7.6.15 title. Clause 7.6.14.1.1: There is some red text that should presumably be in black. Clause 7.6.14.1.3: There are some mathematical symbols in red color that should be black. Table 7.5.6: Discrepancy between chair notes and CR: There is an additional minus sign in the CR, which says -0.08 instead of 0.08. |
| Sharp | Section 4: Near field channel modeling and spatial non-stationarity is supported. – shouldn’t this be changed to Near field channel modeling (i.e., characteristics of spherical wavefront) and spatial non-stationarity is supported like ZTE suggested earlier in the CR v34? This should also resolve concerns from companies and will be consistent with the description we have in Section 6.4. Table 7.2-3: Carrier Frequency: 7GHz instead of 7Ghz Table 7.2-4: BS antenna height hbs seems to be an image can you please change it to an equation to denote hBS Similarly, hUT is also an image, please change hUT to an equation. Table 7.5-6 Part-1: UMa DS NLOS Std – we should point to “see note 9”. Rephrase Note 9: NOTE 9: Value(s) were unchanged even after the considering the measurements submitted as part of Release 19 Study on channel modelling enhancements for 7-24 GHz for NR [25]. UMi ASA LOS STD value should be 0.08 * log10(1+ f) + 0.29. The minus sign before 0.08 should be removed. Table 7.5-6 Part-4: XRP remove the value of 5 from O2I and just keep 3. Section 7.6.0 Change the description from the simulation of diverse cluster environments to simulation of number of cluster variability Section 7.6.15 replaces the number of clusters N – change N to italics to be consistent with Tables 7.5-6’s. The replaced text will be “The selected number of clusters for each link replaces the number of clusters N… Change caption of 7.6.15-1 to Range of number of clusters. Remove the word “for selection” from the Table heading. Table 7.6.15-1: Column 1, row 1 change - Usage scenario to “Scenarios” |
| vivo | We agree that d1 and d2 depend on i. Therefore, the formula should be d1,n,i=sTRP,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sTRP,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N d2,n,i=sUE,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sUE,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N, The index m seems incorrect. |
| vivo2 | Since the first rotation is from the LCS of each element to LCS of UT, in 7.3.2, the paragraph should be modified as Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation and polarization direction of the each of UT antennae to get Fu,θ''θ',ϕ', Fu, ϕ''θ',ϕ' and based on the orientation of the UT in the global coordinate system to get Fu,θθ,ϕ and Fu,ϕθ,ϕ using the method specified in Clause 7.3.2 equation (7.3-3) the following equation Fθ''θ',φ'Fφ''θ',φ'=+cosψu-sinψu+sinψu+cosψuFθ″″θ″,φ″Fφ″″θ″,φ″ and Clause 7.1.3 equation (7.1-11). |
| Moderator (ZTE) | @Ericsson: OK with the clarification on near field and SNS in section 6.4 and 7.6.0. @Daewon: Please help to update as follows, thanks. ## 6.4 Modelling objectives - Support near-field channel propagation (i.e., characteristics of spherical wavefront) and spatial non-stationarity (i.e. antenna element-wise power variation) ### 7.6.0 Introduction of additional modelling components simulations of near-field channel effects (i.e., characteristics of spherical wavefront), simulations of spatial non-stationarity (i.e. antenna element-wise power variation) @vivo: In channel coefficient formula with phase term, there is no sub-cluster index i, that’s the reason that ray index m is used to generate per sub-cluster d1 and d2. With the clarification from Daewon, it should be clear that the rays in each sub-cluster share the same d1 value, i.e. i = 1 corresponds to ray index m = {1, 2, 3, 4, 5, 6, 7, 8, 19, 20}, it would be more convenient to use m instead of i in d1 generation. Hu,s,n,mNLOSt=PnMFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD expj2πd1,n,m-d1,n,m∙ rtx,n,m-dtx,sλ0expj2πrrx,n,mT∙drx,uλ0expj2πrrx,n,mT∙vλ0t (7.6-47) d1,n,m=sTRP,n*d3D+Δτ*c+τn,i*c, for the two strongest clusters, say n = 1 and 2sTRP,n*d3D+Δτ*c+τn*c,for the N – 2 weakest clusters, say n = 3, 4,…, N, The τn,i refers to the delay of i-th sub-cluster mapping to the rays defined in Table 7.5-5. The ray index, m, should be determined based on the sub-cluster information in Table 7.5-5. |
| Huawei, HiSilicon | Thanks Moderators for the great effort! We have the following comments for V34: Comment #1 Seems we’re missed in Table 6.3-1. Suggest to add Huawei to the following cells: - Urban macro, 6-20 GHz - Urban micro, 6-20 GHz - Suburban macro, 6-20 GHz Comment #2 Some editorial suggestion: - “non-stationarity is supported” in section 4 should be changed into “non-stationarity are supported”; - “filed” in section 7.3.2 should be changed into “field”; - “0,48” in Table 7.6.14.1.2-2 should be changed into “0.48”; - The frame of Table 7.6.14.2-2 may need to be adjusted. |
| Moderator (Intel) | Updated CR based on Moderator (ZTE) inputs. @Ericsson: Please check the changes proposed by Moderator (ZTE). On the SMa scenario description. Section 6.2 now captures a short description (first sentence) of the SMa, and the rest of the description has been moved to Section 7.2. Description in 7.6.0 for cluster variability has been updated. Adopted the text from Sharp for 7.6.15. Corrected the text color in Section 7.6.14.1.1 and 7.6.14.1.3. Corrected the sign for UMi LOS ASD. @Sharp: Updated Section 4 to match the description in other sections. Updated GHz typo in Table 7.2-3 Updated picture for hBS and hUT to symbolic equations in Table 7.2-4 Updated UMa DS NLOS Std to add the note 9 Update note 9 based on Sharp’s comment in Table 7.5-6-Part 1. Corrected the sign for UMi LOS ASD. Deleted 5 for SMa XPR O2I parameters. Updated cluster variability text in 7.6.0 Changed N to italic N in Sectio 7.6.15 Changed Table 7.6.15-1 to “range of number of clusters” and updated text of first row to “Scenarios” @vivo: Moderator thinks the subscript ‘m’ is still valid. There is a mapping between ‘i’ and ‘m’ that is based on Table 7.5-5 noted by Moderator (ZTE) that should fix the problem. The equation provided and equation (7.3-3) are identical. So the following updated text should be clear enough “Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation and polarization direction of the each of UT antennae to get Fu,θ''θ',ϕ', Fu, ϕ''θ',ϕ' using equation (7.3-3), and then rotated based on the orientation of the UT in the global coordinate system to get Fu,θθ,ϕ and Fu,ϕθ,ϕ using Clause 7.1.3 equation (7.1-11).” @Huawei: Updated Table 6.3-1 as commented. Updated Section 4 as commented. Fix the comma typo in Table 7.6.4.1.2-2. Table borders of Table 7.6.14.2-2 was updated. Some misc. error corrections were additionally made: Typo in Section A.3 and A.4: Pm -> Pn Typo in Section A.3: cluter -> cluster |
| Moderator (Intel) | Moderator has been asked offline to further clarify the UT orientation for calibration. Currently for NF calibration UT orientation of beta = 45 is specified. However, from moderator understanding this is only for Config B that uses the new handheld UT antenna model and not intended for Config A. For Config A, we should use the existing beta = 90. Table 7.8-7 UT antenna configurations; Config A: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config B: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3. UT antenna pattern; Isotropic. UT Polarized antenna modelling; For UT Config A: Model-2 in Clause 7.3.2 For UT Config B: following the clause 7.3. UT orientation; Config A: ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree Config B: ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree. For Full calibration case, it was clarified that the UT orientation only applies to Config B and C. Config D used to antenna field pattern calibration should use alpha = 0, beta = 0, gamma = 0. Table 7.8-2A UT array orientation; Config B, C: ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 45 degree, ΩUT, = 0 degree Config D: ΩUT, = 0 degree, ΩUT,= 0 degree, ΩUT, = 0 degree. UT antenna configurations; Config B for 7 GHz: 4 antenna port with single polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 (optional) Config C for 15 GHz: 16 antenna port with dual polarization based on handheld device antenna model using candidate antenna locations in (1,2,3,4,5,6,7,8) as described in Clause 7.3 (only for metric 5) Config D: 8 antenna port with single polarization based on handheld device antenna model using candidate antenna locations in (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| QC | Thanks a lot for the draft. A few minor remarks from our side: In 7.6.13, where we say , “otherwise sUE,n =1-sTRP,n” can we also add the note that we had in the agreement to say that this assumes an environment where most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions and most indirect paths have only a single single non-specular interaction. In 7.6.14.1.3: can we change the text for Step 1 to “Determine whether the nth cluster of UT is partially visible” In 7.6.14.1.3: Instead of saying “For the nth cluster, the cluster is assumed to be impacted by the invisibility” can we change to “For the nth cluster, the cluster is assumed to be partially visible”? This occurs at two locations. In 7.6.14.1.3 Step III: Instead of saying “For the nth cluster which is assumed not to be impacted by the invisibility” can we say “For the nth cluster which is assumed to be fully visible” In 7.6.14.1.3 Step III:Please use lower case a and b where we say "xA,n, yB,n is the coordinate of the other corner” In 7.6.16: The note on applicability to Model 2 is missing. Please add this. |
| Moderator (ZTE) | @Qualcomm: Please find the responses to your comments on NF and SNS below: In 7.6.13, where we say , “otherwise sUE,n =1-sTRP,n” can we also add the note that we had in the agreement to say that this assumes an environment where most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions and most indirect paths have only a single single non-specular interaction. Moderator (ZTE): It’s not necessary to additionally add the note to TR 38.901 to clarify the formula, if companies want to understand the logic behind the formulas, it’s easy to trace back to the agreements. In 7.6.14.1.3: can we change the text for Step 1 to “Determine whether the nth cluster of UT is partially visible” In 7.6.14.1.3: Instead of saying “For the nth cluster, the cluster is assumed to be impacted by the invisibility” can we change to “For the nth cluster, the cluster is assumed to be partially visible”? This occurs at two locations. In 7.6.14.1.3 Step III: Instead of saying “For the nth cluster which is assumed not to be impacted by the invisibility” can we say “For the nth cluster which is assumed to be fully visible” Moderator (ZTE): These editorial changes are not critical, prefer to keep current wording at this stage. In 7.6.14.1.3 Step III:Please use lower case a and b where we say "xA,n, yB,n is the coordinate of the other corner” Moderator (ZTE): xa,n, yb,n with lower case a and b refer to the coordinate of the other corner of VR while xA,n, yB,n with upper case A and B refers to the coordinate of the other corner of antenna array, they are different definitions. |
| QC | Thanks for the clarifications. We would like to insist that the note be added to 7.6.13 as otherwise this assumption may be applied to scenarios where the condition sUE,n =1-sTRP,n does not clearly hold. It is a limitation of the model and it should be captured clearly. Regarding “impact by the invisibility” --- while its clear what is meant, it doesn’t read well and not how one would usually phrase this in English. It is suggested to use the simpler wording of “partial visibility” or “partially visible” where appropriate. Thanks for the clarification on xA,n, yB,n --- we misread this. |
| Moderator (ZTE) | @Qualcomm: Please find further responses below We would like to insist that the note be added to 7.6.13 as otherwise this assumption may be applied to scenarios where the condition sUE,n =1-sTRP,n does not clearly hold. It is a limitation of the model and it should be captured clearly. Moderator (ZTE): Although Moderator still feels unnecessary, to address QC’s concern, the following TP can be considered by Daewon: where sUE,n=1, if the nth cluster is one of the k1 strongest cluster, otherwise sUE,n =1-sTRP,n. It’s assumed that only single non-specular interaction along with specular reflection(s) is considered in the propagation channel and at least most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions. Regarding “impact by the invisibility” --- while its clear what is meant, it doesn’t read well and not how one would usually phrase this in English. It is suggested to use the simpler wording of “partial visibility” or “partially visible” where appropriate. Moderator (ZTE): Actually “impacted by the invisibility” refers to 2 cases, i.e. partially visible and totally invisible, the intention of original wording is just to avoid using ‘or’ to describe these 2 case, so it’s not accurate to simply change it to ‘partially visible’. |
| Moderator (Intel) | Fixed typo in Table 7.6.14.1.2-1 (pointed out by Huawei) for UMi and InH. Updated text suggested by Moderator (ZTE). @Qualcomm, ZTE: As for “invisibility”, Moderator (Intel)’s suggestion would be to use “visibility obstruction”. There are five places this is used. Step I: Determine the invisibility visibility obstruction condition for the nth cluster of UT For the nth cluster, the cluster is assumed to be impacted by the invisibility visibility obstruction, if the … For the nth cluster which is assumed to be impacted by the invisibility visibility obstruction, the visibility region is generated … For the nth cluster which is assumed not to be impacted by the invisibility visibility obstruction, the … For the nth cluster which is assumed to be impacted by the invisibility visibility obstruction, the … |
| QC | Thank you to both moderators for the changes. Can we use the word ‘restriction’ instead of ‘obstruction’ as the stochastic model also takes partial scattering/reflections into account and not just partial blockage? Can we also try to address our other comment on 7.6.16 (note on applicability to Model 2 is missing) |
| Moderator (Intel) | @Qualcomm: From the Oxford dictionary, restriction: (noun) a limiting condition or measure, especially a legal one. Obstruction: (noun) the action of obstructing (block (an opening, path, road, etc.); be or get in the way of.) or the state of being obstructed From the definitions, obstruction may be a better language for this compared to restriction. Especially, it is used as visibility XXXX condition. According to the language if we use restriction, it would be similar to stating “visibility limiting condition condition …”. It is not clear from the definitions there is any implications of partial or full blockage. It is simply a state of being obstructed. Updated applicability to Model 2 for 7.6.16. |
| QC | We just wanted to make it clear that the model is not limited to partial or full blockage but also consider partial reflections (due to a glass pane being of a finite size, say). Partial reflections are not an “obstruction” as is clearly pointed out by the Oxford dictionary definitions that you copied above. Visibility limitation or visibility limiting conditions also seem okay to use. |
| Moderator (Intel) | @Qualcomm: While I don’t know if this will make any meangingful difference, if Moderator (ZTE) is ok, let’s go with “visibility limitation”. Update in v39. |
| End of Discussion |  |

# Appendix A: RAN1 Agreements

## Common Agenda for 9.8.1 and 9.8.2

### RAN1 #116-bis (April-2023)

Conclusion

- RAN1 to compile measurement/simulation descriptions from companies into a Tdoc to be added as reference to TR38.901.

  - Rapporteur to update the Tdoc in each meeting based on inputs from companies.

- Rapporteurs to provide a template for the measurement/simulation descriptions capture to RAN1 #117 for initial review and endorsement.

### RAN1 #117 (May-2024)

Agreement

To check and review the following results and measurement data provided in RAN1 #117 and RAN1#116bis for further discussion in next RAN1 meeting. R1-2405646 contains the list of data sources for the results and measurements provided in RAN1 #117.

- measurements for penetration loss for various materials, including drywall/wood, clear glass, IRR glass, and concrete

- measurements for pathloss for following scenarios: InH_office LOS, InH-Office NLOS, InF LOS, InF NLOS, UMi LOS, UMi NLOS, UMa LOS, UMa NLOS, [Outdoor courtyard], RMa LOS, RMA NLOS, SMa NLOS

- measurements for polarization for UMa deployment scenario

- measurements for DS for following scenarios: InH-Office LOS, InH-Office NLOS, UMi LOS, UMI NLOS, UMa LOS, UMa NLOS, InF LOS, Inf NLOS.

- Measurements for angular distributions, such as ZOD, ZOA, AOD, AOA for following scenarios: InH, UMi, UMa

- Measurements for number of clusters for following scenarios: InH, UMi

- Simulations for LOS probability for SMa deployment scenario

- Measurement results regarding near-field model for following deployment scenarios: InH-Office LoS, UMa

- Measurement results regarding spatial non-stationarity for following deployment scenarios: UMa, [UE side]

- Simulation results regarding spatial non-stationarity for UMa deployment scenario

### RAN1 #118 (August-2024)

Agreement

To check and review the following results and measurement data provided in RAN1 #118 for further discussion in next RAN1 meeting. R1-2407251 contains the list of data sources for the results and measurements provided in RAN1 #118, RAN1 #117, and RAN1 #116-bis.

### RAN1 #118-bis (October-2024)

Conclusion

Investigation and potential update of the propagation characteristics of the channel model for 6-7 GHz are part of the study for this item.

### RAN1 #119 (November-2024)

Conclusion

For any new channel modeling parameter updates, replace the existing parameters values (whenever applicable). For parameters that were replaced TR should add information that the parameter value/model equation/procedure was changed in Rel-19.

- Note: If deemed important, it should be still possible to discuss how specific channel model parameters need to be updated.

### RAN1 #120 (February-2025)

Conclusion

To check and review the following results and measurement data provided in RAN1 #120 for further discussion in next RAN1 meeting. R1-2501426 contains the list of data sources for the results and measurements provided since RAN1 #116-bis until RAN1 #120.

Conclusion

For new scenarios, and if changes are made to existing scenarios, interested companies in RAN1 to perform large scale and full calibration based on the following simulation assumptions. Use the following updates to TR38.901 as starting point for further discussion for calibration.

### 7.8.1 Large scale calibration For large scale calibration, fast fading is not modeled. The calibration parameters can be found in Table 7.8-1. The calibration results based on TR 38.900 V14.0.0 can be found in R1-165974. Table 7.8-1: Simulation assumptions for large scale calibration Parameter; Values. Scenarios; UMa, UMi-Street Canyon, Indoor-office (open office). Sectorization; 3 sectors per cell site: 30, 150 and 270 degrees. BS antenna configurations; Mg = Ng = 1; (M,N,P) = (10, 1, 1), dV = 0.5λ. BS port mapping; The 10 elements are mapped to a single CRS port. BS antenna electrical downtilting; 102 degrees for UMa and UMi Street Canyon 110 degrees for indoor. Antenna virtualization; DFT precoding according to TR 36.897 with application of panning and tilting angles. BS Tx power; 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz 35 dBm at 30GHz and 70 GHz for UMa and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies. Bandwidth; 20MHz for 6GHz, and 100MHz for 30GHz and 70 GHz. UT antenna configurations; 1 element (vertically polarized), Isotropic antenna gain pattern. Handover margin (for calibration); 0dB. UT distribution; Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m. UT attachment; Based on pathloss considering LOS angle. UT noise figure; 9 dB. Fast fading channel; Fast fading channel is not modelled. O2I penetration loss; 50% low loss and 50% high loss. Carrier Frequency; 6 GHz, 30 GHz, 70GHz. Wrapping method for UMa and UMi; geographical distance based wrapping (mandatory) radio distance (optional). Metrics; 1) Coupling loss – serving cell (based on LOS pathloss). 2) Geometry (based on LOS pathloss) with and without white noise. Additional calibration parameters can be found in Table 7.8-1A. It is assumed that parameters from Table 7.8-1 is used if unspecified by the additional calibration parameters in Table 7.8-1A. Table 7.8-1A: Simulation assumptions for large scale calibration Parameter; Values. Scenarios; UMa, UMi-Street Canyon, SMa. BS antenna electrical downtilting; FFS degrees for SMa. BS antenna configurations; Config 0 for 6GHz: M = 10,N = 1,P=1; Mg = Ng = 1; Mp = 10, Np = 1; dV = 0.5λ Config 1 for 7GHz : M = 8, N = 16,P = 2; Mg = 1, Ng = 1; Mp = 8, Np = 16; dH = dV = 0.5λ. BS Tx power; FFS dBm for SMa. Bandwidth; 20MHz for 6GHz only for UMa and UMi-Street Canyon, 200 MHz for 7 GHz. UT distribution; For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type.. O2I penetration loss; For SMa, FFS. Carrier Frequency; 6 GHz, 7 GHz. ### 7.8.2 Full calibration The calibration parameters for full calibration including the fast fading modelling can be found in Table 7.8-2. Unspecified parameters in Table 7.8-2 are the same as those in Table 7.8-1. When P=2, X-pol (+/-45 degree) is used for BS antenna configuration 1 and X-pol (0/+90 degree) is used for UT antenna configuration. The calibration results based on TR 38.900 V14.0.0 can be found in R1-165975. Table 7.8-2: Simulation assumptions for full calibration Parameter; Values. Scenarios; UMa, UMi-street Canyon, Indoor-office (open office). Carrier Frequency; 6 GHz, 30 GHz, 60GHz, 70GHz. Bandwidth; 20MHz for 6GHz, and 100MHz for 30GHz, 60 GHz and 70 GHz. BS Tx power; 44 dBm for UMi-Street Canyon, 49 for UMa at 6GHz 35 dBm at 30GHz, 60 GHz and 70 GHz for UMa and UMi-Street canyon 24 dBm for Indoor for all carrier frequencies. BS antenna configurations; Config 1: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ … calibration metrics 1), 2), 3) are calibrated Config 2: Mg = Ng = 1, M = N = 2, P = 1 … calibration metrics 1), 2), 4) are calibrated. BS port mapping; Config 1: all 16 elements for each polarization on each panel are mapped to a single CRS port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna element is mapped to one CRS port. UT antenna configurations; Mg = Ng = 1, M = N = 1, P = 2. UT distribution; Following TR36.873 for UMa and UMi, (3D dropping) uniform dropping for indoor with minimum distance (2D) of 0 m. UT attachment; Based on RSRP (formula) from CRS port 0. Polarized antenna modelling; Model-2 in TR36.873. UT array orientation; ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree. UT antenna pattern; Isotropic. Metrics; 1) Coupling loss – serving cell. 2) Wideband SIR before receiver without noise. 3) CDF of Delay Spread and Angle Spread (ASD, ZSD, ASA, ZSA) from the serving cell (according to circular angle spread definition of TR 25.996). 4) CDF of largest (1st) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale. CDF of smallest (2nd) PRB singular values (serving cell) at t=0 plotted in 10*log10 scale. CDF of the ratio between the largest PRB singular value and the smallest PRB singular value (serving cell) at t=0 plotted in 10*log10 scale. Note: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB.. Additional full calibration parameters can be found in Table 7.8-2A. It is assumed that parameters from Table 7.8-2 is used if unspecified by the additional full calibration parameters in Table 7.8-2A. Table 7.8-2A: Simulation assumptions for full calibration Parameter; Values. Scenarios; UMa, UMi-Street Canyon, SMa. Carrier Frequency; 6 GHz, 7 GHz. BS antenna electrical downtilting; FFS degrees for SMa. BS antenna configurations; Config 1 for 6GHz and only for UMa and UMi: M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ … calibration metrics 1), 2), 3) are calibrated Config 2 for 6GHz and only for UMa and UMi: Mg = Ng = 1, M = N = 2, P = 1 … calibration metrics 1), 2), 4) are calibrated Config 3) for 7: M = 8, N = 16,P = 2, Mg = 1, Ng = 1, Mp = 8, Np = 16,dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated Config 4) for 7: M = 64, N = 16,P = 2, Mg = 1, Ng = 1, Mp = 8, Np = 16,dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated. BS Tx power; FFS dBm for SMa. Bandwidth; 20MHz for 6GHz and only for UMa and UMi-Street Canyon, 200 MHz for 7 GHz. UT distribution; For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type.. UT antenna configurations; Config A for 6GHz and only for UMa and UMi: Mg = Ng = 1, M = N = 1, P = 2 Config B for 7GHz: reference 4 antenna port with single polarization for calibration based on handheld device antenna model described in Clause 7.3 Config C for 7 GHz: Mg = Ng = 1, M = 2, N = 2, P = 2. UT antenna pattern; Config A for 6GHz: FFS Config B and C for 7 GHz: Based on [directional antenna] for UT described in Clause 7.3.. O2I penetration loss; For SMa, FFS. ### 7.8.3 Calibration of additional features The calibration parameters for the calibration of oxygen absorption, large bandwidth and large antenna array, spatial consistency, and blockage, near field channel modeling, and spatial non-stationarity can be respectively found in Table 7.8-3, 7.8-4, 7.8-5, and 7.8-6, 7.8-7, and 7.8-8. Unspecified parameters in these tables are the same as those in Tables 7.8-1 and 7.8-2. When P=2, X-pol (+/-45 degree) is used for BS antenna configuration 1 and X-pol (0/+90 degree) is used for UT antenna configuration. The calibration results based on TR 38.900 V14.0.0 can be found in R1-1700990. Table 7.8-7: Simulation assumptions for calibration for near field channel modeling Parameter; Values. Scenarios; UMi-street Canyon, [Indoor-office (open office)]. Carrier Frequency; [6 or 7 GHz]. Bandwidth; 20 MHz. BS antenna height; UMi: 10m [Indoor-office: 3m]. BS antenna downtilt; 102 degrees for UMi Street Canyon [110 degrees for indoor]. BS Tx power; 44 dBm for UMi-Street Canyon [24 dBm for Indoor for all carrier frequencies]. BS antenna configurations; For UMi scenario: - Config1: (M, N, P,Mg, Ng,MP,NP) = (24,16,2,1,1, 8,16), dh = 0.5λ,dv = 0.7λ; - Config2: (M, N, P,Mg, Ng, MP,NP) = (24,32,2,1,1,8,32), dh = 0.5λ,dv = 0.7λ; [For Indoor-office scenario:FFS]. UE Location; Outdoor/indoor; UMi and UMa: 100% outdoor Indoor-office: 100% indoor. LOS/NLOS; 100% LOS. UE antenna height; UMi and UMa: 1.5m Indoor-office: 1m. UT antenna configurations; - Config 1: Mg = Ng = 1, M = 2, N = 1, P = 2 - Config 2: Mg = Ng = 1, M = 2, N = 2, P = 2 - Config 3: 4 antenna ports with single polarization based on the following antenna placements - Config 4: Mg = Ng = 1, M = 4, N = 2, P = 2. Calibration method; Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric per distance; - Note: the value Z can be (1,2,3,4,5,10)* minimum distance of BS and UT - For UMi, the minimum distance is 10 m (ref. Table 6-1: Scenarios in TR36.873) Component of NF channel to be considered: - Phase of Direct path; - Phase of Non-direct path (optionally);. Metric; 1) CDF of the ratio between the 2nd,3rd,4th,...,xth singular value and the 1st singular value (serving cell) at t=0 plotted in 10*log10 scale with/without SNS at UE side Note: The value of x are the minimum value of (number of BS antenna ports, number of UE antenna ports) Note: The singular value are calculated in the M PRBs, e.g., M=4;. Table 7.8-7: Simulation assumptions for calibration for spatial non-stationarity Parameter; Values. Scenarios; UMi-street Canyon. Carrier Frequency; [6 or 7 GHz]. Bandwidth; 20 MHz. BS Antenna height; UMi: 10m. BS Tx power; 44 dBm for UMi-Street Canyon. BS Ant Config.; M = 4,N = 4,P = 2, Mg = 1, Ng = 2, dH = dV = 0.5λ, dH,g = dV,g = 2.5λ. BS port mapping; Config 1: all 16 elements for each polarization on each panel are mapped to a single port; panning angles of the two subarrays: (0,0) degs; same downtilt angles as used for the large-scale calibrations Config 2: each antenna element is mapped to one port. UT Ant Config.; 4 antenna ports with single polarization based on the antenna placements for 4 elements of handheld devices. UT distribution; Following TR36.873 for UMi, (3D dropping). Metric; 1) CDF of coupling loss (serving cell) received per UE port with/without SNS at UE side 2) CDF of the ratio between the 2nd,3rd,4th,...,xth singular value and the 1st singular value (serving cell) at t=0 plotted in 10*log10 scale with/without SNS at UE side Note: The value of x are the minimum value of (number of BS antenna ports, number of UE antenna ports) Note: The singular value are calculated in the M PRBs, e.g., M=4;.

Note: Email discussion on the simulation assumption for calibration until Mar.5th .

### RAN1 #120bis (April-2025)

Agreement

- Endorse R1-2503060 to be captured as reference for TR38.901.

- R1-2503060 contains the latest update of measurement data sources for channel model as part of the Rel-19 channel modeling enhancements for 7-24 GHz SI.

Agreement

Draft CR R1-2503129 to TR38.901 is endorsed in principle.

### RAN1 #121 (May-2025)

Agreement

- Copy the following summary of observations, conclusions, and agreements to the cover sheet of the measurement data source Tdoc, intended to be captured as reference to the TR.

Based on measurement data provided, RAN1 concludes that following channel modeling parameters are validated and no updates to the TR are made. - Pathloss - InH-Office LOS and NLOS - RMa LOS and NLOS - InF LOS and NLOS - UMi LOS and NLOS - Delay Spread - InH LOS and NLOS - Shadow fading - InH LOS and NLOS - UMi NLOS - Angular spread - UMa LOS ZSD - UMa NLOS ZSD For the following channel modeling parameters, sources have observed both consistent and different measurements compared with model in TR38.901 v18.0.0. Due to lack of consensus, the following channel modeling parameters are not updated. - Pathloss - UMa LOS and NLOS - Delay spread - InF LOS and NLOS - Angular spread - ASD, ASA, ZSA, and ZSD of InH LOS and NLOS - ZSD of UMi LOS and NLOS - ZSA of UMa LOS - Shadow fading - UMi LOS While measurement of ZSD for rural macrocell deployments from a source observed lower ZSD values compared to ZSD for RMa at shorter distances, and measurement of ASD for rural macrocell deployments from a source observed lower ZSD values compared to ASD for RMa, there was no consensus to update RMa ASD and ZSD parameters due to lack of measurement data for each of LOS, NLOS, and O2I cases. For the following channel modeling parameters, RAN1 has identified necessary updates at least for 6 – 24 GHz frequency range. The updated parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI. The data points were divided into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz and weighted for processing. The data sets for each group was used to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1. Each group is given equal weightage. - Delay spread - UMi LOS and NLOS - UMa LOS and NLOS - Angular spread - ASA, ASD, ZSA of UMi LOS and NLOS - ASA, ASD, ZSA of UMa LOS and NLOS - Cluster ASD of UMa LOS, NLOS, and O2I - ASD of UMa O2I - ZSA of UMa NLOS For ASD of UMa O2I scenario, the update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz. UMa O2I ASD values of Rel-14 TR38.901 used values from UMa O2I ASD values of TR36.873. UMa O2I ASD values of TR36.873 used UMi O2I ASD values from ITU-RM.2135-1. IMT-2020 modeling used the same values from TR36.873. TR25.996 does not contain UMa O2I values. The UMi O2I ASD parameters were derived from Winner II report. For Cluster AoD spread of UMa LOS, NLOS, and O2I scenarios, the update parameter for LOS and NLOS was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1. Each group is given equal weightage. The update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz. The number of clusters for UMi was taken from Winner II which used the 95% CDF of the number of clusters assuming K-mean clustering algorithm for counting number of clusters. For material penetration loss model, IRR glass penetration loss model was updated based on single coating for IRR glass. For other materials, standard multi-panel glass, concrete, and wood, model was not updated. However, reference thickness of the materials were clarified. New material penetration loss model for plywood has been agreed. Angle scaling and shifting for CDL based channel model for link-level evaluations was corrected and updated. The number of clusters for UMi was taken from Winner II which appears to have use the 95%-tile value from the CDF of the number of clusters assuming K-mean clustering algorithm for counting number of clusters. For UMa NLOS case, the 90%-tile and maximum number of cluster measurements from sources was observed to be aligned. For UMa LOS and UMi LOS cases, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally higher than the values in current TR38.901 by 1 to 6 clusters. For InH LOS, InH NLOS, UMi NLOS, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally lower than the values in current TR38.901 by 1 to 6 clusters. For UMa LOS/NLOS, UMi LOS/NLOS, and InH LOS/NLOS cases, the mean number of cluster measurements were significantly lower than values in current TR38.901. For other fast fading channel model parameters not mentioned above, no observation and conclusions are made due to lack of measurement data inputs. Beyond changes to the existing channel model, the following new components were added: - new deployment scenario, suburban macro (SMa), - new antenna model for handheld UT and consumer premise equipment (CPE) UT, - absolute time of arrival modeling for InH, UMi, UMa, RMa, and SMa, - nearfield channel propagation, - spatial non-stationarity modeling for BS and UT.

Agreement

- Copy the excel sheet in R1-2504913 as additional tab sheets of the measurement data source excel file, intended to be captured as reference to the TR.

Agreement

- R1-2504696 to be captured as reference for TR38.901 when endorsed.

  - Note: R1-2504696 contains the latest update of measurement data sources for channel model as part of the Rel-19 channel modeling enhancements for 7-24 GHz SI.

Post Email discussion for draft&final CR for TR38.901 (26th -28th, May)

## Agenda 9.8.1

### RAN1 #116-bis (April-2023)

Conclusion

- To provide measurement data, and/or simulation results, and/or available publications with measurement information for frequencies 7 to 24 GHz to validate/update the channel model.

- For frequency continuity of the channel models, Measurement information outside 7 to 24 GHz is also encouraged

Agreement

The following provides list of modelling parameters for 7 – 24 GHz frequencies that could be further studied for validation. The parameters listed are starting point for further discussions and does not imply the parameters require validation nor imply parameters require updates for 7 – 24 GHz frequencies.

- Antenna modelling parameters (e.g. radiation power patterns, directional gain values, etc.)

- Pathloss

- LOS probability

- O-to-I penetration loss

- Delay spread (mean, variance)

- AoD spread (mean, variance)

- AoA spread (mean, variance)

- ZoA spread (mean, variance)

- ZoD spread (mean, variance)

- ZoD offset

- Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions)

- Shadow fading

- K factor (mean, variance)

- LSP cross correlations

- Delay scaling parameter

- XPR

- Number of clusters

- Number of rays per cluster

- Cluster delay spread

- Cluster ASD

- Cluster ASA

- Cluster ZSD

- Cluster ZSA

- Per Cluster shadowing

- Correlation distances

- LSP correlation type (e.g. site-specific or all correlated)

- Oxygen absorption

- Correlation distance for spatial consistency

- Blockage region parameters/blocker parameters

- Spatial correlation for blockages

- Material properties for ground reflector model

- Spatial consistency model A/B

Conclusion

RAN1 to continue discussion on the need for new modelling parameters/scenarios and modelling procedure. The following modelling parameters/aspects for 7 – 24 GHz frequencies that are currently not available in TR38.901 have been identified by companies in RAN1#116bis. At least the following is for further study, but does not imply parameters/scenarios and modelling procedure are required for 7 – 24 GHz frequencies.

- Intra-cluster K factor

- Random power variability in each polarization

- Addition of SMa deployment scenario

### RAN1 #117 (May-2024)

Observation

- Some companies provided information that sub-urban deployments cannot be represented by existing deployments in TR38.901 (such as UMi, UMa, RMa).

Conclusion

The following parameters are used as a starting point for aligning companies understanding of channel model parameters related to suburban use cases.

- BS height: [22.5] m

- Layout:	Hexagonal grid, 19 Macro sites, 3 sectors per site, ISD = [1732] m

- Typical building heights: [Up to two floors for residential buildings, up to five floors for commercial buildings]

- UT height: [1.5 or 4.5 m for residential buildings], [1.5/4.5/7.5/10.5/13.5 m for commercial buildings]

- UT distribution: [Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor]

- FFS: ratio between residential and commercial buildings

- Indoor/Outdoor: [80% indoor and 20% outdoor, FFS on in-car users]

- LOS/NLOS: LOS and NLOS

- Min BS - UT distance(2D): [25] m

Conclusion

- To provide information about motivation and reasons why changes to the channel model are essential.

Agreement

- Further study whether the following parameters for existing deployment scenarios is necessary to be updated:

  - Delay spread, pathloss, penetration loss, AoD/AoA/ZoA/Zod spreads, ZoD offset, Angle distribution characteristics (e.g. exponential, Gaussian, Laplacian distributions), XPR, number of clusters, number of rays per cluster, LSP correlations type (e.g. site-specific or all correlated), UE antenna modeling parameters, K factor (mean, variance)

- Study of updates to other parameters are not precluded and subject to further study.

Agreement

- Further study whether/how to reflect absolute delay between links, or whether/how correlation type of the delay needs to be changed from site-specific to all-correlated type in the model

  - Note: site-specific and all-correlated definitions are provided in TR38.901 Section 7.6.3.4.

  - FFS: impact of ISD on correlation type for the deployment scenario

Agreement

- Further study on correcting the scaling of the angles and other alternative to address angle scaling in TR38.901 Section 7.7.5 to enable accurate desired angle spread.

Agreement

- Further study of whether/how to model the variability of the co- and cross polar powers, in both diagonal and anti-diagonals of the polarization matrix, in the TR 38.901 model.

  - FFS: variability is applied for per ray or per cluster or per link

  - FFS: impact of antenna configurations

  - For example, variability may be random with an i.i.d. zero-mean Gaussian with some standard deviation, via changes to step 9 and eq (7.5-22) and (7.5-28) in clause 7.5 in TR 38.901.

Agreement

- Further study whether/how to model intra-cluster K factor to the TR38.901 models, such as power re-normalization among intra-cluster rays of a cluster so that first intra-cluster ray has K times more average power compared to rest of the intra-cluster rays.

  - FFS: whether same or different intra-cluster K factor is applied for each clusters

  - FFS: which applicable deployment scenarios

Agreement

- Further study whether/how following UE antenna modelling aspects should be considered in the modelling:

  - UE antenna placement, e.g. placement along edges of a rectangle reflecting UE form factor,

  - UE antenna orientation of individual antenna elements, e.g. randomize UE antenna element orientation,

  - Antenna radiation pattern, e.g. consider more realistic antenna patterns, including a phase component, potential reuse the parabolic pattern,

  - Antenna imbalance

- Note: this is only used for calibration.

### RAN1 #118 (August-2024)

Agreement

- Enable necessary model to support “sub-urban macro deployments”. Scenario of interest can be implemented when necessary by one of the following options:

  - Option-1: Define alternate parameters for UMa based on different assumption on building density/heights and corresponding BS and UE height/distributions, ISD, other aspects related to sub-urban macro deployments.

  - Option-2: Define a new scenario, e.g., SMa.

  - FFS parameter commonality and differences between UMa and scenario of interest

  - FFS whether and how to enable frequency continuity beyond 7-24 GHz for scenario of interest

Agreement

- At least for calibration purposes, introduce new UE antenna model that potentially provides updates to following parameters:

  - UE antenna placement

    - E.g. placement along edges of a rectangle reflecting UE form factor.

  - UE antenna orientation

    - E.g. randomize UE antenna orientation

  - Antenna radiation pattern

    - E.g. consider more realistic antenna patterns, including a phase component

    - Consider reusing the parabolic pattern

- Note: not all parameters are necessarily updated from current calibration antenna model.

Agreement

- At least for calibration purposes, for a new UE antenna model, additionally

  - FFS: Antenna imbalance

Observation

- ‘standard’ Glass penetration loss model in TR38.901 seems to be aligned well with measurements in 7-24 GHz conducted by companies.

- Some exceptional cases are wood, concrete and IRR glass penetration losses. It should be noted that assumption of at least material thickness has impact to penetration loss and further alignment of at least material thickness is useful for further validation.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on penetration loss at least for the wood, concrete and IRR glass penetration loss and provide details of experimental setup used for penetration loss measurements.

Observation

- Preliminary study shows pathloss updates may not be needed at least for the following scenarios:

  - InH-Office LOS/NLOS, UMi Street Canyon LOS/NLOS, InF LOS/NLOS, RMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on at least pathloss for the following applicable scenarios, UMa LOS/NLOS.

Observation

- Preliminary study shows delay spread updates may not be needed at least for the following scenarios:

  - InH-Office NLOS

- Preliminary study shows delay spread updates may be needed at least for the following scenarios:

  - UMi LOS/NLOS

- Preliminary study shows different conclusions from different sources on whether delay spread updates are needed at least for following scenarios and further study and validation is needed.

  - InH-Office LOS

  - UMa LOS/NLOS/O2I

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on at least delay spread for applicable scenarios, including any frequency dependency analysis of delay spread.

- Companies are encouraged to provide methodology of frequency dependence analysis of delay spread, if performed

Observation

- Preliminary study shows some updates may be needed for azimuth and zenith angular spread for at least following scenarios:

  - InH LOS/NLOS, UMi LOS/NLOS, UMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on angular spread for applicable scenarios. The following are preliminary examples for identified scenarios:

| Scenario |  | InH @10 GHz |  | UMi @10 GHz |  | UMa @13 GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| ASD lgASD=log10(ASD/1°) | μlgASD | 1.21 | 1.27 | 1.04 | 1.14 | 1.08 | 1.25 |
|  | σlgASD | 0.18 | 0.14 | 0.2 | 0.07 | 0.21 | 0.3 |
| ASA lgASA=log10(ASA/1°) | μlgASA | 1.29 | 1.5 | 1.19 | 1.37 | - | - |
|  | σlgASA | 0.13 | 0.23 | 0.13 | 0.08 | - | - |

| Scenario |  | InH @10.1 GHz |  | UMi @10.1 GHz |  | UMi @ 10GHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| ASD | μlgASD | 8.3 | 24.0 | 16.6 | 22.8 | 36.7 | 17.4 |
|  | σlgASD | 4.8 | 13.7 | 15.8 | 19.3 | 20.4 | 24.0 |
| ASA | μlgASA | 28.4 | 47.6 | 32.8 | 60.2 | 19.2 | 31.7 |
|  | σlgASA | 7.3 | 20.6 | 16.0 | 12.6 | 11.0 | 9.3 |
| ZSD | μlgASD | 10.5 | 6.6 | 6.8 | 7.9 | - | - |
|  | σlgASD | 8.6 | 10.5 | 2.8 | 1.3 | - | - |
| ZSA | μlgASA | 4.4 | 8.3 | 13.5 | 12.6 | - | - |
|  | σlgASA | 1.8 | 6.2 | 3.2 | 3.8 | - | - |

| Scenarios |  | UMa @ 3.5 GHz, 13 GHz, 28 GHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| ASD lgASD=log10(ASD/1) | µlgASD | 0.39 + 0.1114 log10(fc) | 0.83 - 0.1144 log10(fc) | 0.58 |
|  | σlgASD | 0.4 | 0.7 | 0.7 |
| Cluster ASD (cASD) in [deg] |  | 1.5 | 1.5 | 1.5 |

| Scenario |  | InH @15 GHz |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| ASA lgASA=log10(ASA/1°) | μlgASA | 1.57 | 1.78 |
|  | σlgASA | 0.15 | 0.15 |
| ZSA lgASA=log10(ZSA/1°) | μlgASA | 0.94 | 0.94 |
|  | σlgASA | 0.05 | 0.06 |

| Scenario |  | UMa @6.5 GHz |  |
| --- | --- | --- | --- |
|  |  | LOS | NLOS |
| ASD lgASD=log10(ASD/1°) | μlgASD | 0.82 | 1.26 |
|  | σlgASD | 0.28 | 0.27 |
| AOA spread (ASA) lgASA=log10(ASA/1°) | μlgASA | 1.67 | 1.72 |
|  | σlgASA | 0.19 | 0.15 |

Observation

- Preliminary study shows some updates may be needed for number of cluster and/or the threshold for removing lower powered clusters in the channel model for at least following scenarios:

  - InH LOS/NLOS, UMi LOS/NLOS, UMa LOS/NLOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of number of clusters for applicable scenarios.

- The following are preliminary examples for identified scenarios:

  - InH LOS/NLOS number of clusters

    - LOS @ 10 GHz, 500 MHz measurement BW: 15 ⇒ 10

    - LOS @ 10.1 GHz, 500 MHz measurement BW: 15 ⇒ 6

    - NLOS @ 10 GHz, 500 MHz measurement BW: 19 ⇒ 11

  - UMi LOS/NLOS number of clusters

    - LOS @ 10 GHz, 500 MHz measurement BW: 12 ⇒ 5

    - LOS @ 10.1 GHz, 500 MHz measurement BW: 12 ⇒ 8

    - NLOS @ 10 GHz, 500 MHz measurement BW: 19 ⇒ 7

    - NLOS @ 10.1 GHz, 500 MHz measurement BW: 19 ⇒ 10

  - UMa LOS/NLOS number of clusters

    - LOS @ 13 GHz, 400 MHz measurement BW: 12 ⇒ 9

    - NLOS @ 13 GHz, 400 MHz measurement BW: 20 ⇒ 14

- Companies are encouraged to disclose measurement methodology and conditions for determining number of clusters, such as measurement noise floor, capture dynamic range, measurement bandwidth, angular/spatial resolution, etc.

Observation

- 1 source observed existence of dominant angle for each cluster angular power distribution in 13 GHz UMa NLOS scenario. The source noted that dominant angle representation of the channel may be done by introducing an intra-cluster power factor.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

- The following is an example of potential changes:

Hu,s,nNLOSt=ICP∙PnICP+1Frx,u,θθn,1,ZOA,ϕn,1,AOAFrx,u,ϕθn,1,ZOA,ϕn,1,AOATexpjΦn,1θθκn,1-1expjΦn,1θϕκn,1-1expjΦn,1ϕθexpjΦn,1ϕϕFtx,s,θθn,1,ZOD,ϕn,1,AODFtx,s,ϕθn,1,ZOD,ϕn,1,AOD

expj2πrrx,n,1T∙drx,uλ0expj2πrtx,n,1T∙dtx,sλ0expj2πrrx,n,1T∙vtλ0

+PnICP+1M-1m=2MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AOD

expj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vtλ0

where ICP is the intra cluster power ratio.

Conclusion

- To potentially reflect the channel angular domain sparsity, continue study of unequal intra-cluster power distribution for applicable scenarios.

Observation

- Preliminary study shows LOS probability do not need to be updated for the frequency range of interest for all existing scenarios. Further study is needed on LOS probability if existing scenario building/node deployment statistics are updated or if new deployment scenario(s) are adopted.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Observation

- 3 source observed variability in power between co-polarized and cross-polarized antennas.

  - Among the 3 sources, 1 source observed ground reflection model in 38.901 may be used to introduce the polarization variability.

- 1 source observed variability in power between co-polarized and cross-polarized antennas for fix antenna setup. The source noted that UE relative polarization is not fixed in practice and questioned the applicability of the measurements for deployments of interest.

- 1 source observed that power ratio for co-polarization and cross-polarization is influenced by the depolarization effects of the environment and the antenna assumptions. Furthermore, under ±45° antenna slant assumption, the ratios of cross-polarization received power to co-polarization received power have a consistent mean value and therefore existing polarization modeling in TR 38.901 is sufficient.

Conclusion

- Continue study on handling of polarization for applicable scenarios.

Observation

- Preliminary study shows inconclusive results on whether updates may be needed for shadow fading for pathloss parameters of the channel model.

  - 1 source observed shadow fading values, are frequency-independent based on measurements at 6.75, 16.95, 28, and 73 GHz for InH-Office scenario. The discrepancies in shadow fading values are less than 0.3 dB for LOS and less than 2 dB for NLOS channel conditions across the entire frequency range of 0.5-100 GHz.

  - 1 sourced observed frequency dependency of the shadow fading in UMa LOS/NLOS scenario in 7-24 GHz, which the current channel modeling does not have.

- Preliminary study shows inconclusive results on whether updates may be needed for shadow fading per cluster parameters of the channel model.

  - 1 source observed 3dB reduced per cluster shadowing standard deviation for UMi LOS scenario, 1 dB reduced per cluster shadowing standard deviation for InH LOS scenario.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue.

Conclusion

- Continue study on handling of shadow fading parameters for applicable scenarios.

Observation:

- Preliminary study shows some updates may be needed for K-Factor parameters of the channel model for at least following scenarios:

  - InH LOS, UMi LOS

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of K-factor parameters for applicable scenarios. The following are preliminary examples for identified scenarios:

  - InH LOS K-factor

    - Mean 9, std-dev 5 ⇒ Mean 5.1, std-dev 3.2

  - UMi LOS K-factor

    - Mean 7, std-dev 4 ⇒ Mean 8.5, std-dev 3.5

Observation:

- Inconclusive on whether CDL model angle calculation update is needed.

- Note that these are initial observations from RAN1 #118 and does not represent conclusive observations for the SI. RAN1 study and validation is expected to continue, including frequency continuity aspects.

Conclusion

- Continue study on handling of other channel modeling aspects. The following are examples of suggested changes for angle calculation for CDL model by sources:

  - Example 1)

      - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,                  (7.7-5)

      - where

      - ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model,

      - and

      - Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle.

      - ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}.

      - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Example 2)

      - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, and

      - ϕn,intermediate=s* ϕn,model-μϕ,model, where s is a scale factor to achieve desired angular spread.

Conclusion

- Continue study on handling of channel delays between different UE-TRP links. The following are examples of how absolute delays between different UE-TRP link may be applied in 38.901 provided by companies.

  - Example 1) introduce a new correlation type called “physically consistent” that takes the individual UE-TRP distances into account when generating the link-specific delays.

  - Example 2) Reuse absolute delay modelling in section 7.6.9 in TR 38.901 with extension for other scenarios as follows:

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa |
| --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7 | -6.8 |
|  | σlgΔτ | 0.4 | 0.4 | 0.4 | 0.6 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 15 | 50 |

Agreement

The following assumptions are modeling parameters related to suburban use case. For aspects with multiple options, FFS which option(s) to support.

- BS height:

  - Option 1: 22.5 m

  - Option 2: 20 m – 25 m

  - Option 3: 15 m

  - Option 4: 35 m

  - Option 5: 25 m

- Layout:	Hexagonal grid, 19 Macro sites, 3 sectors per site,

  - Option 1: ISD = 1732 m

  - Option 2: ISD = 500 m

  - Option 3: ISD <= 1000 m

  - Option 4: 500 ~ 1000 m

  - Option 5: 1299 m

  - Option 6: ISD < 500 m

- Building density

  - Option: [373, 440, others] buildings/km2

- UT height: 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings

  - Typical building heights: Up to two floors for residential buildings, up to five floors for commercial buildings

- UT distribution:

  - Option 1: Uniform horizontally, 70% indoor residential users are on ground floor, 30% are on upper floor

  - Option 2: 0.5 m for outdoor; 1.5+3(Nfl-1) m for indoor, where Nfl is the floor and Nfl is uniform (1, 2) for residential buildings or is uniform (1, 5) for commercial buildings. (framework in 36.873 for UMa with changes to distribution of Nfl (number of floors).)

- Indoor/Outdoor:

  - Option 1: 80% indoor and 20% outdoor,

    - FFS: ratio between residential and commercial buildings

    - FFS on in-car users

  - Option 2: 40% indoor in residential buildings, 40% indoor in commercial buildings, and 20% outdoor

- LOS/NLOS: LOS and NLOS

- Min BS - UT distance (2D):

  - Option 1: 25 m

  - Option 2: 10 m

  - Option 3: 35 m

- FFS: Penetration model: low-loss penetration model

- FFS: clutter height parameter, e.g. foliage

Conclusion

Study at least the following channel modelling aspects of suburban use case. The sub-bullets describing the detailed equations of the modelling aspect are examples for consideration:

- Pathloss

  - NLOS: A+B⋅log10d+20⋅log10fc+10⋅log10minfc,10

  - LOS: Reuse pathloss model of UMa scenario in TR 38.901

  - LOS: PLLoS=22.9log10(d3D)+19.6log10f+28.6 dB, σ=3.48 [dB]

  - For LOS and NLOS, reuse Winner II

- LOS probability

  - PrLOS=1d2D-out≤10 mexp-d2D-out-1040010 m<d2D-out

- ASD

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) | µlgASD | 0.55 | 0.55 | 0.55 |
|  | σlgASD | 0.25 | 0.25 | 0.25 |
| Cluster ASD (cASD) in [deg] |  | 1.5 | 1.5 | 1.5 |

- ZSD

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1°) | µlgZSD | 0.55 | 0.55 | 0.55 |
|  | σlgZSD | 0.25 | 0.25 | 0.25 |

### RAN1 #118-bis (October-2024)

Agreement

For new UE antenna model, antenna placement candidate locations relative to centre of the handheld device is as follows: - UE antenna modeling (at least for calibration), the UE antennas are placed along the edges of a rectangle reflecting a UE form factor. - The size of the device can be set to be X cm x Y cm x Z cm - FFS (X,Y, Z), e.g., (X,Y,Z) can be15cm x 6 x 0 cm, - The four corners and the centres of each edge are identified as potential locations of the UE antenna. Centre of the device is also identified as potential location of the UE antenna. - Antennas (except the centre of device antenna) are assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location. centre of device antenna is assumed to be oriented in either forward facing or backward facing the plane of the device. - FFS: antenna field pattern with respect to the direction of antenna - FFS: how antenna elements and antenna modules are mapped to antenna location candidates, e.g., for FR1, antenna location candidates are for antenna elements, and for FR2, antenna location candidates are for antenna array modules. - FFS: how to handle polarization aspects of the antenna element/module.

Agreement

For UE antenna radiation pattern modeling, consider the following options for further study:

- Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS |
| Maximum directional gain of an antenna element GE,max | FFS dBi |

- Option 2) half-wavelength dipole.

  - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

    - FFS: range of θ

- Option 3) omni-directional antenna pattern

- Other options

Agreement

The following assumptions are updated modeling parameters related to SMa use case.

| Parameters |  | SMa | Options for FFS |
| --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = FFS) Up to two floors for residential buildings, up to five floors for commercial buildings. FFS: ratio between residential and commercial buildings | ISD Option 1: 1732 m ISD Option 2: 1299 m ISD Option 3: 1000 m ISD Option 4: 250 m Building Type Ratio Option A: 50% residential, 50% commercial Building Type Ratio Option B: 90% residential, 10% commercial |
| BS antenna height hBS |  | FFS | BS Height Option 1: 35 m BS Height Option 2: 25m BS Height Option 3: 15m |
| UT location | Outdoor/indoor | Outdoor and indoor | - |
|  | LOS/NLOS | LOS and NLOS | - |
|  | Height hUT | 1.5 m for outdoor, 1.5 or 4.5 m for residential buildings, 1.5/4.5/7.5/10.5/13.5 m for commercial buildings | - |
| Indoor UT ratio |  | 80% indoor and 20% outdoor |  |
| UT mobility (horizontal plane only) |  | FFS |  |
| Min. BS - UT distance (2D) |  | FFS | Min Distance Option 1: 20m Min Distance Option 2: 25m Min Distance Option 3: 35m |
| UT distribution (horizontal) |  | Uniform | - |
| UT distribution (vertical) |  | FFS | Vertical Distribution Option 1: 70% indoor residential users are on ground floor, 30% are on upper floor, uniform distribution across all floors for commercial Vertical Distribution Option 2: uniform distribution across all floors for a building type |
| Penetration model |  | FFS: low-loss penetration model | - |

Agreement

Further study and down-select cell layout, BS antenna height, and building type ratio for modeling parameters related to SMa use case. The Following are sets of option combinations prioritized for study and down-selection. Other option combinations are not precluded.

- Alt 1) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m

- Alt 2) ISD 1000m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 3) ISD 250m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 4) ISD 1299m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

- Alt 5) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 35m

- Alt 6) ISD 1732 m + Building Type Ratio 90% residential, 10% commercial + BS Height 25m

Agreement

Further study methods of achieving frequency continuity and handling frequency dependency parameters of the channel model.

Observation:

- 4 sources showed wood penetration loss model in TR38.901 seems to with acceptable range of deviation with measurements in 7-24 GHz. 1 source from RAN1 #117 showed wood penetration loss model in TR38.901 deviates from measurement by more than 5 dB for certain frequency of interest within 7 – 24 GHz.

- 3 sources and 1 source from RAN1 #117 showed Concrete penetration loss seems to deviate from measurements by more than 25 dB for certain frequency of interest within 7 – 24 GHz.

- 3 sources show IRR glass penetration loss seems to deviate from measurements for certain frequency of interest within 7 – 24 GHz.

  - It is noted that the spread of measurements from companies could be due to selection of the glass window type (e.g. fixed, sliding, swinging, etc), thickness of the window, and number of panes.

Continue study on penetration loss for Wood, Concrete, and IRR glass taking into account measurement study conducted for the original TR38.901 (available in http://www.5gworkshops.com/5GCMSIG_White%20Paper_r2dot3.pdf)

Agreement

Study possibility of introduction of additional O2I building penetration loss model type that consider the following:

- concrete penetration loss model that is potentially different from existing model

- Note: other aspects are not precluded

Companies are asked to provide information on the need for the different penetration loss model. Companies are encouraged to provide penetration loss per unit thickness (1 cm) of the material.

Observation

- 1 Source suggested updates to UMa NLOS pathloss based on measurement of UMa

- 1 Source suggested updates to UMi LOS and NLOS pathloss based on measurement of UMi

  - Note methodology to obtain the curve fit for measurement data has been questioned.

- For other scenarios InH and RMa, multiple sources reported reasonable alignment of measurements to current model in TR38.901.

Continue study on pathloss for applicable scenarios, UMa NLOS. The following are example of suggests changes:

- UMa NLOS: 13.54 + 38.6 39.09 log10( d3D ) + 20 log10( fc ) – 0.6 (hUT – 1.5)

Observation

- The following delay spread results were observed by 5 sources for UMi and UMa scenarios.

  - The center of the range of the values for a specific source in the figure represent the mean of the log delay spread.

  - The line range of the values for a specific source in the figure represent the standard deviation of the log delay spread.

Conclusion

- RAN1 concludes LOS probability is validated for existing deployment scenarios defined in TR38.901.

  - Note: this does not preclude introduction of new LOS probability for new scenarios of interest and/or different BS height to be supported for new/existing scenarios.

Observation

- 2 sources observed aligned shadow fading parameters in the frequency ranges of interest for InH LOS and NLOS scenarios.

- 1 source observed (from RAN1 #118) shadow fading parameters to deviate from TR38.901 by more than 1 dB for InH LOS, InH NLOS, and UMi LOS scenarios.

- 2 sources observed no need for updates to shadow fading parameters in the frequency ranges of interest.

Continue discuss on shadow fading parameters for InH and UMi scenarios.

Observation

- 1 source observed frequency dependency of K-Factor for UMi LOS scenario.

- 1 source observed (in RAN1 #118) deviation of mean K-factor by 4 dB for UMi LOS scenario and 1.5 dB for InH LOS scenario.

Continue study of K-factor parameters for InH LOS and UMi LOS scenarios.

Agreement

- Introduce absolute delay for UMi and UMa scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - Details of the absolute delay modeling for UMi and UMa FFS.

  - Note: this does not preclude potential introduction of absolute delay for other scenarios.

  - The following parameters are examples:

- Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InF-SL, InF-DL | InF-SH, InF-DH | UMi | UMa |
| --- | --- | --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.5 | -7 | -6.8 |
|  | σlgΔτ | 0.4 | 0.4 | 0.4 | 0.6 |
| Correlation distance in the horizontal plane [m] |  | 6 | 11 | 15 | 50 |

Observation

- 1 Source observed decorrelation distance for DS and KF for UMi LOS are different from measured results, and possibly have frequency dependency.

Continue study on correlation distances for applicable scenarios.

Agreement

Study the possibility of supporting a < 500 m ISD for UMa

- FFS whether any other modeling parameter is required to be updated

Agreement

Further study LOS probability assumption for SMa

- The following options are example of LOS probability assumption for SMa

| Scenario | LOS probability (distance is in meters) |
| --- | --- |
| SMa | Option1: PrLOS=exp-d2D-outexpα∙hBSβ+γhUEδ - FFS: α, β, γ, δ parameters Option 2: (ITU M.2135-1) PLOS=1, d2D-out≤10exp-d2D-out-10200, d2D-out>10 - BS height of 35m Option 3: PLOS=1, d2D-out≤10exp-d2D-out-10562, d2D-out>10 - For BS height of 25m, and UE height of 1.5m |

Working assumption

Use the following pathloss formula assumption for SMa (based on ITU M.2135-1).

- FFS: refinements based on additional new simulations/measurements

- FFS whether pathloss formula can be applicable for hUT higher than 10m

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters | Shadow fading std [dB] | default values | Applicability range |
| --- | --- | --- | --- | --- | --- |
| SMa | LOS | PL1=20log1040πdfc3+min0.03h1.72,10log10d-min0.044h1.72, 14.77+0.002log10hd PL2=PL1dBP+40log10ddBP | σ=4 σ=6 | 10 m < d< dBP dBP < d < 5000 m W = 20m, h = 10m | 0.8 ≤fc≤ 37 GHz 5 m < h < 50 m 5m < W < 50 m 10 m < hBS < [150] m 1 m < hUT < [10] m |
|  | NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 FFS potential changes: PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc+10⋅log10minfc,10-10⋅log102-3.2log1011.75hUT2-4.97 | σ=8 | 10 m < dBP < 5000 m W = 20m, h = 10m |  |

Agreement

Further study the following modeling parameters and associated parameter assumption values for SMa:

- Correlation distance for spatial consistency

- Spatial correlation distance

- Blocking region parameters (For Blockage Model A)

- Fast fading parameters

### RAN1 #119 (November-2024)

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Building type ratio: 90% residential, 10% commercial

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT Vertical Distribution Option 2: uniform distribution across all floors for a building type

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- UT mobility in horizontal plane only: Indoor UTs: 3 km/h, outdoor UTs: 40 km/h

Agreement

For handheld devices adopt the following device dimensions for UE antenna modeling:

- 15 cm x 7 cm x 0 cm

Conclusion

- Based on measurement data provided, RAN1 concludes pathloss models in TR38.901 for the following scenarios are validated and no updates to TR are made.

  - InH-Office LOS

  - InH-Office NLOS

  - RMa LOS

  - RMa NLOS

  - InF LOS

  - InF NLOS

Observation

- For the following scenarios, some sources have observed some differences in delay spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed delay spread consistent with the model in the TR.

  - InF LOS

  - InF NLOS

Conclusion

- For the following scenarios, there is no consensus to update delay models due to lack of consistent and significant observed difference between model and measurements.

  - InF LOS

  - InF NLOS

Agreement

- Study further on the need for introduction of intra-cluster power profile.

  - Potential options for introduction of intra-cluster power profile:

    - Introduce unequal power per ray within a cluster and change the unequal angle offset to equal angle offset among rays within a cluster to maintain the same PAS characteristic as in TR 38.901 (example in R1-2409724)

    - Power scaling within a cluster based on DS associated with the cluster (example in R1-2410213)

Agreement

Further study penetration loss modeling, including the following aspects:

- Need for change and details of the updates to the material penetration loss, for example, if any

|  | Concrete | Wood |
| --- | --- | --- |
| Existing TR | Lconcrete=5+4f | Lwood=4.85+0.12f |
| New Updates | Lconcrete=α∙(5+4f) α=Tconcrete23cm | Lwood=α∙(4.85+0.12f) α=Twood6cm |
| Tconcrete and Twood represent reference (default) thickness of the material. For concrete and wood, the reference thickness are 23 cm and 6 cm, respectively. |  |  |

  - Whether penetration loss can be scaled linearly with thickness for each material

  - For standard multi-pane glass, FFS whether and how to reflect the penetration loss resonation effect, which may relate to thickness of the glass.

  - FFS the impact of updates to material penetration loss to total build penetration loss

- Need for change and details of total building penetration loss, e.g. low-loss and high-loss model, if any

- Impact of material composition on material penetration loss

Agreement

- For the following scenarios, further study path loss model parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- For the following scenarios, further study delay spread parameters for at least 6 – 24 GHz frequency range.

  - UMi LOS

  - UMi NLOS

  - InH-Office LOS

  - InH-Office NLOS

  - UMa LOS

  - UMa NLOS

Agreement

- Based on measurement data provided, RAN1 concludes angular spread parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

Observation

- For the following scenarios, some sources have observed some differences in angular spread in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed angular spread consistent with the model in the TR.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS and NLOS: ZSA, ZSD

  - UMi LOS: ZSA, ZSD

  - UMi NLOS: ZSD

  - UMa LOS: ZSA

Agreement

- For the following scenarios, further study angular spread parameters for at least 6 – 24 GHz frequency range.

  - InH LOS/NLOS: ASA, ASD

  - UMi LOS: ASA, ASD

  - UMi NLOS: ASA, ASD, ZSA

  - UMa LOS: ASD, ASA

  - UMa NLOS: ASA, ASD, ZSA

  - InF LOS and NLOS: ASD, ASA, ZSA, ZSD

Agreement

- Further study number of clusters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6GHz and above 24 GHz, including whether to update values for other frequencies based on measurements

  - Whether changes to number of clusters result in meaningful impact to system performances and evaluation of features

Conclusion

- Based on measurement data provided, RAN1 concludes shadow fading parameters in TR38.901for the following scenarios are validated and no updates to TR are made.

  - InH LOS

  - InH NLOS

  - UMi NLOS

Observation

- For the following scenarios, some sources have observed some differences in shadow fading in the TR and measurement taken for 6 – 24 GHz frequency range. Other sources have observed shadow fading with the model in the TR.

  - UMi LOS

Conclusion

- For the following scenarios, there is no consensus to update shadow fading due to lack of consistent and significant observed difference between model and measurements.

  - UMi LOS

Agreement

- Further study the need for updating the K-factor parameters for existing deployment scenarios (InH, InF, UMi, UMa, RMa) taking into consideration the following:

  - Consistency/continuity across frequencies below 6 GHz and above 24 GHz, including whether to update values for other frequencies based on measurements.

  - Whether changes to K-Factor result in meaningful impact to system performances and evaluation of features.

Working Assumption

- Adopt the following absolute delay for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

- Introduce absolute delay for InH scenarios.

  - Absolute delay is enabled by re-using the absolute time of arrival for InF scenarios.

  - FFS details of the parameter for InH. Consider the following for further study.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

- To address the angle-scaling issue when generating CDL channels for link-level evaluations, adopt the following two-step approach to compute the scaled angles.

  - ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,

  - μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A.

  - WrapTo180(⋅) is a function which wraps an azimuth angle to the half-open interval (-180, 180].

- Down-select among options.

  - Option 1)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - where s is a scale factor chosen to change the distribution of the angles (up to each company to provide information, e.g. s=ASdesiredASmodel).

  - Option 2a)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s=minx≥0{x: ASx=ASdesired}.

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 2b)

    - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

    - s is chosen such that AS(ϕn,intermediate) = AS_desired

    - ASx=-2lnn=1NexpjxAngleϕn,model-μϕ,modelPnn=1NPn

  - Option 3)

    - ϕn,intermediate=ASdesiredASmodel* WrapTo180ϕn,model-μϕ,model,

    - Note: The resulting scaled ray angle may or may not achieve the desired angle mean and the desired rms angle spread.

- FFS whether and how to enable specular rays of CDL-D and CDL-E to be in the direction of the desired vector.

Agreement

For suburban scenario, further consider following options and down-select a value for BS height assumption for calibration purposes:

- Option 1) BS height = 25m

- Option 2) BS height = 35m

Note: It should be understood that BS height selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios may be applicable for a range of BS heights [25 – 35]m.

Agreement

For suburban scenario, further consider following options and down-select a value for ISD assumption for calibration purposes:

- Option 1) ISD = 1299 m

- Option 2) ISD = 1732 m

Note: It should be understood that ISD selected here is for calibration purposes and the channel model developed for suburban macrocell scenarios is applicable for a range of ISD [1200 – 1800]m.

Agreement

For suburban scenario, further study the LOS probability equation.

- Example of LOS probability equation for study:

  - PrLOS=exp-d2D-out-dmexpα∙hBSβ+γhUEδ, where dm, α, β, γ, δ are modeling parameters

- Study includes whether distinction of LOS probability should be made for UEs in residential and commercial buildings, which have different building heights.

- Study includes whether or not vegetation should be explicitly considered and modeled for determining LOS probability, if determined to be explicitly modeled, how to model vegetation impact, e.g. reuse of InF clutter to change the LOS probability.

Agreement

Further study the need of additional low loss outdoor-to-indoor (O2I) building penetration loss model for suburban scenario.

- Potential example of the new low loss O2I building penetration loss:

PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

Agreement

- UMa can also be used for other ISDs between 200m and 500m, including the ISD proposed in ITU-R M.2412 -0

- One ISD value will be selected for calibration with ISD = 500m

- No change to any other parameters due to other ISD values

- FFS: How to reflect it in TR38.901

Agreement

Capture channel modeling updates regarding suburban macrocell scenario as follows:

- Define a new scenario, e.g., SMa

- The key differentiating aspects of SMa (compared to UMa and RMa) in high level description will be highlighted in Section “6.2 Scenario of Interest” of the TR.

Agreement

Further study device dimension and antenna placement applicability for customer premise equipment (CPE) devices, include the need for defining CPE device type dimensions.

Agreement

Further refine previous agreement (made in RAN1#118bis)  on UE antenna radiation patternmodeling as follows:

- For UE antenna radiation pattern modeling, consider the following options for further study:

  - Option 1) Adapt the Table 7.3-1: Radiation power pattern of a single antenna element

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) | AE,Vθ″=-min12θ″-90°θ3dB2,SLAV,θ3dB=FFS°,SLAV=FFS dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hϕ″=-min12ϕ″ϕ3dB2,Am,ϕ3dB=FFS°,Am=FFS dB |
| Combining method for 3D antenna element pattern (dB) | FFS A″(θ″,ϕ″)=-min-AE,Vθ″+AE,Hϕ″,Am |
| Maximum directional gain of an antenna element GE,max | FFS dBi |
| Note: max direction gain and half power beam widths should be selected such that total antenna efficiency should be equal or less than 0dBi. |  |

  - Option 2) half-wavelength dipole.

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: E(θ) = cos(cos(θ)*π /2) / sin(θ), where θ is the angle relative to the +Z-axis

      - FFS: range of θ

  - Option 2b) half-wavelength dipole

    - The normalized E-field radiation pattern of a half-wavelength dipole can be expressed as: sin(π/2*cos(θ))

    - range of θ = -90° to +90°

  - Option 3) omni-directional antenna pattern

Agreement

For UE antenna modeling of handheld devices,

- Further study on default antenna placement locations for calibration purpose for 2, 3, 4, 6, and 8 antenna elements at least for FR1 and 6-24 GHz, including the need to define default antenna placement locations, and applicability of frequency ranges, e.g. FR1, FR2.

  - FFS on handling of antenna array panels

Agreement

For handheld UE device type antenna radiation pattern, further study details of antenna field pattern orientation with respect to the direction of antenna for each antenna candidate locations.

- For example, maximal gain directivity of the antenna radiation pattern to be aligned with direction of the antenna candidate location.

### RAN1 #120 (February-2025)

Conclusion

- Based on measurement data provided, RAN1 concludes pathloss parameters in TR38.901 that following scenarios are validated and no updates to TR are made.

  - UMi LOS/NLOS

Agreement

- Further study whether/how to handle the LOS pathloss convergence regardless of the frequency band beyond the breakpoint distance phenomena.

Conclusion

- Based on measurement data provided, RAN1 concludes that following angular spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are made.

  - UMa LOS: ZSD

  - UMa NLOS: ZSD

Conclusion

- For the following scenarios, there is no consensus to update angular spread models due to lack of consistent and significant observed difference between model and measurements.

  - InH LOS : ASD, ASA

  - InH NLOS: ASD, ASA

  - UMi LOS: ASD

  - UMi NLOS: ASD, ZSA

Agreement

- Confirm working assumption (made in RAN1#119) for proposed absolute delay parameters for UMi and UMa scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | UMi | UMa |
| --- | --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -7.5 | -7.4 |
|  | σlgΔτ | 0.5 | 0.2 |
| Correlation distance in the horizontal plane [m] |  | 15 | 50 |

Agreement

Adopt the following description for suburban scenario:

- In suburban macro-cells base stations are located above the surrounding environment to allow wide area coverage, and mobile stations are outdoors at street level and within commercial and residential buildings. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure.

- FFS: vegetation

Agreement

For suburban scenario, use the BS height and ISD assumption for calibration purposes:

- BS height = 35 m

Agreement

For suburban scenario, adopt the following assumption for calibration purposes:

- Minimum BS to UT distance in 2D:

  - 35m

Agreement

Assume enumeration of candidate antenna placement locations as follows:

Agreement

- Amend the previous agreement (from RAN1 #118bis) on handheld UE antenna placement location modeling.

  - The four corners and the centres of each edge are identified as potential locations of the UE antenna. Centre of the device is also identified as potential location of the UE antenna.

  - Antennas (except the centre of device antenna) are assumed to be oriented along the direction determined by the vector connecting the centre of the rectangle to the antenna location. centre of device antenna is assumed to be oriented in either forward facing or backward facing the plane of the device.

Agreement

For calibration, the antenna placements for 4 elements of handheld devices are defined as follows:

Agreement

Define candidate antenna locations for CPE devices

FFS: Device dimensions for CPE antenna modelling.

Agreement

- For cases when a candidate antenna placement location is used for one antenna field pattern (i.e., not used for two distinct antenna polarization field patterns):

- Reference radiation pattern of the UE antenna model is vertically polarized with all the gain in the theta filed component,

  - Fθ″″θ″,ϕ″=A″θ″,ϕ″ and Fϕ″″θ″,ϕ″=0.

- For cases when a candidate antenna placement location is used for two distinct antenna polarization field pattern:

  - Reference radiation pattern of the UE antenna model is FFS,

    - For first antenna field pattern: Fθ″″θ″,ϕ″=FFS and Fϕ″″θ″,ϕ″=FFS.

    - For second antenna field pattern: Fθ″″θ″,ϕ″=FFS and Fϕ″″θ″,ϕ″=FFS.

- Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation of the each of UE antennae to get Frx,u,θ''(θ',ϕ'), Frx,u, ϕ''(θ',ϕ') and based on the orientation of the UE in the global coordinate system to get Frx,u,θ(θ,ϕ) and Frx,u,ϕ(θ,ϕ) using the methods already specified in TR 38.901, equation (7.3-3)/(7.1-11).

Agreement

- Further study and consider update of power angular spectrum (PAS) of the current cluster modeling from equal powered angle offset to unequal power angle offsets.

  - For the update of power angular spectrum (PAS) of the current cluster modeling from equal powered angle offset to unequal power angle offsets. Consider the following options:

    - Option 1) introduce the update as an optional additional modeling component

    - Option 2) update the existing intra-cluster power distribution and angles

  - Consider power adjustments made by “large bandwidth and large antenna array” additional component and review whether measurement observed can be replaced and modeled by large bandwidth and large antenna array” model.

  - The following are examples of changes

Example 1)

Finally add offset angles m from Table 7.5-3 to the cluster angles φn,m,AOA=φn,AOA+cASAαm, (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 7.5-3: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m; Ray power ratio βm; Basis vector of offset angles m. 1,2; ± 0.0447; 0.1193; ± 0.1134. 3,4; ± 0.1413; 0.0933; ± 0.3403. 5,6; ± 0.2492; 0.0729; ± 0.5671. 7,8; ± 0.3715; 0.0570; ± 0.7940. 9,10; ± 0.5129; 0.0446; ± 1.0208. 11,12; ± 0.6797; 0.0348; ± 1.2477. 13,14; ± 0.8844; 0.0272; ± 1.4745. 15,16; ± 1.1481; 0.0213; ± 1.7014. 17,18; ± 1.5195; 0.0166; ± 1.9282. 19,20; ± 2.1551; 0.0130; ± 2.1551. <unchanged text omitted> Hu,s,nNLOSt=PnMm=1MβmFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.5-22)

Example 2)

Finally add offset angles m from Table 7.5-3 to the cluster angles φn,m,AOA=φn,AOA+cASAαm, (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 7.5-3: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m; Ray power ratio βm; Basis vector of offset angles m. 21; ICPn/(ICPn+1); 0. 1,2; ± 0.0447; 0.2029/(ICPn+1); ± 0.1134. 3,4; ± 0.1413; 0.1679/(ICPn+1); ± 0.3403. 5,6; ± 0.2492; 0.1390/(ICPn+1); ± 0.5671. 7,8; ± 0.3715; 0.1151/(ICPn+1); ± 0.7940. 9,10; ± 0.5129; 0.0952/(ICPn+1); ± 1.0208. 11,12; ± 0.6797; 0.0788/(ICPn+1); ± 1.2477. 13,14; ± 0.8844; 0.0653/(ICPn+1); ± 1.4745. 15,16; ± 1.1481; 0.0540/(ICPn+1); ± 1.7014. 17,18; ± 1.5195; 0.0447/(ICPn+1); ± 1.9282. 19,20; ± 2.1551; 0.0370/(ICPn+1); ± 2.1551. Value and Distribution of ICPn is FFS <unchanged text omitted> Hu,s,nNLOSt=PnMm=1Mβm,nFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.5-22)

Example 2)

Finally add offset angles m from Table 7.5-3 to the cluster angles φn,m,AOA=φn,AOA+cASAαm, (7.5-13) where cASA is the cluster-wise rms azimuth spread of arrival angles (cluster ASA) in Table 7.5-6. Table 7.5-3: Ray offset angles within a cluster, given for rms angle spread normalized to 1 Ray number m; Basis vector of offset angles m; Ray power ratio βm,n; Basis vector of offset angles m. 1,2; ± 0.0447; ICPn/(ICPn+1); -0.0901. 2; 1/(19ICPn+1); -0.3611. 3,4; ± 0.1413; 1/(19ICPn+1); ±0.2729. 5,6; ± 0.2492; 1/(19ICPn+1); ±0.3614. 7,8; ± 0.3715; 1/(19ICPn+1); ±0.4626. 9,10; ± 0.5129; 1/(19ICPn+1); ±05808. 11,12; ± 0.6797; 1/(19ICPn+1); ±0.7227. 13,14; ± 0.8844; 1/(19ICPn+1); ±0.9004. 15,16; ± 1.1481; 1/(19ICPn+1); ±1.1383. 17,18; ± 1.5195; 1/(19ICPn+1); ±1.4995. 19,20; ± 2.1551; 1/(19ICPn+1); ±2.2763. ICPn=b∙10-a∙τnDS FFS values of a and b for each deployment scenario <unchanged text omitted> Hu,s,nNLOSt=PnMm=1Mβm,nFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATexpjΦn,mθθκn,m-1expjΦn,mθϕκn,m-1expjΦn,mϕθexpjΦn,mϕϕ Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT∙drx,uλ0expj2πrtx,n,mT∙dtx,sλ0expj2πrrx,n,mT∙vλ0t (7.5-22)

Agreement

- Adopt the following absolute delay parameters for InH scenarios.

  - Down-select between option 1, 2 and 3.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | Option 1) -7.9 Option 2) -8.6 Option 3) for 40% of the NLOS cluster, Δτ=0; for other 60% of NLOS cluster μlgΔτ = -8.9 |
|  | σlgΔτ | Option 1) 0.3 Option 2) 0.1 Option 3) for 40% of the NLOS cluster with Δτ=0, σlgΔτ is not applicable; for other 60% of the NLOS cluster σlgΔτ = 0.7 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Working Assumption

- Intermediate angle values of the CDL channel is computed by

  - ϕn,intermediate=s* WrapTo180ϕn,model-μϕ,model,

  - where s is a scale factor chosen to change the distribution of the angles. Table X shows required scale factor for typical desired angular spread values for AOD, AOD, ZOA, and ZOD.

Table X: Scale factor values for each CDL model

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-A | 5 | 0.068 | 30 | 0.353 | 5 | 0.240 | 1 | 0.035 |
|  | 10 | 0.136 | 45 | 0.527 | 10 | 0.480 | 3 | 0.106 |
|  | 15 | 0.204 | 60 | 9.525 | 15 | 0.723 | 5 | 0.176 |
|  | 25 | 0.341 |  |  |  |  |  |  |
| CDL-B | 5 | 0.124 | 30 | 0.542 | 5 | 0.652 | 1 | 0.194 |
|  | 10 | 0.248 | 45 | 0.808 | 10 | 1.302 | 3 | 0.582 |
|  | 15 | 0.371 | 60 | 1.071 | 15 | 1.948 | 5 | 0.971 |
|  | 25 | 0.617 |  |  |  |  |  |  |
| CDL-C | 5 | 0.128 | 30 | 0.431 | 5 | 0.648 | 1 | 0.364 |
|  | 10 | 0.257 | 45 | 0.645 | 10 | 1.297 | 3 | 1.093 |
|  | 15 | 0.386 | 60 | 7.749 | 15 | 1.950 | 5 | 1.822 |
|  | 25 | 0.651 |  |  |  |  |  |  |
| CDL-D | 5 | 0.323 | 30 | 9.869 | 5 | 4.327 | 1 | 0.448 |
|  | 10 | 0.665 | 45 | N/A | 10 | 8.887 | 3 | 1.347 |
|  | 15 | 1.059 | 60 | N/A | 15 | 14.034 | 5 | 2.258 |
|  | 25 | 9.354 |  |  |  |  |  |  |
| CDL-E | 5 | 0.395 | 30 | 9.743 | 5 | 6.919 | 1 | 0.971 |
|  | 10 | 0.801 | 45 | N/A | 10 | 14.838 | 3 | 2.918 |
|  | 15 | 1.233 | 60 | N/A | 15 | 27.285 | 5 | 4.877 |
|  | 25 | 7.780 |  |  |  |  |  |  |

Note: the values are computed by formula option 2A defined in RAN1#119.

Agreement

- Further study updates to the mean angle calculation in Annex used for calculation of the mean angle values for CDL-D/E channel model

  - Potential update of the power weighted mean angle is

μϕ=argexpjϕLOSPLOS+n=1Nm=1Mexpjϕn,mPn,m

Agreement

- Discuss further on the following suggested text changes:

< Unchanged text omitted > #### 7.7.5.1 CDL extension: Scaling of angles The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to μφ,desired and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation: φn,scaled=ASdesiredASmodelφn,model-μφ,model+μφ,desired (7.7-5) in which: φn,model is the tabulated CDL ray cluster angle ASmodel is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A μφ,model is the mean angle of the tabulated CDL, calculated using the definition in Annex A μφ,desired is the desired mean angle ASdesired is the desired rms angular spread φn,scaled is the resulting scaled ray cluster angle. The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model. Example scaling values are: - AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees. - AOA spread (ASA) for each CDL model: {30, 45, 60} degrees. - ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees. - ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees. The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival. Note: The azimuth angles of ϕn,model-μϕ,model should be wrapped around to be within [-180, 180] degrees. Note: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees. Note: The resulting scaled ray cluster angle may or may not achieve the desired angle mean and the desired rms angle spread.

Agreement

- RAN1 has identified that standard multi-panel glass penetration loss model may require updates at least for 6-24 GHz frequency range. Further discuss changes.

- For standard multi-panel glass penetration loss model, study the following model changes:

  - Option 1) thickness dependent model without resonation effect

    - Lglass=α∙(2+0.2f)

    - where α=Tglass3 cm+0.41.4

    - Default reference thickness for standard multi-panel glass is 3 cm

    - If model without resonation effect is selected, add note in TR that while glass material measurement may showcase resonation effect as a function of frequency and thickness, the model abstracts away the effect.

  - Option 2) thickness dependent model with resonation effect

    - Lglass=0.08∙α∙f+sin1.05∙α∙f-7.8/α+0.2-0.3α+1.8

      - For α=3, Lglass=0.9+ 0.24∙f+sin3.15∙f-7.8/3.2

    - where α=Tglass1 cm

    - Default reference thickness for standard multi-panel glass is 3 cm

  - Option 3) thickness dependent model without resonation effect and with air interface loss

    - Lglass=a+b∙α∙f, α=TglassX cm

    - Where X is the default thickness of the glass.

    - Parameters a, b, and X are FFS.

  - Note: for all options, consider the impact to overall building exterior penetration loss from the potential changes to glass penetration loss.

Agreement

- RAN1 has identified that IRR glass penetration loss model may require updates at least for 6-24 GHz frequency range. Further discuss changes For IRR glass penetration loss model, study the following model changes:

  - Option 1) coating dependent model without resonation effect

    - LIRRglass=0.11f-0.9α2+8.3α+18

      - For α=1, LIRRglass=25.4+ 0.11f

    - where α=1 for single silver-coating; α=2 for double silver-coating; α=3 for triple silver-coating.

    - Default reference coating for standard multi-panel glass is single coating, i.e. α=1

    - Default reference thickness for IRR glass is 3 cm

    - If model without resonation effect is selected, add note in TR that while glass material measurement may showcase resonation effect as a function of frequency and thickness, the model abstracts away the effect.

  - Option 2) coating dependent model with resonation effect

    - LIRRglass=0.11f-3sin0.5f+1.36-0.9α2+8.3α+18

      - For α=1, LIRRglass=25.4+ 0.11f-3sin0.5f+1.36

    - where α=1 for single silver-coating; α=2 for double silver-coating; α=3 for triple silver-coating.

    - Default reference coating for IRR glass is single coating, i.e. α=1

  - Option 3) thickness dependent model without resonation effect and with air interface loss

    - LIRRglass=a+b∙α∙f, α=TIRRglassX cm

    - Where X is the default thickness of the glass.

    - Parameters a, b, and X are FFS.

  - Note: for all options, consider the impact to overall building exterior penetration loss from the potential changes to glass penetration loss.

  - FFS: whether and how to incorporate thickness dependent affect for the IRR model. In this context, incorporation of thickness impact is considered a minor refinement of option 1 or 2 to better match the measurement data.

Agreement

- Discuss further on potential update of concrete penetration model based on measurement provided by companies.

  - For updates of the concrete penetration loss model, further study between the following model changes:

    - Option 1) model with no separate air to interface insertion loss

      - Lconcrete=α(5+4f)

      - α=Tconcrete23 cm

      - Default reference thickness for concrete is 23 cm

    - Option 2) model with separate air to interface insertion loss

      - Lconcrete=1.4+4.6αf

      - α=Tconcrete23cm

      - Default reference thickness for concrete is 23 cm

Note: for all options, consider the impact to overall building exterior penetration loss from the potential changes to concrete penetration loss.

Agreement

- Discuss further on potential introduction of new material mode “cinder block” penetration model based on measurement provided by companies for concrete based walls.

Agreement

- Discuss further on potential update of wood penetration loss model based on measurement provided by companies. The following is an example of a potential update:

  - Option 1) thickness dependent model without air interface loss

    - Lwood=α(4.85+0.12f)

    - α=Twood6cm

    - Default reference thickness for wood is 6 cm

  - Option 2) thickness dependent model with air interface loss

    - Lwood=a+b∙α∙f

    - α=TwoodX cm

    - Where X is the default thickness of the wood.

    - Parameters a, b, and X are FFS.

Agreement

Discuss further on whether to introduce additional model to handle LOS probability impact from vegetation for SMa and details of the additional modeling component.

For suburban scenario, the down-select among the following LOS probability

- Option 1)

PrLOS=exp(-d2D-out-10exp(0.02044hBS+5.746))

- Option 2)

PLOS=1, d2D≤10exp-d2Dkcommercialexp-d2Dkresidentialexp-d2Dkvegetation, d2D>10

kcommercial=-dcommercialln1-rcommericial∙hBS-hUThcommercial-hUT

kresidential=-dresidentialln1-rresidential∙hBS-hUThresidential-hUT

kvegetation=-dresidentialln1-rresidential∙hBS-hUThresidential-hUT

dcommercial,residential,vegetation=clutter bin size

rcommercial,residential,vegetation=clutter density

hcommercial,residential,vegetation=clutter height

| Parameter |  |
| --- | --- |
| dcommercial,residential,vegetation | 30 m |
| hcommercial | 20 m |
| hresidential | 8 m |
| hvegetation | 15 m |
| rcommercial | 2% |
| rresidential | 18% |
| rvegetation | 0% (no vegetation), 10% (sparse vegetation), or 20% (dense vegetation) |

- Option 3)

PLOS =1,  &d2D-out≤10exp(-d2D-out-10200),  &d2D-out>10

Working Assumption

- For parameters with multiple options, choose from one of the options in meeting RAN1#120-bis.

- For information: O2I are barrowed from TR38.901 UMa O2I, LOS and NLOS parameters are barrowed from ITU M.2135 SMa if values are available. For colored values listed, values are based on measurement data from companies.

| Scenarios |  | SMa |  |  | Reference – ITU M.2135 SMa |  |  | TR38.901 UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | µlgDS | –7.23 | Option 1) -7.12 Option 2) -7.2 Option 3) -6.98 | -6.62 | –7.23 | –7.12 | N/A | -6.955 - 0.0963 log10(fc) | -6.28 - 0.204 log10(fc) | -6.62 |
|  | σlgDS | 0.38 | Option 1) 0.33 Option 2) 0.58 Option 3) 0.74 | 0.32 | 0.38 | 0.33 | N/A | 0.66 | 0.39 | 0.32 |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 0.55 | 0.55 | 0.55 | 0.78 | 0.90 | N/A | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) | 1.25 |
|  | σlgASD | 0.25 | 0.25 | 0.25 | 0.12 | 0.36 | N/A | 0.28 | 0.28 | 0.42 |
| AOA spread (ASA) lgASA=log10(ASA/1) | µlgASA | Option 1) 1.48 Option 2) 1.75 | Option1) 1.65 Option 2) 1.74 Option 3) 1.27 | 1.76 | 1.48 | 1.65 | N/A | 1.81 | 2.08 - 0.27 log10(fc) | 1.76 |
|  | σlgASA | Option 1) 0.20 Option 2) 0.04 | Option 1) 0.25 Option 2) 0.24 Option 3) 0.86 | 0.16 | 0.20 | 0.25 | N/A | 0.20 | 0.11 | 0.16 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 1.31 | Option 1) 1.32 Option 2) -0.388 | 1.01 | N/A | N/A | N/A | 0.95 | -0.3236 log10(fc) + 1.512 | 1.01 |
|  | σlgZSA | 0.02 | Option 1) 0.08 Option 2) 1.17 | 0.43 | N/A | N/A | N/A | 0.16 | 0.16 | 0.43 |
| Shadow fading (SF) [dB] | σSF | 4 | 8 | 7 | 4 | 8 | N/A | See Table 7.4.1-1 | See Table 7.4.1-1 | 7 |
| K-factor (K) [dB] | µK | 9 | N/A | N/A | 9 | N/A | N/A | 9 | N/A | N/A |
|  | σK | 7 | N/A | N/A | 7 | N/A | N/A | 3.5 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0.4 | 0 | 0 | N/A | 0.4 | 0.4 | 0.4 |
|  | ASA vs DS | 0.8 | Option 1) 0.7 Option 2) 0.5 | 0.4 | 0.8 | 0.7 | N/A | 0.8 | 0.6 | 0.4 |
|  | ASA vs SF | –0.5 | Option 1) 0 Option 2) -0.25 | 0 | –0.5 | 0 | N/A | -0.5 | 0 | 0 |
|  | ASD vs SF | –0.5 | -0.4 | 0.2 | –0.5 | –0.4 | N/A | -0.5 | -0.6 | 0.2 |
|  | DS vs SF | –0.6 | Option 1) -0.4 Option 2) -0.13 | -0.5 | –0.6 | –0.4 | N/A | -0.4 | -0.4 | -0.5 |
|  | ASD vs ASA | 0 | 0 | 0 | 0 | 0 | N/A | 0 | 0.4 | 0 |
|  | ASD vs K | 0 | N/A | N/A | 0 | N/A | N/A | 0 | N/A | N/A |
|  | ASA vs K | 0 | N/A | N/A | 0 | N/A | N/A | -0.2 | N/A | N/A |
|  | DS vs K | 0 | N/A | N/A | 0 | N/A | N/A | -0.4 | N/A | N/A |
|  | SF vs K | 0 | N/A | N/A | 0 | N/A | N/A | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | FFS | FFS | 0 | N/A | N/A | N/A | 0 | 0 | 0 |
|  | ZSA vs SF | FFS | -0.47 | 0 | N/A | N/A | N/A | -0.8 | -0.4 | 0 |
|  | ZSD vs K | FFS | N/A | N/A | N/A | N/A | N/A | 0 | N/A | N/A |
|  | ZSA vs K | FFS | N/A | N/A | N/A | N/A | N/A | 0 | N/A | N/A |
|  | ZSD vs DS | FFS | -0.06 | -0.6 | N/A | N/A | N/A | -0.2 | -0.5 | -0.6 |
|  | ZSA vs DS | FFS | FFS | -0.2 | N/A | N/A | N/A | 0 | 0 | -0.2 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 | N/A | N/A | N/A | 0.5 | 0.5 | -0.2 |
|  | ZSA vs ASD | FFS | FFS | 0 | N/A | N/A | N/A | 0 | -0.1 | 0 |
|  | ZSD vs ASA | FFS | FFS | 0 | N/A | N/A | N/A | -0.3 | 0 | 0 |
|  | ZSA vs ASA | FFS | 0.36 | 0.5 | N/A | N/A | N/A | 0.4 | 0 | 0.5 |
|  | ZSD vs ZSA | FFS | FFS | 0.5 | N/A | N/A | N/A | 0 | 0 | 0.5 |
| Delay scaling parameter r |  | 2.4 | 1.5 | 2.2 | 2.4 | 1.5 | N/A | 2.5 | 2.3 | 2.2 |
| XPR [dB] | µXPR | 8 | 4 | 9 | 8 | 4 | N/A | 8 | 7 | 9 |
|  | σXPR | N/A | N/A | 5 | N/A | N/A | N/A | 4 | 3 | 5 |
| Number of clusters N |  | 15 | 14 | 12 | 15 | 14 | N/A | 12 | 20 | 12 |
| Number of rays per cluster M |  | 20 | 20 | 20 | 20 | 20 | N/A | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | N/A | N/A | 11 | N/A | N/A | N/A | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | 11 |
| Cluster ASD (cASD) in [deg] |  | Option 1) 5 Option 2) 1.5 | Option 1) 2 Option 2) 1.5 | 1.5 | 5 | 2 | N/A | 5 | 2 | 5 |
| Cluster ASA (cASA) in [deg] |  | 5 | 10 | 8 | 5 | 10 | N/A | 11 | 15 | 8 |
| Cluster ZSA (cZSA) in [deg] |  | N/A | N/A | 3 | N/A | N/A | N/A | 7 | 7 | 3 |
| Per cluster shadowing std ξ [dB] |  | 3 | 3 | 4 | 3 | 3 | N/A | 3 | 3 | 4 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 10 | 6 | 40 | N/A | 30 | 40 | 10 |
|  | ASD | 15 | 30 | 11 | 15 | 30 | N/A | 18 | 50 | 11 |
|  | ASA | 20 | 30 | 17 | 20 | 30 | N/A | 15 | 50 | 17 |
|  | SF | 40 | 50 | 7 | 40 | 50 | N/A | 37 | 50 | 7 |
|  | K | 10 | N/A | N/A | 10 | N/A | N/A | 12 | N/A | N/A |
|  | ZSA | N/A | N/A | 25 | N/A | N/A | N/A | 15 | 50 | 25 |
|  | ZSD | N/A | N/A | 25 | N/A | N/A | N/A | 15 | 50 | 25 |

| Scenarios |  | SMa LOS | SMa NLOS | SMa O2I | For reference - UMa LOS | For reference - UMa NLOS | For reference – RMa O2I |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | µlgZSD | Option 1) 0.14 Option 2) 0.55 | Option 1) 0.14 Option 2) 0.55 | 0.55 | max[-0.5, -2.1(d2D/1000) -0.01 (hUT - 1.5)+0.75] | max[-0.5, -2.1(d2D/1000) -0.01(hUT - 1.5)+0.9] | max[-1, -0.19(d2D/1000) -0.01(hUT - 1.5) + 0.28] |
|  | σlgZSD | Option 1) 0.16 Option 2) 0.25 | Option 1) 0.16 Option 2) 0.25 | 0.25 | 0.40 | 0.49 | 0.30 |
| ZOD offset | µoffset,ZOD | FFS | FFS | FFS | 0 | e(fc)-10^{a(fc) log10(max(b(fc), d2D)) +c(fc) -0.07(hUT-1.5)} | arctan((35 - 3.5)/d2D) -arctan((35 - 1.5)/d2D) |
| Note: For NLOS ZOD offset: a(fc) = FFS; b(fc) = FFS; c(fc) = FFS; e(fc) = FFS. |  |  |  |  | Note: For NLOS ZOD offset: a(fc) = 0.208log10(fc)- 0.782; b(fc) = 25; c(fc) = -0.13log10(fc)+2.03; e(fc) = 7.66log10(fc)-5.96. |  |  |

Agreement

Further study on introduction of a loss outdoor-to-indoor (O2I) building penetration loss model for SMa:

- PLtw=5-10log100.3⋅10-Lglass10+0.7⋅10-Lwood10

- Consider the impact to overall building exterior penetration loss

Working Assumption

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters | Shadow fading std [dB] | default values | Applicability range |
| --- | --- | --- | --- | --- | --- |
| SMa | LOS | PL1=20log1040πdfc3+min0.03h1.72,10log10d-min0.044h1.72, 14.77+0.002log10hd, h = 10 m, | σ=4 σ=6 | 10 m < d< dBP dBP < d < 5000 m | 0.8 ≤fc≤ 37 GHz 25 m < hBS < 35 m 1 m < hUT < 14 m |
|  | NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 h = 10 m, W = 10 m FFS potential changes: PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc+10⋅log10minfc,10-10⋅log1010-3.2log1011.75hUT2-4.97 h = 10 m, W = 10 m | σ=8 | 10 m < dBP < 5000 m |  |

Corresponding Working Assumption (made in RAN1#119) is dropped.

Agreement

For UE antenna modeling of handheld devices, at least support directional antenna radiation pattern for calibration purposes.

- Further study the following directional radiation pattern parameters

| For directional radiation pattern |  |  |
| --- | --- | --- |
| Vertical Radiation Pattern | Horizontal | Max Gain |
| θ3dB=90° SLAV= 25 dB | ϕ3dB=120°~180° Am= 25 dB |  |
| θ3dB=115° SLAV=25 dB | ϕ3dB=85° Am=25 dB |  |
| θ3dB=131° SLAV=20 dB | ϕ3dB=115° Am=20 | 4.5 ~ 6.5 dBi |
| θ3dB=90° SLAV=25 dB | ϕ3dB=90° Am=25 | ≤ 5 dBi |
| θ3dB=90~120° SLAV=15~25 dB | ϕ3dB=90°~120° Am=15~25 dB | 0 ~ 5 dBi |
| θ3dB=131° SLAV=15 dB | ϕ3dB=131° Am=15 dB | 5 dBi |

Agreement

RAN1 has identified that angular spread for following scenarios require necessary updates at least for 6-24 GHz frequency range. Further discuss necessary changes. Note: For UMa ASD, weak dependency to frequency was observed and the potential necessary changes may include changes to values across wide frequency ranges.

- UMi LOS ASA

- UMi NLOS ASA

- UMa LOS ASD

- UMa LOS ASA

- UMa NLOS ASD

- UMa NLOS ASA

- UMa NLOS ZSA

Agreement

- Conclude in RAN1 #120bis among the following:

- Alt 1)

  - RAN1 has identified that delay spread for UMi LOS/NLOS and UMa LOS/NLOS scenario is smaller compared to delay spread in the TR at least for 6-24 GHz frequency range. Further discuss, necessary changes for UMi LOS/NLOS and UMa LOS/NLOS

  - Proponent companies to provide detailed information on potentially necessary changes to DS for scenarios in question.

    - Data samples for discussion on necessary changes for UMi LOS

| Delay spread For UMi LOS | TR38.901 @ 6.5 GHz | Measured @ 6.5 GHz | TR38.901 @ 10 GHz | Measured @ 10 GHz | TR38.901 @ 13 GHz | Measured @ 13 GHz | TR38.901 @ 13.5 GHz | Measured @ 13.5 GHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| μDS (log_10⁡〖DS/1s〗 ) | -7.35 | -8.16 | -7.39 | -7.47 | -7.42 | -7.85 | -7.41 | -8.19 |
| σDS | 0.38 | 0.83 | 0.38 | 0.28 | 0.38 | 0.54 | 0.38 | 0.91 |

- Alt 2)

  - For the following scenarios, there is no consensus to update delay spread models due to lack of consistent and significant observed difference between model and measurements.

    - UMi LOS/NLOS and UMa LOS/NLOS

Agreement

- Conclude in RAN1 #120bis among the following:

- Alt 1)

  - RAN1 has identified that number of clusters spread for [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS] scenario is smaller compared to delay spread in the TR at least for 6-24 GHz frequency range. Further discuss, necessary changes for [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS].

    - RAN1 to determine which scenarios among [InH LOS/NLOS, UMi LOS/NLOS, and UMa LOS/NLOS] requires necessary changes.

    - Data samples for discussion on necessary changes

| Scenario | InH @10 GHz |  |  |  | Umi @10 GHz |  |  |  | UMa @6.5 GHz |  |  |  | UMa @13 GHz |  |  |  | UMa @15GHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  | TR 38.901 |  | Measurement |  |
|  | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Number of clusters | 15 | 19 | 10 | 11 | 12 | 19 | 5 | 7 | 12 | 20 | 13 | 17 | 12 | 20 | 11 | 15 | 12 | 20 | 7 | 13 |

  - Proponent companies to provide detailed information on potentially necessary changes to number of clusters for scenarios in question.

- Alt 2)

  - For the following scenarios, while sources have observed smaller number of clusters for various deployment scenario from measurement taken for 6 – 24 GHz frequency range, due to impact on frequency continuity outside 6 – 24 GHz frequency range, RAN1 concludes that there is no census to update number of clusters for all existing scenarios.

Agreement

- Conclude in RAN1 #120bis to either (1) introduce an optional modeling component for polarization variability for each cluster for NLOS component of the channel or (2) no consensus to introduce polarization variability for each cluster for NLOS component of the channel.

Agreement

The following agreement replaces conclusion made in RAN1 #120 regarding starting point for further discussion for calibration.

- For new scenarios, and if changes are made to existing scenarios, interested companies to perform large scale calibration based on the following simulation assumptions. Use the following updates to TR38.901 as baseline assumption for calibration.

- Additional calibration parameters can be found in Table 7.8-1A. It is assumed that parameters from Table 7.8-1 (of TR38.901) is used if unspecified by the additional calibration parameters in Table 7.8-1A. In addition, calibration of UMa and UMi-Street Canyon at 6 GHz carrier frequency using simulation assumptions in Table 7.8-1 with updated channel modeling is part of the additional calibration.

Table 7.8-1A: Simulation assumptions for large scale calibration

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, SMa |
| Carrier Frequency | 7 GHz |
| BS antenna electrical downtilting | [95] degrees for SMa |
| BS antenna configurations | Mg = Ng = 1; (M,N,P) = (10, 1, 1), dV = 0.5λ FFS: additional BS antenna configurations |
| BS port mapping | Mp = 1, Np = 1, i.e., 10 elements are mapped to a single port Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization FFS: additional BS antenna configurations |
| BS Tx power | 49 dBm for SMa |
| Bandwidth | [20] MHz |
| UT distribution | For SMa, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type. |
| O2I penetration loss | For SMa, [low-loss model] |
| Wrapping method | For UMa, UMi-Street Canyon, SMa: - geographical distance based wrapping (mandatory) - radio distance (optional) |

- Note 1: Parameters in [ ] are subject for further check and are intent as tentative values for calibration checks for companies.

- Note 2: Calibration of UMa and UMi in 6 and 7 GHz is subject to any changes to UMa and UMi channel modeling parameters. In case no changes are concluded, UMa and UMi in 6 and 7 GHz can be removed from calibration.

Agreement

The following agreement replaces conclusion made in RAN1 #120 regarding starting point for further discussion for calibration.

- For new scenarios, and if changes are made to existing scenarios, interested companies to perform full calibration based on the following simulation assumptions. Use the following updates to TR38.901 as baseline assumption for calibration.

- Additional full calibration parameters can be found in Table 7.8-2A. It is assumed that parameters from Table 7.8-2 (of TR38.901) is used if unspecified by the additional full calibration parameters in Table 7.8-2A. In addition, calibration of UMa and UMi-Street Canyon at 6 GHz carrier frequency using simulation assumptions in Table 7.8-2 with updated channel modeling is part of the additional calibration. For calibration of UMa and UMi-Street Canyon at 6 GHz, the following is additionally assumed.

  - SCS of 15 kHz

  - UT attachment is based on RSRP (formula) from BS port 0

  - BS antenna configuration 1 and 2 both apply

  - UT antenna configuration, pattern, and polarization modeling is labeled as UT antenna config A

Table 7.8-2A: Simulation assumptions for full calibration

| Parameter | Values |
| --- | --- |
| Scenarios | UMa, UMi-Street Canyon, SMa |
| Carrier Frequency | 7 GHz FFS: additional frequency |
| BS antenna electrical downtilting | [95] degrees for SMa |
| BS antenna configurations | Config 3 for 7 GHz: Mg = Ng = 1, M = 8, N = 16, P = 2, dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated (optional) Config 4 for UMa at 7 GHz: Mg = Ng = 1, M = 64, N = 16, P = 2, Mg = 1, Ng = 1, dH = dV = 0.5λ … calibration metrics 1), 2), 3), 4) are calibrated |
| BS Polarized antenna modelling | Model-2 in Clause 7.3.2 of TR38.901 |
| BS port mapping | Config 3 for 7 GHz: Mp = 8, Np = 16, each antenna element is mapped to one port (optional) Config 4 for UMa at 7 GHz: Mp = 16, Np = 16 Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Tx power | 49 dBm for SMa |
| Bandwidth | [20] MHz for 7 GHz (optional) 200 MHz for 7 GHz |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | For SMA, 20% of UT outdoor, 80% of UT indoor. Among indoor UT, 90% of indoor UT are within residential buildings, and 10% of indoor UT in commercial buildings. Indoor UTs are uniformly distributed across all floors for a building type. |
| UT antenna configurations | Config B for 7 GHz: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 FFS: (optional) Config C for [7] GHz: 16 antenna port with dual polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| UT antenna pattern | Config B for 7 GHz: Based on directional antenna for UT described in Clause 7.3 FFS: (optional) Config C for [7] GHz: [Based on directional antenna for UT described in Clause 7.3] |
| UT Polarized antenna modelling | Config B for 7 GHz: Based on directional antenna for UT described in Clause 7.3 FFS: (optional) Config C for [7] GHz: [Model-2 in Clause 7.3.2 of TR38.901] |
| O2I penetration loss | For SMa, [low-loss model] |
| SCS assumption | 30 kHz |

- Note 1: Parameters in [ ] are subject for further check and are intent as tentative values for calibration checks for companies.

- Note 2: Calibration of UMa and UMi in 6 and 7 GHz is subject to any changes to UMa and UMi channel modeling parameters. In case no changes are concluded, UMa and UMi in 6 and 7 GHz can be removed from calibration.

### RAN1 #120bis (April-2025)

Conclusion

- Based on measurement data provided, RAN1 concludes that following delay spread parameters for 6 – 24 GHz frequency range are validated and no updates to TR are needed.

  - InH LOS and NLOS

Agreement

- Update the UMi LOS and NLOS delay spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -0.24 log10(1+ fc) - 7.14 | -0.24 log10(1+ fc) - 6.83 | -0.18 log10(1+ f) - 7.28 | -0.22 log10(1+ f) – 6.87 |
|  | lgDS | 0.38 | 0.16 log10(1+ fc) + 0.28 | 0.39 | 0.19 * log10(1+ f) + 0.22 |

Agreement

- Update the UMa LOS and NLOS delay spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -6.955 - 0.0963 log10(fc) | -6.28 - 0.204 log10(fc) | -0.0794 * log10(f) – 7.067 | -0.134 * log10(f) – 6.47 |
|  | lgDS | 0.66 | 0.39 | 0.026 * log10(f) + 0.57 | 0.39 |

Agreement

- Update the UMi LOS and NLOS AOA spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | -0.08 log10(1+ fc) + 1.73 | -0.08 log10(1+ fc) + 1.81 | -0.07 * log10(1+ f) + 1.66 | -0.07 * log10(1+ f) + 1.76 |
|  | lgASA | 0.014 log10(1+ fc) + 0.28 | 0.05 log10(1+ fc) + 0.3 | 0.021 * log10(1+ f) + 0.26 | 0.05 * log10(1+ f) + 0.27 |

Agreement

- Update the UMa LOS and NLOS AOA spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOA spread (ASA) lgASA=log10(ASA/1) | lgASA | 1.81 | 2.08 - 0.27 log10(fc) | 1.76 | -0.25 * log10(f) + 2.04 |
|  | lgASA | 0.20 | 0.11 | 0.19 | -0.03 * log10(f) + 0.17 |

Working Assumption

- Adopt the following absolute delay parameters for RMa scenarios.

| Scenarios |  | RMa |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -8.33 |
|  | σlgΔτ | 0.26 |
| Correlation distance in the horizontal plane [m] |  | 50 |

Agreement

- Defines upper bound of absolute delay in indoor office scenario as 2L/c where L is the largest dimension of the office hall.

- No upper bound of absolute delay in UMi, UMa, and RMa scenario.

Conclusion

- For the following scenarios, there is no consensus to update pathloss models due to lack of consistent and significant observed difference between model and measurements.

  - UMa LOS/NLOS

Agreement

- Update the UMa LOS and NLOS AOD spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit without frequency dependency. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.06 + 0.1114 log10(fc) | 1.5 - 0.1144 log10(fc) | 0.95 | 1.12 |
|  | lgASD | 0.28 | 0.28 | 0.31 | 0.42 |

Agreement

- For UE antenna modeling, support the following direction antenna radiation pattern for calibration purposes.

| For directional radiation pattern |  |  |
| --- | --- | --- |
| Vertical Radiation Pattern | Horizontal | Max Gain |
| θ3dB=125° SLAV= 22.5 dB | ϕ3dB=125° Am= 22.5 dB | 5.3 dBi |

Agreement

- Reference UE orientation vector of the handheld UE is perpendicular to the plane of the flat UE handheld device, and reference point for near field phase calculation of the UE is assumed to be the center of the plane of the UE handheld.

Agreement

- For cases when a candidate antenna placement location is used for two distinct antenna polarization field pattern:

  - Reference radiation pattern of the UE antenna model is

    - For first antenna field pattern: Fθ″″θ″,ϕ″=A″θ″,ϕ″and Fϕ″″θ″,ϕ″=0.

    - For second antenna field pattern: Fθ″″θ″,ϕ″=0 and Fϕ″″θ″,ϕ″=A″θ″,ϕ″.

Agreement

Update and agree to the following the SMa LOS pathloss working assumption:

| LOS | For 10m ≤ d < dBP, PL1=20log1040πdfc3+min0.03h1.72,10log10d-min0.044h1.72, 14.77+0.002log10hd For dBP ≤ d < 5000m, PL2=PL1dBP+40log10ddBP dBP=2πhBShUTfc/c, fc is the center frequency in Hz, c=3×108m/s | For 10m ≤ d < dBP, σSF=4 For dBP ≤ d < 5000m, σSF=6 |
| --- | --- | --- |

Agreement

Update and agree to the following the SMa NLOS pathloss:

| NLOS | PL=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10d-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.8 ≤fc≤ 37 GHz 10 m < dBP < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |
| --- | --- | --- | --- |

Agreement

For suburban scenario, adopt the LOS probability

PLOS=1, d2D≤10exp-d2Dkcommercialexp-d2Dkresidentialexp-d2Dkvegetation, d2D>10

kcommercial=-dcommercialln1-rcommericial∙hBS-hUThcommercial-hUT

kresidential=-dresidentialln1-rresidential∙hBS-hUTmax⁡(hresidential-hUT ,0)

kvegetation=-dvegetationln1-rvegetation∙hBS-hUThvegetation-hUT

dcommercial,residential,vegetation=clutter bin size

rcommercial,residential,vegetation=clutter density

hcommercial,residential,vegetation=clutter height

| Parameter |  |
| --- | --- |
| dcommercial,residential,vegetation | 30 m |
| hcommercial | 20 m |
| hresidential | 8 m |
| hvegetation | 15 m |
| rcommercial | 2% |
| rresidential | 18% |
| rvegetation | 0% (no vegetation), 10% (sparse vegetation), or 20% (dense vegetation) |

Agreement

- Update the UMa NLOS ZOA spread as follows:

  - Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.

  - Note: Each group is given equal weightage.

| Scenarios |  | TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | lgZSA | 0.95 | -0.3236 log10(fc) + 1.512 |  | -0.2856 log10(fc) + 1.445 |
|  | lgZSA | 0.16 | 0.16 |  | 0.17 |

Agreement

- Adopt the following absolute delay parameters for InH scenarios.

Table 7.6.9-1: Parameters for the absolute time of arrival model

| Scenarios |  | InH |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | -8.6 |
|  | σlgΔτ | 0.1 |
| Correlation distance in the horizontal plane [m] |  | 10 |

Agreement

Adopt the following correlation distance for SMa spatial consistency

Table 7.6.3.1-2 Correlation distance for spatial consistency

| Correlation distance in [m] | SMa |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS | O2I |
| Cluster and ray specific random variables | 40 | 50 | 15 |
| LOS/NLOS state | 50 |  |  |
| Indoor/outdoor state | 50 |  |  |

Agreement

Update SMa description as follows:

- In suburban macro-cells base stations are located above the rooftops and clutter (e.g. vegetation) to allow wide area coverage, and mobile stations are outdoors at street level. Buildings are typically low residential detached houses with one or two floors, or blocks of apartments/condos or commercial buildings with a few floors. Occasional open areas such as parks or playgrounds between the houses make the environment rather open. Streets do not form urban-like regular strict grid structure. In suburban areas, vegetation is more prevalent than in urban areas with a high variability of foliage density across different suburban areas.

Agreement

For suburban scenario, adopt the following assumptions for calibration purposes:

- ISD = 1299m and 1732m

- Note: For evaluation in study items/work items, if needed, admission control policies may be considered in conjunction with the ISD choice to address out of coverage UEs.

Agreement:

- For standard multi-panel glass penetration loss model, clarify that reference thickness is 3 cm. no changes to the model.

- For IRR glass penetration loss model, update the model equation as follows:

  - LIRRglass=25.4+ 0.11f

  - Single coating for IRR glass is assumed.

- For concrete penetration loss model, clarify that reference thickness is 23 cm. no changes to the model.

- For wood penetration loss model, clarify that reference thickness is 3.3cm. no changes to the model.

- For plywood penetration loss model:

  - Lplywood=1.03 + 0.17f

  - Reference thickness is 1.27 cm

  - Note: plywood penetration loss model is intended to be used for SMa penetration loss modeling.

Agreement

To potentially resolve the UMi/UMa pathloss convergence beyond breakpoint distance, RAN1 to further discuss following options:

- Option 1) Set hE to 0 m

- Option 2) Applying only PL1 when performing link analysis

- Option 3) add note that provide information on pathloss convergence phenomena beyond the breakpoint distance across different frequencies.

- Other options are not precluded.

Working assumption

Proposed Working assumption

- Update the UMa LOS, NLOS, and O2I Cluster AOD spread as follows:

  - Note: the update parameter for LOS and NLOS was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit. If a group has fewer data points, higher weight per data point is calculated. All points within a group have same weight. Sum of weights for all groups is equal to 1.  Each group is given equal weightage.

  - Note: the update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz.

| Scenarios | TR 38.901 UMa |  |  | Updated UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| Cluster ASD (CASD) in [deg] | 5 | 2 | 5 | 3.74 | 1.82 | 1.82 |

Working assumption

Proposed Working assumption

- Update the UMa O2I AOD spread as follows:

  - Note: the update parameters were generated from scaling of updated NLOS UMa O2I value with the ratio of NLOS UMa NLOS and O2I measurement data fit from (current) Rel-19 SI at 3.7 GHz.

  - Note: UMa O2I ASD values of Rel-14 TR38.901 used values from UMa O2I ASD values of TR36.873. UMa O2I ASD values of TR36.873 used UMi O2I ASD values from ITU-RM.2135-1. IMT-2020 modeling used the same values from TR36.873. TR25.996 does not contain UMa O2I values. The UMi O2I ASD parameters were derived from Winner II report.

| Scenarios |  | TR 38.901 UMa | Updated UMa |
| --- | --- | --- | --- |
|  |  | O2I | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1) | lgASD | 1.25 | 0.84 |
|  | lgASD | 0.42 | 0.42 |

Conclusion

- RAN1 to continue study the number of clusters for InH, UMi, and UMa scenarios, and intra-cluster power angular profile modeling for all scenarios.

  - Study to further consider distribution of the number of clusters, resolvable clusters and subpaths of the measurements.

Conclusion

- Further study on introduction of polarization variability for each rays of cluster for NLOS component of the channel as an optional modeling component.

  - Polarization variability is introduced by amplitude scaling parameter η for XPR matrix as follows:

  - Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATηn.m.θθexpjΦn,mθθηn,m,θϕκn,m-1expjΦn,mθϕηn,m,ϕθκn,m-1expjΦn,mϕθηn,m,ϕϕexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT⋅drx,uλ0expj2πrtx,n,mT⋅dtx,sλ0expj2πrrx,n,mT⋅vλ0t

  - The study should include distribution of the amplitude scaling parameters, power normalization of the amplitude scaling.

Agreement

Adopt the following equations for scaling of angles for CDL channel model. (This updates existing agreements and working assumption).

- Option 0) Scaling applies to both NLOS and LOS angles for all cluster paths, Mean Angle and AS include both NLOS and LOS cluster paths but not subpaths

  - ϕn,m,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired+cASA/ASD/ZSA/ZSDαm

  - ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desired

  - ϕn,intermediate=s∙WrapTo180ϕn,model-μϕ,model

  - ϕLOS,intermediate=s∙WrapTo180ϕLOS,model-μϕ,model

  - AS=-2lnexpjφLOSPLOS+n=1NexpjφnPnPLOS+n=1NPn

  - μϕ,model=argexpjϕLOS,modelPLOS+n=1Nexpjϕn,modelPn

  - μϕ,intermediate=argexpjϕLOS,intermediatePLOS+n=1Nexpjϕn,intermediatePn

Table X: Example Scale factor values for each CDL model

| CDL Type | Desired AOD Spread（°） | Scale Factor (AOD) | Desired AOA Spread（°） | Scale Factor (AOA) | Desired ZOA Spread（°） | Scale Factor (ZOA) | Desired ZOD Spread（°） | Scale Factor (ZOD) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CDL-A | 5 | 0.0680 | 30 | 0.3531 | 5 | 0.0352 | 1 | 0.0352 |
|  | 10 | 0.1360 | 45 | 0.5268 | 10 | 0.1056 | 3 | 0.1056 |
|  | 15 | 0.2041 | 60 | 0.6981 | 15 | 0.1761 | 5 | 0.1761 |
|  | 25 | 0.3405 |  |  |  |  |  |  |
| CDL-B | 5 | 0.1238 | 30 | 0.5417 | 5 | 0.1940 | 1 | 0.1940 |
|  | 10 | 0.2475 | 45 | 0.8081 | 10 | 0.5822 | 3 | 0.5822 |
|  | 15 | 0.3710 | 60 | 1.0709 | 15 | 0.9705 | 5 | 0.9705 |
|  | 25 | 0.6168 |  |  |  |  |  |  |
| CDL-C | 5 | 0.1281 | 30 | 0.4307 | 5 | 0.3643 | 1 | 0.3643 |
|  | 10 | 0.2568 | 45 | 0.6447 | 10 | 1.0929 | 3 | 1.0929 |
|  | 15 | 0.3864 | 60 | 0.8585 | 15 | 1.8219 | 5 | 1.8219 |
|  | 25 | 0.6513 |  |  |  |  |  |  |
| CDL-D | 5 | 0.3231 | 30 | 9.8888 | 5 | 0.4477 | 1 | 0.4477 |
|  | 10 | 0.6652 | 45 | N/A | 10 | 1.3469 | 3 | 1.3469 |
|  | 15 | 1.0594 | 60 | N/A | 15 | 2.2579 | 5 | 2.2579 |
|  | 25 | 5.8637 |  |  |  |  |  |  |
| CDL-E | 5 | 0.3950 | 30 | 2.9733 | 5 | 0.9714 | 1 | 0.9714 |
|  | 10 | 0.8009 | 45 | N/A | 10 | 2.9180 | 3 | 2.9180 |
|  | 15 | 1.2330 | 60 | N/A | 15 | 4.8774 | 5 | 4.8774 |
|  | 25 | 2.3627 |  |  |  |  |  |  |

Note: the values are computed by

s=minx≥0{x: ASx=ASdesired}.	(A-3)

ASx=-2lnexpjx∙WrapTo180(φLOS-μϕ,model)PLOS+n=1Nexpjx∙WrapTo180ϕn,model-μϕ,modelPnPLOS+n=1NPn	(A-4)

Working Assumption

- SMa O2I will use the same values as SMa NLOS

| Scenarios |  | SMa (7 – 24 GHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | µlgDS | –7.23 | -7.12 | Copy NLOS |
|  | σlgDS | 0.38 | 0.33 | Copy NLOS |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 0.55 | 0.55 | 0.55 |
|  | σlgASD | 0.25 | 0.25 | 0.25 |
| AOA spread (ASA) lgASA=log10(ASA/1) | µlgASA | 1.07 | 1.39 | Copy NLOS |
|  | σlgASA | 0.13 | 0.34 | Copy NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 0.95 | 1.14 | Copy NLOS |
|  | σlgZSA | 0.16 | 0.16 | Copy NLOS |
| Shadow fading (SF) [dB] | σSF | 4 | 8 | 8 |
| K-factor (K) [dB] | µK | 9 | N/A | N/A |
|  | σK | 7 | N/A | N/A |
| Cross-Correlations | ASD vs DS | 0 | 0 | 0 |
|  | ASA vs DS | 0.8 | 0.7 | Copy NLOS |
|  | ASA vs SF | –0.5 | 0 | Copy NLOS |
|  | ASD vs SF | –0.5 | -0.4 | -0.4 |
|  | DS vs SF | –0.6 | -0.13 | Copy NLOS |
|  | ASD vs ASA | 0 | 0 | 0 |
|  | ASD vs K | 0 | N/A | N/A |
|  | ASA vs K | 0 | N/A | N/A |
|  | DS vs K | 0 | N/A | N/A |
|  | SF vs K | 0 | N/A | N/A |
| Cross-Correlations 1) | ZSD vs SF | 0 | 0 | 0 |
|  | ZSA vs SF | -0.8 | -0.47 | -0.47 |
|  | ZSD vs K | 0 | N/A | N/A |
|  | ZSA vs K | 0 | N/A | N/A |
|  | ZSD vs DS | 0.5 | -0.5 | -0.5 |
|  | ZSA vs DS | 0 | 0 | 0 |
|  | ZSD vs ASD | 0.5 | 0.5 | 0.5 |
|  | ZSA vs ASD | 0 | -0.1 | -0.1 |
|  | ZSD vs ASA | -0.3 | 0 | 0 |
|  | ZSA vs ASA | 0.4 | 0.36 | 0.36 |
|  | ZSD vs ZSA | 0 | 0 | 0 |
| Delay scaling parameter r |  | 2.4 | 1.5 | 1.5 |
| XPR [dB] | µXPR | 8 | 4 | 4 |
|  | σXPR | 4 | 3 | 5 3 |
| Number of clusters N |  | 15 | 14 | 14 |
| Number of rays per cluster M |  | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) 11 |
| Cluster ASD (cASD) in [deg] |  | 1.5 | 1.5 | Copy NLOS |
| Cluster ASA (cASA) in [deg] |  | 5 | 10 | 10 |
| Cluster ZSA (cZSA) in [deg] |  | 7 | 7 | 7 |
| Per cluster shadowing std ξ [dB] |  | 3 | 3 | 4 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 40 |
|  | ASD | 15 | 30 | 30 |
|  | ASA | 20 | 30 | 30 |
|  | SF | 40 | 50 | 50 |
|  | K | 10 | N/A | N/A |
|  | ZSA | 15 | 50 | 50 |
|  | ZSD | 15 | 50 | 50 |

Agreement

For suburban scenario, adopt the following ZSD parameters as updated working assumption:

- Values in [ ] are working assumption

| Scenarios |  | SMa LOS (7 – 24 GHz) | SMa NLOS (7 – 24 GHz) | SMa O2I (7 – 24 GHz) |
| --- | --- | --- | --- | --- |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1) | µlgZSD | 0.14 | 0.14 | 0.14 |
|  | σlgZSD | 0.16 | 0.16 | 0.16 |
| ZOD offset | µoffset,ZOD | 0 | atanhBS-hcld2D-atanhBS-hUTd2DhUT<hcl0hUT≥hcl hcl= [3.5] |  |

Working Assumption

Introduce new penetration loss outdoor-to-indoor (O2I) building penetration loss model applicable for SMa:

- Values in [ ] are working assumption

|  | Path loss through external wall: PLtw in [dB] | Indoor loss: PLin in [dB] | Standard deviation σp in [dB] |
| --- | --- | --- | --- |
| Low-loss A model | 5-10log100.3⋅10-Lglass10+0.7⋅10-Lplywood10 | 0.5 d2D-in | 4.4 |

Agreement

Adopt the following absolute time of arrival parameters for SMa

- Values in [ ] are working assumption

| Scenarios |  | SMa |
| --- | --- | --- |
| lgΔτ=log10Δτ1s | μlgΔτ | [-7.702] |
|  | σlgΔτ | [0.4] |
| Correlation distance in the horizontal plane [m] |  | [6 50] |

Agreement

- Adopt the following changes to Clause 7.2 of TR38.901.

< Unchanged parts are omitted > Table 7.2-1: Evaluation parameters for UMi-street canyon and UMa scenarios Parameters; UMi - street canyon; UMa. Cell layout; Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m); Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m1). BS antenna height hBS; 10m; 25m. UT location; Outdoor/indoor; Outdoor and indoor; Outdoor and indoor. LOS/NLOS; LOS and NLOS; LOS and NLOS. Height hUT; Same as 3D-UMi in TR36.873; Same as 3D-UMa in TR36.873. Indoor UT ratio; 80%; 80%. UT mobility (horizontal plane only); 3km/h; 3km/h. Min. BS - UT distance (2D); 10m; 35m. UT distribution (horizontal); Uniform; Uniform. NOTE 1: UMa scenarios with ISDs between 200-500m can be used for evaluations < Unchanged parts are omitted >

- Adopt the following changes to Clause 6.2 of TR38.901.

## 6.2 Scenarios of interest Brief description of the key scenarios of interest identified (see note): (1) UMi (Street canyon, open area) with O2O and O2I: This is similar to 3D-UMi scenario, where the BSs are mounted below rooftop levels of surrounding buildings. UMi open area is intended to capture real-life scenarios such as a city or station square. The width of the typical open area is in the order of 50 to 100 m. Example: [Tx height:10m, Rx height: 1.5-2.5 m, ISD: 200m] (2) UMa with O2O and O2I: This is similar to 3D-UMa scenario, where the BSs are mounted above rooftop levels of surrounding buildings. Example: [Tx height:25m, Rx height: 1.5-2.5 m, ISD: 200m, 500m]

Working Assumption

- Reference orientation of the handheld UE as follows:

  - FFS how to capture the reference orientation of the handheld UE

- For calibration of handheld UE, use the following UE rotation based on reference UE orientation

  - For calibration with blockage:

    - For one-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

    - For dual-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 90 deg,

    - For hand and head blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg,

  - For all other calibration cases:

    - ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

  - Note: UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle).

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 90 deg

Agreement

For CPE devices adopt the following device dimensions for UE antenna modeling:

- (x cm × y cm × z cm) = (0 cm × 20 cm × 20 cm)

- Total 9 candidate antenna locations in 4 corners of the vertical square plane, 4 center of the edges of the vertical square plane, and center of the vertical square plane. The figure below captures the side view on the device.

- Note: a candidate antenna location (e.g., 9) can be used for multiple antennas

Agreement

For UE antenna modeling of handheld devices, introduce optional antenna imbalance modeling as part of antenna field pattern as follows:

- No imbalance is modelled by default.

- If modelled, Randomized loss is applied per UE antenna port. Details of the randomized value and it’s distribution will not be specified in TR 38.901 as part of Rel-19 SI.

- If modelled, Randomized loss can be applied independently for the UL and DL directions.

Agreement

- Confirm the downtilt value for SMa with ISD of 1299m, and introduce downtilt value [93] for SMa with ISD of 1732m

  - Downtilt value for SMa with ISD of 1732m is working assumption

| Parameter | Values |
| --- | --- |
| BS antenna electrical downtilting | 95 degrees for SMa for ISD = 1299m [93] degrees for SMa for ISD = 1732m |

### RAN1 #121 (May-2025)

Agreement

- Update the reference thickness of plywood to 1.75 cm.

Agreement

Add the following note to UMi and UMa pathloss:

- UMi and UMa pathloss formula converges for different frequencies when distances beyond the breakpoint distance is applied.

Conclusion

Angular spread parameters for InF scenarios are not studied further due to lack of measurement data inputs.

Agreement

- Confirm the working assumption (made in RAN1#120b) for absolute delay parameters of RMa scenario.

Agreement

- Adopt the following update for scaling factor for ZOA and ZOD generation

| # clusters | 8 | 10 | 11 | 12 | 14 | 15 | 19 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CθNLOS | 0.889 | 0.957 | 1.031 | 1.104 | 1.1072 | 1.1088 | 1.184 | 1.178 | 1.282 |

Agreement

- Update the physical blocker modeling parameters in Table 7.6.4.1-2 and 7.6.3.1-4 as follows:

Table 7.6.4.1-2: Blocking region parameters.

| Blocker index (k = 1, …, 4) |  |  |  |  | r |
| --- | --- | --- | --- | --- | --- |
| InH scenario | Uniform in [0o, 360o] | Uniform in [15o, 45o] | 90o | Uniform in [5o, 15o] | 2 m |
| UMi, UMa, SMa, RMa scenarios | Uniform in [0o, 360o] | Uniform in [5o, 15o] | 90o | 5o | 10 m |

Table 7.6.4.1-4: Spatial correlation distance for different scenarios.

| Spatial correlation distance dcorr in [m] for the random variable determining the centre of the blocker | UMi/UMa/SMa/RMa |  |  | UMa |  |  | RMa |  |  | InH |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS | O2I | LOS | NLOS |
|  | 10 | 10 | 5 | 10 | 10 | 5 | 10 | 10 | 5 | 5 | 5 |

Agreement

- Confirm the working assumption (made in RAN1#120b) absolute delay parameters of SMa scenario.

Agreement

- Adopt the following updates to LSP correlation values for SMa LOS scenario:

  - Note: this is a copy of LSP correlation values for UMa LOS scenario.

|  | SF | K | DS | ASD | ASA | ZSD | ZSA |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SF |  | 0 | -0.6 -0.4 | -0.5 | -0.5 | 0 | -0.8 |
| K |  |  | 0 -0.4 | 0 | 0 -0.2 | 0 | 0 |
| DS |  |  |  | 0 0.4 | 0.8 | 0.5 -0.2 | 0 |
| ASD |  |  |  |  | 0 | 0.5 | 0 |
| ASA |  |  |  |  |  | -0.3 | 0.4 |
| ZSD |  |  |  |  |  |  | 0 |
| ZSA |  |  |  |  |  |  |  |

Agreement

- Clarify outdoor UT for SMa are in-car deployments and subject to car O2I penetration loss.

Agreement

- For SMa scenario calibration, use value of 0% vegetation for LOS probability determination.

Agreement

- Revise d2D to d2D-out for LOS probability equations of SMa scenario.

Agreement

- Update the SMa pathloss equation as follows:

| SMa | LOS | PLSMa-LOS=PL110m≤d2D<dBPPL2dBP≤d2D≤5km, PL1=20log1040πdfc3+min0.03h1.72,10log10dd3D- min0.044h1.72, 14.77+0.002log10hdd3D, see note 5 PL2=PL1dBP+40log10dd3DdBP, see note 5 | For 10 m ≤ d2D < dBP, σSF=4 For dBP ≤ d2D < 5000 m, σSF=6 | 0.50.8≤fc≤ 37 GHz 10 m ≤ d2D < dBP dBP ≤ d2D < 5000 m 25 m < hBS < 35 m h = 10 m |
| --- | --- | --- | --- | --- |
|  | NLOS | PLSMa-NLOS=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10dd3D-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.50.8 ≤fc≤ 37 GHz 10 m ≤ d2D < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |

Agreement

- For SMa, d2D-in is minimum of two independently generated uniformly distributed variables between 0 and 25 m for UT considered to be inside commercial buildings, and between 0 and 10 m for UT considered to be inside residential buildings.

Agreement

- Low Loss, High loss, and Low Loss A penetration model are applicable for SMa.

  - Low Loss A penetration model is used for SMa scenario calibration purposes.

Agreement

- Confirm Working Assumption (made in RAN1#120b) for penetration loss model (low loss A model) applicable for SMa.

Agreement

- Update delay spread parameters for SMa scenario as follows

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Delay spread (DS) lgDS=log10(DS/1s) | lgDS | -7.23 -7.42 | -7.12 -7.20 | -7.12 -7.20 |
|  | lgDS | 0.38 0.60 | 0.33 0.58 | 0.33 0.58 |

Agreement

- Confirm the working assumption for the following SMa parameters

| Scenarios |  | Sma (7 – 24 GHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| Shadow fading (SF) [dB] | σSF | See Table 7.4.1-1 | See Table 7.4.1-1 | See Table 7.4.1-1 |
| Delay scaling parameter rt |  | 2.4 | 1.5 | 1.5 |
| XPR [dB] | µXPR | 8 | 4 | 4 |
|  | σXPR | 4 | 3 | 3 |
| Number of rays per cluster M |  | 20 | 20 | 20 |
| Cluster DS (cDS) in [ns] |  | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) | max(0.25, 6.5622 -3.4084 log10(fc)) |
| Per cluster shadowing std ξ [dB] |  | 3 | 3 | 3 |
| Correlation distance in the horizontal plane [m] | DS | 6 | 40 | 40 |
|  | ASD | 15 | 30 | 30 |
|  | ASA | 20 | 30 | 30 |
|  | SF | 40 | 50 | 50 |
|  | K | 10 | N/A | N/A |
|  | ZSA | 15 | 50 | 50 |
|  | ZSD | 15 | 50 | 50 |

Agreement

- Confirm working assumption (made in RAN1#120b) for channel bandwidth for large scale and full calibration and remove brackets.

Agreement

Update UMi LOS/NLOS ASD (Mean, Std) as follows

| Scenarios |  | Original TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | -0.05 log10(1+ fc) + 1.21 | -0.23 log10(1+ fc) + 1.53 | -0.05 log10(1+ fc) + 1.21 | -0.24 * log10(1+ fc) + 1.54 |
|  | σlgASD | 0.41 | 0.11 log10(1+ fc) + 0.33 | 0.08 * log10(1+ fc) + 0.29 | 0.10 * log10(1+ fc) + 0.33 |

Update UMi LOS/NLOS ZSA (Mean, Std) as follows

| Scenarios |  | Original TR 38.901 UMi |  | Updated UMi |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | -0.10 log10(1+ fc) + 0.73 | -0.04 log10(1+ fc) + 0.92 | -0.11 log10(1+ fc) + 0.81 | -0.03 log10(1+ fc) + 0.92 |
|  | σlgZSA | -0.04 log10(1+ fc) + 0.34 | -0.07 log10(1+ fc) + 0.41 | -0.03 log10(1+ fc) + 0.29 | -0.05 log10(1+ fc) + 0.35 |

Update UMa LOS ZSA (Mean, Std) as follows

| Scenarios |  | Original TR 38.901 UMa |  | Updated UMa |  |
| --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | LOS | NLOS |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1) | µlgZSA | 0.95 | -0.3236 log10(fc) + 1.512 | 0.96 | Updated in RAN1#120b |
|  | σlgZSA | 0.16 | 0.16 | 0.15 | Updated in RAN1#120b |

Update UMa LOS/NLOS ASD (Mean, Std) as follows

| Scenarios |  | Agreement from RAN1 #120b UMa |  |  | Updated UMa |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | 0.95 | 1.12 | 0.84 | 0.92 | 1.09 | 0.58 |
|  | σlgASD | 0.31 | 0.42 | 0.42 | 0.31 | 0.44 | 0.70 |

Note: Same parameter derivation methodology as noted in RAN1 #120-bis for each parameter were re-used. For UMa ASD, the updates to previous agreement are made based on new measurement data inputs in RAN1 #121.

Agreement

Update the UMa O2I AOD spread as follows.

- Note: This would change the working assumption made in RAN1 #120-bis.

- Note: No UMa O2I data was provided in Rel-14. The UMa ASD O2I values were adopted from UMi ASD O2I in TR 38.901. The UMi ASD O2I in TR 38.901 was in turn adopted from winner II channel model. In the Winner II channel model only a single measurement at 5.25 GHz for UMi O2I ASD.

| Scenarios |  | UMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1) | µlgASD | Not part of this agreement | Not part of this agreement | 0.84 0.58 |
|  | σlgASD | Not part of this agreement | Not part of this agreement | 0.42 0.7 |

Agreement

Update the UMa LOS, NLOS, and O2I Cluster AOD spread as follows.

- Note: This would change the working assumption made in RAN1 #120-bis.

- Note: the update parameter was generated using all measurement and ray tracing data set from Rel-14 SI and (current) Rel-19 SI and dividing the data points into 3 groups, below 6 GHz, 6 to 24 GHz, and above 24 GHz, and weighting the data sets for each group to perform weighted least square curve fit OR compute weighted mean. If a group has fewer data points, higher weight per data point is calculated. All points within a group have the same weight. Sum of weights for all groups is equal to 1.  Each group is given equal weightage.

| Scenarios | UMa |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS | O2I |
| Cluster ASD (cASD) in [deg] | 3.74 3.58 | 1.82 1.8 | 1.82 1.8 |

Agreement

Proposed Observation:

- Measurement of ZSD for rural macrocell deployments from a source observed lower ZSD values compared to ZSD for RMa at shorter distances.

- Measurement of ASD for rural macrocell deployments from a source observed lower ZSD values compared to ASD for RMa.

Conclusion:

- No consensus to update RMa ASD and ZSD parameters due to lack of measurement data for each of LOS, NLOS, and O2I cases.

Agreement

- Introduce an optional Mmin parameter to potentially bound the number of rays per clusters for equation (7.6-8) of intra-cluster angular and delay spreads.

M=minmaxMtMAODMZOD,20 Mmin,Mmax (7.6-8) Mmin can be chosen based on the mean of dominant rays across all clusters for a specific frequency and deployment scenario. For example, the number of dominant rays in a cluster is defined as the minimum number of rays that contain 95% of the total cluster power, when the rays in a cluster are sorted in descending order of power. The default value of Mmin=20 is assumed.

Agreement

Add the following angle scaling formula for CDL-E and CDL-D that allow maintaining the LOS angle as the desired scaled LOS angle:

- ϕn,m,scaled=ϕn,intermediate-μϕ,intermediateϕLOS,intermediate+μϕ,desiredϕLOS,model+cASA/ASD/ZSA/ZSDαm,

- ϕLOS,scaled=ϕLOS,intermediate-μϕ,intermediate+μϕ,desiredϕLOS,model

- ϕn,intermediate=s*WrapTo180ϕn,model-μϕ,model,

- ϕLOS,intermediate=s*WrapTo180ϕLOS,model-μϕ,model,

- Note: This will be an additional (and alternative) formulation for CDL-D and CDL-E angle scaling.

Agreement

- Update ASA and ZSA parameters for SMa scenario as follows:

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOA spread (ASA) lgASA=log10(ASA/1°) | µlgASA | 1.07 1.43 | 1.39 1.63 | 1.39 1.63 |
|  | σlgASA | 0.13 0.12 | 0.34 0.26 | 0.34 0.26 |
| ZOA spread (ZSA) lgZSA=log10(ZSA/1°) | µlgZSA | 0.95 1.18 | 1.14 1.16 | 1.14 1.16 |
|  | σlgZSA | 0.16 0.05 | 0.16 0.14 | 0.16 0.14 |
| Cluster ASA (cASA) in [deg] |  | 5 | 5 10 | 10 |
| Cluster ZSA (cZSA) in [deg] |  | 7 | 7 | 7 |

Note: The updated SMa results for ZSA was based on a single source measurement data.

Agreement

- Update ASD parameters for SMa scenario and confirm the working assumption for ZSD parameters as follows:

| Scenarios |  |  | SMa |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) |  | µlgASA | Not part of this agreement | 0.55 0.73 0.51 | 0.55 0.73 0.51 |
|  |  | σlgASA | Not part of this agreement | 0.25 0.31 0.33 | 0.25 0.31 0.33 |
| ZOD spread (ZSD) lgZSD=log10(ZSD/1°) |  | µlgZSA | 0.14 | 0.14 | 0.14 |
|  |  | σlgZSA | 0.16 | 0.16 | 0.16 |
| Cluster ASD (cASD) in [deg] |  |  | Not part of this agreement | 5 1.75 | 10 1.75 |
| ZOD offset | µoffset,ZOD |  | 0 | atanhBS-hcld2D-atanhBS-hUTd2DhUT<hcl0hUT≥hcl hcl= [3.5] |  |

Agreement

- Revise the Rx heigh example for UMa, UMi, and SMa in scenario description in Section 6.2 as follows:

  - UMi

    - “Example: [Tx height:10m, Rx height with reference to floor height: 1.5-2.5 m, ISD: 200m]”

  - UMa

    - “Example: [Tx height:25m, Rx height with reference to floor height: 1.5-2.5 m, ISD: 200m, 500m]”

  - SMa

    - “Example: [Tx height: 35m, Rx height with reference to floor height: 1.5-2.5m, ISD: 1299m, 1732m]”

Agreement

- Confirm the working assumption for the following SMa parameters

| Scenarios | Sma (7 – 24 GHz) |  |  |
| --- | --- | --- | --- |
|  | LOS | NLOS | O2I |
| Number of clusters N | 15 | 14 | 14 |

Agreement

- Update “UT array orientation” in TR 38.901 as “UT orientation”

Agreement (replaces previous agreement in RAN1 #121)

- Update the SMa pathloss equation as follows:

| SMa | LOS | PLSMa-LOS=PL110m≤d2D<dBPPL2dBP≤d2D≤5km, PL1=20log1040πdd3Dfc3+min0.03h1.72,10log10dd3D- min0.044h1.72, 14.77+0.002log10hdd3D, see note 5 PL2=PL1dBP+40log10dd3DdBP, see note 5 | For 10 m ≤ d2D < dBP, σSF=4 For dBP ≤ d2D < 5000 m, σSF=6 | 0.50.8≤fc≤ 37 GHz 10 m ≤ d2D < dBP dBP ≤ d2D < 5000 m 25 m < hBS < 35 m h = 10 m |
| --- | --- | --- | --- | --- |
|  | NLOS | PLSMa-NLOS=161.04-7.1log10W+7.5log10h-24.37-3.7hhBS2log10hBS+43.42-3.1log10hBSlog10dd3D-3+20log10fc-3.2log1011.75hUT2-4.97 | σSF=8 | 0.50.8 ≤fc≤ 37 GHz 10 m ≤ d2D < 5000 m 1 m < hUT < 14 m h = 10 m, W = 10 m |

Agreement

- Update ASD parameters for SMa scenario as follows:

  - Note: The cluster ASD values in this agreement replaces the previous agreement in RAN1 #121.

| Scenarios |  | SMa |  |  |
| --- | --- | --- | --- | --- |
|  |  | LOS | NLOS | O2I |
| AOD spread (ASD) lgASD=log10(ASD/1°) | µlgASA | 0.55 0.48 | Not part of this agreement | Not part of this agreement |
|  | σlgASA | 0.25 0.27 | Not part of this agreement | Not part of this agreement |
| Cluster ASD (cASD) in [deg] |  | 5 2.08 | 1.75 1.33 | 1.75 1.33 |

Agreement

- Add the following note to SMa description

  - NOTE 1: SMa scenarios with ISDs between 1200-1800m can be used for evaluations

Agreement

Clarify downtilt simulation assumption parameter for calibrations as follows:

- For SMa,

  - downtilt refers to mechanical downtilt and no electrical downtilt applies.

  - Update the “Working assumption of 93 degree” to 92 degree for downtilt for SMa deployment scenario for ISD = 1732m.

- For InH,

  - Downtilt refers to mechanical downtilt and no electrical downtilt applies

- For UMa and UMi,

  - No mechanical downtilt applies, and downtilt values refer to electrical downtilt

Agreement

- Update the reference orientation of the handheld UE as follows:

  - Note: The representation will be same between GCS and UE LCS for ΩUT,α = 0 deg, ΩUT,β = 0 deg, ΩUT,γ = 0 deg,

- For calibration of handheld UE, use the following UE rotation based on reference UE orientation

  - For calibration with blockage:

    - For one-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

    - For dual-hand blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 0 45 deg, ΩUT,γ = 4590 deg,

    - For hand and head blockage, ΩUT,α = 0 – 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg,

  - For all other calibration cases:

    - ΩUT,α = 0 – 360 deg, ΩUT,β = 45 deg, ΩUT,γ = 0 deg,

  - Note: UT array orientation is defined by three angles ΩUT,α (UT bearing angle), ΩUT,β (UT downtilt angle) and ΩUT,γ (UT slant angle).

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 90 deg, ΩUT,γ = 0 deg

  - Example of ΩUT,α = 0 - 360 deg, ΩUT,β = 0 90 deg, ΩUT,γ = 90 deg

Agreement

- Reference radiation pattern of the UE antenna model is vertically polarized with all the gain in the theta field component, Fθ″″θ″,ϕ″=A″θ″,ϕ″ and Fϕ″″θ″,ϕ″=0 and referred to as polarization direction along the Z’’ axis.

- For cases when a candidate antenna placement location is used for one antenna field pattern (e.g., single polarization):

  - The polarization direction is indicated by the arrow in the following figure which is parallel with the plane of the handheld UE and perpendicular to the direction from the UE center to the candidate antenna location

- For cases when a candidate antenna placement location is used for two antenna field patterns (e.g., dual polarization) (not intended for FR1):

  - For the first antenna field pattern, the polarization direction is indicated by the arrows in the above figure but additionally rotated 45 degrees about the direction from the UE center to the candidate antenna location (i.e. rotated using the direction from the UE center to the candidate antenna location as the rotational axis).

  - For the second antenna field pattern, the polarization direction is perpendicular to the polarization direction of the first filed pattern and perpendicular to the direction from the UE center to the candidate antenna location

  - An example for candidate antenna location (6) is given in the figure below.

- RAN1 to perform calibration of antenna field pattern (Fu,θ''(θ',ϕ'), Fu, ϕ''(θ',ϕ')) for 8 antenna handheld UE in the UE local coordinate system

Agreement

Amend the previous agreement (from RAN1 #120-bis) as follows:

- Each polarized field component of the reference radiation pattern Fθ″″θ″,ϕ″ and Fϕ″″θ″,ϕ″ should be rotated according to the orientation and polarization direction of the each of UE antennae to get Frx,u,θ''(θ',ϕ'), Frx,u, ϕ''(θ',ϕ') and based on the orientation of the UE in the global coordinate system to get Frx,u,θ(θ,ϕ) and Frx,u,ϕ(θ,ϕ) using the methods already specified in TR 38.901, equation (7.3-3)/(7.1-11).

Observation:

- The number of clusters for UMi was taken from Winner II which appears to have use the 95%-tile value from the CDF of the number of clusters assuming K-mean clustering algorithm for counting number of clusters.

- For UMa NLOS case, the 90%-tile and maximum number of cluster measurements from sources was observed to be aligned.

- For UMa LOS and UMi LOS cases, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally higher than the values in current TR38.901 by 1 to 6 clusters.

- For InH LOS, InH NLOS, UMi NLOS, the 90%-tile and maximum number of cluster measurements from sources was observed to be marginally lower than the values in current TR38.901 by 1 to 6 clusters.

- For UMa LOS/NLOS, UMi LOS/NLOS, and InH LOS/NLOS cases, the mean number of cluster measurements were significantly lower than values in current TR38.901.

Agreement

For section 7.6, introduce an optional modeling component for number of clusters

- Optional modeling components are used to evaluate a channel propagation environment with diverse cluster ranges.

Table: Number of clusters for the diverse cluster environment

| Scenario | LOS | NLOS | O2I |
| --- | --- | --- | --- |
| UMi | D1: 6, D2: 12 | D1: 6, D2: 19 | D1: 6, D2: 12 |
| UMa | D1: 10, D2: 12 | D1: 15, D2: 20 | D1: 10, D2: 12 |
| InH | D1: 7, D2: 15 | D1: 6, D2: 19 | N/A |

- Number of clusters is chosen between a closed range of [D1, D2]. The selection of the number of cluster for each link is determined by the user. This is used as the number of clusters in Table 7.5.6-Part 1 and Part 2.

- Rapporteur to reference METIS channel model for this optional modeling component.

- Note: D1 is the average of the mean of the number of clusters from the source data. D2 is the values from 38.901

- Note: O2I values for D1 and D2 is the minimum of LOS or NLOS value

Agreement

Introduce additional cluster angle scaling factors as follows:

Table 7.5-2: Scaling factors for AOA, AOD generation

| # clusters | 4 | 5 | 6 | 7 | 8 | 10 | 11 | 12 | 14 | 15 | 16 | 19 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CφNLOS | 0.779 | 0.860 | 0.921 | 0.973 | 1.018 | 1.090 | 1.123 | 1.146 | 1.190 | 1.211 | 1.226 | 1.273 | 1.289 | 1.358 |

Table 7.5-4: Scaling factors for ZOA, ZOD generation

| # clusters | 6 | 7 | 8 | 10 | 11 | 12 | 14 | 15 | 16 | 19 | 20 | 25 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CφNLOS | 0.788 | 0.847 | 0.889 | 0.957 | 1.031 | 1.104 | 1.1072 | 1.1088 | 1.1276 | 1.184 | 1.178 | 1.282 |

Agreement

- Introduce polarization variability for each rays of cluster for NLOS component of the channel as an optional modeling component.

Generate polarization variability powers ηθθ, ηθϕ, ηϕθ and ηϕϕ for each ray m of each cluster n. η is log-Normal distributed. Draw values as

η'n,m,θθ,θϕ,ϕθ,ϕϕ=10Qn,m,θθ,θϕ,ϕθ,ϕϕ/10,	(7.5-21b)

ηn,m,θθ,θϕ,ϕθ,ϕϕ=η'n,m,θθ,θϕ,ϕθ,ϕϕ2+2κn,m-1η'n,m,θθ+η'n,m,ϕϕ+η'n,m,θϕκn,m-1+η'n,m,ϕθκn,m-1	(7.5-21c)

where Qn,m,θθ,θϕ,ϕθ,ϕϕ~N(0,32) is Gaussian distributed.

Note that Qn,m,θθ,θϕ,ϕθ,ϕϕ is independently drawn for each ray, cluster, and polarization component.
Hu,s,nNLOSt=PnMm=1MFrx,u,θθn,m,ZOA,ϕn,m,AOAFrx,u,ϕθn,m,ZOA,ϕn,m,AOATηn,m.θθexpjΦn,mθθηn,m,θϕκn,m-1expjΦn,mθϕηn,m,ϕθκn,m-1expjΦn,mϕθηn,m,ϕϕexpjΦn,mϕϕ

Ftx,s,θθn,m,ZOD,ϕn,m,AODFtx,s,ϕθn,m,ZOD,ϕn,m,AODexpj2πrrx,n,mT⋅drx,uλ0expj2πrtx,n,mT⋅dtx,sλ0expj2πrrx,n,mT⋅vλ0t	(7.5-22)

Note: The model is at least applicable for model-2 antenna polarization modeling.

Agreement

- Add new antenna Config C (with BS config 4) for 15 GHz for UT as optional configuration as part of full calibration assumption

  - (optional) Config C for 15 GHz: 16 antenna port with dual polarization based on handheld device antenna model using feasible candidate antenna locations in (1,2,3,4,5,6,7,8) as described in Clause 7.3.

  - Note: other UT configuration are not applicable for 15 GHz.

## Agenda 9.8.2

### RAN1 #116-bis (April-2023)

Agreement

The antenna array is assumed for the near-field study.

Agreement

For the study of near-field channel modelling, at least following aspects should be considered:

- Whether/How to define the near-field region

- The parameters variation for each ray/cluster across different antenna element pairs

Agreement

The following scenarios defined in TR38.901 should be considered for the study/modelling of near-field.

- UMa,UMi, Indoor office and Indoor factory

- FFS: RMa and other new scenarios

Agreement

For the assumption on the aperture size of antenna array, the following is considered as reference for channel model study.

- up to [TBD] m, or [TBD] lambda for UMi

- up to [TBD] m, or [TBD] lambda for UMa

- up to [TBD] m, or [ TBD] lambda for Indoor office

- up to [TBD] m, or [TBD] lambda for Indoor factory

Agreement

For the near-field channel model:

- The impact of the assumption of wavefront is only considered from the perspective of antenna array.

- The near field for each element within the antenna array is not considered in this SI.

Agreement

For near-field channel model, RAN1 strives to design a unified model to explicitly reflect the new properties of near- and existing properties of far-field under the structure of existing stochastic model TR 38.901.

- FFS: whether the same or different implementations, e.g., procedures/equations, are used for near- and far-field channel realization

Agreement

The near- or far-field condition should be studied for the direct path and non-direct paths between BS and UE.

- The near-/far-field condition for the direct path may be assessed by using the 3D BS-UE distance.

  - FFS: The determination of near-/far-field condition for the non-direct paths

- Note: The direct path is referring to the LoS ray in the TR 38.901 in principle.

- Note: The non-direct paths are referring to the cluster/ray(s) without including LoS ray in the TR 38.901.

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE,

- Angular domain parameters (i.e., AoA, AoD, ZoA, ZoD), Delay, initial phase, Doppler shift, Amplitude

- FFS: Impacts on the polarization

The following options are considered:

- Option-1: Determined by the locations of both TRP and UE.

- Option-2: Determined by the antenna element locations of both TRP and UE

Agreement

The following scenarios defined in TR38.901 should be considered for studying/modelling of spatial non-stationarity

- UMi, UMa, Indoor office and Indoor factory

- FFS: RMa and other new scenarios

Agreement

For the modelling of spatial non-stationarity, at least the following options can be studied to identify the impacted ray/cluster and element-pair link:

- Option 1: Introducing per ray/cluster the visible probability, or visibility region for set of antenna element

- Option 2: Introducing the physical blocker to emulate the blockage impact on the link for each element-pair

Note: The consistency across antenna elements and across clusters should be guaranteed.

### RAN1 #117 (May-2024)

Agreement

For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration:

- Up to 1.5 m for UMa with maximum antenna elements in the array is [5k] for single Polarization.

- Up to 1 m for UMi with maximum antenna elements in the array is [2.22k] for single Polarization.

- Up to [0.71] m for Indoor factory with maximum antenna elements in the array is [1.12k] for single Polarization.

- Up to [0.25 (for rectangular antenna array), 0.5 (for linear antenna array)] m for Indoor office with maximum antenna elements in the array is [138, 24] for single Polarization, respectively.

Working Assumption

For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901:

- Pathloss model, SF, LOS probability

- FFS:DS, ASA, ASD, ZSA, ZSD, K factor

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE,

- Phase

with Option-2 “Determined by the antenna element locations of both TRP and UE”.

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of non-direct path between TRP and UE,

- Angular domain parameters (i.e., AoA, AoD, ZoA, ZoD), Delay, phase, Doppler shift, Amplitude

- FFS: Impacts on the polarization

The following options are considered:

- Option-1: The cluster location-based approach, wherein the cluster location is obtained with following alternatives:

  - Alt-1: cluster location is derived based on at least the distance between the BS/UE and clusters.

  - FFS: How to obtain the distance.

  - FFS: Other parameters.

  - Alt-2: cluster location is directly dropped and generated.

- Option-2: The parameter-based approach with following detailed alternatives:

  - Alt-1: Introduce the model of variation rate of parameter over antenna elements.

  - Alt-2: Modelling the variation by taking the existing spatial consistency procedure of TR 38.901 as baseline.

- Option-3: The curvature-based approach.

Agreement

For the modelling of spatial non-stationarity, if necessary, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair link should be modelled.

- FFS: The value for power variation

- FFS: Impacts on the phase

Agreement

For the modelling of spatial non-stationarity, if necessary, if visible probability (VP) or visibility region (VR) is adopted, at least the following aspects should be considered for definition of VR/VP:

- Granularity of visible probability or visibility region (e.g., per cluster or per ray)

- Determination of visible probability (e.g., distribution) or visibility region (e.g., size, location)

Agreement

For the modelling of spatial non-stationarity, if necessary, if physical blocker-based approach is adopted, the following aspects should be considered for definition of blocker:

- Blocker size/type:

  - FFS: Additional blocker size/type compared to the Table 7.6.4.2-5 in TR 38.901.

  - FFS: Different blocker sizes/types are considered to emulate the antenna element-wise blockage effect at the BS and UE side

- Blocker location, e.g. distribution of the blocker, relative distance between blocker and BS or UE

- FFS: Number of physical blockers to be considered.

Agreement

To align the understanding of the terminology for channel model study, the following figures are considered as the reference:

- For non-direct path:

- For direct path:

Conclusion

For near-field channel, no changes are expected on the following parameters for direct path.

Amplitude, polarization matrix

### RAN1 #118 (August-2024)

Agreement

Previous agreement made in RAN1#117 is updated as:

For the assumption on the aperture size of antenna array, the following is considered for near-field and spatial non-stationarity channel model study, e.g., simulation/measurement and calibration:

- Up to 1.5 m for UMa with maximum antenna elements in the array is 5k for single Polarization.

- Up to 1 m for UMi with maximum antenna elements in the array is 2.22k for single Polarization.

- Up to 0.71 m for Indoor factory with maximum antenna elements in the array is 1.12k for single Polarization.

- Up to 0.25 (for rectangular antenna array), 0.5 (for linear antenna array) m for Indoor office with maximum antenna elements in the array is 256, 80 for single Polarization, respectively.

Agreement

Confirm the following working assumption made in RAN1#117.

Working Assumption

For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901:

- Pathloss model, SF, LOS probability

- FFS:DS, ASA, ASD, ZSA, ZSD, K factor

Agreement

For the near-field channel modeling, no changes are expected on both value and parameter generation procedure of at least following large-scale parameters in existing TR 38.901:

- DS, ASA, ASD, ZSA, ZSD, K factor

Agreement

The spatial non-stationarity characteristics, i.e., the antenna element-wise power variation at least at BS side, is supported in the channel modelling.

FFS: the antenna element-wise power variation at UE side

FFS: the causes and details of methodology for modelling the spatial non-stationarity characteristics

Agreement

For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE:

- Polarization matrix

- FFS: Amplitude

Agreement

For near-field channel, the following formula is adopted to model the phase of direct path between TRP and UE as antenna element-wise channel parameter:

exp⁡(-j2πd3Dλ0)exp-j2π|ru,s|-d3Dλ0,

where the ru,s refers to the vector determined by the location of antenna element u and s. The d3D refers to the 3D distance between reference antenna at TRP and UE side.

Agreement

For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter.

- Phase

- FFS: Doppler shift, Angular domain parameters, delay

Observation

According to the inputs from multiple sources, partial blockage effect may cause the spatial non-stationarity.

Agreement

For the modelling of spatial non-stationarity, the variation (e.g., reduction) of power for the impacted ray/cluster within the element-pair link should be modelled as:

- If visible probability (VP) or visibility region (VR) is adopted,

- A power attenuation factor within [0 1] is introduced.

- FFS: Details on how to determine the exact value for each cluster

- If physical blocker-based approach is adopted:

- Adopt the existing knife edge attenuation model in TR 38.901 in blockage Model-B to model the power attenuation per element with following update as:

- For each ray/cluster, rotating the blocker to ensure the arrival/departure direction at each Receive/Transmit antenna element is always perpendicular to the screen, respectively.

- FFS: applicability and details for blockage Model-A

Agreement

For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, the following additional blocker type can be considered for blockage model B:

- Building edge for outdoor scenario

- Small object, e.g., billboard, street lamp, pillar, for either indoor or outdoor scenario

- FFS: UE-side (self-blockage) blocker for both indoor and outdoor scenario

- Details, e.g., blocker types such as Single hand grip, dual-hand grip, and head with one hand grip.

FFS: the number and the location of the blocker between BS and one specific UE.

FFS: applicability and details for blockage Model-A

Agreement

For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE can be determined by one of the following candidate options:

- Option-1: The antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster.

  - FFS: How to obtain the distance.

  - FFS: Other parameters.

  - FFS: association between a point and a cluster

- Option-2: The antenna element-wise channel parameters are determined based on the existing spatial consistency procedure of TR 38.901 with updates.

  - FFS: Details to obtain the antenna element-wise parameters.

Note: Companies are encouraged to check the Option-3 including the similarity/difference with Option-1.

Agreement

For the modelling of spatial non-stationarity, if visible probability or visibility region is adopted,

- Visible probability or visibility region is modelled per cluster

- FFS Ratio of UEs and clusters that have SNS impact

- Rectangle can be considered as starting point for shape of VR with following alternatives to define the size:

- Alt 1: VR size is defined as number of elements generated by a distribution

- FFS distribution

- Alt 2: VR size is derived based on distance between antenna array of BS and UE/cluster

- Note: Cluster location is required in this alternative

- Alt 3: VR size is randomly generated with a minimum size limit

### RAN1 #118-bis (October-2024)

Conclusion

The spatial non-stationarity characteristics at UE side (e.g., due to the impact of hand(s) and/or head), i.e., the antenna element-wise power variation, is supported in the channel modelling in TR 38.901 with potential updates.

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameters of direct path between TRP and UE,

- Angular domain parameters

- FFS: Doppler shift, Delay

with Option-2 “Determined by the antenna element locations of both TRP and UE”.

Note: Angular domain parameters can be optionally modelled.

Conclusion

For near-field channel, no changes are expected on following parameters of the non-direct path between TRP and UE:

- Amplitude

Agreement

For near-field channel, if necessary, the antenna element-wise channel parameters of non-direct path between TRP and UE is modelled by:

- For the non-direct paths, the antenna element-wise channel parameters are derived based on at least the distance between the BS/UE and a point associated with cluster, i.e.,

  - d1 is used to determine the 1st point associated wit h cluster to calculate the parameter at least for BS side;

  - FFS: d2 is used to determine the 2nd point associated with cluster to calculate the parameter at least for UE side;

  - FFS: whether one point is sufficient

  - FFS: the details to generate the d1, d2,e.g.,if the d2 is needed, both d1 and d2 will be generated pair wise.

  - FFS: The ratio of non-direct path which show the phenomenon with spherical wavefront

  - FFS: The determination of the point associated with cluster is conducted in Cluster-level

Agreement

For near-field channel, if necessary, the following parameters of the non-direct path between TRP and UE should be modeled as antenna element-wise parameter.

- Angular domain parameters, Delay (both can be optionally modelled)

    - Working Assumption

      - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.

- FFS: Doppler shift

Agreement

For near-field channel, if necessary, to model the following antenna element-wise channel parameter of direct path between TRP and UE,

- Delay (can be optionally modelled)

    - Working Assumption

      - The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.

with Option-2 “Determined by the antenna element locations of both TRP and UE”.

Conclusion

RAN1 confirms that the modelling of near-field propagation characteristics (i.e., characteristics of spherical wavefront) is taken as an additional modelling component, and the relevant progress (e.g., the agreement(s) with/without “if necessary”) in AI 9.8.2 will be captured in TR 38.901.

Agreement

For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Billboard | Cartesian: w=2.4m; h=3.6m | Stationary |
| Outdoor | Street lamp | Cartesian: w=0.4m; h=0.8m | Stationary |
| Outdoor | Building edge | Cartesian: w=X m; h=Y m | Stationary |
| Indoor | Pillar | Cartesian: w=0.3m; h=3m | Stationary |

- FFS: the value of X and Y for the blocker dimensions of building edge is needed.

- FFS: the details related to the user hand/head:

| Indoor;Outdoor | FFS: User hand | Cartesian: w=[0.2]m; h=[0.1]m | Stationary |
| --- | --- | --- | --- |
| Indoor;Outdoor | FFS: User head | Cartesian: w=[0.24]m; h=[0.20] m | Stationary |

- FFS:The details of blockage model B to implement the impact of user hand and head.

- FFS: The location of the user hand/head

Agreement

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, for blockage model B, the procedure to determine the number of blockers and locations are same as existing blockage model B.

For the modelling of spatial non-stationarity at UE side, if blocker-based approach is adopted, an attenuation per antenna element is introduced, the following options can be considered with down-selection by RAN1#120:

- Option-1: generated by leveraging the existing blockage Model-B with potential updates.

  - At most 2 hand type blockers and one head type blocker for a specific UE is assumed.

- Option-2: generated by leveraging the existing blockage Model-A with potential updates

- Option-3: generated by a distribution or a fixed value due to the hand grip and/or head proximity

FFS: the details, e.g., how to determine the value of loss for the case as one hand grip, a dual hand grip and/or head proximity

### RAN1 #119 (November-2024)

Agreement

For near-field channel, the following formula is adopted to model the angular domain parameters of direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

Where θLOS,ZOA,u,s,ϕLOS,AOA,u,s, θLOS,ZOD,u,s,ϕLOS,AOD,u,s are the respective antenna element-wise elevation arrival angles, azimuth arrival angles, elevation departure angles and azimuth departure angles of LoS path between the transmit antenna element s and receive antenna element u.

Working Assumption

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected.

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |

  - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge:

    - LdB=20log100.5-F, where F represents one of Fh1, Fh2, Fv1, and Fv2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

Agreement

For the modelling of spatial non-stationarity at BS side, if stochastic based approach, at least for unified visible probability (VP) and visibility region (VR), is adopted, the following steps are considered:

- Step 1: Determine whether a cluster is impacted by SNS

- Step 2: Generate the visibility region

- Step 3: Calculate the power attenuation factor

FFS: the details of each step

Note: Merge of some of the steps is not precluded

Working Assumption

For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details:

- Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip.

  - FFS: probability of each case in the simulation.

- For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with one of following alternatives:

  - Alt 1: Introduce a fixed value for each candidate antenna position per case.

  - Alt 2: Introduce a distribution for candidate antenna position.

    - FFS: distribution per antenna position(s) and per case.

Agreement

For near-field channel, to generate the channel parameter, the reference point is defined as:

- The physical center of the antenna array/center of the device.

Conclusion

For near-field channel, no changes are expected on following parameter for direct path between TRP and UE:

- Doppler shift

Conclusion

For near-field channel, no changes are expected on following parameter for non-direct path between TRP and UE:

- Doppler shift

Agreement

For near-field channel, the following formula is adopted to model the angular domain parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

Frx,u,θθn,m,ZOA,u,ϕn,m,AOA,uFrx,u,ϕθn,m,ZOA,u,ϕn,m,AOA,uTexpjΦn,mθθkn,m-1expjΦn,mθϕkn,m-1expjΦn,mϕθexpjΦn,mϕϕFtx,s,θθn,m,ZOD,s,ϕn,m,AOD,sFtx,s,ϕθn,m,ZOD,s,ϕn,m,AOD,s

where θn,m,ZOA,u,ϕn,m,AOA,u, θn,m,ZOD,s,ϕn,m,AOD,s are the ray-wise angular domain parameters of ray m cluster n between the transmit antenna element s and receive antenna element u.

Agreement

For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the rotation and power variation calculation are conducted in ray level.

Agreement

For near-field channel, for the non-direct paths, the distance d1 between the BS and the 1st point associated with cluster is generated with one of following options that to be selected in RAN1#120:

- Option-1: The distance d1 is generated directly following a specific distribution.

  - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster

  - For other k2 clusters, the distance  d1 is equal or less than the speed of light times the absolute delay of the cluster and generated following a specific distribution

    - FFS: The specific distribution, e.g., log-normal distribution for UMi

    - FFS: The values of k1 and k2

  - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901

    - FFS: The value for UMi, UMa, InH-Office.

- Option-2: The distance d1 is generated by a scaling factor multiplied by the absolute distance corresponding to the absolute delay of the cluster, where the scaling factor is generated from a specific distribution.

  - For the k1 clusters, d1 = speed of light times the absolute delay of the cluster (i.e., the scaling factor is 1)

  - For other k2 clusters, the distance  d1 is equal or less than the speed of light times the absolute delay of the cluster and the scaling factor generated following a specific distribution (i.e., the scaling factor is equal or less than 1)

    - FFS: Distribution of scaling factor, e.g., Beta distribution for UMa

    - FFS: The values of k1 and k2

  - The generation of absolute delay of the cluster is according to the procedure defined in Section 7.6.9 in TR 38.901

    - FFS: The value for UMi, UMa, InH-Office.

Note 1: If d2 is agreed to be considered, the generation of d2 can follow the same principle with the generation of d1.

Working Assumption

For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side.

Agreement

For near-field channel, in principle, one of following options is considered to calculate the antenna element-wise delay of direct path between TRP and UE:

- Option-1: The following formula is used to calculate the antenna element-wise delay of direct path:

τ1,u,s=τ1-d3D-|ru,s|c

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

  - The above formula is applied with the same principle as existing model in Clause 7.6.2 of TR 38.901, and the channel response of LOS ray between Rx antenna u and Tx antenna s at delay τ at time t is given by:

  - Hu,s,1LOS(t;τ)=Frx,u,θθLOS,ZOA,u,s,ϕLOS,AOA,u,sFrx,u,ϕθLOS,ZOA,u,s,ϕLOS,AOA,u,sT100-1Ftx,s,θθLOS,ZOD,u,s,ϕLOS,AOD,u,sFtx,s,ϕθLOS,ZOD,u,s,ϕLOS,AOD,u,s

exp-j2πd3Dλfexp-j2πru,s-d3Dλfexpj2πrrx,LOST∙vλ0tδτ-τ1,u,s

- Option-2: The antenna element-wise delay of direct path is directly modeled based on the distance of direct path between antenna elements at TRP and UE side.

  - Following formula is adopted to model the antenna element-wise delay in the channel coefficient as:

δτ-τ1,u,s, τ1,u,s=τ1-d3D-ru,sc

where ru,s refers to the vector determined by the location of receive antenna element u and transmit antenna element s, d3D refers to the 3D distance between reference antenna at TRP and UE side.

- Option-3: Confirm the following working assumption made in RAN1#118BIS:

  - “The existing model in Clause 7.6.2 of TR 38.901 is used to model the delay.”

- Note: τ1 is same as that is defined in 7.5-30 in TR38.901

### RAN1 #120 (February-2025)

Agreement

For the modelling of spatial non-stationarity at UE side mainly due to close proximity, if blocker-based approach is adopted, the Option-3 is supported with following details:

- Three typical cases are introduced: one hand grip, dual hand grip, head and one hand grip.

  - probability of each case in the simulation is defined with the following table:

| Usage scenario | one hand grip | dual hand grip | head and one hand grip |
| --- | --- | --- | --- |
| Proportion | 58% | 29% | 13% |

- For each case, to model the attenuation for each of antenna in the candidate antenna location defined in the UE antenna model for handheld device with Alt 1:

  - Alt 1: Introduce a fixed value for each candidate antenna position per case.

- The percentage of the UE without impact from the SNS at UE side is 10%.

Related Working Assumption made in RAN1#119 doesn’t need to be confirmed.

Agreement

For the modelling of spatial non-stationarity at UE side, the fixed values of attenuation for candidate antenna locations for handheld device will be defined.

Note:

The fixed values of attenuation for candidate antenna locations for handheld device can be defined as:

| Antenna index | Power attenuation (dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | One hand grip |  |  |  | Dual hand grip |  |  |  | Head and one hand grip |  |  |  |
|  | Set-1@6GHz | Set-2@ [0.7-8.4] GHz | [Set-3@15GHz] |  | Set-1@6GHz | Set-2@ [0.7-8.4] GHz | [Set-3@15GHz] |  | Set-1@6GHz | Set-2@ [0.7-8.4] GHz | [Set-3@15GHz] |  |
|  |  |  | Alt-1 | Alt-2 |  |  | Alt-1 | Alt-2 |  |  | Alt-1 | Alt-2 |
| 1 | 0 | 1 | 1.0 | 0 | 7.31 | 9 | 3.8 | 9.68 | 10.57 | 1 | 4.0 | 14.16 |
| 2 | 0 | FFS | FFS | 0 | 0 | FFS | FFS | 0 | 7.08 | FFS | FFS | 9.87 |
| 3 | 0 | 3 | 3.8 | 0 | 7.31 | 9 | 3.8 | 10.6 | 0 | 3 | 3.3 | 0 |
| 4 | 6.15 | FFS | 3.6 | 9.13 | 6.15 | FFS | FFS | 9.12 | 0 | FFS | 3.3 | 9.13 |
| 5 | 7.31 | 12 | 3.8 | 10.6 | 0 | 5 | 1.0 | 0 | 6.55 | 12 | 3.3 | 10.6 |
| 6 | 0 | FFS | FFS | 0 | 0 | FFS | FFS | 0 | 10.12 | FFS | FFS | 9.70 |
| 7 | 0 | 1 | 1.0 | 0 | 0 | 5 | 1.0 | 0 | 10.57 | 3 | 4.0 | 14.16 |
| 8 | 0 | FFS | 1.0 | 0 | 6.15 | FFS | FFS | 10.62 | 10.67 | FFS | 4.0 | 14.27 |

- Note1: The selection of number of antenna and antenna location is determined as part of assumption in the calibration and simulation.

- Note2: For both one hand grip and Head and one hand grip cases, the handheld is assumed to be held by right hand as example.

- The standardized CTIA hand grip can be considered as reference.

Conclusion

For near-field channel, no changes are expected on following parameter for both direct and non-direct path between TRP and UE:

- Delay

Agreement

For near-field channel, for the non-direct paths, the distance d1 between the TRP and the 1st point associated with cluster is generated by Option-2:

- The distance d1 is generated by a scaling factor (sTRP,n) multiplied by the absolute distance corresponding to the absolute delay of cluster, where the scaling factor (sTRP,n) is generated by:

- For UMa scenarios, a Beta distribution with [α=1.8,β=1.22]:

- For UMi scenarios, a Beta distribution with [α=2.35,β=1.45]:

- For Indoor Office scenarios, a Beta distribution with [α=1.13,β=1.42]

- For the value of  k1 and  k2

- For UMa, the value of  k1 is 2.

- For UMi, the value of  k1 is 2.

- For InH, the value  k1 is [75%, 10.9%] of the total number of clusters for InH

- Note: the  k1 clusters are the strongest of the total number of clusters in the channel

- Note: the value of  k2 is equal to total number of clusters minus k1.

Agreement

Confirm the following working assumption.

Working Assumption (made in RAN1#119)

For near-field channel, for the non-direct paths, the impact of spherical wavefront is optionally considered at UE side.

Agreement

For near-field channel, for number of  k2  non-direct paths, the distance d2 between the UE and the 2nd point associated with cluster is generated by:

- a scaling factor (sUE,n) multiplied by the absolute distance corresponding to the absolute delay of cluster.

Agreement

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, the nearest K blockers from BS are selected.

- For the modelling of spatial non-stationarity at BS side, if physical blocker-based approach is adopted, at least for blockage model B, the following parameters to define the building edge are introduced in the Table 7.6.4.2-5 in TR 38.901:

|  | Typical set of blockers | Blocker dimensions | Mobility pattern |
| --- | --- | --- | --- |
| Outdoor | Building edge | Cartesian: w=50 m; h=20 m | Stationary |

  - In the blockage model B, the following equation is used to calculate the attenuation caused by the building edge:

- LdB=-20log100.5-F, where F represents one of Fh1, Fh2, Fw1 and Fw2 defined in equation 7.6-29 in section 7.6.4.2 of TR 38.901.

Note: Corresponding Working Assumption made in RAN1#119 does not need to be confirmed.

Agreement

For near-field channel, the following formula is adopted to model the phase parameters of non-direct path between TRP and UE as antenna element-wise channel parameter:

- For the TRP side, the element-wise phase is calculated as:

expj2πd1,n-d1,n∙ rtx,n,m-dtx,sλ0

where d1,n is the generated distance of the cluster n. rtx,n,m is the spherical unit vector with azimuth departure angle and elevation departure angle for ray m of cluster n. dtx,s is the vector pointing from reference point to transmit antenna element s, wherein the reference point is the physical center of the antenna array/center at Tx side.

- For the UE side, the element-wise phase is calculated as:

expj2πd2,n-d2,n∙ rrx,n,m-drx,uλ0

where d2,n is the generated distance of the cluster n. rrx,n,m is the spherical unit vector with azimuth arrival angle and elevation arrival angle for ray m of cluster n. drx,u is the vector pointing from reference point to receive antenna element u, wherein the reference point is the physical center of the antenna array/center at Rx side.

Agreement

The following agreement replaces conclusion made in RAN1 #120 regarding starting point for further discussion for calibration.

- For new scenarios, and if changes are made to existing scenarios, interested companies to perform near field channel modeling calibration based on the following simulation assumptions. Use the following updates to TR38.901 as baseline assumption for calibration.

- Additional calibration parameters for near field channel modeling can be found in Table 7.8-7. It is assumed that parameters from Table 7.8-2 is used if unspecified by the additional full calibration parameters in Table 7.8-7.

Table 7.8-7: Simulation assumptions for calibration for near field channel modeling

| Parameter |  | Values |
| --- | --- | --- |
| Scenarios |  | UMi-street Canyon, Indoor-office (open office) |
| Carrier Frequency |  | 7 GHz (optional) 15 GHz |
| Bandwidth |  | 20 MHz |
| BS antenna height |  | UMi-street Canyon: 10m Indoor-office: 3m |
| BS antenna downtilt |  | 102 degrees for UMi-street Canyon 180 degrees for indoor (i.e., array panel downward facing) |
| BS Tx power |  | 44 dBm for UMi-Street Canyon 24 dBm for Indoor |
| BS antenna configurations and port mapping |  | For UMi scenario: (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ FFS: (optional for 15 GHz) (M, N, P, Mg, Ng; MP, NP) = (128, 32, 2, 1, 1; 16, 16), dH = dV = 0.5λ For Indoor-office scenario: (M, N, P, Mg, Ng; MP, NP) = (8, 24, 2, 1, 1; 8, 8), dH = dV = 0.5λ (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling |  | Model-2 in Clause 7.3.2 of TR38.901 |
| UE Location | Outdoor/indoor | UMi-street Canyon: 100% outdoor Indoor-office: 100% indoor |
|  | LOS/NLOS | 100% LOS |
|  | UE antenna height | UMi-street Canyon: 1.5m Indoor-office: 1m |
| UT antenna configurations |  | Config 1: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config 2: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern |  | Config 1: isotropic Config 2: [isotropic] |
| UT Polarized antenna modelling |  | Config 1: Model-2 in Clause 7.3.2 of TR38.901 Config 2: [Model-2 in Clause 7.3.2 of TR38.901] |
| Applicability of Nearfield Modeling |  | Component of NF channel to be considered: - Phase of Direct path - Phase of Non-direct path, applied only at the BS side (optionally) |
| Calibration method |  | Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric for a each distance Z For UMi-street Canyon: Z = 10, 30, 50, 100 m (optional) Z = 20, 40, 80 m For Indoor: Z = 0, 2, 6, 10 m (optional) Z = 4, 8 m |
| Metric |  | 1) CDF of the ratio between the 2nd, 3rd ,4th, ..., xth singular value and the 1st singular value (serving cell) at t=0 plotted in 10*log10 scale Note: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) Note: The singular values of are the eigenvalues of the mean covariance matrix of [4] consecutive and non-overlapping PRBs |

- Note: Parameters in [ ] are subject for further check and are intent as tentative values for calibration checks for companies.

Agreement

The following agreement replaces conclusion made in RAN1 #120 regarding starting point for further discussion for calibration.

- For new scenarios, and if changes are made to existing scenarios, interested companies to perform near field channel modeling calibration based on the following simulation assumptions. Use the following updates to TR38.901 as baseline assumption for calibration.

- Additional calibration parameters for BS side spatial non-stationarity channel modeling can be found in Table 7.8-8. It is assumed that parameters from Table 7.8-2 is used if unspecified by the additional full calibration parameters in Table 7.8-8.

Table 7.8-8: Simulation assumptions for calibration for BS side spatial non-stationarity

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ FFS: (optional for 15 GHz) (M, N, P, Mg, Ng; MP, NP) = (128, 32, 2, 1, 1; 16, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 of TR38.901 |
| UT Antenna Configuration | Config 1: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 FFS: (optional) Config 2: 16 antenna port with dual polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| UT antenna pattern | Config 1: [isotropic] FFS: (optional) Config 2: [Based on directional antenna for UT described in Clause 7.3] |
| UT Polarized antenna modelling | Config 1: [Model-2 in Clause 7.3.2 of TR38.901] FFS: (optional) Config 2: [Model-2 in Clause 7.3.2 of TR38.901] |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the BS | Case 1: Spatial blocker-based approach Case 2: Stochastic-based approach |
| Metric | 1) CDF of coupling loss (serving cell) received per UT port 2) CDF of the ratio between the 2nd, 3rd ,4th, ..., xth singular value and the 1st singular value (serving cell) at t=0 plotted in 10*log10 scale Note: The value of x is the minimum value of (number of BS antenna ports, number of UE antenna ports) Note: The singular values are the eigenvalues of the mean covariance matrix of [4] consecutive and non-overlapping PRBs |

- Note: Parameters in [ ] are subject for further check and are intent as tentative values for calibration checks for companies.

Agreement

The following agreement replaces conclusion made in RAN1 #120 regarding starting point for further discussion for calibration.

- For new scenarios, and if changes are made to existing scenarios, interested companies to perform near field channel modeling calibration based on the following simulation assumptions. Use the following updates to TR38.901 as baseline assumption for calibration.

- Additional calibration parameters for UT side spatial non-stationarity channel modeling can be found in Table 7.8-9. It is assumed that parameters from Table 7.8-2 is used if unspecified by the additional full calibration parameters in Table 7.8-9.

Table 7.8-9: Simulation assumptions for calibration for UT side spatial non-stationarity

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | (M, N, P, Mg, Ng; MP, NP) = FFS one or more of following options, including which options may be “optional” and applicable to which carrier frequency - Option 1: (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ - Option 2: (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ - Option 3: (128, 32, 2, 1, 1; 16, 16), dH = dV = 0.5λ - Option 4: (8, 16, 2, 1, 1; 8, 16), dH = dV =0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 of TR38.901 |
| UT Antenna Configuration | FFS one or more of following options, including which options may be “optional” and applicable to which carrier frequency - Option 1: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1, 7, 3, 5) as described in Clause 7.3 - Option 2: 8 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1, 2, 3, 4, 5, 6, 7, 8) as described in Clause 7.3 - Option 3: 16 antenna port with dual polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| UT antenna pattern | FFS applicability of either <isotropic> or <Based on directional antenna for UT described in Clause 7.3> for each supported UT antenna configuration |
| UT Polarized antenna modelling | FFS applicability of <Model-2 in Clause 7.3.2 of TR38.901> for each supported UT antenna configuration |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the UT | Case 1: one hand grip Case 2: dual hand grip Case 3: head and one hand grip |
| Metric | 1) CDF of coupling loss (serving cell) received per UT port 2) Singular value based metric: FFS between following option - Option 1: use definition of Metric 2 from BS side spatial non-stationarity calibration table - Option 2: use definition of Metric 4 from full calibration table |

- Note: Parameters in [ ] are subject for further check and are intent as tentative values for calibration checks for companies.

### RAN1 #120bis (April-2025)

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS) is determined as:

- For the nth UE, the Prn is generated using a truncated normal distribution within [0,1]

Prn=Nμ, σ2;     where 0≤Prn≤1.

  - FFS: the value of μ, σ.

    - Note: It should be fixed in RAN1#120-bis;

- For kth cluster of the nth user, if the xk,n is smaller than Prn, the cluster is impacted by the SNS, where the xk,n is defined as xk,n~U(0,1))

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, generate the visibility region as a rectangle randomly located at a corner of the antenna array with dimension a*b, where a~U(Vn*W,W), b=Vn*H*W/a, H is the antenna array height in vertical dimension and W is the antenna array width in horizontal dimension, Vn is calculated by

Vn=Α∙e-maxPn-Pnλ+Β+δ.

where A=0.15, B=0.45, λ=33, δ~N(0,0.0015) for UMa scenario, and n is a cluster index

- FFS: The value for other scenarios.

  - Note: It should be fixed in RAN1#120-bis;

- FFS: the details of randomly located VR

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted,

- For clusters without SNS, the power attenuation factor αs,n is set to 1.

- For clusters with SNS, the power attenuation factor αs,n is generated by

αs,n=1                             ,xs-x0,n≤a,ys-y0,n≤be-xs-xa,nDn∙C        ,xs-x0,n>a,ys-y0,n≤b  e-ys-yb,nDn∙C        ,xs-x0,n≤a,ys-y0,n>b e-xs-xa,n2+ys-yb,n2Dn∙C                          ,others

where, Dn=xA,n-xa,n2+yB,n-yb,n2 , xA,n, yB,n is the coordinate of the other corner of the antenna array on the diagonal with reference corner x0,n, y0,n, and xa,n, yb,n is the coordinate of the other corner of the rectangular VR on the diagonal with reference corner x0,n, y0,n. The xs, ys denotes the coordinate of the antenna element out of VR region. C is the roll-off factor between the visible and invisible regions.

- FFS: The value of C.

  - Note: It should be fixed in RAN1#120-bis;

Agreement

For the modelling of spatial non-stationarity at UE side, introduce 3 sets of fixed attenuation values, each set corresponding to a range of frequency,

| Antenna index | Power attenuation (dB) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | One hand grip |  |  | Dual hand grip |  |  | Head and one hand grip |  |  |
|  | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz |
| 1 | - | 0.7 | 1.0 | - | 11.0 | 3.8 | - | 3.7 | 4.0 |
| 2 | - | 4.1 | 2.4 | - | 1.1 | 3.8 | - | 4.1 | 3.7 |
| 3 | - | 3.3 | 3.8 | - | 10.5 | 3.8 | - | 4.3 | 3.3 |
| 4 | 13.6 | 7.2 | 3.6 | 5.8 | 5.6 | 2.6 | 15.1 | 7.8 | 3.3 |
| 5 | - | 10.8 | 3.8 | - | 1.5 | 1.0 | - | 11.7 | 3.3 |
| 6 | - | 9.1 | 2.4 | - | 1.4 | 1.0 | - | 10.1 | 3.7 |
| 7 | - | 0.7 | 1.0 | - | 1.3 | 1.0 | - | 2.9 | 4.0 |
| 8 | 2.4 | 0.6 | 1.0 | 5.6 | 6.2 | 2.6 | 4.9 | 4.2 | 4.0 |

Note: For below 1GHz, the model is only applicable for 2-antenna system with antenna location index 4 and 8

Note: The selection of number of antenna and antenna index will be determined as part of the assumption for simulation.

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, for the Step 1 (whether a cluster is impacted by SNS):

- In UMa, the distribution of Prn is determined as: μ=0.56, σ=0.20;

Agreement

For near-field channel, the scaling factor (sUE,n) used to generate the d2 is:

- For number of k1 non-direct paths, sUE,n=1.

- For number of k2 non-direct paths:

  - Option-1: sUE,n=1-sTRP,n;

    - Note 1: It’s assumed that only single non-specular interaction along with specular reflection(s) is considered in the propagation channel

    - Note 2: At least most of the sources of non-specular interaction are assumed to be a point scatterer or diffractions.

Agreement

For near-field channel, to generate the d1 for the k2 non-direct paths, the distribution of scaling factor sTRP,n is:

- For the UMi scenarios, a Beta distribution with α=1.53, β=1.42

- For the UMa scenarios, a Beta distribution with α=1.93, β=1.33

- For the InH scenarios, a Beta distribution with α=1.25, β=1.27

Conclusion

For near-field channel, to generate the d1 for k2 non-direct paths:

- No additional lower bound (i.e., dmin) of d1 is defined.

Agreement

In the Indoor office scenario for near-field channel, k1= 4.

Agreement

In the Indoor factory scenarios, to generate the d1 non-direct path for the near-field channel model:

- k1= 4;

- For the k2 non-direct paths, the distribution of scaling factor sTRP,n is a Beta distribution with α=1.38, β=1.26

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, to generate the power attenuation factor αs,n for clusters with SNS, the value of C (i.e., the roll-off factor between the visible and invisible regions) is:

- C= 13 for UMa.

Agreement

For the modelling of spatial non-stationarity, the visibility region, i.e., a rectangle, is randomly located at a corner of the antenna array by following approach:

- The location of the VR is jointly determined by following conditions:

  - The VR is located at either left or right part in horizontal domain of antenna array with equal probability of 1/2.

  - The probability of VR at the upper part of antenna array is [0.8] and the probability of VR at the lower part of antenna array is [0.2];

Agreement

For the calibration parameters for the near-field channel,

| Parameter |  | Values |
| --- | --- | --- |
| Scenarios |  | UMi-street Canyon, Indoor-office (open office) |
| Carrier Frequency |  | 7 GHz (Optional) 15 GHz |
| Bandwidth |  | 20 MHz |
| BS antenna height |  | UMi-street Canyon: 10m Indoor-office: 3m |
| BS antenna downtilt |  | 102 degrees for UMi-street Canyon 180 degrees for indoor (i.e., array panel downward facing) |
| BS Tx power |  | 44 dBm for UMi-Street Canyon 24 dBm for Indoor |
| BS antenna configurations and port mapping |  | For UMi scenario: Config 1 for 7GHz - (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; - (Optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config 2 for 15GHz - (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 8, 16), dH = dV = 0.5λ For Indoor-office scenario for 7GHz: - (M, N, P, Mg, Ng; MP, NP) = (8, 24, 2, 1, 1; 8, 8), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling |  | Model-2 in Clause 7.3.2 of TR38.901 |
| UE Location | Outdoor/indoor | UMi-street Canyon: 100% outdoor Indoor-office: 100% indoor |
|  | LOS/NLOS | 100% LOS |
|  | UE antenna height | UMi-street Canyon: 1.5m Indoor-office: 1m |
| UT antenna configurations |  | Config A: Mg = Ng = 1, M = 2, N = 1, P = 2, dH = dV = 0.5λ Config B: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3 |
| UT antenna pattern |  | Isotropic |
| UT Polarized antenna modelling |  | For UT Config A, Model-2 in Clause 7.3.2 of TR38.901 For UT Config B, following the Clause 7.3.2 of TR38.901 |
| Applicability of Nearfield Modeling |  | Component of NF channel to be considered: - Phase of Direct path - Phase of Non-direct path, applied only at the BS side (optionally) |
| Calibration method |  | Drop multiple users in the multiple cells at the fixed horizontal distance Z from the BS, and collect the metric for a each distance Z For UMi-street Canyon: Z = 10, 30, 50, 100 m (optional) Z = 20, 40, 80 m For Indoor: Z = 0, 2, 6, 10 m (optional) Z = 4, 8 m |
| Metric |  | CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

Agreement

The following assumption is used for UE side SNS calibration:

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | (M, N, P, Mg, Ng; MP, NP) = (8, 16, 2, 1, 1; 8, 16), dH = dV =0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 of TR38.901 |
| UT Antenna Configuration | Config A for 7GHz: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1, 7, 3, 5) as described in Clause 7.3 Config B for 15 GHz: 8 antenna port with dual polarization based on handheld device antenna model using all candidate antenna locations (1,2,3,4,5,6,7,8) as described in Clause 7.3. |
| UT antenna pattern | Based on directional antenna for UT described in Clause 7.3 for each supported UT antenna configuration |
| UT Polarized antenna modelling | Following the Clause 7.3.2 of TR38.901 |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the UT | Case 1: one hand grip Case 2: dual hand grip Case 3: head and one hand grip Note: For each UE, the selection of case is determiend by the probability defined in Table 7.5.14-2. |
| Metric | - CDF of coupling loss (serving cell) received per UT port - CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

Agreement

The following assumption is used for BS side SNS calibration:

| Parameter | Values |
| --- | --- |
| Scenarios | UMi-street Canyon |
| Carrier Frequency | 7 GHz (optional) 15 GHz |
| Bandwidth | 20 MHz |
| BS Antenna height | 10m for UMi-Street Canyon |
| BS Tx power | 44 dBm for UMi-Street Canyon |
| BS Antenna Configuration and port mapping | Config1 for 7GHz: - (M, N, P, Mg, Ng; MP, NP) = (24, 32, 2, 1, 1; 8, 32), dH = 0.5λ, dV = 0.7λ; - (optional) (M, N, P, Mg, Ng; MP, NP) = (64, 16, 2, 1, 1; 16, 16), dH = dV = 0.5λ Config2 for 15 GHz: - (M, N, P, Mg, Ng; MP, NP) = (64, 32, 2, 1, 1; 8, 16), dH = dV = 0.5λ Mp and Np are the number of vertical, horizontal TXRUs within a panel and polarization |
| BS Polarized antenna modeling | Model-2 in Clause 7.3.2 of TR38.901 |
| UT Antenna Configuration | Config 1: 4 antenna port with single/linear polarization for calibration based on handheld device antenna model using candidate antenna locations (1,7,3,5) as described in Clause 7.3. |
| UT antenna pattern | Isotropic |
| UT Polarized antenna modelling | Following the Clause 7.3.2 of TR38.901 |
| UT attachment | Based on RSRP (formula) from BS port 0 |
| UT distribution | Following TR36.873 for UMi, (3D dropping) |
| Spatial non-stationarity modeling at the BS | Case 1: Physical blocker-based approach (e.g., K = [1 per sector]) Case 2: Stochastic-based approach |
| Metric | 1) CDF of coupling loss (serving cell). 2) CDF of the ratio between the 2nd, 3rd ,4th, ..., xth (smallest) PRB singular value and the 1st PRB (largest) singular value (serving cell) at t=0 plotted in 10*log10 scale. - Note-1: The PRB singular values of a PRB are the eigenvalues of the mean covariance matrix in the PRB. - Note-2: The value of x is the minimum value of (number of BS antenna ports, number of UT antenna ports) |

### RAN1 #121 (May-2025)

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted:

- To calculate the power attenuation factor for the clusters with SNS, the definition of xs, ys in RAN1#120bis agreement is revised as:

  - The xs, ys denotes the coordinate of the antenna element out of VR region.

- To generate the visibility region, the Pn refers to the power of cluster generated by (7.5-6) in dB scale. In the case of LOS condition, the LOS path is considered as an additional cluster, and the power ratio of the LOS path to NLOS clusters follows the Ricean K-factor generated in Section 7.5.

Agreement

The following value in the agreement of RAN1#120bis is revised as:

- The probability of VR at the upper part of antenna array is [0.8] 0.5 and the probability of VR at the lower part of antenna array is [0.2] 0.5;

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, the following parameters are adopted:

| Scenario | A | B | R | δ | C | PrSNS(μ,σ2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | μ | σ |
| UMa | 0.15 | 0.45 | 33 | Nμ,σ2~N0, 0.0015 | 13 | 0.56 | 0.20 |
| UMi | 0.12 | 0.48 | 50 | N(μ,σ2)~ N(0, 0.001) | 13 | 0.49 | 0.18 |
| InH | 0 | 0.60 | NA | N(μ,σ2)~N0, 0.0011 | 13 | 0.31 | 0.08 |

Agreement

To calculate the absolute time of arrival for d1 and d2, the Δτ refers to the excess delay, which is only applicable

- when it’s not in the LOS case, and generated according to the Section 7.6.9, otherwise Δτ is assumed to be 0.

Agreement

For the modelling of spatial non-stationarity at UE side, the value of the attenuation values for “Head and one hand grip” in the agreement made in RAN1#120bis is revised in Red:

| Antenna index | Power attenuation (dB) |  |  |
| --- | --- | --- | --- |
|  | Head and one hand grip |  |  |
|  | Below 1 GHz | (1-8.4) GHz | 14.5-15.5 GHz |
| 1 | - | 3.7 | 4.0 |
| 2 | - | 4.1 4.6 | 3.7 |
| 3 | - | 4.3 | 3.3 |
| 4 | 15.1 | 7.8 | 3.3 |
| 5 | - | 11.7 | 3.3 |
| 6 | - | 10.1 | 3.7 |
| 7 | - | 2.9 | 4.0 |
| 8 | 4.9 | 4.2 | 4.0 |

Agreement

For the modelling of spatial non-stationarity, if the unified visible probability and visibility region based approach is adopted, the following parameters are adopted:

| Scenario | A | B | R | δ | C | PrSNS(μ,σ2) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | μ | σ |
| RMa | 0.16 | 0.74 | 60 | Nμ,σ2~N0, 0.0016 | 13 | 0.14 | 0.08 |
| SMa | 0.06 | 0.56 | 23 | Nμ,σ2~N0, 0.0013 | 13 | 0.24 | 0.07 |
| InF | 0 | 0.57 | NA | Nμ,σ2~N0, 0.002 | 13 | 0.32 | 0.06 |

Agreement

For the following agreement made in RAN1#121, it’s to confirm that the “new blocker type/size” is only applicable for SNS.

Agreement For the modelling of spatial non-stationarity, if physical blocker-based approach is adopted, at least for blockage model B, the following new blocker type/size can be introduced in the Table 7.6.4.2-5 in TR 38.901: Typical set of blockers; Blocker dimensions; Mobility pattern. Outdoor; Billboard; Cartesian: w=2.4m; h=3.6m; Stationary. Outdoor; Street lamp; Cartesian: w=0.4m; h=0.8m; Stationary. Outdoor; Building edge; Cartesian: w=X m; h=Y m; Stationary. Indoor; Pillar; Cartesian: w=0.3m; h=3m; Stationary. - FFS: the value of X and Y for the blocker dimensions of building edge is needed. - FFS: the details related to the user hand/head: Indoor;Outdoor; FFS: User hand; Cartesian: w=[0.2]m; h=[0.1]m; Stationary. Indoor;Outdoor; FFS: User head; Cartesian: w=[0.24]m; h=[0.20] m; Stationary. - FFS:The details of blockage model B to implement the impact of user hand and head. - FFS: The location of the user hand/head

Agreement

For the modelling of SNS, capture the following observation & recommendation in the CR:

- In Rel-19 Channel Model study,

  - the VR/VP based approach is recommended to be used in the simulation except for the case where it is not applicable

- From the perspective of the simulation requirements:

  - The physical blocker based approach can be considered for simulation where physical accuracy and consistency is desired.

  - The visibility region-based approach (i.e., Stochastic-based approach) can be considered for simulation where computational efficient SnS modelling is desired.

- From the perspective of the model mechanism:

  - To reflect the SNS phenomenon due to partial blockage, the physical blocker-based approach can be considered in simulation.

  - To reflect the SNS phenomenon due to incomplete scattering, the visible probability and visibility region-based approach (i.e., Stochastic-based approach) can be considered in simulation.

Agreement

For the terminology used in the CR:

- Keep the terminology “Near field channel model” with following updates, capturing the following TP in section 6.4:

< Unchanged text omitted > - Support near-field channel propagation (i.e., characteristics of spherical wavefront) and spatial non-stationarity < Unchanged text omitted >