3GPP TSG RAN WG1 #121	R1-2504705

St Julian’s, Malta, May 19th – 23th, 2025

Title 	: Discussion on ISAC channel model calibration

Source 	:  NTT DOCOMO, INC.

Agenda item	:  9.7.1

Document for:  Discussion and Decision

- Introduction

In RAN1 #120bis, two agreements on simulation assumptions for large scale and full calibration for UAV sensing targets are endorsed [1]. In this contribution, we provide our large scale calibration results for UAV sensing targets.

- Discussion on ISAC channel model calibration

In RAN1 #120-bis meeting, an agreement on simulation assumptions for large scale calibration for UAV sensing targets is endorsed as shown in Table 1.

Table 1 Simulation assumptions for large scale calibration for UAV sensing targets

| Parameters | Values |
| --- | --- |
| Scenario | UMa-AV |
| Sensing mode | TRP monostatic, TRP-TRP bistatic, TRP-UE bistatic, UE-UE bistatic |
| Target type | UAV of small size (0.3m x 0.4m x 0.2m) |
| Sectorization | Single 360-degree sector can be assumed |
| Carrier Frequency | FR1: 6 GHz FR2: 30 GHz |
| BS antenna configurations | Single dual-pol isotropic antenna |
| BS Tx power | FR1: 56dBm FR2: 41dBm |
| Bandwidth | FR1: 100MHz FR2: 400MHz |
| BS noise figure | FR1: 5dB FR2: 7dB |
| UT antenna configurations | Single dual-pol isotropic antenna; (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1) |
| UT noise figure | FR1: 9dB FR2: 10dB |
| UT height | 1.5m for terrestrial UTs, |
| UT Tx power | 23dBm |
| UT Distribution | • The overall number of UTs is 30 uniformly distributed in the center cell. • All of the UTs are either terrestrial UTs or aerial UTs, all outdoors. • Vertical distribution of aerial UE: Fixed height value of 200 m. • FR1 is assumed for aerial UE. |
| Sensing target distribution | 1 target uniformly distributed (across multiple drops) within the center cell. Vertical distribution: Fixed height value of 200 m. |
| Component A of the RCS for each scattering point | -12.81 dBsm |
| Minimum 3D distances between pairs of Tx/Rx and sensing target | 10 m |
| Wrapping Method | No wrapping method is used if interference is not modelled, otherwise geographical distance based wrapping |
| Coupling loss for target channel | power scaling factor (pathloss, shadow fading, and RCS component A included): LTX-SPST-RX=PLdBd1+PLdBd2+10lgc24πf2-10lgσRCS,A+SFdB,1+SFdB,2 |
| Sensing Tx/Rx selection | Best N = 4 Tx-Rx pairs to be selected for the target. NOTE1: Based on the Tx-Rx pairs with the smallest power scaling factor of the target channel. |
| Metrics | Coupling loss for target channel Coupling loss for background channel (in case of monostatic sensing, this is the coupling loss between Tx and one reference point) Note: CDFs can be separately generated for target channel, background channel |

Based on these assumptions, we performed a calibration, and the results are shown in Fig. 1, 2 and 3.

(a) FR1                                                                           (b) FR2

Fig. 1. Large scale coupling loss for TRP-TRP bistatic target channel for UAV

(a) FR1                                                                           (b) FR2

Fig. 2. Large scale coupling loss for TRP-UE (terrestrial UE) bistatic target channel for UAV

(a) FR1                                                                           (b) FR2

Fig. 3. Large scale coupling loss for TRP-UE (aerial UE) bistatic target channel for UAV

Proposal 1: Our large scale calibration results for TRP-TRP bistatic and TRP-UE bistatic target channel for UAV sensing targets shown in Fig. 1, 2 and 3 should be considered and incorporated in the TR.

- Conclusion

In this contribution, we provide our large scale calibration results for UAV sensing targets and the following proposal is made:

Proposal 1: Our large scale calibration results for TRP-TRP bistatic and TRP-UE bistatic target channel for UAV sensing targets shown in Fig. 1, 2 and 3 should be considered and incorporated in the TR.

References

[1] R1-2502733, FL Summary #3 on ISAC Scenarios and Calibrations.