3GPP TSG RAN WG1 #121			R1-2504752

St Julian’s, Malta, May 19th – 23th, 2025

Source: 	Moderator (CMCC)

Title:	Summary #4 for coding aspects of physical channel design

Agenda:	9.4.2

Document for:	Discussion & Decision

Introduction

In RAN#107 meeting, a revised work item on solutions for Ambient IoT (Internet of Things) in NR has been approved [1], in which a new A-IoT technology focusing on deployment scenario 1 with topology 1 (D1T1) for device 1 and with carrier wave (CW) node outside topology (-B transmission case), is recommended for standardization.

The following RAN1-led objectives have been identified within the General Scope described in the WID:

- RAN1 scope: - PRDCH and PDRCH, which are the only physical channels in R2D and D2R, respectively. - R2D and D2R signal(s) - Multiplexing/multiple access in R2D is by only TDMA, and in D2R is by only TDMA and FDMA. - R2D supports only OOK-4 modulation, one solution for CP handling. D2R backscattering supports only OOK and BPSK modulations. - R2D transmission supports only the Manchester line code in TR 38.769 - D2R transmission supports: - Either the Manchester line code in TR 38.769 or no line code (one to be down-selected); and - A corresponding small frequency shift method according to the options in TR 38.769. - R2D does not support FEC. D2R supports only convolutional code with generator polynomials as per TS 36.212. Applying or not applying the FEC to D2R is specified by ensuring it is under the reader control and applies to all devices targeted by the reader. - PRDCH and PDRCH both support transmission without CRC, and with CRC as per the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC. Cases to use which length of CRC, or no CRC, to be decided in RAN1. - D2R supports physical layer repetition transmission. R2D does not support physical-layer repetition transmission.

This contribution provides a summary of the submitted contributions, issues for discussion, and outcomes in RAN1#120bis meeting.

Online/offline proposals

Tuesday offline

[H] Proposal 3.1-1-v2

For D2R transmission, support the following values of Tchip and R, satisfying Tb = Tchip×2R:

- The D2R chip duration Tchip are {133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, [1.04μs], 0.69μs}.

- The SFS factor R are {1, 2, 4, 8, 16, 32, 64, [128]}.

- The minimum D2R bit duration Tb is 1.39μs, and the maximum D2R bit duration Tb is 266.67μs.

Majority views want to formulate the proposal in a table format.

No implication to let the device to pre-store.

No down-selection on Tchip and R: CATT

For down-selection of min Tb to 1.39 us, Samsung and Xiaomi has strong concern.

[M] Proposal 4.2-1-v2

- For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, e.g., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dK-10,dK-11,dK-12,  with code rate of 1/3.

[H] Proposal 6.1-v1

- For attaching the CRC, the parity bits are appended to the end of the input bits.

[H] Proposal 3.3-1-v2

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, down-select from the following options:

- Opt. 1:

  - Opt. 1-1: Reader indicates an index of a combination of {Tb, Tchip, R} in the table for Tb, Tchip, and R for each frequency domain resource.

  - Opt. 1-2: Reader indicates a Tb. Device randomly selects a Tchip (or R) from the corresponding set.

  - Opt. 1-3: Pre-define a table where each row refers to a set of combinations of {Tchip, R}. Reader indicates an index.

- Opt. 2: Reader indicates the values of Tchip and R in a combination for each frequency domain resource.

- Opt. 3:

  - Opt. 3-1a: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is Ymax.

  - Opt. 3-1b: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is determined by the indicated Tchip,ref.

  - Opt. 3-2a: Reader indicates a Tb and a set of R using bitmap. The length of bitmap is Ymax.

  - Opt. 3-2b: Reader indicates a Tb and a set of R using bitmap. The length of bitmap is determined by the indicated Tb.

  - Note: Device needs to compute the actual D2R chip duration Tchip = Tb /(2R) or Tchip = Tchip,ref /R. The computation is simple only if R=2n.

- Opt. 4:

  - Opt. 4-1: Reader indicates a Tb and Y.

    - Opt. 4-1a: Device determines Rmax based on the indicated Y. Device randomly selects R starting from the smallest R to the determined Rmax.

    - Opt. 4-1b: Device determines a set of R based on reader indication of the starting value of R and gap value of R. Device randomly selects R from the set.

  - Opt. 4-2: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and Rmax.

  - Opt. 4-3: Pre-define a table where each row refers to a reference Tchip,ref (i.e., Tchip when R = 1) and a corresponding set of R. Reader indicates an index.

  - Opt. 4-4: Pre-define a table where each row refers to a combination of {Tb, a subset of R}, and the subset of R avoids potential interferences among devices. Reader indicates an index.

  - Opt. 4-5: Pre-define a table for each Tb where each row refers to a set of R. Reader indicates Tb and a row index.

  - Note: Device needs to compute the actual D2R chip duration Tchip = Tb /(2R) or Tchip = Tchip,ref /R. The computation is simple only if R=2n.

Note: The indication signaling details will be left to RAN2 design if RAN1 cannot reach any consensus.

Tuesday online

[H] Proposal 3.1-1-v3

For D2R transmission,

- The minimum Tb is 1.39μs.

- Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis.

|  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

Note: This does not imply the device to pre-store the table.

[M] Proposal 4.2-1-v2

- For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, e.g., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dK-10,dK-11,dK-12, with the code rate of 1/3.

[H] Proposal 6.1-v1

- For attaching the CRC, the parity bits are appended to the end of the input bits.

Wednesday online

      - [H] Proposed conclusion 4.1-1-v1

When FEC is applied, only 1/3 code rate is supported.

      - [H] Proposal 5.1-v2

For the number of block-level repetitions, {1, 2} are supported.

- FFS number of repetitions 4 is supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

Thursday online

      - [H] Proposal 3.3-2-v3

For frequency domain resource for Msg 3 transmission is determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, the reader indicates:

- a single bit duration, which is same for all frequency domain resources,

- a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R

  - note: the set of R values could be signalled using a bitmap

The mapping relationship between device and its Msg 3 frequency domain resource is left to RAN2.

  - Note: Device could determine its R value for Msg 3 transmission based on its order of random ID in Msg 2

The detailed signalling design is left to RAN2.

Small frequency shift (SFS)

Background

For D2R transmission, the following parameters and relationships are considered:

- Tb: Time duration corresponding to a bit, where the bit is that after FEC if applied.

- Tchip: Chip duration, where Tb = Tchip×2R.

- R: SFS factor, where the small frequency shift location is determined by ± (R/Tb = 1/2/Tchip) Hz.

- Btx,D2R: D2R transmission bandwidth, where Btx,D2R = 4/Tb = 2/(R*Tchip).

Potential values of Tb, Tchip, and R are recapped as follows:

|  | Freq. shift to CW =1/2Tchip | 3.75 kHz | 7.5 kHz | 15 kHz | 30 kHz | 45 kHz | 60 kHz | 90 kHz | 120 kHz | 180 kHz | 240 kHz | 360 kHz | 480 kHz | 720 kHz | 960 kHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Btx,D2R =4/Tb | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 15 kHz | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  |  |
| 30 kHz | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 60 kHz | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 120 kHz | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 180 kHz | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 240 kHz | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 360 kHz | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 480 kHz | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 720 kHz | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 960 kHz | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 1440 kHz | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 1920 kHz | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 2880 kHz | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 3840 kHz | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

(CLOSED) D2R bit duration, D2R chip duration, SFS factor R

Summary of inputs

In the table of potential values of Tb, Tchip, and R that agreed in the last RAN1 meeting, it has been concluded that the bit duration 266.67μs, 133.33μs and 66.67μs are supported to provide better multiplexing capability in the frequency domain, apart from this, the following remaining issues are still open for further discussion:

#### Issue #1: Down-select between 1.39μs and 1.04μs for min Tb

It is mentioned that both 1.39μs and 1.04μs can achieve a competitive peak data rate higher than 640 kbps. Qualcomm analyzes that both 1.39μs and 1.04μs is feasible to be achieved by clock speed of 1.92 MHz, 2.16 MHz, and 2.88 MHz with a total error less than 12% caused by residual SFO and/or non-integer number of samples. Proponents of the two values and rational behind are summarized as follows:

- 1.39μs (12): Futurewei, Huawei, ZTE, vivo, CATT, CMCC, China Telecom, OPPO, LGE, Qualcomm, Sharp, Docomo. Proponents of 1.39μs provide at least the following reasons:

  - 1.39μs can be supported by a sampling clock (e.g., 2.88 MHz) suitable from both R2D reception perspective and D2R transmission perspective without additionally introducing non-integer counting errors.

  - 1.04μs requires higher sampling rate and leads to higher device cost and power consumption.

  - 1.04μs requires larger bandwidth which is not friendly in practical deployment.

- 1.04μs (6): Spreadtrum, Samsung, Panasonic, Xiaomi, InterDigital, Apple. Proponents of 1.04μs provide at least the following reasons:

  - 1.04μs can be supported by a sampling clock (e.g., 1.92 MHz) which was historically used in 3GPP system.

  - 1.04μs is a multiple division of 2n corresponding to 266.67μs.

##### FL comments

From the inputs, there is a clear majority to support 1.39μs, which can be achieved by a sampling clock suitable from both R2D reception and D2R transmission perspective, and provides a competitive peak data rate higher than 640 kbps with a smaller bandwidth for deployment. Therefore, FL proposes down-selecting to 1.39μs.

#### Issue #2: Determine the supported Tb other than 266.67μs, 133.33μs, 66.67μs, 1.39μs and 1.04μs

- 4 companies (Futurewei, Huawei, CATT, China Telecom) propose to support the full set of Tb and no further down-selection is needed. On the other hand, 12 companies (ZTE, vivo, Spreadtrum, Samsung, Panasonic, CMCC, Xiaomi, OPPO, InterDigital, Apple, Qualcomm, Docomo) propose to down-select Tb. Most companies select 8 or 9 values out of the full set while Apple selects only 4 values out of the full set.

- Companies’ views on remaining Tb are summarized as follows:

Table 3.1.1-1: Summary of proposed Tb other than 266.67μs, 133.33μs, 66.67μs, 1.39μs and 1.04μs

| Tb (μs) | Source |
| --- | --- |
| 33.33 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, Samsung, Panasonic, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Qualcomm (for R2D M = 2), Docomo |
| 22.22 | Futurewei, Huawei, CATT, China Telecom, Qualcomm (for R2D M = 6, 12, 24) |
| 16.67 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, Samsung, Panasonic, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Docomo |
| 11.11 | Futurewei, Huawei, CATT, China Telecom, Qualcomm (for R2D M = 6, 12, 24) |
| 8.33 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, Samsung, Panasonic, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Docomo |
| 5.56 | Futurewei, Huawei, CATT, China Telecom, Qualcomm (for R2D M = 6, 12, 24) |
| 4.17 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, Samsung, Panasonic, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Docomo |
| 2.78 | Futurewei, Huawei, CATT, China Telecom |
| 2.08 | Futurewei, Huawei, Samsung, Panasonic, CATT, Xiaomi, China Telecom, InterDigital, Docomo |

##### FL comments

From the inputs, some companies suggest not pursuing any down-selections on Tb (and also Tchip and R). It may be a possible way forward if in the end companies cannot converge on which values to support or not to support. But note that two adjacent Tb in the table provide a small gap in terms of bit rate and transmission bandwidth, and therefore, it seems not necessary to support every Tb from deployment perspective. FL suggests RAN1 discusses some down-selections in the first place. There is 1 company (Qualcomm) suggests choosing different set of Tb (and Tchip) for different R2D M values. To be specific, for M = 2, support 266.67μs, 133.33μs, 66.67μs, 33.33μs, and for M = 6, 12, 24, support 22.22μs, 11.11μs, 5.56μs, 2.78μs. The rationale is that the D2R bit/chip duration and R2R chip duration has a relationship of 2n, and it is more easily for device to generate D2R chip by doubling or splitting samples from R2D chip. FL understands the intention but it does not necessarily mean that only ratio of 2n is required for device to calibrate D2R clock from R2D CAP. For example, if R2D M = 6 (11.11μs per chip), there are 32 samples per chip assuming a 2.88 MHz clock. For a D2R chip duration of 33.33μs, there are 96 samples per chip, the ratio of samples per D2R chip to R2D chip is 3. The device can simply calibrate the samples of D2R chip using a shifter-adder, as 3 = 21+20. Therefore, the values with more proponents are used as a starting point (i.e., 33.33μs, 16.67μs, 8.33μs, 4.17μs, [2.08μs]) to further discuss the supported Tchip and R.

Please note that RAN1 has already agreed to indicate Tchip from a pre-defined set, so FL formulates the proposal regarding values of Tchip and R (see Proposal 3.1.1-v1).

#### Issue #3: Determine the supported Tchip

- Companies’ views on Tchip are summarized as follows:

Table 3.1.1-2: Summary of proposed Tchip

| Tchip (μs) | Source |
| --- | --- |
| 133.33 | Futurewei, Huawei, ZTE, vivo (in square bracket), Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, LGE, InterDigital, Apple, Docomo |
| 66.67 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 33.33 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 16.67 | Huawei, ZTE, vivo, Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 11.11 | Huawei, TCL, CATT, China Telecom, |
| 8.33 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 5.56 | Huawei, TCL, CATT, China Telecom, Qualcomm (for R2D M = 6, 12, 24) |
| 4.17 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 2.78 | Huawei, TCL, CATT, China Telecom, Qualcomm (for R2D M = 6, 12, 24) |
| 2.08 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, TCL, Samsung, CATT, CMCC, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 1.39 | Huawei, TCL, CATT, China Telecom, Qualcomm (for R2D M = 6, 12, 24) |
| 1.04 | Futurewei, Huawei, ZTE, vivo, Spreadtrum, Samsung, CATT, Xiaomi, China Telecom, OPPO, InterDigital, Apple, Qualcomm (for R2D M = 2), Docomo |
| 0.69 | Futurewei, Huawei, ZTE, vivo, CATT, CMCC, China Telecom, OPPO, LGE, Qualcomm (for R2D M = 6, 12, 24), Sharp, Docomo |
| 0.52 | Spreadtrum, Samsung, Xiaomi, InterDigital, Apple, Qualcomm (for R2D M = 2) |

#### Issue #4: Determine the supported R

- Majority companies propose to support a subset of R values. On the other hand, 3 companies (Ericsson, Huawei, CATT) propose to support all R values in the table without any down-selections.

- For selecting a subset of R values, all 13 companies (Futurewei, ZTE, vivo, Spreadtrum, TCL, Samsung, CMCC, China Telecom, OPPO, LGE, Apple, Qualcomm, Docomo) propose to select values satisfying R = 2n, with at least the following reasons:

  - Avoidance of potential interferences of odd harmonics between multiplexed devices.

  - Much easier for device to generate values satisfying 2n using shift register.

- Furthermore, some companies further discuss restrictions on the R values:

  - R = 2 is not supported: Futurewei, Spreadtrum, NEC, OPPO, Apple. It is mentioned by companies that there is no sufficient gap in the frequency domain between R = 1, 2, and 4, and suggest precluding R = 2 from the set.

  - R = 1 is only used for non-FDMA case (i.e., Y=1) and not supported for FDMA case: ZTE, Qualcomm. ZTE mentions that the odd harmonics of R = 1 will cause interference to the main lobes of R = 2, 4, 8… and suggest precluding R = 1 in case of FDMA.

  - R = 1 for max Tchip = 133.33μs is FFS: vivo. The max Tchip should avoid too small frequency shift which suffers serious CW interference.

  - R > 32 for Ymax = 4 or R > 64 for Ymax = 6 is not supported: CMCC. Given a Tb, larger R values lead to smaller chip durations which have no benefit for the D2R link performance. Support Rmax = 32 or Rmax = 64 can provide a multiplexing capability of Ymax = 4 or Ymax = 6, respectively, which is sufficient from system perspective.

##### FL comments

For a given Tb, Tchip and R that to be supported are related. Therefore, for the suggested Tb of 266.67μs, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, [2.08μs] and 1.39μs, the corresponding values of Tchip when R = 1, i.e., 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, [1.04μs], 0.69μs, are naturally supported. 1 company (vivo) has concern on supporting Tchip =133.33μs, but there are 15 companies propose this value and do not think it is problematic for a gNB reader to handle the phase noise. Therefore, FL doesn’t see the need to preclude 133.33μs and R = 1 from specification perspective.

With the above values of Tchip, at least R=2n up to 128 can be supported, which is also proposed by majority views. In addition,

- On whether to support other R values satisfying R=2n and R≠2n: With values of R=2n, it can already support up to 7 devices multiplexed in the frequency domain, which seems sufficient from system perspective.

- On whether to preclude certain R values from the set of R=2n: Some companies suggest precluding R = 2 as the main lobe overlaps with that of R = 1 and R = 4, while some other companies suggest precluding R = 1 as the odd harmonics overlaps with main lobes of other R values. From specification perspective, FL doesn’t see the need to restrict the reader scheduling flexibility. For a reader capable of handling phase noise, it can select R = 1, 4, 8, …, but for a reader incapable of handling phase noise, it can use R = 2, 8, 16, … Regarding the concerns on the odd harmonics of R = 1 overlaps with main lobes of other R values, some companies provide simulation results using R = 1, 4, 8, … and it does not show any problem in D2R performance [24]. In fact, as long as the main lobes of different R values are non-overlapped, the reader can first filter out the main lobe of one device, reconstruct the odd harmonic components and cancel the corresponding interference. The reader can then filter out the main lobe of the other device.

Therefore, FL proposes R=2n as a starting point, and no value is precluded from the set.

Round 1 discussion

      - [H] Proposal 3.1-1-v1

For D2R transmission, support the following values of Tchip and R, satisfying Tb = Tchip×2R:

- The D2R chip duration Tchip are {133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, [1.04μs], 0.69μs}.

- The SFS factor R are {1, 2, 4, 8, 16, 32, 64, [128]}.

- The minimum D2R bit duration Tb is 1.39μs, and the maximum D2R bit duration Tb is 266.67μs.

| Company | Comments |
| --- | --- |
| FL | Please note that from majority views, the current proposed D2R chip duration has 9 values, which implies that 1 bit will be additionally added for indication of bit/chip duration. FL adds a square bracket on 1.04μs, which has less supporters than other values, companies please provide your views on whether this value can be removed or not. |
| FUTUREWEI | For the D2R durations, we are flexible for the values as long as we target at most 8 values for signaling considerations. We indicated support for 0.69us based on having R=96. We would be fine dropping 0.69us and keeping 1.04us based on the supported R values. We can support 128 for the R value. Our observation is that it may be possible to consider joint coding if there are fewer than 32 combinations of Tchip and R values. |
| OPPO1 | Support. We suggest to support Tchip=1.04μs and R=128 that are currently in the square brackets. |
| Spreadtrum | For the Tb, we support the minimum D2R bit duration Tb is 1.04μs, as it can be supported by a sampling clock (e.g., 1.92 MHz) which was historically used in 3GPP system. The value of Tb should be list in the proposal too, Tb = {266.67, 133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 2.08, 1.04}μs. For the chip duration, we support the 1.04μs and 0.52μs. For the SFS factor R, we support the maximum value of R is 256 based on the agreement in the last meeting. |
| Panasonic | For Tchip, we suggest considering 1.04μs or 0.52μs instead of 0.69μs as the shortest chip duration, so the higher chip durations can be derived by integer multiplication of 1.92 MHz clock. If we take 0.69 us, our preference is not mandating all devices. The device only supporting 0.69us should response Paging via Msg1. Then either of the devices based on 1.92 MHz clock and the device based on 2.88 MHz sampling clock are possible. In addition, the device not supporting 0.69 us could reduce the power consumption by using slower circuit and lower voltage design. |
| vivo | We support 1.04us without bracket, and also support R= 128 without bracket. Regarding FL’s comment on additional 1 bit to support 9 values of {133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, [1.04μs], 0.69μs}, it highly depends on the detailed signalling design, i.e., whether Tb, reference Tchip is indicated or the actual Tchip is indicated. Whether additional 1 bit is needed, depends on how to indicate the bit or reference chip duration. If bit duration is indicated, 3 bit is sufficient to indicate {266.67, 133.33, 66.67, 33.33, 16.67, 8.33, 4.17 and 1.39}us which can support 9 values of chip duration as shown below. Alternatively, if reference chip duration with R=1 is indicated, still, 3 bit is sufficient, because 1.04us Tchip does not support R=1, the reference chip duration candidate is only 8, but the actual chip duration of 1.04us is still possible with R>1. Only if actual chip duration is indicated, then, 4 bits is needed to support 9 values. However, as pointed out by FL in section 3.3.1, the signalling overhead reduction would be the next step optimization. Therefore, we prefer to keep 1.04us and R=128 without bracket. If we have to go with 8 candidate chip duration, we prefer to delete 133.33 us, due to poor performance of very small frequency shift close to CW suffering CW interference. Please note, not only vivo, but also Qualcomm do not support 133.33us. |
| HONOR | Fine with the proposal |
| Huawei, HiSilicon | In our view, apart from the selection of the minimum bit duration of 1.39μs captured in the last bullet, the motivation for further down-selection of the R and Tcjip values is not clear, since signalling overhead is not a limiting factor as it uses higher layer signalling. If we are to down-select from the values, in the first bullet, we prefer to keep Tchip=1.04μs since it would enable the use of R=128, giving the devices another frequency resource to transmit on. On the R values in the second bullet, if we are selecting a smaller list, then we would prefer to retain R=128, and remove R=2. While it is possible for the reader to avoid choosing R=2 along with R=1 or R=4 due to overlapping of the main lobes, since we are selecting a smaller set, it makes sense to avoid the use of such R values that would cause main lobe overlaps, and it does not add to the reader scheduling flexibility, but would rather be a restriction, since the reader cannot use R=2 if it has a maximum of 7 devices to be FDMed. We are fine with the last bullet. |
| Samsung | We still prefer 0.52μs as the minimum chip duration. It offers several advantages — for example, it aligns well with binary-based timing (since it's a 2ⁿ division of other chip durations) which makes it easier to implement using simple logic like binary dividers. As for concerns about power consumption and bandwidth: we believe they won’t be a critical issue in practice. The minimum chip duration is mainly needed when supporting the peak data rate, which won’t be the case most of the time. In typical system configurations, longer chip durations will likely be used, so the actual power impact would be quite limited. Also, for Rel-19, we’re focusing on fairly closed network environments where coexistence isn’t a big concern. So we don’t see the slightly larger bandwidth requirement of 0.52μs as a serious drawback — especially since this only matters when aiming for the highest possible data rate. |
| ZTE, Sanechips | Since the A-IoT devices support SFS as a mandatory ability, there is no need to choose R=1 which is with CW-interference problem and harmonic interference issue with other R values. Therefore, other R values are preferred for FDMA. In our understanding, R=1 can be used to achieve the peak rate for a single device, where the shifted frequency is slightly larger and no impact on devices. Besides, the maximum value of R is related to the multiplexed number Y, which is related to Proposal 3.2-1-v1. Therefore, we suggest to firstly draw the conclusion that SFS factor R are 2n in this proposal, while the specific values are discussed with proposal 3.2-1-v1. As for the D2R chip durations, 133.33μs can be used only along with R=1 for Tb=266.67μs, while there are sufficient combinations of Tchip and R that can be used for Tb=266.67μs. Furthermore, Tchip=133.33μs along with R=1 suffers the CW-interference problem. Thus, Tchip=133.33μs is suggested to be precluded. Besides, Tchip=1.04μs is naturally suitable for the R being 2n , so Tchip=1.04μs is suggested to be kept since it can be used for multiple Tb, especially for small Tb (e.g., 16.67μs, 8.33μs, 4.17μs) to increase the number of multiplexed devices. However, if we have to consider candidate R value set and max Y value together in this proposal, it should be clarified that the minimum Tchip is 1.04us along with the maximum value of R being 128, 64 or 32, instead of 0.69us shown in Table 3.2.1-1. Besides, since the supported minimum Tb is 1.39us along with the minimum Tchip being 0.69us, and Tchip=1.04μs, which is larger than 0.69us, is naturally suitable for the R being 2n, there is no reason not to support Tchip=1.04us. Furthermore, it’s okay for the devices to supported R=128. Consequently, R=128 should be supported for the maximum number of multiplexed devices. Therefore, we suggest to update the proposal as below: For D2R transmission, support the following values of Tchip and R, satisfying Tb = Tchip×2R: - The D2R chip duration Tchip are {[133.33μs], 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, [1.04μs], 0.69μs}. - The SFS factor R are 2n , e.g. {1, 2, 4, 8, 16, 32, 64, [128]}. - The minimum D2R bit duration Tb is 1.39μs, and the maximum D2R bit duration Tb is 266.67μs. |
| Xiaomi | For the minimum D2R bit duration, 1.04us is supported according to the sampling clock of 1.92 MHz. The minimum chip duration is correspondingly 0.52us. The maximum SFS factor R is 256. Considering about the effort on indication and how many values should be kept as the candidate chip/bit duration value set, it is related to indication method. It can be discussed and determined together with indication method. |
| QC | Supported R value We think R=2 could be always used instead of R=1. It is far enough from CW and there is no need to support together with R=1. R=1 is necessary only for the highest bit rates. For all other cases for FDM or non-FDM, R=2 could be starting position. Tb {266.67us, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, 1.39us}. It would be good to have 8 different Tb values for 3bit indication. Tchip {66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, 1.04μs, 0.69μs, 0.52us}. The number of Tchip or R do no need to be 2^n bits for bitmap or index-indication based method. No need to capture relation between Tb and Tchip and R since they are already determined by table. - [H] Proposal 3.1-1-v1 For D2R transmission, support the following values of Tchip and Tb and R, satisfying Tb = Tchip×2R: - The D2R chip duration Tchip are {133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, 1.04μs, 0.69μs, 0.52us}. - The D2R bit duration Tb are {266.67us, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, 1.39us}. - The SFS factor R are {1, 2, 4, 8, 16, 32, 64, [128]}. - The minimum D2R bit duration Tb is 1.39μs, and the maximum D2R bit duration Tb is 266.67μs. |
| NEC | For D2R chip duration, {11.11, 5.56, 2.78, 1.39, 0.69} should also be supported. Besides, for R values, R=2 should be precluded in FDM due to insufficient gap. |
| DOCOMO | Support. For the values with brackets, we support both 1.04us for Tchip and 128 for R which can ensure the multiplexing capacity. |
| Sharp | We think it should be made clear that this proposal is intended to be a down-selection of the combinations of Tb, Tchip and R values agreed in the table in the last meeting, i.e. no new combination is intended (or implied) by this proposal. |
| FL | Some responses from FL: @Spreadtrum @Qualcomm @Xiaomi, FL agrees with what Sharp commented that this proposal is to down-select values from what we have agreed in the last meeting, and in the agreed table, there is no R = 256. I understand that R can be 256 if Tb = 266.67us and Tchip = 0.52us, but when we agree the table in the last meeting, no comments to add additional R values. Let’s try not to open the door to add new Tb, Tchip or R values. @Huawei @ZTE @Qualcomm, regarding R=2, Huawei (and also several companies proposed in their papers) wants to delete R=2 from the set, as the main lobes of R=1, 2 and R=2,4 are overlapped and cannot be used at the same time. In contrast, ZTE and Qualcomm want to delete R=1 except for the peak data rate case (i.e., Tb=1.39us and Tchip=0.69us), as R=1 brings potential CW interference problem and harmonic interferences to other R values. From FL’s perspective, it seems not possible for one group to convince the other, so suggest supporting both R=1 and R=2 in specification and let the reader do scheduling. @Qualcomm, per your comment that no need to specify the relationship but the table can tell. My current thinking is that, how the spec captures the supported values of Tb, Tchip or R in RAN1, whether shows in a table format or specifies the values and relationship, it can be up to the editor. In the agreement, FL formulates it in this way, not using a table as what we have done in the last meeting, is to avoid the implication that RAN1 agrees a table and the device has to pre-store such table. @Panasonic @Spreadtrum @Samsung @Xiaomi, regarding the minimum Tb, from the comments so far, 9 companies support this proposal (1.39us), 1 company can live with 1.04us. Still, majority view is to support 1.39us. The only benefit clarified by supporting 1.04us is that it can support R=256, which is not even in the agreed table in the last meeting. Even if this value can be added in this meeting, provide one more multiplexed device, the gain is esxpected to be marginal but it will consume a larger bandwidth. In addition, many proponents of 1.04us claims that device can use a sampling clock of 1.92 MHz, which is not appropriate from R2D reception part, the number of samples per each chip for M = 6, 12, 24 are non-integer. So, FL does not change the original proposal regarding the minimum Tb. In addition, from the comments so far, more companies prefer to remove the square bracket for Tchip = 1.04 and R = 128. Implement it in the updated proposal v2. - [H] Proposal 3.1-1-v2 For D2R transmission, support the following values of Tchip and R, satisfying Tb = Tchip×2R: - The D2R chip duration Tchip are {133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 2.08μs, [1.04μs], 0.69μs}. - The SFS factor R are {1, 2, 4, 8, 16, 32, 64, [128]}. - The minimum D2R bit duration Tb is 1.39μs, and the maximum D2R bit duration Tb is 266.67μs. |

(CLOSED) Number of multiplexed devices Y

Summary of inputs

Based on the contributions submitted in this meeting, the proposed maximum number of multiplexed devices in the frequency domain along with the maximum value of R and minimum Tchip are summarized as follows.

Table 3.2.1-1: Summary of proposed Ymax

| Ymax | Rmax | Tchip (μs) | Source |
| --- | --- | --- | --- |
| 8 | 128 | 0.69 | Futurewei, Huawei, China Telecom*, Honor*, LGE*, Sharp |
| 8 | 256 | 0.52 | Spreadtrum, Apple, Qualcomm (in square bracket) |
| 7 | 128 | 0.69 | ZTE, vivo, OPPO |
| 6 | 64 | 0.69 | CMCC |
| 4 | 32 | 0.69 | CMCC |

##### FL comments

From the inputs, although the three companies (China Telecom, Honor, LGE) proposes Ymax = 8, they also propose R values satisfying R=2n, since R = 1, 2, or R = 2, 4 have overlapped main lobes (assuming device SFO) and are not preferable to be used simultaneously, and hence only Ymax = 7 can be supported.

From the analysis and proposals in Section 3.1, with minimum Tchip = 0.69μs and R=2n, it is proposed to define Ymax = 7 if 1.04μs is supported.

Round 1 discussion

      - [M] Proposal 3.2-1-v1

The maximum number of devices to be multiplexed in the frequency domain is [7].

| Company | Comments |
| --- | --- |
| FUTUREWEI | The value will be based on what we select for R |
| OPPO1 | Agree with FL’s analysis/comments, we don’t think the upper limit of Y should be less than 7. It is also questionable there is a need to restrict the allowable Y to a lower number. If a reader only needs to schedule / multiplex a number of devices less than 7 in a time domain resource, it can indicate a smaller Y value. |
| Spreadtrum | As we comment in the Proposal 3.1-1-v1, we support the value of R up to 256, so the maximum number of devices to be multiplexed in the frequency domain is [8]. The specific value of R is {1, 4, 8, 16, 32, 64, 128, 256}. |
| Panasonic | Support |
| vivo | Support |
| HONOR | The detail number of used frequency resources can be configured but the maximum number should consider the maximum case of different available Tb. So we prefer the Ymax=8 |
| Huawei, HiSilicon | This decision would depend on the number of R values agreed. If we are supporting the R values with minimum or no down-selection, we prefer to support a maximum of 8 devices, and if we are considering a smaller set of R values using R=2n, then only a maximum of 7 devices can be supported. |
| Samsung | This value can be determined based on the selected R values, meaning that the maximum number of devices may vary depending on each Tb. |
| ZTE, Sanechips | First of all, it should be clarified that the minimum Tchip is 1.04us along with the maximum value of R being 128, 64 or 32, instead of 0.69us shown in Table 3.2.1-1. Besides, since the supported minimum Tb is 1.39us along with the minimum Tchip being 0.69us, and Tchip=1.04μs, which is larger than 0.69μs, is naturally suitable for the R being 2n, there is no reason not to support Tchip=1.04μs. As we commented in Proposal 3.1-1-v1, R=1 is not recommended for the multi-devices transmission since R=1 introduce harmonic interference to other R values. Furthermore, it’s okay for the devices to supported R=128. Consequently, R=128 should be supported for the maximum number of multiplexed devices. And the maximum number of devices to be multiplexed in the frequency domain is 7. |
| Xiaomi | The maximum number of R values can be supported in the system/table is [9] based on the previous proposal, {1,2,4,8,16,32,64,128,256} when supporting minimum bit duration of 1.04us. In real-time deployment, the maximum FDMed device at the same time is determined by reader by considering all the conditions such as interference, capacity and spectrum efficiency. The exact number of FDMed devices should be no larger than than [9]. Secondly, it is also related to indication method by considering indication method and overhead. At current stage, we think the maximum number of devices to be multiplexed in frequency domain is 9. |
| QC | The max Y could be decided after we determine Tchips or R values. We prefer 8 based on R=2,4,8,16,32,64,128,256. |
| Ericsson | The value will be dependent on the number of agreed R values |
| NEC | The maximum number will be obtained when the Tb-R-Tchip table is fixed. |
| DOCOMO | It depends on the supported values for R. |
| FL | Some responses from FL: @Samsung, my understanding of Ymax is the maximum multiplexing capability in the frequency domain for the system, which is achieved using the maximum Tb (i.e., minimum transmission bandwidth). @ZTE, with Proposal 3.3.1-v1 the relationship of Tb, Tchip and R in the main bullet, it is clear that when Tb = 266.67us, 133.33us, 66.67us, the corresponding Tchip is 1.04su for R=128, 64 or 32. @Xiaomi, even if we eventually agree to include R=256, Ymax should be 8, not 9, because R=1,2 or R=2,4 cannot be used at the same time due to the overlapped main lobes. Many comments mention that this proposal is dependent on the discussion on R values. True. Once we can have some progress on R values, this proposal is straightforward to support. Based on the v2 version of Proposal 3.1-1, the proposal is not changed, the number is still 7. |

(CLOSED) Indication signaling for FDMA resources when Y≥1

Background

As per Chair guidance, the indication signaling for frequency domain resources will be handled in this agenda item.

In previous RAN1 meetings, the following agreement regarding indication for frequency domain resource has been achieved:

Agreement The following is supported for indicating the D2R chip duration: - The D2R chip duration is indicated in R2D control information from predefined a set of D2R chip duration values

Note that the following agreement has a remaining FFS regarding Msg 3 resource allocation, based on coordination, related proposals and discussions are summarized and handled in AI 9.4.4:

Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

Summary of inputs

From reviewing the contributions in this meeting, Huawei proposes that RAN1 agrees the values of Tb, Tchip, and R (as discussed in Section 3.1), and the indication signaling will be handled by RAN2. However, all the other companies consider this discussion to be handled by RAN1 and provide design details in this agenda item.

Regarding the methodology for the indication signaling design, Panasonic and vivo propose to have a common design for Msg 1 and Msg 3 transmissions in case of FDMA and in case of non-FDMA to reduce complexity. As a contrast, many companies (e.g., ZTE, CATT, OPPO, Qualcomm) propose different solutions for Msg 1 and Msg 3 transmissions in case of FDMA, or in case of non-FDMA, to provide better scheduling flexibility or to reduce indication overhead. In addition, 3 companies (vivo, CMCC, Panasonic) mention a very important issue that, when RAN1 designs the signaling, the design should minimize or avoid the device to additionally store a pre-defined table. However, it seems that most companies entirely ignore this critical factor and treat signaling overhead reduction optimization as the first priority.

#### Issue #1: Indication signaling for Msg 1 transmissions when Y≥1

At least the following solutions are proposed by companies.

Table 3.3.1-1: Summary of proposed indication signaling for Msg 1 transmission when Y≥1

| Source | Solutions | Device storage | Device computation | Signaling overhead |
| --- | --- | --- | --- | --- |
| Futurewei, China Telecom, LGE, Docomo | Opt. 1-1: Device pre-stores the table of Tb, Tchip, and R. Reader indicates a combination entry of {Tb, Tchip, R} for each device. | YES. Device needs to pre-store a table as agreed in RAN1#120bis | NO. Device obtains the actual Tchip and R. | log2(N1)*Ymax. N1 is the total number of combinations in the table. |
| Samsung | Opt. 1-2: Reader indicates a Tb. Device randomly selects a Tchip (or R) from the corresponding set. | YES. Device needs to pre-store a table as agreed in RAN1#120bis. | NO. Device can obtain the actual Tchip and R from the pre-stored table. | log2(NTb) NTb is the total number of supported Tchip,ref or the total number of supported Tb. |
| ZTE | Opt. 1-3: Pre-define a table (a new table), where each row refers to a set of combinations of {Tchip, R}. Reader indicates an index to devices. An example is shown below: | YES. Device needs to pre-store a table, similar as the one shown in the example. | NO. Device obtains the actual Tchip and R. | log2(N2) N2 is the total number of indices in the table. |
| Ericsson, CMCC, Panasonic | Opt. 2: Reader indicates values of Tchip and R in a combination for each device. | NO. Device obtains the values of Tchip and R and no need to pre-store table. | NO. Device obtains the values of Tchip and R. | (log2(NTchip)+log2(NR))*Ymax NTchip is the total number of supported Tchip. NR is the total number of supported R. |
| vivo | Opt. 3-1: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R values using bitmap. The length of bitmap can be Ymax or determined by the indicated Tchip,ref. | NO. Device generates values of R based on shift register and no need to pre-store table. | YES. Device needs to compute its actual Tchip = Tchip,ref /R. | log2(NTb)+Ymax, or log2(NTb)+Yi Yi is the number of R values corresponding to indicated Tchip,ref. or Tb. |
| Spreadtrum, Apple, Qualcomm, Docomo | Opt. 3-2: Reader indicates a Tb and a set of R values using bitmap. The length of bitmap can be Ymax or determined by the indicated Tb. | NO. Device generates values of R based on shift register and no need to pre-store table. | YES. Device needs to compute its actual Tchip = Tb /(2R). |  |
| OPPO, Sharp | Opt. 4-1a: Reader indicates a Tb and Y. Device derives Rmax based on the indicated Y and randomly selects R from the smallest R. | YES. Device needs to pre-store a table as agreed in RAN1#120bis. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(NTb)+log2(Ymax) |
| Qualcomm | Opt. 4-1b: Reader indicates a Tb, Y, starting value of R and gap value of R. | YES. Device needs to pre-store a table as agreed in RAN1#120bis. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(NTb)+log2(Ymax)+2* log2(Rmax) |
| NEC | Opt. 4-2: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and Rmax or Btx,D2R,max. | YES. Device needs to pre-store a table as agreed in RAN1#120bis. | YES. Device needs to compute its actual Tchip = Tchip,ref /R. | log2(NTb)+log2(Rmax) |
| ZTE, Xiaomi | Opt. 4-3: Pre-define a table (a new table), where each row refers to a reference Tchip,ref (i.e., Tchip when R = 1) and corresponding set of R values. Reader indicates an index to devices. An example is shown below [6]: | YES. Device needs to pre-store a table, similar as the one shown in the example. | YES. Each row refers to a reference Tchip,ref (i.e., Tchip when R = 1) common to all devices. Device needs to compute its actual Tchip = Tchip,ref /R. | log2(N3) N3 is the total number of indices in the table. |
| CATT | Opt. 4-4: Pre-define a table (a new table), where each row refers to a combination of {Tb, a subset of R} and the subset of R values avoids potential interferences among devices. Reader indicates an index to devices. An example is shown below: | YES. Device needs to pre-store a table, similar as the one shown in the example. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(N4) N4 is the total number of indices in the table. |
| Qualcomm | Opt. 4-5: Pre-define a table (a new table) per each Tb, each row refers to a set of R. Reader indicates Tb and a row index. An example is shown below: | YES. Device needs to pre-store NTb number of tables, similar as ones shown in the example. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(NTb)+log2(N5) N5 is the number of row indices in the table. |
| CATT | Reader indicates a Tb and a set of R values which avoids potential interferences among devices. *Note: No details on how to indicate R. CATT proposes that R=2n can be defined. If the indicated R value includes a value of 2n, if the exact value is indicated, the device does not need to pre-store R values; otherwise, if bitmap/codepoint is used to indicate R (to reduce overhead), device needs to pre-store a mapping relationship. | NO, if exact R value is indicated. Device obtains the values of Tb and R and no need to pre-store table. YES, if exact R value is not indicated. Device needs to pre-store a mapping table of codepoint and R values. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(NTb)+log2(Rmax) log2(NTb)+log2(NR) |
| Qualcomm | Reader indicates a Tb, number of R and optionally Rmax. Device determines R based on a rule. *Note: No details on the rule. | YES. Device needs to pre-store a table as agreed in RAN1#120bis. | YES. Device needs to compute its actual Tchip = Tb /(2R). Device needs to determine R based on some rule. | log2(NTb)+log2(Ymax) |

##### FL comments

From the inputs, the design details from different companies are quite diverse. But regardless of different designs, the indication signaling should consider the impact on at least the following aspects:

- Aspect 1: Whether or not the device needs to pre-store a table for Tb, Tchip, and/or R.

- Aspect 2: Whether or not the device needs to perform additional computation to obtain the actual Tchip and R used for D2R transmission.

- Aspect 3: Indication overhead.

FL understands that the Aspect 1 and Aspect 2 are very critical for device cost, power consumption, and implementation complexity. Therefore, the indication signaling design should avoid or minimize the device storage and device computation. For Aspect 3, as the indication of frequency resources for Msg 1 transmission is carried in paging, which applies for the entire paging procedure, the indication overhead can be considered as a next step optimization but should not be prioritized in the design. FL formulates a Question 3.3.1-1 to collect companies’ views on the listed Aspects and design principles.

The proposed solutions are further divided into 4 groups due to different impacts on Aspect 1 and/or Aspect 2:

- Opt. 1: The device needs to pre-store a table for Tb, Tchip, and/or R (a table as agreed in RAN1#120bis or a new table to be defined in RAN1). The device does NOT need to perform additional computation to obtain the actual Tchip and R used for D2R transmission.

- Opt. 2: The device does NOT need to pre-store tables. The device does NOT need to perform additional computation to obtain the actual Tchip and R used for D2R transmission.

- Opt. 3: The device does NOT need to pre-store tables. The device needs to additional compute actual Tchip based on the indicated Tb or reference Tchip,ref  (i.e., Tchip when R = 1) and selected R. Note that if R = 2n, the computation is very simple.

- Opt. 4: The device needs to pre-store a table for Tb, Tchip, and/or R (a table as agreed in RAN1#120bis or a new table to be defined in RAN1). The device needs to additional compute actual Tchip based on the indicated Tb or reference Tchip,ref  (i.e., Tchip when R = 1) and selected R.

The categorized solutions are recapped as follows and formulated in Proposal 3.3.1-v1 for further down-selection:

- Opt. 1:

  - Opt. 1-1 (Futurewei, China Telecom, LGE, Docomo): Reader indicates an index of a combination of {Tb, Tchip, R} in the table for Tb, Tchip, and R for each frequency domain resource.

  - Opt. 1-2 (Samsung): Reader indicates a Tb. Device randomly selects a Tchip (or R) from the corresponding set.

  - Opt. 1-3 (ZTE): Pre-define a table where each row refers to a set of combinations of {Tchip, R}. Reader indicates an index.

- Opt. 2 (Ericsson, CMCC, Panasonic): Reader indicates the values of Tchip and R in a combination for each frequency domain resource.

- Opt. 3:

  - Opt. 3-1a (vivo): Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is Ymax.

  - Opt. 3-1b (vivo): Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is determined by the indicated Tchip,ref.

  - Opt. 3-2a (Spreadtrum, Apple, Qualcomm, Docomo): Reader indicates a Tb and a set of R using bitmap. The length of bitmap is Ymax.

  - Opt. 3-2b (Spreadtrum, Apple, Qualcomm, Docomo): Reader indicates a Tb and a set of R using bitmap. The length of bitmap is determined by the indicated Tb.

- Opt. 4:

  - Opt. 4-1: Reader indicates a Tb and Y.

    - Opt. 4-1a (OPPO, Sharp): Device determines Rmax based on the indicated Y. Device randomly selects R starting from the smallest R to the determined Rmax.

    - Opt. 4-1b (Qualcomm): Device determines a set of R based on reader indication of the starting value of R and gap value of R. Device randomly selects R from the set.

  - Opt. 4-2 (NEC): Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and Rmax.

  - Opt. 4-3 (ZTE, Xiaomi): Pre-define a table where each row refers to a reference Tchip,ref (i.e., Tchip when R = 1) and a corresponding set of R. Reader indicates an index.

  - Opt. 4-4 (CATT): Pre-define a table where each row refers to a combination of {Tb, a subset of R}, and the subset of R avoids potential interferences among devices. Reader indicates an index.

  - Opt. 4-5 (Qualcomm): Pre-define a table for each Tb where each row refers to a set of R. Reader indicates Tb and a row index.

#### Issue #2: Indication signaling for Msg 3 transmissions Y≥1

Companies’ proposals for frequency domain resource indication for Msg 3 transmissions in AI 9.4.4 are also captured here. At least the following solutions are proposed by companies.

Table 3.3.1-2: Summary of proposed indication signaling for Msg 3 transmission when Y≥1

| Source | Solutions |  | Device storage | Device computation | Signaling overhead |
| --- | --- | --- | --- | --- | --- |
| Ericsson, CATT | Opt. A-1a: Reader indicates values of Tchip and R in a combination for each device. | Explicit indication (Option 2) | NO. Device obtains the values of Tchip and R and no need to pre-store table. | NO. Device obtains the values of Tchip and R. | (log2(NTchip)+log2(NR))*Ymsg3 Ymsg3 is the number of devices for Msg 3 transmission. |
| CATT | Opt. A-1b: Reader indicates values of Tb and R in a combination for each device. | Explicit indication (Option 2) | NO. Device obtains the values of Tchip and R and no need to pre-store table. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(NTb)+log2(Ymax) |
| vivo | Opt. A-2a: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R values using bitmap. The length of bitmap can be Ymax or determined by the indicated Tchip,ref. Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2. | Explicit indication (Option 2) | NO. Device generates values of R based on shift register and no need to pre-store table. | YES. Device needs to compute its actual Tchip = Tchip,ref /R. | log2(NTb)+Ymax, or log2(NTb)+Yi Yi is the number of R values corresponding to indicated Tchip,ref. or Tb. |
| OPPO | Opt. A-2b: Reader indicates a Tb and a set of R values using bitmap with bit length of Ymax. Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2. | Explicit indication (Option 2) | NO. Device generates values of R based on shift register and no need to pre-store table. | YES. Device needs to compute its actual Tchip = Tb /(2R). | log2(NTb)+log2(Ymax) |
| Huawei | Opt. B-1: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1. For a Msg 2 corresponds to one Msg 1, reader indicates a combination of {Tchip, R} from the indicated combinations of {Tchip, R} for Msg 1 transmission. For a Msg 2 corresponds to multiple Msg 1, reader indicates a combination of {Tchip, R} from the indicated combinations of {Tchip, R} for Msg 1 transmission for each device. | Explicit indication (Option 2) | Depends on how device obtains combination of {Tchip, R} for Msg 1 transmission. | NO. Device obtains the values of Tchip and R. | log2(YMsg1)*Ymsg3 YMsg1 is the number of frequency resources for Msg 1. |
| Huawei | Opt. B-2: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1. For a Msg 2 corresponds to multiple Msg 1, use a bitmap with length Y to indicate N resources for Msg 3 transmissions. Device determines its own frequency resource for Msg 3 transmission based on the order of its device ID in Msg 2. Y is the number of frequency resources for Msg 1 and N is the number of devices transmitting Msg 3. | Explicit indication (Option 2) | Depends on how device obtains combination of {Tchip, R} for Msg 1 transmission. | NO. Device obtains the values of Tchip and R. | YMsg1 |
| CMCC | Opt. B-3: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1. For a Msg 2 corresponds to one Msg 1, use a bitmap to indicate the order and number of Msg 3. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. | Explicit indication (Option 2) |  |  |  |
| ZTE, CMCC, NEC | Opt. C: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. | Determines based on a rule. | Depends on how device obtains combination of {Tchip, R} for Msg 1 transmission. | Depends on how reader indicates the frequency domain resources. | N/A |

##### FL comments

From previous RAN1 meeting, in AI 9.4.4, the following agreement on frequency domain resource for Msg 3 transmission has been achieved.

Agreement Support the following option(s) for frequency domain resource for Msg3 transmission: - Option 2: The frequency domain resource for Msg3 can be determined based on explicit indication in the PRDCH for Msg2 transmission for one or multiples devices. - FFS: support option 1 as a default if PRDCH does not provide the indication or based on a rule - Option 1: the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device

It has been agreed that the frequency domain resource for Msg 3 transmission can be determined based on explicit indication in Msg 2 for one or multiple devices. On the other hand, there is still an FFS bullet on how to determine the frequency domain resource for Msg 3 transmission if Msg 2 does not provide any indication, whether Option 1 (i.e., device reuses the frequency domain resource for Msg 1) is supported or the device can determine its frequency domain resource based on a rule.

From the inputs, companies’ proposals regarding Option 2 can be divided into two groups. One is that the frequency domain resources for Msg 3 transmissions is separately indicated to devices and irrespective with the frequency domain resources for Msg 1 transmissions (Opt. A). This option can provide better scheduling flexibility but requires a higher signaling overhead. The other is that the set of frequency domain resources for Msg 3 transmissions is same as that for Msg 1 transmissions, and additional indication is used for devices to determines its own resources for Msg 3 transmission (Opt. B). This option largely reduces the signaling overhead but has limitations on scheduling flexibility (as the frequency domain resources for Msg 1 is reused, the bit rate for Msg 3 keeps the same for Msg 1. However, during an inventory round, the benefit or even possibility to schedule different bit rates for Msg 1 and Msg 3 is questionable, the gain of reducing signaling overhead for Opt. B stands). The proposed solutions are recapped as follows and formulated in Proposal 3.3.2-v1 for further down-selection:

- Opt. A:

  - Opt. A-1a (Ericsson, CATT): Reader indicates values of Tchip and R in a combination for each device.

  - Opt. A-1b (CATT): Reader indicates values of Tb and R in a combination for each device.

  - Opt. A-2a (vivo): Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R values using bitmap. The length of bitmap can be Ymax or determined by the indicated Tchip,ref. Device determines its R value for Msg 3 transmission based on its order of device ID in Msg 2.

  - Opt. A-2b (OPPO): Reader indicates a Tb and a set of R values using bitmap with bit length of Ymax. Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2.

- Opt. B: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1.

  - Opt. B-1 (Huawei): Reader indicates a combination of {Tchip, R} from the indicated combinations of {Tchip, R} for Msg 1 transmission for each device.

  - Opt. B-2 (Huawei): Reader indicates frequency resources for Msg 3 transmissions using bitmap. The length of bitmap is YMsg1. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. YMsg1 is the number of frequency resources for Msg 1.

  - Opt. B-3 (CMCC): Reader indicates the order and number of Msg 3 transmission using bitmap. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2.

In addition, regarding the remaining FFS bullet when there is no indication in Msg 2, a few companies provide detailed rule on how device determines its frequency domain resources for Msg 3 transmission (Opt. C). When compared to Option 2, the direction of Msg 2 carrying no frequency domain resource indication has a benefit on signaling overhead. Then, when compared to Option 1 (device reuses the frequency domain resources for Msg 1), the solution to determine frequency domain resources for Msg 3 based on the proposed rule has a benefit on link performance. Given a Tb, a larger R corresponds to a shorter Tchip, which negatively affects the link performance. In this sense, when the device randomly selects a frequency domain resource with larger R (or shorter Tchip) for Msg 1 transmission, this solution is possible to re-select a new frequency domain resource using smaller R (or longer Tchip) for Msg 3 transmission. The proposed solution is recapped as follows and formulated in Proposal 3.3.3-v1:

- Opt. C (ZTE, CMCC, NEC): Set of frequency domain resources for Msg 3 transmission is the same as Msg 1. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2.

#### Issue #3: Indication signaling for other D2R transmissions when Y=1

At least the following solutions are proposed by companies.

| Source | Solutions |
| --- | --- |
| Vivo | Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a R value using bitmap. The length of bitmap can be Ymax or determined by the indicated Tchip,ref. |
| Vivo | Reader indicates a Tchip and a R. |
| Qualcomm | Reader indicates a Tb. Reader indicates R = 1 for peak data rate or R = 2 otherwise. |

##### FL comments

Only a few companies provide inputs regarding other D2R transmissions in non-FDMA case. Note that some frequency domain resource indication solution for Msg 1 and Msg 3 can be reused or is applicable for non-FDMA case, therefore, we can first focus on the indication designs for Msg 1 and Msg 3. Pending the progress, FL can further draft the proposal for this issue.

Round 1 discussion

      - [H] Question 3.3-1-v1

Please provide your views on at least the following principles when down-selecting the indication signaling design:

- Principle #1: The indication signaling design should minimize or avoid the device to pre-store a table for Tb, Tchip, and/or R.

- Principle #2: The indication signaling design should minimize or avoid the device to perform additional computation to obtain the actual Tchip and R used for D2R transmission.

- Principle #3: The indication of frequency domain resources for Msg 1 transmission is carried via paging. The indication overhead should not be prioritized in the design but considered as an optimization.

- Others, if any.

| Company | Principle | Comments |
| --- | --- | --- |
| FUTUREWEI |  | Regardless of the principles, this is a RAN2 issue of signaling once RAN1 decides what combinations are supported. |
| OPPO1 | #1 | For Principle #2, the conversion from Tchip to Tb (Tchip = Tb /(2R)) is a trivial operation in our understanding for a device. Therefore, Principle #1 and #3 should not be considered. |
| Spreadtrum | 2 or 3 | Our first preference is principle #2. As mentioned in our contribution [R1-2503516], device can pre-store the table can get the Tchip based on the bitmap of R. In addition, we also support principle # 3 (2nd preference), it is up to reader to ensure the non-overlap with main lobes of different R values, leaving the complexity to the reader and making device implementation simple. |
| Panasonic | #1 and #2 | We think all these principles, especially principles 1 and 2, are important to reduce the complexity and power consumption. Given the target of device 1 short operating range, principle#3 is not so important as the frequency can be reused by the multiple of devices by the spatial isolation. |
| vivo | #1 | For principle #1, it depends on how much information to be stored. We agree that a large number of table(s) or a table with large number of entries should be avoided to reduce storage impact. For principle #2, we think simple computation, e.g., by simply shift (2^n) to derive actual Tchip based on Tb or reference Tchip does not cause any material burden to the device. For principle #3, we think signaling overhead, e.g., 40+ bits vs 10+ bits, has non-negligible impact on R2D performance and transmission efficiency, no matter the R2D is paging or Msg 2 (to schedule Msg 3). Therefore, it should be considered as an important aspect rather than optimization. |
| HONOR |  | We prefer RAN1 just define the table and RAN2 to design the signaling. |
| Huawei, HiSilicon |  | The indication overhead is not a priority since the signaling is being handled by higher layers. We do not see an issue with the device being able to pre-store a table with the R and Tchip values as it would avoid the device having to calculate these values. |
| Samsung |  | We believe that reducing signaling overhead for indication purposes should be a key focus. For example, if a total of 8 Tb values are supported, only 3 bits are required to indicate one of them. Based on the indicated Tb, the device can select a suitable pre-stored Tc (or R) value for Msg1 FDMA. However, if Tc were to be directly indicated, the corresponding R value would also need to be signaled, which would inevitably result in a significant increase in signaling overhead. |
| ZTE, Sanechips |  | It should be clarified that whether Principle #2 is a mandatory principle for A-IoT design. If so, should the other signaling design, for example, the interval of midamble follow Principle #2? Besides, Principle #1 and Principle #2 are naturally contradictory. More specifically, more values are usually stored in advance in order to avoid computation. On the contrary, storage overhead is usually reduced through computation. How to determine the priority of Principle #1 and Principle #2 needs to be clarified. Since FDMA resources indication is not only used for msg1, but also for other D2R transmission such as msg3, the indication overhead should be with high priority. If the indication overhead is increased with the number of multiplexed devices, the signaling needs to be defined according to the maximum number of multiplexed devices (denoted as Ymax). However, the number of multiplexed devices in practice is usually less than Ymax considering the performance and bandwidth, which leads to a significant waste of signaling overhead in practical applications. Moreover, in R2D transmission, there is no FEC, larger OH will also be proportionally degraded performance, which should be also considered. Therefore, the indication overhead should be the principle with high priority. |
| QC |  | For “additional computation to obtain the actual Tchip”, we think this is not correct. Tchip does not need to be calculated any options. What device needs to know is the number of clock cycles to perform switching for a given D2R chip duration. This does not require actual “computation” of Tchip. Since different device uses clocks with different speed (implementation issue), the number of clock cycles during a D2R chip is different and needs to be stored in table form anyway. Computing Tchip by multiplication/division is not likely or necessary and it is way more complex operation. The principle we think important is as follows. - Common/same information to all devices should be transmitted only once to reduce overhead. - Flexible indication on R is preferred since RAN1 cannot determine/agree any necessary combination of R values for a given Tb. - Better to minimize big and many tables. But small table should be okay. Computation can be simply replaced by small table. |
| Ericsson | Principle#1 | If device cannot pre-store the Tb,Tchip,R table, then the indication sent in paging will have to be larger. A combination tuple {R,Tchip} per device can be configured with minimal number of bits (table indices) only if the device pre-stores the table |
| Ericsson | Principle#2 | Agree. Computation should be done at the reader and indication should just involve the device choosing one of the combinations |
| Ericsson | Principle#3 | Agree that FDMA indication should be in paging. How frequency is indicated should be in the standard. |
| Ericsson | Others | Rules and restriction associated with the possible combination tuple {R,Tchip} for Msg1 should be calculated at the reader and indicated to the device via paging |
| NEC | #2 | Signalling overhead should be minimized for D2R scheduling, and Principle #3 should not be discussed. |
| DOCOMO |  | We tend to agree that principle#1 can be prioritized over principle#3 if the overhead reduction gain is marginal while we think which aspects should be prioritized depends on the details, how many values should be stored for principle#1, how complex calculation is expected for principle#2 and overhead reduction gain for principle#3. |

      - [H] Proposal 3.3-1-v1

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, down-select from the following options:

- Opt. 1:

  - Opt. 1-1: Reader indicates an index of a combination of {Tb, Tchip, R} in the table for Tb, Tchip, and R for each frequency domain resource.

  - Opt. 1-2: Reader indicates a Tb. Device randomly selects a Tchip (or R) from the corresponding set.

  - Opt. 1-3: Pre-define a table where each row refers to a set of combinations of {Tchip, R}. Reader indicates an index.

- Opt. 2: Reader indicates the values of Tchip and R in a combination for each frequency domain resource.

- Opt. 3:

  - Opt. 3-1a: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is Ymax.

  - Opt. 3-1b: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is determined by the indicated Tchip,ref.

  - Opt. 3-2a: Reader indicates a Tb and a set of R using bitmap. The length of bitmap is Ymax.

  - Opt. 3-2b: Reader indicates a Tb and a set of R using bitmap. The length of bitmap is determined by the indicated Tb.

  - Note: Device needs to compute the actual D2R chip duration Tchip = Tb /(2R) or Tchip = Tchip,ref /R. The computation is simple only if R=2n.

- Opt. 4:

  - Opt. 4-1: Reader indicates a Tb and Y.

    - Opt. 4-1a: Device determines Rmax based on the indicated Y. Device randomly selects R starting from the smallest R to the determined Rmax.

    - Opt. 4-1b: Device determines a set of R based on reader indication of the starting value of R and gap value of R. Device randomly selects R from the set.

  - Opt. 4-2: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and Rmax.

  - Opt. 4-3: Pre-define a table where each row refers to a reference Tchip,ref (i.e., Tchip when R = 1) and a corresponding set of R. Reader indicates an index.

  - Opt. 4-4: Pre-define a table where each row refers to a combination of {Tb, a subset of R}, and the subset of R avoids potential interferences among devices. Reader indicates an index.

  - Opt. 4-5: Pre-define a table for each Tb where each row refers to a set of R. Reader indicates Tb and a row index.

  - Note: Device needs to compute the actual D2R chip duration Tchip = Tb /(2R) or Tchip = Tchip,ref /R. The computation is simple only if R=2n.

| Company | Opt. | Comments |
| --- | --- | --- |
| FL |  | Please note that RAN1 needs an agreement for the signaling overhead for RAN2 to design the MAC PDU format. Once the indication signaling option is selected, FL will further draft another proposal. The proposal will be based on the signaling overhead per each option summarized in Table 3.3.1-1. Companies are welcomed to check. At this stage, please first focus on down-selection of listed options. Thanks. |
| OPPO1 | 4-1a | We believe this provides the least amount of signaling overhead. |
| Spreadtrum | Opt.3-2b(1st preference); Opt.2(2nd preference) | As we comment in the Question 3.3-1-v1, our first preference is Opt.3-2b based on the principle #2, and our second preference is Opt.2 based on the principle#3. |
| Panasonic | Opt. 3-2a | This option is straight forward and provides flexibility on assigning different frequency allocations. |
| vivo | Opt 3 | To simplify discussion, it may be helpful to structure the proposal with following high-level categories. For indication of frequency domain resources for Msg 1 transmissions when Y≥1, down-select from the following approaches: Approach 1: Separate indication of {Tb, R} or {Tchip, R} for each device - Approach 1-1: Separate indication of Tb or Tchip, and R. - Approach 1-2: Joint indication of Tb or Tchip, and R Approach 2: Indication of single Tb or reference Tchip applicable to all devices, and each R for each device. Approach 3: Indication of single Tb or reference Tchip applicable to all devices, and single reference R, e.g., Rmax, to all devices. We prefer approach 2 based on principles discussed in Question 3.3-1. Regarding options listed in the proposal, our views are as below. Opt 1 requires larger overhead and larger storage. Opt 2 requires largest overhead. For option 4, Option 4-3 ~ 4-5 requires to define table for each Tb, which would be difficult to converge within one meeting, and it also requires storage. For option 4-1 & 4-2, it is less flexible than option 3, and it may not work if both R=1 and R=2 is supported as candidate R value for FDMA. Option 3 provides reasonable tradeoff between device complexity and signaling overhead. Thus, we support option 3. |
| HONOR | Combine Opt.3-2a Opt.4-4 | We prefer pre-define a table where each row refers to a combination of {Tb, R} like the current agreed table, Reader indicates an index and a set of R using bitmap. The length of bitmap is Ymax. |
| Huawei, HiSilicon |  | Looking at the options listed out, they are indeed signaling optimizations in order to reduce the overhead. But as we had already mentioned in Question 3.3-1, the indication overhead is not a priority since the signaling is being handled by higher layers. Hence, we would like to add an option 5 leaving the signaling details to RAN2. In the end, if RAN1 cannot converge to one Option, the way out is adopting Option 5, send LS to RAN2 about this. Companies will then consider what is appropriate to propose in the context of RAN2 papers and RAN2 agreements, etc. == For indication of frequency domain resources for Msg 1 transmissions when Y≥1, down-select from the following options: - Opt. 1: - … - Opt. 2: Reader indicates the values of Tchip and R in a combination for each frequency domain resource. - Opt. 3: - … - Opt. 4: - … - Opt. 5: leave the signaling details to RAN2 |
| Samsung | Opt. 1-2 Opt. 3-2 Opt. 4-1 | For Msg1 transmission, it was agreed that in the case of FDMA of multiple Msg1 transmissions in response to a single R2D transmission triggering random access, only the same data rate should be supported across the FDMed Msg1 transmissions. Based on this agreement, we believe that indicating Tb directly is a more efficient way to reduce signaling overhead. Depending on how we down-select Tb values in Section 3.1, it may or may not be necessary to additionally indicate FDMA-related parameters. We are open to further discussion on this point. |
| ZTE, Sanechips |  | For Option 1, there are also some methods to reduce storage. For example, stores only the maximum set of R values and then determines the used values in the set based on the index. Another possible way is to calculate Tchip based on R and Tb instead of directly storing Tchip. Both two methods can be implemented by the device to reduce storage. Option 2 indicates the Tchip and R for each frequency domain resource. The indication overhead of Option 2 depends on the maximum number of multiplexed devices, which is (log2(NTchip)+log2(NR))*Ymax as illustrated in Table 3.3.1-1. However, when there are fewer multiplexed devices, this signaling overhead is wasted. In addition, the device also needs to store all possibilities of Tchip and R, and similarly, the storage overhead will be necessary. Option 3 has high overhead and includes the potential combination of many useless cases. For example, when multiplexing 2 devices, instead of provide indications of all possible combinations, specifying to use the smallest two R values with is sufficient, which can also reduce the OH. Option 4 may have similar requirement in terms of storage and may have some redundant indication states for some sub-options. Moreover, several general comments are as below: - Device storage complexity. In our understanding, even a table is defined, it doesn’t mean device has to implement it by storing the table. Device may choose to implement it in another way with a proper design of the bit/chip duration and R values. - For option 4, some of the sub-option has some high level similarity with slightly different. - Regarding the solution of indication Tb, our understanding is that in the Feb meeting, we agreed in 944 that chip duration is indicated for D2R, instead of bit duration. Given the current options are too divergent for convergence, we share the similar feeling with vivo that we can go step by step. We can first discuss whether to indicate the same/separate chip duration, R value/set for the FDM-ed device. The detailed wording can be further refined. |
| Xiaomi | Opt. 3-2a | This option allows flexible scheduling. Using bitmap to directly indicate bit duration and set of R values does not require device to derivation. |
| QC | Opt. 3: | For Msg1, all devices are supposed to use the “same” bit rate. So, it is the most efficient to signal Tb only once which is common to all devices. Instead of listing all possible options, as vivo mentioned, we recommend to get high level idea and start with high level approaches. Once we decide high level option, then, we can further discuss sub options Option 3-x, x=1a,1b,2a,2b,… We prefer to have following simpler formulation – modified from vivo’s proposal. Approach 1: Separate indication of {Tb, R} or {Tchip, R} for each device - Approach 1-1: Separate indication of Tb or Tchip, and R. - Approach 1-2: Joint indication of Tb or Tchip, and R Approach 2: Indication of single Tb or reference Tchip applicable to all devices, and each set of R values for each device. Approach 3: Indication of single Tb or reference Tchip applicable to all devices, and single reference R, e.g., Rmax, to all devices. |
| Ericsson | Option 2 | Option 1-1: {Tb, Tchip, R} all the three not needed. Device ends up transmitting in data rate Tb if it is tuned to a certain Tchip, R Option 1-2: Random choosing of Tchip or R means the data rate can differ. The combination needs to be such that Tchip1*R1=Tchip2*R2= .. Option 1-3: Predefining works but might need additional restrictions based on IM3 and harmonics. Option 3-1a,b: A reference Tchip and then a combination of R does not ensure Tchip1*R1=Tchip2*R2= … This does not work. Option 3-2 a,b: Device needs to adjust its chip rate and R. With Tb and R indication, Tchip will have to be additionally computed by the device. More computational overhead. Option 4 will not work due to the constraint on combination of Tchip,R that the devices can choose from such that the following equation is satisfied. Tchip1*R1=Tchip2*R2= … |
| NEC | Opt.4-2 | By indicating Rmax, the overhead of bitmap is saved. |
| DOCOMO | Opt.3-2 | For Msg1 for CBRA, it has already been agreed that the same Tb is used for FDMed resources. Then, Tb can be indicated and we are open to discuss further how to indicate the set of R values. In our view, it is not necessary to limit the possible combinations of R for each Tb and it should be up to reader’s choice. In addition, it should be clarified whether the same indication method as Msg1 for CBRA can be applied to Msg1 for CFRA where Y=1. |
| FL |  | FL don’t think that all companies understand the situation well. We are talking about indication designs for an extremely low cost and low power consumption device, not a normal UE. When we do the designs, we cannot jump out of nowhere to say that this solution is preferred only because it has less indication overhead. We should first make sure that a proposed solution does not bring burden for the device to implement, and the identified issues, as FL listed in Question 3.3-1-v1, at least include requiring device to pre-store tables and to do complicated computation. @vivo @Qualcomm, thanks for your good suggestions on how we can proceed the discussion in different ways, but still, FL thinks that such classification may not clearly show what and how a proposed solution affect the device implementation. For the current categorization on proposed solutions: Opt. 2, as the reader explicitly indicates the values of actual Tchip and R in a combination for each frequency domain resource, clearly, it does not require the device to store any table. The device will simply transmit the indicated Tchip and repeat it R times for a D2R transmission. For all the other options, they are actually optimizations on top of Opt. 2 for signaling overhead reduction: - For Opt. 1 and Opt. 4, the device has to pre-store tables, the only difference is that Opt. 1 reuses the table we have agreed in the last meeting, but Opt. 4 proposes some new tables and RAN1 need additional workload to agree on them. - For Opt. 3, as long as we follow the current proposals that R=2n, the device does not need to store any table, and it is very easy for a device to acquire the actual Tchip using shift register. However, it should be noted that if RAN1 cannot reach consensus on down-selections on Tb, Tchip and R, and support R values like 12, 24, 48, it would be difficult for the device to get Tchip using a commonly indicated value of Tb or Tchip,ref. For Opt. 5 that proposed by Huawei, and also echoed by Futurewei and Honor, to leave the signaling details to RAN2. My understanding is that it may be the only way to go, if in the end RAN1 cannot reach any consensus on the indication design. So, FL adds a note in the updated proposal for round 2 discussion. At least from the first round of feedback, clearly, Opt. 3 has more proponents (8 companies), so companies are encouraged to provide your views on whether you can live with the direction of Opt. 3. If no, what are exactly your concerns, and if yes, please further provide your preference on the sub options under Opt. 3. - [H] Proposal 3.3-1-v2 For indication of frequency domain resources for Msg 1 transmissions when Y≥1, down-select from the following options: - Opt. 1: - Opt. 1-1: Reader indicates an index of a combination of {Tb, Tchip, R} in the table for Tb, Tchip, and R for each frequency domain resource. - Opt. 1-2: Reader indicates a Tb. Device randomly selects a Tchip (or R) from the corresponding set. - Opt. 1-3: Pre-define a table where each row refers to a set of combinations of {Tchip, R}. Reader indicates an index. - Opt. 2: Reader indicates the values of Tchip and R in a combination for each frequency domain resource. - Opt. 3: - Opt. 3-1a: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is Ymax. - Opt. 3-1b: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R using bitmap. The length of bitmap is determined by the indicated Tchip,ref. - Opt. 3-2a: Reader indicates a Tb and a set of R using bitmap. The length of bitmap is Ymax. - Opt. 3-2b: Reader indicates a Tb and a set of R using bitmap. The length of bitmap is determined by the indicated Tb. - Note: Device needs to compute the actual D2R chip duration Tchip = Tb /(2R) or Tchip = Tchip,ref /R. The computation is simple only if R=2n. - Opt. 4: - Opt. 4-1: Reader indicates a Tb and Y. - Opt. 4-1a: Device determines Rmax based on the indicated Y. Device randomly selects R starting from the smallest R to the determined Rmax. - Opt. 4-1b: Device determines a set of R based on reader indication of the starting value of R and gap value of R. Device randomly selects R from the set. - Opt. 4-2: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and Rmax. - Opt. 4-3: Pre-define a table where each row refers to a reference Tchip,ref (i.e., Tchip when R = 1) and a corresponding set of R. Reader indicates an index. - Opt. 4-4: Pre-define a table where each row refers to a combination of {Tb, a subset of R}, and the subset of R avoids potential interferences among devices. Reader indicates an index. - Opt. 4-5: Pre-define a table for each Tb where each row refers to a set of R. Reader indicates Tb and a row index. - Note: Device needs to compute the actual D2R chip duration Tchip = Tb /(2R) or Tchip = Tchip,ref /R. The computation is simple only if R=2n. Note: The indication signaling details will be left to RAN2 design if RAN1 cannot reach any consensus. |

      - [H] Proposal 3.3-2-v1

For frequency domain resource for Msg 3 transmission is determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, down-select from the following options:

- Opt. A:

  - Opt. A-1a: Reader indicates values of Tchip and R in a combination for each device.

  - Opt. A-1b: Reader indicates values of Tb and R in a combination for each device.

  - Opt. A-2a: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R values using bitmap. The length of bitmap can be Ymax or determined by the indicated Tchip,ref. Device determines its R value for Msg 3 transmission based on its order of device ID in Msg 2.

  - Opt. A-2b: Reader indicates a Tb and a set of R values using bitmap with bit length of Ymax. Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2.

- Opt. B: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1.

  - Opt. B-1: Reader indicates a combination of {Tchip, R} from the indicated combinations of {Tchip, R} for Msg 1 transmission for each device.

  - Opt. B-2: Reader indicates frequency resources for Msg 3 transmissions using bitmap. The length of bitmap is YMsg1. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. YMsg1 is the number of frequency resources for Msg 1.

| Company | Opt. | Comments |
| --- | --- | --- |
| FL |  | Please keep in mind that for down-selections of indication design for Msg 3, the principles in Question 3.3-1-v1 should also be considered. |
| FUTUWEI |  | It is unclear why RAN1 needs to consider any options. It is a scheduling and signal design question for RAN2 |
| OPPO1 | A-1b, A-2b, or B-2 | Within Opt. A-1b, only a common/reference Tb value is needed for all devices. For each scheduled device, the reader indicates the exact R value. When the number of scheduled devices is small, the required number of bits (3 or 6 bits) can be smaller than using a bitmap (7 bits). For Opt. A-1b and B-2, the benefit of using this option is when the number of scheduled devices is large using a bitmap (7 bits). |
| Spreadtrum | Opt.A-1a or Opt.A-1b | Either Opt.A-1a or Opt.A-1b is acceptable for us. |
| Panasonic | Opt. A-1a | This option is more aligned with the frequency allocation for Msg1(i.e., Option 3-2a). |
| vivo | Opt A-2a/2b | We don’t support option B, as it introduces unnecessary restriction on Msg 3. The contents, payload size, w/ or w/o collision, total number of FDMed device is different for Msg 1 and Msg 3, it is not reasonable to couple Msg 1 and Msg 3 frequency resource allocation. Regarding option A-1a/1b, it is more flexible than option B, but it results in larger overhead than option A-2a/2b. We prefer option A-2a/2b as unified design for Msg 1 and Msg 3. |
| HONOR |  | It is no clear whether RAN2 need to scheduling Msg3 again or just use the resources as Msg1 |
| Huawei, HiSilicon | Opt. B | We support Opt. B that the set of frequency domain resources for Msg 3 transmission is the same as Msg 1, this can simplify the design a lot. In separate Msg2 case: this Msg2 only needs to schedule 1 device, so Opt. B-1 can be adopted, i.e., use log2Y bits to indicate which one pair of {R, Tchip} among the ones for Msg1 is reused for Msg3. In common Msg2 case: this common Msg2 needs to schedule N devices, Opt. B-2 can be adopted. == For frequency domain resource for Msg 3 transmission is determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, down-select from the following options: - Opt. A: - Opt. A-1a: Reader indicates values of Tchip and R in a combination for each device. - Opt. A-1b: Reader indicates values of Tb and R in a combination for each device. - Opt. A-2a: Reader indicates a reference Tchip,ref (i.e., Tchip when R = 1) and a set of R values using bitmap. The length of bitmap can be Ymax or determined by the indicated Tchip,ref. Device determines its R value for Msg 3 transmission based on its order of device ID in Msg 2. - Opt. A-2b: Reader indicates a Tb and a set of R values using bitmap with bit length of Ymax. Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2. - Opt. B: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1. - In separate Msg2 case, adopt Opt. B-1: Reader indicates a combination of {Tchip, R} from the indicated combinations of {Tchip, R} for Msg 1 transmission for each device. - In common Msg2 case, adopt Opt. B-2: Reader indicates frequency resources for Msg 3 transmissions using bitmap. The length of bitmap is YMsg1. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. YMsg1 is the number of frequency resources for Msg 1. |
| Samsung |  | For Msg3, it was agreed that the frequency domain resource can be determined based on explicit indication in the PRDCH for Msg2 transmission, either for one or multiple devices. Based on this, we believe it is appropriate to indicate a Tc and the R value for each device. However, aside from which values are to be indicated, we think the discussion on how to deliver this information should take place in RAN2, given that it would be transmitted via higher-layer signaling. |
| ZTE, Sanechips |  | Option A-1 has too much overhead since the signaling needs to be defined according to the maximum number of multiplexed devices. For example: if the indication overhead of single frequency domain resource is 6 bits (3 bits for Tb/Tchip and 3 bits for R value), then multiplexing 8 devices requires 48 bits. However, considering the performance and frequency domain bandwidth, the multiplexed devices in practice are usually less than the maximum number, which leads to a significant waste of signaling overhead in practical applications. Moreover, as we commented on msg1 indication solution, higher OH also means worse performance. On the other hand, Option A/B-2 requires fewer bits to indicate the resources set, and reusing the indication signaling design for Msg1 also reduces the discussion of individual device indications considering that there are still many unresolved issues to discuss in this is last meeting. It is recommended to consider the design rule of Option A/B-2, wherein the exact frequency domain resource is determined by the ID. Regarding whether the R value set is determined by a bitmap or codepoint can be further discussed. For example, reader can indicate the Y value with a codepoint, while the resource set is the Y smallest R values. Moreover, similar comments on msg1, in the Feb meeting, we agreed in 944 that chip duration is indicated for D2R, instead of bit duration. |
| Xiaomi | Opt. B-2 | Option B-2 requires the lowest indication overhead. In the same inventory procedure, there is no necessary to change/switch the occupied resources for Msg 1 and Msg 3. |
| QC | Opt. A: | We prefer to have high level decision first. We prefer Option A. We think Option B is not efficient since in slotted Aloha random access, the optimal slot utilization is around 36%. This means that with Option B, the frequency resources with collision may not be utilized for Msg3, which makes msg3 multiplexing very inefficient. Once RAN1 has consensus, then, we can further discuss sub options of Option A. We think “reference Tchip,ref” is fundamentally the same as Tb. There is no need to introduce new name or parameter. |
| Ericsson | OptionA-1a | Msg3 frequency resource should be separately configurable as part of Msg2 scheduling. We think this can be variable as well among FDMA’ed devices as well. Reader can configure scheduling restrictions associated with no frequency overlaps, no harmonics, no IM3 etc better if Option A-1a is followed. |
| NEC | A-2a with comments or B-2 | In Opt. A-2a, by determining the length of bitmap with the indicated Tchip,ref (a.k.a a pair of Tchip and R = 1), in some cases with certain specified rules like automatically increase/decrease R by default interval for other devices, the length of bitmap can be reduced to minimum as zero, thus saved signalling overhead in Msg2. In Opt. B-2, the set of frequency domain resources for Msg 3 should not be different from Msg1 which has already optimized selected/indicated by the reader, besides, the additional storage and signaling overhead can be avoided if the set of frequency resources to be unchanged. |
| DOCOMO |  | For Msg3, different from Msg1 for CBRA, only X=1 is supported and the number of transmission occasions should be only for the successful reception of Msg1 at reader, and also the message size/Tb can be different from Msg1. In that sense, it is not necessary that the same FDM resource as Msg1 should be assumed for Msg3 and we prefer option A for better flexibility. For the sub-options, it is preferable to reuse the similar indication framework as Msg1 for CBRA. |

      - [H] Proposal 3.3-3-v1

For frequency domain resource for Msg 3 transmission, if the PRDCH for Msg 2 transmission does not provide any frequency domain resource indication, and if device determines the frequency domain resource for Msg 3 transmission based on a rule is supported,

- The set of frequency domain resource for Msg 3 transmission is the same as Msg 1, and device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2.

| Company | Comments |
| --- | --- |
| FUTUREWEI | Is MAC signaling design a RAN1 issue? |
| OPPO1 | Not supportive. We are supportive of the set of frequency domain resource for Msg.3 transmission is the same as Msg.1, but this applies to the case only when X=1 for Msg.1 and does not depend on the order of random ID in Msg.2. Otherwise, when X=2, it is preferrable that the reader has the full flexibility of the number of frequency domain resource(s) for Msg.3 transmission, which device(s) to be scheduled, and which frequency position to be used for each device. |
| Spreadtrum | We think this proposal should be keep same with the agreement achieved in the RAN1#120. We support reuses the frequency domain resource for Msg1 for the same device if PRDCH does not provide the indication for the frequency domain resource for Msg3. Considering this is the last meeting, we don’t have more time to discuss a rule. The modification is list as following. - [H] Proposal 3.3-3-v1 For frequency domain resource for Msg 3 transmission, if the PRDCH for Msg 2 transmission does not provide any frequency domain resource indication, and if device determines the frequency domain resource for Msg 3 transmission based on a rule is supported, - The set of frequency domain resource for Msg 3 transmission is the same as Msg 1 for the same device, and device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. |
| Panasonic | We prefer the explicit indication, as the above rule requires storing the configuration for the previous Msg1 transmission. |
| vivo | We don’t support the proposal. The main motivation is to reduce the signaling overhead. However, additional signaling needs to inform device whether the frequency domain resource allocation field for Msg3 transmission is preset or absent, then the overhead reduction vanishes. And it requires device to store Msg 1 parameter, and then use the indicated resource or stored Msg 1 resource according to received Msg 2, which increases device complexity. |
| HONOR | Whether as the proposal or just use the same frequency resources as Msg1 regardless the order, that seems a RAN2 issue. |
| Huawei, HiSilicon | If Msg2 does not provide any frequency resource indication, Msg3 uses {R, Tchip} that are used for Msg1. That’s all. There is no need to consider other complicated designs. == For frequency domain resource for Msg 3 transmission, if the PRDCH for Msg 2 transmission does not provide any frequency domain resource indication, and if device determines the frequency domain resource for Msg 3 transmission based on a rule is supported, - The set of frequency domain resource for Msg 3 transmission is the same as Msg 1, and device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. |
| Samsung | This discussion is based on the assumption that Msg2 may not include frequency domain resource information. However, whether such a Msg2 transmission is even allowed in the first place is under RAN2’s scope. Therefore, we believe RAN1 does not need to discuss this issue. |
| ZTE, Sanechips | It should be clarified that what the last meeting agreed in AI9.4.4 is that ‘the frequency domain resource for Msg3 reuses the frequency domain resource for Msg1 for the same device’, instead of ‘the set of frequency domain resource for Msg 3 transmission is the same as Msg 1’. In our understanding, there are some differences between these interpretations. In the original wording, the R value and bit duration should be the same. While in the current FL proposal, the R value may be different, since for Msg1, the R value is randomly selected by device, while for msg 3, the R value is determined by the order. Furthermore, considering the goal of saving indication overhead, we suggest that the repetition, bit rate, and amble configuration of Mg3 should also be the same as those of Mg1 in this case to further reduce the overhead. |
| Xiaomi | We support this proposal. |
| QC | We do not support this. We don’t understand why FL has this proposal, which is already discussed in Proposal 3.3-2. |
| Ericsson | If no indication is provided then same frequency resource as Msg1 might be used, but this is very suboptimal from a resource configuration point of view due to the following reasons- 1) in Msg1 there might be collisions, therefore many FDMA resources can remain unused if the above proposal is generalized. 2) There are two TDMAed occasions in Msg1, and 1 TDMA’ed occasion for Msg3. Therefore even uncollided Msg1 transmissions can collide for Msg3 3) Different data rate configurations are feasible for Msg3 (due to better knowledge of the channel/coverage conditions). Therefore using the same frequency configurations as Msg1 is not efficient. |
| NEC | The proposal is somehow different from the original version in AI 9.4.4. From our perspective, when there is no scheduling information in Msg2, not only the resource set should be same, but also the Msg3 frequency domain resource should be same to Msg1, which is not based on the order of random ID; however, when there is scheduling information, a bitmap can be additionally used for the indication based on random ID order, as in Opt. B-2 from the previous proposal. |
| DOCOMO | We don’t see the need to support implicit indication in addition to the explicit indication. |

Round 2 discussion

      - [H] Proposal 3.3-1-v2

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, down-select from the following options:

- Opt. 1: Reader indicates an index corresponding to a combination of {Tchip, R} defined in a table for each frequency domain resource.

- Opt. 2: Reader indicates the values of Tchip and R in a combination for each frequency domain resource.

- Opt. 3: Reader indicates a single bit duration or chip duration which is same for all frequency domain resources, and a set of R values using bitmap.

  - FFS: Reader indicates a single bit duration Tb or a single reference chip duration Tchip,ref (i.e., Tchip when R = 1).

  - FFS: The length of bitmap is fixed (i.e., Ymax) or determined by the indicated single bit or chip duration.

- [Opt. 4: Reader indicates a single bit duration or chip duration which is same for all frequency domain resources. Device determines the set of R values based on a reader indicated parameter.]

  - FFS: Reader indicates a single bit duration Tb or a single reference chip duration Tchip,ref (i.e., Tchip when R = 1).

  - FFS: For determination of the set of R values, the reader indicates Rmax, or Y or other parameters.

| Company | Opt. | Comments |
| --- | --- | --- |
| FL |  | Based on offline discussion, FL reformulates the proposal. Some high level descriptions for each option has been summarized, hopefully that companies can understand how each option work. In the last online session, we have agreed that values for Tb, Tchip and R, and for R values, please note that no values are precluded from the set of 2n (i.e., both R = 1, 2, 4 are kept). From this perspective, Opt. 4 may not work well, as device determines the set of R based on, for example, the indicated Rmax=128, then different multiplexed device may select R=1 and R=2 or R=2 and R=4 at the same time, causing interference issue. Therefore, FL adds a square bracket on Opt. 4. |

      - [H] Proposal 3.3-2-v2

For frequency domain resource for Msg 3 transmission is determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, down-select from the following options:

- Opt. A-1: Reader indicates a single Tb and R for each device.

- Opt. A-2: Reader indicates a single bit duration or chip duration which is same for all frequency domain resources, and a set of R values using bitmap. Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2.

  - FFS: The length of bitmap is fixed (i.e., Ymax) or determined by the indicated single bit duration or chip duration.

- Opt. B: Set of frequency domain resources for Msg 3 transmission is the same as Msg 1.

  - In case of separate Msg 2 corresponds to multiple Msg 1 transmissions, reader indicates a combination of {Tchip, R} from the indicated combinations of {Tchip, R} for Msg 1 transmission for each device.

  - In case of a common Msg 2 corresponds to multiple Msg 1 transmissions, reader indicates frequency resources for Msg 3 transmissions using bitmap. Device determines its own frequency resource for Msg 3 transmission based on the order of its random ID in Msg 2. The length of bitmap is YMsg1. YMsg1 is the number of frequency resources for Msg 1.

For frequency domain resource for Msg 3 transmission is determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, the reader indicates:

- a single bit duration, which is same for all frequency domain resources,

- a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R

  - note: the set of R values could be signalled using a bitmap

The mapping relationship between device and its Msg 3 frequency domain resource is left to RAN2.

  - Note: Device could determine its R value for Msg 3 transmission based on its order of random ID in Msg 2

The detailed signalling design is left to RAN2.

WF1: Device determines its R value for Msg 3 transmission based on its order of random ID in Msg 2.

WF2: How the device determines its R value for Msg 3 transmission is left to RAN2.

WF3: Add a third bullet point of mapping relationship.

Agreement

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, the reader indicates

- a single bit duration Tb which is same for all frequency domain resources

- a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R

  - note: the set of R values could be signalled using a bitmap

The detailed signalling design is left to RAN2.

| Company | Opt. | Comments |
| --- | --- | --- |

D2R FEC

(CLSOED) FEC code rate

Background

In RAN1#120bis, the following intermediate FL proposal has been discussed in the last online session on Friday without reaching any consensus.

[H] Proposal 4.2-1-v3 If FEC is applied for a D2R transmission, the code rate of FEC additionally supports values 1/2. - The 1/2 code rate is achieved by generating dk0,dk1 according to the polynomial G0 and G1 in Figure 5.2-1 in TS 38.291 v0.0.1. Figure 5.2-1: Rate 1/3 tail biting convolutional encoder

It was suggested that in this meeting, FL will come back to this issue early in the week, if proponents of additionally supporting FEC code rate of 1/2 cannot confirm the group, RAN1 will conclude that only 1 and 1/3 are supported.

Summary of inputs

Based on the contributions submitted in this meeting, on additionally support FEC code rate of 1/2 when FEC is applied, there is more preference on NOT to support it:

- No (11+1): vivo, Spreadtrum, Samsung, CMCC, Xiaomi, Honor, OPPO, LGE, Qualcomm, Sharp, Docomo, Fujistu (can live with not supporting 1/2 code rate).

- Yes (7): Ericsson, Futurewei, Nokia, ZTE, Panasonic, NEC, China Telecom.

In addition, ZTE, Fujistu and Qualcomm analyze that the performance gap between not applying FEC and applying FEC with 1/3 code rate is very large (~10 dB [6] or ~ 6 dB [24]), and it is better to have an intermediate code rate in between. ZTE proposes to support 1/2 code rate to fill in the gap, however, Fujistu and Qualcomm argue that the performance gap between 1/2 code rate and 1/3 code rate is too trivial (e.g., 0.5 dB ~ 1dB [24]). Qualcomm then proposes to support a code rate of 2/3, but Fujistu suggests to give up 1/2 code rate (support only 1/3 code rate when FEC is applied) to finish the work item on time.

Figure: Results of multiple code rates [24]

#### FL comments

From the inputs, seems that more companies are not convinced by adding an additional code rate other than 1/3 if FEC is applied. Considering that this is the last RAN1 meeting we have, FL suggests concluding that only 1/3 FEC code rate is supported, if FEC is used.

Round 1 discussion

      - [H] Proposed conclusion 4.1-1-v1

When FEC is applied, only 1/3 code rate is supported.

| Company | Comments |
| --- | --- |
| FUTUREWEI | We are sacrificing system performance – it is clear that the performance of rate-1/2 FEC is at least several dB better than using two repetitions. Given all the impairments (e.g. clock timing, frequency offset) which may not mitigated with repetition alone, we are sacrificing a low complexity solution. In addition, the performance gap between R=1/2 and R=1/3 shown above increases with faded channels. In many instances, we can use R=1/2 instead of R=1/3 to reduce latency and for device availability considerations. |
| Huawei, HiSilicon | We support the proposal. |
| Spreadtrum | Support. |
| Panasonic | Support |
| vivo | Support the proposal |
| HONOR | Support this proposal |
| Samsung | Agreed |
| ZTE, Sanechips | It is not reasonable to support 2 times of repetition instead of 1/2 code rate, since it is clearly that using FEC provides several dBs gain (i.e., 5dB based our evaluation) It is recommended to also support 1/2 code rate. |
| OPPO | Support |
| Xiaomi | Support |
| QC | In general, we support multiple coding rate. The best option (in addition to 1/3) would be 2/3. But, if this is not agreeable, we are also okay with 1/2. As FW mentioned, if coding gain of 1/3 and ½ are similar, then, using ½ could help reduce latency. |
| Ericsson | Fine. |
| NEC | We support both {1/2, 1/3}, and 1/2 could outperform 2 repetitions without FEC. |
| DOCOMO | Support the FL proposal. Meanwhile we don’t see the necessity of additional coding rate considering that various chip duration and block-level repetition are supported for D2R, if additional coding rate is necessary for any specific scenario, we think other code rate such as 2/3 or 4/5 which has sufficient gap from both no FEC and 1/3 coding rate makes more sense rather than 1/2 coding rate. |
| Nokia | We prefer to additionally support code rate 1/2 and agree with Futurewei’s/ZTE’s reasoning. |

Round 2 discussion

      - [H] Proposed conclusion 4.1-1-v1ming

When FEC is applied, only 1/3 code rate is supported.

| Company | Comments |
| --- | --- |
| FL | From the first round of comments, still, companies supporting only 1/3 code rate are not convinced and supporting an additional 1/2 code rate is minority. The proposal is not changed. Please keep commenting if you have strong concern on this proposal. No need to repeat comments from the 1st round. |

(CLOSED) Bit collection

Summary of inputs

In the current version of TS 38.291, the bit collection of FEC is pending RAN1 agreement. From reviewing the contributions submitted in this meeting, 6 companies (Futurewei, Huawei, ZTE, vivo, CATT, Qualcomm) provide views, and all the inputs propose that the output bits for each input bits will be collected sequentially.

Round 1 discussion

      - [M] Proposal 4.2-1-v1

- For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, i.e., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dD-10,dD-11,dD-12.

- Support the following text proposal for Section 5.1.x.x Channel coding or omission for TS 38.291:

- 5.1.x.x Channel coding or omission <Unchanged parts are omitted> The sequence of bits e0,e1,…, eE-1 for transmission, where E=3D, is generated by: ek= d00,d01,d02,d10,d11,d12,…,dD-10,dD-11,dD-12. <Unchanged parts are omitted>

| Company | Comments |
| --- | --- |
| FUTUREWEI | We note that the proposal is only for R=1/3. If R=1/2 is supported, a similar proposal is …, where E=2D, is generated by ek= d00,d01,d10,d11,…,dD-10,dD-11 |
| Huawei, HiSilicon | We support the first bullet of the proposal. The exact way as to how it is captured in the TS can be left up to the editor. |
| Spreadtrum | Support. |
| Panasonic | Support |
| vivo | Support the proposal |
| HONOR | Ok with the proposal |
| ZTE, Sanechips | Similar comments as FW to add the bit collection for 1/2 code rate. |
| OPPO | Support |
| Xiaomi | Support |
| QC | Support |
| DOCOMO | Support. |
| Nokia | Support |

D2R repetition

(CLOSED) Number of repetitions

Summary of inputs

From reviewing the contributions in this meeting, companies’ views on the number of repetitions are summarized:

- Maximum number of repetitions is 2 (11): Ericsson, Futurewei, Huawei, vivo, Spreadtrum, Samsung, Honor, Qualcomm, Sharp, Docomo, IIT Kanpur. Proponents provide at least the following reasons:

  - The link budget analysis in the study item was done without considering D2R repetition and can meet the coverage requirement.

  - Not to support other candidate repetition number to reduce the signaling overhead.

- Maximum number of repetitions is larger than 2 (13): Huawei, Nokia, ZTE, TCL, Panasonic, CATT, CMCC, NEC, China Telecom, Fujistu, OPPO, LGE, MediaTek. All 13 companies propose that repetition number of 4 should be supported. 2 companies (CMCC, CATT) propose to additionally consider the maximum number to be 8. Proponents provide at least the following reasons:

  - Support larger repetition number can add design flexibility to ensure safer practical deployment, and make the system future proof.

- In addition, when more than 2 times of repetition is supported, one more bit is needed to indicate the repetition number. When the maximum repetition number is 4, OPPO suggests reserving 1 indication field for forward compatibility (i.e., the repetition number supported in Rel-19 is {1, 2, 4}). Huawei and CMCC suggest supporting repetition number {1, 2, 3, 4}. When the maximum repetition number is 8, CMCC and CATT suggest supporting repetition number {1, 2, 4, 8}.

Round 1 discussion

      - [H] Proposal 5.1-v1

For the number of block-level repetitions, down-select from the following options:

- Option 1: {1, 2}

- Option 2: {1, 2, 4}

  - FFS: whether additional value of 3 or 8 or reserved.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

| Company | Comments |
| --- | --- |
| FUTUREWEI | Because at least option 1 is supported, the proposal should be For the number of block-level repetitions, down-select from the following options: at least 1 and 2 repetitions are supported - if 4 repetitions are supported - FFS: whether additional value of 3 or 8 or reserved. - Note: When the number of block-level repetition is 1, it indicates no repetition. Note with the same amount of signaling (2 bits for repetition and 1 bit for FEC), we can have improved system performance (rate 1/4 with R=1/2 FEC and 2 repetitions performs much better than 4 repetitions) |
| Huawei, HiSilicon | If we are intending to support a higher number of repetitions for flexibility and making the system future proof, we should support {1,2,3,4} repetition numbers. If we are focusing only on achieving the coverage targets for this release, then we support option 1. |
| Spreadtrum | We support Option 1. Option 1 is a simple way to support block-level repetition, only 1 bit is needed to indicate whether the repetition is needed and the repetition number. |
| Panasonic | Option 2. Additional value should be reserved depends on the signaling design. |
| vivo | We prefer option 1, because up to 2 repetition is sufficient to meet performance target. |
| HONOR | Support Option 1. |
| Samsung | We continue to prefer Option 1. Repetition is not the only method for coverage extension. Since the SI, we have further discussed alternatives for enhancing coverage, such as FEC, X-amble insertion, and longer data durations. Therefore, we believe that supporting more than two repetitions is unnecessary. In addition, if only two repetition levels are supported, this can be indicated with just 1 bit. |
| ZTE, Sanechips | As we commented above, we think it is not reasonable to support 2 times repetition without FEC, instead of 1/2 code rate. |
| OPPO | We support option 2. More repetition number is benefit for performance. Whether additional repetition number is supported or not can be discussed further. |
| Xiaomi | We support Option 1. Block level repetition with number of 2 can provide good performance and can reach the coverage target. Number 4 does not provide much more performance gain than number 2. Furthermore, {1,2,4} requires 1 more bit for indication overhead. |
| QC | Support Option 1. |
| Ericsson | Option 1 |
| NEC | We support Option 2 with 4 repetitions for coverage. |
| DOCOMO | Support option 1. The target coverage of Rel-19 can be already achieved by repetition factor 2, then we think it is not necessary to support any further coverage in Rel-19. If further coverage enhancement is necessary, it can be done in future release. |
| Nokia | Support Option 2 |

Round 2 discussion

      - [H] Proposal 5.1-v2

For the number of block-level repetitions, {1, 2} are supported.

- FFS number of repetitions 4 is supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

| Company | Comments |
| --- | --- |
| FL | From the first round of comments, Option 1 has more preferences. But FL tends to agree proponents of Option 2 that having a larger number can provide flexibility and making the system future proof. Please provide further comments. No need to repeat comments from the 1st round. |

R2D/D2R CRC

(CLOSED) CRC attachment

Summary of inputs

In the current version of TS 38.291, the description of attachment order and position of CRC parity bits is removed and pending RAN1 agreement. From the submitted contributions, 2 companies provide different views on this issue. Huawei proposes to reuse the mechanism in TS 38.212, i.e., CRC parity bits are appended to the end of the input bits. In contrast, Qualcomm proposes to add CRC parity bits before last (K-1) information bits to avoid the additional processing time to finish CRC encoding.

#### FL comments

In previous RAN1 meetings, companies are not convinced to use the first (K-1) information bits to initialize the shift register of the CC encoder although it can avoid the additional processing time introduced to finish CRC encoding. In the last RAN1 meeting, companies reached consensus to handle the processing time under the discussion on timing relationship in AI 9.4.4. In this sense, FL does not see a strong need to further optimize CRC attachment for the same reason.

Round 1 discussion

      - [H] Proposal 6.1-v1

- For attaching the CRC, the parity bits are appended to the end of the input bits.

- Support the following text proposal for Section 7.1 CRC calculation for TS 38.291.

- 7.1 CRC calculation <Unchanged parts are omitted> The bits after CRC attachment are denoted by b0,b1,…,bB-1, where B=A+L. The relation between ak and bk is: bk=akfor k=0,1,2,…,A-1 bk=pk-Afor k=A, A+1,…, A+L-1. <Unchanged parts are omitted>

| Company | Comments |
| --- | --- |
| FUTUREWEI | Ok |
| Huawei, HiSilicon | We support the proposal, but the exact TP can be left up to the TS editor. |
| Spreadtrum | Support. |
| Panasonic | Our preference is to revise the previous agreement to use the first (K-1) information bits to initialize the shift register of the CC encoder. |
| vivo | Support the proposal |
| HONOR | Ok |
| Samsung | Agreed |
| ZTE, Sanechips | OK |
| OPPO | OK |
| Xiaomi | OK |
| DOCOMO | Support. |

(CLOSED) Joint or separate CRC

Summary of inputs

Based on the contributions submitted in this meeting, a few companies provide views regarding joint or separate CRC if L1 control is supported:

- Joint CRC (1): CATT

- Separate CRC (5): Samsung, OPPO, LGE, ASUSTeK, IIT Kanpur. Samsung proposes to use CRC-6 while others have no clear proposals on whether CRC-6 or CRC-16 is used.

#### FL comments

So far, it seems that RAN1 has no clear understanding on whether and which scheduling information is needed to be carried in L1 control. FL captures the proposed options in the proposal and once any progress is made in AI 9.4.4, we can further discuss this issue under this agenda item.

Round 1 discussion

      - [M] Proposal 6.2-v1

If L1 R2D control information is supported, for CRC attachment, down-select from the following options:

- Option 1: Joint CRC is attached.

- Option 2: Separate CRC is attached. CRC-6 is used for R2D L1 control information.

| Company | Opt. | Comments |
| --- | --- | --- |
| FUTUREWEI |  | As the FL noted, we need to decide where L1 R2D is supported first. |
| Huawei, HiSilicon |  | It is necessary to discuss this only if L1 R2D control information has been agreed to be supported. |
| Spreadtrum |  | The proposal should be postponed until an agreement is achieved for L1 R2D control information. |
| Panasonic |  | As discussed in agenda 9.4.2, our preference is to have separate CRC in the MAC layer. Scheduling information including the device ID is protected by separate CRC. Separate CRC is not required to limit to the case to use L1 control information. Such design allows to manage different device ID length and different formats of PRDCH while allowing non-scheduled device not to receive the remaining PRDCH. In addition, the error rate of scheduling information can be reduced as the scheduling information is shorter length than the data. |
| vivo |  | Similar view with other companies to first determine whether L1 control. |
| HONOR | 2 | If R2D L1 control is supported. |
| Samsung | Opt. 2 | We fundamentally prefer Option 2. However, we are also fine with waiting for the decision in A.I. 9.4.4. |
| ZTE Sanechips | Option 1 | We suggest Option1 for the fewer overhead. |
| OPPO | Option 2 |  |
| Xiaomi | Option 1 |  |
| QC | 2 | We support separate CRC for control and data. |
| NEC |  | Prefer to defer the discussion, since the existence of L1 R2D control has not been agreed. |
| DOCOMO | 2 | The motivation to have L1 control information is early termination of subsequent procedure upon the success on CRC check for separate CRC for L1 control. Therefore, if L1 control information is supported, CRC should be separately attached, otherwise, no major difference from having control information in higher layer. Given that the L1 control information is expected to be smaller than 24 bits, we are fine to fix the CRC length as 16 bits for L1 control information. |

Others

| Source | Observations / Proposals |
| --- | --- |
| Ericsson | Observation 1: OOK modulation without Manchester encoding has several drawbacks, such as synchronization issues, threshold sensitivity, and susceptibility to noise. Proposal 1: Support Manchester line code as specified in TR 38.769 for both R2D and D2R with OOK modulation. |
| TCL | Observation 2: It can be observation from RAN 1 #118b that there are different Btx,D2R and transmission power of the D2R transmissions due to XOR operation. Proposal 4: Support option 1 or no D2R line code (e.g., by using a square-wave corresponding to the small frequency-shift) Observation 4: As the impact of SFO of device 1, each repetition block needs to consider the potential time offset (△t) to avoid the impact of SFO, which is up to the device’s capability and each block length. Proposal 7: Consider D2R block-level repetition based on following aspects, - Frequency resource for each repetition block, e.g., if hopping frequency could be considered for each block - How to indicate frequency resource and repetition number (R) Observation 5: Limited by one bit length, the impact of SFO during one or multiple bit may be ignored, which is up to repetition number of bit-level repetition. Observation 6：For the number of bit-level repetition, it is up to coverage and access performance and it will impact on D2R frame structure selection. Proposal 8: Consider D2R bit-level repetition based on following aspects, - Repetition number (R) - How to indicate time/frequency resource and repetition number (R) |
| CATT | Observation 1: With Manchester line coding, the R2D information data rate can be described as 1/(2*R2D chip length), and the R2D chip length is determined by the M value of OOK-4 and the OFDM symbol duration with 15k SCS. Proposal 3: From the perspective of robustness of TBCC to SFO, D2R preamble design should ensure that residual timing error does not exceed 1000ppm. Observation 2: With same coding rate, TBCC code has about 6dB coding gain than that of Manchester code at BLER=10%. Observation 3: Compared with CC code only, CC concatenated with Manchester code would introduce unnecessary redundancy, while its performance is poor than that of convolutional code with repetition operation in case of SFO=1000ppm. Proposal 3: Manchester line code should not be supported together with CC coding. Observation 4: The phase discontinuity issue of OOK demodulation by the reader using the BPSK demodulation method after DC removal would severely degrade the OOK demodulation performance if the reader does not know the D2R waveform is OOK or BPSK. Proposal 4: In order not to degrade the OOK demodulation performance, the reader should be aware of the modulation scheme of OOK or BPSK for the D2R transmission. Proposal 15: Although SFS is equivalent to Manchester code bit level repetition in bit mapping perspective, it cannot provide redundancy bits to obtain repetition coding gain. Manchester codeword repetition using for SFS purpose should not be supported as part of repetition coding scheme. Proposal 16: Bit level repetition should not be supported for Rel-19 A-IoT. |
| China Telecom | Proposal 8: Sub-block interleaving is not considered for Ambient IoT Device 1. |
| Qualcomm | Observation 12: For random access, additional processing time due to CRC for msg1 and 3 are not needed. Observation 13: For command, processing time needs to consider CRC compute time. Observation 14: Adding additional zeros at the end of block could convert LTE TBCC to ZTCC and reduce processing time by initializing shift register with zero tail bits. Observation 15: Adding CRC before last K-1 information bits can reduce processing time by initializing SR with information bits (instead of CRC). Proposal 10: Regarding CRC position and processing time, discuss whether/how to improve current TBCC. |
| Docomo | Proposal 1: For small frequency shift of D2R transmission, discuss whether FDMA is supported for other D2R transmissions than Msg1 and Msg3. Proposal 14: For D2R, whether to apply block-level repetition should be controlled by reader. Proposal 15: For D2R, block-level repetition should be applicable to any PDRCH transmission regardless of message type. - Whether to apply block-level repetition can be indicated in any R2D which schedules D2R. |

Collection of agreements

RAN1#120

Agreement

For R2D Manchester line code, the specification is according to TR 38.769 as follows:

- A chip corresponds to one modulated symbol.

- For Manchester encoding, the bit-to-chip mapping is: bit 0→chips{10}, bit 1→chips{01}

Agreement

For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length):

- the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation

- the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation.

- Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

Agreement

For D2R repetition, block level repetition from TR38.769 is supported.

Agreement

RAN1 concludes that generator polynomials of LTE CC with constraint length of 7 and coding rate of 1/3 from TS36.212 are reused as the mother code for Ambient IoT.

Agreement

RAN1 concludes that the generator polynomials in TS 38.212 for 6-bit CRC and 16-bit CRC are reused for Ambient IoT.

Agreement

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS impact of segmentation, if any

  - Note: impact may not be in RAN1

Agreement

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS impact of segmentation, if any

  - Note: impact may not be in RAN1

Agreement

One or both of the following options are supported to determine when no CRC is used,

- Option 1: A threshold of number of information bits Y. When the number of information bits is ≤ Y bits, no CRC is used. Down-selection from the following for Y:

  - Alt. 1: 16

  - Alt. 2: 8

  - Alt. 3: 6

- Option 2: Specified condition(s), e.g., device transmits PDRCH for Msg 1 upon receiving a PRDCH triggering random access. FFS specified condition(s) and/or how to determine the specified condition(s).

Agreement

For the initial values of the shift register of the CC encoder, down-select by RAN1#120bis from the following two options:

- Option 1: The initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits defined in TS 36.212.

- Option 2: The initial values of the shift register of the CC encoder are set to the values corresponding to the first (K-1) information bits, and the first (K-1) bits are appended to the end such that the initial state and ending state of the shift register are the same.

- Note: K is the constraint length.

RAN1#120bis

Agreement

For the initial values of the shift register of the CC encoder, the initial values of the shift register of the CC encoder are set to the values corresponding to the last (K-1) information bits defined in TS 36.212.

- Note: K is the constraint length.

- For discussion on timing relationships in 9.4.4, the entire processing time for encoding (including finishing CRC encoding) should be taken into account

Agreement

When CRC is attached to a PRDCH or PDRCH transmission, when the number of information bits is ≤ 24 bits, CRC-6 is used. Otherwise, when the number of information bits is > 24 bits, CRC-16 is used.

Agreement

There is no case in D2R or R2D where CRC is not attached.

Conclusion

RAN1 will not address the FFS in the agreement from RAN1#120:

Agreement

When CRC is attached to a PRDCH or PDRCH transmission,

- When the number of information bits is ≤ X bits, CRC-6 is used. Otherwise, when the number of information bits is > X bits, CRC-16 is used. Down-selection by RAN1#120bis from the following for X considering the balance of overhead and probability of undetected error:

  - Alt. 1: 24

  - Alt. 2: 56

- FFS impact of segmentation, if any

  - Note: impact may not be in RAN1

Agreement

The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table:

- FFS further down-selections of Tchip, Tb, and R to be supported

- Note: Detailed indication signaling of D2R scheduling information is discussed in AI 9.4.4.

|  |  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
|  | 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
|  | 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
|  | 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
|  | 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
|  | 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
|  | 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
|  | 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
|  | 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
|  | 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
|  | 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
|  | 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
|  | 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
|  | 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
|  | 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |
| Note: For further down-selection regarding the bit duration, - bit duration 266.67μs, 133.33μs and 66.67μs to be kept, which provides better multiplexing capability in the frequency domain. - Down-select to 1 between bit duration 1.39μs and 1.04μs, which achieves the competitive peak data rate larger than 640 kbps. - The remaining bit durations will be further discussed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Agreement

For block-level repetition of a D2R transmission, the repeated blocks are transmitted in the single PDRCH of the D2R transmission.

RAN1#121

Agreement

For D2R transmission,

- The minimum Tb is 1.39μs.

- Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis.

|  | Tchip (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tb (μs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133.33 | 66.67 | 33.33 | 16.67 | 11.11 | 8.33 | 5.56 | 4.17 | 2.78 | 2.08 | 1.39 | 1.04 | 0.69 | 0.52 |
| 266.67 | R=1 | R=2 | R=4 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |  | R=256 |
| 133.33 |  | R=1 | R=2 | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 | R=96 | R=128 |
| 66.67 |  |  | R=1 | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 | R=48 | R=64 |
| 33.33 |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 | R=24 | R=32 |
| 22.22 |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  | R=16 |  |
| 16.67 |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 | R=12 | R=16 |
| 11.11 |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  | R=8 |  |
| 8.33 |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 | R=6 | R=8 |
| 5.56 |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |  |
| 4.17 |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  | R=4 |
| 2.78 |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |  |
| 2.08 |  |  |  |  |  |  |  |  |  |  |  | R=1 |  | R=2 |
| 1.39 |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |  |
| 1.04 |  |  |  |  |  |  |  |  |  |  |  |  |  | R=1 |

Note: This does not imply the device to pre-store the table.

Agreement

For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, e.g., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dK-10,dK-11,dK-12, with the code rate of 1/3.

Agreement

For attaching the CRC, the parity bits are appended to the end of the input bits, according to the order in TS38.212.

Agreement

For the number of block-level repetitions, {1, 2} are supported.

- Note: When the number of block-level repetition is 1, it indicates no repetition.

Agreement

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, the reader indicates

- a single bit duration Tb which is same for all frequency domain resources

- a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R

  - note: the set of R values could be signalled using a bitmap

The detailed signalling design is left to RAN2.

Agreement

For frequency domain resource for Msg 3 transmission determined based on explicit indication in the PRDCH for Msg 2 transmission for one or multiple devices, the reader indicates:

- a single bit duration, which is same for all frequency domain resources,

- a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R

  - note: the set of R values could be signalled using a bitmap

The mapping relationship between device and its Msg 3 frequency domain resource is left to RAN2.

  - Note: Device could determine its R value for Msg 3 transmission based on its order of random ID in Msg 2

The detailed signalling design is left to RAN2.

References

- RP-250796, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, 3GPP TSG RAN Meeting #107.

- R1-2503221	Coding aspects for Ambient IoT	Ericsson

- R1-2503225	Coding aspects for A-IoT physical channel	FUTUREWEI

- R1-2503295	Physical channel design on channel coding	Huawei, HiSilicon

- R1-2503300	AIoT Physical channels design - line coding, FEC, CRC, repetition aspects	Nokia

- R1-2503312	Discussion on Ambient IoTcoding and SFS	ZTE Corporation, Sanechips

- R1-2503359	Remaining issues on line coding, FEC, CRC and repetition for A-IoT	vivo

- R1-2503516	Discussion on line coding, FEC, CRC, repetition aspects for Ambient IoT	Spreadtrum, UNISOC

- R1-2503537	Discussion on other aspects for Ambient IoT physical design	TCL

- R1-2503567	Views on Physical channels design – line coding, FEC, CRC, repetition aspects	Samsung

- R1-2503618	Discussion on Physical Channel Designs for A-IoT	Panasonic

- R1-2503794	Ambient IoT channel coding and small frequency shift	CATT

- R1-2503832	Discussion on coding aspects of physical channel design	CMCC

- R1-2503883	Discussion on line coding, FEC, CRC and repetition aspects for Ambient IoT	Xiaomi

- R1-2503925	Physical layer design – line coding, FEC, CRC, repetition aspects	NEC

- R1-2504047	Discussion on physical channels design about line coding, FEC, CRC, repetition aspects for Ambient IoT	China Telecom

- R1-2504089	Discussion on coding aspects	Fujitsu

- R1-2504099	Discussion on Physical channels design for Ambient IoT-other aspects	HONOR

- R1-2504206	Discussion on physical channels design for A-IoT	OPPO

- R1-2504244	A-IoT PHY layer design - line coding, FEC, CRC and repetition aspects	LG Electronics

- R1-2504261	A-IoT - PHY line coding, FEC, CRC, repetition aspects	MediaTek Inc.

- R1-2504288	Remaining issues on coding aspects for Ambient IoT	InterDigital, Inc.

- R1-2504319	On remaining coding aspects for Ambient IoT	Apple

- R1-2504395	Physical channels design – line coding, FEC, CRC, repetition aspects	Qualcomm Incorporated

- R1-2504434	Discussion on coding aspects	Sharp

- R1-2504474	Discussion on A-IoT Physical channels design	ASUSTeK

- R1-2504502	Discussion on coding and CRC aspects of physical channel design for Ambient IoT	NTT DOCOMO, INC.

- R1-2504634	Discussion on other aspects of Phy design for AIoT	IIT Kanpur

- Appendix: Summary of proposals

Small frequency shift

#1 D2R bit duration, chip duration, and R

| Source | Proposals |
| --- | --- |
| Ericsson | Proposal 4: The reader can indicate the D2R chip duration and repetition during device-specific scheduling restriction; hence, no additional down-selection among the R values is required in RAN1. |
| Futurewei | Proposal 4: For D2R transmission, down select bit duration Tb=1.39 μs with the chip duration corresponding to Tchip=0.69 μs; the rest of the bit duration and chip duration along with the SFS factor R are given in the following table: Proposal 5: For D2R transmission, a set of feasible SFS factors is R={1, 4, 8, 16, 32, 64, 96, 128}, which can be signaled with 3 bits. Proposal 6: For D2R transmission, the set of feasible chip intervals is Tchip=133.33, 66.67, 33.33, 8.33, 4.17, 2.08, 1.04, 0.69 μs, which can be signaled with 3 bits. |
| Huawei | Observation 1: The minimum bit length Tb,min can be selected as 1.39 μs for achieving comparable peak rate with RFID and saving power consumption compared with Tb,min = 1.04 μs. The corresponding minimum chip length Tchip,min is 0.69 μs. Proposal 1: For D2R: - The D2R chip durations are {133.33, 66.67, 33.33, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39, 1.04, 0.69}μs. - The minimum information bit duration Tb is 1.39 μs. Observation 2: There is no need to further optimize the candidate set of R value since the difference of indication overhead is marginal and RAN1 does not need to specify the reader’s implementation. Proposal 2: The following R set is supported as long as the reader by implementation ensures that overlapping R values are not selected for devices to use at the same time. - The D2R small frequency shift R values are {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64, 96, 128}. |
| ZTE | Observation 1: In the agreed table, the R value of 12, 24, 48, 96 is three times of 4, 8, 16, 32, respectively. Thus the performance would be affected by the corresponding 3rd harmonic. Observation 2: The interference due to partial main-lobe overlap has impact on the multi-access performance, including: - The interference of partial main-lobe overlap between R={1, 2} results in about 2dB loss @10%BLER for R=1 and about 1dB loss @1%BLER for R=2; - The interference of partial main-lobe overlap between R={6, 8} results in about 0.7dB loss @1%BLER for R=6 and about 1dB loss @1%BLER for R=8; - The interference of partial main-lobe overlap between R={12, 16} results in about 0.6dB loss @1%BLER for R=12 and about 0.6dB loss @1%BLER for R=16; - The interference of partial main-lobe overlap between R={24, 32} results in about 1dB loss @10%BLER and more than 6dB @1%BLER for R=24, and about 1dB @10%BLER and more than 6dB @1%BLER for R=32. Proposal 1: Considering the performance loss caused by the main-lobes overlap in the presence of SFO, it’s recommended not to use the following R values simultaneously: - R=1 and R=2; - R=6 and R=8; - R=12 and R=16; - R=24 and R=32. Observation 3: The interference due to 3rd harmonic has non-negligible impact on the multi-access performance, including: - The performance of R=6 is degraded about 0.4dB at 10%BLER and more than 3dB at 1%BLER, resulting from the interference of 3rd harmonic of R=2; - The performance of R=12 is degraded about 1dB at 10%BLER and more than 3dB at 1%BLER, resulting from the interference of 3rd harmonic of R=4; - The performance of R=24 is degraded more than 4dB at 1%BLER, resulting from the interference of 3rd harmonic of R=8; - The performance of R=48 is degraded about 1.5dB at 10%BLER and more than 3dB at 1%BLER, resulting from the interference of 3rd harmonic of R=16; - The performance of R=96 is degraded about 3dB at 10%BLER, resulting from the interference of 3rd harmonic of R=32. - Proposal 2: Considering the performance loss caused by the 3rd harmonic interference, it’s recommended not to use R values of 6, 12, 24, 48 and 96. - Proposal 3: Further considering the harmonics interference resulting from R=1, R={2,4,8,16,32,64,128} is proposed for the FDM-ed D2R transmission, while R=1 is proposed to be used for the single-device transmission. Observation 4: If the D2R bit durations sorted in descending order progressively decreases by 1/2, the number of D2R chip durations can be reduced. Proposal 4: Considering the indication overhead and the implementation complexity, the down-selected potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: |
| Vivo | Proposal 2: Support table 2 as candidate set for D2R chip duration Tchip and R, supporting up to Y=7 FDMed D2R transmission. |
| Spreadtrum | Observation 1: The number of 8 values with {1, 4, 8, 16, 32, 64, 128, 256} for R is sufficient to meet the system capacity requirements. Proposal 1: The potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, are shown in the following table: |
| TCL | Observation 1: The user experienced data rate target is in the range of 0.1kbps~ 5kbps for R2D and D2R transmission. Proposal 1: The maximum D2R chip duration is 133.33 μs (assuming 1.92MHz sampling clock). Proposal 2: The minimum D2R chip duration is 1.39 μs. Proposal 3: The set of D2R chip duration is {133.33, 66.67, 33.33, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39}μs and further down-selection could be considered if possible. Observation 3: SFS factor setting is associated with SCS/harmonic components/guard band defined in RAN4. Proposal 5: Support R=2n as common SFS factor and no special R should be down-selected in Rel-19. |
| Samsung | Proposal 5: For D2R small frequency shift, only cases where R=2n are to be supported. Proposal 6: Down-selection of Tb, Tchip and R according to the table below. |
| Panasonic | Proposal 1: The minimum D2R chip duration should be 1.04 μs. The maximum SFS factor of R=128 should be supported to accommodate up to 8 FDMed devices. Tb of 2.78 μs, 5.56 μs, 11.11 μs, 22.22 μs are not required to be supported. |
| CATT | Proposal 5: The D2R chip duration of {133.33, 66.67, 33.33, 16.67, 11.11, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39, 1.04, 0.69, 0.52} μs should be supported for Rel-19 A-IoT. Proposal 6: The D2R bit duration could support both of 1.39μs and 1.04μs for Rel-19 A-IoT. If one D2R bit duration needs to be down-selected between 1.39μs and 1.04μs, D2R bit duration of 1.39μs should be supported. Observation 5: D2R transmissions with SFS factor R=2n have similar BLER performance without FDM performance degradation; however, the number of available D2R channels is reduced tremendously. This will reduce the spectrum utilization. Observation 6: The BLER performance of D2R channels with SFS factor 3*R, 5*R and 7*R, due to odd harmonics interference, would be degraded by about 3dB, 1.5 dB and 0.8dB, respectively without interference cancellation at the reader. However, reader can perform interference mitigation with simple harmonics interference cancellation to ensure the BLER performance. Observation 7: The BLER performance of D2R channels with 3rd harmonics interference cancellation is improved by 2dB compared with that of the D2R channel without harmonic interference cancellation. Proposal 7: Considering very poor FDMA capacity performance of SFS factor Alt.2, R=2n only should not be supported for determination of the value of SFS factor in Rel-19 A-IoT. Proposal 8: SFS factor Alt.1: should be supported with following R values |
| CMCC | Observation 1: A 2.88 MHz clock can provide a comparable and slightly higher sampling rate than UHF RFID, where the device can implement a 2.56 MHz sampling clock. Observation 2: The values of D2R chip duration or bit duration should be related to (e.g., a multiple of or a factor of) the R2D chip duration, as the device performs the D2R clock calibration from R2D CAP. Observation 3: RAN1 supports M = 2, 6, 12, 24 for R2D transmission, a 2.88 MHz sampling speed is a common multiple of both 2 and 3, which can avoid non-integer counting errors when device calibrates samples for D2R chip duration. Proposal 1: Support the following 8 values of D2R bit duration and corresponding D2R chip duration (when R = 1): - For the D2R bit duration: 266,67 μs, 133.33 μs, 66.67 μs, 33.33 μs, 16.67 μs, 8.33 μs, 4.17 μs, 1.39 μs. - Notes: the first three values have been agreed in RAN1#120bis. - For the D2R chip duration (R = 1): 133.33 μs, 66.67 μs, 33.33 μs, 16.67 μs, 8.33 μs, 4.17 μs, 2.08 μs, 0.69 μs. Observation 4: From reader reception perspective, as long as the main lobe of the frequency domain resources between different multiplexed devices are non-overlapped, the reception performance can be guaranteed. Observation 5: Using R = 2n, the selection of R can be more flexible and provide more multiplexing capability in the frequency domain. Observation 6: Smaller R values can be selected using R = 2n than using R = 2n, which are beneficial for D2R link performance, as smaller R values correspond to larger D2R chip durations. Proposal 2: The following R values and the maximum number of multiplexed devices in the frequency domain are supported: - R = {1, 2, 4, 8, 16, 32}, if the maximum number of multiplexed devices Ymax = 4. - R = {1, 2, 4, 8, 16, 32, 64}, if the maximum number of multiplexed devices Ymax = 6. |
| Xiaomi | Observation 1: For D2R FDMA, R=2 and R=4 (M=4 and M=8), the interference between main lobes and side lobes from different devices has negative impact on FDMA performance. Observation 2: For D2R FDMA, R=2 and R=6 (M=4 and M=12), and R=4 and R=6 (M=8 and M=12), the interference between main lobes and side lobes from different devices can be negligible. Proposal 1: For D2R FDMA, the case of R=2 and R=4 (M=4 and M=8) is not selected. Proposal 3: For D2R chip duration supports value set of {133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 2.08, 1.04, 0.52} μs. |
| NEC | Proposal 2: The value R=2 should be removed from the Tchip-Tb-R table. |
| China Telecom | Proposal 1: Support R values based on 2n. Proposal 2: Delete the value of 1.04μs for D2R bit duration. Proposal 3: Delete the value of 0.52μs for D2R chip duration. Proposal 4: Support the following table for D2R chip duration, bit duration and SFS factor R. |
| OPPO | Proposal 1: For the down-selection of Tchip, Tb, and R to be supported in Rel-19 A-IoT, we propose keep R values in black colour and remove all red ones in Table 1. |
| LGE | Proposal 4: The set of D2R chip duration values supported for D2R transmissions are determined considering the following aspects: - The minimum D2R chip duration value should be comparable to or no larger than 1 / (640*2 kHz) = 0.78 us - The maximum D2R chip duration value should be comparable to or no smaller than 133.33 us. - The D2R chip duration values should be easily derived by R2D chip duration values Proposal 5: The minimum D2R bit duration for D2R transmission is 1.39 us Proposal 6: The minimum D2R chip duration for D2R transmission is 0.69 us Proposal 7: The maximum D2R chip duration for D2R transmission is 133.33 us Proposal 8: For D2R small FS, support small FS factor R = {1, 2, 4, 8, 16, 32, 64, 128}. |
| InterDigital | Proposal 1: The following D2R chip duration Tchip, bit duration Tb, and SFS factor R are supported: |
| Apple | Proposal 1: Adopt following values for D2R chip duration Tchip, bit duration Tb, and SFS factor R: |
| Qualcomm | Proposal 1: Define Frequency shift as FS = 1/2*chip rate (Hz). Observation 1: - Fractional number of device clock cycles still could be used for D2R transmission with frequency uncertainty. - Reader should allow certain levels of uncertainty of frequency shift considering 1) limited resolution of device clock cycle and 2) device clock SFO. - RAN4 should specify frequency tolerance considering these above two aspects. - To support chip durations 0.69*2^n, 0.52*2^n for n=0,1,2,… , clock speed of 1.92MHz, 2.16MHz or 2.88MHz are feasible and good options. Observation 2: - 0.69us can support bit rate up to 720kbps. - 0.52us chip duration can support bit rate up to 960kbps. Observation 3: - Considering that some device can be implemented based on method 1, it would be good to have 2^n ratio between R2D chip duration and D2R chip duration. Observation 4: - If M=2 is indicated in R2D, D2R can easily support D2R chip duration of {133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 2.08, 1.04, 0.52}us, which are black coloured numbers. - If M=6, 12, or 24 is indicated in R2D, D2R can easily support D2R chip duration of {11.1, 5.56, 2.78, 1.39, 0.69}us. Observation 5: - For low R2D data rate, black numbers {133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 2.08, 1.04, 0.52} could be easily used for D2R, - For high R2D data rate, red numbers {11.1, 5.56, 2.78, 1.39, 0.69} could be easily used. Observation 6: Given that max D2R rate is likely to be used with M=24, D2R max bit rate could be 720kbps (with R=1) (chip duration of 0.69us is 1/4 of the R2D chip duration with M=24). Proposal 2: Support max bit rate of 720kbps. Proposal 4: - RAN1 support following table for D2R transmission. - Shortest Tb is 1.39us (720kbps). - Shortest chip duration is 0.52us. - Allows up to 8 FDM positions - Tb = {266.67, 133.33, 66.67, 33.33, 22.22, 11.11, 5.56, 1.39} - 8 different bit duration to assign 3 bits for Tb indication - Tchip = {66.67, 33.33, 16.67, 8.33, 5.56, 4.17, 2.78, 2.08, 1.39, 1.04, 0.69, 0.52} |
| Sharp | Proposal 1: Regarding down-selection between 1.39 μs and 1.04 μs for D2R bit length, adopt 1.39 μs. Proposal 2: In the table providing the potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, remove the column for Tchip = 0.52 μs. |
| NTT Docomo | Proposal 2: For D2R, the minimum value on information bit duration Tb should be 1.39μs. Proposal 3: For D2R, the minimum value on chip duration Tchip should be 0.69μs. Proposal 4: For D2R, the minimum value on small frequency shift factor R should be 1. Proposal 5: For D2R, supported values on small frequency shift factor R should be 2n, where n=0, 1, 2,…,7. Proposal 6: For D2R, the maximum value on chip duration Tchip should be 133.33μs. Proposal 7: For D2R, supported values on chip duration Tchip should be {133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 2.08, 1.04, 0.69}μs. |

#2 Number of multiplexed devices

| Source | Proposal |
| --- | --- |
| Huawei | Proposal 3: The supported Y values are {1, 2, 4, 8}. |
| Vivo | Proposal 2: Support table 2 as candidate set for D2R chip duration Tchip and R, supporting up to Y=7 FDMed D2R transmission. |
| CMCC | Proposal 2: The following R values and the maximum number of multiplexed devices in the frequency domain are supported: - R = {1, 2, 4, 8, 16, 32}, if the maximum number of multiplexed devices Ymax = 4. - R = {1, 2, 4, 8, 16, 32, 64}, if the maximum number of multiplexed devices Ymax = 6. |
| China Telecom | Proposal 6: Support the maximum number of multiplexed devices Y as 8. |
| Honor | Proposal 3: The maximum value of Y for FDMA should equal to or less than 8. |
| LGE | Proposal 9: Support Ymax = 8 for A-IoT D2T transmissions. - Ymax: maximum number of multiplexed devices in frequency domain |
| Qualcomm | Observation 7: In D2R, 8 devices with bit rate of 3.75kbps can be supported. Proposal 3: Support max number of FDMed devices up to [8]. |
| Sharp | Proposal 3: The maximum number (Ymax) of FDM’ed devices is 8. Proposal 4: In the table providing the potential values of D2R chip duration Tchip, bit duration Tb, and SFS factor R, for each D2R bit length, down-select to no more than Ymax values of R (and corresponding Tchip). |

#3 Initialization signaling for FDMA resources

| Source | Proposal |
| --- | --- |
| E/// (9.4.4) | Proposal 5: Scheduling FDMA for Msg1 involves the reader providing a set of tuples containing (Tchip, R) such that their product Tchip x R remains constant for all the scheduled resources. Proposal 6: Additional restrictions on configuring combinations of (Tchip, R) are to be implemented by the reader so as to prevent overlaps in main lobes and restrictions due to harmonics. Observation 6: Since Msg3 scheduling is device-specific, there is a possibility to assign device specific parameters on preamble lengths, midamble fields, FEC, block repetitions etc. Observation 7: Device-specific scheduling might benefit from supporting different data rates as well. Proposal 7: Device-specific D2R scheduling can support same as well as different D2R data rates. Observation 8: Supporting a common D2R chip duration and a reader-configured set of R values can provide frequency resources supporting variable BWs which can be FDMA’ed without overlaps. However, overall resource allocation might not be very efficient in this case. Proposal 8: Indicating D2R chip duration and repetition separately during device-specific scheduling might be more efficient in terms of overall resource allocation. |
| Futurewei | Observation 2: Since the combination of chip intervals, bit durations, SFS factors is less than 32, joint coding with 5 bits is also possible. |
| Huawei | Proposal 4: The detailed signalling of indicating R, chip length and Y will be handled by RAN2. Proposal 5: Values of D2R chip duration Tchip, bit duration Tb, and SFS factor R in the following table are supported: |
| ZTE | Observation 5: The indication overhead of Alt.1 for Msg1 transmission is 5 bits. Observation 6: The indication overhead of Alt.2 for Msg1 transmission is 6 bits. Observation 7: The indication overhead of Alt.3 for Msg1 transmission is 6 bits. Proposal 5: For the indication of SFS resources for Msg1 transmission, Alt.1 (i.e., indicate a common Tchip combined with a R set) is preferred to reduce the indication overhead. Observation 8: The indication overhead of Alt.1 is 5*N bits for N Msg3 transmissions, and it increases with the increase of the number of Msg3 transmission. Observation 9: Alt.2 requires less indication overhead which is fixed for different number of Msg3 transmission. Proposal 6: For the indication of SFS resources for Msg3 transmission, Alt.2 (i.e., indicate a SFS resources set shared for all devices’ Msg3 transmission) is preferred to reduce the indication overhead. |
| Vivo | Proposal 3: For indicating chip duration and R for FDMA of Y D2R resource at least for Msg1 and Msg3 transmission, support following indications - Single reference chip duration for Y D2R resources - A bitmap with bit length Ymax for the set of candidate R values, e.g., {Ri}, where each Ri value is associated with one bit location, the bit set to 1 indicates that the Ri is used for a D2R resource; the bit set to 0 indicates that the Ri is NOT used - Ymax is determined by the maximum number of FDMed devices associated with the indicated reference chip duration, or determined by the maximum number of FDMed devices associated with any one of the candidate reference chip durations. Proposal 4: For indicating chip duration and R for D2R transmission other than Msg1 and Msg3 transmission, i.e., for non-FDMA D2R transmission, down-select from the following indication options - Option 1: Same indication method as used for Msg3 transmission - Option 2: 3-bit indication for D2R chip duration and 3-bit indication for R value |
| Spreadtrum | Observation 2: Reader indicate Y, D2R bit length, and R to the device to obtain the frequency domain resource for the corresponding D2R transmission. Proposal 2: For the same data rate for FDMed transmissions, e.g., FDMed Msg1 transmission, 12 bit is needed to indicate the frequency domain resource for the corresponding D2R transmission based on the potential value in the table. - 3 bit is needed for 8 candidate values of Tb within the table, i.e., 266.67μs, 133.33μs, 66.67μs, 33.33μs, 16.67μs, 8.33μs, 4.17μs, 1.04μs - 9 bit with a bitmap is needed for R and Y, each one bit corresponds to a Tchip |
| Samsung | Proposal 7: The configuration of Tchip and R shall be provided via higher-layer signaling via the corresponding PRDCH. Proposal 8: Different Tb values lead to different multiplexing capacities in the frequency domain. For each Tb, the set of available Tb values shall be predefined accordingly. Proposal 9: The reader can transmit a Tb value via an R2D message that triggers random access. Based on the indicated Tb, the device can randomly select one Tchip value from the corresponding set. (9.4.4) Proposal 19: The frequency domain resource for Msg3 is always determined based on explicit indication in the PRDCH providing Msg2. |
| Panasonic | Proposal 2: Explicit indication of transmission parameters should be used for scheduling Msg1 and Msg3 transmissions. |
| CATT | Observation 9: For the D2R channel by a small frequency shift, the device needs to know any two of the bit duration Tb, D2R chip duration and SFS factor R. Proposal 10: SFS factor R could be included for the indication of frequency channel in the R2D control information. Proposal 11: To indicate D2R chip duration Tchip and SFS factor R, there are two schemes can be considered - Option a: The D2R chip duration is implicitly indicated by deriving from the control signaling of bit duration Tb and SFS factor R. - Option b: The D2R chip duration and SFS factor R are explicitly indicated for the D2R transmission. Proposal 12: When the D2R chip duration is implicitly indicated by deriving from the control signaling of bit duration Tb and SFS factor R, three potential Options for D2R transmission indication should be considered, and the Option a-3 could be preferable for flexible indication. - Option a-1: The value/index of the bit duration Tb is explicitly indicated based on the pre-defined table, in which it also pre-defines the relationship of the bit duration Tb and corresponding chip duration Tchip and SFS factors R. - Option a-2: The value/index of the bit duration Tb and the SFS factor R are jointly explicitly indicated for D2R transmission via the control information of PRDCH. The R could be expressed as Rmax for FDMed Msg1 transmission, while it could be the dedicated R value for Msg3 or CFRA. - Option a-3: The value/index of the bit duration Tb is explicitly indicated and the corresponding subset of the R values are also indicated via the control information of PRDCH or based on the pre-defined table. Proposal 13: When the explicit indication of D2R chip duration is supported, the following two potential options should be considered and Option b-1 is preferable due to simple implementation. - Option b-1: The D2R chip duration index and the SFS factor R are carried in the control information in PRDCH. The D2R bit duration and corresponding R value(s) could be derived via the indicated chip duration and the R value (e.g. maximum/minimum R value) according to the predefined R value set or TBS related information. - Option b-2: The joint indication of the D2R chip duration and subset of R values for D2R transmission is carried in the control information in PRDCH. (9.4.4) Proposal 18: The frequency domain resource for each Msg3 transmission can be indicated in Msg2 by following two options: - Option 1: {bit duration, R} - Option 2: {chip duration, R} |
| CMCC | Proposal 3: For a D2R transmission, the combination of Tchip and corresponding R value is indicated in a combination. Proposal 4: To support FDMA with Y≥1 D2R transmissions, - If Ymax = 4, 6 bits is used to indicate a combination of Tchip and R for a D2R transmission, and 24 bits in total is needed to provide frequency domain resources to multiplex up to 4 devices. - If Ymax = 6, 6 bits is used to indicate a combination of Tchip and R for a D2R transmission, and 36 bits in total is needed to provide frequency domain resources to multiplex up to 6 devices. (9.4.4) Proposal 13: The frequency domain resource for Msg3 can be determined based on successfully reception state of Msg1 and predefined frequency resources orders. |
| Xiaomi | Proposal 2: Besides D2R chip duration, SFS factor R is indicated in R2D control information predefined a set of D2R SFS factor R values. Proposal 4: Joint indication on D2R chip duration and SFS factor R is supported. Proposal 5: For FDMed D2R transmission, predefine a set of combination of chip durations and SFS factor R, and indication is according to the predefined combination index. Proposal 6: The predefined set of combination of chip durations and SFS factor R should support target of multiple devices and single devices. Proposal 7: To indicate the maximum number of Msg 1 resources with X=2 and Y, the same SFS factor R values set is applied on both time domain occasions for X=1 and X=2. |
| NEC | Observation 1: The size of small frequency shift depends of Tchip only. Observation 2: For 2SB D2R, the frequency position of each sideband could be {3.75, 7.5, 15, 30, 45, 60, 90, 120, 180, 240, 360, 480, 720, 960} in kHz. Proposal 3: For D2R scheduling, Btx,D2R should be explicitly or implicitly indicated to device(s), and the adopted Tchip or R should be restricted by Btx,D2R. Proposal 4: For Msg1 scheduling, the bit duration Tb could be obtained by indicated Tchip and R, and the potential value of R could be determined by at least one of the following alternatives. - Alt 1: potential value of R could be selected by the row of Tb, and restricted by the indicated R; - Alt 2: potential value of R could be selected by the row of Tb, and restricted by Btx,D2R; Proposal 5: The FDRA field in Msg2 is a bitmap whose length is N which represents the number of candidate R values indicated by A-IoT paging message, the value set is same as Msg1. - When the nth bit in the bitmap is “1”, it means the nth SFS factor value Rn-1 in the SFS candidate value set {R0, R1…RN} is used for the corresponding ith device to determine the frequency domain resource: - Where i corresponds to the order in which the device ID appears in Msg2. |
| China Telecom | Proposal 4: Support the following table for D2R chip duration, bit duration and SFS factor R. Proposal 5: Support using 6 bits to jointly indicate the value of D2R chip duration, bit duration and SFS factor R. |
| Honor | Proposal 4: Use two parts signaling to indicate the D2R chip duration Tchip and SFS factor R values based on the table in which the first part indicates the Tb index and the second part indicates the available R values. (9.4.4) Proposal 7: Use the same bitmap to indicate the frequency resources for Msg3 of all responding users. |
| OPPO | Observation 1: In R2D control information, indication of allocated SFS factors R for FDMA of D2R transmissions is equivalent to the indication of D2R chip durations Tchip, since one can be derived from the other based on the supported data rate (Tb). Therefore, for a given bit duration / data rate (Tb), it is sufficient to indicate in the R2D control information either the SFS factors R or the D2R chip durations Tchip. Proposal 2: - Same Tb, repetition factor and FEC coding rate are applied to all FDMed Msg.3 transmission in response to a PRDCH for Msg.2 transmission. - Same Tb, repetition factor and FEC coding rate are applied to all FDMed Msg.1 transmission (if supported) in response to R2D transmission triggering CFRA. Observation 2: For Msg.1 transmission, the payload for indicating FDMA frequency positions in R2D control information can be significantly different between different indication approaches. In case of Tb = 266.67µs, (Btx_D2R = 15kHz), the maximum R2D payload is - Option 1 (Y only approach): 3 bits - Option 2 (index-based approach): at most 21 bits - Option 3 (bitmap-based approach): 7 bits Observation 3: For Msg.3 transmission, since the reader needs to flexibly schedule devices in different SFS frequency positions, the Y only approach (Option 1) cannot provide a full control to the reader for scheduling a specific frequency position (R) to a given device (RN16). For Option 2 and Option 3, the required payload for indicating FDMA frequency positions in Msg.2 control information in case of Tb = 266.67µs, (Btx_D2R = 15kHz), - Option 1 (Y only approach): not applicable - Option 2 (index-based approach): 3*number of scheduled Msg.3 in one PRDCH bits - Option 3 (bitmap-based approach): 7 bits Proposal 3: For scheduling of FDMed Msg.1 transmissions, the signalling design in the paging R2D control information should be based on the following: - Indication of Tb, repetition factor, and FEC coding rate(1 or 1/3) (all devices should have the same data rate) - Indication of Y value (total number of allocated SFS frequency positions) Proposal 4: For scheduling of FDMed Msg.3 transmissions, the signalling design in the Msg.2 R2D control information should be based on the following: - Indication of Tb , repetition factor, and FEC coding rate (1 or 1/3)(all devices should have the same data rate) - Indication of one or multiple of {device ID (RN16), R} |
| LGE | Proposal 1: Based on the D2R chip duration provided by a reader, an A-IoT device determines the small FS value for D2R transmission. - The information on D2R chip duration can be carried by R2D control information. Proposal 2: Based on the D2R chip duration and the small FS factor R provided by a reader, an A-IoT device determines the data rate (1/Tb) for D2R transmission. - The information on the small FS factor R can be carried by R2D control information. Proposal 3: The D2R FDM with the small FS can be scheduled with one of the following: - D2R chip duration Tc and the small FS factor R are indicated by R2D control information - D2R bit duration Tb and the small FS factor R are indicated by R2D control information Proposal 10: For D2R small FS, the set of combinations of supported Tc, Tb and R values are predefined in the spec and a reader indicates one combination (e.g., for CFRA) or a subset of the combinations (e.g., for CBRA) of the supported Tc, Tb and R values. (9.4.4) Proposal 1: If MSG2 does not provide explicit indication to the frequency resource index (e.g., frequency shift value) of Msg.3 transmission, when both FDMA of Msg.1 transmission and FDMA of Msg.3 transmission are scheduled, agree that the frequency resource index (e.g., frequency shift value) of Msg.3 transmission is defined to be the same as the frequency resource index of Msg.1 transmission for the same device. |
| InterDigital | Proposal 1: The following D2R chip duration Tchip, bit duration Tb, and SFS factor R are supported: |
| Apple | Proposal 2: Adopt a 2-bit long codepoint to indicate the D2R bit duration Tb Proposal 3: For signaling the SFS R factor, adopt one of the following options for boths cases of Y =1 and Y> 1 - Option 1: A fixed size bitmap independent of the D2R bit duration Tb, where each bit corresponds to R =1, 4, 8, 16, 32, 64, 128, 256 - Option 2: A variable size bitmap depending on the D2R bit duration Tb - For both options, “1” is used to indicate the application of corresponding SFS R factor from the bitmap |
| Qualcomm | Proposal 5: Following restriction is applied between R2D M and D2R Tb. - R2D with M=2 can indicate D2R Tb >= 33.33 - R2D with M>2 can indicate D2R Tb <=22.22 Observation 8 - Tb bit duration (or bit rate) is essential information to be signaled. - Y is not necessary to be signaled if reader allows all possible positions. If readers want to avoid or limit FS positions for some reason (e.g., reduce interference), then, Y could be directly or indirectly indicated. Proposal 6: Down-select from following options for FD resource allocation - Option 1: Bit map-based scheme - Tb: bit rate for D2R transmission - Bitmap for FS position R - Option 2: Rule-based scheme - Tb - Y: number of FS positions for D2R transmission - Rmax (optional) - Spec defines a simple rule to determine FS positions or R values - Option 3: Codepoint based scheme - Tb - Row index from a table TTb to indicate FS positions [R1, R2, … RY] - Option 4: Starting position and gap indication - Tb - Y: number of FS positions for D2R transmission - Rmin: starting FS position R - G: Gap between two neighboring FS positions Proposal 7: For non-FDM case (e.g., unicast D2R data or D2R feedback), FDRA indicates - Tb (3bits) - R: predefined as R=1 for Tb=1.39us (720kbps) and R=2 other Tb values Observation 9: For FDM resource allocation for Msg3 and CFRA, fewer positions could be used resulting in fewer bits for signaling. |
| Sharp | Proposal 5: At least for the case of same data rate (i.e. same D2R bit length of Tb) for Y FDM’ed D2R transmissions in response to a R2D transmission, Tb and Y are indicated in the D2R scheduling information carried in the R2D transmission, - The allocated values of R are determined such that the spacing between any two adjacent values of R is maximized in the pre-defined set of R values for Tb. - For each value of R, the corresponding D2R chip duration is Tb/(2R). |
| NTT Docomo | Proposal 8: For D2R, chip duration Tchip and small frequency shift factor R can be indicated - Option 1: independently, e.g., by codepoint/bitmap. - Option 2: by joint indication, e.g., codepoint indicates a predefined combination of R and Tchip. |

D2R FEC

#1 FEC code rate

| Source | Proposal |
| --- | --- |
| Ericsson | Proposal 2: When FEC is applied to a D2R transmission, the coding rate can be set to either 1/2 or 1/3. |
| Futurewei | Proposal 3. For Ambient IoT devices, support a rate Rcode=1/2 convolutional encoder with the output ordering d00,d01,d10,d11,… , where - dk0 is the output corresponding to the polynomial 133 (octal) dk1 is the output corresponding to the polynomial 173 (octal) |
| Nokia | Proposal 1: For D2R channel coding, support code rate ½ in addition to code rate 1/3 and code rate 1 (no channel coding). |
| ZTE | Observation 10: In LTE convolutional coding, for the code rate of 1/2, all bits of 1st and the 2nd parity bit streams are output. Observation 11: The SNR performance gap between code rate of 1/3 and no FEC is about 10dB. Observation 12: Compared to repeating twice without FEC, using a FEC of 1/2 coding rate can provide 5dB performance gain and reduce power consumption by about 70%. Proposal 7: Support code rates of Option 2: {1/2, 1/3} for convolutional coding. Proposal 8: R=1 and Rep=2 should not be supported. |
| Spreadtrum | Proposal 3: Only support the code rate of FEC with 1/3 if FEC is applied for a D2R transmission. Observation 3: To achieve the goals with supporting a higher performance, or a higher code rate, or a flexible code rate, it may cause redundant to discuss other alternative schemes like puncturing. |
| Samsung | Observation 1: Given that no FEC is already supported in D2R, there is limited justification for introducing puncturing to achieve higher code rates. Observation 2: Since block-level repetition has already been agreed upon, additionally repeating coded bits is unnecessary and redundant. Proposal 1: There is no consensus to support FEC code rate 1/2 in Rel-19. Observation 3: It can be difficult for the reader to determine whether each device is close enough to operate without FEC, based on proximity determination solution 1. Proposal 2: The use of FEC is configured via A-IoT paging. It is assumed that all devices operate in the same manner with respect to FEC usage within the same inventory or command round. - Note: This issue can be addressed under A.I. 9.4.4 or within this A.I. |
| Panasonic | Proposal 3: The FEC code rate of 1/2 should be supported by using 2 out of 3 output bits of CC polynomial generator. |
| CMCC | Proposal 5: If FEC is applied for a D2R transmission, only the 1/3 code rate is supported. |
| Xiaomi | Proposal 8: For D2R FEC, the target coding rate is the same as mother code with coding rate 1/3. |
| NEC | Proposal 1: For code rate of FEC, Option 2 with both {1/2, 1/3} is preferred. |
| China Telecom | Proposal 7: Support 1/2 and 1/3 code rate of FEC. - For the code rate of 1/2, it is achieved by bypassing the third parity chain with polynomial (165). |
| Fujistu | Observation 1: - Supporting at least one intermediate code rate between no FEC and 1/3 code rate can provide more flexibility to readers and bigger probability to achieve better network spectrum efficiency. - The 1/2 code rate is not perfect. If the AIoT item had time, it would be a good direction to further study which code rate is better than 1/2 as the intermediate code rate in between no FEC and 1/3 code rate. |
| Honor | Proposal 1: If FEC is applied for a D2R transmission, only support code rate 1/3. |
| OPPO | Proposal 5: only coding rate {1/3, 1} is supported for A-IoT in Rel-19. |
| LGE | Proposal 11: Applying or not applying the FEC for D2R is explicitly indicated by a reader by R2D control information. Proposal 12: For D2R, the convolutional code rate other than 1/3 is NOT supported in Rel-19 Ambient IoT. - Other code rates can be supported in combination with block-level repetitions, e.g., with the support of repetition number 2 and 4, the combined code rates of {1/2, 1/3, 1/4, 1/6} can be supported. |
| Qualcomm | Observation 10: Convolutional code can be punctured to generate various code rates including 4/5, 2/3, 1/2, etc. Observation 11: BLER performance of CC rate 1/2 and 1/3 are within 0.5 ~ 1dB. Proposal 9: Support multiple coding rates for D2R CC 2/3 and 1/3. |
| Sharp | Proposal 6: If FEC is applied for a D2R transmission, for the code rate of FEC, adopt Option 1 (i.e. 1/3). |
| NTT Docomo | Proposal 9: For FEC on PDRCH, other code rate than 1/3, e.g., 1/2 by puncturing, is not necessary. Proposal 10: For D2R, FEC should be applicable to any PDRCH transmission regardless of message type. - Whether to apply FEC can be indicated in any R2D which schedules D2R. |
| IIT Kanpur | Proposal 1: In addition to the existing 1/3 code rate, a code rate of 1/2 should be supported for FEC in D2R transmissions. |

#2 Bit collection

| Source | Proposal |
| --- | --- |
| Futurewei | Proposal 2. For Ambient IoT devices, the output of the rate R=1/3 convolutional encoder is ordered d00,d01,d02,d10,d11,d12,… , where - dk0 is the output corresponding to the polynomial 133 (octal) - dk1 is the output corresponding to the polynomial 173 (octal) - dk2 is the output corresponding to the polynomial 165 (octal). The possible text proposal for TS 38.291 is The sequence of bits e0,e1,…, eE-1 for transmission, where E=3D, is generated by: ek= d00,d01,d02,d10,d11,d12,…,dD-10,dD-11,dD-12.. Proposal 3. For Ambient IoT devices, support a rate Rcode=1/2 convolutional encoder with the output ordering d00,d01,d10,d11,… , where - dk0 is the output corresponding to the polynomial 133 (octal) - dk1 is the output corresponding to the polynomial 173 (octal) |
| Huawei | Proposal 7: For bit collection after FEC, the output bits for each input bit are arranged sequentially in accordance with the input bits, i.e., for input bits c0,c1,…,cK-1, the output of the FEC is d00,d01,d02,d10,d11,d12,…,dK-10,dK-11,dK-12. |
| ZTE | Proposal 9: Clarify that output bit sequence of convolutional coding is for code rate 1/3 and for code rate 1/2 , wherein bit corresponds to the 1st parity bit stream with polynomial(133), bit corresponds to the 2nd parity bit stream with polynomial(171), and bit corresponds to the 3rd parity bit stream with polynomial(165). Proposal 10: It is proposed that the bit collections in the TS_38.291 as in [3] can be described as below: After encoding the bits are denoted d0i,d1i, …, dD-1i, i=0,1,2, where i is the coded stream index and D is the number of bits in each coded stream, i.e., D=BR. The sequence of bits e0,e1,…, eE-1 for transmission, where E=ID and I=1R, is generated by: eIk+i=dki k=0, 1, …, D-1 i=0, 1, …, I-1.. |
| Vivo | Proposal 1: For FEC, the sequence of bits after channel coding and bit collection is d00,d01,d02, d10, d11,d12, …, dD-10, dD-11,dD-12, where dji is j-th encoded bits corresponding to i-th branch. |
| CATT | Proposal 1: For CC in A-IoT device 1 - Repetition operation should be integral multiples of CC codeword. - Bit collection should be performed based on natural order of three encoder output parity streams. - Note: - No need to support the sub-block interleaver(s) and puncturing operation of rate matching in LTE-TBCC. |
| Qualcomm | Proposal 11: For FEC bit collection, the sequence of FEC output bits e0,e1,…, eE-1 is generated by: e3k+i=dki for k=0, …, D-1 and for i=0,1,2 |

D2R repetition

#1 Number of repetitions

| Source | Proposal |
| --- | --- |
| Ericsson | Proposal 3: Support FEC with a coding rate of 1/2, which can be achieved through puncturing, instead of using simple repetition. |
| Futurewei | Observation 1. It is preferable to use convolutional codes alone or combine convolutional codes with at most 2 repetitions (Rblock=2) over repetitions alone. Proposal 1: For the number of block-level repetitions Rblock, Rblock=1 and Rblock=2 are supported. - Note: When Rblock=1, it indicates no repetition - Note: Rblock=2 is only used when channel coding is applied |
| Huawei | Proposal 6: For the D2R transmission of Ambient IoT, the maximum repetition number for block repetition is 2 or 4, with the supported repetition numbers being {1, 2} or {1, 2, 3, 4} respectively. |
| Nokia | Proposal 2: For D2R block-level repetition, support number of repetitions (Rblock) in { 1, 2, 4 }. |
| ZTE | Proposal 11: If R=1/2 is supported, at least support following code rates and repetitions combinations: {1, 1}, {1/2, 1}, {1/3, 1}, {1/3, 2}. Proposal 12: If R=1/2 is not supported, at least support following code rates and repetitions combinations: {1, 1}, {1/3, 1}, {1/3, 2}, {1/3, 4}. |
| Vivo | Observation 1: Up to 2 repetitions can be sufficient. |
| Spreadtrum | Proposal 4: For the number of block-level repetitions, {1, 2} repetitions are supported. - Note: When the number of block-level repetition is 1, it indicates no repetition. |
| TCL | Proposal 6: Support option 2 as the number of block-level repetitions. - Option 2: {1, 2, 4} - Note: When the number of block-level repetition is 1, it indicates no repetition. |
| Samsung | Proposal 4: For the number of block-level repetitions, {1, 2} repetitions are supported. - Note: When the number of block-level repetition is 1, it indicates no repetition. - Signalling details can be discussed under A.I. 9.4.4 or within this A.I. |
| Panasonic | Proposal 4: A block repetition number of 4 should be supported to achieve a desired coverage. |
| CATT | Observation 10: For D2R data rate of 240k, when the number of repetitions is less than 8, the target coverage distance of 15m cannot be reached. Proposal 14: 2 bits should be included PRDCH to indicate D2R block repetition number {1, 2, 4, 8}. |
| CMCC | Proposal 6: For the number of block-level repetitions in D2R transmission, Option 2: {1, 2, 4} is supported. If the option size must be expanded to four values, either {1, 2, 3, 4} or {1, 2, 4, 8} can be supported from our perspective. |
| NEC | Proposal 6: For block-level repetition, Option 2 with both {1,2,4} is supported. |
| China Telecom | Proposal 9: For the number of block-level repetition, support {1, 2,4}. |
| Fujistu | Proposal 1: Support {1, 2, 4} for D2R block-level repetitions. |
| Honor | Proposal 2: For the number of D2R block level repetition, support Option 1 {1, 2}. |
| OPPO | Proposal 6: The repetition factor supported for A-IoT in Rel-19 is {1,2,4}. One codepoint is reserved for later release. |
| LGE | Proposal 16: For the number of D2R block-level repetitions, {1, 2, 4} repetitions are supported. |
| MediaTek | Observation 1: For a worse residual SFO assumption of 1000ppm, it is observed the performance is unacceptable for D2R packet sizes (including TB and CRC-16) of {416, 816, 1016} bits. Observation 2: For a better residual SFO assumption of 100ppm, it is observed the performance is able to achieve 1% BLER for D2R packet sizes (including TB and CRC-16) up to 1016 bits. Observation 3: Depending on the evaluation assumptions, e.g., message size and residual SFO value at the reader side, it is observed a bad D2R performance is possible thus resulting in a poor coverage, which calls for the necessity of a large repetition number in addition to 1 and 2. Observation 4: It would be safe to have one additional bit to enable the scheduling flexibility of a larger repetition number in addition to 1 and 2 considering the necessity. Observation 5: The sustainability duration of device 1 is assumed to be long enough to support a larger repetition number in addition to 1 and 2. Proposal 1: For the number of block-level repetitions, {1, 2, 4} repetitions are supported. - Note: When the number of block-level repetition is 1, it indicates no repetition. |
| Qualcomm | Proposal 8: Max repetition factor is 2. |
| Sharp | Proposal 7: For the number of D2R block-level repetitions, adopt Option 1 (i.e. {1, 2}). |
| NTT Docomo | Proposal 11: For D2R, for repetition factor of block-level repetition, it is not necessary to support multiple repetition factors, i.e., specify single value of repetition factor for D2R block-level repetition. Observation 1: For D2R, 15 m coverage can be achieved with 5-7 kbps data rate. Proposal 12: For D2R, for repetition factor of block-level repetition, consider maximum chip length Tchip, information bit length Tb and corresponding data rate. Proposal 13: For D2R, for repetition factor of block-level repetition, 2 times repetition should be supported. |
| IIT Kanpur | Proposal 2: Block-level repetition factor 4 should not be supported for the D2R link in Rel-19. |

R2D/D2R CRC

#1 CRC attachment

| Source | Proposal |
| --- | --- |
| Huawei | Proposal 8: For attaching the CRC, the parity bits are appended to the end of the input bits, similar to what is done in TS 38.212. |
| QC | Observation 12: For random access, additional processing time due to CRC for msg1 and 3 are not needed. Observation 13: For command, processing time needs to consider CRC compute time. Observation 14: Adding additional zeros at the end of block could convert LTE TBCC to ZTCC and reduce processing time by initializing shift register with zero tail bits. Observation 15: Adding CRC before last K-1 information bits can reduce processing time by initializing SR with information bits (instead of CRC). Proposal 10: Regarding CRC position and processing time, discuss whether/how to improve current TBCC. |

#2 Joint or separate CRC

| Source | Proposal |
| --- | --- |
| Samsung | Observation 4: If L1 R2D control information is defined, it must have a separate CRC so that a device can first read R2D control information to acquire information necessary to read the following R2D data. Proposal 3: If L1 R2D control information is supported, the L1 control information is attached with a separate CRC-6. If L1 R2D control information is not supported, no need for further discussion. |
| CATT | Proposal 17: Joint CRC should be applied to both R2D control information and R2D data. |
| OPPO | Proposal 7: If L1 R2D control information is supported & carried by PRDCH, separate CRC is attached to L1 R2D control information and R2D data respectively is preferred. |
| LGE | Proposal 13: Support L1 control information for PRDCH, i.e. Option 2. Proposal 14: Support separate CRCs for PRDCH, i.e. the first CRC for L1 R2D control information and the second CRC for R2D data which may or may not contain L2 control information. Proposal 15: Consider fixed size CRC for L1 R2D control (6 or 16 depending on the L1 R2D control information size) and variable size CRC for R2D data. - CRC size for R2D data to be determined based on the R2D data size, or to be signaled via L1 R2D control |
| ASUSTek | Proposal 1: For PRDCH generation, L1 R2D control information and R2D data with separate CRC attachment are carried together in single PRDCH. Proposal 2: For CRC attachment, the 24 bits thresholds of number of information bits are applied to L1 R2D control information (if any) and R2D block separately. |
| IIT Kanpur | Proposal 3: If L1 control information is supported, a separate CRC should be used for control information and R2D data. |

Others

| Source | Proposal |
| --- | --- |
| E/// | Observation 1: OOK modulation without Manchester encoding has several drawbacks, such as synchronization issues, threshold sensitivity, and susceptibility to noise. Proposal 1: Support Manchester line code as specified in TR 38.769 for both R2D and D2R with OOK modulation. |
| Nokia | Proposal 3: In D2R control signaling, whether FEC is applied, which FEC code rate (if multiple code rates are supported) and the number of block level repetitions can be jointly encoded. |
| NEC | Proposal 7: Support joint indication of FEC code rate and repetition number with 3 bits using Table below: |
| China Telecom | Proposal 8: Sub-block interleaving is not considered for Ambient IoT Device 1. |
| Honor | Observation 1: For D2R code rate, {1, 1/2, 1/3, 1/6} is supported according to the repetition number and FEC code rate. |
| TCL | Observation 2: It can be observation from RAN 1 #118b that there are different Btx,D2R and transmission power of the D2R transmissions due to XOR operation. Proposal 4: Support option 1 or no D2R line code (e.g., by using a square-wave corresponding to the small frequency-shift) Observation 4: As the impact of SFO of device 1, each repetition block needs to consider the potential time offset (△t) to avoid the impact of SFO, which is up to the device’s capability and each block length. Proposal 7: Consider D2R block-level repetition based on following aspects, - Frequency resource for each repetition block, e.g., if hopping frequency could be considered for each block - How to indicate frequency resource and repetition number (R) Observation 5: Limited by one bit length, the impact of SFO during one or multiple bit may be ignored, which is up to repetition number of bit-level repetition. Observation 6：For the number of bit-level repetition, it is up to coverage and access performance and it will impact on D2R frame structure selection. Proposal 8: Consider D2R bit-level repetition based on following aspects, - Repetition number (R) - How to indicate time/frequency resource and repetition number (R) |
| CATT | Proposal 15: Although SFS is equivalent to Manchester code bit level repetition in bit mapping perspective, it cannot provide redundancy bits to obtain repetition coding gain. Manchester codeword repetition using for SFS purpose should not be supported as part of repetition coding scheme. Proposal 16: Bit level repetition should not be supported for Rel-19 A-IoT. |
| CATT | Observation 1: With Manchester line coding, the R2D information data rate can be described as 1/(2*R2D chip length), and the R2D chip length is determined by the M value of OOK-4 and the OFDM symbol duration with 15k SCS . Proposal 3: From the perspective of robustness of TBCC to SFO, D2R preamble design should ensure that residual timing error does not exceed 1000ppm. Observation 2: With same coding rate, TBCC code has about 6dB coding gain than that of Manchester code at BLER=10%. Observation 3: Compared with CC code only, CC concatenated with Manchester code would introduce unnecessary redundancy, while its performance is poor than that of convolutional code with repetition operation in case of SFO=1000ppm. Proposal 3: Manchester line code should not be supported together with CC coding. Observation 4: The phase discontinuity issue of OOK demodulation by the reader using the BPSK demodulation method after DC removal would severely degrade the OOK demodulation performance if the reader does not know the D2R waveform is OOK or BPSK. Proposal 4: In order not to degrade the OOK demodulation performance, the reader should be aware of the modulation scheme of OOK or BPSK for the D2R transmission. Observation 8：Transmission bandwidth of D2R channel without small frequency shift is 2/Tb , which is half of transmission bandwidth of D2R channel with SFS factor R=1. Proposal 9: To improve frequency spectrum efficiency, D2R channel without small frequency shift should be supported for Msg1/Msg3 transmission with the D2R bit duration being equal to the chip duration. |
| NTT Docomo | Proposal 1: For small frequency shift of D2R transmission, discuss whether FDMA is supported for other D2R transmissions than Msg1 and Msg3. Proposal 14: For D2R, whether to apply block-level repetition should be controlled by reader. Proposal 15: For D2R, block-level repetition should be applicable to any PDRCH transmission regardless of message type. - Whether to apply block-level repetition can be indicated in any R2D which schedules D2R. |