3GPP TSG RAN WG1 #121			R1- 2504776

St.Julians, Malta, May 19th – 23rd, 2025

Agenda Item:	9.1.3

Source: 	Moderator (LG Electronics)

Title: 	Summary #3 of CSI prediction

Document for:	Discussion and decision

Introduction

This is the summary document for agenda item 9.1.3 CSI prediction.

Contact information

Please provide your contact information or any updates.

| Company | Name | Email |
| --- | --- | --- |
| Moderator (LG Electronics) | Haewook Park | haewook.park@lge.com |
| New H3C | Lei Zhou | Zhou.leih@h3c.com |
| Samsung | Ameha Tsegaye Abebe | amehat.abebe@samsung.com |
| Indian Institute of Technology Madras (IIT Madras) | Sai Prasad Pirati | venkatasiva@5gtbiitm.in |
| SK telecom | Yunesung Kim | yunesung.kim@sktelecom.com |
| MediaTek | Pedram Kheirkhah Sangdeh Yuching Huang | Pedram.kheirkhah@mediatek.com yuching.huang@mediatek.com |
| Panasonic | Tetsuya Yamamoto Xuan Tuong Tran (Henry) | yamamoto.tetsuya001@jp.panasonic.com xuantuong.tran@sg.panasonic.com |
| vivo | Hao Wu | hao.wu@vivo.com |
| Huawei, HiSilicon | Keyvan Zarifi, Yuan Li | Keyvan.Zarifi@huawei.com liyuan3@huawei.com |
| NEC | Peng Guan, Yi Jiang | guan_peng@nec.cn y-jiang_ct@nec.com |
| Fujitsu | WANG Guotong (David) | wangguotong@fujitsu.com |
| Qualcomm | Jay Kumar Sundararajan | jsundara@qti.qualcomm.com |
| OPPO | Wendong Liu | liuwendong1@oppo.com |
| NTT DOCOMO | Xin Wang Haruhi Echigo | wangx@DOCOMOlabs-beijing.com.cn haruhi.echigo.fw@nttDOCOMO.com |
| Spreadtrum | Shijia Shao Hualei Wang | Shijia.shao@unisoc.com Hualei.wang@unisoc.com |
| Ericsson | Jingya Li Siva Muruganathan Jianwei Zhang | Jingya.li@ericsson.com Siva.muruganathan@ericsson.com Jianwei.zhang@ericsson.com |
| ZTE | Xingguang WEI Yunqi Sun | wei.xingguang@zte.com.cn sun.yunqi@zte.com.cn |
| IIT Kanpur | Shyam Vijay Gadhai Abhishek Kumar Singh | svgadhai@iitk.ac.in abhishekks@iitk.ac.in |
| AT&T | Isfar Tariq | Isfar.tariq@att.com |
| Xiaomi | Liuzhengxuan Liumin | liuzhengxuan@xiaomi.com liumin10@xiaomi.com |
| Intel | Victor Sergeev | Victor.sergeev@intel.com |
| CMCC | Yuhua Cao Yi Zheng Dan Song | caoyuhua@chinamobile.com zhengyi@chinamobile.com songdan@chinamobile.com |
| Lenovo, MotM | Ahmed Hindy Vahid Pourahmadi | ahmedhindy@motorola.com vpourahmadi@lenovo.com |
| CEWiT | Shiv Shankar | shivshankar@cewit.org.in |
| NVIDIA | Xingqin Lin | xingqinl@nvidia.com |
| CATT | Qianrui Li | liqianrui@catt.cn |
| TCL | Didi Zhang | didi.zhang@tcl.com |
| LGE | Junhyuk Yoo | Junhyuk.yoo@lge.com |
| Sony | Sam Atungsiri | Sam.atungsiri@sony.com |
| InterDigital | Moon-il Lee | moonil.lee@interdigital.com |
| ETRI | Youngjoon Yoon Anseok Lee | youngjoon.yoon@etri.re.kr alee@etri.re.kr |
| Mavenir | Fan Yang Yuanlong Yang | fan.yang@mavenir.com yuanlong.yang@mavenir.com |
| Apple | Huaning Niu | huaning_niu@apple.com |
| Tejas Networks | Pavan Kalyan D | pavankalyand@tejasnetworks.com |
| Sharp | Makoto Kitahara Liqing Liu | kitahara.makoto@sharp.co.jp liu.liqing@sharp.co.jp |

Work item description

In RAN#105, it is concluded that CSI prediction moves to normative phase starting from Q1 of 2025. Following objectives are captured in updated WID [1] as below

- CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): - Functionality-based LCM leveraged from other use cases, when necessary and applicable, - Study, and if necessary, specify consistency of training/inference, - Note: Normative work in RAN1 for CSI prediction will start from Q1 of 2025

In this meeting, we discuss on the specification impact regarding functionality-based LCM.

Specification support for functionality-based LCM

General functionality-based LCM

- Summary

In this meeting, proposals/observations related to functionality-based LCM are copied below:

| Ericsson | Observation 1 For UE-sided CSI prediction use case, signaling and procedures for functionality-based LCM can largely reuse the framework being discussed and specified for the UE-sided beam management use cases, when applicable. |
| --- | --- |
| Huawei/HiSi | Proposal 18: For functionality alignment, the general signalling procedure agreed by RAN2 (Step 1~Step 5) and the Option A and Option B agreed by RAN1 (except for the associated ID part) for the beam management case can be reused for CSI prediction. Proposal 19: For functionality alignment, regarding the set of inference related parameters provided by NW in Step 3 subject to Option B, consider CSI prediction specific parameters, including: • Measurement related information, e.g., CSI-ResourceConfig. • Report related information, e.g., CodebookConfig-r18, reportConfigType. • Note: Strive to reuse the CSI prediction specific IEs subject to CSI-ReportConfig or referred by CSI-ReportConfig. |
| CATT | Proposal 10: For CSI prediction using UE-sided model, the procedure of applicable functionality reporting for AI/ML-based beam management excluding associated ID can be reused. |
| NEC | Proposal 22: For model switching, model selection and model update, if the input or/and output (e.g., observation window length, the number of measurement time instance, prediction window length, the number of future time instances) of the new AI/ML model change, UE is necessary to report the change(s) to NW. |
| Panasonic | Proposal 5: Functionality-based LCM from time-domain beam prediction (BM-Case 2) with UE-sided model can be leveraged for CSI prediction with UE-sided model. Proposal 6: At least observation window, prediction window, and processing unit related information should be included for inference related parameters |
| NVIDIA | Proposal 1: The inference of one-sided AI/ML model for CSI prediction can be performed at either gNB or UE. Besides studying CSI prediction at UE side, companies are encouraged to study CSI prediction at gNB side to understand the potential gains of performing CSI prediction at gNB side vs. UE side. |
| Lenovo | Proposal 1 The higher-layer parameter introduced to differentiate the Rel-19 AI/ML-based CSI prediction from the legacy Rel-18 CSI prediction is not part of the codebook configuration. |
| Fujitsu | Proposal 1: • Regarding LCM for AI/ML based CSI prediction with UE side model, the LCM design for BM Case-2 with UE side model should be reused as much as possible. Proposal 2: • For AI/ML based CSI prediction with UE side model, the TDCP reporting could be used to facilitate the determination of applicable functionalities. |
| NTT Docomo | Observation 1 - The AI/ML-based BM session is working on the procedures for the UE capability report, model applicability report, and corresponding configurations for the UE. - The same procedures can be reused since both BM and CSI prediction are under the CSI framework. - There is no benefit from introducing differential signaling designs to the CSM framework for BM and CSI predictions. Proposal 1 - Reuse the same LCM framework for the applicability report, CSI report configurations, and report activation specified for the AI/ML-based BM.  Do not introduce different signaling designs to the CSI framework for BM and CSI predictions, i.e., the associated ID is also supported for CSI prediction use cases naturally. |

- (on hold) Issue 2.1-1. Applicable functionality reporting

Signalling procedure (see the attached figure) on applicable functionality reporting for beam management UE-sided model in R2-2407848: - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: - 1) UE is allowed to do UAI reporting via OtherConfig. - 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. - 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - (Between “Step 3” and “Step 4”) UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. - “Step 4”: UE reports applicable functionality in the following scenarios: - 1) Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI - 2) As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration). - Step 5: - 1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5). - 2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not. Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3

Form the discussion in the last meeting and proposals submitted this meeting, many companies think LCM design for BM Case-2 with UE side model can be reused for CSI prediction. In this context, applicable functionality report agreed in RAN2 and further agreements regarding direction A) and B) in BM agenda can be a starting point of discussion. In this regard, I propose to reuse existing framework, however, Nokia argue that this issue should be handled by RAN2 first. So, let’s wait for the progress in RAN2.

Please provide your input/suggestion on Issue 2-1-1.

| Company | Views |
| --- | --- |

Data collection related aspects

- Summary

In this meeting, proposals/observations related to data collection are copied below:

| Ericsson | Observation 2 For UE-sided CSI prediction use case, signaling and procedures for data collection can largely reuse the framework being discussed and specified for the UE-sided beam management use cases, when applicable Observation 3 For UE-sided CSI prediction use case, for data collection, there is no need of specifying assistance information for categorizing the data. Observation 4 Without separate configuration of CSI-RS resources for training data collection for model input and ground-truth CSI, it is not always possible to train a model that corresponds to a supported CSI report configuration for inference, e.g., model inference using aperiodic CSI-RSs for CMR. Proposal 1 For UE-sided CSI prediction use case, at least for training data collection, support separate configurations of CSI-RS resource(s) for measurements for model input and CSI-RS resource(s) for measurements for ground-truth CSI Proposal 2 For UE-sided CSI prediction use case, at least for training data collection, support indication of the association between CSI-RS resources used for measurements in an observation window to create a model input and CSI-RS resource(s) used for measurements in a prediction window to create a ground-truth label. Proposal 3 For UE-sided CSI prediction use case, at least for training data collection, support configuration of a pair of CSI-RS resource sets, where a. The first CSI-RS resource set consists of K CSI-RS resources, with K>1, where the measurements on the K CSI-RS resources are used for creating a model input b. The second CSI-RS resource set consists of N4 CSI-RS resources, with N4>=1, where the measurements on the N4 CSI-RS resources are used for creating a ground-truth label associated to the model input c. The pair of CSI-RS resource sets are configured in a same CSI report configuration, but with different resource configuration IDs. d. Reuse the candidate values for K and N4 as specified for Rel-18 “typeII-Doppler-r18” codebook. Observation 5 For training a model that is used for model inference with a burst of AP CSI-RS measurements as input, enhancement is needed to reduce CSI-RS signaling overhead. Proposal 4 For CSI prediction using UE-side model, for data collection for training, For CSI-RS resource type for CMR, support periodic/semi-persistent CSI-RS burst configuration a. The first periodic/semi-persistent CSI-RS burst is for measurements for model input b. The second periodic/semi-persistent CSI-RS burst is for measurements for ground truth CSI |
| --- | --- |
| Huawei/HiSi | Proposal 1: For training data collection of UE-side model, one CSI-ResourceConfig can be configured for UE measurement to obtain both model input and label. • Note: label corresponding to the time slots without CSI-RS of CSI-ResourceConfig can be obtained by UE side implementation with extrapolation or interpolation. Proposal 2: For training data collection of UE-side model, the ReportQuantity can be set to ‘none’ to indicate without CSI report in CSI-ReportConfig. Proposal 3: For training data collection, gNB indicates the preferred inference related information as guidance to facilitate UE side training, e.g., • Prediction window related information, including the candidate time gap between the earliest of N4 predicted instances and the reference time (e.g., CSI-RS occasion), the time unit of predicted instance (i.e., d), and number of predicted time instances (i.e., N4). • Observation window related information, including the number of A-CSI-RS for channel measurement (i.e., K) at the inference phase. |
| Vivo | Proposal 3: If data collection is configured using CSI framework, it does not have any report to the NW, and it does not occupy CPU. Proposal 4: For CSI prediction, when a UE requests training data collection, it needs to report the UE's preferred CSI-RS configuration and parameters related to prediction window |
| TCL | Proposal 1 : For training data collection, a new RRC parameter is introduced to distinguish between CSI reporting configurations for data collection and inference Proposal 2 : For CSI prediction, when a UE requests data collection for training, the reuse of the SR mechanism should be supported, such as using a specific SR. |
| Spreadtrum/UNISOC | Proposal 1: For data collection for training, for resource configuration, whether to separately configure CSI-RS resource(s) for model input and for ground-truth CSI depends on whether measurement interval and prediction interval are required to be the same. |
| Samsung | Observation#3: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection purpose, The UE is not expected to report CSI for data collection purpose, i.e., the higher layer parameter for reportQuantity can be set to ‘none’. Moreover, such configuration does not require parameter for codebookType. RAN1 need to address how to manage the processing unit and their occupancy for CSI report configuration for data collection purpose. RAN1 need to address how to determine the active CSI-RS resources and the active CSI-RS ports when the resources are referred by a report configuration for data collection. Whether the same scaling factor Kp∈{1,2,4} is applied as in the case wherein the CSI-ReportConfig configured with the higher layer parameter codebookType set to 'typeII-Doppler-r18'. Observation#4: For the AI/ML based CSI prediction using UE-side model, the CSI report configuration for training data collection purpose needs to be differentiated from other report configurations with reportQuantity set to ‘none’. Proposal#2: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection purpose, The UE is not expected to report CSI for data collection purpose, i.e., the higher layer parameter reportQuantity is set to ‘none’, Codebook (‘codebookType’) is not configured. To differentiate the report configuration from other report configurations with reportQuantity set to ‘none’, consider explicit indication of resources (CMRs) for data collection purpose, e.g., higher layer parameter resourcesForDataCollection ‘CSI-ResourceConfigId’. Proposal#3: For the AI/ML based CSI prediction using UE-side model, for CSI report configuration for training data collection O_CPU=Ks processing units are counted toward the legacy CSI processing unit (CPU), where Ks is the number of CSI-RS resource(s) in the CSI-RS resource set for channel measurement. For each of P/SP CSI-RS transmission occasions, the CPU is occupied from the first symbol of the earliest CSI-RS resource until 〖Z'〗_3 symbols after the last symbol of the latest CSI-RS resource for data collection. FFS: whether new value for 〖Z'〗_3. For the active CSI-RS resources and active CSI-RS ports counting, a scaling factor is not applied, i.e., K_p=1. |
| ZTE | Observation 6: For UE side training data collection, separate configuration of CSI-RS would cause twice as much CSI resource configuration overhead. Proposal 9: For UE side training data collection, support single configuration for model input and ground truth CSI. Observation 7: For UE side training data collection, if to support separate configuration for model input and ground truth CSI, consider at least clarifying the binding method and the necessary parameters needed to be indicated to the gNB for resource allocation. ̵ The binding method can be explicitly indicated by indicating a resource ID corresponding to the CSI-RS for inference and a time domain offset d. ̵ The time domain offset d should be exactly divided by the periodicity of the CSI-RS configured for inference data collection. |
| Quectel | Proposal 6: When considering AI/ML-based data collection scenarios, using ReportQuantity = "none" may serve as a valid configuration to indicate that no CSI feedback is required, since the primary objective shifts to raw channel data acquisition for model training rather than traditional link adaptation Proposal 7: The system requires explicit RRC layer reconfiguration to reactivate CSI reporting including PMI/RI/CQI after completing the AI/ML training phase. |
| CATT | Proposal 17: For CSI prediction using UE-sided model, for data collection for training, an indication can be configured in the CSI report with report quantity set to “none”. Proposal 18: For CSI prediction using UE-sided model, for data collection for training, support the following options for RS configuration:  Option 1: Configure a single resource in the CSI report configuration, in case the intervals between instances within the measurement window are identical to those in the prediction window;  Option 2: Configure two separate resources in the CSI report configuration, in case the intervals between instances within the measurement window are different to those in the prediction window. Proposal 19：For CSI prediction using UE-sided model, for data collection for training, UE can request the specific RS configuration for data collection. |
| CMCC | Proposal 13: For CSI prediction using UE-side model, for training data collection, the RRC configuration reportQuantity of CSI-ReportConfig is set to ‘none’. |
| Xiaomi | Proposal 7: Support to reuse existing parameter, i.e., reportQuantity set to ‘none’ for indication of without CSI report for training data collection, or leave it to RAN2 design. Proposal 8: Support to configure one CSI-RS resource for data collection for model training. Otherwise, multiple CSI-RS resources could be configured to obtain model input and ground truth CSI. |
| NEC | Proposal 1: For data collection for training, support to configure single P/SP CSI-RS resource(s) used for measurements for model input and measurements for ground-truth CSI. Proposal 2: For CSI report configured for data collection for training, reportQuantity of CSI-ReportConfig can be configured as ‘none’. Proposal 3: Support enhanced CSI-RS configurations based on the CSI variation point. Proposal 4: Support multiple CSI configuration timing sets in a CSI measurement configuration. Proposal 5: Support multiple CSI periodicities in a CSI report configuration. |
| LG Electronics | Proposal #1: Conclude that separate configuration of CSI-RS resources for model input and ground truth CSI is not supported. Proposal #2: For configuring the resource for training data collection, CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report. - Introduce new reportConfigType-r19 with “none” |
| Panasonic | Proposal 7: The necessity of UE preference reporting on the appropriate CSI measurement configuration to the network other than UE’s capability / functionality report should be discussed. |
| Google | Proposal 6: Do not support to configure separate CSI-RS for model input and model output for data collection Proposal 7: Support the UE to report the UE capability for the preferred number of consecutive transmission occasions and intervals for the CSI-RS resources for data collection Proposal 8: Support to configure the data collection window • CSI-RS for data collection is transmitted in the data collection window • CSI-RS for data collection is not transmitted outside the data collection window Proposal 9: Support a CSI prediction taking 1 processing unit for AI/ML PU (e.g., APU) and 1 CPU Proposal 10: For multiple AI/ML functionalities, support the UE to report which of the following options the UE supports • Option 1: Separate models for different AI/ML functionalities o The processing units are not shared for different AI/ML functionalities • Option 2: Common model for one inference for some AI/ML functionalities, e.g., CSI prediction, CSI compression or positioning o The processing units are shared for different AI/ML functionalities o N processing units are required for the inference for N AI/ML functionalities • Option 3: Common model for simultaneous inference for some AI/ML functionalities, e.g., one model can be used to generate the inference results for both CSI prediction and CSI compression, or for both CSI prediction and positioning o The processing units are shared for different AI/ML functionalities o 1 processing unit is required for the inference for N AI/ML functionalities |
| Lenovo | Proposal 2 For data collection under AI/ML-based CSI prediction, the UE is configured with a CSI report configuration corresponding to channel measurement via CMR(s) over a time window. Proposal 3 For Rel-18 eType-II CB configured with K AP CSI-RS resources spaced by m slots, the corresponding CSI report configuration for data collection comprises at least K P/SP CSI-RS resources with the offset between two consecutive CSI-RS transmissions from two CSI-RS resources is m slots. |
| Sony | Observation 4: When P-CSI-RS or SP-CSI-RS are configured with a periodicity that matches the time between desired CSI prediction slots, both the CSI-RS resource(s) used for measurements for model input and for measurements for ground-truth can be provided by one configuration. Proposal 3: RAN1 to conclude that at least for data collection for training, a single CSI configuration of periodic or semi-persistent CSI-RS of adequate periodicity can be sufficient to provide the CSI-RS measurements resources for both model input and ground-truth. |
| Fujitsu | Proposal 9: • Regarding training data collection for AI/ML based CSI prediction with UE side model, new report quantity could be introduced to indicate “without CSI report” in CSI-ReportConfig. Proposal 10: • Regarding training data collection for AI/ML based CSI prediction with UE side model, it should be discussed whether periodic CSI-RS and semi-persistent CSI-RS should be enhanced to support inference with aperiodic CSI-RS resource. Proposal 11: • Regarding training data collection for AI/ML based CSI prediction with UE side model, the CSI-RS periodicity should be collected and included in the dataset. |
| HONOR | Proposal 1: Reuse the existing parameter (i.e., reportQuantity set to 'none') to enable UE-side data collection. |
| Nokia | Proposal 10: Consider the option to configure for high speed UEs two or more SP CSI-RS and/or P CSI-RS resources with a relative delay of, e.g. 2.5 ms to fulfill the Nyquist criterion for high speed UEs. Observation 6: For data collection it might be beneficial if the UE estimates and reports the best fitting CSI RS configuration of one, two or even more parallel SP CSI-RSs and/or P CSI-RSs. The UE might use TRS to analyze the current time domain channel variance as basis for the proposed CSI RS configuration. Proposal 11: For data collection configure the CSI-ReportConfig with the higher layer parameter reportQuantity set to 'none-CSI-r19', with the effect that then the UE shall not report any quantity for the CSI-ReportConfig. Proposal 12: The configuration of P and one or more SP CSI-RS resources should support the data collection over multiple time instances to form tracks of, e.g., 100 successive channel observations. Proposal 13: For the collected data content, RAN1 to consider explicit CSI as one option. Observation 7: Defining data content as explicit CSI and considering uncompressed explicit CSI (in the data collection framework) can minimize specification effort. Observation 8: Compressed data format (e.g., with an enhanced TypeII format) can be considered as well even so this will require a more detailed further analysis. |
| ETRI | Proposal 11: To indicate no CSI report in ‘CSI-ReportConfig’, reuse the existing parameter by setting the ‘reportQuantity’ field to ‘none’ Proposal 12: Support the single configuration of model input and ground truth CSI. |
| OPPO | Proposal 1: Support configure the reportQuantity as ‘none’ to indicate without CSI report in CSI-ReportConfig. Observation 1: No additional downlink signaling is required when one CSI-RS resource is configured for both of model input and ground-truth CSI measurements Observation 2: Additional downlink signaling is required to associate two CSI-RS resources which are separately configured for model input and ground-truth CSI measurements. Proposal 2: Support configure one CSI-RS resource for both of model input and ground-truth CSI measurements. Proposal 3: Reuse current CSI-RS resource configuration, no enhancement is supported in Rel-19 WI phase. |
| MediaTek | Proposal 11. Propose using the existing parameter by setting ReportQuantity to 'none' in CSI-ReportConfig for configuring resources for data collection without requiring a CSI report, thus avoiding the need to introduce an additional parameter. Proposal 12. For CSI prediction using UE-sided model, the data collection mechanism can leverage the Rel-19 AI/ML-based beam management cases as much as possible. |
| Apple | Proposal 6: For AI based CSI prediction, for data collection for training, support CSI-RS resource set burst with long periodicity to enable efficient data collection. Separate CSI-RS resource set for measurement window and prediction window can be configured. Proposal 7: For AI based CSI prediction, for data collection for performance monitoring, a separate CSI-RS resource set is configured for CMR in the prediction window, to facilitate ground truth measurement. • CSI-RS resource set for prediction window is explicitly linked to CSI-RS resource set for the measurement used for inference. Proposal 8: For CSI-RS prediction, when a UE requests data collection procedure for training, the UE can recommend a CSI-RS related configuration including: time domain information, frequency domain information, density, and association ID if specified. |
| Qualcomm | Observation 1: Setting reportQuantity to be “none” is normally used for Rx beam sweeping or TRS. UE behaviours need to be clarified if use reportQuantity to be “none” for UE side data collection. Proposal 1: For UE side data collection, consider either setting reportQuantity to be “none” or setting reportQuantity to be a new value. If reportQuantity set to “none” is used, it is needed to distinguish UE behaviour of data collection from Rx beam sweeping or TRS. Observation 2: In the inference configuration, the gap between last symbol of measurement occasion to the first symbol of the prediction instance is not the same as the measurement resource periodicity. Proposal 2: For UE side data collection, configure two separate resources or resource sets for model input measurement and ground-truth measurement separately. |
| CEWiT | Proposal-3: For CSI prediction using UE-side model, for training data collection, support, reportQuantity set to ‘none’. |

In the previous meeting, following regarding data collection was agreed.

Agreement For CSI prediction using UE-side model, for data collection for training, - For resource configuration, - For CSI-RS resource type for CMR, periodic and semi-persistent are supported. - FFS on support of aperiodic CSI-RS resource - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead. - FFS on CSI report configuration Conclusion For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported. Agreement For CSI prediction using UE-side model, for training data collection, support - CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report - FFS on how to indicate without CSI report in CSI-ReportConfig

As shown above, there are two open issues to be resolved.

- Issue 2.2-1. Whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

For this issue, two different views are observed. The proponent of single configuration think separate configuration for model input and GT CSI is not needed since the prediction window can overlap with the next observation window. If there are some mismatch due to some delay, it could be resolved by UE-side implementation. While proponents of separate configuration believe that observation window and prediction window may not be well-matched due to delay, e.g., scheduling and/or processing delay. Also, in case of separate configuration, the linkage is needed between two configurations. Followings are summarized companies’ position.

- Single configuration (11): OPPO, ETRI, NEC, Huawei/HiSi, OPPO, Sony, MediaTek, ZTE, Xiaomi, Google, LG Electronics

- Separate configurations (5): Ericsson, CMCC, NTT Docomo, Apple, QC

It seems majority companies think a single configuration seems sufficient. Therefore, I propose following conclusions.

- (updated) Conclusion 2.2-1.

For CSI prediction using UE-side model, for data collection for training, a single configuration of CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI is supported.

For CSI prediction using UE-side model, for data collection for training, separately configuring CSI-RS resource used for measurements for model input and CSI-RS resource used for measurements for ground-truth CSI is not supported.

Please provide your input/suggestion on Conclusion 2-2-1.

| Company | Views |
| --- | --- |
| vivo | OK |
| Samsung | Ok |
| Huawei, HiSilicon | Support. Further, only one CSI-ResourceConfig can be configured for UE measurement to obtain both model input and label. If labels are required for the time slots without CSI-RS, they can be obtained by UE implementation using, eg, interpolation or extrapolation of the measured CSIs on the CSI-RS occasions in the neighbouring slots. |
| OPPO | Support |
| ETRI | Support |
| Fujitsu | Ok with the conclusion. |
| NEC | Support |
| CMCC | Support. We want to modify our position to single configuration. We think the potential mismatch from single configuration can be handled by UE-side implementation during training. |
| Xiaomi | Support |
| NTT DOCOMO | This issue is highly correlated to Issue 2.2-3. If P/SP CSI-RS enhancements are considered, the separated resources are necessary. We suggest discussing Issue 2.2-3 first. |
| Sharp | Support |
| Sony | Support |
| HONOR | Support. |
| Apple |  |
| ZTE | Support single configuration. The UE could have different implementations for using the CSI-RS resources to increase the model training precision by applying a sliding observation window. For example, if the gNB configures a CSI-RS with N resources for M CSI input and K CSI output model training, if N>M+K, then the UE has the flexibility to decide how to generate a prediction model using the N resources with respect to the reporting config indicated by the gNB. |
| Ericsson | Not support. It has been agreed in RAN1#120 that AP CSI-RS is supported for inference. For AP CSI-RS based model inference, the value of m and d can be different according to the 'typeII-Doppler-r18’ codebook configuration (e.g., m=2, d=1). In addition, the distance between the first prediction time instance in the prediction window and the last CSI-RS measurement in the measurement window does not necessarily to be an integer of m. Hence, it is not always possible to have a single configuration of CSI-RS resource for collecting both the model input and ground-truth for training data collection. The UE implementation based method mentioned by some companies can result in prediction performance degradation due to inaccurate ground truth CSI for model training, and this scheme has not been evaluated in the study phase. |
| CATT | OK |
| TCL | OK |
| Qualcomm | During offline discussion, we think the messages exchanged among companies are not clear, more discussion is needed. The idea of “single resource(s)” configuration should be clarified, is there just a single resource or multiple, or their periodicity / offset can be different? |
| CEWiT | OK |
| SPRD | Agree with QC. “single resource(s)” configuration is not clear. |
| Mod | I revise the proposal as above, please check this can be acceptable. Your wording suggestion is always welcomed. |

- (updated) Conclusion 2.2-1.

For CSI prediction using UE-side model, for data collection for training,

- at least two CSI-RS resources can be configured in a single resource set in CSI report configuration.

- OCPU=1

- the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

Please provide your input/suggestion on Conclusion 2-2-1.

| Company | Views |
| --- | --- |

- (closed) Issue 2.2-2. Report configuration for training data collection

From the submitted proposals in this meeting, following candidates are proposed by companies.

- Option 1-1: reportQuantity set to ‘none’

- Option 1-2: reportQuantity set to ‘none-r19’

- Option 2: reportConfigType-r19 with “none”

Option 1-1 seems the most popular option among the companies. However, as Qualcomm and Samsung pointed out, it is needed to distinguish UE behaviour of data collection from Rx beam sweeping or TRS. I think option 1-2 and 2 can address this issue by adding suffix (r19). Among two options, Option 1-2 seems majority, so I propose to go with option 1-2 as below.

- Proposal 2.2-2.

For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-r19’.

Please provide your input/suggestion on Proposal 2-2-2.

| Company | Views |
| --- | --- |
| Qualcomm | Option 1-2 sounds a cleaner way as there are various description of “none” in 214 spec, if adopt option 1-1, we need to distinguish data collection with those descriptions. |
| vivo | OK |
| Samsung | Ok |
| Huawei, HiSilicon | OK. |
| OPPO | OK |
| ETRI | OK |
| Fujitsu | We need to check with BM session. There is proposal in BM which also sets reportQuantity to none-r19 for data collection. The report quantity for CSI prediction data collection should be different with BM data collection. |
| NEC | OK |
| Xiaomi | OK. |
| NTT DOCOMO | Support |
| Sharp | OK |
| Sony | OK |
| HONOR | Support. |
| ZTE | Support. |
| Ericsson | Support |
| CATT | OK |
| TCL | OK |
| CEWiT | OK |

- (closed) Issue 2.2-3. Enhancement of P/SP CSI-RS configurations

Regarding following FFS bullet, several companies are provided some proposals.

- FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

For example, Ericsson and Apple propose burst-like P/SP CSI-RS, and Nokia and Lenovo proposed to configure two or more P/SP CSI-RS with gap, e.g., 2.5ms to support high speed UE. This issue was discussed in the last meeting, but some companies think that this it is out of scope. If you have different view, please provide your view in the below.

Please provide your input/suggestion on Issue 2-2-3.

| Company | Views |
| --- | --- |
| OPPO | Not necessary to discuss this issue. |
| CMCC | Support. |
| Huawei, HiSilicon | We are open to introduce smaller periodicity of 1 or 2 slots for P/SP-CSI-RS, so that the A-CSI-RS with m=1/2 slots for inference can be enabled. |
| NTT DOCOMO | We believe this feature is beneficial, especially for AI/ML-based CSI prediction. Therefore, this issue can be treated in this session. |
| Lenovo/ MotM | For CSI prediction there are two different CSI-RS configurations needed: - CSI-RS configuration for inference/monitoring: This can be optimized yet legacy can be used. Not a priority CSI-RS configuration for data collection at UE: This requires multiple CSI-RSs triggered with no CSI quantity reported. As of the current spec the only way to configure this is via a large number of AP CSI-RS resources, leading to unnecessarily complicating the triggering process. This needs to be discussed. |
| HONOR | Out of scope. |
| Apple | Support. For m=1/2. In addition, for resource efficiency and data collection efficiency, bursty configuration can be used. |
| ZTE | We are supportive of the idea that some enhancement for reducing the signalling overhead should be included. We think the problem is strongly coupled with UE side model implementation, including model structures and hyper parameters. It is difficult for the spec to capture all the personalised UE implementation by supporting different types of CSI-RS configurations. In such cases, we think it would be better to setup a general signalling procedure to support the UE model implementation flexibility, such as indicating a total number of CSI-RS resources for UE. Therefore, different UEs could share the same CSI-RS resource configuration and the CSI-RS signalling overhead could be reduced. |
| Ericsson | Support. The issue is necessary to be discussed and addressed to reduce the signaling overhead for training data collection and to ensure the quality of the collected training data (accurate ground truth CSI). |
| CATT | It is enhancement, not necessary to discuss. |
| TCL | We are open to discussing this issue. |
| SPRD | We think it is not necessary to discuss. |

Inference related aspects

- Summary

In this meeting, proposals/observations related to inference reporting are copied below:

| TCL | Proposal 3 :For CSI prediction using UE-side model, dynamically update the number of predicted CSI should be supported. |
| --- | --- |
| Xiaomi | Proposal 1: For model inference, CSI-RS resource configuration for Rel-18 eType II Doppler codebook could be reused. |
| NEC | Proposal 6: For P/SP CSI report for inference, support to configure observation window and prediction window to avoid unnecessary signaling overhead. Proposal 7: For CSI prediction, the CSI reporting periodicity may be updated autonomously upon reaching a significant point of variation (determined by time, location or distance). Proposal 8: If there is a mismatch between the CSI-RS configuration parameters and the model input requirements then support to UE can either fall back to legacy mode or request the gNB to reconfigure the parameters. |
| LG Electronics | Proposal #5: For CQI calculation in inference report, reuse the mechanism of Rel-18 tdCQI. |
| Nokia | Proposal 1: Define a new RRC parameter for the CSI report configuration like vectorLengthDD_ML, which configures the value of N_4, to indicate AI/ML based UE-sided channel prediction. |
| Sharp | Proposal 1: For Rel-19 AI/ML CSI prediction, the parameters δ,d,N for CSI prediction window are configured by gNB via RRC signaling from candidate values reported by UE depending on UE capability where slot (n+δ) is location of the first slot of first time interval, d is the time duration of time interval, N is the number of time intervals for CSI prediction. |

In the RAN1#120, following was agreed.

Agreement For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused. - For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported. - For inference report, - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Also, in the last meeting, following is agreed to distinguish AI-CSI prediction and non-AI-CSI prediction.

Agreement For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction. - Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion - Detailed parameter name is upto RAN2

One issue can be regarding CQI calculation. In the last meeting, during UE feature discussion, there was discussion whether to include UE capability regarding ‘tdCQI’ or not. In this context, LG proposed to reuse Rel-18 tdCQI for AI based CSI prediction. From the moderator’s view, reusing Rel-18 CSI framework can include reusing tdCQI as well, so I think there is no need to discuss this issue.

Please provide your input/suggestion on inference operation, if any.

| Company | Views |
| --- | --- |
| Lenovo/ MotM | Not a priority. Rel-18 already supports reporting two CQI values corresponding to two periods within the prediction window |

CSI processing criteria and timeline

- Summary

In this meeting, proposals/observations related to CSI processing criteria and timeline are copied below:

| Futurewei | Observation 1: Supporting the sharing of AI/ML-based computing resources among different AI/ML features and use cases is a more efficient, flexible, and scalable approach. It optimizes resource utilization, potentially reduces power consumption, simplifies UE capability reporting, and aligns with industry trends in AI hardware design. Proposal 1: Support the sharing of AI/ML-based computing resources (i.e., AI/ML Processing Units) among different AI/ML features and use cases, including AI/ML-based beam management, CSI compression/prediction, and positioning, for R19. Proposal 2: Define a new timeline for APU calculation and reporting. Proposal 3: UE considers model (un)loading, (de)activation, switching, and inference time when calculating the time needed for AI/ML-based processing, but only reports the total time needed as UE capability for APU-based timeline reporting. Observation 2: The time required for model inference, (de)activation, and (un)loading can vary significantly over time due to many reasons. Therefore, the time it takes for a UE to calculate and report the number of APUs may vary by the different types of UEs, and it could vary from time to time even for the same UE. The new timeline framework should allow the UE to report updates of its APU timeline, enabling the network to configure/control CSI reporting parameters more accurately. Proposal 4: For CSI prediction using UE-sided model, regarding whether OAPU and OCPU are based on defined rules and/or reported by UE for inference: • For Option 1 and Option 3, OAPU is reported by the UE, while OCPU can be based on defined rules. • For Option 2, OCPU is reported by the UE. |
| --- | --- |
| Ericsson | Proposal 7 For CSI prediction using UE-side model, for the aspect of CSI processing criteria for model inference, down-select to support a single option, preferably option 1, where the number of occupied APU, OAPU, and the total number of APUs are reported as part of the UE capability. Observation 7 For AI/ML based CSI reporting features, the inference latency for generating CSI report may be shortened compared to legacy CSI reporting due to dedicated hardware and better parallelization for CSI processing, while model loading or model switching at UE, when needed, may introduce additional delay for CSI reporting. Proposal 8. For defining the CSI processing timeline of an AI-based CSI reporting feature, support the following A shortened processing timeline for the CSI report (except for the initial CSI reporting occasion) of an AI-based CSI reporting feature, comparing to the corresponding legacy non-AI based CSI report feature. An additional delay, δ_T, can be introduced in the CSI processing timeline for the initial CSI reporting occasion of an AI-based CSI reporting feature to account for latency introduced by model switching. Observation 8 The processing timeline and AI-CPU counting for CSI performance monitoring (CSI-PM) report depends on how a performance monitoring report is configured and triggered. Proposal 9 Reuse the legacy framework regarding active CSI-RS resource and port counting for AI CSI prediction use case. |
| Huawei/HiSi | Proposal 8: Regarding the occupancy of AI/ML PU and legacy CPU for AI/ML-based CSI report, for inference, support both Option 2 and Option 3 with UE capability to report the selected option. • Option 2: only legacy CPU is occupied. • Option 3: both dedicated AI/ML PU and legacy CPU are occupied.  Equal occupancy duration can be assumed for AI/ML PU and legacy CPU. Proposal 9: Regarding the AI/ML PU pool over use cases, at least support the total number of dedicated AI/ML PU is shared among CSI report related use cases, e.g., AI/ML-based BM and AI/ML-based CSI prediction. Proposal 10: For PU occupancy Option 3 (both dedicated AI/ML PU and legacy CPU are occupied), the PU occupancy order for two pools can be updated as: among the simultaneous CSI reports ordered from high priority to low priority, • For a specific CSI report subject to Option 3:  if both the unoccupied AI/ML PU and the unoccupied legacy CPU can satisfy the required AI/ML PU and required legacy CPU of the CSI report, respectively, the CSI report is required to update, and both PUs are occupied;  otherwise, if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied. • If the AI/ML PU pool is fully occupied, the CPU pool, if not fully occupied, can still serve the CSI reports that only require the legacy CPU. Proposal 11: Regarding the CPU occupancy rule for LCM procedures: • For inference: reuse the occupancy duration of Rel-18 CSI prediction, i.e.,  from DCI till the scheduled PUSCH for A-CSI inference report.  from the Kp-th latest consecutive P/SP-CSI-RS occasion till the corresponding PUSCH for SP-CSI inference report. • For training data collection, reuse the legacy CPU occupancy rule subject to report quantity of ‘none’, i.e., from the first symbol of each P/SP-CSI-RS occasion, till Z3’ symbols after the P/SP-CSI-RS occasion.  AI/ML PU is not occupied. • For monitoring Type 3, if a dedicated CSI report is configured to the monitoring CSI, the CPU for a monitoring CSI report is occupied:  from DCI till the scheduled PUSCH for A-CSI monitoring report.  from the N4-th latest consecutive P/SP-CSI-RS occasion till the corresponding PUSCH for SP-CSI monitoring report.  AI/ML PU is not occupied. Proposal 12: Regarding the active resource/port counting, legacy rule can be reused, i.e., no need to introduce AI/ML-specific active resource/port capability. • For training data collection and monitoring, the active resource/port is counted multiple times based on N4, considering separate measurements are needed to generate labels for monitoring N4 predicted instances. Observation 1: Activation delay is needed for deploying the model to the AI/ML processing memory before performing inference. • Activation of an AI/ML model for CSI prediction could be achieved in TTI level delay. • To achieve efficient usage of AI/ML processing memory, the model should be flushed from the AI/ML processing memory after inference is completed. Proposal 13: For A-CSI report subject to a functionality, the CSI processing timeline is impacted by the active/inactive state of the functionality. When the A-CSI report is triggered: • If the functionality is subject to “active state”, the CSI processing is subject to a shorter timeline (Zref). • If the functionality is subject to “inactive state”, the CSI processing is subject to a longer timeline (Zref+ΔT) as it additionally includes the activation delay of ΔT. Proposal 14: For determining the active/inactive state of a functionality, consider following cases: • Case 1: An activation window is configured/activated by the NW for the functionality. The functionality is subject to active state in the activation window, and is subject to inactive state otherwise. • Case 2: Active/inactive state is determined by another SP-CSI report subject to the same functionality. The functionality is subject to active state for a certain time duration before the uplink symbol of per SP-CSI report, and is subject to inactive state otherwise. Proposal 15: The association of multiple CSI report configurations and/or multiple sets of inference related parameters corresponding to a same functionality should be reported by UE during the functionality report procedure. Observation 2: UE may need to deploy multiple functionalities in the AI/ML processing memory, each of which may correspond to an individual required memory storage. Proposal 16: Discuss the memory occupancy alignment mechanism to align the availability of memory storage for updating the AI/ML-based CSI report. • The memory is occupied when it requires computation and is flushed when the CSI report is transmitted.  For A-CSI report, the occupation lasts from DCI to the CSI report.  For SP-CSI report, the occupation lasts for a certain time duration before the uplink symbol of per SP-CSI report. • The UE does not need to update an AI/ML-based CSI report if the unoccupied memory cannot support its required memory. Proposal 17: For AI/ML model at the UE-side, differentiate the priority of AI/ML-based CSI reporting types over LCM procedures. - E.g., update the CSI priority formula as PriiCSIy,m,k,c,s=2∙Nm∙Ncells∙Ms∙y+2∙Ncells∙Ms∙m+Ncells∙Ms∙k+Ms∙c+s, where m=0 for monitoring CSI, m=1 for inference CSI or legacy CSI, and m=2 for training data collection CSI |
| Vivo | Proposal 5: For UE-side model, for AI/ML based CSI prediction, for processing of a CSI report for inference, support the following options: Option 1: only dedicated AI/ML PU is occupied, O_APU=N1 is reported by UE capability Option 2: only legacy CPU is occupied, O_CPU=M it is reported by UE capability, where M is a multiple of the number of CPUs occupied by legacy non-AI CSI prediction report with the same configuration. Note: The supported option by UE is reported by UE capability, Proposal 6: Define a maximum total number of simultaneous AI and non-AI CSI reports in UE capability report in addition to the legacy CPUs and AI CPUs. All CSI reports can be reported only if all the conditions are met, including not exceeding legacy CPUs, not exceeding AI CPUs and not exceeding such maximum total number of simultaneous AI and non-AI CSI reports. Proposal 7: For CSI priorities, Pnon-AI>PAI≥Pmonitoring. Proposal 8: For AIML functionality switching/activation, consider the following steps for defining timeline:  Step 1: loading model to AI engine (e.g., APU/CPU/GPU) - Example 1: Loading model from flash storage to AI engine - Example 2: Loading model from Global memory to AI engine - Example 3: model has been loaded in AI engine memory  Step 2: Initialization and scheduling of AI resources in AI engine  Step 3: running the model based on the input data  Step 4: return output results  Step 5: release model/AI resource in AI engine - Proposal9: for AIML functionality switching/activation, consider the following value range for defining timeline in step1/3: Step 1; Example 1; loading model from flash storage to AI engine; Hundreds MB/s; Hundreds ms. Example 2; loading model rom Global memory to AI engine; Hundreds GB/s; Few ms. Example 3; model has been loaded in AI engine memory; /; few us. Step 3; Inference timing; Tflops to Gflops; few us to hundreds of us. Proposal 17: For CSI prediction inference report based on SP CSI-RS or AP CSI-RS, extend the active resource counting time for the inference report to the end of the associated monitoring report to reflect the extra buffering size. |
| TCL | Proposal 4 : For inference, we prefer option 1 and/or option 2. If multiple options are supported, the applied option will depend on the UE’s capability. Proposal 5 : For CSI prediction using a UE-side model, the CSI processing criteria and timeline should be further discussed for different stages of model operation. Proposal 6 : For AI-based CSI prediction, the priority value calculation for inference CSI reporting can reuse the legacy formula Prii,CSI(y,k,c,s). Proposal 7 : For the same functionality, AI-based CSI reporting has a higher priority than legacy CSI reporting regardless of the value of Prii,CSI(y,k,c,s). |
| Spreadtrum/UNISOC | Proposal 3: For processing of a CSI report for inference, support Option 2: only legacy CPU is occupied. |
| Samsung | Observation#3: The UE may execute the configurations for AI/ML related features/functionalities partly or fully on a different hardware unit (processing resource) than legacy CSI. Proposal#10: For CSI prediction using UE side model, for inference, - Support Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - Follow the legacy rule for OCPU as defined for the case when the UE is configured with CSI report configuration with codebookType set to ‘typeII-Doppler-r18’ - The UE reports OAPU from a specified candidate values. |
| ZTE | Observation 4: For CSI prediction using UE side model, for inference, whether the UE supports option 1, option 2 and/or option 3 is up to UE capability. Observation 5: For CSI prediction using UE side model, for inference, option 1 and option 2 have smaller specification impact than option 3. Proposal 7: For CSI prediction using UE side model, for inference, support option 1 and option 2 for UE side AI/ML PU management. Proposal 8: For CSI processing timeline for CSI prediction using UE side model, reuse the legacy CSI processing timeline as a starting point for both option 1 and option 2. |
| CATT | Proposal 1: The dedicated AI/ML PU should be shared among CSI related AI/ML features/functionalities. Proposal 2: For CSI prediction, for inference, all three options for processing unit are supported. UE should report supported option(s) subject to UE capability.  If UE supports multiple options, NW can configure one of the UE supported options to determine the kind of occupied processing units. Proposal 3: For UE side AI/ML-based CSI processing, UE should report the model complexity or UE capability related information to determine the occupied AI/ML PU or legacy CPU. Proposal 4: To determine the occupation duration of AI/ML PU and legacy CPU, reuse the legacy occupation time of non-AI CSI prediction. Proposal 5: For determination of OCPU for performance monitoring of CSI prediction, consider: • the value of N4; • the configuration for calculating performance metric, e.g., how many layers and how many samples are used. Proposal 6: For data collection of AI/ML based CSI prediction, OCPU=0. Proposal 7: For AI/ML based CSI reporting, whether a different timeline is needed when functionality switches/activates is based on UE capability. Proposal 8: For CSI report of AI/ML-based CSI prediction, reuse legacy framework for active CSI-RS resource and port counting.  UE reports the {Max # of Tx ports in one resource, Max # of resources, total # of Tx ports} for AI/ML-based CSI report;  UE reports the scaling factor for active resource counting of AI/ML-based CSI report. Proposal 9: The parameter k in legacy priority rule can be extended according to reportQuantity and other information in CSI-ReportConfig to distinguish the AI CSI report and legacy CSI report, or to distinguish the inference or monitoring report. |
| CMCC | Proposal 8: For CSI prediction using UE side model, for inference, support Option 1 and Option 2:  Option 1: only dedicated AI/ML PU (OAPU) is occupied  Option 2: only legacy CPU (OCPU) is occupied  Note: The supported option by UE is reported by UE capability and UE can only support either of them, not both  The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. Proposal 9: For CSI prediction using UE side model, for inference, the number of occupied OAPU and OCPU in Option 1 and Option 2 are reported by UE capability. Proposal 10: For CSI prediction using UE-side model, support to adjust the timeline for inference, i.e. the value of Z and Z’ can be modified. Proposal 11: For CSI prediction using UE-side model, Not support to introduce a different timeline when functionality switches/activates. Proposal 12: For CSI prediction using UE-side model, the legacy framework for active CSI-RS resource and port counting in Rel-18 CSI prediction can be reused. |
| Xiaomi | Proposal 2: The following three options on CSI processing criteria for AI-based CSI prediction could be supported according to UE capability − Option 1: only dedicated AI/ML PU (OAPU) is occupied − Option 2: only legacy CPU (OCPU) is occupied − Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied Proposal 3: The following three options on number of APU/CPU for AI-based CSI prediction could be supported according to UE capability Option 1: only dedicated AI/ML PU is occupied, O_APU=M1 is reported by UE Option 2: only legacy CPU is occupied, O_CPU=K is reported by UE. Option 3: both dedicated AI/ML PU and legacy CPU are occupied, O_APU=M2 is reported by UE And O_CPU=⌈X_2 N_4 ⌉, X_1∈{0.25,0.5,1,1.5} reported by UE. Proposal 4: For Option 1(only dedicated AI/ML PU is occupied) and Option 2(only legacy CPU is occupied), CPU/APU occupation time for Rel-18 eType II Doppler codebook based CSI reporting could be reused. Proposal 5: For Option 3(both dedicated AI/ML PU and legacy CPU are occupied), both CPU and APU occupation time could be defined as follows: For aperiodic CSI reporting, APU occupation time is from the first symbol after the PDCCH triggering the CSI report until the end of the referent time, while CPU occupation time is from the first symbol after referent time until the last symbol of the scheduled PUSCH carrying the report. For semi-persistent CSI reporting, APU occupation time is from the first time of KP-th latest consecutive periodic/semi-persistent CSI-RS occasions no later than CSI reference resource until the end of the referent time, while CPU occupation time is from the first symbol after referent time until the last symbol of the PUSCH carrying the report, where the value of K_P∈{1,2,4} is indicated by UE capability. Note: The reference time could be FFS. Proposal 6: Legacy active CSI-RS resource counting for Rel-18 eType II Doppler codebook could be reused for AI-based CSI prediction. |
| NEC | Proposal 17: Support common and shared AI/ML processing units among AI/ML related features/functionalities to reflect general UE capability of AI/ML operations, at least for AI/ML use cases in physical layer. Proposal 18: AI/ML based CSI reporting occupies one set of CPUs and the other set of APUs, i.e., support Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied. And UE only updates an AI/ML CSI report if both conditions on CPU and APU are satisfied. Proposal 19: Counting of processing units should be discussed separately for CSI report for inference and CSI report for monitoring. Proposal 20: For CSI prediction, study CPU occupation timing for the observation window. Proposal 21: CSI computation time for the predicted CSI should consider inference delay. |
| RUIJIE NETWORKS | Proposal 2: For CSI prediction using UE side model, for inference, consider the following options for potential down selection  Option 1: only dedicated AI/ML PU (OAPU) is occupied  Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied  FFS whether OAPU and OCPU are based on defined rule and/or reported by UE  Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. • Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately. |
| InterDigital | Observation 6: Option 2 (only legacy CPU (OCPU) is occupied) may lead to inaccurate counting of the AI/ML PU resources. Proposal 5: For AI/ML CSI prediction using UE side model, for inference, Option 2 (only legacy CPU (OCPU) is occupied) should not be considered. Proposal 6: For AI/ML CSI prediction using UE-side model, for inference, down-select Option 3 (both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied.) Proposal 7: Support separate PU occupancy counting between legacy (non-AI/ML) CSI reporting and AI/ML-based CSI. Proposal 8: Support CPU pool sharing between legacy CSI reporting and AI/ML-based CSI. |
| LG Electronics | Proposal #3: Support Option 1, where only dedicated AI/ML PU(OAPU) is occupied for CSI prediction inference operations. Proposal #4: For performance monitoring, OCPU=K where K is the number of monitoring RS occasions. |
| Panasonic | Proposal 1: For processing unit for AI/ML features for UE, all options are supported in the specification. The supported option(s) by UE and the number of occupied PUs are reported by UE capability. Proposal 2: In addition to UE capability, the AI/ML PU (O_APU) and legacy CPU (O_CPU) can be adjusted by functionality applicability reporting over UE Assistance Information of RRC. Proposal 3: Before proceeding a discussion of CSI processing unit, a relationship between model mapping and 3GPP-specified procedure relation should be concluded. Proposal 4: When a UE reports capability of UE-side AI/ML model in UECapabilityInformation message, it means that the UE-side AI/ML model is ready to be used, i.e., it is not required to download from UE side server. |
| Lenovo | Observation 3 Based on UE implementation, the CSI measurement corresponding to a time window up to the latest slot carrying the CSI-RSs may or may be pursued before the input to the AI/ML model, or as part of the AI/ML model for CSI prediction. Proposal 9 For PU calculation for AI/ML-based CSI prediction, support Option 3: Both dedicated APU(s) and legacy CPU(s) can be occupied for AI/ML-based CSI prediction. • Note: the value of either the occupied APU(s) or CPU(s) can be zero. Proposal 10 For the active CSI-RS resource counting (ARC) for AI/ML-based CSI prediction, legacy procedure is supported. |
| Fujitsu | Proposal 12: • For CSI prediction with UE-side model, regarding CSI processing criteria for inference, all the three options could be supported subject to UE capability and RAN1 could further discuss the occupation time for CPU and APU. Proposal 13: • For CSI prediction with UE-side model, regarding CSI processing criteria for performance monitoring and training data collection, it’s not necessary to occupy APU. |
| HONOR | Proposal 11: For UE-side model, for AI/ML based CSI prediction, for processing of a CSI report for inference, support option3: both dedicated AI/ML PU (O_APU) and legacy CPU (O_CPU) are occupied. Proposal 12: For the inference of CSI prediction, regarding the CPU occupancy for Option3, reuse the CPU occupancy rules of the R18 Doppler codebook. Proposal 13: For the inference of CSI prediction, regarding the APU occupancy for Option3, the APU occupancy time is the same as the CPU occupancy time. Proposal 14: The CPU is shared between legacy CSI and AI/ML-based CSI reporting, and AI/ML PU is shared among different AI/ML functionalities/use cases. Proposal 15: Regarding CPU occupancy for training data collection, support O_cpu = 1. Proposal 16: For CPU occupancy time of monitoring report, introduce an additional time or scaling factor on top of legacy codebook (e.g., Rel-16 eType II codebook). Observation 2: Discarding or not updating an inference report will affect performance monitoring. Proposal 17: If the inference report requiring performance monitoring is discarded or not updated, the corresponding monitoring report should also be discarded or not updated. |
| Nokia | Proposal 14: For CSI prediction using UE side model support Option 3) where both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied. Proposal 15: For AI/ML-based channel prediction use case, • OAPU and OCPU may be occupied for the same time duration, where OCPU occupying time durations is already defined in Rel-18. • Rel-18 CSI processing timeline for CSI prediction shall be applicable for Rel-19 AI/ML based channel prediction. |
| ETRI | Proposal 10: For CSI prediction using UE side model, for inference, support Option 3, that both dedicated AI/ML PU and legacy CPU are occupied. |
| OPPO | Proposal 8: Support Option 3, wherein both dedicated APU and legacy CPU are occupied for inference. O_APU and O_CPU are reported by UE Observation 3: Different AI/ML models for the same AI/ML functionality/feature may occupy different APUs Proposal 9: Support Option 2 with only legacy CPU occupation for Type 3 monitoring. |
| MediaTek | Proposal 15. For AI/ML-based CSI prediction inference, support Option 1 only to avoid interference between AI CSI and legacy CSI. Proposal 16. Support separate discussions of PU for different use cases (CSI/BM/POS) to adequately address their distinct requirements. |
| Apple | Proposal 1: For AI/ML processing criteria and timeline discussion with UE side model, to enable flexible mapping of various AI use cases and hardware resources, define maximum simultaneous AI processing per CC and all CC for AI based CSI prediction. Proposal 2: For AI based CSI prediction, both dedicated AI/ML PU and CPU are occupied. AI/ML processing unit is occupied for active CSI-RS resource and port counting, similar to legacy framework. Proposal 3: Define UE capability report on the number of AI processing units used for AI based CSI prediction inference. Proposal 4: Additional processing time should be added for AI based CSI prediction report. Proposal 5: Apply CPU occupancy counting rule for data collection for training without CSI report, for AI based CSI prediction. • Ocpu = 1 • The CPU is occupied from the first symbol of the earliest one of each transmission occasion of periodic or semi-persistent CSI-RS resource for channel measurement, until Z3’ symbols after the last symbol of the latest one of the CSI-RS/SSB resource for channel measurement in each transmission occasion. Z3’ value can be further discussed. |
| Qualcomm | Observation 3: There may be high-end UEs which have independent hardware for AIML than legacy algorithms, and also low-end UEs which may use common hardware for AIML and legacy algorithms. Observation 4: For UEs using independent hardware for AIML operation, the pre-processing and postprocessing may still run in the hardware used for non-AI features. Proposal 3: For processing units counting of AI/ML-based CSI prediction using UE side model, support option 2 and option 3 upto UE capability. Observation 5: CSI processing units depends on the model complexity, which may vary significantly across different use cases such as beam management, CSI prediction and CSI compression. Proposal 4: For processing units counting of AI/ML-based CSI prediction using UE side model, O_APU and O_CPU should be reported by UE. Observation 6: CPU and AI/ML processing units occupation duration should consider the additional buffering time of the measurement results and computation results. Proposal 5: For the duration of the CPU and AIML processing units, support the following: • Reuse the starting time of the legacy CPU for both the CPU and AIML processing unit duration of the AIML-based CSI prediction. • Extend the ending time of both the CPU and AIML processing units to the last symbol of monitoring report. Proposal 6: Reuse the rule and framework for active CSI-RS resource and port counting. For AIML-based CSI prediction, UE report CSI-RS triplets for both AIML processing alone and concurrency processing with non-AIML CSI codebooks. Proposal 7: For the active duration of the aperiodic CSI-RS, extend the ending time to the last symbol of the monitoring report. The timeline for each individual case depends on the specific model choice and hardware architecture. A common offset relative to legacy (Z, Z’) values can be reported by UE as the only difference is AIML-based CSI predication and it applies to both time requirement for the measurement resource and CSI request. Support UE capability reporting for the timeline of each individual use case. For CSI prediction, UE may report an offset value t to the existing (Z,Z’) values. For N4=1, the timeline is (Z_(alml-csi-pred)^',Z_(aiml-csi-pred)+w); for N4=1, the timeline is (2Z_(alml-csi-pred)^',Z_(aiml-csi-pred)+w+Z_(alml-csi-pred)^' ) or (Z_(alml-csi-pred)^',Z_(aiml-csi-pred)+w) upto UE capability where Z_(alml-csi-pred)^'=Z_2^'+t and Z_(aiml-csi-pred)=Z_2+t, w=14(K-1)m if measurement resource is aperiodic or w=56(K_p-1) if measurement resource is periodic or semi-persistent. Observation 9: Absorbing the model switching and loading time into nominal timeline of each individual use case would result in conservative timeline which reduces the benefits of CSI prediction. Observation 10: Defining rule for model switching and loading time requires much higher specification impact and is complicated for UE and NW implementation. Observation 11: Model switching and loading time may be always needed if switching between proprietary AIML feature and standardized feature. Proposal 10: Consider the additional latency of model switching time being absorbed into the nominal timeline for each individual case as benchmark. • Study the necessity of defining rule for defining additional latency of model switching considering whether the latency is negligible compared to nominal inference timeline. |
| NTT Docomo | Proposal 2 - Down-select the x-PU occupation to Option 3, i.e., both PUs are occupied.  A common timeline should be adopted for the occupation of both PUs. Proposal 3 - The value of OCPU is based on defined rules, and the value of OAPU can be based on defined rules together with a UE-reported scaling factor.  The number of candidate scaling factors should be limited to reduce the network burden when managing UE with different scaling factors.  For defined rules, reuse the existing counting rules for CPU as a baseline. - A common timeline should be specified for all UEs, regardless of which options are used for the UE. |
| Continental Automotive | Proposal 1: Specify the unified measurement of AI/ML processing capability (e.g., CSI compression, CSI prediction, etc.) and its reporting. Proposal 2: Support configuration of processing unit to support common and dedicated AI/ML operations to optimize computing resource efficiency. Proposal 3: Support Option 1 with priority. |

Background of Rel-18 Doppler CSI

CPU occupancy

- # of occupied CPU

  - For P/SP CSIRS CMR, Ocpu=Y*N4, where Y is reported by UE capability where Y={1,2,3}

    - When N4=1, OCPU =4

  - For AP CSIRS CMR, Ocpu=Y*K, where Y is reported by UE capability where Y={1,2,3}

    - When K=12, OCPU =8

- CPU occupation duration

  - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP ={1, 2 4} is indicated by UE capability.

  - Aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

Timeline

- Z

  - Capability 1:

    - For AP CSI-RS: Z=legacy Z+14.(K–1).m

    - For P/SP CSI-RS: Z= legacy Z+w where w>0, where w (symbols) is reported by UE

  - Capability 2:

    - For AP CSI-RS: Z= legacy Z+14.(K–1).m + r, where r=legacy Z'

    - For P/SP CSI-RS: Z= legacy Z+w+r , where r=legacy Z'

- Z'

  - For N4=1: reuse legacy Z’ values

  - For N4>1, introduce two UE capabilities:

    - Capability 1: Reuse legacy Z’ values

    - Capability 2: Legacy Z’ values + r, where r=legacy Z'

Active resource counting

- AP CSI-RS configuration parameters for CMR

  - # of active CSIRS resources/ports is counted K={4,8,12} times

- P/SP CSI-RS configuration parameters for CMR

  - # of active CSIRS resources/ports is counted Kp times, where Kp={1,2,4} is reported by UE capability

- Issue 2.4-1. CPU/APU occupancy for inference

In the last meeting, followings were agreed regarding processing criteria and timeline as below.

Agreement Introduce a dedicated AI/ML PU for AI/ML features for UE, - The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction. Agreement For CSI prediction using UE side model, for inference, consider following options for potential down selection - Option 1: only dedicated AI/ML PU (OAPU) is occupied - Option 2: only legacy CPU (OCPU) is occupied - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied - FFS whether OAPU and OCPU are based on defined rule and/or reported by UE - Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported. The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. - Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Based on the reviewing submitted contribution, Moderator think that different companies consider different UE implementations for AI/ML based CSI prediction. For instance, proponents of Option 1 think that AI engine can include functionality of CSI measurement and/or CSI calculation. Also, proponents of Option 2 think that inference can be done within the modem, and it may provide less-optimized inference performance compared to Option 1 and 3. For option 3, proponents think that CPU handle the process of channel measurements and/or CSI calculation, and APU only handles the inference operation. Based on Tdoc review, I summarized the companies’ position on each option.

|  | Supporting companies |
| --- | --- |
| Option 1 | Ericsson, vivo, TCL, ZTE, CATT, CMCC, LG, Xiaomi, Ruijie Networks, Panasonic, Fujistu, MediaTek, Continental Automotive, Futurewei |
| Option 2 | Huawei/HiSi, vivo, TCL, ZTE, Spreadtrum/UNISOC, CATT, CMCC, Xiaomi, Panasonic, Fujistu, Qualcomm, Futurewei |
| Option 3 | Samsung, Huawei/HiSi, CATT, Xiaomi, NEC, Ruijie Networks, InterDigital, Panasonic, Lenovo, Fujistu, HONOR, Nokia, ETRI, OPPO, Apple, Qualcomm, NTT Docomo, Futurewei |

Given above situation, Moderator think it seems hard to down-select to one or two options, since it will restrict the freedom of UE implementation. Also, Panasonic, Fujitsu, CATT, Xiaomi, Lenovo, Futurewei proposed to support all three options based on UE capability.

Value of OCPU and OAPU

For OAPU of Option 1 and 3, most of companies are proposed that it can be based on the UE capability.

For OCPU of Option 2 and 3, some companies think it can be based on UE capability, and some companies think it can be a based on defined rule and UE capability similar to legacy. In legacy, due to some higher layer parameter, e.g., N4 and K, CPU occupancy rule are defined as below.

Occupancy duration of CPU and APU

For the occupancy duration of either CPU and APU, most of companies think that legacy occupancy duration can be reused captured as below.

- CPU occupation duration

  - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP ={1, 2 4} is indicated by UE capability.

  - Aperiodic CSI report occupies CPU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

For Option 3, Huawei, HONOR, Nokia, Apple proposed to support the same occupancy duration for APU and CPU. Meanwhile, Qualcomm think the ending time of occupancy duration of CPU and APU is extended to the last symbol of monitoring report. For Qualcomm’s proposal, I fail to see the necessity of extension of CPU/APU occupancy time, it is more likely to CPU occupancy time of monitoring report.

Occupancy rule

For APU, most of company think that the mechanism similar to legacy CPU occupancy can be reused.

The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC in a component carrier, and simultaneousCSI-ReportsAllCC across all component carriers. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0,…,N-1 corresponds to OCPUn, the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤Nis the largest value such that n=0M-1OCPUn≤NCPU-L holds.

In addition, vivo and Huawei proposed that if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied. I think that is valid argument, otherwise, it is hard to manage PU.

Based on the above discussion, following is proposed.

Proposal 2.4-1.

For CSI prediction using UE-side model, for inference

- For PU occupancy, support following options subject to UE capability

  - Option 1: only dedicated AI/ML PU (OAPU) is occupied, OAPU = N1 is reported by UE

  - Option 2: only legacy CPU (OCPU) is occupied, OCPU = M1 is reported by UE

  - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied, OAPU = N2 and OCPU = M2 are reported by UE

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied

- For occupancy duration of PU, reuse legacy occupancy duration, i.e.,

  - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP =[{1, 2, 4}] is indicated by UE capability.

  - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

Please provide your input/suggestion on Proposal 2.4-1.

| Company | Views |
| --- | --- |
| vivo | Our first preference is the first two options. We think Opt 3 is over-design for PU occupation. With Opt 1, we can simply let UE handle the processing resources of operations like channel estimation, e.g., by considering these resources when counting AI/ML PU numbers. Hence we think the first two options should be sufficient. |
| Samsung | First preference is Opt3. Ok in general for the above, but Option 1 and Option 2 can be supported under Option 3 based on UE’s report on OAPU and OCPU. |
| Huawei, HiSilicon | We are supportive of Option 2 and 3 but have serious reservation regarding Option 1. Option 1 assumes that the AI/ML-based CSI processing only occupies the dedicated AI/ML PU pool, which is not accurate. Taking AI/ML-based CSI report procedure of CSI prediction for instance, the measurement result generation and doppler domain CB generation still consumes legacy CPU processing resources, and cannot be replaced by the AI/ML operation. If Option 1 is adopted, UE has to always preserve fixed accompanied CPU resource for potential AI/ML-based CSI reports, which may cause waste of legacy CPU resources if no AI/ML-based CSI report actually occurs. We are in principle supportive of the intention behind the note. However, the note in its current form is only applicable to Option-3 where both APU and CPU are involved. In Option 1 and Option 2 only one of the APU or CPU is involved. |
| OPPO | Similar view to Samsung. We support Option 3, Option 1 and Option 2 can be supported under Option 3 by reporting O_APU =0 or O_CPU = 0 |
| Fujitsu | Generally fine with the proposal. |
| CMCC | Our first preference is only Option 1 and Option 2. For any one of the two options, the extra consideration of PU occupation in Option 3 can be handled or absorbed in the value of OAPU or OAPU. And for the occupancy duration of PU, for the case of initial SP CSI report on PUSCH after PDCCH triggering the report, we think it starts from the first symbol after the PDCCH until the last symbol of the PUSCH carrying the report. |
| Xiaomi | We are general fine with the proposal. For Option 3, since both OAPU and OCPU are occupied, we think CSI report should be not required to update the CSI reports with lowest priority if unoccupied APU or CPU cannot satisfy the corresponding required PU or CPU respectively. Although PU for AI-based CSI prediction is not occupied, the PU may be occupied by other AI-based CSI reporting. “neither of the PUs is occupied” is not corrected. Hence, we suggest the note could be reworded as follows. - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied, OAPU = N2 and OCPU = M2 are reported by UE - Note: if any of the unoccupied PU or CPU cannot satisfy the corresponding required PU or CPU respectively by the CSI report, the remained CSI report with lowest priority is not required to update, and neither of the PUs is occupied |
| NTT DOCOMO | We suggest selecting only 1 option, either Option 1 or Option 3 is acceptable, and we don’t support the O_APU number can be arbitrarily reported by the UE. We don’t think the NW is capable on accurately managing every piece of computing resources of each individual UE. As long as we have a clear rule on APU occupation given a CSI report, it’s up to UE implementation on how to map UE’s computational resources to the N_APU. That is also how CPU works. O_CPU = 2 does not mean double computational resources are consumed compared with the reports with O_CPU=1 in UE’s implementation. For O_CPU numbers, we understand that there are differences in the implementation of AI/ML, and some flexibility should be supported. However, if the UE reports both N_APU and O_CPU, there will be no common understanding on the unit of APU between NW and UE. For example, if one UE reports N_APU=10 and another UE reports N_APU=5, it does not mean the first UE is more powerful on AI/ML because it may also report a larger number for O_APU. If we go this way, the N_APU becomes UE’s lucky number without any meaning. I don’t know how the NW understands and handles these numbers from different UEs. At least one number between N_APU and O_APU should be constrained. To allow some flexibility, we can compromise on a solution that UE can report O_APU from a limited number of candidates, e.g., 3 candidates, to reflect the UE implementation with large/medium/small scale AI/ML models. In this way, some common understanding on N_APU between NW and UE still exists. |
| Lenovo/ MotM | Share similar views with Samsung |
| HONOR | Our first priority is Option 3. We can live with Option 2 subject to UE capability. As commented by Huawei, Option 1 has some problems that may against UE processing procedures. In any case, legacy CPU should be occupied for RS measurement and UCI encoding. |
| Apple | First, we would like to discuss the proposal of define separate APU for CSI prediction use case, as it has RRC impact on UE capability. For this particular proposal: FL please clarify CSI that “for inference” includes not only model inference, but also the PMI calculation after CSI prediction inference. For Option 3, the OCPU is used to calculate the legacy codebook, therefore, we can follow legacy definition. We do not see whether M2 is used. Please clarify. Or we add - Option 4: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied, OAPU = N1 is reported by UE, OCPU is defined as in R18 Doppler codebook. For CSI prediction using UE-side model inference, to calculate the report using Doppler codebook, … |
| Apple2 | For the 1st part, we suggest the similar proposal from BM, to include different UE implementation options. - For PU occupancy, support following options subject to UE capability - Option 1: only dedicated AI/ML PU (OAPU) is occupied, OAPU = N1 is reported by UE - Option 2: only legacy CPU (OCPU) is occupied, OCPU = M1 is reported by UE - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied, OAPU = N2 and OCPU = M2 are reported by UE - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied - The max number of AI/ML PU for CSI prediction can be reported by UE capability optionally. - If max number of AI/ML PU for CSI prediction is reported, UE does not expect to be configured/triggered with more than the max number of AI/ML PU for CSI prediction. |
| CATT | Support the proposal. |

- (closed) Proposal 2.4-1.

For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook,

- For PU occupancy, support

  - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied,

    - OAPU= 0 or N is reported by UE

    - OCPU=0 or M is reported by UE

    - Note: Detailed values of N and M can be further discussed in UE feature.

    - Note: OAPU= 0 and OCPU=0 is not allowed

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied.

- For occupancy duration of CPU and APU,

  - the same occupancy duration is used if both CPU and APU are reported non-zero value,

  - reuse following legacy occupancy duration at least if monitoring report is not configured

    - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP =[{1, 2, 4}] is indicated by UE capability.

      - Note: Detailed values of KP can be further discussed in UE feature.

    - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability

- The max number of AI/ML PU for CSI prediction can be reported by UE capability optionally.

  - If max number of AI/ML PU for CSI prediction is reported, UE does not expect to be configured/triggered with more than the max number of AI/ML PU for CSI prediction.

  - Note: No additional priority rule is introduced.

Please provide your input/suggestion on Proposal 2.4-1.

| Company | Views |
| --- | --- |
| CATT | Support the proposal. |
| TCL | Support |
| OPPO | Generally fine, but with two minor questions requiring further clarification for the occupancy duration of CPU and APU. Q1: as per our understanding, APU is only occupied when model inference is activated. The CSI measurement and pre-processing seem only occupy CPU. Therefore, for the CSI prediction at UE-side, when both of O_CPU and O_APU are reported by UE, it seems CPU is firstly occupied, and then APU is occupied. Using the same occupancy duration may cause some resource waste on CPU and APU. Actually, we understand the situation that designing a new framework to distinguish the CPU and APU occupation duration may be time consuming in current stage. Reusing the same framework is acceptable for us. Q2: when UE reports K_p = {1,2,4}, it seems only K_p CSI measurements can be used as the model input? How to handle the condition when the observation window is larger than K_p? Is my understanding right? Mod: Thanks for the question. For Q1, as I explained during offline, I agree that it is not optimized occupancy duration, but it is simple occupancy duration. And, it will give more degree of freedom for UE implementation as it is quite open for UE to implement APU occupancy. For Q2, that is why I put bracket on Kp values. To be more clear, I put note with purple color. |
| Xiaomi | Since both APU and CPU could be occupied at same time, if one of APU and CPU could not satisfy the required APU or CPU, CSI report with lowest priority should be not required to update. Hence, we suggest the second note needs to be reworded as follows. - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied, OAPU = N2 and OCPU = M2 are reported by UE - Note: if any of the unoccupied PU or CPU cannot satisfy the corresponding required PU or CPU respectively by the CSI report, the remained CSI report with lowest priority is not required to update, and neither of the PUs is occupied |
| SPRD | We think the first part is OK. However, for the occupancy duration of CPU and APU, if the CPU and APU are both non-zero, is that means both APU and CPU are occupied in a period of time? Mod: Yes, your understanding is correct |

During the online discussion, we did not have enough time to discuss the issue on whether UE can report maximum # of PU per use case. So, please make your comment based on the proposal below.

- (new) Proposal 2.4-1B.

For CSI prediction using UE-side model, for inference,

- The max number of AI/ML PU for CSI prediction can be reported by UE capability optionally.

Please provide your input/suggestion on Proposal 2.4-1B.

| Company | Views |
| --- | --- |

- (closed) Issue 2.4-2. Time line

For CSI processing time, (Z, Z’) values are defined where Z is minimum CSI processing time after receiving Aperiodic CSI triggering DCI until reporting and Z’ is minimum CSI processing time after receiving CSI-RS for channel/interference measurement until reporting.

For Rel-18 CSI prediction, followings are supported.

- (Z2+14K-1m,Z2'), with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or - (Z2+w,Z2'), with (Z2,Z2') of table 5.4-2, where w=56.(KP –1) or 56.KP symbols, according to the reported UE capability, where the value of 𝐾𝑃 ∈{1,2,4} is indicated by UE capability, if the CSI report is configured with N4=1, codebookType is set to 'typeII-Doppler-r18' or 'typeII-Doppler-PortSelection-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or - (Z2+14K-1m,Z2') or (Z2+14K-1m+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is aperiodic with K CSI-RS resources, or - (Z2+w,Z2') or (Z2+w+Z2',2Z2'), according to UE reported capability, with (Z2,Z2') of table 5.4-2, if the CSI report is configured with N4>1, codebookType is set to 'typeII-Doppler-r18' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is periodic or semi-persistent with a single CSI-RS resource, or

Regarding this issue, there are several proposals from companies. Ericsson thinks that shortened timeline (except for the initial CSI reporting occasion) can be considered and additional delay by model switching can be introduced. Huawei thinks that CSI processing timeline can be different according to the state of functionality (e.g., active state, inactive state), and this state can be controlled by NW. Qualcomm think common offset relative to legacy (Z, Z’) value can be reported by UE and also thinks additional latency for model switching needs to be considered. Meanwhile, ZTE proposed the same timeline as Rel-18 CSI prediction, and CMCC, NEC, NTT Docomo think no need to introduce a different timeline when functionality switch/activate.

- Question 2.4-2.

- Q1: Do you think timing relaxation (e.g., additional timing offset) is needed compared to the time line of Rel-18 CSI prediction?

- Q2: Do you think different timeline or additional offset is needed when functionality is switched or activated. If yes, how to define timeline?

Please answer Question 2.4-2.

| Company | Views |
| --- | --- |
| Qualcomm | Q1: we don’t think anyone can say R18 timeline is sufficient for inference using UE side model. The only way is to define an offset value for UE to report it as capability, we can further decide the candidate values in UE feature session, so that the issue will not delay the progress as this is the last meeting. Q2: we think the proper way is to have the model switching time absorbed into the nominal timeline for simplicity. Besides, there could be proprietary models in level x, it is not possible to define the switching time if switching from these level-x model to model for CSI prediction. In other words, the model switching time may always exists. |
| vivo | We are open to further consider these two questions. |
| Samsung | Q1: Open to discuss. Q2: yes. |
| Huawei, HiSilicon | Q1: We are open to introduce AI/ML specific values of Zref and Z'ref Q2: Yes. The model needs to be deployed to the AI/ML processing memory before performing inference (functionality activation). Considering multiple models supported by the UE share the AI/ML processing memory, storing a specific model in the AI/ML processing memory all the time would cause a waste of the storage memory. Therefore, the model should be flushed after each single inference occasion is finished (functionality deactivation). Functionality activation introduces a latency (typically around a couple of TTIs) which should be considered in the processing timeline. Taking A-CSI report for instance, if the corresponding functionality is subject to inactive state, the CSI processing timeline should count both the functionality activation delay (ΔT) and the inference delay (Zref), while otherwise the CSI processing timeline only counts the inference delay (Zref). |
| OPPO | Q1: need to further discuss. Whether timing relaxation is needed cannot be decided now. Q2: generally yes, we think additional offset is required when a nee functionality/UE-side model is activated, which is different when different models are deployed. How to define the timeline may still require further discussion. |
| Xiaomi | In generally, if model has been loaded into AI engine, additional timing offset is not needed since AI-based CSI prediction may need less processing time due to parallelization computation. However, if model has not been loaded into AI engine, additional offset is needed. In order to make the timeline definition be simple, we suggest the additional time offset is reported by UE, and the timeline is defied as Z/Z’+δoffset, where value of Z/Z’ is defined as the legacy (Z, Z’) value of Rel-18 Type II Doppler codebook, δoffset is additional time offset reported by UE. |
| LGE | Q1: Yes. Additional timing offset can be considered. Q2: No |
| Lenovo/ MotM | Q1: Yes Q2: Agree with QC. This should be captured within the AI/ML CSI prediction |
| HONOR | Q1: Yes. The timeline may depend on the model complexity. Time offset can address this issue. Q2: Open to further discuss. However, in functionality-based LCM, UE side model is transparent to network, it’s easy to define what’s active functionality but it’s hard to define what’s active model. |
| Apple | For Q1, UE report additional inference time on top of R18 value. For Q2, we can go with the proposal that the additional switching is part of the value in Q1. |
| ZTE | For Q1, we think whether timing relaxation is needed should be considered by . For Q2, we think the model switching issue is strongly related to the AI engine/model architecture. As is mentioned by companies, loading AI model from hard drive would cause latency up to several hundreds of ms. While loading from global memory or AI dedicated memory could significantly reduce the latency to several ms or even us level. For us level latency, the additional latency caused by model switching would be negligible. So we should treat the different timeline or additional offset up to UE capability. |
| CATT | Q1: Can be further discussed Q2: We think whether a different timeline is needed when functionality switches/activates is based on UE capability. |
| Mod | Based on the answer, I propose the following. |

- (new) Proposal 2.4-2.

For CSI prediction using UE-side model, regarding Z/Z’

- For N4=1,

  - For Z value, Z2+t+w

  - For Z’ value, Z’2'+t

- For N4>1,

  - For Z value, according to UE capability

    - Z2+t + w+ Z’2 + t, or

    - Z2+t+w

  - For Z’ value, according to UE capability

    - 2(Z’2'+t), or

    - Z’2’+t

- UE may report the value of t

  - Detailed value of t can be discussed in UE feature

- Note:  (Z2, Z’2) of table 5.4-2 in TS38.214, w is the same value for Rel-18 doppler codebook

Please provide your input/suggestion on Proposal 2.4-2.

| Company | Views |
| --- | --- |
| TCL | We believe that the timeline of Rel-18 CSI prediction can be reused; however, it is necessary to define the specific values of Zref and Z'ref for AI-based CSI prediction. Moreover, we are concerned that introducing an offset value based on the timeline of Rel-18 may result in a coupled design. |
| Xiaomi | According to the proposal, does it means that the total delay is Z/Z’+ t. If so, we are OK. But it is better to clarify this. Mod: yes, your understanding is correct. I revised the proposal |
| ZTE | Ok with the proposal. |
| SPRD | Ok with the proposal. |
| Mod | I slightly modify the proposal based on current specification. The intension is to replace legacy (Z2, Z’2) with (Z2 +t, Z’2 +t) where t is reported by UE capability. |
| Huawei, HiSilicon | We are OK with introducing an offset to the legacy values to provide inference-specific values for Z2/Z2’. However, we think RAN1 needs to support an "active functionality window" where outside this window a further \Delta value is added to the processing time to represent the upload/activation latency of the model. Mod: The intension of proposal is to introduce offset value to absorb the functionality switching time, rather than introduce different time line according to model switching, and this seems majority view. |
| Qualcomm | If following the formulation of R18, for N4 > 1, it seems that Z=z2+t+w or Z2+t+w+Z2’+t, we don’t need 2(Z2+t)+w+Z2’+t. Mod: Thanks for pointing this out, it is fixed. |

- (closed) Issue 2.4-3. Active resource/port counting

For active resource/port counting, LG, Ericsson, HW, CATT, CMCC, Xiaomi, Lenovo, Apple think legacy active resource/port counting can be reused. Meanwhile, vivo and Qualcomm think that active resource counting time for inference needs to be extended to the end of monitoring report as monitoring requires the results of inference. I think this is valid issue, so I would like to hear more view on this issue.

- Question 2.4-3.

- Q1: For active resource/port counting for inference, do you think that active resource counting time for inference needs to be extended from the end of inference report to the end of monitoring report?

Please answer Question 2.4-3 including other solution, if any.

| Company | Views |
| --- | --- |
| Qualcomm | Yes if L1 signalling is adopted, same for CPU occupation. But we also need to consider the case if monitoring report will never be triggered because UE will not buffer the CSI forever. This issue should be discussed with monitoring together. |
| vivo | We think it is crucial to extend the active resource counting time to the end of monitoring report to make monitoring report in CSI work. To calculate SGCS metric, UE needs to buffer the inference output or its associated PMIs, which has large size. Without addressing this buffering issue, UE cannot afford to have monitoring report in UCI. In the current specification, the buffering size of CSI processing is reflected by active resource counting. With extending the active resource counting time to the end of the associated monitoring report, this extra buffering is explicit to NW, and NW can control the total CSI buffering will not exceed the UE capability. |
| Samsung | Yes. |
| OPPO | We do not think the extension to monitoring report is required. |
| Fujitsu | We think the case of inference and monitoring should be discussed separately. At least for inference, there is no need to introduce enhancement. |
| Xiaomi | If monitoring is reported via L1 signalling, the active resource counting time for inference needs to be extended to to the end of monitoring report. Otherwise, it is not needed. We suggest the issue could be discussed after Question 2.5-3. |
| Huawei, HiSilicon | Q1: We can agree to extend the duration of the active resources, but not till the monitoring report, because for A-CSI monitor report, UE is not aware of the time position of the monitor CSI report before it successfully receives the DCI – then, the UE behaviour is unclear, if there has no been monitoring DCI received for monitoring an inference CSI report – is it supposed to store the CSI buffer till a later time, or flush it immediately after the inference CSI report? To us, a simpler way is to assume UE extends the active resource duration of the inference CSI report till the end of the last predicted instance (which is a fixed duration regardless of the time location of the monitoring CSI report), if it is configured with accompanied monitoring CSI report. |
| HONOR | According to current discussion, inference report and monitoring report are two separate reports. active resource/port counting for inference relies on monitoring report may have other impact on network scheduling on the triggering timeline between two reports. |
| Apple | If L1 reporting for performance monitoring is used. For performace monitoring L1 report, it should be from CSI-RS for inference to end of monitoring report. For inference report only, no need to extend. |
| ZTE | We think this issue is linked with at least the two following aspects, i.e., the performance metric determination and the indication of the monitoring RS and the inference RS. For performance metric determination, if the metric is not determined per inference instance, the resource could be released at those instances that does not require metric determination. Whether the memory occupation is considered as the active resource counting is separate discussion. Besides, the active resource counting manner is binding the performance monitoring RS and the inference RS together, which treated after clarifying the whether the binding between te inference RS and the monitoring RS is needed. |
| Ericsson | Before discussing this proposal, we need to firstly discuss issues in section 2.5 and agree on how a performance monitoring report is configured. |
| CATT | For inference report, no need to extend. For monitoring report, open to discuss. |

- (closed) Proposal 2.4-3.

For CSI prediction using UE-side model, for inference, regarding active resource/port counting,

- Reuse legacy active resource/port counting at least if monitoring report is not configured

Please provide your input/suggestion on Proposal 2.4-3.

| Company | Views |
| --- | --- |
| TCL | Support |
| Xiaomi | Support |
| ZTE | support |
| SPRD | support |

- Issue 2.4-4. CPU occupancy for monitoring

Regarding CPU occupancy for performance monitoring, in BM agenda, following was agreed.

Agreement For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, OCPU=1. Note: the occupation duration is a separate discussion.

Regarding this issue, Huawei, CATT, HONOR, LG provide their view. For the value of OCPU, CATT, LG think it can be based on the monitoring RS occasion, e.g., N4, and HONOR think scaling factor on top of legacy Rel-16 codebook can be introduced. For occupancy duration, Huawei think that CPU is occupied from DCI till the scheduled PUSCH for A-CSI monitoring report, and from the N4-th latest consecutive P/SP-CSI-RS occasion till the corresponding PUSCH for SP-CSI monitoring report, if dedicated monitoring report is configured. Moderator think this issue can be handled after we have some progress on performance monitoring report.

Please provide your view on issue 2-4-4, if any.

| Company | Views |
| --- | --- |
| Qualcomm | We should discuss monitoring mechanism first. If L3 signaling is considered, CSI processing criterion can be relaxed as there is no need to satisfy the stringent timeline. If L1 signaling is used, at least UE would need to compute the CSI for the ground-truth using R18 codebook, so CPU cannot be one, no need to say there is a buffering issue and reconstruct the CSI of the buffered predicted CSI. |
| Huawei, HiSilicon | For monitoring Type 3, assuming a dedicated CSI report is configured to the monitoring CSI session, for A-CSI monitoring report, the CPU occupancy starts from DCI to the scheduled PUSCH, similar to legacy. For SP-CSI monitoring report, N4 individual measurements are needed to generate labels for the comparison with each of the N4 prediction results; therefore, similar to the CPU occupancy duration for Rel-18 MIMO where the CPU is occupied from the Kp-th latest consecutive P/SP-CSI-RS occasion till the PUSCH, the CPU is occupied from the N4-th latest consecutive P/SP-CSI-RS occasion till the corresponding PUSCH. |
| Xiaomi | Agree with FL, it could be discussed after more progress on performance monitoring report. |
| HONOR | It’s highly related to how we define the PMI for ground-truth CSI as discussed in Question 2.5-1. |
| Apple | Do not agree. If L1 report is used, to calculate performance metric, comparing to BM, much higher complexity is observed. It is much higher than inference, as the following operation are invloded. - To calculate the SGCS, the UE needs to perform the following operations based on the current SGCS definition: - Reconstruct the predicted precoder using stored codebook coefficients. - Estimate the channel based on the CSI-RS in prediction window - Calculate the PMI (Precoding Matrix Indicator) coefficients based on the configured codebook. - Reconstruct the precoder from the coefficients. - Calculate SGCS. If option 2 of performance monitoring is used, the above steps are needed for the sample and hold channel as well. So it is roughly 3 times more of inference delay of Z/Z’. |
| Ericsson | Before discussing this proposal, we need to firstly discuss issues in section 2.5 and agree on how a performance monitoring report is configured. |
| CATT | For determination of OCPU for performance monitoring of CSI prediction, it at least can be based on N4. |
| Mod | I will prepare the proposal, after we have some progress in monitoring. |

- Issue 2.4-4. CPU occupancy for data collection

For CPU occupancy of training data collection, there are two different views (CPU occupancy is needed v.s. not needed). CATT thinks there is no need to occupy CPU for training data collection. Meanwhile, Huawei, HONOR, Apple think CPU occupancy is needed. In addition, for the value of OCPU, HONOR and Apple propose to have OCPU =1. For the occupancy duration, Huawei and Apple think legacy rule subject to report quantity of ‘none’, i.e., from the first symbol of each P/SP-CSI-RS occasion, till Z3’ symbols after the P/SP-CSI-RS occasion, can be reused. For the occupancy duration, I’m not sure reusing legacy seems sufficient/efficient or not, since the legacy rule is for the Rx beam sweeping.

- Question 2.4-4

- Q1: For CPU occupancy for training data collection, does CPU needs to be occupied?

- Q2: If answer of Q1 is yes, is it sufficient to have OCPU =1 and reusing legacy occupancy duration?

Please answer Question 2.4-4.

| Company | Views |
| --- | --- |
| vivo | Q1: Our preference is it does not occupy CPU, as how to process the training data collection is totally up to UE. |
| Samsung | Q1: yes. It is crucial to consider CPU occupancy. Considering the measured CSI is with possibly large number of ports, the CSI report for data collection would occupy non-trivial computational requirement. |
| Huawei, HiSilicon | Q1: Yes. Q2: We can use the same legacy principle: Since UE does not report the CSI for training data collection, similar to legacy, the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion. Our FL concern can be addressed by supporting AIML-specific values for Z3’ but there is no reason to completely deviate from legacy design. |
| ETRI | Q1: Yes Q2: Yes |
| Fujitsu | Q1: Yes. Q2: open for discussion |
| CMCC | Q1: Our preference is CPU is not needed to occupy for data collection. |
| Xiaomi | Since there is no CSI calculation, we think CPU occupation is not needed. |
| LGE | Q1: Our preference is CPU doesn’t needed to be occupied for data collection. |
| HONOR | Q1: Yes Q2: Yes. Prefer to reuse the legacy CPU mechanism for Rx beam sweeping. |
| Apple | Q1: yes. Q2: Yes. The Z’ value can be further discussed, as comparing to legacy with “none” report, which is used for P3 procedure, the operation here is much eaiser. |
| ZTE | For Q1, whether the CPU needs to be occupied is a related question to the UE model architecture. For example, if the UE uses received signal as model input, the training data collection would occupy no CPU. While taking channel matrix as a model input would occupy CPU. So the answer is better to leave this for UE capability reporting. For Q2, if the CPU is occupied, it is sufficient to have Ocpu = 1 and reusing the legacy occupancy duration as a starting point. |
| Ericsson | Q1: No, since the UE is not reporting the measurements to the NW for training data collection, there is no need to count the CPU. |
| CATT | Q1:No. Whether to measure the configured resources is up to UE, and it can be considered as Ocpu =0. |
| Mod | Based on the input from companies, I summarized the companies’ position as below OCPU=N (7) : Samsung, Huawei/HiSi, ETRI, Fujitsu, HONOR, Apple, ZTE(?) OCPU=0 (6) : vivo, CMCC, Xiaomi, LG, Ericsson,CATT I propose the following based on the majority view |

- (new) Proposal 2.4-3.

For CSI prediction using UE-side model, for training data collection,

- OCPU=X, FFS X value

- the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

  - FFS on Z3’ values

Please answer Question 2.4-3.

| Company | Views |
| --- | --- |
| TCL | Support |
| Xiaomi | We are open to discuss. The motivation for occupied CPU more than zero is not clear to us. |
| ZTE | Open to discuss. We still think it’s related to UE implementation. The value could be more or less than N for CPU occupation for training data collection. Maybe it should be considered as a UE capability report. Mod: Is it ok for you? |
| ZTE | Thanks feature leader for the update. We are fine with the current proposal. |

- (closed) Issue 2.4-5. Priority rule

Regarding priority rule for AI based CSI prediction, vivo, Huawei, and TCL provide their view. Vivo think that priority rule should be given by non-AI CSI report > AI CSI report > monitoring report, meanwhile TCL think AI CSI report should have higher priority than non-AI CSI report. Also, Huawei think priority can be given as monitoring report > inference report > data collection.

In NR, there are 4 level of priority is defined as

- Level 1: Time-domain behaviour / channel (AP-CSI > SP-CSI on PUSCH > SP-CSI on PUCCH > P-CSI)

- Level 2: CSI content (Beam related report > other report)

- Level 3: cell id

- Level 4: CSI report id

I think, without any enhancement, current priority rule can be applied to AI based CSI report. Therefore, we need to first discuss on whether differentiation between AI based CSI report and other CSI report is needed.

- Question 2.4-5

- Q1: Do you think enhancement of current priority rule is needed for AI/ML CSI prediction?

- Q2: If your answer of Q1 is yes, what is your preference on priority rule?

Please answer Question 2.4-5.

| Company | Views |
| --- | --- |
| Qualcomm | No need, don’t see any issue of existing rule. |
| vivo | In addition to legacy CSI priorities, we think we should further clarify the priority between monitoring report and inference report. Our understanding is the priority of monitoring report should be always lower than inference report. As monitoring report KPI is calculated based on inference output, the meaning of monitoring report is not clear if inference report is dropped. |
| Samsung | Priority between inference report and monitoring report should be determined. |
| Huawei, HiSilicon | Q1: Yes. The priority rule needs to be updated considering the newly introduced LCM components, namely training data collection, inference, and monitoring. As an example, the monitoring result is used to reflect whether the network is still working properly, and should be more critical than inference report or non-AI/ML CSI report. As another example, for training data collection, UE does not report the measurement of CSI-RS, and the training data collection is non-real time, so the CSI priority for training data collection can be lower than other types of CSI measurement/report (e.g., inference CSI report) when considering the CPU mechanism. Q2: This may be achieved by introducing additional priority calculation variables in the legacy formula. For instance: PriiCSIy,m,k,c,s=2∙3∙Ncells∙Ms∙y+2∙Ncells∙Ms∙m+Ncells∙Ms∙k+Ms∙c+s, where m=0 for monitoring CSI, m=1 for inference CSI or legacy CSI, and m=2 for training data collection CSI. |
| Fujitsu | Agree with vivo. |
| NEC | Yes. If L1 signaling is supported to report SGCS, we think that at least priority rule for CSI report for monitoring needs to be defined. Additionally, we have the similar view with Huawei/HiSilicon, additional priority calculation variables in the legacy formula can be considered. |
| CMCC | No, current spec is enough to prioritize specific CSI report. |
| Xiaomi | Q1: enhancement of current priority rule is needed for AI/ML CSI prediction. Since both AI-based CSI reporting and non-AI based CSI reporting for the same CSI feedback functionality may be exist at the same time, it is necessary to define their priorities. Q2: New parameter k'could be introduced to define the priorities. E.g., the priority is defined as PriiCSIy,k',k,c,s= α∙Ncells∙Ms∙Mk'∙y+β∙Ncells∙Ms∙Mk'∙k+γ∙Ncells∙Ms∙k'+Ms∙c+s, where Mk' denotes the number of beam reporting. |
| LGE | Yes. Monitoring is essential to prioritize monitoring reports over inference reports to ensure prompt and reliable AI/ML performance feedback. |
| Lenovo/ MotM | CSI report for inference priority should follow legacy Type-II, Level 4 CSI report for monitoring should have higher priority, Level 2 |
| Apple | For inference report only, legacy rule can be used to AI CSI prediction. If L1 report for performance monitoring is used, agree with Samsung, priority between inference report and monitoring report needs to be discussed. In addition, monitoring report versus legacy report needs to be discussed. |
| Ericsson | Similar view as Qualcomm, we don’t see the need of enhancement. |
| CATT | Q1:Yes. Q2: The parameter k in legacy priority rule can be extended according to reportQuantity to distinguish the AI CSI report and legacy CSI report, or to distinguish the inference or monitoring report. |
| Mod | This issue will be handled after we have some progress on monitoring. |
| TCL | Q1: No need. However, one thing we need to consider is how to handle a collision between the AI CSI report and the legacy CSI report if we reuse the legacy priority rules. |

Performance monitoring

- Summary

In this meeting, performance monitoring related proposals/observations are listed below:

| Ericsson | Observation 6 For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, Alt 2 is not supported, since a. Alt2 causes high computational complexity at UE b. NW has other low complexity methods to compare the AI CSI prediction performance with nearest historic CSI baseline approach c. even if SGCS comparation between AI CSI prediction and nearest historic CSI baseline is needed, it can be supported via Alt1 by NW implementation. For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, support only Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI, where A SGCS value of an inference reporting is averaged over all configured sub-bands For rank>1 or N_4>1, the SGCS value is reported per MIMO layer per prediction instance. Report quantized SGCS in the range of 0.3 to 1 with 0.1 granularity. Proposal 6 For CSI prediction using UE-side model, support a dedicated CSI report configuration for performance monitoring reporting in L1 signaling, which includes at least a. an inference report configuration ID, or a CSI-RS resource set for obtaining model input b. a CSI-RS resource set for obtaining ground truth CSI. |
| --- | --- |
| Huawei/HiSi | Proposal 4: For Type 2 monitoring, to enable the UE to avoid the disordered layers between the inference CSI and the measured CSI subject to the monitoring CSI report, the linkage between the inference CSI report and the monitoring CSI report should be indicated to UE. • Note: The format of the ground-truth CSI can be the same as a legacy CSI codebook. Proposal 5: For the report of monitoring results for Type 3 monitoring, consider L1 signaling considering: • More stringent latency requirements for monitoring report. • Ensuring a unified signaling framework with UE assisted monitoring beam management which has already agreed to adopt L1 signaling. Proposal 6: For Type 3 monitoring, support Alt.1 (one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI). • The SGCS can be calculated individually for each layer (l=1,…,v) and each prediction instance (n4=1,…,N4), and averaged over subbands (n3=1,…,N3). Proposal 7: For the CSI report configuration of monitoring Type 3: • Reuse the agreed mechanism for the beam management case, i.e., a dedicated CSI report for performance monitoring is configured for monitoring the inference results of the inference CSI report.  To indicate the linkage between the monitoring CSI report and the inference CSI report, the associated CSI-ReportConfigId of the inference CSI report configuration is configured in the monitoring CSI reporting configuration.  For each of N4 predicted instances subject to the associated inference CSI report, the monitoring RS falling into its prediction time unit can be used for comparing with the corresponding predicted result. • The SGCS values for all the N4 prediction instances of the associated inference CSI report are included in the monitoring CSI report. |
| Vivo | Proposal 10: For the monitoring of N4>1 inference, only one (the f-th) from the N4 DD units is monitored, where f is configured in CSI report config for monitoring. Proposal 11: SGCS is defined as wideband. Proposal 12: SGCS is defined per layer. Proposal 13: Support the SGCS report method Alt 1. Proposal 14: Supports reuse of existing CSI reporting framework for performance monitoring report. - Add SGCS as a new report quantity and report SGCS in L1 signaling - Quantize the SGCS in linear domain or dB domain, and form a sequence of maximum 4 layers in order, with each layer occupying 4bits, and the total bit width is 16 bits Proposal 15: A dedicated CSI report configuration is configured for monitoring which is associated to the inference configuration. Each monitoring RS occasion is linked to an inference occasion. Details follow the mechanism defined for AI beam management Case 2 as following. Proposal 16: Support that the codebook configuration for ground truth CSI is an eType II codebook configured in the monitoring report config. |
| TCL | Proposal 8 : For CSI prediction using the UE-side model, the periodic or semi-persistent CSI-RS resource can be utilized for both model inference and performance monitoring simultaneously. Proposal 9 :For aperiodic CSI measurement and reporting, additional aperiodic CSI-RS resources should be configured to enable performance monitoring. Proposal 10 : For the reporting contents of the UE-assisted performance monitoring, Alternative 1 is sufficient. Proposal 11 : For reporting contents of UE-assisted performance monitoring, support reporting per prediction instance. Proposal 12 : For the reporting contents of UE-assisted performance monitoring, support calculating the SGCS value based on the averaged over subbands. Proposal 13 : For reporting contents of UE-assisted performance monitoring, at least per-layer reporting should be supported. Proposal 14 : For performance monitoring metric reporting, equal interval quantization is sufficient. Proposal 15 :For performance monitoring metric reporting, only the legacy CPU is utilized, and O_cpu=1. Proposal 16 :For the priority of performance monitoring metric reporting, both AI-based beam management and AI-based CSI prediction should be discussed together. The priority determination should take into account whether the associated CSI reporting carrier includes L1-RSRP or L1-SINR. |
| Spreadtrum/UNISOC | Proposal 2: For data collection for performance monitoring, dedicated resource set(s) for monitoring and report configuration for monitoring can be configured in a dedicated CSI report configuration • The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration. Proposal 5: For the reporting contents of UE assisted performance monitoring, support Alt 3: statistic of reporting contents of inference reporting instances within configured monitoring window. |
| Samsung | Proposal#4: For the AI/ML based CSI prediction using UE-side model, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring o The CSI report Configuration ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration Observation#5: For the AI/ML based CSI prediction using UE-side model, among the alternatives of the reporting contents of UE assisted performance monitoring, - Alt2, as compared to Alt1, provides more information for the network to preform LCM decision, e.g., fall-back to non-predicted CSI. - Alt2, if no restriction is applied, requires a large specification effort, e.g., codebook and CSI timeline for non-predicted CSI - Alt2, if no restriction is applied, may increase UE complexity, UE requires to compute three PMIs for each of monitoring instances. Observation#6: For the AI/ML based CSI prediction using UE-side model, for Alt2 of the reporting contents of UE assisted performance monitoring, when the latest CMR for inference report and the earliest MMR for the corresponding monitoring report are located in the reference slot of the inference reporting instance, i.e., by configuring higher layer parameter predictionDelay as δ=-nref, - The baseline CSI (non-predicted precoder) is the same as the precoder corresponding to n4=0 of predicted PMI W0n3 in the inference report. - For n4∈1,..,N4-1, the UE may calculate the second SGCS values based on the predicted PMI in the inference report, i.e., precoders corresponding to n4=0, W0n3, and the ground-truth precoding vectors corresponding to n4>0,Wn4n3n4=1N4-1. Note: The second SGCS in Alt2 is based on the ground truth CSI and the non-predicted CSI corresponding to the latest CSI-RS transmission occasion not later than the CSI reference resource of the inference-reporting instance. Proposal#5: Considering RAN1#121 is the last meeting of Rel-19 normative work and to address UE complexity, for the reporting contents alternatives of UE assisted performance monitoring, support Alt 2, only if prediction delay is configured as δ=-n_ref. the latest CMR for inference report instance and the earliest MMR for the corresponding monitoring are in the reference slot of the inference reporting instance. Proposal#6: For the AI/ML based CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, support - Performance monitoring KPI reporting per MIMO layer, l=0,…,v-1, where v is the reported rank for the corresponding inference reporting instance. - Performance monitoring KPI reporting per prediction instance, n4=0,1,…,N4-1. - The UE determines the value N4 for monitoring report based on the higher layer parameter vectorLengthDD in the CSI report configuration for inference. - Performance monitoring KPI reporting per wideband frequency granularity by averaging across the subband level SGCS values calculated per PMI reporting subbands indexed by n3∈0,…,N3-1. Proposal#7: For the AI/ML based CSI prediction using UE-side model, for performance monitoring, the network configures at least one monitoring measurement resource (MMR) in the prediction window defined as the slots between the earliest slot to the last slot, in time domain, for the linked predicted PMIs. The UE measures up to N_4 MMRs in the prediction window. The UE is not expected to measure MMR outside the prediction window. Proposal#8: For the AI/ML based CSI prediction using UE-side model, for the quantization of the reported SGCS values, support linear quantization of the SGCS values as monitoring KPIs - The network configures the UE to report SGCS with a certain range [ymin,ymax] with quantization level ∆y and reporting bitwidth B. The range [ymin,ymax] is configured per layer per prediction instance. - The UE may determines the quantization level ∆y=ymax-ymin2B-1 . - The reported bits bB-1,,…,b1,b0 wherein bB-1and b0reperenst the MSB and LSB bits, respectively, correspond to a value, c=i=1B-1bi2i. - For the calculated value, KPIreported, the UE determines and report the bB-1,,…,b1,b0 corresponding to the value c=⌈KPIreported-ymin∆y⌉ifKPIreported≥ymin0ifKPIreported<ymin Proposal#9: For the AI/ML based CSI prediction using UE-side model, for monitoring report configuration, RAN1 to support L1 reporting and address the following issues - Processing unit and occupancy - UCI construction and reporting priority for monitoring quantities. |
| ZTE | Observation 1. For performance monitoring Alt. 2 for AI/ML based CSI prediction, the SGCS determined by two CSI measured at different CSI-RS transmission occasion does not have prior information about the UE side model. Observation 2. For performance monitoring Alt. 3 for AI/ML based CSI prediction, more open issues need to be clarified compared with Alt.1 and Alt.2  How to indicate the time/frequency/spatial domain samples;  How to determine the averaged SGCS, e.g., apply equal weight to time/frequency/spatial domain, or apply different weight to time/frequency/spatial domain, but apply equal weight within time/frequency/spatial domain; Proposal 2. For performance monitoring for AI/ML based CSI prediction, support alt 1 as the baseline for reporting content discussion. Proposal 3. For reporting contents for performance monitoring, in addition to the wideband SGCS as default configuration, UE can further support SGCS reporting on the first/last time instance, the layer with the largest singular value. Proposal 4. For reporting contents for performance monitoring, for quantization, support at most a 4-bit quantization for SGCS reporting. Proposal 5. For AI/ML based CSI prediction, for performance monitoring, support L1 signaling for monitoring report. ̵ For priority rule of monitoring report, reuse other quality metric reporting rule such as CQI as a starting point. Proposal 6. For data collection for performance monitoring, support P/SP/AP CSI-RS configuration. Observation 3: For AI/ML based CSI prediction, the complexity for SGCS determination scales with N_t^5, which should be considered for PU occupation discussion. |
| Quectel | Proposal 1: For AI-based CSI prediction systems, preserving the layer-distinctive characteristics becomes critical for maintaining spatial multiplexing gains and beamforming accuracy, while proper SGCS calculation methodologies must account for per-layer performance variations to ensure meaningful monitoring metrics. SGCS can be calculated separately for each layer. Proposal 2: Massive MIMO systems with high layer counts can employ layer-averaged SGCS reporting to significantly reduce uplink signalling overhead while maintaining system-level accuracy assessment, since the spatial-domain averaging method preserves macroscopic performance trends through strong correlation with end-to-end throughput Proposal 3: In MIMO systems, specifically configuring SGCS reporting solely for the first layer is neither necessary nor recommended since the training and inference behaviours in the first layer don't generalize to other layers and the layer-wise SGCS reporting approach inherently includes first-layer data. Proposal 4: From a temporal domain perspective, CSI prediction accuracy naturally degrades for farther prediction instances due to channel aging effects, per-instance SGCS calculation is essential for meaningful performance evaluation. Proposal 5: The frequency granularity for SGCS reporting can be configured through per wideband, per subband, averaged over subband or selected subband, each offering distinct suitability to scenarios and tradeoffs between accuracy and signaling overhead. |
| CATT | Proposal 11: For CSI prediction using UE-sided model, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling.  Configure a dedicated CSI report for performance monitoring, and the ID of an inference report is configured in the monitoring report. Proposal 12: For performance monitoring of AI/ML-based UE side CSI prediction, both network configured reporting and event-triggered reporting can be considered.  For network configured reporting: periodic, semi-periodic, or aperiodic reporting can be configured;  For event-triggered reporting: the specific event can be defined by reference to beam failure event. Proposal 13: For report content of Type 3 performance monitoring of CSI prediction, support Alt 1, where the reported SGCS can be averaged over multiple samples. Proposal 14: For report content of Type 3 performance monitoring of CSI prediction, the SGCS is reported with the following granularities:  report SGCS of selected instance(s) from N4 prediction instances, where the specific instance(s) can be configured by the network;  report SGCS averaged over subband(s);  report SGCS of per layer or only the first layer. Proposal 15: For performance monitoring of AI/ML based CSI prediction, support the following options for configuration of monitoring resource:  Option 1: Reuse the resource configured in inference report;  Option 2: Configure dedicated resource(s) for monitoring in the monitoring report. Proposal 16: For performance monitoring of CSI prediction, the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring  measurement result of a transmission occasion of the CSI-RS resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS resources for monitoring  Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS resources for monitoring are no later than the CSI reference resource corresponding to the CSI report for monitoring. |
| CMCC | Proposal 2: For CSI prediction using UE-side model, support Alt 1 as reporting contents of UE assisted performance monitoring. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI Observation 1: If consider statistic of reporting contents of inference reporting instances within configured monitoring window as reporting contents of UE assisted performance monitoring, it only can be used for semi-persistent CSI report for inference associated with a semi-persistent CSI report for monitoring with periodic or semi-persistent CSI-RS resources for monitoring. Observation 2: If consider statistic of reporting contents of inference reporting instances within configured monitoring window as reporting contents of UE assisted performance monitoring, then we should study whether multiple CSI-RS resources for monitoring are needed to associate with multiple CSI report occasions for inference in one CSI report. Proposal 3: Support SGCS is reported per prediction instance or selected prediction instance(s) for AI/ML based CSI prediction. Proposal 4: Support SGCS is reported per layer, for AI/ML based CSI prediction. Proposal 5: Support SGCS can be averaged across subband or per wideband, for AI/ML based CSI prediction. Proposal 6: The reported SGCS is a uniformly quantized value between the NW configured or fixed range, and the resolution of reported SGCS can be NW configured or fixed. Proposal 7: For AI based CSI prediction, for Type 3 monitoring, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. |
| Xiaomi | Proposal 9: For obtaining ground truth CSI for performance monitoring, support CSI-RS resource configuration on partial subband or partial instance to save CSI-RS overhead. Proposal 10: If the SGCS is calculated per N4 prediction instances, Rel-18 eType II Doppler codebook could be used to represent PMI used for ground truth CSI. Otherwise, Rel-16 eType II codebook is applied. The largest value of parameter combination for Rel-18 eType II Doppler codebook or Rel-16 eType II codebook is configured according to the rank. Proposal 11: Support to report SGCS calculated by using Alt 1, i.e., one SGCS is calculated based on predicted CSI for one inference reporting and ground truth CSI. Proposal 12: Support to report SGCS per wideband. Proposal 13: Support to report SGCS for the selected prediction instance(s) or per prediction instance. Observation 2: SGCSs of different layers for some users are similar, while SGCSs of different layers for some users are quite diverged. Proposal 14: Support to report SGCS per layer. Proposal 15: Support to L1 signalling for reporting SGCS, and aperiodic, periodic, semi-persistent or UE-event reporting for monitoring performance could be considered. Proposal 16: The number of occupied CPU for monitoring performance metric reporting could be equal to number of predicted instances or selected predicted instances in the prediction window. Proposal 17: The time delay for SGCS reporting is defined as Z ̅+14(N_4-1)d if SGCS is calculated for each predicted instance, where Z ̅ is delay of PMI of ground truth CSI and SGCS calculation delay, N_4 and d respectively denote the number of predicted instance and the interval of adjacent predicted instance. |
| NEC | Proposal 9: Support to report the calculated SGCS per time instance. Proposal 10: Support to report the calculated SGCS per layer. Proposal 11: For reporting contents of UE assisted monitoring, support the following alternatives: - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window Proposal 12: For reporting contents of UE assisted monitoring, the following options can be considered: • If Alt 1 is supported: - Option 1: indication of range of the calculated SGCS. - Option 2: indication of comparison result between the calculated SGCS and a threshold configured by NW. • If Alt 3 is supported: - Option 1: indication of range of the statistical (e.g., mean) value of the calculated SGCSs. - Option 2: indication of comparison result between the statistical (e.g., mean) value of the calculated SGCSs and a threshold configured by NW. - Option 3: indication of the number (or proportion) of ‘correct samples’ within the monitoring window, wherein the ‘correct sample’ can be defined as one whose corresponding SGCS is higher than a threshold. Proposal 13: Support to configure the same (or single) P/SP CSI-RS resource for monitoring (i.e., measurement) and inference. Observation 1: If the inference results derives from an inference report linked with the monitoring report, the inference results may encounter the issue of being unavailable. Proposal 14: Calculation and reporting of SGCS should be based on valid monitoring instance, rather than invalid monitoring instance. • FFS: how to define ‘valid monitoring instance’. Proposal 15: Multiple reportConfigIDs of model inference reports can be associated with a single reportConfigID of performance monitoring. Proposal 16: Study how to refine the performance monitoring procedure when the target timing of predicted CSI is not aligned with the timing of corresponding ground-truth CSI. |
| RUIJIE NETWORKS | Proposal 1: For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, support Alt 1. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI. - Report averaged over predication instances, subband and layer |
| InterDigital | Observation 1: Metrics for monitoring the UE-side CSI prediction model that are more indicative of end-to-end performance should be used to reduce the model switching, activation/deactivation overhead. Observation 2: Using out-of-distribution metrics in conjunction with SGCS may be beneficial for UE-side model monitoring. Proposal 1: For performance monitoring, out-of-distribution metrics should be used in conjunction with SGCS. Observation 3: One SGCS based reporting content for UE assisted performance monitoring (Alt 1) may not lead to desirable decisions at the NW. Observation 4: Alt 2 of UE assisted performance monitoring enables better complexity-performance trade-off decisions at the NW-side. Proposal 2: Down-select Alt-2 for reporting contents of UE assisted performance monitoring. Observation 5: Reporting SGCS values on a per-layer basis would improve decisions at the NW. Proposal 3: Support reporting SGCS on a per-layer basis. Proposal 4: Support separate configuration for CSI-RS for monitoring, especially when aperiodic CSI-RS is used for CSI prediction. |
| LG Electornics | Proposal #6: For report contents of UE-assisted performance monitoring, support Alt 1 where a single SGCS value is calculated based on predicted CSI and ground-truth CSI. Proposal #7: For SGCS reporting, the reported value range is X to 1 with Y step size. - FFS on X, Y values Proposal #8: Consider following SGCS calculation - per prediction instance with NW configured prediction instance(s), - averaged over subband, - the first layer only. Proposal #9: For monitoring reporting, support to reuse CSI framework for the configuration for monitoring result report in L1 signaling. - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration Proposal #10: For monitoring resource type, periodic CSI-RS, semi-persistent CSI-RS, and aperiodic CSI-RS are supported. Proposal #11: For monitoring resource type, following combination of inference report and monitoring report are supported. |
| Panasonic | Proposal 8: For reporting contents of UE assisted performance monitoring, either Alt.1 or Alt.2 is supported. Proposal 9: For performance monitoring of CSI prediction, UE needs to be indicated / configured with the configuration for ground-truth measurement. Proposal 10: For performance monitoring of CSI prediction, performance monitoring metric for the previous predicted CSI can be piggybacked in the predicted CSI report. |
| NVIDIA | Proposal 4: For reporting contents of UE assisted performance monitoring in CSI prediction using UE-sided model, support Alt-3, with support for signaling related to the configurations sent by the NW to the reporting UE(s) and UE capability signaling for the following parameters: - Monitoring window - Details of prediction instance (s) - Reporting frequency granularity - Details of layer(s) to be reported - Reporting mechanism |
| Google | Proposal 2: Support to reuse the framework for performance monitoring configuration for AI/ML based BM for AI/ML based CSI prediction, i.e., • Support the NW configures one CSI report configuration for the inference of the AI/ML based CSI prediction, and configure another CSI report configuration for the monitoring of the AI/ML based CSI prediction which is linked to the CSI report configuration for the inference • The UE calculates the performance monitoring metric based on the inference results from the CSI report configuration for the inference and the ground-truth results from the CSI report configuration for the monitoring Proposal 3: Support UE to report the SGCS based on predicted CSI for one inference reporting and ground truth CSI • UE reports the SGCS for the layer with strongest channel energy • UE reports the SGCS for the subbands configured by the network o The maximum number of subbands is a UE capability • UE reports the SGCS for one instance configured by the network Proposal 4: Support to report the SGCS by UCI Proposal 5: UE does not need to receive the CSI-RS for SGCS report during the cell DTX inactive period, since the CSI-RS is also a type of CSI-RS for CSI acquisition. |
| Lenovo | Proposal 4 Evaluate the specification impact corresponding to AI/ML model monitoring, considering the following monitoring decisions: (i) No model change, (ii) CSI parameters update, (iii) Model switching, and (iv) Fallback to non-AI/ML scheme. Observation 1 Type 3 monitoring implies that the processing of the performance metric for a model adaptation decision is pursued at the network side, whereas for Type 1 monitoring the UE computes an auxiliary recommendation based on the measured performance metric to assist the network with the model adaptation decision. Observation 2 The SGCS value corresponding to the monitoring process is expected to drop significantly over the reporting window period. Proposal 5 For SGCS reporting for monitoring purposes, the SGCS metric is averaged over SBs, layers, but not over reporting instances corresponding to the prediction window. Proposal 6 For performance monitoring under UE-based CSI prediction, three reference time instants are considered: (i) the first slot of the prediction window, (ii) the median slot of the prediction window, and (iii) the last slot of the prediction window. Proposal 7 For SGCS reporting, support a single reference for performance monitoring corresponding to realistic CSI according to the latest measurement occasion. Proposal 8 SGCS is reported over a region of valid values, e.g., [0.75,0.95], with an additional codepoint corresponding to values lower than the smallest value in the region. • FFS: whether the values are quantized uniformly in linear or log domain. |
| Sony | Observation 1: The gNB can use monitoring reports from all prediction instances within the prediction window to select an appropriate time resource allocation for transmissions to the UE or to generate statistics on the monitoring metric. Proposal 1: RAN1 to allow raw monitoring reports per prediction instance. What to do with these should be up to the network. Observation 2: Alt1 as described does not preclude one SGCS for each CSI prediction instance. Observation 3: The ground truth at a given prediction instance will always provide the best possible CSI for that instance Proposal 2: RAN1 selects Alt1 with an SGCS report per prediction instance in the monitoring period and leave the statistical computation in Alt3 to network implementation. Observation 5: Measurements of ground-truth for performance monitoring are likely less frequently needed than measurements for model input in inference. Proposal 4: RAN1 should allow separate configuration of CSI-RS resources for model input at inference and model monitoring ground-truths. Proposal 5: RAN1 should allow the use of aperiodic CSI-RS for use in performance monitoring of CSI prediction using UE-side model. |
| Fujitsu | Proposal 3: • For AI/ML based CSI prediction with UE-side model, regarding the reporting content for performance monitoring, Alt 1 is preferred, i.e., - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI Proposal 4: • For AI/ML based CSI prediction with UE-side model, regarding the reporting content for performance monitoring, the SGCS could be averaged over prediction instances or the SGCS could be reported for one selected prediction instance. Proposal 5: • For AI/ML based CSI prediction with UE-side model, regarding the frequency granularity of the reporting content for performance monitoring, the SGCS could be averaged over sub-bands, or the SGCS could be averaged over selected sub-bands. Proposal 6: • For AI/ML based CSI prediction with UE-side model, regarding the reporting content for performance monitoring, the SGCS could be averaged over layers or the SGCS could be reported for the first layer. Proposal 7: • For AI/ML based CSI prediction with UE-side model, periodic CSI-RS and semi-persistent CSI-RS could be supported for performance monitoring. There is no need to introduce dedicated resource configuration for performance monitoring, i.e., the resource configuration for inference could be re-used. Proposal 8: • For AI/ML based CSI prediction with UE-side model, CSI-ReportConfig could be used to configure the reporting for performance monitoring. New report quantity could be introduced for performance metric reporting. |
| HONOR | Proposal 2: Use the Rel-16 eType II codebook to represent PMI of ground-truth CSI for performance monitoring. Proposal 3: To mitigate the impact of quantization loss, PMI of the ground-truth CSI should have a higher quantization resolution (e.g., using default parameter combination). Proposal 4: Codebook parameters should be aligned between monitoring report and inference report (e.g., subband number, antenna port number, layer number). Proposal 5: For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, support Alt1 while deprioritizing Alt2 and Alt3. Observation 1: Reporting per subband will significantly increase signalling overhead and computational complexity. Proposal 6: At least support reporting one prediction instance averaging over subbands and layers, wherein the prediction instance can be configured by network or determined by UE by default. Proposal 7: Support reporting per layer averaging over subbands which can be configured by the network subject to additional UE capability. Proposal 8: The monitoring window can be configured by network to increase the reliability of monitoring report. Proposal 9: Support L1 signalling for reporting monitoring results. Proposal 10: Introduce time restriction for triggering monitoring reports. e.g., when an inference report is triggered, and if the report is linked to a monitoring report, the monitoring report should be triggered before the inference report is transmitted. |
| Nokia | Observation 1: Alt 2) seems to be significantly better suited to evaluate the monitoring performance then Alt 1) which suffers from the strong variation of median SGCS values ranging from 0.1 to 0.9 for different UEs like cell center and cell edge UEs. This makes it difficult to define a suitable single threshold value to decide the AI/ML model performance as needed for Alt 1). Contrary, for Alt 2) the strong correlation of the SGCS statistics for non-predicted CSI and AI/ML predicted CSI allows to differentiate effects of the radio channel conditions versus failures of the AI/ML model itself. Observation 2: Alt 2) can be realized by reporting of delta SGCS values so that the UE reports the difference of the SGCS values of the non-predicted and the AI/ML predictor. This combines the benefits of reporting relative to a reference predictor while the reporting overhead is the same as for Alt 1). Observation 3: For Alt 2) due to the lower value range of delta SGCS values, the number of bits per report might be reduced compared to Alt 1) for a given quantization error. Proposal 2: For reporting of performance monitoring content we propose Alt 2), i.e., reporting of SGCS values for predicted CSI as well as non-predicted CSI, preferably as delta values between the predicted and non-predicted SGCS values. Proposal 3: Consider a non-linear quantization of SGCS values within a limited reporting window, e.g. ranging from 0.5 to 0.9 and including an ‘out-of-range’ indication. Proposal 4: UE should report median SGCS values in combination with the related reliability, where the reliability might be, for example, estimated from the variance of the SGCS values as well as other means like the SINR. Proposal 5: If the UE calculates the 〖SGCS〗_i value of one inference reporting instance over multiple layers l and multiple prediction time instances n_4 then it should report the weighted sum over all layers and prediction time instances. Proposal 6: As advanced and low overhead mode consider reporting SGCS values over multiple reporting instances I ≫ 1 with inherent error correction e_(i-1) so that one can achieve simultaneously a low bit quantization in combination with a very low estimation error e_(I=10). Proposal 7: Consider as another optimization a codebook approach for reporting of relative domain specific SGCS reliabilities. Proposal 8: For performance monitoring, consider configuration of monitoring CSI-RS resources independent from configuration of inference CSI-RS resources (e.g., configured for the observation window). Proposal 9: For the performance monitoring configure the CSI-ReportConfig with the higher layer parameter reportQuantity set to 'SGCS-r19' to generate the related pre-defined monitoring reports. Observation 4: For performance monitoring schemes leading to few number of bits per inference report instance one should consider L1 reporting in combination with a low priority. In case of more complex schemes leading to a significantly higher number of bits per report one should consider L2 reporting. Observation 5: Requirements for performance monitoring might vary depending on channel conditions, ML model types, energy efficiency requirements, etc. so that the gNB should be able to configure the performance monitoring reporting scheme. It might combine different modes like event driven, gNB triggered and/or periodic reporting with varying periodicities. |
| ETRI | Proposal 4: For CSI prediction using UE-sided model, for reporting contents of UE assisted performance monitoring, support Alt 1, that UE only reports one SGCS calculated based on predicted CSI for one inference reporting, ground truth CSI. Poposal 5: For reporting contents of UE assisted performance monitoring, UE reports the SGCS value averaged across all layers, prediction instances, and subbands, in order to reduce uplink reporting overhead. Proposal 6: For the quantization of the reported SGCS, the network can configure the number of uniformly divided range intervals over [0, 1] – ‘N’. Proposal 7: For the reporting contents of UE assisted performance monitoring, UE reports the index of the configured interval that corresponds to the calculated SGCS value. Proposal 8: Introduce a new quantity for SGCS in the CSI report to support performance monitoring of CSI prediction. Proposal 9: For the reporting contents of UE assisted performance monitoring, the size of the CSI field associated with SGCS reporting is ⌈〖log〗_2⁡N ⌉, where N is the number of uniformly divided range intervals over [0, 1], as configured in ‘CSI-ReportConfig’ |
| OPPO | Proposal 4: Regarding the reporting content of UE assisted performance monitoring, only support Alt 1. Proposal 5: Regarding the reporting content, support one SGCS reporting calculated by the average over all sub-bands, the first R layers and N4 prediction instances, where R is the rank indicator within the inference reporting, N4 is configured by RRC. Proposal 6: Support dedicated CSI report configuration for monitoring, with association of CSI report configuration of inference. Proposal 7: Regarding the monitoring CSI-RS type,  P/SP/AP CSI-RS type are supported for P CSI-RS type for inference  SP/AP CSI-RS type are supported for SP CSI-RS type for inference  AP CSI-RS type is supported for AP CSI-RS type for inference |
| MediaTek | Proposal 4. Deprioritize Alt 1 for the monitoring report content since a single SGCS value is meaningless. Proposal 5. Deprioritize Alt 3 for the monitoring report content to avoid complicated configurations and potential divergence in the limited discussion time. Proposal 6. Propose only Alt 2 for the monitoring report content. Proposal 7. For Alt 2, an extra bit is needed to differentiate between the SGCS values corresponding to predicted or non-predicted CSI values. Proposal 8. Report per prediction instance if the N4>1, to ensure accurate monitoring across predicted instances. Proposal 9. Reporting frequency granularity is unnecessary due to effective generalization over single/joint RB(s). Proposal 10. For layer-common models, report SGCS averaged over layers; for layer-specific models, report SGCS per layer. To simplify, reporting SGCS per layer for all cases is recommended. |
| Apple | Proposal 9: For performance metric, support Alt 3: statistics of SGCS within a configured window. Proposal 10: For performance metric reporting, support L3 based reporting using UAI, as it is a simple and low overhead approach. Proposal 11: For performance metric reporting, do not support L1 based reporting due to • Performance monitoring is not delay sensitive. • L1-based report introduces additional UE complexity • L1-based reporting requires additional specification work for report configuration, CPU occupancy, processing timelines, and prioritization between performance monitoring report and inference report/non-AI based CSI reports. Proposal 12: For SGCS quantization, use uniform quantization between [x 1] with step size of 0.01, where x is specified (e.g., 0.5). |
| Qualcomm | Observation 12: The objective of performance monitoring is to identify data drift, it will be hard to make any monitoring decision unless prediction accuracy is observed over enough number of samples to get a reliable statistic. Monitoring latency could be tens or even hundreds of milliseconds. Observation 13: Single SGCS value averaged across subbands, layers, prediction instances is sufficient as the PMI representation of precoders will spread out the prediction error in frequency, spatial and temporal domain. Proposal 11: Support reporting SGCS averaged within a configured monitoring window (Alt3). • The monitoring window size can be defined by N reporting instances in the past where N is reported as UE capability. • The reported SGCS value is averaged across subbands, layers and prediction instances. Observation 14: UE assisted monitoring is not a self-contained CSI process, it requires the UE to buffer previous inference results. The buffering cost may scale with the number of CCs. Observation 15: The buffering burden can be relaxed if L3 signaling and mechanism is used for performance monitoring. Observation 16: L3 signaling and mechanism is sufficient for detecting performance drift as the detection needs reliable measurement across multiple occasions. Observation 17: L3 signaling and mechanism has much less specification effort than L1 as there is no need to define L1 timeline, CPU counting, or active resource / port counting rule for performance monitoring. Observation 18: UE assisted monitoring can reuse the NW side data collection framework being discussed in RAN2. Proposal 12: Support L3 signalling or mechanism for performance monitoring report, e.g., via NW side data collection procedure being defined in RAN2. Observation 19: Reporting SGCS statistics and separate configuration / activation of monitoring resource for target CSI and the monitoring report may be helpful in addressing the buffering issue as it allows more flexible UE implementation using L1-related hardware when capable or even using low-cost hardware. Observation 20: L1-signalling for report SGCS statistics based on target CSI measured using periodic or semi-persistent CSI-RS can be considered to address the buffering issue. Observation 21: If L1 report is supported, especially for aperiodic report based on aperiodic CSI-RS, specification enhancements are needed to either eliminate the buffering, or allowing UE to know whether buffering is needed as early as possible. Proposal 13: If L1 report is supported for monitoring, support one of the following to address the buffering issue: • Option 0: separate configuration / activation of measurement resource for target CSI (e.g., aperiodic CSI-RS) and monitoring report. • Option 1: Configure paired resources in monitoring report, one is for model input measurement and another is for ground-truth (target CSI) measurement • Option 2.1: Configure one measurement resource for target CSI only, but the triggering of monitoring report should be close to the inference report triggering o e.g., next available downlink slot, or no later than the measurement resource of the model input • Option 2.2: define a timeline between monitoring trigger and SGCS report to include both the measurement resource for ground-truth and the measurement resource for model input |
| Sharp | Proposal 2: For CSI prediction using the UE-side model, for performance monitoring, measured precoding matrix is represented based on the TypeII-Doppler-r18 codebook. Proposal 3: For CSI prediction using the UE-side model, for reporting contents of UE assisted performance monitoring, support Alt 1 and report per prediction instance. Proposal 4: For CSI prediction using UE-side model, support to reuse CSI framework for the configuration for monitoring result report in L1 signalling:  Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring  The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration  UE measures the dedicated resource set(s) for monitoring. |
| NTT Docomo | Proposal 4 - Support a dedicated CSI report for performance monitoring. Proposal 5 - Support the reporting content Alt. 3 (Statistics of reporting contents of inference reporting instances within the configured monitoring window).  The Monitoring window is configured in the dedicated CSI report.  Mean SGCS is supported. Proposal 6 - Support the reporting per selected (first) layers and/or subbands.  The reported layers/subbands can be specified (e.g., the first one) or configured in the dedicated CSI report. Proposal 7 - If a few bits (e.g., 1-2 bits) are reported for each SGCS value, support the quantization levels configured from the NW to report the performance monitoring results. |
| CEWiT | Proposal-1: For CSI prediction using UE sided model, for reporting contents of UE assisted performance monitoring, support option Alt-1. Proposal-2: For CSI prediction using UE sided model, for reporting contents of UE assisted performance monitoring, deprioritize Alt-2. |

- Issue 2.5-1. Reporting contents

In the last meeting, regarding reporting contents, following was agreed.

Agreement For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121. - Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI - Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance. - Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window - Monitoring window can be configured by NW - FFS on signalling details - FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances - FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband) - FFS on whether to report per layer, the first layer, averaged over layer - FFS on how to quantize and report mechanism

As shown above, we need to down-select one alternative in this meeting. Alt 1 is a simple and lower complexity solution by reporting SGCS calculated from predicted CSI and ground-truth CSI. Alt 2 reports two SGCS values, one is the same value as in Alt 1 and another one is SGCS calculated from sample and hold approach. For this approach, InterDigital and Nokia provides their evaluation results, and from the results, it can be observed that there is non-negligible probability that the value of  SGCS of sample and hold can be larger than the value of SGCS obtained from predicated CSI, meaning that using sample and hold is beneficial than using AI/ML predictor. Compared to Alt 1, it requires more SGCS calculation which increases computational complexity. Alt 3 is reporting statistical SGCS value and it requires some window configuration. For this approach, some companies think that Alt1 is less beneficial for NW to determine functionality fallback while some proponents of Alt 1 think oppositely.

Based on the Tdoc review, I summarized the companies’ position as below:

|  | Supporting companies |
| --- | --- |
| Alt 1 | Ericsson, HW, VIVO, TCL, ZTE, CATT, LG, CMCC, Xiaomi, NEC, Ruijie Networks, Panasonic, Lenovo, Sony, Fujitsu, HONOR, ETRI, OPPO, Sharp, CEWiT |
| Alt 2 | Samsung, IDC, Panasonic, Nokia, MediaTek |
| Alt 3 | Spreadtrum, NEC, NVIDIA, Apple, Qualcomm, NTT Docomo |

Given above, I will propose to down-select to Alt 1 which is majority companies’ support.

- Proposal 2.5-1A.

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select to Alt 1.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

Please provide your input/suggestion on Proposal 2.5-1.

| Company | Views |
| --- | --- |
| Qualcomm | We believe Alt3 is the right way to go, NW cannot make the decision based on the SGCS of one sample. Another possibility is to merging Alt1 and Alt3 by configuring number of inference reporting instances to be considered in the monitoring report. The number of inference reporting instances is also subject to UE capability. |
| vivo | Support |
| Samsung | Alt2 would be useful for the network to make LCM decisions, e.g., fallback. We also sympathize with opponents of Alt2 that it may increase UE’s and specification/configuration complexity. As an example, in hindsight, the UE might be required to determine three PMIs, i.e., predicted CSI, baseline CSI, and ground truth CSI. It may also mean CB and other configurations have to be defined for all the three CSIs. In order to address the concern from opponents, we suggest to restrict Alt2 for the case wherein the earliest prediction slot interval (corresponding to n4=0) to be the one that contains the latest CMR for inference reporting instance. This can be achieved with the Rel-18 configurations by NW’s configuration/implementation with predictionDelay=-nref and transmission of the CMR in the n-nref slot (for CSI report in slot n). Or better flexibility can be achieved by defining a new value for predictionDelay, e.g., m1, where the value indicates the first slot corresponding to n4=0 is the slot that contains the latest CMR for the inference prediction instance. Base on the above configuration, the baseline CSI and the earliest predicted CSI would be the same. UE needs to determine two PMIs (as opposed to three) and report N4-1 SGCS values per layer for the first and second set of SGCS values. Moreover, two codebook configurations are sufficient. |
| Huawei, HiSilicon | Support. Alt.1 has relatively lower complexity than Alt.2 and Alt.3, as Alt.2 needs UE to calculate two SGCS values, and Alt.3 needs UE to buffer a number of SGCS values for a long time to generate the statistic result. On the other hand, Alt.1 can work be a shot reporting manner where each monitoring CSI is reported for its corresponding inference CSI report. |
| OPPO | Support. |
| ETRI | Support |
| Fujitsu | Generally fine with the proposal. |
| CMCC | Support. |
| Xiaomi | Support |
| NTT DOCOMO | We don’t support Alt. 1 because the per-sample SGCS is reflected by channel random small-scale variations, which does not carry meaningful information that can reflect the AI/ML model performance. Meanwhile, it increases the UL report occasions, which is a burden to the NW. To address the RI changing issue during the averaging window, the SGCS report for the selected layer or the SGCS report averaging across layers can be considered. Finally, Alt. 1 can be a special case of Alt. 3. Even if the NW really wants to know the per-sample SGCS, it can configure the averaging window to be 1. |
| Sharp | Support. In this stage of the WI, we prefer a simple way. |
| Sony | Support. Alt1 reports from multiple instances can be used by network to derive the statistics engendered in Alt3 thereby avoiding specification and UE complexity. |
| LGE | Support |
| HONOR | Support |
| Apple | Option 3 should be the simple and easy way to go. Considering this is the last meeting, using L1 reporting create much more specification work as shown in earlier proposals. For L3 report format of average SGCS metric, it can be up to RAN2 design. |
| Ericsson | Support |
| CATT | Support. |
| TCL | Support |
| CEWiT | Support |
| SPRD | Support |

Reporting granularity

For each alternative, the remaining issue is report granularity.  In the following, I summarized companies’ preference.

- For time domain (e.g., prediction instances)

  - Per instance (11): Ericsson, Huawei, vivo, TCL, CMCC, Xioami, NEC, Lenovo, MediaTek, Samsung, Quectel

  - Per selected instance (8) : ZTE, CATT, Xiaomi, Google, Lenovo, Fujitsu, HONOR, LG

    - Pre-determined instance, e.g., 1st and last instance

    - By NW configuration

  - Average over prediction instance (6): Ruijie Networks, Fujitsu, Nokia, ETRI, OPPO, Qualcomm

- For spatial domain (e.g., layers)

  - Per layer (12): Ericsson, Huawei, vivo, TCL, CMCC, Xiaomi, NEC, Nokia(N4=1), MediaTek, Inter Digital, Samsung, Quectel

  - 1st layer (5): ZTE, Google, Fujitsu, NTT Docomo, LG

  - Averaged (8): Ruijie Networks, Lenovo, Fujitsu, HONOR, Nokia(N4>1), ETRI, OPPO, Qualcomm

  - For options to determine # of rank,

    - Explicit configuration by NW

    - Implicitly determined by associated inference report

- For frequency domain (e.g., subbands)

  - Averaged: Ericsson, Huawei, TCL, CMCC, CATT, Ruijie Networks, Lenovo, Fujistu, HONOR, ETRI, OPPO, QC, NTT, LG, Google, Samsung, Quectel

  - Wideband: vivo, ZTE, Xiaomi

As shown above, regarding, prediction instance and layer, there are diverged views, meanwhile regarding frequency domain granularity, it seems no one wants option of per SBs. Between averaging and wideband, in my understanding, wideband SGCS means it is calculated from WB channel information, while the other option  calculates SGCS per SB first and then averages them out. For time domain and spatial domain, I could understand the rationale of each proponent, and each sub-option is workable, it is just matter of performance and overhead trade-off. Therefore, I made the proposal below based on the majority supports. Also, we need new RRC parameter regarding SGCS report.

- (updated) Proposal 2.5-1B.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- SGCS is reported

  - per monitored prediction instance, and the total # of monitored prediction instance is (1 or N4)

  - per layer where the total number of layers is (the same as the reported RI in the associated inference report or 1, first layer)

  - wideband frequency granularity

- Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’

- (new) Qustion 2.5-1B.

For prediction instance, can we reuse framework of BM case2?

Working Assumption For BM-Case 2, at least support to report one beam prediction accuracy for one configured time instance, configured by one CSI-ReportConfig for monitoring, - only one resource set is configured in the CSI-ReportConfig - the one configured time instance (i.e. f-th time instance of the time instance in one inference report) for metric calculation is configured in the CSI-ReportConfig for monitoring - FFS on whether to configure more than one time instance - the performance metric of the f-th time instance is calculated based on N latest transmission occasion(s) of monitoring resource with linked time instance, no later than CSI reference resource corresponding to the CSI report for monitoring - N (N>=1) is configured in the CSI-ReportConfig - FFS on additional rule for counting N linked pair - measurement result of a transmission occasion of the CSI-RS/SSB resources for monitoring is linked with the f-th time instance for prediction, where the f-th time instance has the minimal slot offset to the transmission occasion of the CSI-RS/SSB resources for monitoring. - Wherein, the corresponding inference reports, and the transmission occasions of the CSI-RS/SSB resources for monitoring, [FFS on the f-th time instances] are no later than the CSI reference resource corresponding to the CSI report for monitoring - FFS: whether to introduce a threshold X, and whether it is optionally configured by RRC, where the minimal slot offset k is no larger than X; otherwise, the transmission occasion for monitoring has no linked time instance

Please provide your input/suggestion on Proposal 2.5-2 and question 2.5-1B.

| Company | Views |
| --- | --- |
| Qualcomm | We think a single SGCS value averaged across all dimension is sufficient. The reason is that we adopt R18 PMI for SGCS calculation, the prediction error is averaged across subbands, layers, prediction instances after the R18 PMI compression. |
| vivo | Regarding time instance, to reuse the mechanism in AI BM Case 2, our position is each monitoring report only report SGCS for one prediction time instance, where the reported time instance index (f-th time instance from N4 instances) is configured in monitoring report configuration. This is related with the association between inference and monitoring. In AI BM Case2, we only agree for an associated mechanism for one prediction instance in a monitoring report configuration. This should be reused in CSI prediction. For “averaged over subbands”, this is UE implementation. From spec perspective, we only need to say “wideband”. |
| Samsung | Support |
| Huawei, HiSilicon | Support. |
| OPPO | We share similar view to QC to reduce the SGCS reporting complexity. One average result is enough for monitoring . |
| ETRI | Support |
| Fujitsu | With the current formulation (“SGCS is calculated”), does it mean the SGCS will be reported per prediction instance and per layer? If so, there would be a lot of overhead for the reporting, which is not preferred. |
| CMCC | Support. |
| Xiaomi | Support |
| NTT DOCOMO | As discussions for the previous proposal, the average across all layers or selected layers should be supported. |
| Lenovo/ MotM | We are fine with this proposal. Also OK to average over layers |
| HONOR | Support. To reduce report overhead, restrict to a single prediction for monitoring is preferred. |
| Apple | At least for per layer, we strongly prefer only 1st layer. Considering RI can be different per report, and SGCS needs to be monitoring over a window, higher layers will not have stable report to average. Agree with average over subbands. |
| ZTE | Thanks feature leader for the proposal. We are generally fine with the layer and averaged subbands SGCS determination. However, we hold concerns about calculating SGCS per prediction instance. We think calculating SGCS per prediction instance causes the CSI-RS transmission over every prediction instances. This would not only cause huge CSI-RS overhead but also increases the UE complexity for determining every CSI upon the receiving of CSI-RS. Besides, the overall gain by applying AI/ML to CSI prediction is decreased because UE needs to measure the CSI at every instance which should be reliably predicted by an AI/ML model. |
| Ericsson | Support |
| CATT | Support in principle. Whether SGCS is calculated per instance or per selected instance can be configured by the gNB. |
| TCL | Support |
| CEWiT | Generallly support this proposal. For SGCS calculation, OK to average over subbands |
| SPRD | Support |
| Lenovo/ MotM | The figure below plots the SGCS of the strongest eigenvector of the channel in UMa at 3.5 GHz, which compares the SGCS of the strongest channel eigenvector at slot k, where k∈{1,2,…,50}, with that of slot 1, at UE speed of 30 km/h. The severe drop in SGCS value vs. slot index implies the expected reduction in SGCS value of prediction instances, which can span up to 16 slots for AP CSI-RS, and up to 8p slots assuming P/SP CSI-RS, where p denotes the CSI-RS periodicity value. Given that, averaging across reporting instances will severely degrade the SGCS value, which would be dominated by the SGCS values at the end of the reporting window |

SGCS quantization

Regarding of SGCS value quantization, there are some proposals from companies.

- Ericsson : SGCSlower=0.3, quantization bits =3 bits

- vivo: each layer occupy 4bits

- Samsung: [ymin,ymax], with quantization level ∆y and reporting bitwidth B

- ZTE : [0,1], 4 bit quantization

- Nokia : [0.5, 0.9], including an ‘out-of-range’ indication.

- ETRI : [0, 1], N quantization, 1/N

- Apple : [x, 1] with step size of 0.01

- LG: [x, 1] with step size of Y

Based on the above proposals, I propose the following

- Proposal 2.5-1C.

For CSI prediction using UE-side model, for UE assisted performance monitoring, support N-bit quantization of SGCS value [per layer and per monitored prediction instance]

- 2N-1 codepoints are used to uniformly quantize (x 1] in linear scale.

- One codepoint is used to indicate [0 x]

- FFS on N and x values

Please provide your input/suggestion on Proposal 2.5-3

| Company | Views |
| --- | --- |
| vivo | OK |
| Samsung | Ok |
| Huawei, HiSilicon | Good starting point but we suggest to remove the suggested values of [3, 4] for N from the proposal. |
| ETRI | Same view with HW to remove the suggested values. Moreover, we think that N can be configured by NW. |
| Fujitsu | Fine |
| CMCC | Support and the quantization step is also needed to specify. |
| NTT DOCOMO | Generally support the proposal. For x, it can be network configured since the NW can have better knowledge about the benchmark performance given a deployment area. |
| HONOR | Support. |
| Apple | OK |
| ZTE | Thanks feature leader for the proposal. We think low SGCS value could also has value for the NW side to decide the performance of the UE side model, especially considering the temporally channel varying. For example, two SGCSs with {0.1, 0.3} are indicating that the model performance is improved. While reporting of two SGCS which are smaller than a certain threshold would lose the information. Besides, we suggest leaving an open left boundary for interval [x 1]. Therefore, we suggest the following updates of the proposal. - Proposal 2.5-3. For CSI prediction using UE-side model, for UE assisted performance monitoring, support N [3 or 4]-bit quantization of SGCS value [per layer and per monitored prediction instance] - 2N-1 codepoints are used to uniformly quantize [(x 1] in linear scale. - One codepoint is used to indicate [0 x] - FFS on N and x values |
| Ericsson | Support |
| TCL | Support |
| OPPO | Generally fine. Prefer only one SGCS reporting with N= 3 or 4 bits quantization. |
| CEWiT | Generallly support this proposal. For SGCS calculation, OK to average over subbands |

- Issue 2.5-2. SGCS definition

Agreement For the definition of SGCS, Note: How to handle layer mapping mismatch, if any, is up to UE implementation

In the last meeting, SGCS definition is agreed as captured above. During the discussion, an issue of assumption of codebook for ground-truth CSI is raised. Regarding this, vivo and HONOR proposed to use Rel-16 eType II codebook.

- Question 2.5-2

- Q1: Do you think clarification is needed for the assumption of codebook for ground truth CSI? If yes, what is your preference?

Please answer Question 2.5-2.

| Company | Views |
| --- | --- |
| vivo | Yes, it has to be defined. Otherwise what SGCS means is not clear to NW. Re the codebook, it is related with proposal 2.5-1 and the association between inference and monitoring. If we have only 1 time instance in a monitoring report, the only choice we have is Rel-16 eType II codebook. |
| Samsung | Yes. It may follow the CB configuration for predicted CSI. However, clarification is needed. |
| Huawei, HiSilicon | We don’t see any need for such clarification though we are open to further discussions from proponents. |
| OPPO | It seems that use the similar definition to AI/ML reporting is enough. |
| Fujitsu | Open for discussion |
| CMCC | Yes, since we have agreed to use Rel-18 Doppler eType-II codebook for inference, it is reasonable to reuse it here, even for only one prediction instance, Rel-18 Doppler eType-II with N4=1 can be adopted. |
| Xiaomi | Yes. If the SGCS is calculated per N4 prediction instances, Rel-18 eType II Doppler codebook could be used to represent PMI used for ground truth CSI. Otherwise, Rel-16 eType II codebook is applied. |
| NTT DOCOMO | Yes, we think the alignment is necessary between NW and UE. No strong preference on the codebook choice as long as how to apply the codebook is clearly specified. |
| Sharp | Yes, we suggest to do this. Defining a codebook for the ground truth CSI allows the UE to derive the SGCS based on the same criteria. In other words, the SGCS calculated by different UEs are based on the same criteria. We recommend supporting the Rel-18 type II Doppler codebook due to its flexibility. |
| Lenovo/ MotM | Agree with Xiaomi’s comment |
| HONOR | Yes. It should be defined. Otherwise, we don’t see how RAN4 can define the test for monitoring. For flexibility, Rel-16 eType II is suggested as it only includes a prediction instance. |
| Apple | Yes. Reuse the same CB configuration for inference. |
| ZTE | Based on our understanding, the ground truth CSI should be aligned with the predicted CSI for performance metric determination. Since the predicted CSI is determined in a Rel-18 doppler codebook, we would kindly suggest to further clarify the necessity to introduce Rel-16 eType-II codebook. |
| Ericsson | Yes. The codebook for ground truth CSI shall be Rel-18 eType-II, which is the same as for inference and it covers the cases of N4=1 and N4>1. |
| CATT | Yes. The CB configuration for the linked inference report can be used. |
| Mod | This will be handled after we have some progress on issue 2.5-1 |

- Issue 2.5-3. Reporting mechanism

For reporting mechanism, several options are proposed by companies. Most popular option (supported by LG, Ericsson, Huawei, vivo, Samsung, ZTE, CATT, Xiaomi, Google, HONOR, Nokia, Sharp) is using UCI for carrying monitoring contents which is same as BM use case.

Agreement At least for the monitoring Type 1 Option 2 of UE-side model monitoring (when applicable), support to reuse CSI framework for the configuration for monitoring result report in L1 signaling: - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration - FFS how to identify the connection between RSs in the resource set(s) for monitoring and Set A beams - FFS on whether to support all the combination on time domain behavior of the reportConfigType for inference report and the reportConfigType for monitoring report - FFS on the timing related issues - UE measures the dedicated resource set(s) for monitoring.

Other option includes relying on L3 signalling, e.g., via UAI reporting, which is supported by Apple and Qualcomm. The rational of supporting this option is its simplicity, and low buffering burden at UE side and low RAN1 specification impact.  Given the situation that majority companies are supporting L1 based monitoring reporting, I propose to support L1 signaling for monitoring report.

One discussion point is whether dedicated monitoring resources are needed. For instance, some companies think dedicated resources are not needed in case of P/SP CSI-RS resources. Also, Qualcomm raised concern on UE buffering issue for performance monitoring and proposed some options to address UE buffering issue, if L1 signalling is supported. So, I want to hear more view on these issues.

- Question 2.5-3.

- Q1: If L1 signalling is used for monitoring report, do you think dedicated CSI-RS is needed? And which type of CSI-RS, e.g., P/SP/AP CSI-RS, is needed for monitoring RS?

- Q2: If L1 signalling is used for monitoring report, which type of CSI report, e.g., P/SP/AP CSI report, is needed for monitoring report?

Please provide your input on Question 2.5-3 including other issues which are not listed above.

| Company | Views |
| --- | --- |
| Qualcomm | We think the right way is to use L3 signalling as the objective is to determine data drift which cannot be done by a single sample. Thus, tens or even hundreds of millisecond would be needed to provide a solid SGCS evaluation. If L1 signaling is used, we don’t think A-CSI-RS should be considered. UE should be allowed to measure the resources, till there are sufficient measurements are gathered, then report the results separately. |
| vivo | For these two questions, we can simply reuse the conclusions from AI BM Case 2. |
| Samsung | Q1, the NZP CSI-RS with P/SP/AP time-domain property can be configured for monitoring measurement resource (MMR)- parameter name can be used to differentiate from CMR, IMR. Q2, given that Type II CSI is supported for SP/AP CSI report, monitoring report may follow these time-domain properties. |
| OPPO | Regarding these two questions, reuse the design framework agreed in BM case 2. |
| ETRI | Same view with HW. |
| Fujitsu | Q1: We think P/SP CSI-RS is sufficient for monitoring RS. Therefore, there is no need to introduce dedicated CSI-RS. The CSI-RS for inference could be reused. Q2: Open to consider all the P/SP/AP report. |
| CMCC | Q1: Yes, dedicated CSI-RS is needed, and all types of CSI-RS can be considered. Q2: All types can be considered |
| Xiaomi | Q1: For P/SP CSI-RS, monitoring RS may be not needed if the interval of adjacent P/SP CSI-RS transmission in the observation window is same as that of adjacent CSI-RS transmission in the prediction window. Otherwise, monitoring RS is needed. Q2: P/SP/AP CSI report could be supported. |
| Huawei, HiSilicon | Q1: A dedicated CSI report is needed for monitoring, which follows the same rule as AI BM. Q2: We think SP/AP CSI report types should be supported for monitoring in specification, since inference CSI report only supports SP/AP. To FL: we think the signaling for reporting the monitoring result should be prioritized for discussion than the content of the metric. We cannot accept different signaling mechanisms between AI BM (which already adopts L1 signaling) and AI CSI prediction – that means we (from NW vendor perspective) need to build separate policies for different AI CSI features, which doubles the efforts of product development. Moreover, considering CSI prediction is with dense reporting frequency for inference, the monitoring report periodicity is also small – considering a statistic metric calculation over 10 monitoring samples each with 5ms time gap means 50ms reporting periodicity, which is smaller than the RRC msg periodicity of 120ms; if we introduce denser RRC report periodicity, that will cause severe RLF problem. |
| NTT DOCOMO | For Q2, all 3 types can be supported. For P/SP reports, the report periodicity can be different from the inference since the report for monitoring is independently configured. |
| Apple | If L1 signaling is used, dedicated CSI-RS for ground truth is needed. This configuration can be reused for data collection for training as well. As other agenda mentioned, we do not specify the use case of CSI-RS configuration. Again, L3 is a simpler from spec point of view. The detailed L3 report procedure can be done in RAN2. |
| ZTE | For Q1, dedicated CSI-RS could be supported for performance monitoring. For RS types for monitoring RS, we think all P/SP/AP CSI-RS could be supported in the following manner, such that only AP CSI-RS need dedicated CSI-RS, while P/SP CSI-RS could be more flexible configured by the reporting configuration. For Q2, we support P/SP/AP CSI report with respect to the CSI-RS configuration. |
| Ericsson | For Q1: does “dedicated CSI-RS” mean a different CSI-RS comparing the CSI-RS configured for inference? For Q2: support SP/AP CSI report for monitoring, since only SP/AP CSI reporting is supported for inference. |

- (closed) Proposal 2.5-3.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

  - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, [periodic, semi-persistent, aperiodic] are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |

- UE measures the dedicated resource set(s) for monitoring

  - For monitoring resource type for measurement, [periodic, semi-persistent, aperiodic] are supported.

- UE measures the dedicated resource set(s) for monitoring

Please provide your input on Proposal 2.5-3

| Company | Views |
| --- | --- |
| CATT | OK |
| TCL | Support |
| Qualcomm | In general, we think inference remaining issues should be discussed with higher priority. And reporting mechanism should be discussed even after the monitoring metric For reporting mechanism, we believe L3 signalling is the proper way with the following reasons - The objective is to identify data drift, so gathering sufficient samples for SGCS calculation is needed. This would need tens or hundres of millisecond, so L3 signalling is sufficient. - L3 signalling is beneficial for UE implementation, as there is no need stringent timeline and the resource and report may be separately activated. UE is able to use low-cost hardware to perform the monitoring once the resource is activated, and bring the report when there is RRC request. - L1 signalling needs more specification effort than L3, we need to discuss CPU, timeline, active resource port counting for L1 mechanism, which may not be able to finish even in next meeting. For L3, we can just finalize the monitoring metric discussion and send LS to RAN2, RAN2 has existing container such as NW side data collection framework to request the SGCS report. If proceed with L1 signalling, we think the monitoring trigger should be close to the inference trigger (e.g., no later than the measurement resource of the inference report), so that the UE is aware of the whether there is a need of buffering or UE may be able to jointly process the inference and monitoring. If UE does not receive the monitoring trigger prior to the inference measurement resource, UE is not expected to be triggered with monitoring report. Also, the active resource counting and CPU of the inference report needs to be extended to the end of monitoring report if monitoring trigger is received. |
| Xiaomi | We have same question with Ericsson commented in last round. I.e., regarding dedicated resource set(s) for monitoring, does “dedicated CSI-RS” mean a different CSI-RS comparing the CSI-RS configured for inference? In our view, dedicated CSI-RS for monitoring could be same or different from the CSI-RS for inference when P/SP CSI-RS resource is configured. |
| ZTE | We share similar questions as Ericsson commented and Xiaomi. The meaning of dedicated P/SP CSI-RS should be clarified. Does the word ‘dedicated’ mean that it is implicitly bonded with the ‘dedicated’ monitoring report? |

- (new) Proposal 2.5-3B.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report, the first inference report not earlier than the monitoring report trigger

- If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report;

  - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report

Please provide your input on Proposal 2.5-3B

| Company | Views |
| --- | --- |

UE capability

- Summary

In this meeting, UE capability related proposals/observations are listed below:

| Company | Views |
| --- | --- |
| MediaTek | For AI/ML-based CSI prediction, UE needs to report the prediction capability to NW. The prediction capability can be the same format as the N_4 value defined in the Rel-18 MIMO. To distinguish between AI and non-AI capabilities, an extra bit can be added to the existing N_4 value. For AI/ML-based CSI prediction, UE needs to report the model monitoring related capabilities to NW. For example, one bit to indicate whether it supports model fine-tuning, one bit to indicate whether it supports non-AI based CSI prediction. |
| Qualcomm | Proposal 14: Support UE capability signalling for CSI-RS triplets concurrent processing of AIML-based CSI predication and other legacy CSI reports. The signalling granularity should be per-band and also per band-combination. |

In this meeting, there will be UE capability discussions for AI/ML features in A.I. 9.15.1. So, let’s discuss this issue in A.I. 9.15.1.

Please provide your input, if any.

| Company | Views |
| --- | --- |
| Apple | We strongly request to discuss the proposal of dedicated APU for CSI prediction use case, as part of UE capability of APU/CPU. |

Consistency between training and inference

- Summary

This section is for consistency issue. Although we intensively discussed in the previous meetings, we fail to achieve a consensus. As promised, this issue will not be treated in online, unless there is a consensus. Please keep that in mind, and please be flexible.

Followings are proposals/observations related to consistency issues submitted in this meeting.

| Ericsson | Observation 16 There is no need of specification enhancement to ensure consistency between model training and model inference for CSI prediction use case (UE-sided model) regarding at least the following aspects: UE speed, deployment scenario, carrier frequency, and UE distribution. Observation 17 A localized model gives in most cases only minor gain compared to a model trained on general data. Table 1 Throughput gain of AI-based prediction over baseline#1 (nearest historical) and baseline#2 (non-AI prediction) at 20% and 50% resource utilization, at 30km/h, with observation window of 5/5ms and prediction window of 4/5ms/5ms, comparing AI model trained with general dataset and localized, site-specific, dataset. 20%; 50%. Mean; 5th percentile; Mean; 5th percentile. #1; #2; #1; #2; #1; #2; #1; #2. AI, Trained with localized dataset; 8%; 6%; 21%; 18%; 15%; 12%; 26%; 15%. AI, Trained with general dataset; 7%; 5%; 17%; 15%; 11%; 7%; 15%; 4%. Observation 18 The inconsistency issue identified for the UE-sided spatial beam prediction use case (impact on UE assumptions on beams of Set A/Set B) is not appliable for the UE-side CSI prediction use case, which is for temporal CSI prediction. Table 2 Performance degradation of generalization Case 2 w.r.t. Case 1 on antenna tilt Antenna down tilt; AI model trained for; 20%; 50%. Mean; 5th percentile; Mean; 5th percentile. 10 deg; 0 deg; 2%; 4%; 0%; 1%. 0 deg; 10 deg; 0%; 1%; 0%; 0%. Observation 19 Compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain NW antenna down tilt value #B applied for inference with the same NW antenna tilt value #B • For generalization Case 2, generalized performance can be achieved for NW antenna tilt value #A o 0-4% degradation when the aspects #A is (NW antenna tilt = 0 degree) and the aspects #B is (NW antenna tilt is 10 degrees). o 0-1% degradation when the aspects #A is (NW antenna tilt = 10 degree) and the aspects #B is (NW antenna tilt is 0 degrees). Observation 20 The NW antenna down tilt value has negligible impact on the UE-side CSI prediction model design and performance. Table 3 Performance degradation of generalization Case 2 w.r.t. Case 1 on TXRU mapping Subarray size; AI model trained for; 20%; 50%. Mean; 5th percentile; Mean; 5th percentile. 1x1; 4x1; 1; -0; -1; 1. 6x1; 0; -3; -2; -3. 4x1; 1x1; 0; 0; 0; 3. 6x1; 0; 0; -1; 0. 6x1; 1x1; 0; 1; 1; 3. 4x1; 0; 1; 0; 3. Observation 21 When all cells use the same TXRU mapping, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain NW TXRU mapping #B applied for inference with the same NW TXRU mapping #B • For generalization case 2, generalized performance can be achieved for NW TXRU mapping #A o -1~1% degradation when the aspects #A is (subarray size is 4x1) and the aspects #B is (subarray size is 1x1). o -3~0% degradation when the aspects #A is (subarray size is 6x1) and the aspects #B is (subarray size is 1x1). o 0~3% degradation when the aspects #A is (subarray size is 1x1) and the aspects #B is (subarray size is 4x1). o -1~0% degradation when the aspects #A is (subarray size is 6x1) and the aspects #B is (subarray size is 4x1). o 0~3% degradation when the aspects #A is (subarray size is 1x1) and the aspects #B is (subarray size is 6x1). o 0~3% degradation when the aspects #A is (subarray size is 4x1) and the aspects #B is (subarray size is 6x1). Table 4 Performance degradation of generalization Case 2 a/b/c w.r.t. Case 1 on TXRU mapping, when different cells may have different subarray sizes. Cell group #1 deploys 1x1 subarray, Cell group #2 deploys 4x1 subarray. AI model used in cell group #1; AI model used in cell group #2; 20%; 50%. Mean; 5th percentile; Mean; 5th percentile. Case 2-a; 1x1; 1x1; 0; 0; 0; -1. Case 2-b; 4x1; 4x1; 0; 0; 0; 2. Case 2-c; 4x1; 1x1; 0; 2; 0; 4. Observation 22 When cells use different TXRU mapping (the first group of cells use 1x1 subarray, the second group of cells uses 4x1 subarray), compared to the generalization Case 1, the generalization Case 2-a/b/c introduces -1~4% degradation. Observation 23 The NW TXRU mapping has negligible impact on the UE-side CSI prediction model design and performance, no matter different cells use the same TXRU mapping or different TXRU mappings. Proposal 24 Conclude that there is no need of specification enhancement to ensure consistency between model training and model inference for CSI prediction use case (UE-sided model). |
| --- | --- |
| ZTE | Table 1. Simulation results of generalization across different down tilt angles Train; Test; SGCS. Dataset B; Dataset B; 0.9602. Dataset A; Dataset B; 0.9575. Dataset A+B; Dataset B; 0.9726. Note: 32 ports: (8,8,2,1,1,2,8), UE speed 30 km/h Dataset A = down tile 102 degrees, Dataset B = down tile 120 degrees. Table 2. Simulation results of generalization across different TXRU mappings with TXRU virtualization Train; Test; SGCS. Dataset B1; Dataset B1; 0.9362. Dataset A1; Dataset B1; 0.8785. Dataset A1+B1; Dataset B1; 0.9531. Note: down tile 102 degrees, UE speed 30 Km/h Dataset A1 = 32 ports: (8,8,2,1,1,2,8) Dataset B1 = 32 ports: (12,8,2,1,1,4,4). Table 3. Simulation results of generalization across different TXRU mappings without TXRU virtualization Train; Test; SGCS. Dataset B2; Dataset B2; 0.949. Dataset A2; Dataset B2; 0.8825. Dataset A2+B2; Dataset B2; 0.9655. Note: down tile 102 degrees, UE speed 30 Km/h Dataset A2 = 32 ports: (2,8,2,1,1,2,8) Dataset B2 = 32 ports: (4,4,2,1,1,4,4). Table 4. Simulation results of generalization across different TXRU virtualizations Train; Test; SGCS. Dataset A1; Dataset A1; 0.9541. Dataset A2; Dataset A1; 0.9484. Note: UE speed 30 Km/h Dataset A1 = 32 ports: (8,8,2,1,1,2,8) without TXRU virtualization Dataset A2 = 32 ports: (2,8,2,1,1,2,8) with TXRU virtualization. Table 5. Simulation results of generalization across different TXRU mapping with 60km/h UE speed Train; Test; SGCS. Dataset A; Dataset A; 0.7405. Dataset A; Dataset B; 0.5945. Dataset A+B; Dataset A; 0.7413. Note: UE speed 60 Km/h Dataset A = 32 ports: (2,8,2,1,1,2,8) without TXRU virtualization. Dataset B = 32 ports: (4,4,2,1,1,2,8) without TXRU virtualization.. Proposal 1: Considering RAN1#121 is the last RAN1 meeting of Rel-19, RAN1 prioritizes other essential issues in AI CSI prediction agenda instead of spending time on this controversial additional condition issue. |
| CATT | Table 1 Simulation results for generalization over various tilt angles Configuration; Performance (SGCS). Case 1: trained by #B => tested by #B; Case 2: trained by #A => tested by #B. #A: tilt angle=102° #B: tilt angle=110°; 0.951; 0.950 (-0.1%). #A: tilt angle=110° #B: tilt angle=102°; 0.939; 0.939 (-0%). Observation 1: For CSI prediction use case, the configuration of various tilt angles at network side has negligible impact on the performance. Table 2 Simulation results for generalization over various TXRU mapping Configuration; Performance (SGCS). Case 1: trained by #B => tested by #B; Case 2: trained by #A => tested by #B. #A:[2, 8, 2] #B: [4, 4, 2]; 0.971; 0.977 (+0.6%). #A:[4, 4, 2] #B: [2, 8, 2]; 0.969; 0.957 (-1.2%). Observation 2: For CSI prediction use case, the configuration of various TXRU mapping at network side has negligible impact on the performance. Proposal 20: For CSI prediction use case, conclude that there is no need to ensure consistency between training and inference regarding antenna down tilt and TXRU mapping configuration at network side. |
| Vivo | - Generalization performance case2 loss for various TXRU virtualization (20% outdoor users) . - Generalization performance case2 loss for various TXRU virtualization (100% outdoor users) - Generalization performance case3 loss for various TXRU virtualization (100% outdoor users) Observation 1: at least for 60km/h, or when the proportion of high-speed data collected is high, significant performance loss can be caused by the mismatch in terms of TXRU virtualization mapping model between training and the inference data. - When the user distribution ratio is 8:2, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case2 ranges from -5.6% to -9.4% in generalization performance case2; - When the user distribution ratio is 100% outdoor, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case2 ranges from -13.0% to -44.4%. - When the user distribution ratio is 100% outdoor, the SGCS loss of different TXRU virtualization mappings in the generalization evaluation case3 ranges from -3.7% to -14.6%. - UE average throughput los for Generalization performance case3 - UE 5% throughput los for Generalization performance case3 Observation 2: At the system-level simulation evaluation level, the generalization of the model for different TXRU virtualisation mappings has a more severe performance degradation at higher speeds, even after using the model with mixed data. - When the user distribution ratio is 100% outdoor, the UE average throughput loss of different TXRU virtualization mappings in the generalization evaluation case3 ranges from -6.3% to -27.3%. - When the user distribution ratio is 100% outdoor, the UE 5% throughput loss of different TXRU virtualization mappings in the generalization evaluation case3 ranges from -15.5% to -38.3%. Proposal 1: At least TXRU mapping is identified as one NW-side additional condition which will have significant impact on generalization performance. Proposal 2: Follow the solution based on associated ID as BM use case to address the consistency issue of training and inference for UE-side model. |
| Google | Proposal 1: Support to reuse the inference configuration procedure for AI/ML based BM for AI/ML based CSI prediction to maintain the consistency of training and inference with regard to the supported/applicable functionalities for AI/ML based CSI prediction. |
| LG Electronics | Table 2. Generalization performance over various gNB tilt angles Generalization case 1; Train/k; 102 degrees / 9; 110 degrees / 9. Test/k; 102 degrees /1; 110 degrees / 1. SGCS gain (%); 100%; 100%,. Generalization case 2; Train/k; 102 degrees / 9; 110 degrees / 9. Test/k; 110 degrees / 1; 102 degrees /1. SGCS gain (%); 98.6%; 97.4%. Observation#3: Regarding gNB antenna tilt angles, for generalization Case 2, generalization performance is achieved. Proposal #8: For CSI prediction using UE-sided model, conclude that specification support is not needed to ensure consistency between training and inference with respect to gNB antenna tilt angles. Table 3. Generalization performance over various TXRU mappings Generalization case 1; Train/k; [2, 8, 2] / 9. Test/k; [2, 8, 2] /1. SGCS gain (%); 100%. Generalization case 2; Train/k; [2, 8, 2] / 9. Test/k; [4, 4, 2] /1. SGCS gain (%); 95.8%. Observation#4: Regarding TXRU mapping, for generalization Case 2, marginal loss is observed. |
| NVIDIA | Observation 1: Inconsistency of training/inference arises when using data generated from stochastic channel models for training an AI/ML based CSI prediction model and deploying the AI/ML model for inference in the field. Observation 2: Network digital twin with ray tracing can be used to generate data for the training of the AI/ML model for CSI prediction to ensure consistency across training and inference. Proposal 2: Conclude that there is a need for consistency of training/inference in AI/ML-based CSI prediction. Proposal 3: Specify solutions to ensure consistency of training/inference for AI/ML-based CSI prediction. |
| Panasonic | Proposal 11: RAN1 should conclude whether NW-side additional condition is necessary or not for ensuring the consistency between training and inference based on the observations and/or evaluation results considering the aspects agreed in RAN1#118bis. Proposal 12: For ensuring the consistency between training and inference of CSI prediction, the discussion / conclusion in beam prediction can be reused. Proposal 13: For UE-side data collection, in order to identify the scenario / configuration, how to share the NW-side additional condition should be studied. Proposal 14: Instead of informing actual configuration, some kind of configuration ID corresponding to associated ID is necessary. Proposal 15: Data-transfer-based method as not to inform NW-side additional condition at the time of the training should be studied. Proposal 16: RAN1 is not required to discuss how UE collected data is transferred to UE side (server). |
| Samsung | Figure 1 Gain of AI/ML-based CSI prediction over no prediction (sample-and-hold) for generic and sector-specific models (UE speed 30kmphr). Observation#1: Site-specific AI/ML prediction model trained based on dataset collected from a single drop on a single sector with spatial consistency turned on and a large number of UE per drop shows better performance (SGCS) as compared to generic model. Table 3 Performance impact of TXRU mapping for AI/ML based CSI prediction on 60Kmhr with measurement and prediction at 1ms/1ms/5 Case 1: Model trained on Setting#2and tested on Setting#2; Case 2: Model trained on Setting#1 and tested on Setting#2. (10kmph); (10kmph). n (x5ms); Sample and hold (SGCS); AI Prediction SGCS (A); Gain %; n (x5ms); Sample and hold (SGCS); AI Prediction SGCS (B); % gain from Case 1. (B-A)/A. Gain %. 1; 0.9337; 0.9824; 5.22%; 1; 0.9337; 0.9674; 3.61%; -1.53%. 2; 0.8132; 0.9396; 15.54%; 2; 0.8132; 0.9231; 13.51%; -1.76%. 3; 0.7088; 0.8682; 22.49%; 3; 0.7088; 0.8224; 16.03%; -5.28%. 4; 0.6317; 0.6969; 10.32%; 4; 0.6317; 0.6568; 3.97%; -5.75%. 5; 0.5638; 0.6074; 7.73%; 5; 0.5638; 0.5773; 2.39%; -4.96%. Table 4 Performance impact of TXRU mapping for AI/ML based CSI prediction on 60Kmhr with measurement and prediction at 1ms/1ms/5 Case 1: Model trained on Setting#2 and tested on Setting#2; Case 2: Model trained on Setting#1 and tested on Setting#2. (60kmph); (60kmph). n (x1ms); Sample and hold (SGCS); AI Prediction SGCS (A); Gain %; n (x5ms); Sample and hold (SGCS); AI Prediction SGCS (B); % gain from Case 1. (B-A)/A. Gain %. 1; 0.9144; 0.9769; 6.84%; 1; 0.9144; 0.9742; 6.54%; -0.28%. 2; 0.7777; 0.9212; 18.45%; 2; 0.7777; 0.9085; 16.82%; -1.38%. 3; 0.6785; 0.7886; 16.23%; 3; 0.6785; 0.7525; 10.91%; -4.58%. 4; 0.6032; 0.6894; 14.29%; 4; 0.6032; 0.6151; 1.97%; -10.78%. 5; 0.5388; 0.5588; 3.71%; 5; 0.5388; 0.5062; -6.05%; -9.41%. Table 5 Performance impact of TXRU mapping for AI/ML based CSI prediction on 60Kmhr with measurement and prediction at 1ms/1ms/5 Case 1: Model trained on Setting#1 and tested on Setting#1; Case 2: Model trained on Setting#2 and tested on Setting#1. (60kmph); (60kmph). n (x1ms); Sample and hold (SGCS); AI Prediction SGCS (A); Gain %; n (x5ms); Sample and hold (SGCS); AI Prediction SGCS (B); % gain from Case 1. (B-A)/A. Gain %. 1; 0.9099; 0.9715; 6.77%; 1; 0.9099; 0.9527; 4.70%; -1.94%. 2; 0.7671; 0.906; 18.11%; 2; 0.7671; 0.8775; 14.39%; -3.15%. 3; 0.6613; 0.7461; 12.82%; 3; 0.6613; 0.6886; 4.13%; -7.71%. 4; 0.5782; 0.6102; 5.53%; 4; 0.5782; 0.5907; 2.16%; -3.20%. 5; 0.509; 0.4982; -2.12%; 5; 0.509; 0.474; -6.88%; -4.86%. Observation#2: For performance evaluation of UE-side CSI prediction with respect to data distribution mismatch that may arise from NW-side additional conditions, considering TRP antenna settings with different TXRU mapping, e.g., Setting#1: [N1, N2, P] = [2, 8, 2] with virtualization (2x8x2) antenna elements and Setting#2: [N1, N2, P] = [2, 8, 2] with virtualization (12x8x2) antenna elements - Case1: When an AI/ML model is trained and tested on dataset from a single TRP antenna setting, a similar level of performance gain (5%-22%) over Baseline#1 (sample-and-hold) is observed irrespective of the antenna settings. - Case 2: When an AI/ML model is trained on dataset from one TRP antenna setting and tested on dataset from a different TRP antenna setting, a lower performance gain over Baseline#1 (sample-and-hold) is observed and a significant performance loss of up to 10.8% is observed as compared to a model trained on a single antenna setting. Proposal#1: In CSI prediction use case using UE-sided model, consider implicit indication of TRP related aspects for network-side additional condition indication. |
| ETRI | Observation 1: Ensuring the consistency between training and inference of UE-sided AI/ML model is essential for preventing non-negligible performance degradation. Proposal 1: For consistency of training/inference of UE sided model in CSI prediction, support associated ID. Proposal 2: In CSI prediction, UE may assume the same NW-side additional conditions of DL RS associated with the same associated ID. Proposal 3: For consistency of UE sided model in CSI prediction, RAN1 to study the method for configuration of associated ID. - The associated ID can be configured through the CSI report configuration or CSI resource configuration from a higher layer, with the UE assuming that the configured CSI-RS within the configuration shares the same associated ID. |
| MediaTek | Proposal 1. For consistency between training and inference for CSI prediction using UE-sided model, consider the use of the associated ID as the baseline. Proposal 2. Reuse the existing discussion and agreements on the beam management agenda for the associated ID framework and procedures as much as possible to reduce the discussion required for CSI prediction. Proposal 3. Consider performance monitoring-based methods as an alternative approach, allowing the UE to dynamically select and switch between multiple models based on real-time performance monitoring to identify the best model for CSI prediction. |
| Apple | Proposal 13: To identify the NW-side additional conditions, generalization case 2 can be evaluated. If performance degradation is observed in generalization case 2, consistency signaling is needed. Table I: SGCS gain over sample and hold benchmark Generalization case 1; Generalization case 2. SGCS at layer 1; 52%; -33%. Observation 1: When spatial domain correlation is used in UE side model, prediction performance is sensitive to antenna virtualization. Proposal 14: Define assisted information/identifier to abstract the antenna virtualization/mTRP to ensure consistency of training and inference. |
| TCL | Proposal 17 : No specification enhancements are needed to ensure consistency between model training and model inference. |

Based on the submitted results for several previous meeting and this meeting, I summarized the results as below.

Observation 3-1 (Tilt angle)

For CSI prediction using UE-sided model, for consistency between training and inference regarding tilt angle at NW-side, till the RAN1#120bis meeting, compared to generalization case 1

- For generalization case 2,

  - in terms of UPT,

    - 1 source [Ericsson] observes

      - 0% ~2% performance degradation in terms of mean UPT

      - 0% ~4% performance degradation in terms of 5% UE UPT

  - in terms of SGCS,

5 sources [ZTE, CATT, InterDigital, LG Electronics, MediaTek] observe 0~2.6% performance degradation,

  - in terms of NMSE,

1 source [MediaTek] observes 1.4 ~ 2.2dB performance degradation,

- For generalization case 3,

  - in terms of SGCS

    - 3 sources [Huawei, ZTE, MediaTek] observe 0% ~3% performance degradation

  - in terms of NMSE

    - 1 source [MediaTek] observes 0.2~0.9dB performance degradation,

Observation 3-2 (TXRU mapping)

For CSI prediction using UE-sided model, for consistency between training and inference regarding TXRU mapping at NW-side, till the RAN1#120bis meeting, compared to generalization case 1

- For generalization case 2,

  - For 10km/h, in terms of SGCS,

    - 1 source [Samsung] observes 1.5%~5.8% performance degradation,

  - For 30km/h

    - in terms of SGCS,

      - 4 sources [CATT, LG Electronics, Xiaomi, MediaTek] observe 0%~4.2% performance degradation,

      - 1 source [ZTE] observes 6~7% performance degradation,

      - 1 source [Nokia] observes -8.7~4.8% performance degradation,

      - 1 source [Qualcomm] observes 0~7.5% performance degradation with consideration of different gNB tilt and TXRU mapping,

    - in terms of UPT

      - 1 source [Ericsson] observes -2% ~1% loss in terms of mean UPT, and -3% ~3% loss in terms of 5% UE UPT

    - in terms of NMSE

      - 2 sources [Xiaomi, MediaTek] observe 0~1.3dB performance degradation,

  - For 60km/h,

    - in terms of SGCS

      - 2 sources [Xiaomi, Fujitsu] observe 4.1%~6.4% performance degradation,

      - 1 source [Qualcomm] observes 1.6%~4.6% performance degradation with consideration of different gNB tilt and TXRU mapping,

      - 1 source [Samsung] observes 0.3~10.8% performance degradation,

      - 1 source [ZTE] observes 19.7% performance degradation,

      - 1 source [vivo] observes 13%~41% performance degradation,

    - in terms of NMSE,

      - 2 sources [MediaTek, Xiaomi] observe 0.1~1.5 dB performance degradation,

- For generalization case 3,

  - For 30km/h,

    - in terms of SGCS,

      - 4 sources [ZTE, Huawei, Xiaomi, MediaTek] observe -1%~1.8% performance degradation,

    - in terms of NMSE,

      - 2 sources [MediaTek, Xiaomi] observe 0~0.6 dB performance degradation,

  - For 60km/h

    - in terms of SGCS,

      - 2 sources [ZTE, Xiaomi] observe 0~-2% performance degradation,

      - 1 source [vivo] observes 3~15% performance degradation,

    - in terms of UPT,

      - 1 source [vivo] observe 6% ~27% performance degradation in terms of mean UPT, and 15% ~38% performance degradation in terms of 5% UE UPT

    - in terms of NMSE,

      - 1 source [Xiaomi] observes 0~0.6 dB performance degradation,

I assume that the position of company seem to be the same as in the last meeting as captured below.

| TXRU mapping and/or tilt angle causes performance degradation (i.e., non-negligible performance impact). Therefore, it is necessary to introduce associated id. | Supporting companies |
| --- | --- |
| Yes | 15 Companies: Spreadtrum/ UNISOC, vivo, Tejas Networks, NVIDIA, Xiaomi, Apple, Panasonic, ETRI, Fujistu, MediaTek, AT&T, Qualcomm, Samsung, NTT Docomo, Google, |
| No | 10 Companies: Huawei/HiSi, CATT, CMCC, OPPO, InterDigital, Nokia, Ericsson, LG Electronics, ZTE, Lenovo/MotM, TCL |

Please let me know if your position has changed, and please provide your view, if any.

| Company | Views |
| --- | --- |

Others

Please provide your input, if there are any essential issues that are not handled above.

| Company | Views |
| --- | --- |

Summary and proposals for offline session

Discussions for Monday.

Offline working assumption

For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’.

- Conclusion 2.2-1.

For CSI prediction using UE-side model, for data collection for training, a single configuration of CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI is supported.

- (updated) Proposal 2.4-1.

For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook,

- For PU occupancy, support

  - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied,

    - OAPU= 0 or N is reported by UE

    - OCPU=0 or M is reported by UE

    - Note: OAPU= 0 and OCPU=0 is not allowed

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied.

- For occupancy duration of CPU and APU,

  - the same occupancy duration is used if both CPU and APU are reported non-zero value,

  - reuse following legacy occupancy duration at least if monitoring report is not configured

    - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP =[{1, 2, 4}] is indicated by UE capability.

    - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability

- The max number of AI/ML PU for CSI prediction can be reported by UE capability optionally.

  - If max number of AI/ML PU for CSI prediction is reported, UE does not expect to be configured/triggered with more than the max number of AI/ML PU for CSI prediction.

  - Note: No additional priority rule is introduced.

- Proposal 2.5-3.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

  - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, [periodic, semi-persistent, aperiodic] are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |

- UE measures the dedicated resource set(s) for monitoring

  - For monitoring resource type for measurement, [periodic, semi-persistent, aperiodic] are supported.

- UE measures the dedicated resource set(s) for monitoring

Discussions for Wednesday

- (new) Proposal 2.4-2.

For CSI prediction using UE-side model, regarding Z/Z’ for inference

- For N4=1,

  - For Z value, Z2+t+w

  - For Z’ value, Z’2'+t

- For N4>1,

  - For Z value, according to UE capability

    - Z2+nt + w+ Z’2, or

      - FFS n=1 or 2

    - Z2+t+w

  - For Z’ value, according to UE capability

    - 2Z’2'+nt, or

      - FFS n=1 or 2

    - Z’2’+t

- UE may report the value of t per SCS

  - Detailed value of t can be discussed in UE feature

- Note: (Z2, Z’2) of table 5.4-2 in TS38.214, w is the same value for Rel-18 doppler codebook, i.e., w=14(K-1)m for AP-CSI-RS, w=56(KP-1) or 56 KP symbols according to UE capability for P/SP CSI-RS

- (new) Proposal 2.4-3.

For CSI prediction using UE-side model, for training data collection,

- OCPU= 1

- the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

- (new) Proposal 2.4-1B.

For CSI prediction using UE-side model, for inference,

- The max number of simultaneous AI/ML PUs for CSI prediction can be reported by UE capability optionally.

Concerned by: Nokia, NTT,

- (new) Proposal 2.5-3.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling

  - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling,

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, semi-persistent and aperiodic are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | SP report | AP report |
| --- | --- | --- |
| AP report | Not support | Support |
| SP report | Support | Support |

  - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

- For AP monitoring report with AP CSI-RS and for AP inference report with AP CSI-RS,

  - UE is expected to receive the monitoring report trigger no later than the first symbol of the inference measurement resource;

    - The active resource /port counting for inference report is extended to the end of the associated monitoring report

    - The active resource / port counting for monitoring, adopt OCPU=X and reuse legacy active resource/port counting time.

      - FFS on X values

  - Otherwise, UE is not expected to be triggered with monitoring report for the associated inference report

    - The active resource/port counting for inference report reuse the legacy counting time

- Proposal 2.5-1A.

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select to Alt 1.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

|  | Supporting companies |
| --- | --- |
| Alt 1 | Ericsson, HW, VIVO, TCL, ZTE, CATT, LG, CMCC, Xiaomi, NEC, Ruijie Networks, Panasonic, Lenovo, Sony, Fujitsu, HONOR, ETRI, OPPO, Sharp, CEWiT |
| Alt 2 | Samsung, IDC, Panasonic, Nokia, MediaTek |
| Alt 3 | Spreadtrum, NEC, NVIDIA, Apple, Qualcomm, NTT Docomo |

- Proposal 2.5-1B.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- SGCS is calculated

  - per prediction instance,

  - per layer where the total number of layers is the same as the reported RI in the associated inference report,

  - averaged over subbands

- Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’

- Proposal 2.5-1C.

For CSI prediction using UE-side model, for UE assisted performance monitoring, support N [3 or 4]-bit quantization of SGCS value [per layer and per prediction instance]

- 2N-1 codepoints are used to uniformly quantize (x 1] in linear scale.

- One codepoint is used to indicate [0 x]

- FFS on N and x values

Proposals for online session

Proposals for Tuesday.

Offline working assumption

For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’.

Proposal 2.4-1.

For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook,

- For PU occupancy, support

  - Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied,

    - OAPU= 0 or N is reported by UE

    - OCPU=0 or M is reported by UE

    - Note: Detailed values of N and M can be further discussed in UE feature.

    - Note: OAPU= 0 and OCPU=0 is not allowed

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report is not required to update, and neither of the PUs is occupied.

    - [Note: if any of the unoccupied PU or CPU cannot satisfy the corresponding required PU or CPU respectively by the CSI report, the remained CSI report with lowest priority is not required to update]

- For occupancy duration of CPU and APU,

  - the same occupancy duration is used if both CPU and APU are reported non-zero value,

  - reuse following legacy occupancy duration at least if monitoring report is not configured

    - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP =[{1, 2, 4}] is indicated by UE capability.

      - Note: Detailed values of KP can be further discussed in UE feature.

    - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability

- The max number of AI/ML PU for CSI prediction can be reported by UE capability optionally.

  - If max number of AI/ML PU for CSI prediction is reported, UE does not expect to be configured/triggered with more than the max number of AI/ML PU for CSI prediction.

Proposal 2.4-3

For CSI prediction using UE-side model, for inference, regarding active resource/port counting,

- Reuse legacy active resource/port counting at least if monitoring report is not configured

Proposal 2.5-3

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- support to reuse CSI framework for the configuration for monitoring result report in L1 signaling:

  - Dedicated resource set(s) for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, [periodic, semi-persistent, aperiodic] are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | P report | SP report | AP report |
| --- | --- | --- | --- |
| AP report | Not support | Not support | Support |
| SP report | Not support | Support | Support |

- UE measures the dedicated resource set(s) for monitoring

  - For monitoring resource type for measurement, [periodic, semi-persistent, aperiodic] are supported.

- UE measures the dedicated resource set(s) for monitoring

Proposal 2.5-1A.

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select to Alt 1.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

Proposals for Wednesday.

Proposal 2.4-2.

For CSI prediction using UE-side model, regarding Z/Z’ for inference

- For N4=1,

  - For Z value, Z2+t+w

  - For Z’ value, Z’2'+t

- For N4>1,

  - For Z value, according to UE capability

    - option 1: Z2+t + w+ Z’2+t, option 2: [Z2+t + w+ Z’2], or

    - Z2+t+w

  - For Z’ value, according to UE capability

    - option 1: 2(Z’2'+t), option 2: 2Z’2'+t, or

    - Z’2’+t

- UE may report the value of t per SCS

  - Detailed value of t can be discussed in UE feature

Note: (Z2, Z’2) of table 5.4-2 in TS38.214, w is the same value for Rel-18 doppler codebook, i.e., w=14(K-1)m for AP-CSI-RS, w=56(KP-1) or 56 KP symbols according to UE capability for P/SP CSI-RS

Proposal 2.5-3.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling

  - Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling,

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, semi-persistent and aperiodic are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | SP report | AP report |
| --- | --- | --- |
| AP report | Not support | Support |
| SP report | Support | Support |

  - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

- If the inference measurement resource is periodic or semipersistent, UE is expected to receive the monitoring report trigger, no later than the first symbol of the last Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the first inference report not earlier than the monitoring report trigger

- If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, no later than the first symbol of the last K inference measurement resources that are no later than the CSI reference resource of the inference report;

  - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report

- Proposal 2.5-1A.

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select to Alt 1.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

|  | Supporting companies |
| --- | --- |
| Alt 1 | Ericsson, HW, VIVO, TCL, ZTE, CATT, LG, CMCC, Xiaomi, NEC, Ruijie Networks, Panasonic, Lenovo, Sony, Fujitsu, HONOR, ETRI, OPPO, Sharp, CEWiT |
| Alt 2 | Samsung, IDC, Panasonic, Nokia, MediaTek |
| Alt 3 | Spreadtrum, NEC, NVIDIA, Apple, Qualcomm, NTT Docomo |

- Proposal 2.5-1B.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- SGCS is calculated

  - per prediction instance,

  - per layer where the total number of layers is the same as the reported RI in the associated inference report,

  - averaged over subbands

- Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’

- Proposal 2.5-1C.

For CSI prediction using UE-side model, for UE assisted performance monitoring, support N [3 or 4]-bit quantization of SGCS value [per layer and per prediction instance]

- 2N-1 codepoints are used to uniformly quantize (x 1] in linear scale.

- One codepoint is used to indicate [0 x]

- FFS on N and x values

Proposals for Thursday.

- (new) Proposal 2.5-3B.

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- If the inference measurement resource is aperiodic CSI-RS, UE is expected to receive the monitoring report trigger, no later than the first symbol of the earliest occasion of K inference measurement resources that are no later than the CSI reference resource of the associated inference report;

  - Additional active resource/port counting time for the aperiodic resource of the inference report is from the end of the inference report to the end of the triggered monitoring report

- If inference report is aperiodic and the inference measurement resource is periodic or semi-persistent, UE is expected to receive the monitoring report trigger, no later than the first symbol of the earliest occasion of the most recent Kp inference measurement resource transmission occasions that are no later than the CSI reference resource of the associated inference report, the first inference report not earlier than the monitoring report trigger

- Proposal 2.5-1A.

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select to Alt 1.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

|  | Supporting companies |
| --- | --- |
| Alt 1 | Ericsson, HW, VIVO, TCL, ZTE, CATT, LG, CMCC, Xiaomi, NEC, Ruijie Networks, Panasonic, Lenovo, Sony, Fujitsu, HONOR, ETRI, OPPO, Sharp, CEWiT |
| Alt 2 | Samsung, IDC, Panasonic, Nokia, MediaTek |
| Alt 3 | Spreadtrum, NEC, NVIDIA, Apple, Qualcomm, NTT Docomo |

- Proposal 2.5-1B.

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring,

- SGCS is reported

  - wideband frequency granularity

  - per monitored prediction instance, and the total # of monitored prediction instance is (FFS: 1 or N4)

  - per layer, and the total number of layers is (FFS: the same as the reported RI in the associated inference report or 1, i.e., first layer)

- Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’

- Each SGCS value is quantized with N-bit

  - 2N-1 codepoints are used to uniformly quantize (x y] in linear scale.

  - One codepoint is used to indicate [0 x]

  - FFS on value of N, x and y

- Introduce new RRC parameter for reportQuantity, e.g., ‘SGCS-r19’

- (updated) Proposal 2.2-1.

For CSI prediction using UE-side model, for data collection for training,

- at least two CSI-RS resources can be configured in a single resource set in CSI report configuration.

- OCPU=1

- the CPU occupancy starts from the first symbol of each P/SP-CSI-RS occasion till Z3’ symbols after the P/SP-CSI-RS occasion.

- (new) Proposal 2.4-1B.

For CSI prediction using UE-side model, for inference,

- The max number of AI/ML PU for CSI prediction can be reported by UE capability optionally.

Proposals for Friday.

Agreements

Agreements in RAN1#121

Agreement

For CSI prediction using UE-side model, for data collection for training, reportQuantity is set to ‘none-CSI-r19’.

Agreement

For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook,

- For PU occupancy, support

  - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied,

    - OAPU= 0 or N is reported by UE

    - OCPU=0 or M is reported by UE

    - Note: Detailed values of N and M can be further discussed in UE feature.

    - Note: Combination of OAPU= 0 and OCPU=0 is not allowed

    - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied

- For occupancy duration of CPU and APU,

  - the same occupancy duration is used if both CPU and APU are reported non-zero value

  - if associated monitoring report is not configured, reuse following legacy occupancy duration

    - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability.

      - Note: Detailed values of KP can be further discussed in UE feature.

    - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report.

- The total number of dedicated AI/ML PU for AI/ML is reported by UE capability

Agreement

For CSI prediction using UE-side model, for inference, regarding active resource/port counting,

- Reuse legacy active resource/port counting at least if associated monitoring report is not configured

Agreement

For CSI prediction using UE-side model, for inference, in addition to legacy Z/Z’ for doppler codebook, UE may report the value of t per SCS

- Detailed value of t can be discussed in UE feature

Agreement

For CSI prediction using UE-side model, for UE assisted performance monitoring,

- Support to reuse CSI framework for the configuration for monitoring result report in L1 signaling

  - Dedicated resource set for monitoring and report configuration for monitoring are configured in a dedicated CSI report configuration used for monitoring

    - The ID of an inference report configuration is configured in the configuration for monitoring to link the inference report configuration and monitoring report configuration

    - For monitoring report type, semi-persistent and aperiodic are supported

    - The following combination for inference report type and monitoring report type are supported

| Monitoring report type Inference report type | SP report | AP report |
| --- | --- | --- |
| AP report | Not support | Support |
| SP report | Support | Support |

  - For monitoring resource type for measurement, periodic, semi-persistent and aperiodic are supported.

Agreements in RAN1#120bis

Agreement

Introduce a dedicated AI/ML PU for AI/ML features for UE,

- The AI/ML PU is used at least for quantifying the simultaneous processing of multiple CSI reports subject to CSI-related AI/ML use case(s), e.g., CSI compression (if supported), CSI prediction, BM spatial prediction, BM temporal prediction.

Conclusion

For CSI prediction using UE-side model, for data collection for training, aperiodic CSI-RS resource for CMR is not supported.

Agreement

For CSI prediction using UE-side model, for training data collection, support

- CSI-ReportConfig can used for configuring the resources for data collection purpose without CSI report

  - FFS on how to indicate without CSI report in CSI-ReportConfig

Agreement

For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring

Agreement

For CSI prediction using UE-side model, for performance monitoring type 3, support SGCS as a performance metric.

Agreement

For the definition of SGCS,

Note: How to handle layer mapping mismatch, if any, is up to UE implementation

Agreement

For CSI prediction using UE-side model, for reporting contents of UE assisted performance monitoring, down-select one alternative by RAN1#121.

- Alt 1: one SGCS is calculated based on predicted CSI for one inference reporting, ground truth CSI

- Alt 2: one SGCS is calculated based on predicted CSI for one inference reporting, and ground truth CSI, another SGCS is based on ground truth CSI and CSI (non-predicted) corresponding to the latest CSI-RS transmission occasion not later than CSI reference resource of the inference reporting instance.

- Alt 3: statistic of reporting contents (e.g., mean, x% CDF of SGCS values) of inference reporting instances within configured monitoring window

  - Monitoring window can be configured by NW

    - FFS on signalling details

- FFS on whether to report per prediction instance, selected prediction instance(s), averaged over prediction instances

- FFS on report frequency granularity (e.g., per wideband or per subband or averaged over subband or selected subband)

- FFS on whether to report per layer, the first layer, averaged over layer

- FFS on how to quantize and report mechanism

Agreement

For CSI prediction using UE side model, for inference, consider following options for potential down selection

- Option 1: only dedicated AI/ML PU (OAPU) is occupied

- Option 2: only legacy CPU (OCPU) is occupied

- Option 3: both dedicated AI/ML PU (OAPU) and legacy CPU (OCPU) are occupied

- FFS whether OAPU and OCPU are based on defined rule and/or reported by UE

- Note: The supported option(s) by UE is reported by UE capability, if multiple options are supported.

The total number of dedicated AI/ML PU for AI/ML is reported by UE capability.

- Note: The total number of Use case specific dedicated AI/ML PU could be discussed separately.

Agreement

For CSI prediction using UE-side model, at least for inference, introduce new RRC parameter for CSI report configuration to distinguish CSI report of AI-CSI prediction and non-AI CSI prediction.

- Note: terminology of “AI-CSI prediction” and “non-AI CSI prediction” is separate discussion

- Detailed parameter name is upto RAN2

Agreements in RAN1#120

Agreement

For CSI prediction using UE-side model, at least for inference, Rel-18 CSI framework is reused.

- For CSI-RS resource type for CMR, periodic, semi-persistent, and aperiodic CSI-RS are supported.

- For inference report,

  - for N4>=1, support Rel-18 codebook ('typeII-Doppler-r18')

Agreement

For CSI prediction using UE-side model, if performance monitoring type 1 or 3 is supported, for calculation of monitoring metric, support

- Based on intermediate KPI

  - Down select between SGCS and NMSE by RAN1#120bis

  - FFS on the definition of SGCS/NMSE and how to calculate monitoring metric

Agreement

For CSI prediction using UE-side model, for CSI processing criteria and timeline, at least for inference further study on

- Whether the CPU should be shared or separately counted between legacy CSI reporting and AI/ML-based CSI reporting

- Whether the Processing Unit should be shared or separately counted among AI/ML related features/functionalities.

- Whether new timeline is needed/updated for inference, and whether a different timeline is needed when functionality switches/activates.

- Whether legacy framework for active CSI-RS resource and port counting can be reused

Note: Strive to study CSI processing criteria considering both BM and CSI case, and take the existing solutions as starting point.

Agreement

For CSI prediction using UE-side model, for data collection for training,

- For resource configuration,

  - For CSI-RS resource type for CMR, periodic and semi-persistent are supported.

    - FFS on support of aperiodic CSI-RS resource

  - FFS on whether to separately configure CSI-RS resource(s) used for measurements for model input and CSI-RS resource(s) used for measurements for ground-truth CSI

  - FFS whether/how to enhance CSI-RS resource configuration to reduce CSI-RS signalling overhead.

- FFS on CSI report configuration

Agreements in RAN1#118bis

Agreement

For consistency between training and inference, study to identify which potential NW-side additional conditions, if any, may impact on UE assumption for CSI prediction using UE-sided model, resulting non-negligible degradation on model generalization performance.

Note: Companies are encouraged to provide generalization performance evaluation on the impact of NW-side additional conditions, if considered.

Agreement

For generalization evaluation to identify potential NW-side additional conditions to ensure consistency between training and inference, consider one or more of the following aspects, if evaluated:

- [Optional evaluation] Various tilt angle (e.g., [102] degrees, [110] degrees)

- [Optional evaluation] Various TXRU mapping

  - With TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (8x8x2) and (12 x 8 x 2) antenna elements)

  - Without TXRU virtualization (e.g., for 32 port CSI-RS, [N1, N2, P] = [2, 8, 2] (baseline), [4,4,2] (optional) with (2x8x2) and (4 x 4 x 2) antenna elements, respectively)

- Other configurations are not precluded

Note: for other evaluation assumptions, reuse baseline of TR 38.843

Note: report how to map antenna port to antenna elements

Note: report the backbone/structure of AI/ML model used

Agreement

The TP1, TP2 and TP3 to TR38.843 in Section 3 of R1-2409236 are endorsed in principle.

Agreements in RAN1#118

Observation

For the generalization verification of CSI prediction using UE sided model over various UE speeds, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h and

    - UE speed#A is 30 km/h or 120km/h, 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -5.5% degradation, and 1 source [LG Electronics] observes -16.2% degradation.

    - UE speed#A is 60 km/h, 2 sources [CMCC, LG Electronics] observe -26.79%~-13.3% degradation.

  - If UE speed#B is 30 km/h and

    - UE speed#A is 60km/h, 4 sources [Huawei, CATT, Ericsson, vivo] observe a generalized performance of -11.4%~-2.7% degradation and 4 sources [CMCC, Nokia, LG Electronics, ZTE] observe a generalized performance of -33.6%~-19% degradation.

    - UE speed#A is 10km/h or 120km/h, 3 sources [CMCC, MediaTek, Fujitsu] observe a generalized performance of -72.37%~-51.5% degradation.

  - If UE speed#B is 60 km/h and

    - UE speed#A is 10km/h, 1 source [LG Electronics] observes a generalized performance of -16.4 degradation

    - UE speed#A is 30km/h, 1 source [ZTE] observes a generalized performance of -5.4% degradation and 6 sources [Huawei, Fujitsu, MediaTek, Ericsson, vivo, LG Electronics, CATT] observe a generalized performance of -56.3%~-13.8% degradation

  - If UE speed#B is 120 km/h and

    - UE speed#A is 30km/h, 1 source [MediaTek] observes a generalized performance of -56.3% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-3.8% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 8 sources [Fujitsu, Huawei, CATT, vivo, Apple, Ericsson, ZTE, LG Electronics]

  - If UE speed#B is 10 km/h

    - 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -0.2% degradation, 1 source [LG Electronics] observe -5.1% degradation, and 1 source [CMCC] observes -16.87% degradation.

  - If UE speed#B is 30 km/h

    - 9 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson, MediaTek, ZTE, LG Electronics] observe a generalized performance of less than -3.8% degradation, 1 source [CMCC] observes a generalized performance of -15.44% degradation.

  - If UE speed#B is 60 km/h and

    - 8 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson, ZTE, LG Electronics] observe a generalized performance of less than -3.63% degradation, 3 sources [CMCC, MediaTek] observe a generalized performance of -13.5%~-6.77% degradation.

  - If UE speed#B is 120 km/h and

    - 1 source [MediaTek] observes a generalized performance of -43.6% degradation

  - Note: For generalization Case 3, 2 sources [CMCC, MediaTek] observe performance degradations (-43.6%~-13.5% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, 120 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 4 sources [vivo, Ericsson, MediaTek, ZTE] consider spatial consistency. Other sources do not consider spatial consistency.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - Note: Results refer to Table 3-1 of R1-2407338

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from perspective of phase discontinuity modelling

- If phase discontinuity modelling is not adopted

  - For 30km/h UE speed,

    - 2 sources [vivo, Fujitsu] observe 4.24%~4.3% gain with N4=1

    - 1 source [vivo] observes 3.1% ~ 40.5% with N4=4

    - 1 source [Qualcomm] observes -1.2%~6.3% with N4=1 depending on filter updates between 10 and 70 learning window size

  - For 60km/h UE speed,

    - 1 source [vivo] observe 13.5% gain with N4=1, and 1 source [vivo] observe 8.2%~34.6% gain with N4=4

    - 1 source [Qualcomm] observes 0%~8% with N4=1 depending on filter updates between 10 and 70 learning window size

- If phase discontinuity modelling is adopted,

  - For 30km/h UE speed,

    - Without phase adjustment,

      - 2 sources [vivo, Fujitsu] observe 7.26%~7.8% gain with N4=1

      - 1 source [vivo] observes 6.5%~23.5% gain with N4=4

      - 1 source [Qualcomm] observes -0.8%~8% with N4=1 depending on filter updates between 10 and 70 learning window size

    - With phase adjustment,

      - 1 source [Qualcomm] observes 1.2%~7.7% gain with N4=1 depending on filter updates between 10 and 70 learning window size

      - 1 source [Fujitsu] observes 25.6% gain

  - For 60km/h UE speed,

    - Without phase adjustment,

      - 1 source [vivo] observe 6.6% gain with N4=1 and 6.4%~55% with N4=4

      - 1 source [Qualcomm] observes 5.7%~17.5% gain with N4=1 depending on filter updates between 10 and 70 learning window size

    - With phase adjustment, 1 source [Qualcomm] observes 4.2%~11.9% gain with N4=1 depending on filter updates between 10 and 70 learning window size

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix as model input is assumed.

  - The performance metric is SGCS in linear value for layer 1.

  - Note: Results refer to Table 2-12 of R1-2407338

- Note: N4 refers to the number of predicted CSI instances

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from channel estimation perspective

- If ideal channel estimation is adopted

  - For N4=1

    - 9 sources [vivo, Nokia, Intel, Fujitsu, InterDigital, Qualcomm, OPPO, Spreadtrum, Lenovo] observe -1.2%~6.3% gain

    - 1 source [Apple] observes -2% ~17% gain depending on filter complexity and filter update

  - For N4=4

    - 2 sources [OPPO, Intel] observe -1.6%~5.56%

    - 1 source [InterDigital] observes -4.5%~6.93%

    - 1 source [vivo] observes 3.1%~40.5%

- If realistic channel estimation is adopted,

  - For N4=1

    - 5 sources [Intel, Fujitsu, Ericsson, MediaTek, CATT] observe 0.43%~7.26% gain

    - 2 sources [vivo, Nokia] observe 15.2%~19.5 gain

    - 1 source [InterDigital] observes 68% gain

    - 1 source [ZTE] observes 5.8%~16.4% gain depending on traffic load

  - For N4=4

    - 2 sources [Intel, MediaTek] observe 0.1%~1.4% gain

    - 1 source [Ericsson] observes 5%~29% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix as model input and UE speed of 30km/h is assumed.

  - 3 sources [vivo, ZTE, Ericsson] consider spatial consistency, and other sources do not consider spatial consistency.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - The performance metric is SGCS in linear value for layer 1.

- Note: Results refer to Table 2-3 of R1-2407338

- Note: N4 refers to the number of predicted CSI instances

Conclusion

For computational complexity of both AI/ML and non-AI/ML based CSI prediction, to report the number of FLOPs assuming whole bandwidth and one prediction sample.

Observation (Update of the observation made in RAN1#117)

- For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective,

  - If spatial consistency is not adopted, and if N4=1

    - For 10km/h UE speed, 1 source [InterDigital] observes 6% gain

    - For 30km/h UE speed, 9 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, Qualcomm, NTT Docomo] observe 5.9%~20.6% gain and 4 sources [Intel, LG Electronics, ZTE, Fujitsu] observe 23.2%~35.4% gain. 2 sources [Nokia, InterDigital] observe 54%~106% gain

    - For 60km/h UE speed, 4 sources [Qualcomm, China Telecom, Huawei, ZTE] observe 10.5%~26.53% gain

  - If spatial consistency is adopted, and if N4=1

    - For 30km/h UE speed, 2 sources [Ericsson, ZTE] observes 22.93%~23% gain, 1 source [vivo] observe 68.5% gain

    - For 60km/h UE speed, 3 sources [Ericsson, vivo, ZTE] observe 16.2~24.3% gain

  - If spatial consistency is not adopted, and if N4=4

      - For 10km/h UE speed, 1 source [LG Electronics] observes 9.1%~29.7% gain

      - For 30km/h UE speed, 1 source [NTT Docomo] observes 4.3%~12.3% gain, and 2 sources [OPPO, Intel] observe 17.6%~35.4%gain

  - If spatial consistency is adopted, and if N4=4

      - For 10km/h UE speed, 1 source [Samsung] observes -1.61%~62.9% gain

      - For 30km/h UE speed, 1 source [Ericsson] observes 23%~34% gain, 1 source [MediaTek] observe 20.9%~76.4% gain

      - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 5.96%~-22% gain,

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 20ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 7 sources [ZTE, Ericsson, Intel, vivo, Fujitsu, Samsung, MediaTek] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [NTT Docomo] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, 1 source [Samsung] considers per layer raw channel matrix after pre-processing, and other sources do not consider pre/post processing.

  - The performance metric is SGCS in linear value for layer 1.

- Note: N4 refers to the number of predicted CSI instances

Note: Results refer to Table 2-1 of R1-2407338

Observation (Update of the observation made in RAN1#117)

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#1 of the nearest historical CSI:

- For FTP traffic with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1

    - 2 sources [Intel, NTT Docomo] observe -0.2%~1% gain

    - 4 sources [Ericsson, Fujitsu, Apple, Nokia] observe 4.4%~9% gain.

  - For 60km/h UE speed, and N4=1

    - 2 sources [Huawei, Ericsson] observe 1.2%~5% gain;

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 0.3% gain.

    - 1 source [Ericsson] observes 7% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 5% gain.

- For FTP traffic with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1

    - 2 sources [Intel, NTT Docomo] observe 0.9%~2.3% gain

    - 1 source [Apple] observe 8.5% gain

    - 2 sources [Ericsson, Fujitsu] observe 22.3%~37% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observe 1.8%~3.5% gain;

    - 1 source [Ericsson] observes 21% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 2.3% gain.

    - 1 source [Ericsson] observes 29% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 21% gain.

- For FTP traffic with high RU (RU>=70%)

  - For 30km/h UE speed and N4=1

    - 4 sources [Intel, NTT Docomo, OPPO] observe 2%~3.6% gain

    - 2 sources [Fujitsu, Apple] observe 8.5%~31.7% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 7.2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 4.2% gain;

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 3 sources [Lenovo, Fujitsu, vivo] observe 27%~51% gain.

    - 2 sources [MediaTek, Intel] observe 1.2%~8.7% gain.

  - For 60km/h UE speed and N4=1

    - 1 source [vivo] observe 12.1% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [Fujitsu] observes 11.6% gain.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, OPPO, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-5/2-7 of R1-2407338

Observation (Update of the observation made in RAN1#117)

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of 5% UE UPT, gains are observed compared to Benchmark#1 of the nearest historical CSI:

- For FTP traffic with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1

    - 1 source [NTT Docomo] observes 1.5% gain.

    - 4 sources [Intel, Fujitsu, Apple, Nokia] observe 6.7%~15.7% gain.

    - 1 source [Ericsson] observes 27% gain.

  - For 60km/h UE speed, and N4=1

    - 2 sources [Huawei, Ericsson] observe 9%~18.3% gain;

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 3.6% gain.

    - 1 source [Ericsson] observes 14% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 4% gain.

- For FTP traffic with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1

    - 1 source [NTT Docomo] observes 4.1% gain.

    - 2 sources [Intel, Fujitsu, Apple] observe 14.3%~38.9% gain.

    - 1 source [Ericsson] observes 100% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 11.3% gain;

    - 1 source [Ericsson] observes 45% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [NTT Docomo] observes 10% gain.

    - 1 source [Ericsson] observes 77% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 26% gain.

- For FTP traffic with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 1 source [Fujitsu] observes 51.7% gain.

    - 3 sources [Intel, NTT Docomo, Apple] observe 7.8%~15.8% gain

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observe 17.8% gain;

  - For 30km/h UE speed, and N4=4

    - 1 source [NTT Docomo] observes 6.2% gain

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 4 sources [Fujitsu, MediaTek, Lenovo, Intel] observe 7.7%~10.5% gain.

    - 1 source [vivo] observes 39.7% gain.

  - For 60km/h UE speed and N4=1

    - 1 source [vivo] observes 20.6% gain.

  - For 30km/h UE speed and N4=4

    - 1 source [Fujitsu] observes 2.1% gain.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, OPPO, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-5/2-7 of R1-2407338

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from UE speed perspective

- If spatial consistency is not adopted, if N4=1

  - For 10km/h UE speed, 1 source [InterDigital] observes 0% gain

  - For 30km/h UE speed,

    - 7 sources [Lenovo, Spreadtrum, Intel, CATT, Fujitsu, ZTE, Nokia] observe 1%~19.5% gain,

    - 1 source [InterDigital] observes 68% gain

    - 2 sources [Apple, Qualcomm] observe -2% ~17% gain depending on filter complexity and filter update

  - For 60km/h UE speed,

    - 3 sources [Huawei, CATT, Qualcomm, ZTE] observe 3.6%~26.35% gain,

    - 1 source [InterDigital] observes 35% gain

    - 1 source [Qualcomm] observe 0% ~8.7% gain depending on filter complexity and filter update

- If spatial consistency is adopted, if N4=1

  - For 30km/h UE speed,

    - 1 source [Ericsson] observes 5% gain,

    - 2 sources [vivo, ZTE] observe 15.2%~16.4%

  - For 60km/h UE speed,

    - 3 sources [Ericsson, vivo, ZTE] observes 3%~13.5% gain,

- If spatial consistency is not adopted, if N4=4

  - For 10km/h UE speed, 1 source [InterDigital] observes 0%~1.4% gain

  - For 30km/h UE speed, 3 sources [InterDigital, Intel, OPPO] observe -1.6%~6.93% gain,

  - For 60km/h UE speed, 1 source [InterDigital] observes -4%~7.3% gain

- If spatial consistency is adopted, if N4=4

  - For 30km/h UE speed,

    - 1 source [MediaTek] observes 0.43%~0.97% gain

    - 1 source [Ericsson] observes 5%~29% gain

    - 1 source [vivo] observes 28.9%~43.6% gain

  - For 60km/h UE speed,

    - 1 source [MediaTek] observes -1.01%~1.52% gain

    - 1 source [Ericsson] observes 3%~16% gain

    - 1 source [vivo] observes 10.0%~32.3% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 6 sources [ZTE, Ericsson, Intel, vivo, Fujitsu, CATT] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - All sources consider Raw channel matrix as model input.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing

  - The performance metric is SGCS in linear value for layer 1.

- Note: N4 refers to the number of predicted CSI

- Note: Results refer to Table 2-3 of R1-2407338

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction approach:

- For FTP traffic, with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1,

    - 2 sources [Ericsson, InterDigital] observe 7.6%~9% gain.

    - 4 sources [CATT, Intel, Fujitsu, Nokia] observe -2.41%~1.1% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [InterDigital] observe -3.4% gain

    - 2 sources [Huawei, CATT] observes 0.9%~1.2% gain;

    - 1 source [Ericsson] observes 11% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 13% gain.

    - 2 sources [InterDigital, MediaTek] observe 0%~0.3% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson,] observes 13%

    - 1 source [MediaTek] observes 0.14% gain

- For FTP traffic, with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1,

    - 1 source [Ericsson] observes 24% gain.

    - 3 sources [InterDigital, Intel, Fujitsu] observe 0.2%~5.1% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [InterDigital] observes -29.4% gain;

    - 1 source [Huawei] observes 3.1% gain;

    - 1 source [Ericsson] observes 31% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 35% gain.

    - 2 sources [MediaTek, InterDigital] observe -0.25%~2% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [InterDigital] observes -16.2% gain.

    - 1 source [MediaTek] observes 0.25% gain.

    - 1 source [Ericsson] observes 32% gain.

- For FTP traffic, with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 4 sources [InterDigital, Intel, Fujitsu, OPPO] observes -0.2%~9.2% gain;

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 2.5% gain;

  - For 30km/h UE speed and N4=4,

    - 2 sources [InterDigital, MediaTek] observes -0.8%~0.11% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [InterDigital] observes -19.1% gain.

    - 1 source [MediaTek] observes 0.92% gain.

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 2 sources [Lenovo, ZTE] observe 10.6%~24% gain.

    - 2 sources [Fujitsu, vivo] observe 7.8%~9.7% gain.

    - 3 sources [CATT, MediaTek, Intel] observe -0.6%~1.2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [CATT] observes 0.2% gain

    - 1 source [vivo] observes 8.4% gain

  - For 30km/h UE speed and N4= 4

    - 1 source [Fujitsu] observes 7% gain.

    - 1 source [vivo] observes 6.8% gain

  - For 60km/h UE speed and N4=4

    - 1 source [vivo] observes 11.6% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-247339

- Note: The following boxchart shows the median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of 5% UE UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction approach:

- For FTP traffic, with low RU (RU<=39%)

  - For 30km/h UE speed and N4=1,

    - 2 sources [Ericsson, InterDigital] observe 18%~26% gain.

    - 4 sources [CATT, Fujitsu, Intel, Nokia] observe -1.1%~7.6% gain.

  - For 60km/h UE speed, and N4=1

    - 3 sources [Huawei, InterDigital, CATT] observe 1.9%~5.2% gain;

    - 1 source [Ericsson] observes 17% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 23% gain.

    - 1 source [InterDigital] observes -3.7% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observe 19%gain.

    - 1 source [InterDigital] observes 4%gain.

- For FTP traffic, with mid RU (40<=RU<=69%)

  - For 30km/h UE speed and N4=1,

    - 1 source [Ericsson] observes 46% gain.

    - 1 source [InterDigital] observes 18.7% gain.

    - 2 sources [Intel, Fujitsu] observe 4%~6.6% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei] observes 8.6% gain;

    - 2 sources [Ericsson, InterDigital] observe 23%~66% gain.

  - For 30km/h UE speed and N4=4,

    - 1 source [Ericsson] observes 73% gain.

    - 1 source [InterDigital] observes -9% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [Ericsson] observes 56% gain.

    - 1 source [InterDigital] observes 11.5% gain.

- For FTP traffic, with high RU (RU>=70%)

  - For 30km/h UE speed, and N4=1

    - 1 source [Intel] observes 1.9% gain;

    - 2 sources [InterDigital, Fujitsu] observe 20.7%~26.3% gain;

  - For 60km/h UE speed, and N4=1

    - 1 source [Huawei, InterDigital] observes 14.8%~16% gain;

  - For 30km/h UE speed and N4=4,

    - 1 source [InterDigital] observes 0.9% gain.

  - For 60km/h UE speed, and N4=4

    - 1 source [InterDigital] observes 22.8% gain.

- For full buffer traffic:

  - For 30km/h UE speed and N4=1

    - 2 sources [vivo, ZTE] observe 15.7%~16.1% gain.

    - 4 sources [Lenovo, Fujitsu, CATT, Intel] observe 0.2%~6% gain.

    - 1 source [MediaTek] observes -2% gain.

  - For 60km/h UE speed, and N4=1

    - 1 source [CATT] observes 0.4% gain

    - 1 source [vivo] observes 11.6% gain

  - For 30km/h UE speed and N4= 4

    - 1 source [Fujitsu] observes 6.3% gain.

    - 1 source [vivo] observes 21% gain

  - For 60km/h UE speed and N4=4

  - 1 source [vivo] observes 26.7% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

  - 5 sources [Ericsson, Fujitsu, CATT, MediaTek, vivo] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-2407339

- Note: The following boxchart shows the median, 0.75 quantile, 0.25 quantile, outliers, and min/max values excluding outliers, for low RU, Mid RU, High RU and full buffer with UE speed of 30km/h.

Observation

- From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. The actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations.

- From a perspective of complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss. For example, 7 sources adopt the algorithm subject to the computational complexity of filter updates and inference in units of FLOPs from 0.47M to 106M and 0.067M to 3M, respectively.

- Results refer to Figure 2-1, Table 2-9, and Table 2-10 in R1-2407339.

Observation

For the generalization verification of CSI prediction using UE sided model over various deployment scenarios, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain deployment scenario#B applied for inference with a same deployment scenario#B

- For generalization Case 2, generalized performance may be achieved for some certain combinations of deployment scenario#A and deployment scenario#B but not for others:

  - For deployment scenario#B is UMa

    - 2 sources [Ericsson, MediaTek] observe -1.88%~0% degradation

    - 1 source [MediaTek] observe -6.8% degradation

  - For deployment scenario#B is UMi

    - 1 source [MediaTek] observe 0% degradation

    - 3 sources [Ericsson, vivo, ZTE] observe -4.85%~-3.03% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved (-1.95%~0% loss) for deployment scenario#B subject to any of UMa and UMi, if the training dataset is constructed with data samples subject to multiple deployment scenarios including deployment scenario#B as observed by 3 sources.

  - Minor loss (0%~-1.95%) are observed by 3 sources [vivo, ZTE, MediaTek].

  - Note: Moderate degradations of -5.2% are observed by 1 source [vivo] for deployment scenario#B subject to Uma

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - Note: Results refer to Table 3-2 of R1-2407339

Observation

For the generalization verification of CSI prediction using UE sided model over various carrier frequency, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain carrier frequency#B applied for inference with a same carrier frequency#B

- For generalization Case 2, significant degradations are suffered in general from the perspective of the layouts of antenna ports, as observed by 3 sources:

  - For carrier frequency#B is 2GHz

    - 1 source [MediaTek] observe -11.4% degradation

    - 1 source [vivo] observe -80.53% degradation

  - For carrier frequency#B is 3GHz or 4GHz

    - 2 sources [MediaTek, vivo] observe -34.23%~-80.53% degradation

    - 1 source [ZTE] observe -4.21% degradation

- For generalization Case 3, generalized performance may be achieved for some certain combinations of carrier frequency#A and carrier frequency#B but not for others, if the training dataset is constructed with data samples subject to multiple carrier frequencies including carrier frequency#B

  - For carrier frequency#B is 2GHz

    - 1 source [MediaTek] observe -0.5% degradation

    - 1 source [vivo] observe -9.27% degradation

  - For carrier frequency#B is 3GHz or 4GHz

    - 2 sources [MediaTek, ZTE] observe -1.93%~-5.1% degradation

    - 1 source [vivo] observe -14.94% degradation

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 2 sources [vivo, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - Note: Results refer to Table 3-3 of R1-2407339

Observation

For the generalization verification of CSI prediction using UE sided model over multiple aspects, till the RAN1#118 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to certain aspects #B applied for inference with the same aspects #B,

- For generalization Case 2,

    - 1 source [NTT DOCOMO] observes -9.8% ~ -1.5% degradation when the aspects #A is (2 GHz carrier frequency, 100% outdoor UE) and the aspects #B is (4GHz carrier frequency, 20% outdoor UE+80% indoor UE)

    - 1 source [NTT DOCOMO] observes -10.7%~-1.8% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 100 outdoor UE)

    - 1 source [NTT DOCOMO] observes -21.2%~-2.4% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

    - 1 source [Nokia] observe -3% degradation when aspects#A is (30km/h UE speed, 100% UE in a car, 2GHz carrier frequency) and the aspects#B is (3km/h UE speed, 20% outdoor UE, 4GHz carrier frequency)

- For generalization Case 3,

    - 1 source [NTT DOCOMO] observes -1.1%~0% when the aspects#A is (2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects #B is (4GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution) the

    - 1 source [NTT DOCOMO] observes -2.4%~-0.8% when aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 100 outdoor UE)

    - 1 source [NTT DOCOMO] observes -3.4%~-0.3% when the aspects#A is (UMa, 2 GHz carrier frequency, 100% outdoor UE distribution) and the aspects#B is (UMi, 4 GHz carrier frequency, 20% outdoor UE+80% indoor UE distribution)

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 1 source [NTT Docomo] considers eigenvector as model input, and 1 source [Nokia]s considers Raw channel matrix as model input.

  - Note: Results refer to Table 3-4 of R1-2407339

Observation

For the CSI prediction using CSI-RS with 20ms periodicity, till the RAN1#118 meeting, in terms of mean and 5% UE UPT, gains are observed compared to Benchmark #1 of the nearest historical CSI and Benchmark #2 of non-AI/ML based CSI prediction,

- Compared to Benchmark 1

  - In terms of UPT gain

    - For N4=1 and UE speed of 3km/h, 1 source [Ericsson] observes -18%~-7% and -26%~-8% degradation in terms of mean and 5% UE UPT, respectively.

    - For N4=4 and UE speed of 3km/h, 1 source [Ericsson] observes 1%~6% and 6%~17% gain in terms of mean and 5% UE UPT, respectively.

    - For N4=4 and UE speed of 30km/h, 1 source [Ericsson] observes 2%~17% and 13%~19% gain in terms of mean and 5% UE UPT, respectively, by using one 20ms periodic CSI-RS and three aperiodic CSI-RS.

- Compared to Benchmark 2

  - For N4=1 and UE speed of 3km/h,

    - 1 source [Ericsson] observes 10%~19% and 23%~31% gain in terms of mean and 5% UE UPT, respectively.

    - 1 source [InterDigital] observes -2.7%~-1% and -10.4%~-4.9% degradation in terms of mean and 5% UE UPT, respectively.

  - For N4=1 and UE speed of 10km/h, 1 source [InterDigital] observes 0.6%~1% gain and -14.2%~-8% gain in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 3km/h, 1 source [Ericsson] observes 14%~30% and 19%~56% gain in terms of mean and 5% UE UPT, respectively.

  - For N4=4 and UE speed of 30km/h,

    - 1 source [ZTE] observes 17.3% and 16.1 gain in terms of mean and 5% UE UPT, receptively.

    - 1 source [Ericsson] observes 5%~19% and 13%~40% gain in terms of mean and 5% UE UPT, respectively, by using one 20ms periodic CSI-RS and three aperiodic CSI-RS.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - 1 source [ZTE] considers full buffer model, other sources consider FTP traffic model

  - Raw channel matrix is used as the model input.

  - 5 sources [Ericsson, ZTE] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - The performance metric is SGCS in linear value for layer 1/2/3/4 and mean UPT, 5% UPT.

  - 2 sources [Ericsson] consider spatial consistency. Other sources do not consider spatial consistency.

  - Note: Results refer to Table 2-13, 2-14, 2-15, 2-16 of R1-2407339

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of mean UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction, from channel estimation perspective:

- For FTP traffic, with low RU (RU<=39%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Nokia, InterDigital] observe -2.41%~1.8% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [InterDigital, Huawei] observe -3.4%~0.9% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 0.3% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Ericsson, InterDigital] observe 7.6%~9% gain.

      - 3 sources [CATT, Intel, Fujitsu] observe 0%~1.1% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [InterDigital, CATT] observe -3.4%~1.2% gain

      - 1 source [Ericsson] observes 11% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 13% gain.

      - 1 source [MediaTek] observes 0% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 13%

      - 1 source [MediaTek] observes 0.14% gain

- For FTP traffic, with mid RU (40<=RU<=69%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes -4.5% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -7.1% gain

      - 1 source [Huaweil] observes 3.1% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -2% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Ericsson] observe 24% gain.

      - 3 sources [InterDigital, Intel, Fujitsu] observe 0.2%~5.1% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Ericsson] observes 31% gain

      - 1 source [InterDigital] observes -29.4% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 35% gain.

      - 2 sources [MediaTek, InterDigital] observe -0.25%~1.1% gain

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 32%

      - 1 source [MediaTek] observes 0.25% gain

- For FTP traffic, with high RU (RU>=70%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes -4.8% gain

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -9% gain

      - 1 source [Huaweil] observes 2.5% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -0.8% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 3 sources [InterDigital, Intel, OPPO] observe 0%~0.8%

      - 1 source [Fujitsu] observes 9.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -9% gain

    - For 30km/h UE speed and N4=4,

      - 2 sources [MediaTek, InterDigital] observe 0%~0.1% gain

    - For 60km/h UE speed, and N4=4

      - 1 source [MediaTek] observes 0.92% gain

- For full buffer model,

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [Lenovo] observes 24%

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1

      - 3 sources [Fujitsu, vivo, ZTE] observe 7.8%~10.6% gain.

      - 3 sources [CATT, MediaTek, Intel] observe -0.6%~1.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 0.2% gain

      - 1 source [vivo] observes 8.4% gain

    - For 30km/h UE speed and N4= 4

      - 1 source [Fujitsu] observes 7% gain.

      - 1 source [vivo] observes 6.8% gain

    - For 60km/h UE speed and N4=4

      - 1 source [vivo] observes 11.6% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-2407340

Observation

For the CSI prediction using UE-sided model, till the RAN1#118 meeting, in terms of 5% UE UPT, gains are observed compared to Benchmark#2 of a non-AI/ML based CSI prediction, from channel estimation perspective:

- For FTP traffic, with low RU (RU<=39%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observe -5.5% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [InterDigital, Huawei] observe 4%~4.3% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -3.7% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Ericsson] observe 17% gain.

      - 2 sources [CATT, InterDigital] observe 0%~4% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 1.9% gain

      - 1 source [Ericsson] observes 17% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 23% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 19%

- For FTP traffic, with mid RU (40<=RU<=69%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes -12.9% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes 2.6% gain

      - 1 source [Huaweil] observes 8.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes -9% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [Intel, Fujitsu] observe 4%~6.6% gain.

      - 1 source [InterDigital] observes 18.7% gain.

      - 1 source [Ericsson] observes 46% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Ericsson] observes 66% gain

      - 1 source [InterDigital] observes 2.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 73% gain.

      - 1 source [InterDigital] observes 18.7% gain

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 56%

- For FTP traffic, with high RU (RU>=70%)

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [InterDigital] observes 3.6% gain

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes -10.7% gain

      - 1 source [Huaweil] observes 14.8% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 0.9% gain.

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1,

      - 2 sources [InterDigital, Fujitsu] observes 20.7%~26.3%

      - 1 source [Intel] observes 1.9% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [InterDigital] observes 3.6% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 0.9% gain

- For full buffer model,

  - With ideal channel estimation

    - For 30km/h UE speed and N4=1,

      - 1 source [Lenovo] observes 0.2%

  - With realistic channel estimation

    - For 30km/h UE speed and N4=1

      - 2 sources [vivo, ZTE] observe 15.7%~16.1% gain.

      - 3 sources [Fujitsu, CATT, Intel] observe 2.6%~7.7% gain.

      - 1 source [MediaTek] observes -2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 0.4% gain

      - 1 source [vivo] observes 11.6% gain

    - For 30km/h UE speed and N4= 4

      - 1 source [Fujitsu] observes 6.3% gain.

      - 1 source [vivo] observes 21% gain

    - For 60km/h UE speed and N4=4

      - 1 source [vivo] observes 26.7% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Nokia] considered antenna(port)-delay domain transformation/ antenna(port)-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 1 source [Nokia] considers 100% in car UE distribution and other sources consider 100% outdoor UE distribution.

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-6/2-8 of R1-2407340

Observation

The following aspects have been studied for CSI prediction using UE-sided model:

- From the perspective of basic performance gain over non-AI/ML benchmark (without considering generalization),

  - It has been studied with corresponding observations on:

    - the metrics of SGCS, mean UPT, 5% UPT;

    - the benchmarks of nearest historical CSI and non-AI/ML based CSI prediction.

    - The impact of channel estimation error, phase discontinuity, spatial consistency, UE Speed, observation window, prediction window, CSI-RS periodicity.

- It has been studied with corresponding observations on complexity for both AI/ML based CSI prediction and non-AI/ML based CSI prediction.

- It has been studied on localized model including evaluation methodology, but is lack of observations.

- From the perspective of generalization over various scenarios,

  - It has been studied with corresponding observations on (with the metric of SGCS):

    - the scenario including various UE speeds, deployment scenarios, carrier frequency

- From the perspective of performance monitoring, it has been studied on boundary between Type 1 and 3 performance monitoring, and potential specification impact for each performance monitoring type 1,2 and 3.

Observation

Based on the evaluation for CSI prediction, the following high-level observations are provided:

- From the perspective of basic performance gain over benchmark of non-AI/ML based CSI prediction, under the same UE speed for training and inference

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 0%~7.8% gain depending on traffic model, in terms of mean UPT, is observed by 7 sources

  - for AI/ML based CSI prediction over non-AI/ML based CSI prediction, 3.8%~20.7% gain depending on traffic model, in terms of 5% UE UPT, is observed by 7 sources.

- From a perspective of AI/ML complexity, 19 sources adopt the model subject to the computational complexity in units of FLOPs from 0.05M to 3000M. For complexity of non-AI/ML benchmark, 16 sources adopt the algorithm (e.g., Kalman filter, Auto-regression, Wiener filter) subject to the computational complexity in units of FLOPs from 0.14M to 107M. The ratio of FLOPs (AI/ML over benchmark 2) ranges from 1 to 35, which is used by majority sources. For non-AI/ML benchmark, main computation complexity is dominated by filter updates, which may not be need to be updated per inference at the expense of performance loss.

- From the perspective of performance impact on channel estimation error and phase discontinuity, compared to non-AI/ML CSI prediction, higher gain is observed by 10 sources for AI/ML based CSI prediction in the presence of channel estimation error or phase discontinuity.

- From the perspective of generalization over various scenarios/configurations (e.g., various UE speed, deployment scenario, carrier frequency) that have been evaluated, compared to generalization Case 1 where the AI/ML model is trained with dataset subject to a certain scenario#B/configuration#B and applied for inference with a same scenario#B/configuration#B

  - For generalization Case 2 where the AI/ML model is trained with dataset from a different scenario#A/configuration#A, generalized performance may be achieved for some certain combinations of scenario#A/configuration#A and scenario#B/configuration#B but not for others.

  - For generalization Case 3 where the training dataset is constructed with data samples subject to multiple scenarios/configurations including scenario#B/configuration#B, generalized performance of the AI/ML model can be achieved.

Agreement

From RAN1 perspective, study of CSI prediction has been completed and performance improvement is observed with increased complexity.

Agreements in RAN1#117

Agreement

For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively.

- The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured.

Observation

For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843,

- Type 1

  - Definition/configuration of performance metric

  - Definition of threshold criterion, if configured

  - Definition/configuration and report of monitoring output, and corresponding report mechanism

- Type 2

  - Definition/configuration and report of ground truth CSI, and corresponding report mechanism.

- Type 3

  - Definition/configuration and report of performance metric, and corresponding report mechanism.

- For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

Agreement

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding training,

- The k-th local model is trained on region #B_k (the k-th local region), 1<=k<=N.

- The generalized model is trained on Region #A that may be constructed via any of the following methods that is appropriate for the given generalized/local region modeling approach.

  - Region #A is the same as the union of regions #B_1, …, #B_N.

  - Region #A is a proper superset of the union of regions #B_1, …, #B_N.

  - Region #A is generated separately from regions #B_1, …, #B_N.

  - Note: companies to report which method was used.

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, regarding testing,

- The trained generalized model, local model, and the non-AI/ML benchmark are tested on the regions #B_1, …, #B_N.

- In case N>1, when reporting the results, companies may report the performance of the generalized model, the local models, and the non-AI/ML benchmark, by averaging the performance over the regions #B_1,…,B_N. Companies to report the value of N.

Observation

- For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#1 of the nearest historical CSI, in terms of SGCS, from UE speed perspective,

  - If spatial consistency is not adopted, and if N4=1

    - For 10km/h UE speed, 1 source [Samsung] observes 9.55% gain

    - For 30km/h UE speed, 8 sources [Lenovo, Apple, Samsung, China Telecom, Spreadtrum, OPPO, CMCC, Qualcomm] observe 8.5%~20.6% gain and 2 sources [Intel, Fujitsu] observe 23.2%~31.65% gain.

    - For 60km/h UE speed, 3 sources [Qualcomm, China Telecom, Huawei] observe 2.8%~18% gain

  - If spatial consistency is adopted, and if N4=1

    - For 30km/h UE speed, 2 sources [Ericsson, ZTE] observes 17.83%~34.66% gain, 2 sources [vivo, MediaTek] observe 62.8%~76.4% gain

    - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 16.2~22% gain, 1 source [vivo] observes 90.6% gain

    - For 120km/h UE speed, 1 source [vivo] observes 68.2% gain

- If spatial consistency is not adopted, and if N4=4

    - For 30km/h UE speed, 1 source [OPPO] observes 19.7%~25.7% gain

- If spatial consistency is adopted, and if N4=4

    - For 10km/h UE speed, 1 source [Samsung] observes -1.61%~62.9% gain

    - For 30km/h UE speed, 1 source [Ericsson] observes 23%~34% gain, 1 source [MediaTek] observe 20.9%~76.4% gain

    - For 60km/h UE speed, 2 sources [Ericsson, MediaTek] observe 5.96%~-22% gain,

- If phase discontinuity is modelled, for 30km/h UE speed, 1 source [Fujitsu] observe 52.87% gain.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 20ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 8 sources [ZTE, Ericsson, Intel, vivo, Fujitsu, Samsung, CATT, MediaTek] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [Fujitsu] modelled phase discontinuity, and other sources do not consider phase discontinuity modelling.

  - 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

  - 2 sources [Ericsson, Intel] consider beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, 1 source [Samsung] considers per layer raw channel matrix after pre-processing, and other sources do not consider pre/post processing.

  - The performance metric is SGCS in linear value for layer 1.

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-1 of R1-2405492

Observation

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, compared to the Benchmark#2 of non-AI based CSI prediction, in terms of SGCS, from UE speed perspective

- If spatial consistency is not adopted, if N4=1

  - For 10km/h UE speed, 1 source [InterDigital] observes 3% gain

  - For 30km/h UE speed,

    - 6 sources [Qualcomm, Lenovo, Spreadtrum, Intel, CATT, OPPO] observe 0%~10.7% gain,

    - 2 sources [Fujitsu, InterDigital] observe 25.6%~48% gain

    - 1 source [Apple] observes -2% ~18% gain depending on filter complexity and filter update

  - For 60km/h UE speed,

    - 1 source [Qualcomm] observes -20.6% gain,

    - 2 sources [Huawei, CATT] observe 3.6%~7.8% gain,

    - 1 source [InterDigital] observes 42% gain

- If spatial consistency is adopted, if N4=1

  - For 30km/h UE speed,

    - 2 sources [Ericsson, MediaTek] observe 0.43%~5% gain,

    - 2 sources [vivo, ZTE] observe 2.54%~26.45%

  - For 60km/h UE speed,

    - 2 sources [Ericsson, MediaTek] observe -1%~3% gain,

    - 1 source [vivo] observes 48.8% gain

  - For 120km/h UE speed, 1 source [vivo] observes 20.6% gain

- If spatial consistency is not adopted, if N4=4

  - For 10km/h UE speed, 1 source [InterDigital] observes 1%~5% gain

  - For 30km/h UE speed, 1 source [InterDigital] observes -14%~48% gain,

  - For 60km/h UE speed, 1 source [InterDigital] observes -11%~42% gain

- If spatial consistency is adopted, if N4=4

  - For 30km/h UE speed,

    - 1 source [MediaTek] observes 0.43%~0.97% gain

    - 1 source [Ericsson] observes 5%~29% gain

  - For 60km/h UE speed,

    - 1 source [MediaTek] observes -1.01%~1.52% gain

    - 1 source [Ericsson] observes 3%~16% gain

- If phase discontinuity is modelled, for 30km/h UE speed, 2 sources [Fujitsu, vivo] observe 25.6% ~48.8% gain

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms or 5ms instance from the prediction output is considered for calculating the metric.

  - 6 sources [ZTE, Ericsson, Intel, vivo, Fujitsu, CATT] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 2 sources [Fujitsu, vivo] modelled phase discontinuity, and other sources do not consider phase discontinuity modelling.

  - 1 source [Qualcomm] considers eigenvector as model input, and other sources considers Raw channel matrix as model input.

  - 2 source [Ericsson, Intel] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources do not consider pre/post processing.

  - The performance metric is SGCS in linear value for layer 1.

- Note: N4 refers to the number of predicted CSI

- Note: Results refer to Table 2-3 of R1-2405492

Observation

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, in terms of mean UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark#1 of the nearest historical CSI:

  - For FTP traffic with low RU (RU<=39%)

    - For 30km/h UE speed and N4=1

      - 1 source [Ericsson] observes 9% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [Huawei, Ericsson] observe 1.2%~5% gain;

    - For 30km/h UE speed and N4=4

      - 1 source [Ericsson] observes 7% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 5% gain.

  - For FTP traffic with mid RU (40<=RU<=69%)

    - For 30km/h UE speed and N4=1

      - 1 source [Ericsson] observes 37% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observe 1.8%~3.5% gain;

      - 1 source [Ericsson] observes 21% gain.

    - For 30km/h UE speed and N4=4

      - 1 source [Ericsson] observes 29% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 21% gain.

  - For FTP traffic with high RU (RU>=70%)

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observe 2.5%~4.2% gain;

  - For full buffer traffic:

    - For 30km/h UE speed and N4=1

      - 1 source [Lenovo] observes 51% gain.

      - 1 source [Fujitsu] observes 27% gain.

      - 1 source [MediaTek] observes 8.7% gain.

    - For 30km/h UE speed and N4=4

      - 1 source [Fujitsu] observes 11.6% gain.

- Compared to the benchmark#2 of non-AI/ML based CSI prediction:

  - For FTP traffic, with low RU (RU<=39%)

    - For 30km/h UE speed and N4=1,

      - 1 source [Ericsson] observes 9% gain.

      - 1 source [CATT] observes -2.1% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observes 0.7%~0.9% gain;

      - 1 source [Ericsson] observes 11% gain.

      - 1 source [CATT] observes -6.1% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 13% gain.

      - 1 source [InterDigital] observes 2.1% gain.

      - 1 source [MediaTek] observes 0.02% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson, InterDigital] observes 13%

      - 1 source [MediaTek] observes 0.14% gain

      - 1 source [Ericsson, InterDigital] observes -13.1%

  - For FTP traffic, with mid RU (40<=RU<=69%)

    - For 30km/h UE speed and N4=1,

      - 1 source [Ericsson] observes 24% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observes 2.3%~3.1% gain;

      - 1 source [Ericsson] observes 31% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 35% gain.

      - 2 sources [MediaTek, InterDigital] observe -0.25%~2.5% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 32% gain.

      - 1 source [MediaTek] observes 0.25% gain.

      - 1 source [InterDigital] observes -19.6% gain.

  - For FTP traffic, with high RU (RU>=70%)

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observes 2%~2.5% gain;

    - For 30km/h UE speed and N4=4,

      - 2 sources [InterDigital, MediaTek] observes 0.11%~5.7% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [InterDigital] observes -17.2% gain.

      - 1 source [MediaTek] observes 0.92% gain.

  - For full buffer traffic:

    - For 30km/h UE speed and N4=1

      - 1 source [Lenovo] observes 24% gain.

      - 1 source [Fujitsu] observes 9.7% gain.

      - 2 sources [CATT, MediaTek] observe 0.6~1.2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 0.2% gain

      - 1 source [vivo] observes 20.6% gain

    - For 30km/h UE speed and N4=3 or 4

      - 1 source [Fujitsu] observes 7% gain.

      - 1 source [vivo] observes 24.7% gain with phase discontinuity modelling.

    - For 60km/h UE speed and N4=3

      - 1 source [vivo] observes 41.3% gain with phase discontinuity modelling.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 4 sources [Ericsson, Fujitsu, CATT, MediaTek] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-5/2-4/2-4/2-8 of R1-2405492

Observation

For the CSI prediction using UE-sided model, till the RAN1#117 meeting, in terms of 5% UE UPT, gains are observed compared to both Benchmark#1 of the nearest historical CSI and Benchmark#2 of a non-AI/ML based CSI prediction approach:

- Compared to the benchmark#1 of the nearest historical CSI:

  - For FTP traffic with low RU (RU<=39%)

    - For 30km/h UE speed and N4=1

      - 1 source [Ericsson] observes 27% gain.

    - For 60km/h UE speed, and N4=1

      - 2 sources [Huawei, Ericsson] observe 4.5%~18.3% gain;

    - For 30km/h UE speed and N4=4

      - 1 source [Ericsson] observes 14% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 4% gain.

  - For FTP traffic with mid RU (40<=RU<=69%)

    - For 30km/h UE speed and N4=1

      - 1 source [Ericsson] observes 100% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observe 8.6%~11.3% gain;

      - 1 source [Ericsson] observes 45% gain.

    - For 30km/h UE speed and N4=4

      - 1 source [Ericsson] observes 77% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 26% gain.

  - For FTP traffic with high RU (RU>=70%)

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observe 13.4%~17.8% gain;

  - For full buffer traffic:

    - For 30km/h UE speed and N4=1

      - 1 source [Lenovo] observes 10.5% gain.

      - 1 source [Fujitsu] observes 9.9% gain.

      - 1 source [MediaTek] observes 7.7% gain.

    - For 30km/h UE speed and N4=4

      - 1 source [Fujitsu] observes 2.1% gain.

- Compared to the benchmark#2 of non-AI/ML based CSI prediction:

  - For FTP traffic, with low RU (RU<=39%)

    - For 30km/h UE speed and N4=1,

      - 1 source [Ericsson] observes 18% gain.

      - 1 source [CATT] observes 50.1% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observes 2.5%~5.2% gain;

      - 1 source [Ericsson] observes 17% gain.

      - 1 source [CATT] observes 47.1% gain

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 23% gain.

      - 1 source [InterDigital] observes 5.7% gain.

    - For 60km/h UE speed, and N4=4

      - 2 sources [Ericsson, InterDigital] observe 9.5%~% 19%gain.

  - For FTP traffic, with mid RU (40<=RU<=69%)

    - For 30km/h UE speed and N4=1,

      - 1 source [Ericsson] observes 46% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observes 7%~8.6% gain;

      - 1 source [Ericsson] observes 66% gain.

    - For 30km/h UE speed and N4=4,

      - 1 source [Ericsson] observes 73% gain.

      - 1 source [InterDigital] observes 6.8% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [Ericsson] observes 56% gain.

      - 1 source [InterDigital] observes 27.2% gain.

  - For FTP traffic, with high RU (RU>=70%)

    - For 60km/h UE speed, and N4=1

      - 1 source [Huawei] observes 6.7%~14.8% gain;

    - For 30km/h UE speed and N4=4,

      - 1 source [InterDigital] observes 28.6% gain.

    - For 60km/h UE speed, and N4=4

      - 1 source [InterDigital] observes 27.2% gain.

  - For full buffer traffic:

    - For 30km/h UE speed and N4=1

      - 3 sources [Lenovo, Fujitsu, CATT] observe 0.2%~6% gain.

      - 1 source [MediaTek] observes -2% gain.

    - For 60km/h UE speed, and N4=1

      - 1 source [CATT] observes 0.4% gain

    - For 30km/h UE speed and N4=3 or 4

      - 1 source [Fujitsu] observes 6.3% gain.

      - 1 source [vivo] observes 81.9% gain with phase discontinuity modelling.

    - For 60km/h UE speed and N4=3

      - 1 source [vivo] observes 48.9% gain with phase discontinuity modelling.

- Note: the above results are based on the following assumptions

  - The observation window considers to start as early as 15ms~50ms.

  - A future 4ms ~ 20ms instance from the prediction output is considered for calculating the metric.

  - Raw channel matrix is considered as model input

  - The performance metric is mean UPT for Max rank 1.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency, and other sources do not consider spatial consistency.

  - 4 sources [Ericsson, Fujitsu, CATT, MediaTek] consider realistic channel estimation, and other sources consider ideal channel estimation.

  - 1 source [vivo] are modelled phase discontinuity, and other sources do not consider phase discontinuity

- Note: N4 refers to the number of predicted CSI instances

- Note: Results refer to Table 2-5/2-4/2-4/2-8 of R1-2405492

Observation

For the generalization verification of CSI prediction using UE sided model over various UE speeds, till the RAN1#117 meeting, compared to the generalization Case 1 where the AI/ML model is trained with dataset subject to a certain UE speed#B and applied for inference with a same UE speed#B,

- For generalization Case 2, generalized performance may be achieved for some certain combinations of UE speed#A and UE speed#B but not for others:

  - If UE speed#B is 10 km/h and

    - UE speed#A is 30 km/h or 120km/h, 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -5.5% degradation.

    - UE speed#A is 60 km/h, 1 source [CMCC] observes -26.79% degradation.

  - If UE speed#B is 30 km/h and

    - UE speed#A is 60km/h, 4 sources [Huawei, CATT, Ericsson, vivo] observe a generalized performance of -11.4%~-2.7% degradation and 1 source [CMCC] observes a generalized performance of -25.6%~-32% degradation.

    - UE speed#A is 10km/h or 120km/h, 3 sources [CMCC, MediaTek, Fujitsu] observe a generalized performance of -51.5%~-72.37% degradation.

  - If UE speed#B is 60 km/h and

    - UE speed#A is 30km/h, 1 source [CATT] observes a generalized performance of -1.9% degradation and 5 sources [Huawei, Fujitsu, MediaTek, Ericsson, vivo] observe a generalized performance of -28.6%~-56.3% degradation

  - If UE speed#B is 120 km/h and

    - UE speed#A is 30km/h, 1 source [MediaTek] observes a generalized performance of -56.3% degradation

- For generalization Case 3, generalized performance of the AI/ML model can be achieved in general (0%~-3.63% loss) for UE speed#B subject to any of 10 km/h, 30 km/h, 60 km/h and 120 km/h, if the training dataset is constructed with data samples subject to multiple UE speeds including UE speed#B, as observed by 6 sources [Fujitsu, Huawei, CATT, vivo, Apple, Ericsson]

  - If UE speed#B is 10 km/h

    - 2 sources [Fujitsu, MediaTek] observe a generalized performance of less than -0.2% degradation, and 1 source [CMCC] observes -16.87% degradation.

  - If UE speed#B is 30 km/h

    - 7 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson, MediaTek] observe a generalized performance of less than -2.77% degradation, 1 source [CMCC] observes a generalized performance of -15.44% degradation.

  - If UE speed#B is 60 km/h and

    - 6 sources [Huawei, CATT, vivo, Apple, Fujitsu, Ericsson] observe a generalized performance of less than -3.63% degradation, 2 sources [CMCC, MediaTek] observe a generalized performance of -6.77%~-13.5% degradation.

  - If UE speed#B is 120 km/h and

    - 1 source [MediaTek] observes a generalized performance of -43.6% degradation

  - Note: For generalization Case 3, 2 sources [CMCC, MediaTek] observe performance degradations (-13.5%~-43.6% loss) for UE speed#B subject to 10 km/h, 30 km/h, 60 km/h, 120 km/h, but compared with generalization Case 2, in general the performance are still improved.

- Note: the above results are based on the following assumptions besides the assumptions of the agreed EVM table

  - Raw channel matrix is used as the model input.

  - The performance metric is SGCS in linear value for layer 1/2/3/4.

  - 3 sources [vivo, Ericsson, MediaTek] consider spatial consistency. Other sources do not consider spatial consistency.

  - 1 source [Ericsson] considers beam-delay domain transformation/antenna-frequency domain transformation as pre/post processing, and other sources considers no pre/post processing.

- Note: Results refer to Table 3-2 of R1-2405492

Agreements in RAN1#116bis

Agreement

- For the AI/ML based CSI prediction, adopt following assumptions as a baseline for evaluation purpose

  - UE speed: 30km/h, 60km/h

    - Others can be additionally submitted, e.g., 10km/h, 120km/h

  - Observation window (number/distance): 5/5ms,10/5ms

    - Others can be additionally submitted, e.g., 4/5ms, 15/5ms

  - Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance):  1/5ms/5ms, 4/5ms/5ms

    - Others can be additionally submitted, e.g., 2/5ms/5ms, 3/5ms/5ms, 1/5ms/10ms

  - For other assumptions, reuse Rel-18 baseline

Agreement

- For the AI/ML based CSI prediction, for CSI report, adopt following as a baseline for evaluation purpose

  - N4 value: 1, 4e

    - Others can be additionally submitted, e.g., 2, 8

  - paramCombination-Doppler-r18: 6,7 or paramCombination -r16 = 5,6 (for Benchmark 1)

    - Others can be additionally submitted.

    - Note: The same selected parameter combination shall be applied for benchmarks.

  - CSI report periodicity: 5ms, 20ms (encouraged)

    - Others can be additionally submitted, e.g., 10ms

Conclusion

Consider error modelling in TR36.897 Table A.1-2 as a baseline if channel estimation error is modeled.

  - Other modelling is not precluded, and companies should report how to model channel estimation error if other modelling is considered.

Conclusion

If phase discontinuity is modeled, it is modelled as a uniform distribution between -ϕmax,ϕmax within a time window o, where ϕmax=40 degrees and Twindow=20ms can be a baseline.

- Other modelling is not precluded, and companies should report how to model phase discontinuity if other modelling is considered, and additional Twindow.if adopted

Conclusion

For the phase discontinuity modelling, it is clarified that

- A fixed phase for all CSI-RS observations within the time window, and another fixed phase for the next time window. The phases are according to uniform distribution.

Conclusion

- For evaluation of the UE-sided model based CSI prediction, UE distribution of (80% indoor, 20% outdoor) can be optionally simulated.

Note: Indoor speed is 3 km/h, outdoor speed is chosen from the following options: 30 km/h, 60 km/h. Assumption on O2I car penetration loss and spatial consistency follow the Rel-18 AI/ML based CSI prediction

Agreement

For the results template used to collect evaluation results for UE -sided model based CSI prediction, adopt Table 6 used in Rel-18 as starting point with the following addition:

- Assumption

  - UE distribution (Baseline: 100% outdoor, Optional: 80% indoor, 20% outdoor)

  - Whether/how channel estimation error is modelled

  - Whether/how phase discontinuity is modelled

  - Methods used to handle the phase discontinuity (if applied)

- Benchmark 2

  - FLOPs/M

  - Details of complexity calculation, e.g., complexity of prediction and complexity of filter update

Agreement

- For the results template used to collect evaluation results for UE-sided model based CSI prediction using localized models, adopt Table 6 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model:

- Dataset description

  - Local region modelling: e.g., Option 1 or Option 2, and further details

  - Temporal modelling: e.g., how temporal variation is modelled in train and test sets

  - Dataset description for generalized model

Agreement

For the UE-sided model based CSI prediction, for optional evaluation using AP CSI-RS, consider following assumption on observation window (number/distance)

- Observation window: 12/2ms, 8/2ms, 4/2ms

- Others can be additionally submitted

Agreement

For AI/ML based CSI prediction, at least for inference, legacy CSI-RS configuration can be a starting point. Further study on whether there is a need for specification enhancement.

Agreement

- At least for inference, for UE-sided model based CSI prediction, legacy feedback mechanism using codebook type set to “typeII-Doppler-r18” is a starting point of discussion. Study the necessity and potential specification impacts including at least following aspects:

- CSI processing criteria and timeline

Agreement

For performance monitoring for functionality-based LCM, further study on details of type 1,2 and 3, e.g., potential specification impact, pros/cons aspects.

- To clarify the boundary between type 1 and type 3

- To clarify definition of monitoring output and performance metric

Agreements in RAN1#116

Agreement

For Rel-19 study on CSI prediction, consider EVM agreed in Rel-18 CSI prediction based on UE-sided model as a starting point.

- FFS on additional assumptions, e.g., channel estimation error, phase discontinuity, CSI-RS periodicity.

- Note: Rel-18 CSI-RS configuration/reporting can be reused.

- Note: additional EVM and corresponding template to collect the results can be updated.

Agreement

For Rel-19 study on CSI prediction, companies are encouraged to evaluate throughput performance by comparing performance with non-AI/ML based CSI prediction.

- R18 eType II doppler codebook is assumed for CSI report for both AI/ML and Non AI/ML prediction.

- Companies to report the assumption for N4, which could be 1, 2, 4, 8.

Note: Non-AI/ML based CSI prediction (Benchmark 2) can include statistical model based CSI prediction (e.g., based on Kalman filter, Wiener filter, Auto-regression).

Agreement

For evaluation, to report computational complexity in unit of FLOPs including additional complexity if applicable, e.g., update of filter, and their assumption on non-AI based CSI prediction when performance results are provided.

Conclusion

For the evaluation of the AI/ML based CSI prediction, it is up to companies to choose the modelling method and companies should report if ‘Channel estimation’ and/or ‘phase discontinuity’ is/are considered by companies.

Agreement

For the evaluation of the AI/ML based CSI prediction, consider following CSI-RS configuration

- Periodic: 5 ms periodicity (baseline), 20 ms periodicity (encouraged)

- Aperiodic: Optional, CSI-RS burst with K resources and time interval m slots (based on R18 MIMO eType-II)

Note: Companies to report observation window (number/distance) and prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance) on their evaluation.

Conclusion

For Rel-19 study on CSI prediction only, consider UE-sided model only.

Agreement

- For CSI prediction evaluations, to verify the generalization/scalability performance of an AI/ML model over various configurations, to evaluate one or more of the following aspects:

- Various UE speeds (e.g., 10km/h, 30km/h, 60km/h, 120km/h)

- Various deployment scenarios

- Various carrier frequencies (e.g., 2GHz, 3.5GHz)

- Various frequency granularity assumptions

- Various antenna port numbers (e.g., 32 ports, 16 ports)

- To report the selected configurations for generalization verification

- To report the method to achieve generalization over various configurations and/or to achieve scalability of the AI/ML input/output, including pre-processing, post-processing, etc.

- To report generalization cases where multiple aspects (e.g., combination of above) are involved in one dataset, if adopted.

- To report the performance and requirement (e.g., updating filter parameters, convergence of filter) for non-AI/ML-based CSI prediction to handle the various scenarios/configurations.

Agreement

For the evaluation of AI/ML-based CSI prediction using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region:

- Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901.

    - E.g., Dropped in a specific cell or within a specific boundary.

- Option 2: By using a scenario/configuration specific to the local region.

    - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc.

Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset.

Reference

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm

- R1-2503232	Discussion on CSI Processing Unit for AI/ML-based CSI prediction	FUTUREWEI

- R1-2503246	AI/ML for CSI prediction	Ericsson

- R1-2503253	Discussion on AIML for  CSI prediction	Huawei, HiSilicon

- R1-2503349	Remaining issues on specification support for CSI prediction	vivo

- R1-2503448	Discussion on AI-based CSI prediction	TCL

- R1-2503507	Discussion on AIML for CSI prediction	Spreadtrum, UNISOC

- R1-2503552	Views on AI/ML based CSI prediction	Samsung

- R1-2503650	Discussion on specification support for AI CSI prediction	ZTE Corporation, Sanechips

- R1-2503749	Specification support for CSI prediction	Quectel

- R1-2503772	Discussion on AI/ML-based CSI prediction	CATT

- R1-2503822	Discussion on AI/ML for CSI prediction	CMCC

- R1-2503874	Further discussion on remained issues for AI/ML model based CSI prediction	Xiaomi

- R1-2503922	Discussion on specification support for CSI prediction	NEC

- R1-2503952	Discussion on specification support for CSI prediction	Ruijie Networks Co. Ltd

- R1-2503976	On AI/ML-based CSI prediction	InterDigital, Inc.

- R1-2503982	Discussions on CSI prediction	LG Electronics

- R1-2503988	Discussion on AI/ML-based CSI prediction	Panasonic

- R1-2503998	Specification support for AI-enabled CSI prediction	NVIDIA

- R1-2504026	AI/ML based CSI Prediction	Google

- R1-2504041	Specification support for CSI prediction	Lenovo

- R1-2504060	Specification support for UE-side AI/ML CSI prediction model monitoring	Sony

- R1-2504082	Discussion on specification support for CSI prediction	Fujitsu

- R1-2504094	Discussion on AI/ML for CSI prediction	HONOR

- R1-2504114	AI/ML for CSI Prediction	Nokia

- R1-2504131	Discussion on specification support for CSI prediction	ETRI

- R1-2504225	On specification for AI/ML-based CSI prediction	OPPO

- R1-2504259	AI/ML - Specification support for CSI Prediction	MediaTek Inc.

- R1-2504310	Discussion on AI based CSI prediction	Apple

- R1-2504386	Specification support for CSI prediction	Qualcomm Incorporated

- R1-2504466	Discussion on specification support for AI/ML based CSI prediction	Sharp

- R1-2504493	Discussion on AI/ML for CSI prediction	NTT DOCOMO, INC.

- R1-2504597	Discussion on specification support for CSI prediction	CEWiT

- R1-2504649	Discussion on AI/ML CSI prediction	Continental Automotive