3GPP TSG-RAN WG1 Meeting #122	R1-2505213

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	10.3.2

Source:	Huawei, HiSilicon

Title:	Study of air interface for A-IoT device 2b/C

Document for:	Discussion and Decision

# Introduction

In Rel-20 Ambient IoT [1], due to the inclusion of support for active devices, outdoor scenarios with larger coverage requirements and DO-A, enhancements to the general aspects of the physical layer based on the Rel-19 air interface needs to be identified and studied.

(Copied from RP-251884 [1]) A-IoT Devices These are the A-IoT Devices referred to in the objectives: - Device 1: As standardized in the Rel-19 WI Ambient_IoT_Solutions. ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither R2D nor D2R amplification in the device. The device’s D2R transmission is backscattered on a carrier wave provided externally. - Device 2b: ≤ a few hundred µW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - Device C: ≤ 1 mW to ≤ 10 mW peak power consumption, has energy storage, IF envelope detector receiver or ZIF receiver, initial sampling frequency offset (SFO) up to 10Y ppm, R2D and/or D2R amplification in the device. The device’s D2R transmission is generated internally by the device. - SFO value 10Y is assumed to be better than any SFO value considered for Device 1 standardized in Rel-19. Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

In this contribution, the general aspects of the physical layer design for Rel-20 Ambient IoT are discussed, including physical modulation and waveform, multiple access, and channel coding, physical channels and signals and the necessary physical procedures.

# The overview of the air interface for device 2b/C

According to the SID [1], there are 3 key aspects that would require necessary and feasible changes to the Rel-19 air interface to support Rel-20 A-IoT:

- 1) Coverage enhancement aspects for outdoor scenarios

- 2) Necessary changes to support the active device

- 3) Necessary physical procedure for DO-A

The air interface design of Rel-19 is the baseline of the Rel-20 air interface design. The following table summarizes the parts that need enhancements based on the above three key aspects. Those parts that need  changes compared with the Rel-19 basis are shown in the table with the “red” mark. The detailed discussions are shown in the following sections.

Table 1. The necessary changes for Rel-20 air interface on the basis of Rel-19 air interface

|  |  | Rel-19 Ambient IoT |  | Rel-20 Ambient IoT Outdoor (Changes in Red) |  | Motivation for changes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | R2D | D2R | R2D | D2R |  |
| General PHY aspects | Time resource definition | Chip <-> OOK symbol | Chip <-> modulated symbol | Chip <-> OOK symbol further study | Chip <-> modulated symbol further study | 2) Active device 3) DO-A |
|  | Frequency resource definition | 1/2/3/4 RBs | 15/30/60/120/240/480/960/2880kHz | 1/2/3/4 RBs | further study | 1) Coverage enhancement for outdoor 2) Active device |
|  | Waveform | OFDM-based OOK | Non-OFDM single carrier | OFDM-based OOK | Non-OFDM single carrier |  |
|  | Modulation | OOK | OOK/BPSK | OOK | Only BPSK | 1) Coverage enhancement for outdoor |
|  | Multiple access | TDMA | TDMA&FDMA | TDMA | TDMA&FDMA without SFS | 1) Coverage enhancement for outdoor 2) Active device |
|  | Line coding | Manchester | Manchester/square wave for small frequency shift | Manchester | No need to support line coding | 2) Active device |
|  | Scrambling | None | None | None | further study | 1) Coverage enhancement for outdoor |
|  | FEC/Repetition | None | 1/3 CC without interleaving Block repetition {1, 2} | 1) Support FEC 2) Support repetitions | 1) Support lower code rate CC 2) Support larger block repetition numbers 3) further study interleaving | 1) Coverage enhancement for outdoor |
|  | CRC | len-6/len-16 |  | len-6/len-16 |  |  |
| PHY channel and signal | Channel | One channel, PRDCH | One channel, PDRCH | PRDCH | PDRCH | 2) Active device 3) DO-A |
|  | Signal | SIP+CAP Postamble | Preamble (Len-7 and len-31 m sequence) Midamble | 1) Preamble for on-demand block 2) Periodic block for synchronization, containing a periodic preamble and periodic PRDCH 3) SIB-like transmission 4) Periodic signal for CFO calibration 5) No SIP, CAP and postamble | 1) Longer Preamble 2) Comb-like midamble | 1) Coverage enhancement for outdoor 2) Active device |
| PHY procedures | Cell search | None |  | Periodic transmission block Introduce cell identifier |  | 1) Coverage enhancement for outdoor 2) Active device 3) DO-A |
|  | Random access | Paging triggered RACH(DT or DO-DTT traffic) |  | Paging triggered RACH(DT or DO-DTT traffic) RACH without paging triggered (DO-A traffic) |  | 2) Active device 3) DO-A |
|  | Power control | None |  | Open-loop power control for D2R |  | 1) Coverage enhancement for outdoor 2) Active device 3) DO-A |
|  | Scheduling | R2D scheduling info determined by postamble and/or higher layer D2R grant carried in R2D Msg1 resource determined by paging message |  | R2D scheduling info carried by L1 D2R grant carried in MAC Msg1 resource determined by the system information |  | 1) Coverage enhancement for outdoor 2) Active device 3) DO-A |
|  | Timing relationship | R2D to D2R: Toffset-x D2R to R2D: TD2Rmin |  | R2D to D2R: Toffset-x D2R to R2D: TD2Rmin Reader indicates time offset |  | 2) Active device |
|  | Measurement | None |  | Support, Define measurement quantity such as RSRP |  | 1) Coverage enhancement for outdoor 2) Active device |

# R2D general aspects

## R2D waveform detection method

The Rel-19 R2D waveform detection method is based on falling or rising edge detection. The device relies on regular transition edges provided by line coding to keep the chip-level time tracking due to the large SFO level assumed for device 1 e.g., 104~105 ppm. While edge detection can provide an adequate performance for indoor scenarios, however, for larger distances in outdoor scenarios, the required SNR for the receiver is far lower than the indoor scenarios, and it has to be studied whether edge detection can be used to achieve the coverage distances for outdoor scenarios.

Based on our analysis, assuming a Rel-19 air interface with no FEC, no repetitions and a chip length M = 2, devices using edge detection show that it can successfully receive the R2D transmission in the high SNR region, as seen in Figure 1. This would result in edge detection being able to be used only for limited small coverage scenarios, and would not be sufficient to handle the outdoor scenarios.

Observation 1: Devices using edge detection can successfully receive R2D transmissions only within a limited coverage distance since it works only for high SNRs.

One alternative to using edge detection is that the device can utilize the correlation method in which the accumulated power of each chip is used to detect information. However, for the device to perform correlation detection, it requires high clock accuracy to obtain the accurate samples of each chip, and hence, the SFO needs to be smaller, e.g., 10 ppm. Since the initial clock accuracy for device 2b is already defined in the TR 38.769 [2] and the residual SFO value is quite high (103 – 104 ppm), it would result in the device not being able to successfully detect the PRDCH. It means that device 2b cannot use correlation detection, but would have to use edge detection for the waveform detection and maintain the chip-level timing tracking using the transition edge. This would result in device 2b not being able to achieve the coverage distances for outdoor scenarios since the transition edge will become more and more blurry when the SNR becomes lower.

On the other hand, device C has a higher power consumption and is capable of supporting a higher accuracy clock. This would allow device C to operate the correlation detection method based on its small SFO level after the clock calibration, e.g., 10 ppm, thereby enabling it to achieve larger coverage distances since the correlation method is not limited by the detection of the transition edge, but by the accumulated power of each chip. As seen in Figure 1, device C using correlation detection can successfully detect the R2D transmissions at a lower SNR region, translating into it being able to achieve the larger coverage distances for outdoor scenarios.

Observation 2: While device C can utilize a high accuracy clock to support correlation detection for large coverage distances, device 2b can use only edge detection and not correlation detection due to the clock accuracy value being low, which results in the device not being able to achieve the coverage distances for outdoor scenarios.

Figure 1. Performance comparison of device C with M = 2 using the correlation detection method and device 2b using the edge detection method

Proposal 1: For R2D waveform detection, the correlation detection method is supported for the Rel-20 device C.

## R2D bandwidth

Considering convenient and flexible deployment, especially at an early-stage of the study, a minimum transmission bandwidth of 1 PRB can be regarded as the baseline for Rel-20 R2D transmissions. It is desirable to study designing a system that makes the minimum requirement on the spectrum, to help swift deployment of the Rel-20 Ambient IoT. In order to achieve this, at least the transmission bandwidth of the initial access signal of one cell e.g., the periodic block for synchronization, can be defined as 1 PRB with the minimum overhead for the limited spectrum.

Proposal 2: Support the use of 1 PRB as the baseline R2D transmission bandwidth for Rel-20.

In addition to TDMA, FDMA may be considered for R2D transmission for active device since ZIF receiver filter is supported at device side. From device perspective, it can distinguish the different PRB located on different frequencies. However, due to the CFO exists at device side, guard-band is needed for R2D transmission. Whether there is sufficient frequency resource allocated for R2D transmission to support FDMA in practical deployment also needs to be taken into consideration.

## R2D modulation

### OOK-4 M value

Rel-19 R2D transmissions are based on the DFT-s-OFDM waveform with OOK-4 modulation. An M value set of {2, 6, 12, 24} was adopted for the PRDCH in Rel-19 and the minimum M value is M = 2 since there was no need to adopt a smaller M value as it satisfied the indoor coverage requirements for Rel-19. Thus, with M = 2, the SNR will be higher than 10.0 dB to achieve BLER 10% for a Rel-19 device using edge detection for indoor scenarios. As seen in Figure 2, the performance of M = 2 for the Rel-20 device C using the correlation detection method is 10.0 dB to achieve BLER 10%, which still may not be sufficient for the coverage enhancements to cater to the outdoor scenarios.

Thus, for the Rel-20 device C using the correlation detection method, it is natural to additionally introduce the longer chip length choice with M = 1 to improve the R2D performance to be able to larger coverage distances. For the following figure, the performance for M = 2 and M = 1 using the Rel-20 correlation detection method is compared and it can be observed that the application of M = 1 yields ~3dB performance gain with 10% BLER compared with M = 2.

Figure 2. Performance comparison of the M = 1 and M = 2 using the correlation detection method.

Observation 3: The introduction of M = 1 provides ~3 dB performance gain compared with M = 2 with the Rel-20 correlation detection method, enabling the device to increase its coverage distances.

Proposal 3: M = 1 is additionally supported for Rel-20.

## R2D coding

### FEC

For Rel-19, FEC is not supported for R2D since the FEC decoder usually requires complicated arithmetic or logical operations and cache memory of a certain size which is difficult for device 1 to implement. Furthermore, the distance for Rel-19 indoor scenario is very limited with only 10m-50m and the FEC is not necessary.

As discussed in section 3.3.1, the introduction of M = 1 could enhance the R2D transmission performance, but the R2D will still need the further coverage enhancement for the outdoor coverage requirement which is necessary to achieve a receiver sensitivity ≤ -100 dBm. In order to satisfy the requirement, FEC needs to be introduced in R2D.

Observation 4: FEC can improve the coverage for the Rel-20 R2D transmission.

For the Rel-20 device C, the device C is defined with ≤ 1 mW to ≤ 10 mW peak power consumption. This allows active devices to support the decoder of 3GPP FEC codes such as convolutional codes (CC) and polar codes. For example, as demonstrated in [3], the power consumption for the CC Viterbi decoder can be limited within 100-200 μW.

Observation 5: It is feasible for the Rel-20 device C with higher power consumption to perform FEC decoding which is not feasible for device 1 in Rel-19.

In the following figure, the R2D performance with 1/3 TBCC and M = 1 and with only M = 1 using the correlation detection method for the device C are analysed. It can be observed that the 1/3 TBCC can provide ~4.0 dB performance gain and the SNR can achieve the SNR requirement for the maximum coverage target.

Figure 3. Performance comparison of 1/3 TBCC and no FEC for Rel-20 R2D

Proposal 4: FEC is additionally supported e.g., 1/3 TBCC for Rel-20 device C compared with Rel-19.

If FEC is adopted for the R2D transmission of Rel-20, interleaving can also be considered to further obtain the time diversity gain and enhance the R2D coverage. Especially for large TBS transmissions, interleaving can be expected to demonstrate more diversity gain and provide a robust resistance for the burst channel fading. However, the de-interleaving procedure for the device needs additional power consumption and storage for the device. Hence, the feasibility of interleaving for the R2D FEC also needs further study.

Proposal 5: For R2D FEC interleaving, the feasibility of it being implemented on the device and its necessity based on the achieved performance gain is further studied.

### Repetition

Besides the smaller M value and the FEC techniques discussed in section 3.3.1 and section 3.4.1, repetitions can also be a candidate technique to be supported to further enhance the R2D coverage for the outdoor coverage requirement.

Proposal 6: R2D repetition is additionally supported compared with Rel-19.

The repetition technique includes block repetition and bit repetition. For the block repetition, the device is required to store the received blocks, combine all these blocks and decode them to obtain the repetition gain. However, for bit repetition, such block storage is not required. Meanwhile, the block can obtain the time diversity gain and may expect more performance than the bit repetition. Thus, the feasibility for this storing, combining operations and performance gains for the active device needs to be studied in Rel-20.

Proposal 7: Type of repetition used, i.e., bit or block repetition, is further studied for Rel-20.

## R2D signals

### Preamble for on-demand block

In Rel-19, the device was designed to obtain the starting time of the PRDCH using the R-TAS, which comprised of the SIP, to determine the start of the R2D transmission, and the CAP, to determine the chip length to be used for the PRDCH. They were designed as simple ON-OFF transmissions for the device to perform edge detection discussed in section 3.1, since the Rel-19 device 1 is extremely low-power device with only ~1μW peak power consumption and could support only edge detection.

However, for Rel-20 active devices, which are aimed to support the outdoor scenarios with a larger coverage distance, the required receiver sensitivity is far lower than the Rel-19 device 1 e.g., -30dBm. In this case, it would be difficult for devices using edge detection to achieve the required performance for larger coverage distances due to the weaker received signal power.

As can be seen in [4], when considering device 2b using the edge detection method to detect the Rel-19 R-TAS, an FDR performance of ≤ 1% can only be achieved for an SNR of ~12 dB for M = 2 for the indoor scenarios which is still insufficient to cater to outdoor scenarios in Rel-20. If the same R-TAS design is used for a device C using correlation detection, the required SNR is 17.0 dB to achieve MDR ≤ 1% and FAR ≤ 1%, which is also not sufficient for the coverage enhancement for outdoor scenarios. It means that the Rel-19 R-TAS design is unsuitable for Rel-20 active devices catering to outdoor scenarios and necessitates the redesign of the R2D preamble.

Table 1. Performance of Rel-19 R-TAS using sequence-correlation detection (using M = 2 CAP as an example)

| SNR (dB) | MDR | FAR |
| --- | --- | --- |
| 17.0 | 0.98% | 0.97% |

Observation 6: Rel-19 R-TAS design needs to be redesigned to achieve the coverage distances for outdoor scenarios, since it would require the device to support a weaker received signal power for which edge detection is not sufficient.

In order to overcome this issue, the Rel-20 device C can use correlation detection by correlating the adjacent two chips with the Manchester codeword to detect the information bit, which is suitable for the low-power received signal. However, as discussed in section 3.1, correlation detection may not be suitable for device 2b which assumes a higher value the residual SFO, resulting in the device being able to perform only edge detection.

On the other hand, device C is capable of supporting correlation detection because in Rel-20, it is being discussed in [5] that the residual of CFO is ~10 ppm which also means the SFO for device C is 10 ppm with the same original clock, significantly improving the clock accuracy. With device C defined to have a higher peak power consumption, they can support the sequence correlation operations, which means that an R2D preamble using a sequence-based design can be considered for the device to determine the start of an R2D transmission.

Observation 7: It is possible for device C to support an R2D preamble using a sequence-based design due to its increased power consumption and improved clock accuracy being able to support correlation detection.

Figure 4. Diagram of Rel-20 on-demand R2D transmission block

In order to design the new sequence-based R2D preamble, based on the discussions for the D2R preamble in Rel-19, a 0/1 binary sequence can be used for the definition of an R2D preamble, where a Golay- or m- sequence can be considered with a good auto-correlation characteristic for the sequence detection.

In order to increase the transmission efficiency in outdoor scenarios, different sequence lengths catering to different coverage levels can be considered for the data transmission of PRDCH. Thus, a maximum 256-length sequence corresponding to the larger coverage distance can be considered. For a shorter coverage distance, a shorter length sequence e.g., 64-length can also be considered. The following table shows the performance of the example design (based on the Golay sequence) for a preamble length of 256 and 64.

Table 2. Performance of on-demand preamble signal with the 0/1 binary sequence

| Sequence length | SNR (dB) | MDR | FAR |
| --- | --- | --- | --- |
| 256 | 4.0 | 1.00% | 0.99% |
| 64 | 8.2 | 0.90% | 0.99% |

In order to differentiate the R2D preamble used for the PRDCH transmission carrying data from the periodic preamble used in the periodic block for synchronization, described in the next section, we label the R2D preamble used for the PRDCH transmissions as an on-demand preamble.

Observation 8: The 256 and 64-length binary sequence can achieve an SNR performance of 4.0 dB and 8.2 dB with the MDR ≤ 1% and FAR ≤ 1% respectively.

Proposal 8: For the R2D preamble, a binary sequence preamble, e.g., Golay- or m- sequence, is supported. Use of different sequence lengths for different coverage levels is studied.

### Periodic block for synchronization

Figure 5. Diagram of the periodic transmission block of Rel-20 R2D

The Rel-20 active devices are applied for outdoor scenarios and will be required to perform the cell search procedure discussed in the section 5.1. This would require the device to receive system information from a periodic transmission that not only contains cell information to stay at one specific cell to communicate, but also provides time synchronization. The reasons are listed below:

- The active device has its internal LO frequency and has the capability to calibrate its LO frequency and achieve the corresponding clock accuracy. Thus, the periodic time synchronization can be applied for the active device to calibrate the time accuracy and determine the RO position and the scheduling offset for other signals and transmissions, such as a SIB-like transmission, with limited timing error.

- Apart from the cell information, the device needs to obtain the timeline related information of the system e.g., frame number and slot number, in order to perform the initial access procedure, be aware of the timeline and determine ROs with the information configured in the system information.

- Since DO-A traffic is supported in Rel-20, the device needs to operate the periodic time domain synchronization to maintain the clock accuracy and operate the initial access through the RO time position.

Observation 9: For the Rel-20 active devices to handle outdoor scenarios, they require a periodic transmission that would provide not only system information for the devices to perform the cell search procedure, but also provide them with time domain synchronization in order to determine the timeline of the initial access procedure and ROs for other signals and transmissions.

Structure of periodic sync block

The proposed periodic block for synchronization contains two parts – a periodic preamble, which provides timing information, and a periodic PRDCH, which contains the necessary system level information for the device to perform the cell search procedure, described in Section 5.1.

- Periodic preamble: The periodic preamble enables the device to calibrate its clock accuracy and the device can determine the RO position and the scheduling offset with limited timing error. For example, the device C can utilize the periodic preamble to calibrate its clock accuracy to keep the ~10 ppm residual SFO level. The periodic preamble is transmitted with a fixed period and the device detects it according this fixed period, which is different from the on-demand preamble discussed in section 3.5.1. They can differ not only in periodicity, but also in the sequence used, where a different sequence can be used for the periodic preamble such that the device can differentiate between an on-demand and periodic preamble.

- Periodic PRDCH: After detecting the periodic preamble, the device needs to obtain the necessary system information e.g., MIB of the cell to continue the cell search procedure. For this, the reader needs to transmit a periodic PRDCH carrying the MIB to the devices such that they can obtain the necessary cell search information. Same as the periodic preamble, the periodic PRDCH is also transmitted with a fixed period.

Design of periodic preamble

As discussed in section 3.1, an increase in the power consumption allows device C to perform the sequence correlation operation, while device 2b is limited to edge detection. Similar to the on-demand R2D preamble described in section 3.5.1, a periodic preamble signal based on the sequence is used to allow the device to correct the timing error within a predefined time period. A similar binary sequence as described in section 3.5.1 can be used for defining the periodic preamble signal, where a Golay- or m- sequence can be considered with a good performance. Meanwhile, the cross-correlation of the periodic preamble signal and the on-demand preamble signal should also be considered by using different length sequences, since the device needs to know whether the subsequent PRDCH carries the system information e.g., MIB or the on-demand PRDCH information.

Design of periodic PRDCH

The periodic PRDCH can contain the cell identifier (ID), the system frame number (SFN), the potential time and frequency resource allocation information for the SIB-like transmission discussed in section 3.5.3 and other necessary system information such as the power control parameters. The periodic PRDCH can be transmitted immediately after the periodic preamble and the interval is also fixed which is the same as the period of the periodic preamble.

Proposal 9: Support the use of a periodic block for synchronization, containing a periodic preamble and a periodic PRDCH for the Rel-20 device C.

### SIB-like transmission for DO-A

The necessary initial system parameters are signalled by the periodic block discussed in section 3.5.2. However, since DO-A traffic is supported in Rel-20, the device needs additional resource allocation related information for it to determine the resources to be used for the initial D2R transmission for DO-A. Since the information signalled by the periodic block is not sufficient for the device to determine resources for its initial transmission, the solution is that the reader transmits a SIB-like transmission to the device which contains the necessary parameters for the DO-A transmission. This is different from that of the information provided in the periodic block because while the information in the periodic block provides the device with system information to handle different traffic types, the SIB-like transmission provides information for the DO-A traffic alone, and hence is transmitted separately.

Observation 10: Apart from the periodic block for synchronization, in order to support DO-A traffic, the reader also needs to provide the device with resource allocation information of the ROs for its initial D2R transmission for DO-A.

The SIB-like transmission can contain the necessary DO-A procedure parameters, such as:

- Time and frequency resource allocation of the RO for the device to transmit its initial transmission for DO-A,

- Cell-specific power control parameters,

- Absolute radio frequency channel number (ARFCN) -like frequency points information for D2R,

- RSRP threshold/level for the R2D measurement and other necessary parameters for DO-A and the active device.

The SIB-like transmission can also be transmitted with a fixed periodicity, but the period can be different from the periodic block for synchronization, e.g., a larger period can be used for the SIB-like transmission since the cell-level parameters in the SIB may change less frequently.

Figure 6. Different periodicities for the periodic block for synchronization and the SIB-like transmission

Proposal 10: Support the use of a SIB-like transmission, separate from the periodic block for synchronization and with a different periodicity, which provides the device with resource allocation information for its initial D2R transmission for DO-A.

### Periodic signal for CFO calibration

In Rel-19, for device 1, the D2R transmission is based on backscattering using an external carrier wave and the device did not have the LO component. In this case, the RF frequency of the D2R transmission has no CFO since the LO of the CW node is accurate. Thus, for device 1, the signal for CFO calibration is not required for the transmission of D2R. However, for active devices in Rel-20, the D2R transmission is based on the internal carrier wave and the device owns the LO component. Hence, if the initial CFO of the LO is not calibrated with a large value, the overhead of the guard-band for D2R transmission is large. For example, the guard-band for an initial CFO of 100 ppm is 90 kHz at the two sides of D2R transmission, resulting in a 180 kHz guard band, with a target LO frequency of 900 MHz, resulting in low resource utilization efficiency.

Observation 11: Since active devices use a LO, if the initial CFO of the LO is not calibrated, the overhead of a guard-band for D2R transmissions is large.

Thus, apart from the periodic block for synchronization, yet another signal is required by the active devices in order to calibrate the LO frequency of the device and reduce the residual carrier CFO to an integer value of approximately a few tens of ppm, e.g., 10 ppm.

For this purpose, we introduce a CFO calibration signal that will reduce the residual CFO, which causes increased frequency offset of the D2R transmission and hence increases the guard-bandwidth of D2R transmission. Since the device may use an IF/ZIF envelope detector for receiving R2D transmissions, the R2D demodulation performance is not impacted by the CFO.

The CFO calibration signal is used for the LO frequency calibration for active devices. The LO frequency calibration cannot rely on the baseband clock to fulfil the calibration procedure and the calibration signal should be utilized for the RF-end clock. Thus, a baseband sequence cannot be assumed as the CFO calibration signal and, rather, a single-tone signal which occupies one RE is supported for the analog circuit tracking and locking its RF frequency. At the baseband side, this single-tone signal is a continuous ON signal.

As demonstrated above, the main functionality of the CFO calibration signal is for the D2R transmission with a limited guard-band overhead. Hence, the time domain position for the periodic signal for CFO calibration can be close to the D2R transmission.

Proposal 11: Support the use of a periodic CFO calibration signal for Rel-20 with the following characteristics:

- Single-tone periodic signal.

- Transmitted close to the D2R transmission.

### L1 control and postamble

In Rel-19, the R2D postamble was used to indicate the end of the R2D transmission to the device. Although Rel-19 supports the indication of the TBS via higher layer signalling as well, it is not signalled using L1 control because there were no other parameters considered for L1 signalling. Since in Rel-19, the device 1 determines the M value according the CAP of the R-TAS and FEC/repetition is not used for the R2D. However, as discussed in section 3.4.1 and 3.4.2, FEC and repetition would be required for the Rel-20 R2D transmission for coverage enhancement. This would require the R2D MCS-like information including the FEC code rate, the repetition number and M value to be indicated to the device to decode the R2D data accordingly. Thus, L1 control information with a separate CRC is supported to indicate the MCS-like information to the device before the device detect the payload of the PRDCH.

Proposal 12: L1 control information with a separate CRC is supported for the Rel-20 R2D transmission.

However, if L1 control information is adopted for the R2D transmission, then the TBS indication can also be included in the L1 control information. Hence, the postamble is not necessarily needed for the Rel-20 R2D transmission.

Figure 7. Diagram of L1 control information and payload of PRDCH

Proposal 13: R2D postamble is not necessarily supported for the Rel-20 R2D transmission if the L1 control information includes the TBS indication field.

## Summary of R2D

Similar to Rel-19, only one physical channel, e.g., physical reader-to-device channel, PRDCH, is supported for R2D in Rel-20. However, based on the discussion above, the procedure of R2D generation is different from that in Rel-19, which is summarized as the following.

- Correlation detection is used for R2D waveform detection in Rel-20.

- Channel coding and repetition technique are additionally introduced in Rel-20.

- M = 1 for OOK-4 modulation is additionally introduced in Rel-20, with a minimum R2D bandwidth of 1 PRB.

- The preamble for on-demand block, the periodic block for synchronization containing the periodic preamble and periodic PRDCH, the SIB-like transmission and the periodic signal for CFO calibration is introduced for Rel-20.

- The L1 control information with a separate CRC is introduced for Rel-20. If the L1 control information includes the TBS indication, R2D postamble is not necessary to be supported.

# D2R general aspects

## D2R modulation and waveform

### D2R signal generation

According to [1], device C has the ability to generate the D2R transmission internally. Therefore, the generation of D2R signals is different between Rel-19 and Rel-20. The detailed procedure of D2R signal generation in Rel-19 and Rel-20 is shown in Figure 9 and Table 3.

Figure 9. The difference of D2R signal generation in Rel-19 and Rel-20.

Table 3. The difference of D2R signal generation in Rel-19 and Rel-20.

|  | Rel-19 | Rel-20 |
| --- | --- | --- |
| Frequency shift | - By using the line coding, e.g., Manchester coding, with different value of SFS factor R | - Reader can indicate different target frequency for different devices |
| CW generation | - Using the external CW | - Based on the center frequency of the cell (f0) and the frequency offset (f1) scheduled for the device, device can use LO to generate CW, i.e., ej2πf0+f1t, internally |

Observation 12: The procedure of D2R signal generation between Rel-19 and Rel-20 is different as stated in Table 3.

### D2R multiple access

In R19, SFS (Small Frequency Shift) is applied to realized FDMA, which means FDMed devices should use different R values.

As shown in Figure 9 and Table 3, device C has the ability to generate the CW internally. In this case, reader can indicate different frequency shift values for FDMed devices. Then, the device can use the scheduled specific frequency offset to generate the CW, and SFS is not needed.

As shown in Figure 10, devices should use different R values to realize FDMA in Rel-19. For example, R = 1 for device #1, R = 4 for device #2, and R = 8 for device #3.

In Rel-20, devices can realize FDMA directly by changing the frequency of the CW with scheduled frequency offset. For example, no frequency offset is used by device #1, frequency offset f1 is used by device #2, and frequency offset f2 is used by device #3.

Figure 10. The comparison of FDMA between Rel-19 and Rel-20.

Proposal 14: SFS (Small Frequency Shift) is not supported for device C in Rel-20. FDMA is supported as follows without relying on SFS:

- Device can adjust to target frequency indicated by the reader based on CW generated from LO.

### D2R waveform

The D2R waveform based on the design in Rel-19 in frequency domain is shown in Figure 11(a) for DSB transmission. It can be observed that there are two main lobes located on either side of the carrier frequency. The transmission bandwidth is defined as Btx,D2R = 2 / (R * Tchip ) = 4 / Tb, where Tb is the time duration of the D2R bit.

(a) Waveform in frequency domain in Rel-19		  (b) Waveform in frequency domain in Rel-20

Figure 11. Waveform in frequency domain in Rel-19 and Rel-20.

The waveform shown in Figure 11(a) is generated using Manchester line coding. According to the discussion in Rel-19, the main functions of Manchester line coding is to avoid the CW interference and realize FDMA. As discussed in section 4.1.1 and 4.1.2, since the CW can be generated by the device internally, CW interference no longer exists for device C and FDMA can be realized without relying on SFS. Therefore, line coding is no longer needed for D2R transmission in Rel-20 for device C.

Proposal 15: Line coding is not supported for D2R transmission in Rel-20 for device C.

The D2R waveform in frequency domain for device C is different from that in Rel-19. A shown in Figure 11(b), there only exists one main lobe in the frequency domain. The bandwidth of the main lobe is 15 kHz, , then Tb is 133.33 us, i.e., Btx,D2R = 2 / Tb. The D2R baseband signal of R19 and R20 in time domain are shown in Figure 12.

Figure 12. D2R baseband signal of R19 and R20 in time domain.

Observation 13: DSB transmission can be supported for device C in Rel-20 D2R transmission with only one main lobe exists in the frequency domain.

### D2R modulation

In Rel-19, both BPSK and OOK are supported for D2R transmission and it is up to device implementation to use BPSK or OOK. Such scheme is workable due to indoor with limited short coverage distance. However, it is not suitable for Rel-20 outdoor. BPSK has better performance gain than OOK. Only BPSK is suitable for outdoor macro cell edge. It can also be used in cell center and no need for extra modulation type configuration. So only BPSK is preferred for Rel-20 outdoor.

Proposal 16: Only BPSK is supported for D2R transmission for device C in Rel-20.

## D2R bit length and bandwidth

### The impact of CFO on bandwidth

According to [5], the carrier frequency offset (CFO) needs to be considered for device C. To avoid interference between devices caused by CFO, the bandwidth allocated for D2R transmission for each device should be larger than the transmission bandwidth. Therefore, the guard band needs to be considered.

As shown in Figure 13, when considering CFO, the bandwidth allocated for D2R transmission can be represented by the Occupied bandwidth, and Occupied bandwidth = Transmission bandwidth + Guard bandwidth. Note that the guard band is needed for both side of the transmission band since the CFO can be either positive or negative.

Figure 13. Transmission bandwidth and occupied bandwidth when considering CFO = 10 ppm.

Observation 14: Guard band needs to be considered for D2R transmission of device C in Rel-20 due to CFO.

### Bit length

In Rel-19, the candidate D2R bit length set is {266.67, 133.33, 66.67, 33.33, 16.67, 8.33, 4.17, 1.39} us, and the corresponding D2R transmission bandwidth is {15, 30, 60, 120, 240, 480, 960, 2880} kHz calculated by Btx,D2R =  4 / Tb. As discussed in section 4.1.3, in Rel-20, the transmission bandwidth is calculated as Btx,D2R =  2 / Tb with no line coding.

The smaller the D2R bit length, the larger the D2R transmission bandwidth. For example, when assuming the CFO is 10 ppm, the guard bandwidth for one side should be no less than 9 kHz when the carrier frequency is 900 MHz based on the discussion in section 4.2.1. Then, as shown in Figure 14, 5 FDMed devices can be supported in 180 kHz when Btx,D2R  = 15 kHz. If the Btx,D2R  can be reduced as 10 kHz, 6 FDMed devices can be supported in 180 kHz. Therefore, more devices can be FDMed within the same channel bandwidth when the D2R transmission bandwidth is smaller.

Based on the discussion above, the candidate value of D2R bit length for device C in Rel-20 can be further studied.

Figure 14. FDMA with transmission bandwidth 10 kHz or 15 kHz.

Proposal 17: The candidate value of D2R bit length for device C in Rel-20 needs further study.

## D2R FEC enhancement

Sub-block interleaving is not supported in Rel-19 D2R transmission since the peak power consumption of device 1 is limited as ~ 1 µW. The peak power consumption of device C in Rel-20 is much higher than that of device 1 in Rel-19. Therefore, device C has the ability to support 1/3 TBCC with sub-block interleaving in terms of power consumption. Sub-block interleaving can provide time diversity gain, and can be further studied.

Proposal 18: Study interleaving for Rel-20 device C D2R transmission.

For code rate, only 1/3 TBCC is supported in Rel-19. Since CC with lower code rate can further improve the performance (See Figure 15), lower code rate can be further supported in Rel-20.

Figure 15. Performance comparison with 1/3 TBCC and 1/4 TBCC

Proposal 19: Lower code rate e.g., 1/4 code rate is additionally supported for Rel-20 device C D2R transmission on the basis of Rel-19.

## D2R Repetition

Block repetition with repetition number {1, 2} is supported in Rel-19 [7]. The performance using block repetition with repetition number Rep = 2 is shown in Figure 16. Since the coverage distance target in Rel-19 is much lower than that in Rel-20, larger block repetition number can be applied to further improve the performance. It can be observed from Figure 16 that the performance with repetition number Rep = 4 has ~2.5 dB gain compared with that of repetition number Rep = 2 at BLER 10% and has ~3 dB gain compared with that of repetition number Rep = 2 at BLER 1%.

Figure 16. Block repetition performance comparison

Proposal 20: Larger block repetition number is additionally supported for Rel-20 device C D2R transmission on the basis of Rel-19.

## D2R scrambling

Since larger coverage distance needs to be supported in outdoor scenario in Rel-20 [1], more devices are required to be connected with the reader. Then, the interference caused by other Ambient IoT devices, including inter-cell interference and intra-cell interference, will be more serious than that in Rel-19. In order to mitigate the impact of interference, scrambling can be studied to randomize inter-cell or intra-cell interference, especially for repetition case.

Figure 17. Example of D2R scrambling

Proposal 21: Study scrambling for Rel-20 device C D2R transmission.

## D2R signals

### Preamble

Preamble with length-7 and length-31 m-sequence are supported in Rel-19 [8] for time synchronization and channel estimation. However, the coverage distance requirement for Rel-19 in indoor scenario is very limited, and the coverage distance target in Rel-20 is much higher. Therefore, the length of the preamble sequence needs to be increased to prevent time synchronization from becoming a bottleneck.

According to [5], the CFO after calibration can be 10 ppm for device C. Longer preamble can also help reader performing CFO estimation more accurately.

The performance using different preamble length is shown in Figure 18. It can be observed that increasing the length of the preamble from 31 bits to 511 bits can provide 3.5 dB gain at BLER 10% and 5.2 dB gain at BLER 1%.

Figure 18. Performance comparison with different preamble length

Proposal 22: Longer preamble is additionally supported for Rel-20 device C D2R transmission on the basis of Rel-19 considering CFO.

### Midamble

Block-wise midamble with the same length and sequence as the preamble is supported in R19 [8]. The minimum supported value of the interval between consecutive midambles, and between the preamble and the first midamble, is 48 bits with Tb = 266.67 us. To overcome CFO impact, a new type of midamble, e.g., Type-2 midamble, needs to be supported in Rel-20 for device C. The main difference between Type-2 midamble in Rel-20 and midamble in Rel-19 is that Type-2 midamble is not block-wise as in Rel-19, but comb-like.

Figure 19. Block structure with Type-2 midamble.

Firstly, comb-like means denser insertion (i.e., shorter period). The interval between consecutive Type-2 midambles, and between the preamble and the first Type-2 midamble should be much smaller than that in Rel-19. The reason is that considering 10 ppm CFO at device C, i.e., almost 9 kHz at 900M carrier frequency, preamble cannot provide an ideal estimate of the CFO at reader side. Then, Type-2 midambles need to help reader track and estimate the residual CFO to ensure that channel estimation is not affected by the residual CFO. For reference, the CFO considered in the NB-IoT system is 0.2 ppm, which is about 180 Hz at 900M carrier frequency. And DMRS is inserted every 7 OFDM symbols to track and estimate the CFO, which means the time interval between consecutive DMRS is nearly ~ 0.5 ms. Therefore, much smaller time interval between Type-2 midambles for device C is needed.

Secondly, Type-2 midamble can be shorter than the midamble in Rel-19. In Rel-19, the length of the midamble is the same as preamble. Based on the discussion in section 4.6.1, longer preamble needs to be supported in Rel-19. Then, if the Type-2 midamble are still required to keep the same length as preamble, the overhead of amble signals is too large. Therefore, shorter Type-2 midamble can be considered. Besides, the channel estimation will not be affected by the shorter Type-2 midamble because more than one Type-2 midamble can be jointly used for channel estimation.

Proposal 23: Comb-like midamble with shorter insertion time interval and shorter length is supported for Rel-20 device C for D2R transmission.

## Summary of D2R

Similar to Rel-19, only one physical channel, e.g., physical device-to-reader channel, PDRCH, is supported for D2R in Rel-20 for device C. Based on the discussion above, the procedure of D2R generation is different from that in Rel-19, which is summarized in Figure 20.

- Channel coding with interleaving can be studied in Rel-20, and it should be done before the block repetition. Besides, lower code rate is supported.

- Block repetition should be done after the channel coding in Rel-20 since the scrambling sequences for different blocks can be different. Besides, larger block repetition number is supported.

- A new D2R generation step, e.g., scrambling, can be studied in Rel-20.

- A new type of midamble, e.g., Comb-like Type2-midamble, is introduced with shorter insertion time interval and shorter length for Rel-20.

- Line coding and SFS is not supported in Rel-20.

- BPSK is the only supported modulation method in Rel-20.

Figure 20. Comparison of D2R generation steps between Rel-19 and Rel-20

# Analysis of necessary physical procedures

## Cell search procedure

In Rel-20, the active devices are expected to support outdoor scenarios where the devices could be communicating with multiple readers, and it would need to determine the cell it would be using for the intended communication with a given reader. This would require the device to perform a cell search procedure, which is detailed below.

The device is expected to be provided with a pre-defined sync raster set within which it blindly searches for the sync raster, within which the periodic block for synchronization is transmitted. When the device successfully detects the periodic preamble in one sync raster, then the device locates this frequency point and determines the central frequency of its receiver and detects the following periodic PRDCH. The device then obtains the cell ID and SFN information carried by the periodic PRDCH, as well as information on where to expect the SIB-like transmission. The periodicity of the periodic block for synchronization is fixed and is known to the device before operating the cell search procedure, enabling the device to detect the next periodic transmission block according to this fixed period.

The device then determines the time start of the SIB-like transmission as indicated by the periodic block. On receiving this SIB-like transmission, the device can determine the RO resource allocation information for it to transmit its initial transmission for DO-A. The detailed RO time and frequency resource information is demonstrated in section 5.2. The device then operates the RACH procedure according the information indicated by the SIB information.

Figure 21. Basic cell search procedure diagram of the active device

Proposal 24: Support the introduction of a cell search procedure, where the device uses the periodic block for synchronization to obtain resource information to receive the SIB-like transmission, which contains information for the ROs that the device can use for its initial DO-A transmission.

## Random access procedure and scheduling transmission for DO-A traffic

Traffic types with DO-DTT and DT are considered in Rel-19, and the new traffic type with DO-A is considered in Rel-20 [1]. The comparison of the random-access procedure related to DO-A and DO-DTT is shown in Figure 22.

Figure 22. Comparison of random access with DO-DTT and DO-A

The resource of Msg 1 with DO-DTT is carried by the Paging message or Access Trigger message. However, for DO-A, there is no preceding R2D transmission to provide resources for Msg1. Hence, the resource allocated of Msg 1, which can be called as Random Access Occasion (RO), is carried by a periodically broadcast R2D message which is referred to in Section 3.5.3 as the SIB-like transmission. A set of ROs will be allocated, and the device should choose the appropriate RO to send Msg 1. These ROs are required only for the initial transmission for DO-A traffic, because every subsequent D2R transmission will be preceded by a R2D transmission, which can carry the resource allocation information for the subsequent D2R transmission, similar to what is being done in Rel-19.

Proposal 25: The SIB-like transmission containing information of the ROs to be used for DO-A is required to only provide information for the initial transmission, all subsequent transmissions are scheduled by the preceding R2D transmission, similar to Rel-19.

Figure 23. Example of ROs

As shown in Figure 23, the RO includes time resource and frequency resource information. As for time resource information, as shown in Figure 24, it refers to the start time of each RO, which is related to the time offset between the start of the RO and the beginning of the dedicated frame or slot, and the periodicity of ROs should be indicated. For frequency resource information, a set of frequency resources will be provided and the device can select one of them randomly.

Figure 24. Example of RO’s time resource

Another important issue to be addressed is that the coverage distance in Rel-20 is much larger than that of Rel-19. In order to allow devices with varying distances from the reader realize random access efficiently, multiple coverage levels can be defined and each coverage level corresponds to different resource allocation of ROs, as seen in Figure 25. For example, devices located far from the reader can be configured with lower code rate and higher block repetition number.

Proposal 26: The random-access procedure with DO-A traffic is different from that with DO-DTT/DT in Rel-19 with following characteristics:

- R2D periodic block for synchronization and a SIB-like transmission are broadcast periodically.

- Parameters related to the time and frequency resources of ROs for Msg1 for the initial transmission for DO-A is carried by the SIB-like transmission.

- Device selects appropriate RO from the indicated set of ROs to send Msg1.

- Different coverage levels have different ROs.

## Scheduling Aspects

### R2D scheduling

For PRDCH, as analyzed in section 3.4.1, FEC code is supported and different code rates for different coverage levels can be considered. Thus, it is necessary for the device to know the FEC code rate or the MCS-like information previously before detecting the PRDCH. Thus, the L1 control information with the separate CRC can be supported, as mentioned in section 3.5.5. The L1 control information would indicate TBS, FEC code rate, M value and the potential repetition number to the device, and the device can use this information to detect the subsequent PRDCH. The fields included in the L1 control information can be seen as the following table, with example values:

Table 4. The example design of L1 control information in Rel-20

| Field | Example indicator information |
| --- | --- |
| [a] FEC code rate | {1, 1/3 and other code rates} |
| [b] Repetition number | {1, 2, 4, …} |
| [c] TBS | 7 bits (byte-level ≤1000 bits) |
| [d] M value | {1, 2, 6, …} |

Proposal 27: L1 control indication is supported for Rel-20 device C R2D scheduling.

### D2R scheduling

The D2R scheduling information is indicated by the MAC layer in Rel-19. This can be reused in R20.

## Power control

D2R transmission relies on backscatter in Rel-19, therefore power control cannot be achieved. Power control is also not necessary in R19 since it’s indoor scenario and the coverage distance is very small.

In Rel-20, device’s D2R transmission is generated internally by the device, therefore open-loop power control can be supported in D2R transmission for mitigating interference from other devices. For example, the D2R received power of the device which located near the reader will be higher than that of the device which located far from the reader when no power control is performed. In this case, the interference caused by the former will be extremely severe for the latter. By performing open-loop power control, the D2R transmit power is determined by the pathloss between reader and device. Then, device with poor channel condition can increase the transmit power and the device with small path loss can decrease the transmit power. As a result, the interference caused by this “near-far” effect can be mitigated.

In order to estimate the pathloss between the reader and the device, device needs to perform the RSRP measurement based on the periodic block for synchronization and some parameters related to the power control such as the RSRP threshold should be indicated by the reader.

Proposal 28: Open-loop power control is supported for Rel-20 device C D2R transmission.

## Measurement

In this subsection, we analyze the feasibility of performing measurements by device C.

As shown in TR 38.869 [9], for RRM serving cell measurements performed in LP-WUS, an ADC of at least 4 bits is required to satisfy the AGC accuracy and RSRP/RSSI measurement accuracy. As analyzed in [10], a multi-bit ADC (<6-bit) with a sampling rate of ~1 MHz will consume about 14 μW power, which cannot be supported by devices with ~1 μW peak power consumption.

In addition, as given in [11], a data-aided scheme is used to estimate the RSRP/RSRQ effectively based on M1V estimator, where the M1V estimator is based on the first moment (M1) and the first central moment (V) of the received baseband signal sequence corresponding to the preamble and received baseband signal rk with

M1=Erk=2σ2+A2                                                       (Eqn. 1)

V=Erk2-Erk2=4σ4+4σ2A2                                             (Eqn. 2)

where A is the amplitude of the signal and σ2 is the noise variance. Then, we have

- RSRP A2=M1-2σ2 where 2σ2=M1-M12-V

- RSRQ ρ=A2/2σ2

From the equations above, RSRP or RSRQ calculation at least contains squaring and rooting steps and multipliers are the necessary component for this. For the device C, the adders and comparators for the baseband processing can be afforded and the multiplier can also be afforded as the digital part can significantly impact the chip size and power consumption. Therefore, measurement is feasible for device C.

Proposal 29: Measurement is supported for device C.

## Time relationship

### Time offset between R2D and corresponding D2R

In Rel-19, the time offset between R2D and corresponding D2R, e.g., TR->D, is limited by the clock frequency accuracy of device 1 since the SFO can reach up to 105 ppm. In Rel-20, the clock frequency of device C is much more accurate than that of device 1. For example, the SFO of device C is only 10 ppm after calibration according to [5]. Therefore, reader can indicate TR->D, i.e., scheduling delay, for increasing the flexibility of scheduling. The device can follow that indication based on a more accurate clock.

Proposal 30: The time offset between R2D and corresponding D2R can be indicated by the reader.

# Conclusions

In this contribution, the general aspects of physical layer and physical procedures for Rel-20 Ambient IoT outdoor are discussed and following observations and proposals are made accordingly.

Observation 1: Devices using edge detection can successfully receive R2D transmissions only within a limited coverage distance since it works only for high SNRs.

Observation 2: While device C can utilize a high accuracy clock to support correlation detection for large coverage distances, device 2b can use only edge detection and not correlation detection due to the clock accuracy value being low, which results in the device not being able to achieve the coverage distances for outdoor scenarios.

Observation 3: The introduction of M = 1 provides ~3 dB performance gain compared with M = 2 with the Rel-20 correlation detection method, enabling the device to increase its coverage distances.

Observation 4: FEC can improve the coverage for the Rel-20 R2D transmission.

Observation 5: It is feasible for the Rel-20 device C with higher power consumption to perform FEC decoding which is not feasible for device 1 in Rel-19.

Observation 6: Rel-19 R-TAS design needs to be redesigned to achieve the coverage distances for outdoor scenarios, since it would require the device to support a weaker received signal power for which edge detection is not sufficient.

Observation 7: It is possible for device C to support an R2D preamble using a sequence-based design due to its increased power consumption and improved clock accuracy being able to support correlation detection.

Observation 8: The 256 and 64-length binary sequence can achieve an SNR performance of 4.0 dB and 8.2 dB with the MDR ≤ 1% and FAR ≤ 1% respectively.

Observation 9: For the Rel-20 active devices to handle outdoor scenarios, they require a periodic transmission that would provide not only system information for the devices to perform the cell search procedure, but also provide them with time domain synchronization in order to determine the timeline of the initial access procedure and ROs for other signals and transmissions.

Observation 10: Apart from the periodic block for synchronization, in order to support DO-A traffic, the reader also needs to provide the device with resource allocation information of the ROs for its initial D2R transmission for DO-A.

Observation 11: Since active devices use a LO, if the initial CFO of the LO is not calibrated, the overhead of a guard-band for D2R transmissions is large.

Observation 12: The procedure of D2R signal generation between Rel-19 and Rel-20 is different as stated in Table 3.

Observation 13: DSB transmission can be supported for device C in Rel-20 D2R transmission with only one main lobe exists in the frequency domain.

Observation 14: Guard band needs to be considered for D2R transmission of device C in Rel-20 due to CFO.

Proposal 1: For R2D waveform detection, the correlation detection method is supported for the Rel-20 device C.

Proposal 2: Support the use of 1 PRB as the baseline R2D transmission bandwidth for Rel-20.

Proposal 3: M = 1 is additionally supported for Rel-20.

Proposal 4: FEC is additionally supported e.g., 1/3 TBCC for Rel-20 device C compared with Rel-19.

Proposal 5: For R2D FEC interleaving, the feasibility of it being implemented on the device and its necessity based on the achieved performance gain is further studied.

Proposal 6: R2D repetition is additionally supported compared with Rel-19.

Proposal 7: Type of repetition used, i.e., bit or block repetition, is further studied for Rel-20.

Proposal 8: For the R2D preamble, a binary sequence preamble, e.g., Golay- or m- sequence, is supported. Use of different sequence lengths for different coverage levels is studied.

Proposal 9: Support the use of a periodic block for synchronization, containing a periodic preamble and a periodic PRDCH for the Rel-20 device C.

Proposal 10: Support the use of a SIB-like transmission, separate from the periodic block for synchronization and with a different periodicity, which provides the device with resource allocation information for its initial D2R transmission for DO-A.

Proposal 11: Support the use of a periodic CFO calibration signal for Rel-20 with the following characteristics:

- Single-tone periodic signal.

- Transmitted close to the D2R transmission.

Proposal 12: L1 control information with a separate CRC is supported for the Rel-20 R2D transmission.

Proposal 13: R2D postamble is not necessarily supported for the Rel-20 R2D transmission if the L1 control information includes the TBS indication field.

Proposal 14: SFS (Small Frequency Shift) is not supported for device C in Rel-20. FDMA is supported as follows without relying on SFS:

- Device can adjust to target frequency indicated by the reader based on CW generated from LO.

Proposal 15: Line coding is not supported for D2R transmission in Rel-20 for device C.

Proposal 16: Only BPSK is supported for D2R transmission for device C in Rel-20.

Proposal 17: The candidate value of D2R bit length for device C in Rel-20 needs further study.

Proposal 18: Study interleaving for Rel-20 device C D2R transmission.

Proposal 19: Lower code rate e.g., 1/4 code rate is additionally supported for Rel-20 device C D2R transmission on the basis of Rel-19.

Proposal 20: Larger block repetition number is additionally supported for Rel-20 device C D2R transmission on the basis of Rel-19.

Proposal 21: Study scrambling for Rel-20 device C D2R transmission.

Proposal 22: Longer preamble is additionally supported for Rel-20 device C D2R transmission on the basis of Rel-19 considering CFO.

Proposal 23: Comb-like midamble with shorter insertion time interval and shorter length is supported for Rel-20 device C for D2R transmission.

Proposal 24: Support the introduction of a cell search procedure, where the device uses the periodic block for synchronization to obtain resource information to receive the SIB-like transmission, which contains information for the ROs that the device can use for its initial DO-A transmission.

Proposal 25: The SIB-like transmission containing information of the ROs to be used for DO-A is required to only provide information for the initial transmission, all subsequent transmissions are scheduled by the preceding R2D transmission, similar to Rel-19.

Proposal 26: The random-access procedure with DO-A traffic is different from that with DO-DTT/DT in Rel-19 with following characteristics:

- R2D periodic block for synchronization and a SIB-like transmission are broadcast periodically.

- Parameters related to the time and frequency resources of ROs for Msg1 for the initial transmission for DO-A is carried by the SIB-like transmission.

- Device selects appropriate RO from the indicated set of ROs to send Msg1.

- Different coverage levels have different ROs.

Proposal 27: L1 control indication is supported for Rel-20 device C R2D scheduling.

Proposal 28: Open-loop power control is supported for Rel-20 device C D2R transmission.

Proposal 29: Measurement is supported for device C.

Proposal 30: The time offset between R2D and corresponding D2R can be indicated by the reader.

# References

[1] RP-251884, “New SID on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#108, June, 2025.

[2] 3GPP TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”, December 2024.

[3] C. Desset and A. Fort, “Selection of channel coding for low-power wireless systems”, The 57th IEEE Semiannual Vehicular Technology Conference, VTC 2003-Spring.

[4] R1-2502235, “Physical signals design”, RAN1#120bis, Wuhan, China, Huawei, HiSilicon.

[5] R1-2505212, “Evaluation for active A-IoT device in outdoor scenario”, RAN1#122, Bengaluru, India, Huawei, HiSilicon.

[6] R1-2501623, “9.4.1 Final FL summary for Ambient IoT “9.4.1 Physical channels design-modulation aspects.”, RAN1#120, Huawei, HiSilicon.

[7] R1-2504750, “Summary #2 for coding aspects of physical channel design”, RAN1#121, CMCC.

[8] R1-2503123, “Final FL Summary on timing acquisition & synchronization for Ambient IoT”, RAN1#120bis, Apple.

[9] 3GPP TR 38.869, “Study on low-power wake-up signal and receiver for NR (Release 18)”, December 2023.

[10] R1-2210909, “Discussion on architecture of LP-WUS receiver”, RAN1#111, Toulouse, France, Huawei, HiSilicon.

[11] V. Sridharan, “Interference cancellation techniques for low power wireless sensors”.

# Appendix

The simulation assumptions of the R2D and D2R is shown in the following tables.

Table 6. The simulation assumptions of the R2D

| Channel model | TDL-C 300ns |
| --- | --- |
| UE velocity | 3 km/h |
| SFO | 10 ppm for device C; 103 ppm for device 2b |
| TBS | 20 bits |
| CRC | 6 bits |
| FEC | None or 1/3 TBCC |
| Line coding | Manchester |
| Repetition | None |
| Waveform | OFDM based OOK-4 M = 1 or 2 |

Table 7. The simulation assumptions of the D2R

| Channel model | TDL-C 300ns |
| --- | --- |
| UE velocity | 3 km/h |
| CFO and SFO | Device C: 10 ppm CFO and 10 ppm SFO |
| TBS | 96 bits |
| CRC | CRC-16 |
| FEC | 1/3 TBCC with or without interleaving |
| Line coding | None |
| Block repetition factor | 2 or 4 |
| Preamble length | 31 or 127 or 511 |