3GPP TSG-RAN WG1 Meeting #122	R1-2505230

Bengaluru, India, August, 25th-29th, 2025

Agenda Item:	8.12.2

Source:	Huawei, HiSilicon

Title:	Maintenance of Rel-19 low band CA via switching

Document for:	Discussion and Decision

Introduction

Based on the WI description in RP-250247 [1], the following objective has been agreed on low band carrier aggregation via switching:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In the last meeting, RAN1 discussed several issues and reached some agreements in [2]. Further discussion on Rel-19 low-band CA via switching is provided in this contribution.

Discussion

Regarding the collision handling between switching pattern and existing signal/channel, no matter how the switching pattern is designed, the impact of Rx switching on contention-based random access (CBRA) is inevitable. In legacy CBRA procedure, it is up to UE to select the preamble resource and time occasion to start the procedure, for both UE autonomous PRACH and PDCCH-order PRACH. Upon receiving a preamble, it is impossible for gNB to tell whether the preamble comes from a legacy UE or a Rel-19 UE configured with the switching pattern. As a result, the resource scheduling in the RACH procedure must be no difference between legacy UEs and the Rel-19 new UEs. Additionally, from UE perspective, it is meaningless to ask a UE to monitor SCell PDCCH or receive SCell PDSCH according to switching pattern when the UE is under RACH procedure due to out of synchronization or RRC reestablishment. Therefore, the semi-static switching pattern cannot be applied to Rel-19 UE’s CBRA procedure.

The simplest approach is to apply an exception on the semi-static pattern for CBRA. After UE sends out the preamble, the switching pattern should be temporarily suspended and the UE monitor Msg2/MsgB in the same way as a legacy UE does. The switching pattern can be resumed when the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

Proposal:  For R19 low band switching, if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

Conclusion

In this contribution, the proposal is summarized as the following:

Proposal:  For R19 low band switching, if a RACH procedure is triggered by a UE itself or by network signalling, the semi-static switching pattern, if configured, is temporarily suspended until the last DL reception or UL transmission involved in the RACH procedure is successfully completed.

- References

- RP-250247, “Revised WID on Low NR band carrier aggregation via switching”, RAN#107, March, 2025.

R1-2504350, “FL summary #3 of Low band carrier aggregation via switching”, RAN1#121, May 19th – 23rd, 2025.