3GPP TSG RAN WG1 Meeting #122		R1-2505316

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Maintenance on Support of RedCap and eRedCap UEs with NR NTN operating in FR1-NTN bands

Agenda Item:	8.11.2

Document for:	Discussion and Decision

- Introduction

In this contribution, we will provide a text proposal for some remaining issues of Rel-19 enhancement of RedCap and eRedCap UEs in NTN.

- Text Proposals

2.1 Text Proposal #1 for timeline condition

Reason for change: Current timeline conditions for cancellation of the UL transmission cannot be used in NTN.

Current timeline descriptions in TS 38.213 are as the following:

If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then

-	the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2  relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214].

-	the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols.

Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH.

In NTN, the gNB cannot verify whether the timeline conditions for cancellation of the UL transmission is met on the UE side because of the misalignment of TA between gNB and UE. Consequently, it must be specified that the gNB determines the relevant timeline based on the TA reported by the UE.

Summary of change: In the section of 17.2 of TS 38.213, clarify the timeline conditions for cancellation of the UL transmission is needed.

Consequences if not approved: The timeline conditions for cancellation of the UL transmission are unclear and it misleads the behaviors of UE and gNB.

Proposal 1: Adopt the following TP for TS 38.213.

TS 38.213 V19.0.0 - 17.2 Half-Duplex UE in paired spectrum *** Unchanged parts are omitted *** If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception based on reported TA where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214]. - the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception based on reported TA where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH. *** Unchanged parts are omitted ***

- Conclusion

In this contribution, we presented a text proposal for remaining issues of Rel-19 enhancement of RedCap and eRedCap UEs in NTN:

Proposal 1: Adopt the following TP for TS 38.213.

- Reason for change: Current timeline conditions for cancellation of the UL transmission cannot be used in NTN. If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214]. - the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH. In NTN, the gNB cannot verify whether the timeline conditions for cancellation of the UL transmission is met on the UE side because of the misalignment of TA between gNB and UE. Consequently, it must be specified that the gNB determines the relevant timeline based on the TA reported by the UE. - Summary of change: In the section of 17.2 of TS 38.213, clarify the timeline conditions for cancellation of the UL transmission is needed. - Consequences if not approved: The timeline conditions for cancellation of the UL transmission are unclear and it misleads the behaviors of UE and gNB.

TS 38.213 V19.0.0 - 17.2 Half-Duplex UE in paired spectrum *** Unchanged parts are omitted *** If a HD-UE is configured by higher layers to transmit SRS, or PUCCH, or PUSCH in a set of symbols and the HD-UE detects a DCI format indicating to the HD-UE to receive CSI-RS or PDSCH in a subset of symbols from the set of symbols, then - the HD-UE does not expect to cancel the transmission of the PUCCH or PUSCH in the set of symbols if the first symbol in the set occurs within Tproc,2 relative to a last symbol of a PDCCH reception based on reported TA where the HD-UE detects the DCI format; otherwise, the HD-UE cancels the PUCCH, or the PUSCH, or an actual repetition of the PUSCH [6, TS 38.214], determined from clauses 9 and 9.2.5 or clause 6.1 of [6, TS 38.214]. - the HD-UE does not expect to cancel the transmission of SRS in symbols from the subset of symbols that occur within Tproc,2 relative to a last symbol of a PDCCH reception based on reported TA where the HD-UE detects the DCI format. The HD-UE cancels the SRS transmission in remaining symbols from the subset of symbols. Tproc,2 is the PUSCH preparation time for HD-UE processing capability 1 [6, TS 38.214] assuming d2,1=1 and μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH carrying the DCI format and the SCS configuration of the SRS, PUCCH, PUSCH. *** Unchanged parts are omitted ***

- Reference

TS 38.213 V19.0.0