3GPP TSG-RAN WG1 Meeting #122	R1-2505350

Bengaluru, India, August, 25th-29th, 2025

Agenda Item:	9.12

Source:	Huawei, HiSilicon

Title:	UE features for low band CA via switching

Document for:	Discussion and Decision

# Introduction

In this contribution, our views on UE features for low band CA via switching are provided.

# Discussion

In RAN1#120bis and RAN1#121 meeting, the following agreements were achieved.

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases, - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell. - SCS 15KHz on both carriers. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2. - The pattern starts from the beginning of SFN 0 of PCell. - Restriction of maximum X switch(es) within Y slot(s). - FFS: detailed value and relation of X and Y. - FFS: other restriction. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, down-select one of the following solutions: - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier. - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier. - Alt 3: Switching gap is always assumed at the SDL carrier. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed). - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL. - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA. - FFS: whether it has specification impact. Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. - Switching gap ends at the end of the last slot on the “switch from” carrier Agreement Draft LS R1-2504868 is endorsed in principle. Agreement Final LS R1-2504869 is endorsed. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA.

In RAN4#114bis, the following agreements [2] were achieved.

RAN4#114bis Issue 1: Switching period capability - The switching period capability can be defined per band pair per band combination.

With the above agreements, only one feature group seems necessary and its essences could be the support semi-static carrier switching with a configured switching pattern for inter-band CA. The key difference from the switching period capability that has been introduced in RAN4 is the RAN1 functionality with the semi-static switching pattern. Those two capabilities may end up with single capability signaling. Therefore, a note can be added which is whether the FG can be combined with RAN4 FG x-x as single capability signal is up to RAN2 for signaling design.

Proposal 1: For low NR band carrier aggregation via switching, one FG 68-1 is introduced with the following:

- Component:

  - Support of semi-static carrier switching between two inter-band carriers with a configured switching pattern in which a UE is not expected to be operated simultaneously with both carriers for any downlink reception and uplink transmission with the required switching period and uplink TA taken into account.

  - FFS: others

- Note:

  - Note: It is up to RAN2 to determine whether the FG can be combined with RAN4 FG x-x as single capability signal.

Similar to FG 22-1 for R16 UL Tx switching, for the new FG, its prerequisite FG is FG 6-5, which is the basic DL NR-NR CA operation, and the switching period introduced by RAN4.

Proposal 2: For low NR band carrier aggregation via switching, the prerequisite of FG 68-1 is FG 6-5 and the switching period introduced in RAN4.

Because it is a CA operation, the new FG is per BC straightforwardly. As a result, it is optional capability and the columns of “Need of FDD/TDD differentiation”, “Need of FR1/FR2 differentiation” and “Capability interpretation for mixture of FDD/TDD and/or FR1/FR2” become not applicable, i.e. “N/A”

Proposal 3: For Low NR band carrier aggregation via switching, the granularity of FG 68-1 is per BC.

- Optional with capability signaling

To have a full picture of the new FG, a table in Appendix is provided.

Proposal 4: The table in the Appendix is taken as the starting point for further UE feature discussion on Rel-19 low band CA via switching.

# Conclusions

In this contribution, the following are proposed:

Proposal 1: For low NR band carrier aggregation via switching, one FG 68-1 is introduced with the following:

- Component:

  - Support of semi-static carrier switching between two inter-band carriers with a configured switching pattern in which a UE is not expected to be operated simultaneously with both carriers for any downlink reception and uplink transmission with the required switching period and uplink TA taken into account.

  - FFS: others

- Note:

  - Note: It is up to RAN2 to determine whether the FG can be combined with RAN4 FG x-x as single capability signal.

Proposal 2: For low NR band carrier aggregation via switching, the prerequisite of FG 68-1 is FG 6-5 and the switching period introduced in RAN4.

Proposal 3: For low NR band carrier aggregation via switching, the granularity of FG 68-1 is per BC.

- Optional with capability signaling

Proposal 4: The table in the Appendix is taken as the starting point for further UE feature discussion on Rel-19 low band CA via switching.

# References

- RP-250247, Revised WID on Low NR band carrier aggregation via switching

- R4-2505223, WF on UE RF requirements for LB CA via switching, RAN4#114bis

# Appendix: UE features for low band CA via switching

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 68. NR_LBCA_Sw | 68-1 | Carrier aggregation via switching | 1. Support of semi-static carrier switching between two inter-band carriers with a configured switching pattern in which a UE is not expected to be operated simultaneously with both carriers for any downlink reception and uplink transmission with the required switching period and uplink TA taken into account. 2. FFS: others | 6-5 and RAN4 FG x-x (switching period for CA via switching) | Yes | N/A | Operation of Carrier aggregation via switching is not supported | Per BC | N/A | N/A | N/A | Note: It is up to RAN2 to determine whether the FG can be combined with RAN4 FG x-x as single capability signal | Optional with capability signaling |