3GPP TSG RAN1 #122			R1-2505445

Bengaluru, India, Aug 25th – 29th, 2025

Source: 	     Xiaomi

Title:                  UE feature for SBFD

Agenda item:    9.1.3

Document for:  Decision

Introduction

In previous RAN1 meetings, UE features for SBFD were discussed. The updated RAN1 UE features list for Rel-19 NR after RAN1 #121 is endorsed in [1].

In this contribution, we provide our views on remaining issues on UE features for SBFD operation.

Discussion

SBFD TX/RX/measurement procedures

The following agreements were made related to UE features for SBFD TX/RX/measurement procedures.

Agreement For CSI processing timeline in SBFD symbols to process the CSI-RS across two DL subbands, - Option 3: It is up to UE capability to indicate one of the following: - CSI processing timeline and CPU counting are the same as legacy. - Scale the CSI processing timeline Z/Z’ by factor of 2 and keep the same CPU counting as legacy. Agreement For a PRG that overlaps the subband boundary, the part of the DL PRG inside the DL subband can be used subject to UE capability - UE capability reports the maximum number of supported partial PRGs in SBFD symbols. Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, - Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability. - SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis. - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP. - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP. - For SRS, only Configuration 1 is applicable.

The following agreements on UE features were made in RAN1#120.

Agreement: Introduce following basic FG for SBFD, and FFS on some potential components (that may be separate FGs). X-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one[/two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; [Per Band]; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling. Agreement: Add the following component in basic FG for SBFD. - “Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1)”

The following agreements on UE features were made in RAN1#120bis.

Agreement: Introduce following FG for [Transmissions and] Receptions across SBFD and non-SBFD symbols (Configuration 2) as follows, and FFS on whether transmission across SBFD and non-SBFD symbols is defined as separate FG or not - Alt.1: Configuration 2 for transmission and reception should be defined in a single FG - Alt.2: Configuration 2 for transmission and reception should be defined in separate FGs 60-2; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2); 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) [2. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2)] FFS: Other component(s) FFS: whether component 2 is defined as separate FG; YES; n/a; [Transmission and] Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Adopt “Per band” type for FG 60-1. Agreement: Adopt “Per band” type for FG 60-3. Agreement: Adopt “Per band” type for FG 60-4 Agreement: Add the following components in FG 60-1 (basic FG for SBFD operation) - Support of separate PUCCH frequency resource configuration in SBFD symbols and non-SBFD symbols - [FFS: Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols] - Support of separate PUSCH frequency hopping offset in SBFD symbols and non-SBFD symbols - [FFS: Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols] Agreement: Update FG 60-1 as follows: - Confirm “cell-common” in component 1 - Confirm “UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols” - Confirm Component 3: “3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling” - Add “Support of separate CSI reports for SBFD and non-SBFD symbols 60-1; Basic feature for SBFD-aware UE TX/RX/measurement behaviour and procedures; 1. Support of cell-common configuration of time and frequency location of SBFD subbands 2. Support of UL transmission in one UL subband and DL reception in one [and two] DL subband in SBFD symbols 3. Support of link direction determination based on configured/scheduled transmissions/receptions and collision handling 4. Support of UL transmissions or DL receptions across SBFD symbols and non-SBFD symbols in different slots, where the transmissions or receptions are restricted to SBFD symbols or non-SBFD symbols (Configuration 1) 5. Support of separate CSI reports for SBFD and non-SBFD symbols FFS: Whether to include two DL subband support in component 2 FFS: other component (which may be separate FG); YES; n/a; Basic feature for SBFD-aware UE TX/RX/ measurement behaviour and procedures is not supported; Per band; TDD only; n/a; n/a; [Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s)] [Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one.]; Optional with capability signalling.

The following agreements on UE features were made in RAN1#121.

Agreement: For FG 60-1, regarding FFS for support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols, - Confirm the FFS as it is: - “[FFS: 7. Support of separate UL power control parameters based on unified TCI framework in SBFD symbols and non-SBFD symbols]” - Add the following in Note column - “Component 7 applies only to UEs supporting FG 23-1-1h” Agreement: For FG 60-1, regarding FFS for support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols, - Confirm the FFS as it is - “[FFS: 9. Support of separate SRS resource set configuration in SBFD symbols and non-SBFD symbols]” - Add the following note in Note column - “Note: Component 9 is only applicable for SRS resource sets with usage codebook, nonCodebook and beamManagement.” Agreement: Remove the following two Notes in FG 60-1: - Note: Only UL transmissions are allowed within UL subband and only DL receptions (except for CLI measurement) are allowed within DL subband(s) - Note: The maximum number of UL subbands for SBFD operation in an SBFD symbol within a TDD carrier is one. Agreement: - Adopt “per band” for FG 60-2 Type - Add FG 60-1 as a prerequisite FG for FG 60-2

For the basic FG 60-1, the remaining issues is whether to include two DL subband support in component 2. From our perspective, we are fine to include two DL subband support for PDSCH receptions considering the limited additional complexity compared with one DL subband. However, for CSI reception across two DL subband, it is proposed to define it as a separate FG considering the additional UE complexity.

Proposal 1: For FG 60-1, include two DL subband support for PDSCH in component 2. CSI-RS reception across two DL subband is defined as a separate FG.

For FG 60-2, the main remaining issue is whether to include transmission across SBFD and non-SBFD symbols. Given that the mechanisms for DL receptions and UL transmissions are quite different for Configuration 2, it is proposed to not include transmission in FG 60-2. Instead, a separate FG is defined for transmission for Configuration 2.

Proposal 2: Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) is defined as a separate FG from FG 60-2.

SBFD random access operation

The following agreement was made in RAN1#120 for SBFD random access.

Agreement: Introduce following FGs for SBFD random access, one based on single RACH configuration (Option 1) and the other based on two RACH configurations (Option 2), and FFS on whether UE supporting SBFD random access needs to support at least Option 1 or Option 2. - Alt.1: UE supporting SBFD random access needs to support at least Option 1 - Alt.2: UE supporting SBFD random access needs to support at least Option 2 - Alt.3: UE supporting SBFD random access supports Option 1 or Option 2 (i.e., no default option) X-3; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 1) FFS: Other component(s) which may be separate FG; X-1; YES; n/a; SBFD random access operation based on one single RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. X-4; SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode; Support of SBFD random access operation based on two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration in RRC_IDLE/INACTIVE/CONNECTED mode (RACH configuration Option 2) FFS: Other component(s) which may be separate FG; [X-1, X-3]; YES; n/a; SBFD random access operation based on two separate RACH configurations in RRC_IDLE/INACTIVE/CONNECTED mode is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

The following agreements were made in RAN1#121.

Agreement: Introduce the following two FGs: 60-5; Preamble repetition within additional-Ros for RACH configuration Option 1; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs 2. For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.; 60-3, [54-1]; YES; n/a; Preamble repetition within additional-ROs for RACH configuration Option 1 is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. 60-5a; Preamble repetition within additional-Ros for RACH configuration Option 2; 1. Determination of the set of Npreamblerep valid additional-ROs and the time period of the set(s) of Npreamblerep valid additional-ROs 2. For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.; 60-4, [54-1]; YES; n/a; Preamble repetition within additional-ROs for RACH configuration Option 2 is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling.

For FG 60-4, one remaining issue is the prerequisite. According to our understanding, FG 60-4 is a more advanced UE capability compared with FG 60-3. The main difference is to support different preamble formats for SBFD and non-SBFD symbols. For UE supporting FG 60-4, UE should be able to support FG 60-3.

Proposal 3: FG 60-1 and 60-3 are the prerequisite of FG 60-4.

For FG 60-5 and 60-5a, the remaining issue is whether FG 54-1 is one of the prerequisites. FG 54-1 is basic FG of PRACH coverage enhancements introduced in Rel-18. Given that Rel-18 PRACH repetition scheme is used for PRACH repetition in additional ROs, it is reasonable to define FG 54-1 as prerequisite for FG 60-5 and 60-5a.

Proposal 4: FG 54-1 is prerequisite of FG 60-5 and 60-5a.

CLI handling

The following agreements were made in RAN1#120 related to CLI handling.

Agreement If non-transparent UL resource muting is supported for gNB-to-gNB CLI handling, Option 1 is recommended to be specified - Option 1: Comb-2 for both DFT-S-OFDM and CP-OFDM - Note: Additional details can be discussed within RAN1#117. - Note: Power boosting is assumed for REs in the symbol with UL resource muting. FFS: Details. - Above is subject to separate UE capability (including separate capabilities for DFT-S-OFDM and CP-OFDM).

The following agreements were made in RAN1#120bis related to CLI handling.

Agreement: Introduce following FG for L1 SRS-RSRP measurement/reporting. 60-9; L1 SRS-RSRP measurement and aperiodic reporting; 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and [periodic and] aperiodic SRS-RSRP measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 SRS-RSRP measurement and reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce a new FG for FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource as follows 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce following FG for L1 CLI-RSSI measurement/reporting. 60-8; L1 CLI-RSSI measurement and [aperiodic] reporting; 1. Aperiodic L1 CLI-RSSI reporting on PUSCH [and periodic L1 CLI-RSSI reporting on PUCCH] 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. [Periodic and] aperiodic CLI-RSSI measurement resource FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG; FFS; YES; n/a; [Aperiodic] L1 CLI-RSSI measurement and reporting is not supported; [Per Band]; TDD only; [n/a]; [n/a]; Optional with capability signalling. Agreement: Define the following FG 60-9b; Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 1. Support of semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting; 60-9; YES; n/a; Semi-persistent L1 SRS-RSRP measurement resource and aperiodic reporting is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. Agreement: Introduce the following FGs for UL resource muting 60-7; UL resource muting for CP-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for CP-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for CP-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling. 60-7a; UL resource muting for DFTS-OFDM waveform; 1. Support semi-static configuration of time location and frequency location of UL resource muting for DFTS-OFDM waveform - Up to two UL muting symbols within a slot for PUSCH - A configurable comb offset {0, 1} for the UL muting symbol [- FFS: Whether this FG only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH] 2. Support dynamic on/off indication of the configured UL muting symbols by TDRA field in DCI for DG-PUSCH and Type-2 CG PUSCH 3. Support semi-static determination of UL muting symbols for Type-1 CG PUSCH FFS: other potential component/FG/additional restriction; [FFS]; YES; n/a; UL resource muting for DFTS-OFDM waveform is not supported; [Per Band]; TDD only; n/a; n/a; Optional with capability signalling.

The following agreements were made in RAN1#121.

Agreement: Adopt “Per band” for FG 60-7 and 60-7a Agreement: Adopt “Per band” for FG 60-8 Agreement: Confirm “Periodic and” in Component 3 of FG 60-8 Agreement: On whether “[and periodic L1 CLI-RSSI reporting on PUCCH]” in Component 1 of FG 60-8 should be confirmed or defined as a separate FG from FG 60-8, support Alt-2: - Alt-1: “[and periodic L1 CLI-RSSI reporting on PUCCH]” in Component 1 of FG 60-8 should be confirmed - Alt-2: “[and periodic L1 CLI-RSSI reporting on PUCCH]” in Component 1 of FG 60-8 should be removed, and define a separate FG for “support of periodic L1 CLI-RSSI reporting on PUCCH” Agreement: Add a component for “Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs” in FG 60-8 - FFS: Whether/how to define maximum number of configured L3 and L1 CLI-RSSI measurement resources if simultaneous configuration of L3 and L1 CLI-RSSI measurements is supported Agreement: Define new FG for support of semi-persistent L1 CLI-RSSI measurement resource - Component: Support of semi-persistent CLI-RSSI measurement resource - Prerequisite FG: FG 60-8 - Type: Per band Support of semi-persistent L1 CLI-RSSI measurement resource; 1. Support of semi-persistent L1 CLI-RSSI measurement resource; 60-8; YES; n/a; Semi-persistent L1 CLI-RSSI measurement resource is not supported; Per Band; TDD only; n/a; n/a; Optional with capability signalling. Agreement: - Confirm [Periodic and] in Component 1 of FG 60-9. - Confirm “60-9” as a prerequisite FG for FG 60-9b Agreement: Add a component for “Maximum number of L1 SRS-RSRP measurement resources across all CCs” in FG 60-9 - FFS: Whether/how to define maximum number of configured L3 and L1 SRS-RSRP measurement resources if simultaneous configuration of L3 and L1 SRS-RSRP measurements is supported Agreement: Adopt “Per band” for FG 60-9/9a/9b

For FG 60-7 and 60-7a, one of the remaining issues is whether the FGs only support one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH. On one hand, it is agreed that separate UL muting patterns can be configured for DG/Type 2 CG PUSCH and Type 1 CG PUSCH, as below.

Agreement For the semi-statically configured time location and frequency location of UL muting symbol(s) for PUSCH, - Separate configurations of up to two UL muting symbols can be provided per UE for DG PUSCH/Type 2 CG PUSCH and Type 1 CG PUSCH - Separate configuration for each of multiple Type 1 CG PUSCH configurations is supported

One the other hand, when we discussed the options to determine the time location of UL muting symbols for DG/Type 2 CG PUSCH, Option 2 was down-selected from the three options. There was a lot of debate among the options and there was strong concern from UE vendors on Option 3 where the time location of UL muting symbols can be dynamically changed based on DCI indication. Following the concern, it is better to assume one common pattern of UL muting for the basic FG.

Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, one of the following options is selected for DG PUSCH and Type 2 CG PUSCH - Option 1: The time location of each of one or two UL muting symbols is semi-statically configured and cannot be dynamically indicated - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI - Option 3: The time location of each of one or more UL muting symbols is semi-statically configured, and none, one or two the time location of UL muting symbol(s) is dynamically indicated by TDRA field in DCI To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the time location of each of one or two UL muting symbols is semi-statically configured for Type 1 CG PUSCH. FFS: details of semi-static configuration and dynamic indication (if supported) FFS: how to handle UL resource muting for PUSCH repetition and for multi-PUSCH scheduled by a single DCI Agreement To determine the time location of UL muting symbol(s) in a slot for a PUSCH, the following option is selected for DG PUSCH and Type 2 CG PUSCH - Option 2: The time location of each of one or two UL muting symbols is semi-statically configured, and muting all of the semi-statically configured time location of UL muting symbol(s) can be dynamically turned ON/OFF by TDRA field in DCI

Proposal 5: For FG 60-7 and FG 60-7a, only one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH is supported.

Conclusion

In this contribution, we discussed remaining issues on UE features for SBFD operation, with the following proposals.

Proposal 1: For FG 60-1, include two DL subband support for PDSCH in component 2. CSI-RS reception across two DL subband is defined as a separate FG.

Proposal 2: Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) is defined as a separate FG from FG 60-2.

Proposal 3: FG 60-1 and 60-3 are the prerequisite of FG 60-4.

Proposal 4: FG 54-1 is prerequisite of FG 60-5 and 60-5a.

Proposal 5: For FG 60-7 and FG 60-7a, only one common pattern of UL muting for DG/Type 2 CG PUSCH and Type 1 CG PUSCH is supported.

- Reference

- R1-2504673, Updated RAN1 UE features list for Rel-19 NR after RAN1 #121, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#121