3GPP TSG-RAN WG1 Meeting #122	R1- 2505478

Bengaluru, India, Aug 25th – 29th, 2025

Source:	TCL

Title:	Discussion on other aspects for CSI compression

Agenda item:	10.1.1.2

Document for:	Discussion and Decision

Introduction

The study items on Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface[1][2] explored the benefits of augmenting the air-interface with features enabling improved support of AI/ML. The project studied the 3GPP framework for AI/ML for air-interface corresponding to spatial/temporal/frequency CSI compression (hereinafter referred to as AI/ML-based CSI compression) regarding aspects such as performance, complexity, and potential specification impact.

As for the use case of AI/ML-based CSI compression, the CSI spatial/frequency compression without temporal aspects (Case 0) is recently approved as one objective in the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements in RAN#108 meeting[3]. Furthermore, the WID identifies the following specification support aspects related to AI/ML-based CSI compression:

CSI feedback enhancement, encompassing (two-sided model) [RAN1, RAN2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air, including: - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In this contribution, we discuss the other aspects related to AI/ML-based CSI compression, focusing on data collection for training, performance monitoring, and model paring.

Discussion

  - Data collection for training

Data collection for training can be performed at both NW side and UE side. For the data collection at NW side, the gNB can configure the UE to report the training data, i.e., the target CSI. The training data can be collected in both an opportunistic and proactive manner. The opportunistic manner means the gNB collects available data from ongoing CSI report sessions and the proactive manner means the CSI report is triggered only for data collection. The gNB can then pre-train the two-side model jointly.

For the data collection at UE side, each UE can locally collect training data (i.e., the target CSI) over time. It can also collect data in both an opportunistic and proactive manner. The opportunistic manner means the CSI report is configured by gNB and the proactive manner means the UE requests channel measurements from gNB when it needs data collection. In practice, the UE can log training data periodically or during specific channel conditions to capture a wide range of scenarios. This on-device data collection leverages the UE’s direct perception of the channel, ensuring the generalization performance of the model. The training data can be also provided by gNB for the initial training of the encoder. In one mechanism, the gNB sends a dataset to the UE containing the target CSI and corresponding labels, i.e., expected compressed representations produced by encoder. By receiving such data, the UE can train or fine-tune its encoder model to align with the gNB’s decoder expectations. This approach is flexible. The provided training data could originate from the NW’s own measurements, simulations, or even other UEs (e.g., crowd-sourced data), allowing cross-user or network-wide learning without the UE needing to independently experience every channel condition.

To achieve effective CSI compression, the encoder/decoder models must be trained on a suitable dataset of channel state information. Hence, the quality indicator can be provided along with training data to denote its reliability. Then the NW side or UE side can choose data for model training according to the quality indicator.

Observation 1: The training data, i.e., the target CSI and expected compressed representations produced by encoder, can be provided by gNB for the initial training of the encoder.

Proposal 1: NW side and UE side data collection for training can be performed in both an opportunistic and proactive manner.

Proposal 2: The quality indicator can be provided along with training data to denote its reliability.

  - Performance monitoring

Once the AI/ML-based CSI compression is operational, it is critical to monitor its performance and ensure the model continues to meet accuracy requirements. The performance monitoring for CSI compression includes end-to-end monitoring and root-cause analysis, enabling timely detection of performance degradation and triggering the model update.

End-to-end monitoring

The end-to-end performance monitoring was fully discussed in the SI stage and the conclusions are captured as follows[4][7].

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. Conclusion UE-side monitoring is feasible. Observation Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

The performance monitoring can be performed at NW side or UE side. For the NW side performance monitoring, the gNB evaluates the quality of the reconstructed CSI (output of the decoder) against the label to gauge end-to-end performance. For CSI spatial/frequency compression without temporal aspects, this label could be the actual measured CSI. The label acquisition can be based on the existing CSI report framework. For example, both codebook-based PMI and compressed CSI are configured as the report quantities and the periodical report type is configured so the gNB can compare the reconstructed CSI with the actual measured CSI periodically reported by the UE to assess overall accuracy.

For the UE side performance monitoring, multiple options were considered. For example, the UE side monitoring can be performed based on the precoded reference signals transmitted from NW based on the reconstructed CSI. If the UE fails to decode data based on the associated reference signal, it can be interpreted as performance degradation. The UE side monitoring can be even performed based on the output of the CSI reconstruction model at the UE or via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI, which can reduce the signaling overhead. Overall, the UE side performance monitoring can be performed by UE implementation without specification impact.

For the sake of root-cause analysis and model control, the decision-making can be performed at NW side. In this way, the UE needs to report the monitoring-related information to the gNB for final decision-making and two kinds of report quantities can be considered. One option is that the UE reports monitoring metric values and the gNB judges the model performance. It is necessary to specify the monitoring metrics which can be configured by gNB. The other option is that the UE judges the model performance by itself based on the threshold configured by gNB. The UE only needs to report the monitoring outcome, e.g., good performance or bad performance, to the gNB. Using threshold-based triggering ensures that the UE reports are event-driven, i.e., only when performance falls outside desired bounds, thereby reducing unnecessary signaling. The UE’s report could be a binary flag or a small codepoint representing the performance category, further minimizing overhead.

Observation 2: The UE side end-to-end performance monitoring can be performed by UE implementation.

Proposal 3: For the NW side end-to-end performance monitoring, the label acquisition could be based on the existing CSI report framework.

Proposal 4: The UE needs to report the monitoring metric values or monitoring outcome if the final decision-making is performed at NW side.

Root cause analysis

The root cause analysis was discussed in the SI stage and the conclusion is captured as follows[6].

Conclusion For NW-first training, in inter-vendor training collaboration Direction A and C, identification of root cause for performance issues can be achieved at least by the following - Data distribution mismatch that is identified by either NW side or UE side - Via target CSI and CSI feedback pair report, e.g., with the understanding that NW side runs inference using the NW side reference CSI generation part and compare with the inference using UE side CSI generation part. - Note that this conclusion does not preclude other methods. - Note that triggering of root cause detection may be after detecting performance degradation

If the end-to-end performance degrades, it may infer the issue lies with distribution shift in the input data, i.e., data drift. Hence, the root cause analysis can be triggered to further identify whether the data drift affects the autoencoder or the quantization. In such a case, the gNB can request additional UE-side monitoring data. For instance, the UE might be requested to send samples of target CSI, encoder output and quantizer output for analysis. The gNB can firstly compare the quantizer output with the output from the NW side reference CSI generation part to identify if the decoder generalization has degraded, or rather, is out-of-date. Then the gNB can compare the target CSI with the decoder output which is derived from encoder output directly to identify if the autoencoder generalization has degraded. Similarly, the gNB can continue to compare the encoder output with the de-quantizer output which is derived from quantizer output to identify if the quantization generalization has degraded. The acquisition of samples of target CSI, encoder output and quantizer output can be all configured based on the existing CSI report framework. Overall, the system can identify whether a performance drop is due to the AI model effects or quantization effects, ensuring that appropriate actions can be taken with confidence.

Proposal 5: The request of target CSI, encoder output and quantizer output could be based on the existing CSI report framework for the root cause analysis.

  - Model paring

To successfully deploy AI/ML-based CSI compression, the encoder and the decoder must correspond to each other. The model paring was discussed in the SI stage and the achieved agreements are captured as follows[5][7].

Agreement For study of MI-Option2 (i.e., model identification with dataset transfer) for the two-sided model, - ID-X can be used for pairing the UE-part and the NW-part of a two-sided model - FFS: other information needed for pairing Agreement In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned - Note: whether the purpose for pair will be specified will be discussed separately. Agreement In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned or how the ID is specified - Note: whether the purpose for pair will be specified will be discussed separately.

It is concluded that the paring ID should be introduced for pairing the UE-part and the NW-part of a two-sided model. In Options 3a-1 and 4-1, the paring ID can be exchanged along with the model parameters or dataset for model identification. Using the paring ID, the gNB and UE follow a clear procedure to ensure they are using a paired model. Whenever the gNB decides to use a new encoder due to an update or adaptively switching for changing conditions, it will signal the paring ID of the corresponding encoder to the UE. For example, if the network later provides incremental training data, it can tag it with the same paring ID to indicate that those samples are to be used for the encoder identified by that ID. Additionally, if multiple encoders are applicable, e.g., one for low mobility and another for high mobility scenarios, the paring ID allows the UE to distinguish them and apply the correct one as instructed by the gNB with model activation/deactivation commands including the paring ID of the encoder to activate or switch to. The encoder at UE and the decoder gNB thus maintain alignment.

Considering there exists many identifiers in the previous discussions about AI/ML for NR air interface, such as model ID, functionality ID, ID-X, paring ID, associated ID. We propose to standardize a unified ID, termed here as a quasi-common model identifier (QCM ID), in order to make the specification concise. The QCM ID is defined as a deterministic mapping from multiple identifiers mentioned above. The UE should support recovering the identifiers from the QCM ID by de-mapping. For instance, the UE can recover the paring ID from QCM ID so that the encoder and decoder remain paired. In this way, the UEs with the same QCM ID can be deemed to be configured with the same identifier set or have the similar model conditions, which unifies the ID-related configuration.

Observation 3: The paring ID can be exchanged along with the model parameters or dataset for model identification.

Proposal 6: It is recommended to introduce a unified ID to streamline the ID-related configuration.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1: The training data, i.e., the target CSI and expected compressed representations produced by encoder, can be provided by gNB for the initial training of the encoder.

Observation 2: The UE side end-to-end performance monitoring can be performed by UE implementation.

Observation 3: The paring ID can be exchanged along with the model parameters or dataset for model identification.

Proposal 1: NW side and UE side data collection for training can be performed in both an opportunistic and proactive manner.

Proposal 2: The quality indicator can be provided along with training data to denote its reliability.

Proposal 3: For the NW side end-to-end performance monitoring, the label acquisition could be based on the existing CSI report framework.

Proposal 4: The UE needs to report the monitoring metric values or monitoring outcome if the final decision-making is performed at NW side.

Proposal 5: The request of target CSI, encoder output and quantizer output could be based on the existing CSI report framework for the root cause analysis.

Proposal 6: It is recommended to introduce a unified ID to streamline the ID-related configuration.

References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 6 – 9, 2022.

- RP-250308, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2, RAN #107, March 12 – 14, 2025.

- RP-251870, New WI: Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface enhancements, RAN #108, June 9 – 13, 2025.

- RAN1 Chair’s Notes, RAN1 #117, May 20th – 24th, 2024.

- RAN1 Chair’s Notes, RAN1 #119, Nov. 18th – 22nd, 2024.

- RAN1 Chair’s Notes, RAN1 #120, Feb. 17th – 21st, 2025.

- RAN1 Chair’s Notes, RAN1 #120-bis, April 7th – 11th, 2025.