3GPP TSG RAN WG1 #122		R1-2505547

Bengaluru, India, 25th – 29th Aug, 2025

Agenda item:	8.6.3

Source:	Samsung

Title:	Remaining issues on LP-WUS operation in CONNECTED modes

Document for:	Discussion and Decision

# Introduction

This contribution discusses the remaining issues on supporting LP-WUS operation in RRC_CONNECTED mode.

# Configuration for LP-WUS Monitoring Occasion(s)

For Option 1-2, as shown in Figure 1, one or multiple MOs can be supported per periodicity, and there is a FFS on how to determine the time domain occasions when multiple MOs are supported. Similar to MO configuration in Option 1-1 and our proposal for MO configuration in RRC_IDLE/INACTIVE mode, supporting a configurable window would improve the flexibility on the time domain locations of the MOs, and may also save the UE power by properly configuring the relationship between the periodicity and window duration for monitoring MOs. In this sense, we prefer to support a configurable time domain interval between neighboring MOs in the multiple MOs, wherein the time domain interval is provided by an additional RRC parameter.

Figure 1 Illustration of configurations for LP-WUS MOs in Option 1-2.

Proposal 1: For MO configuration in Option 1-2, support a configurable time interval between two consecutive MOs by RRC, when multiple MOs are configured per periodicity.

- Adopt TP1 for TS 38.213.

=================================== TP #1 for TS 38.213 ===================================

Reason for change: The UE procedure is not specified for determining the remaining MOs when multiple MOs are configured in a period.

Summary of changes: Add the UE procedure for determining the remaining MOs when multiple MOs are configured in a period.

Consequence if not approved: The UE procedure is unclear for determining the remaining MOs when multiple MOs are configured in a period.

=================================== Start of TP #1 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

A UE can be provided by WUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity, provided by numMO-perPeriodicity-Option 1-2. A subsequent WUS monitoring occasion, if any, in the remaining WUS monitoring occasions within the same period is intervalMO-CONNECTED-Option1-2 from a previous WUS monitoring occasion. The UE reports a number of slots [18, TS 38.306] and expects that a time gap, from a last WUS monitoring occasion from the number of WUS monitoring occasions per periodicity to the slot where the wus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts wus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS monitoring occasion from the number of WUS monitoring occasions per periodicity.

=============================== Unchanged Text Omitted ===================================

==================================== End of TP #1 =======================================

# LP-WUS Monitoring

For Option 1-2, the UE behavior on LP-WUS monitoring after the reception of the wake-up indication needs to be further clarified. For example, as shown in Figure 2, after a UE receives a wake-up indication (e.g., in the blue MOs in the figure), the UE may start the new timer after the time timeOffsetCONNECTEDOption1-2, wherein within the timeOffsetCONNECTEDOption1-2, there can be additional MOs (e.g., the red MOs in the figure) associated with other new timer. If the UE continues to monitor the MOs within the timeOffsetCONNECTEDOption1-2, the UE behaviour on the time duration for PDCCH monitoring after wake-up would be confusing. Hence, we propose to stop LP-WUS monitoring in the MOs within the time timeOffsetCONNECTEDOption1-2.

Figure 2 Illustration of LP-WUS monitoring in Option 1-2.

Proposal 2: For Option 1-2, after the reception of wake-up indication, a UE does not need to monitor LP-WUS in the remaining MOs within the time timeOffsetCONNECTEDOption1-2.

- Adopt TP2 for TS 38.213.

=================================== TP #2 for TS 38.213 ===================================

Reason for change: The UE behavior for WUS monitoring after waking-up is ambiguous.

Summary of changes: Add the UE behavior for WUS monitoring after waking-up.

Consequence if not approved: The UE behavior for WUS monitoring after waking-up is not specified.

=================================== Start of TP #2 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. If the UE is provided WUS-MOCONNECTED-Option1-2, the UE does not monitor WUS in the remaining WUS monitoring occasions within the time provided by timeOffsetCONNECTEDOption1-2.

==================================== End of TP #2 =======================================

# QCL source for LP-WUS

There is one remaining issue in TS 38.213 on which RRC parameter to use for distinguishing whether Rel-17 unified TCI framework is capable or configured, and the legacy RRC parameter dl-OrJointTCI-StateList shall be used to be aligned with RAN1 agreement.

Proposal 3: Adopt TP3 for TS 38.213.

=================================== TP #3 for TS 38.213 ===================================

Reason for change: The RRC parameter for QCL assumption of LP-WUS is not defined.

Summary of changes: Add the RRC parameter for QCL assumption of LP-WUS.

Consequence if not approved: The RRC parameter for QCL assumption of LP-WUS is not specified.

=================================== Start of TP #3 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

If a UE is provided dl-OrJointTCI-StateList, the UE receives WUS based on the quasi co-location information of the TCI states indicated by a most recent DCI format or MAC CE, after a respective application time; otherwise, the UE receives WUS based on the quasi co-location information of the TCI states for a CORESET with controlResourceSetId value that is same as the one indicated by WUS_TCI_states_CONNECTED.

=============================== Unchanged Text Omitted ===================================

==================================== End of TP #3 =======================================

# Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: For MO configuration in Option 1-2, support a configurable time interval between two consecutive MOs by RRC, when multiple MOs are configured per periodicity.

- Adopt TP1 for TS 38.213.

=================================== TP #1 for TS 38.213 ===================================

Reason for change: The UE procedure is not specified for determining the remaining MOs when multiple MOs are configured in a period.

Summary of changes: Add the UE procedure for determining the remaining MOs when multiple MOs are configured in a period.

Consequence if not approved: The UE procedure is unclear for determining the remaining MOs when multiple MOs are configured in a period.

=================================== Start of TP #1 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

A UE can be provided by WUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity, provided by numMO-perPeriodicity-Option 1-2. A subsequent WUS monitoring occasion, if any, in the remaining WUS monitoring occasions within the same period is intervalMO-CONNECTED-Option1-2 from a previous WUS monitoring occasion. The UE reports a number of slots [18, TS 38.306] and expects that a time gap, from a last WUS monitoring occasion from the number of WUS monitoring occasions per periodicity to the slot where the wus-PDCCHMonitoringTimer would start, is no smaller than the reported number of slots. If the UE determines to monitor PDCCH based on a detected WUS, the UE starts wus-PDCCHMonitoringTimer [11, TS 38.321] after a time, provided by timeOffsetCONNECTEDOption1-2, with respect to the start of the first WUS monitoring occasion from the number of WUS monitoring occasions per periodicity.

=============================== Unchanged Text Omitted ===================================

==================================== End of TP #1 =======================================

Proposal 2: For Option 1-2, after the reception of wake-up indication, a UE does not need to monitor LP-WUS in the remaining MOs within the time timeOffsetCONNECTEDOption1-2.

- Adopt TP2 for TS 38.213.

=================================== TP #2 for TS 38.213 ===================================

Reason for change: The UE behavior for WUS monitoring after waking-up is ambiguous.

Summary of changes: Add the UE behavior for WUS monitoring after waking-up.

Consequence if not approved: The UE behavior for WUS monitoring after waking-up is not specified.

=================================== Start of TP #2 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group. If the UE is provided WUS-MOCONNECTED-Option1-2, the UE does not monitor WUS in the remaining WUS monitoring occasions within the time provided by timeOffsetCONNECTEDOption1-2.

==================================== End of TP #2 =======================================

Proposal 3: Adopt TP3 for TS 38.213.

=================================== TP #3 for TS 38.213 ===================================

Reason for change: The RRC parameter for QCL assumption of LP-WUS is not defined.

Summary of changes: Add the RRC parameter for QCL assumption of LP-WUS.

Consequence if not approved: The RRC parameter for QCL assumption of LP-WUS is not specified.

=================================== Start of TP #3 =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

=============================== Unchanged Text Omitted ===================================

If a UE is provided dl-OrJointTCI-StateList, the UE receives WUS based on the quasi co-location information of the TCI states indicated by a most recent DCI format or MAC CE, after a respective application time; otherwise, the UE receives WUS based on the quasi co-location information of the TCI states for a CORESET with controlResourceSetId value that is same as the one indicated by WUS_TCI_states_CONNECTED.

=============================== Unchanged Text Omitted ===================================

==================================== End of TP #3 =======================================