3GPP TSG RAN WG1 #122	        		             R1-2505550

Bangalore, India, August 25th – 29th, 2025

Agenda item:	8.9.1

Source: 	Samsung

Title: 	Remaining issues on Rel-19 LTM enhancements

Document for:	Discussion and Decision

# Introduction

In RAN#121 [1], this work item is complete. There are, however, still a few remaining issues on various design aspects related to Rel-19 LTM enhancements, which will be discussed and addressed in this contribution.

# Discussions on remaining issues related to Rel-19 LTM enhancements

In RAN1 #121 [1], the following agreement was made:

Agreement When SpCellInclusion is configured for gNB configured reporting for L1-RSRP based on CSI-RS, the following rule is used to determine the CSI-RS for current SpCell: - NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

In our view, in addition to or rather than exploiting the QCL relation (highlighted in the above agreement), the frequency information of a candidate cell configured with NZP CSI-RS(s) for LTM L1-RSRP reporting can actually be determined as ssb-Frequency for the SSBs associated with the (same) candidate cell – note that Rel-18 LTM supports explicit associations between SSBs and candidate cells, which to our view should be the intention. And, the QCL relation highlighted in the agreement (also captured in the TS 38.214) cannot cover this possibility. Hence, we propose to adopt the following TP #1 in the TS 38.214.

TP #1:

- 38.214-j00

- 5.2.1.4.2	Report quantity configurations

<Unchanged parts are omitted>

- 	if the UE is configured with spCellInclusion with reportQuantity set to 'cri-RSRP’, the UE shall report in a single reporting instance nrOfReportedRS-PerCell different CRI for the current SpCell and each of the nrOfReportedCells -1 candidate cells. Otherwise, the UE shall report in a single reporting instance nrOfReportedRS-PerCell different CRI for each of the nrOfReportedCells candidate cells,

-	where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated [ltm-CSI-NZP-CSI-RS-ResourceList] in the corresponding [ltm-CSI-NZP-CSI-RS-ResourceSet],

-	if spCellInclusion is configured, NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs or associated with the candidate cell) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList)) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

<Unchanged parts are omitted>

Proposal 1: Adopt TP #1 in clause 5.2.1.4.2 of the TS 38.214.

During CSI measurement before CSC, a UE does not know whether a candidate cell will become the actual target cell. Without limitation, the UE may need to store all the acquired CSI for multiple candidate cells before cell switch command, which would result in additional burden to the UE. The UE only needs to store CSI with up to a number of candidate cell(s) indicated by a corresponding UE’s capability.

Proposal 2: Regarding CSI measurement(s) for candidate cell(s) before receiving LTM CSC, support the UE to only keep/store CSI for a limited number of candidate cell(s) before cell switching command, subject to corresponding UE’s capabilities.

Regarding CSI reporting, the UE should send the CSI report at least a time gap after reception of the LTM CSC MAC CE, i.e., the trigger for early CSI acquisition in LTM. The time gap can be provided by the network subject to a corresponding UE’s capability to ensure that the reported CSI would not become obsolete at the time of cell switching hence can be used by the target cell for proper DL/UL scheduling (i.e., the CSI should not be reported too early) and enough processing time at the UE and

Proposal 3: Regarding early CSI acquisition for LTM, support the UE to send the CSI report at least a time gap after reception of the LTM CSC, where the time gap is provided by the network subject to a corresponding UE’s capability.

Regarding conditional LTM (C-LTM), RAN2 has made the following agreement in RAN2 #129:

RAN2#129 meeting Agreements on C-LTM: 4. When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM. 5. For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition. 6. For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

RAN2 design focuses on MAC entity that handles whether/when C-LTM executive condition is satisfied with at least one candidate beam. From RAN1’s perspective, which TCI state the UE needs to apply for DL/UL channel(s)/signal(s) in a candidate cell after C-LTM triggers a LTM cell switch but before a new TCI state is indicated needs to be clarified. In particular,

- For RACH-less LTM triggered in C-LTM, as in the RAN2’s agreement, the MAC entity can determine a RS that meets C-LTM execution condition. This RS can be signaled from higher layer to lower layer, and the UE can determine the TCI state in CandidateTCI-State or CandidateTCI-UL-State whose QCL RS has the same value as the aforementioned RS

- For RACH-based LTM triggered in C-LTM, as there is no RS signaled from higher layer to lower layer, the UE can determine the TCI state as the one associated with the SSB determined during RACH procedure.

Hence, we propose the following for TCI state(s) determination for C-LTM.

Proposal 4: For C-LTM, after the LTM cell switch is triggered,

- For RACH-less LTM, the UE determines the TCI state in CandidateTCI-State or CandidateTCI-UL-State whose QCL RS has the same value as the RS signaled from higher layer to lower layer that meets the C-LTM execution condition.

- For RACH-based LTM triggered in C-LTM, the UE determines the TCI state as the one associated with the SSB determined during RACH procedure.

# Conclusions

In this contribution, we provide our views on some remaining maintenance issues related to Rel-19 LTM operations, focusing on necessary components to support CSI-RS for LTM measurement/reporting, event triggered L1 measurement reporting and CSI acquisition on candidate cell(s). In particular, we present the following proposals focusing on various design aspects related to the Rel-19 LTM enhancements:

Proposal 1: Adopt TP #1 in clause 5.2.1.4.2 of the TS 38.214.

TP #1:

- 38.214-j00

- 5.2.1.4.2	Report quantity configurations

<Unchanged parts are omitted>

- 	if the UE is configured with spCellInclusion with reportQuantity set to 'cri-RSRP’, the UE shall report in a single reporting instance nrOfReportedRS-PerCell different CRI for the current SpCell and each of the nrOfReportedCells -1 candidate cells. Otherwise, the UE shall report in a single reporting instance nrOfReportedRS-PerCell different CRI for each of the nrOfReportedCells candidate cells,

-	where CRI k (k ≥ 0) corresponds to the configured (k+1)-th entry of the associated [ltm-CSI-NZP-CSI-RS-ResourceList] in the corresponding [ltm-CSI-NZP-CSI-RS-ResourceSet],

-	if spCellInclusion is configured, NZP-CSI-RS resources in [ltm-CSI-NZP-CSI-RS-ResourceList] associated with the current SpCell are the entries where PCI (given by ltm-CandidatePCI) and frequency information (given by ssb-Frequency for the SSBs QCLed with NZP-CSI-RSs or associated with the candidate cell) of the candidate cell associated with the LTM-CandidateId (given by the corresponding entry in ltm-CandidateIdList)) is equal to the PCI and center frequency of cell-defining SSB of the current SpCell.

<Unchanged parts are omitted>

Proposal 2: Regarding CSI measurement(s) for candidate cell(s) before receiving LTM CSC, support the UE to only keep/store CSI for a limited number of candidate cell(s) before cell switching command, subject to corresponding UE’s capabilities.

Proposal 3: Regarding early CSI acquisition for LTM, support the UE to send the CSI report at least a time gap after reception of the LTM CSC, where the time gap is provided by the network subject to a corresponding UE’s capability.

Proposal 4: For C-LTM, after the LTM cell switch is triggered,

- For RACH-less LTM, the UE determines the TCI state in CandidateTCI-State or CandidateTCI-UL-State whose QCL RS has the same value as the RS signaled from higher layer to lower layer that meets the C-LTM execution condition.

- For RACH-based LTM triggered in C-LTM, the UE determines the TCI state as the one associated with the SSB determined during RACH procedure.

# References

[1] Chairman Notes, 3GPP RAN1#121.