3GPP TSG RAN WG1 #122	 																		R1-2505667

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	9.1

Source: 	Ofinno

Title: 	Views on Rel-19 UE features for AI/ML for NR Air Interface

Document for:	Discussion/Decision

- Views on UE features for AI/ML beam management for NR Air Interface

In this contribution, we present our views on UE features for AI/ML for NR air interface. We propose:

[AI/ML Beam management]

- Proposal:

  - Add the following component in 58-1-4 as:

    - 14: Supported combinations of value(s) of valid time duration for each predicted time instance and the number of future time instance(s)

  - Related agreements:

Agreement @120

For inference, for BM-Case 2 of UE-side model,

- The time gap between two consecutive future time instances is configured by RRC, and the number of future time instance(s) N is configured by RRC.

  - time gap is [10ms, 20ms, 40ms, 80ms, 160ms]

  - N = [1, 2, 4, 8]

- Reference time of the earliest time instance for the predicted results is based on the most recent occasion of the CSI-RS/SSB resource in Set B for measurement

  - Where the most recent occasion of the CSI-RS/SSB resource of set B is the latest CSI-RS/SSB occasion no later than the corresponding CSI reference resource of the corresponding inference report.

Agreement @120bis

For BM-Case 2 of UE-side model, one RRC parameter represents the time gap configured for between two consecutive future time instances and also represents the time gap between the reference time and the first future time instance for prediction.

  - Reason

    - Regarding inference, a UE may use a different model for inferencing based on different time gap and/or different number of future time instance(s). Based on the related agreements as shown in the above, the configured value of the time gap and the number of future time instances can be commonly used for all the inference models. Then, such UE may run the same number of inferences for the same number of future time instances regardless of the property of each inference model. To handle this issue, combinations between the time gap and the number of future time instance(s) can be considered. For example, the UE can support the following combinations where whole inference time (i.e. time gap value × N) covers 80ms, i.e. {(10ms, 8), (20ms, 4), (40ms, 2), (80ms, 1)}

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 4. Maximum number of inference report(s) activated for BM-Case2 per BWP [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] 5. Maximum number of inference report(s) triggered for BM-Case2 per BWP [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported CSI-RS resource types for Set [A/B]: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration [20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s)] | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: Further partitioning of this FG based on existing and future agreements FFS: CPU/AIMLPU related information FFS: candidate values for components | Optional with capability signalling |