3GPP TSG RAN WG1 #122	R1-2505676

Bengaluru, India, August 25th - 29th, 2025

Source:	Ofinno

Title:	Initial views on 6G frame structure

Agenda item:	11.3.2

Document for:	Discussion and Decision

Introduction

During RAN#108 a new study item on 6G radio (6GR) was approved [1]. This contribution discussed our initial views on the 6G frame structure as related to the relevant SID objective:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4]

  - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

  - Energy efficiency and energy saving: both for network and device.

  - Enhanced spectral efficiency.

  - Enhanced overall coverage, focus on cell-edge performance and UL coverage.

  - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz.

  - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band.

  - Target scalable and forward compatible design for diverse device types.

  - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

  - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs.

  - Aim at a harmonized 6G Radio design for TN and NTN, including their integration.

  - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc.

Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

- Physical Layer structure for 6GR,

  - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4]

  - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1]

  - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1]

  - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4]

  - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2]

  - MIMO operation [RAN1, RAN4]

  - Duplexing [RAN1, RAN4]

  - Initial access [RAN1, RAN2, RAN4]

    - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4]

    - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4]

  - 6GR spectrum utilization and aggregation.  [RAN1, RAN2, RAN4]

  - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4]

  - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1].

    - RAN4 can be involved, if necessary, based on the LS from RAN1

Discussion

We think an objective of SID “Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting” should be considered for 6G frame structure discussion. Based on the objective “Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range)” and the principles, we propose:

Proposal 1. RAN1 to further study a feasibility and drawback to support “At most one subcarrier spacing for a frequency band”.

Frame structure and Multi-RAT Spectrum Sharing

This section clarifies the structural requirements for MRSS (Multi-RAT Spectrum Sharing), which differs fundamentally from DSS (Dynamic Spectrum Sharing). While DSS is to allow opportunistic resource utilization by NR on existing LTE spectrum as a migration path, design goal of MRSS should consider fair coexistence between 5G and 6G while maximizing the efficiency. MRSS should consider PRB level of resource partitioning as well as cross-interference mitigation techniques between 5G and 6G.

Some mechanisms of DSS can be considered for MRSS (e.g., rate match of 6G resource around always-on 5G signaling) with further optimization. For example, in DSS, 4G/5G resource sharing are rather limited due to 4G always-on signals such as CRS/PDCCH. In contrast, flexible and lean always-on signal design of 5G such as the absence of CRS and on-demand SSBs, provides much more tighter foundation for coordination/coexistence with 6G systems. This allows for a more efficient and integrated approach to MRSS than what was possible with DSS.

As mentioned by multiple companies, we consider aligning frame structure between 5G and 6G are beneficial. Aligning timing and/or frequency resource grid between 5G and 6G will allow more tight cross-link interference handling and dynamic resource sharing. In this sense, we propose:

Proposal 2. 6G frame structure is designed based on 5G frame structure.

- Keep frame length of 10 msec and subframe length of 1 msec.

- Keep scalable numerology based on 15 kHz.

- A PRB structure including common RB grid of 5G is used as a baseline of 6G resource grid.

Flexible/dynamic TDD operation has become increasingly necessary as operators and vendors have observed performance bottlenecks in 5G deployments caused by static TDD slot patterns. These include uplink starvation/delay, inefficient UL/DL scheduling, particularly in dense deployments.. Though 5G supports dynamic TDD from Rel-15, practically due to CLI, regulation restriction, etc., coordinated semi-static TDD has been widely used. It is noted that 3GPP studied/specified CLI aspects for dynamic TDD later release, which becomes a bit challenging to be widely deployed.

To address these known issues, 6G should aim to support frame-level flexibility that allows dynamic TDD configurations, preferably per cell or per carrier, along with CLI support from Day1. Such designs may enable traffic-aware switching of DL/UL direction, while considering CLI mitigation mechanisms such as guard symbols and interference measurements/power control. These enhancements would be especially important for uplink-intensive services or networks with asymmetric traffic patterns. Moreover, CLI migration between 5G and 6G should be supported to support MRSS effectively.

Proposal 3. Study CLI between 5G and 6G as well as between 6G RANs.

An anchor/non-anchor carrier structure is discussed by multiple companies [2-4]. The anchor/non-anchor carrier structure refers to a framework where certain carriers (anchor carriers) host essential signals such as synchronization (SSB), common channels (e.g., type0-PDCCH), paging, and system information, while other carriers (non-anchor carriers) carry user data and other control/data. Always-on signals may be limited to the anchor carrier, while non-anchor carriers can adopt various on-demand transmissions of various signals.

A few contributions from 6G workshop proposed anchor/non-anchor carrier structure for MRSS. By centralizing essential/always-on signals on anchor carriers, overhead in supporting coexistence between 5G and 6G can be contained within the anchor frequency layers, while based on almost no or lean always-on signals of non-anchor carriers, 6G can coexist with 5G more efficiently in non-anchor frequency layer.

Additionally, this structure also potentially provides better UE and network energy saving. For example, extensive cell searches or periodic monitoring by UEs can be limited to anchor carriers. Based on network assistance information, UE can measure aperiodic/on-demand signals on non-anchor carriers with less power consumption. That is, tight/periodic management on measurement and report to support radio link quality is required on the anchor carrier, whereas a more relaxed/aperiodic management is acceptable for non-anchor carrier.

Proposal 4. RAN1 further study on anchor/non-anchor architecture, where at least always-on signals are limited to anchor cells/frequency layers.

Numerology and Slot/symbol structure

In this section, the most fundamental timing and frequency grid of 6G, forming the basis for MRSS compatibility, frame alignment, and scheduling flexibility is discussed.

5G-NR carrier can be associated with multiple subcarrier spacings, e.g., to support various service requirements, e.g., uRLLC, eMBB, and mMTC. For example, a part of a carrier can be configured to be lower SCS to provide a larger coverage and other part of the carrier can be configured to be higher SCS to achieve lower latency from network point of view. However, multiple numerology in a band/cell may complicate CLI as well as UE / base station complexity. Based on operator inputs, it seems multiple numerologies for a cell do not bring considerable gain compared to the overhead. While RAN1 can focus on a single numerology/subcarrier per frequency band, study on remaining issues how to effectively support various services may be still necessary.

Other potential issue induced by having single numerology per carrier/frequency band in 6G network may be on the rate-matching for MRSS. Since 5G supports flexible configuration of the numerology per carrier/frequency band, 5G network support flexibility to indicate time/frequency resources, to possibly avoid where 6G data is scheduled. But the opposite way, i.e., 6G network rate-matching to avoid where 5G data is scheduled, may cause waste of the resource for 6G network. However, to minimize the impact of it, at least scalable numerology of 15kHz×2n is still desirable, at least for the case when 5G and 6G networks has same numerology.

Proposal 5. 6G study focus on:

- Scalable numerology of 15 kHz x 2n

- Focus a SCS per frequency band, evaluate candidate SCSs per frequency range/per subset of frequency range/frequency band

- Consider how to address various service requirements for verticals with a single SCS

Although maximum size of FFT is related to the maximum channel bandwidth, there are some of the other factors that need to be considered, for example, the relationship between the occupied bandwidth and the FFT sampling rate, including guard bands and implementation margins, and possible differences between channel bandwidth and maximum processing bandwidth of the UE. Note that the maximum channel bandwidth is typically defined by regulatory requirements for the operating frequency band. Considering this, discussion on maximum FFT size can focus on aspects related to such regulatory bandwidth definitions, together with 6G specific constraints.

For example, if simultaneous transmission and/or reception on non-contiguous frequency bands (to utilize fragmented spectrum) is to be supported, it should be clarified whether the maximum processing bandwidth of the UE is defined as the maximum span covering the non-contiguous spectrum, or the maximum bandwidth defined separately for each non-contiguous part, as in the current NR CA specification where the maximum supported bandwidth is defined per component carrier and not over the entire aggregated span. Such definition impacts UE capability signalling, RF/baseband architecture assumptions, and the calculation of the maximum FFT size. The FFT sizes of 2048, 4096, and 8192, together with channel bandwidths of 100 MHz, 200 MHz, and 400 MHz, are selected to cover the range from typical FR1 deployments to the current NR maximum (4096 FFT with 400 MHz) and potential 6G extensions, ensuring alignment with both existing capabilities and future scalability.

Accounting for those aspects, we propose to evaluate the followings in consideration of UE complexity for FFT size and channel bandwidth.

Proposal 6. RAN1 to evaluate FFT size of 2048, 4096, and 8192 with channel bandwidth of 100 MHz, 200 MHz and potentially 400 MHz.

In 5G, Bandwidth Part (BWP) was introduced primarily to enable UE power saving by allowing the device to operate over a reduced bandwidth instead of continuously processing the full channel bandwidth. It should be noted that there are several additional benefits on top of that. It enables frequency-domain resource optimization by allowing the network to assign different BWPs to different UEs, therefore enabling efficient multiplexing across the cell. Moreover, BWP can improve interference management by properly restricting reception to narrower frequency ranges, which can be beneficial in scenarios with strong interference outside the allocated BWP. And BWP allows adaptation to UE capability limitations, enabling UEs with different maximum processing bandwidths to coexist within the same cell without impacting each other’s operation. Furthermore, it can reduce the CSI measurement burden by transmitting CSI-RS over a narrower BWP, thereby lowering measurement overhead and processing complexity for the UE.

In the context of 6G with a single SCS per carrier, BWP configuration can be simplified without associated with a specific SCS. Considering the advantages, such as resource optimization, interference management, capability adaptation, and reduced measurement overhead, in our view, BWP framework can be considered for 6G design. For example, BWP can be still considered to adapt frequency resources, e.g., for power saving, by dynamic switching from narrow bandwidth to larger (or vice versa). Nevertheless, current BWP operation introduces certain drawbacks, such as configuration complexity due to per-UE-specific BWP switching and additional switching latency caused by the need for RRC reconfiguration or updates. To address these aspects, BWP operation can be further enhanced or simplified. For example, instead of per-UE-specific BWP switching, group-based or cell-specific BWP switching can be considered to reduce complexity, and BWP switching latency can be reduced by simplifying the necessary RRC reconfiguration or update procedures.

Proposal 7. Study simplified BWP mechanism to allow frequency region adaptation while minimizing UE complexity and signalling overhead.

While 6G could adopt different slot and symbol durations from 5G, reusing existing 5G hardware is desirable, not only to reduce migration complexity but also to enable more efficient MRSS. A shared hardware platform for 5G and 6G allows tighter alignment in frame structure and timing, reducing cross-generation scheduling conflicts and simplifying coexistence procedures. In contrast, differing slot and symbol durations would begin another discussion. For example, timeline for procedures, such as HARQ-ACK timing, CSI measurement and/or reporting, need to be revised, and relevant higher-layer timing parameters need to be modified. Moreover, oscillators, sampling rates, and filtering characteristics in 5G may not be reused which may increase cost and effort hardware design. On the other hand, maintaining compatibility in slot and symbol structure simplifies scheduling and frame alignment for coexisting systems, and enables smoother MRSS operation between 5G and 6G. Under the shared hardware assumption, reusing 5G NR slot/symbol duration is desirable.

The extended cyclic prefix (CP), introduced in 5G NR to support scenarios with very large delay spreads (e.g., certain macro-cell or high-delay multipath environments), has seen limited practical use in commercial deployments. Supporting both normal and extended CP modes increases transmitter and receiver design complexity, e.g., buffer management due to different symbol length including CP, and numerology handling, since each CP length is coupled with specific symbol durations and subcarrier spacings. In 6G, it may be worth reconsidering whether extended CP is really needed. Removing it could make the physical layer design simpler, and the impact on performance would likely be small if other techniques are used to handle inter-symbol interference in difficult propagation environments. For example, based on the expected delay spread, adding guard symbols, using adaptive guard periods can be considered.

Proposal 8. Discuss the followings options for harmonized slot and symbol structure:

- Reuse 5G NR frame/subframe/slot/symbol durations

- Evaluate the necessity of extended CP in 6G

Multi-Carrier and Spectrum aggregation

A few companies have proposed enhancements to the current carrier aggregation (CA) framework. One example is to allow a single cell to be associated with multiple downlink carriers, multiple uplink carriers, and/or uplink-only carriers, instead of the current assumption where a single cell is associated with a single downlink carrier.

According to TS 38.300, a cell is described as follows: “The multi-carrier nature of the physical layer is only exposed to the MAC layer, for which one HARQ entity is required per serving cell” and “In both uplink and downlink, there is one independent hybrid-ARQ entity per serving cell and one transport block is generated per assignment/grant per serving cell in the absence of spatial multiplexing. Each transport block and its potential HARQ retransmissions are mapped to a single serving cell.”

One potential benefit of associating multiple carriers with a single cell is the possibility to transmit or retransmit a single TB across multiple carriers, which may improve spectral efficiency and reduce HARQ entity overhead. However, this would also require enhancements to MAC and physical layer procedures, and could increase complexity on the UE side in terms of carrier-specific synchronization, channel estimation, and HARQ process mapping.

In our view, before discussing such CA enhancements directly, it is important to identify the pain points of the current CA framework. For example, companies have mentioned redundant RRC configurations across multiple cells. In a virtual or logical cell grouping multiple carriers (e.g., using the cell group concept in 5G), redundant RRC configuration may be suppressed. Another motivation is to enable more flexible control channel scheduling. Although 5G already supports many flexible scheduling options, such as multi-cell scheduling, multi-slot scheduling, and cross-carrier and self-carrier scheduling for the PCell, there are still limitations. For example, dynamic switching from self-carrier to cross-carrier scheduling (or vice versa) for a cell is not supported, and the scheduling cell for a given cell is relatively static.

In a 6G multi-carrier scenario, the scheduling framework may need to support dynamic selection of the scheduling cell based on spectrum availability, instantaneous traffic demand, and network energy-saving strategies such as cell on/off operations. This would require coordination mechanisms to ensure HARQ timing consistency and to minimize control channel signaling overhead during scheduling cell changes.

Proposal 9. Discuss limitations of 4G/5G carrier aggregation frameworks and investigate potential enhancements including

- Introduction of a logical single-cell across multiple carriers

- Flexible/dynamic self-/cross-carrier scheduling switching and scheduling cell switching

- Cell/carrier grouping to share common configuration parameters/functionalities across multiple cells/carriers.

Physical layer control and scheduling

In 5G NR, the continuous presence of always-on control channels, such as PDCCH monitoring across the entire CORESET, introduces a significant power consumption burden on the UE. 5G offers various control channel monitoring adaptation such as search space set switching, PDCCH skipping, different CORESET configurations via BWP switching and cell DTX/DRX. In 6G, further enhancements on power saving can be considered. For example, based on dynamic on/off of multi-TRP and/or cells/carriers, monitoring behavior of control channel/search space can be accordingly adapted including dynamic coreset/search space set on/off based on predicted traffic patterns or beam management events.

Another source of control overhead in 5G NR is the frequent transmission of broadcast signals such as SSB and paging. While this ensures robust initial access and idle mode tracking, it also results in substantial overhead, particularly in low-mobility or infrequent-access scenarios. In 6G, longer SSB periodicity or asynchronous/on-demand SSB transmission can reduce such overhead while still ensuring timely system information delivery. For instance, SSB transmission could be triggered only upon beam change or mobility events. These schemes are particularly beneficial for energy-constrained devices and sparse traffic use cases, and may also be beneficial for interference management in dense deployments by reducing constant synchronization signal transmissions.

In 5G NR, HARQ processes are managed independently per serving cell, which can cause uplink control channel collisions when HARQ feedback occasions from different carriers overlap. Allowing cross-carrier HARQ process multiplexing and re-sequencing, i.e., reassigning HARQ process IDs and feedback timing to avoid such overlaps, could improve resource utilization, reduce collision probability, and better support dynamic scheduling cell selection in multi-carrier operation. This approach could also enhance overall scheduling flexibility in scenarios where carriers are dynamically activated or deactivated for spectrum efficiency or energy-saving purposes.

Another aspect for physical layer control and scheduling is small data transmission. Current grant-based scheduling is inefficient for small or sporadic transmissions, as the signaling overhead can exceed the data payload. 6G should support lightweight grant-free mechanisms or one-step random access procedures optimized for small packets, enabling reduced latency and overhead. This could include uplink resource pre-allocation for known small-packet patterns, and flexible mini-slot or sub-slot allocations to avoid full-slot occupancy.

Proposal 10. Discuss the followings for physical layer control and scheduling:

- Adopt control channel adaptation techniques such as search space set switching with potential simplification and enhancements

- Reduction of always-on signals based on on-demand signals

- Efficient HARQ operation e.g., cross-carrier HARQ operation, simplification of feedback timing

Duplexing

In NR, three main duplexing modes are supported: time division duplex (TDD), frequency division duplex (FDD), and subband full duplex (SBFD). In current deployments, TDD operation follows a fixed DL/UL slot configuration within each carrier, while FDD maintains a fixed separation of uplink and downlink spectrum with no bandwidth adaptation. SBFD, introduced in Rel-19, allows simultaneous uplink and downlink in different subbands within the same carrier bandwidth. However, the UL/DL subband allocation pattern is statically configured and fixed for the duration of operation.

In 6G discussions, dynamic TDD has been proposed to adjust the DL/UL split on a subframe- or slot-basis to respond to traffic demand or interference conditions. Adaptive FDD aims to dynamically adjust uplink and downlink bandwidth allocation within a paired spectrum, enabling more efficient utilization when traffic is asymmetric. If the current SBFD constraints on static UL/DL subband allocation were relaxed to allow time and frequency domain reconfiguration (e.g., reassigning subbands between UL and DL dynamically) SBFD could achieve similar benefits to both dynamic TDD and adaptive FDD. This relaxation would allow UL/DL pattern adaptation on a per-slot or per-symbol basis, enabling interference coordination with higher time/frequency resolution and more efficient resource utilization under varying traffic and interference conditions, while reusing much of the existing NR SBFD framework. Moreover, in 5G SBFD, half-duplex is assumed for a UE while network can perform SBFD. Considering wider carrier bandwidth in 6G, a larger frequency separation between a UL subband and a DL subband (e.g., a guard band) in a carrier may become feasible such that SBFD operation at UE side could be also feasible without complicated self-interference cancellation/management.

On the other hand, in-band full duplex (IBFD) operation, enabling simultaneous transmission and reception on the same frequency, offers potential benefits in terms of spectral efficiency and latency reduction. However, realizing IBFD in practice requires advanced self-interference cancellation, high levels of frequency-domain isolation, and tightly coordinated scheduling, all of which remain technically challenging. In addition, implementation complexity and hardware cost could be significant, particularly for wideband operation. Therefore, in 6G it would be reasonable to further study IBFD, including their feasibility, potential gains, and practical deployment considerations.

Another duplex scenario requiring attention in 6G is non-terrestrial network (NTN) integration. Satellites typically operate in FDD with long round-trip times, while many NTN IoT devices are limited to half-duplex capability. To accommodate these constraints, 6G frame structures should enable seamless switching between terrestrial and non-terrestrial operation, support flexible scheduling, and allow for extended transmission time intervals (TTIs) when necessary to handle large propagation delays.

While SBFD has so far considered mainly at the gNB side, enabling UE-side SBFD is essential for maximizing the potential of subband-based duplexing in 6G. For example, in a wideband carrier, simultaneous reception and transmission via DL and UL subbands which are apart sufficiently may be considered. Frequency resources between DL and UL subbands can be considered as guard bands.

Proposal 11. For 6G, TDD, FDD and SBFD are supported.

- For TDD, dynamic TDD is supported.

- For FDD, consider whether dynamic adaptation of DL and UL carrier bandwidth and/or locations and/or pairing are supported

- For SBFD, consider dynamic UL/DL subband allocation as well as switching between SBFD and normal DL/UL slots. Further, consider feasibility of UE side SBFD.

- Design NTN-aware duplex structures for long delays and half-duplex UEs

Conclusion

In this paper we make the following proposals:

Proposal 1. RAN1 to further study a feasibility and drawback to support “At most one subcarrier spacing for a frequency band”.

Proposal 2. 6G frame structure is designed based on 5G frame structure.

- Keep frame length of 10 msec and subframe length of 1 msec.

- Keep scalable numerology based on 15 kHz.

- A PRB structure including common RB of 5G is used as a baseline of 6G resource grid.

Proposal 3. Study CLI between 5G and 6G as well as between 6G RANs.

Proposal 4. RAN1 further study on anchor/non-anchor architecture, where at least always-on signals are limited to anchor cells/frequency layers.

Proposal 5. 6G study focus on:

- Scalable numerology of 15 kHz x 2n

- Focus a SCS per frequency band, evaluate candidate SCSs per frequency range/per subset of frequency range/frequency band

- Consider how to address various service requirements for verticals with a single SCS

Proposal 6. RAN1 to evaluate FFT size of 2048, 4096, and 8192 with channel bandwidth of 100 MHz, 200 MHz and potentially 400 MHz.

Proposal 7. Study simplified BWP mechanism to allow frequency region adaptation while minimizing UE complexity and signalling overhead.

Proposal 8. Discuss the followings options for harmonized slot and symbol structure:

- Reuse 5G NR frame/subframeslot/symbol durations

- Evaluate the necessity of extended CP in 6G

Proposal 9. Discuss limitations of 4G/5G carrier aggregation frameworks and investigate potential enhancements including

- Introduction of a logical single-cell across multiple carriers

- Flexible/dynamic self-/cross-carrier scheduling switching and scheduling cell switching

- Cell grouping to share common configuration parameters/functionalities.

Proposal 10. Discuss the followings for physical layer control and scheduling:

- Adopt control channel adaptation techniques such as search space set switching with potential simplification and enhancements

- Reduction of always-on signals based on on-demand signals

- Efficient HARQ operation e.g., cross-carrier HARQ operation, simplification of feedback timing

Proposal 11. For 6G, TDD, FDD and SBFD are supported.

- For TDD, dynamic TDD is supported.

- For FDD, consider whether dynamic adaptation of DL and UL carrier bandwidth and/or locations and/or pairing are supported

- For SBFD, consider dynamic UL/DL subband allocation as well as switching between SBFD and normal DL/UL slots. Further, consider feasibility of UE side SBFD.

- Design NTN-aware duplex structures for long delays and half-duplex UEs

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO (Moderator).

- 6GWS-250004, “6G Radio design principles”, Nokia

- 6GWS-250036, “Vision and Technologies for 6G Radio”, Samsung

- 6GWS-250052, “Vision & priorities for 6G radio technology”, NTT DOCOMO